FURNITURE UPHOLSTERER

KM02 UPHOLSTERED FURNITURE FRAME PREPARATION

ASSESSMENT GUIDE

TABLE OF CONTENTS

ASSESSMENT REQUIREMENTS	3

TOPIC ELEMENTS TO BE COVERED INCLUDE:	4

KM-02-KT01: Specifications for upholstered furniture (10%)	5

Integrated Assessment – KM-02-KT01: Specifications for Upholstered Furniture	6

KM-02-KT02: Upholstery raw materials and consumables types and characteristics (15%)	8

Integrated Assessment – KM-02-KT02: Upholstery Raw Materials and Consumables	9

KM-02-KT03: Raw materials and consumables used in the frame padding process (5%)	12

KM-02-KT04: Use of chemicals: adhesives and solvents (5%)	16

KM-02-KT05: Upholstery tools and equipment (10%)	19

🔧 Integrated Assessment – KM-02-KT05: Upholstery Tools and Equipment	20

Integrated Assessment – KM-02-KT07: Upholstered Furniture Manufacturing Processes	25

KM-02-KT08: Frame preparation: knocking-on principles (5%)	27

KM-02-KT09: Knocker on techniques (15%)	30

Integrated Assessment – KM-02-KT09: Knocker-On Techniques	31

KM-02-KT10: Foaming-up or padding-up principles (10%)	32

Integrated Assessment – KM-02-KT10: Foaming-up or Padding-up Principles	32

KM-02-KT11: Foaming-up or padding-up techniques (10%)	35

Integrated Assessment – KM-02-KT11: Foaming-up or Padding-up Techniques	36

KM-02-KT12: Joints and bracings of the frame (5%)	39

Facilitator Briefing Note	39

Integrated Assessment – KM-02-KT12: Joints and Bracings of the Frame	41

# ASSESSMENT REQUIREMENTS

Integrated Formative Assessment:

The skills development provider will use the curriculum to guide them on the stipulated internal assessment criteria and weighting. They will also apply the scope of practical skills and applied knowledge as stipulated by the internal assessment criteria. This formative assessment leads to entrance into the integrated external summative assessment.

Integrated Summative Assessment:

An external integrated summative assessment, conducted through the relevant QCTO Assessment Quality partner is required for the issuing of this qualification. The external integrated summative assessment will focus on the exit level outcomes and associated assessment criteria. The external assessment model requires that the external assessment will be conducted through a combination of a written assessment and practical task at an accredited trade test centre. The written examination will be concluded at an accredited trade test centre and marked by registered assessors. Practical tasks will also be assessed by registered assessors. The combination of the written and practical assessment will be conducted over a period of two working days.

# TOPIC ELEMENTS TO BE COVERED INCLUDE:

Purpose of the Knowledge Modules

The main focus of the learning in this knowledge module is to build an understanding of upholstery design, construction specifications, materials, equipment and production techniques associated with preparing an upholstered furniture frame for covering by hand building the suspension.

The learning will enable learners to demonstrate an understanding of:

KM-02-KT01: Specifications for upholstered furniture (10%)

KM-02-KT02: Upholstery raw materials and consumables types and characteristics (15%)

KM-02-KT03: Raw materials and consumables used in the frame padding process (5%)

KM-02-KT04: Use of chemicals: adhesives and solvents (5%)

KM-02-KT05: Upholstery tools and equipment (10%)

KM-02-KT06: Upholstery measurements, measuring tools, techniques and calculations (5%)

KM-02-KT07: Upholstered furniture manufacturing processes (5%)

KM-02-KT08: Frame preparation: knocking-on principles (5%)

KM-02-KT09: Knocker on techniques (15%)

KM-02-KT10: Foaming-up or padding-up principles (10%)

KM-02-KT11: Foaming-up or padding-up techniques (10%)

KM-02-KT12: Joints and bracings of the frame (5%)

# KM-02-KT01: Specifications for upholstered furniture (10%)

Formative assessment

IAC0101 Various types of product specifications used in the upholstery industry are compared and differentiated.

IAC0102 Various aspects of basic sketches and engineering drawings such as isometric, oblique and orthographic views and projections, sectional views, component parts, dimensioning, scaling, line structures and geometry, drawing labelling, curves and radiuses are identified and each function explained.

IAC0103 Aspects included in industry and workplace product specification concepts such as raw material standards, drawing standards, engineering symbols, abbreviations and conventions are identified.

IAC0104 The concepts of compliance and non-compliance is explained.

IAC0105 The consequences for the upholstery process of prepared and covered frames not meeting the specified product standard are evaluated

(Weight 10%)

## Integrated Assessment – KM-02-KT01: Specifications for Upholstered Furniture

Weight: 10%

Assessment Type: Integrated Formative Assessment using a variety of instruments

Assessment Instruments

- Multiple Choice Questions (IAC0101, IAC0103): Specification types and industry concepts
- Short Answer Questions (IAC0102, IAC0104): Drawings and compliance concepts
- Practical Identification Task (IAC0101, IAC0102, IAC0103): Drawing components and symbols
- Case Study Scenario (IAC0104, IAC0105): Compliance and consequences

Section A: Multiple Choice Questions (5 marks)

1. Which of the following is NOT a type of product specification?
   A) Sketch
   B) Job card
   C) Angle grinder
   D) Technical drawing

2. A cutting list is mainly used to:
   A) Describe upholstery fabrics
   B) Guide frame measurements and materials
   C) List employee job roles
   D) Replace technical drawings

3. Engineering symbols are used in:
   A) Marketing reports
   B) Upholstery foam grading
   C) Specification drawings
   D) Delivery notes

4. The abbreviation “Ø” on a drawing refers to:
   A) Radius
   B) Diameter
   C) Depth
   D) Circumference

5. A prototype is best described as:
   A) A final product ready for sale
   B) A visual sketch
   C) A first version of a design for testing
   D) A material sample

Section B: Short Answer Questions (15 marks)

1. Explain the difference between orthographic and isometric views. (5)
2. Define “compliance” and provide one example from the upholstery process. (5)
3. What is the purpose of drawing standards in the furniture manufacturing industry? (5)

Section C: Practical Identification Task (15 marks)

Instructions: Learners receive a printed technical drawing and must:
- Label: orthographic projection, isometric view, sectional view (6 marks)
- Identify and explain three symbols used in the drawing (9 marks)

Section D: Case Study Scenario (10 marks)

Scenario: Kyle, a junior upholsterer, is preparing a set of ottoman frames... (text summarised)

Questions:
1. Identify compliance or non-compliance and justify. (2)
2. List three consequences of Kyle’s mistake. (3)
3. Suggest two preventative actions. (2)
4. What documents could Kyle have consulted? (3)

Model Answers

Provided for all sections A to D as per chat content.

Marking Memo

Section A: 5 marks
Section B: 15 marks
Section C: 15 marks
Section D: 10 marks
Total: 45 marks
Pass Mark: 31.5 (70%)

Assessment Rubric

Criteria evaluated on a 1–5 scale: Specification types, Drawing interpretation, Industry concepts, Compliance awareness, Evaluation of consequences.

# KM-02-KT02: Upholstery raw materials and consumables types and characteristics (15%)

Formative assessment

IAC0201 The use of specifications and workplace instructions to determine the type, quality, sizes and quantities of raw materials and consumables for the frame preparation process is reasoned.

IAC0202 Various types of frames and their raw materials and type of foundation materials to be used for each component are identified.

IAC0203 Types and gauges of various springs needed for the knocker-on process are identified including coil springs and zig-zag springs.

IAC0204 Types, sizes, quantity and quality of consumables such as clips, nails, staples, edge rolls, twine and cord needed for the springing up process are identified.

IAC0205 Differences between the types of webbing needed for the knocker on process, such as manmade elastic and rubber webbing, and natural fibres like jute webbing are evaluated.

IAC0206 Types, sizes, quantity and quality of consumables such as clips, tacks, and staples needed for the webbing up process are identified.

IAC0207 Methods for identifying density and thickness of various types of foam required for the foaming up process are described.

IAC0208 Consumables required for the foaming up process are listed.

IAC0209 Raw materials required for the padding and covering of coil springs are listed.

IAC0210 Fabrics used for covering webbing, springs, foam and padding including calico, hessian (burlap), polypropylene, Dacron, cambric and flock are listed.

IAC0211 Principles and concepts of safe materials handling and storage methods are reasoned.

(Weight 15%)

## Integrated Assessment – KM-02-KT02: Upholstery Raw Materials and Consumables

Weight: 15%

Assessment Type: Integrated Formative Assessment – Case Study with Scenario-Based Questions

Case Study

Sibongile is a junior upholsterer in a mid-sized furniture production workshop. Her team receives an order to produce a batch of ten high-end lounge chairs for a corporate client. Each chair must be built using high-density foam, coil springs for the seats, jute webbing on the base, and specific fabrics such as hessian and Dacron for internal covering.

The production supervisor provides her with a job card detailing the frame type (a reinforced hardwood structure), and a materials list including spray adhesive, edge rolls, twine, and a set number of staples and clips per chair. All materials are stored in the back storeroom, which has poor ventilation and inconsistent labelling on shelves.

Sibongile begins the process but struggles with identifying spring gauges and mistakenly uses 9-gauge zig-zag springs instead of coil springs. She also uses elastic webbing in place of jute because the jute was stored in an unlabelled box.

Scenario-Based Questions

1. Based on the job card, what steps should Sibongile have taken to ensure she selected the correct type, quality, and quantity of raw materials and consumables? (IAC0201)

2. Identify the type of frame specified and explain why a reinforced hardwood structure is suitable for high-end lounge chairs. (IAC0202)

3. What is the difference between the 9-gauge zig-zag springs Sibongile used and the coil springs specified? When is each type appropriate? (IAC0203)

4. List the consumables Sibongile should have used during the springing-up process. Comment on quantity and quality expectations. (IAC0204)

5. Evaluate the implications of using elastic webbing instead of jute webbing for this type of furniture. (IAC0205)

6. Identify the correct consumables for the webbing-up process and explain how they differ in purpose or application from those used in springing-up. (IAC0206)

7. How could Sibongile determine the correct density and thickness of foam for the lounge chairs? (IAC0207)

8. List the consumables and materials she would need for the foaming-up process. (IAC0208)

9. What raw materials should be used to pad and cover the coil springs before applying the outer fabric? (IAC0209)

10. List the fabrics used to cover webbing, springs, foam and padding in this scenario. (IAC0210)

11. Discuss the risks associated with poor materials handling and storage, as observed in the workshop’s back storeroom. Suggest improvements. (IAC0211)

Model Answers

Answers correspond to the scenario-based questions and should demonstrate reasoning, identification, listing, and evaluation skills aligned to IAC0201–IAC0211.

Marking Memo

Each question is worth approximately 5 marks. Full marks awarded for clear, technically accurate, and complete answers. Partial marks may be awarded for partially correct responses.

Assessment Rubric

| Criteria | Excellent (5) | Competent (3-4) | Needs Support (1-2) |
| --- | --- | --- | --- |
| Understanding of material specifications and job instructions (IAC0201) | Demonstrates excellent understanding and applies all concepts correctly with relevant examples. | Generally correct responses with minor errors or omissions. | Limited understanding; key concepts misunderstood or missing. |
| Identification of frame types and materials (IAC0202) | Demonstrates excellent understanding and applies all concepts correctly with relevant examples. | Generally correct responses with minor errors or omissions. | Limited understanding; key concepts misunderstood or missing. |
| Understanding of spring types and gauges (IAC0203) | Demonstrates excellent understanding and applies all concepts correctly with relevant examples. | Generally correct responses with minor errors or omissions. | Limited understanding; key concepts misunderstood or missing. |
| Listing and describing consumables (IAC0204–IAC0206) | Demonstrates excellent understanding and applies all concepts correctly with relevant examples. | Generally correct responses with minor errors or omissions. | Limited understanding; key concepts misunderstood or missing. |
| Foam identification and application (IAC0207–IAC0208) | Demonstrates excellent understanding and applies all concepts correctly with relevant examples. | Generally correct responses with minor errors or omissions. | Limited understanding; key concepts misunderstood or missing. |
| Fabric and padding application (IAC0209–IAC0210) | Demonstrates excellent understanding and applies all concepts correctly with relevant examples. | Generally correct responses with minor errors or omissions. | Limited understanding; key concepts misunderstood or missing. |
| Storage safety and reasoning (IAC0211) | Demonstrates excellent understanding and applies all concepts correctly with relevant examples. | Generally correct responses with minor errors or omissions. | Limited understanding; key concepts misunderstood or missing. |

# KM-02-KT03: Raw materials and consumables used in the frame padding process (5%)

Formative assessment

IAC0301 Types of natural raw materials used as padding for upholstered furniture are identified and the respective uses and characteristics are evaluated.

IAC0302 Types of man-made raw materials used as padding for upholstered furniture are identified and the respective uses and characteristics are evaluated.

IAC0303 The importance of safe handling and storage of raw materials is justified.

(Weight 5%)

Integrated Assessment – KM-02-KT03: Raw Materials and Consumables Used in the Frame Padding Process

Weight: 5%

Assessment Instruments Overview

MCQs: IAC0301 & IAC0302
Matching Activity: IAC0301 & IAC0302
Short Answer Questions: IAC0303
Case Study & Scenario: IAC0301–IAC0303

Section A: Multiple Choice Questions (5 marks)

1. Which of the following is a natural padding material?
   A) Memory foam
   B) Polyester fibre
   C) Kapok
   D) Polyurethane

2. Coir is made from:
   A) Cotton lint
   B) Tree bark
   C) Coconut husk fibres
   D) Plastic recycling

3. Which of the following is a man-made padding material?
   A) Dacron
   B) Coir
   C) Wool
   D) Hessian

4. Foam density refers to:
   A) The colour of the foam
   B) Its ability to float
   C) The firmness of the foam
   D) The mass per cubic metre

5. Which of the following is most suitable for lightweight cushion filling?
   A) Jute
   B) Dacron
   C) Kapok
   D) Edge roll

Section B: Matching Activity (6 marks)

Match the padding material in Column A with its correct use in Column B. (1 mark each)

Column A:
1. Coir
2. High-density foam
3. Kapok
4. Dacron
5. Low-density foam
6. Cotton felt

Column B:
A. Final smoothing layer for outer fabric
B. Lightweight cushion filling
C. Used in traditional firm seat foundations
D. Used for soft armrests or backrests
E. Enhances comfort and reduces fabric friction
F. Smooths spring lines in premium furniture

Section C: Short Answer Questions (9 marks)

1. Explain two safety practices that should be followed when storing kapok or coir in a workshop. (3 marks)

2. List two potential risks of storing foam incorrectly. (2 marks)

3. Describe how proper labelling and organisation of raw materials can improve safety and efficiency. (4 marks)

Section D: Case Study Scenario (5 marks)

Scenario: Thabiso is working with a new team and is asked to prepare padding for a batch of ottomans. He selects unlabelled padding from a shelf and uses it across all ten units. The customer later complains that the ottomans feel too soft and do not hold their shape. On inspection, the team realises Thabiso used low-density foam instead of the specified high-density foam and did not apply a Dacron layer to smooth the finish.

Questions:
1. Identify two mistakes Thabiso made. (2 marks)
2. What effect did these mistakes have on product quality? (2 marks)
3. How could these issues have been prevented through safe handling and storage? (1 mark)

Marking Memo

Total Marks: 25
Pass Mark (70%): 18

Assessment Rubric

| Criteria | Excellent (5) | Competent (3–4) | Needs Support (1–2) |
| --- | --- | --- | --- |
| IAC0301: Natural padding materials | Clearly identifies and evaluates materials with correct uses. | Generally accurate but lacks some detail or depth. | Limited or incorrect understanding of material or concept. |
| IAC0302: Man-made materials | Clearly identifies and evaluates materials with correct uses. | Generally accurate but lacks some detail or depth. | Limited or incorrect understanding of material or concept. |
| IAC0303: Safe handling and storage | Clearly identifies and evaluates materials with correct uses. | Generally accurate but lacks some detail or depth. | Limited or incorrect understanding of material or concept. |

# KM-02-KT04: Use of chemicals: adhesives and solvents (5%)

Formative assessment

IAC0401 The various types of chemicals used in upholstery processes are named.

IAC0402 Characteristics and properties of the chemicals used in upholstery processes are described.

IAC0403 The purpose of each of these chemicals used in the process is explained.

IAC0404 Various types of solvents used in upholstery processes are listed.

IAC0405 Characteristics and properties of these solvents are explained.

IAC0406 The purpose of each solvent used in the process is explained.

IAC0407 The application techniques and sequence used for chemicals and solvents are described.

IAC0408 Safety procedures, hazards and risks associated with handling of chemicals and solvents are described.

IAC0409 Standard operating procedures dealing with spillages, waste and effluent are explained.

IAC0410 Types, usage and care of personal protective equipment used in textile wet product preparation processes are described.

(Weight 5%)

Integrated Assessment – KM-02-KT04: Use of Chemicals – Adhesives and Solvents

Qualification: Furniture Upholsterer (SAQA ID 103199)
Knowledge Module: KM-02-KT04
Assessment Type: Integrated Formative Assessment using a Case Study
Weight: 5%

Case Study

Lungile is working in the foaming-up section of a busy upholstery workshop. Her task is to bond layers of foam and Dacron for a batch of seat cushions. She selects a solvent-based spray adhesive due to its fast-drying properties. She works in a poorly ventilated corner of the workshop without a respirator or gloves. She places the open adhesive can near a fan heater for warmth.

Later, she moves on to another task and leaves the spray gun and half-empty solvent container open. A co-worker using a heat gun nearby unknowingly ignites the vapour-rich air, causing a flash fire. The fire is quickly contained, but the supervisor initiates an investigation and safety review.

During the review, it is revealed that Lungile was not fully trained on the application sequence for adhesives, the risks associated with solvents, or the safe storage of chemical products. The fire could have been avoided had the correct PPE, ventilation, and storage procedures been followed.

📝 Scenario-Based Questions

- List three types of chemicals commonly used in upholstery and give one example of each. (IAC0401)

- Describe two characteristics or properties of solvent-based adhesives. (IAC0402)

- Explain the purpose of each of the following chemicals:
a) Contact adhesive
b) Spray adhesive
c) Cleaning solvent (IAC0403)

- Identify two types of solvents that might be present in an upholstery workshop. (IAC0404)

- Describe the properties of the two solvents you listed in Question 4. (IAC0405)

- For each solvent above, explain its purpose in the upholstery process. (IAC0406)

- Outline the correct sequence of application when using contact adhesive. (IAC0407)

- Identify three hazards present in the case study and describe how each one could have been avoided. (IAC0408)

- State two standard operating procedures for dealing with spillages, waste, or chemical effluent. (IAC0409)

- List two types of personal protective equipment (PPE) Lungile should have worn. Describe how each item protects her during chemical use. (IAC0410)

Marking Memo

| Question | Marks |
| --- | --- |
| Q1 | 3 |
| Q2 | 2 |
| Q3 | 3 (1 per item) |
| Q4 | 2 |
| Q5 | 2 |
| Q6 | 2 |
| Q7 | 4 |
| Q8 | 3 |
| Q9 | 2 |
| Q10 | 2 |
| Total | 25 |
| Pass mark (70%) | 18 |

📊 Assessment Rubric

| Criteria | Excellent (5) | Competent (3–4) | Needs Support (1–2) |
| --- | --- | --- | --- |
| Knowledge of chemical types and solvents (IAC0401–IAC0406) | Accurately identifies, explains, and applies knowledge to scenario | Mostly accurate with minor gaps | Limited understanding or vague references |
| Understanding of application techniques (IAC0407) | Demonstrates clear procedural knowledge and reasoning | General steps correct with minor errors | Steps confused or incomplete |
| Hazard awareness and safety (IAC0408–IAC0410) | Risks clearly identified, and appropriate PPE and SOPs provided | Identifies some risks or PPE but lacks full justification | Missing or incorrect safety responses |

# KM-02-KT05: Upholstery tools and equipment (10%)

Formative assessment

IAC0501 Hand tools used by the frame preparing department are identified and their purposes are explained.

IAC0502 Parts of the hand tools are identified.

IAC0503 Maintenance procedures for each tool explained.

IAC0504 The consequences of not handling, cleaning and maintaining hand tools correctly are assessed.

IAC0505 Pneumatic power tools used in the frame preparing department are identified and their purposes explained.

IAC0506 Parts and safety mechanisms of the pneumatic power tools are identified and the maintenance procedures for each tool is explained.

IAC0507 The consequences of not handling, cleaning and maintaining pneumatic power tools correctly are assessed.

IAC0508 Electric power tools used in the frame preparation department are identified and respective purpose is explained.

IAC0509 Parts and safety mechanisms of the electric power tools are identified and maintenance procedures for each tool is assessed.

IAC0510 Consequences of not handling, cleaning and maintaining electric power tools correctly is evaluated.

IAC0511 The identification, selection and loading staples, nails, bits and other consumables into the relevant power tool are explained.

IAC0512 The consequences of not cleaning the spray equipment and spray booth thoroughly after use are assessed.

Formative assessment

IAC0601 Various units, conversions and terminology for measurements, including quantity, deflection, and compressed air volume and pressure are explained and applied.

IAC0602 Calculations required during the upholstery process are explained and applied.

IAC0603 Measuring tools such as steel tape measure, cloth tape measure, squares, steel rules, vernier calliper, templates and patterns, and pressure gauges and their parts are identified and their functions analysed.

IAC0604 Calculations involving dimensions, spacing, quantities, volume, and air pressure using consistent units and conversions and appropriate mathematical operation are completed, recorded and checked.

IAC0605 Consequences of inaccurate measurements on the upholstery process and the output of the department are assessed.

(Weight 5%)

## 🔧 Integrated Assessment – KM-02-KT05: Upholstery Tools and Equipment

Qualification: Furniture Upholsterer (SAQA ID: 103199)
Knowledge Module: KM-02-KT05
Assessment Type: Integrated Formative Assessment using a Variety of Instruments
Weight: 10%

Assessment Instruments Overview

| Instrument Type | Focus Area |
| --- | --- |
| Multiple Choice Questions (MCQs) | Tool identification, safety, purpose (IAC0501, IAC0505, IAC0508, IAC0513) |
| Matching Activity | Parts and components (IAC0502, IAC0506, IAC0509) |
| Short Answer Questions | Maintenance, fault-finding, SOPs (IAC0503, IAC0507, IAC0510, IAC0512) |
| Practical Simulation / Oral Questions | Loading staples and consumables, safe handling (IAC0511, IAC0504, IAC0507) |
| Case Study Scenario | Risk analysis, safety consequences, PPE, frame preparation (IAC0513, IAC0506) |

📝 Section A: Multiple Choice Questions (6 marks)

Choose the correct answer for each question. (1 mark each)

- Which of the following is a pneumatic tool?
A) Mallet
B) Electric drill
C) Staple gun connected to air supply
D) Hot glue gun

- Which PPE item is essential when using a spray adhesive in a booth?
A) Dust mask
B) Ear protection
C) Welding goggles
D) Respirator

- What is the main consequence of not cleaning spray equipment after use?
A) Increased adhesive strength
B) Equipment overheating
C) Blockages and equipment failure
D) Lower electricity use

- Which hand tool is used to tension jute webbing?
A) Tack hammer
B) Staple remover
C) Webbing stretcher
D) Glue spatula

- Which tool is used for cutting foam accurately?
A) Spray gun
B) Upholstery shears
C) Pneumatic gun
D) Dust mask

- What is a key safety mechanism found in electric staple guns?
A) Trigger lock
B) Temperature gauge
C) Wire loop
D) Rotary switch

🔀 Section B: Matching Activity (6 marks)

Match each tool in Column A to its part or component in Column B. (1 mark each)

| Column A | Column B |
| --- | --- |
| 1. Pneumatic staple gun | A. Nozzle, compressor hose, trigger |
| 2. Upholstery tack hammer | B. Magnetic head, grip handle |
| 3. Foam cutting electric knife | C. Blades, power cord, safety guard |
| 4. Adhesive spray gun | D. Pressure tank, nozzle, trigger |
| 5. Portable power drill | E. Chuck, speed setting dial, battery pack |
| 6. Electric staple gun | F. Power cord, trigger switch, safety tip |

✍️ Section C: Short Answer Questions (8 marks)

- Describe two maintenance procedures for hand tools used in frame preparation. (2 marks)

- What are two consequences of not cleaning or maintaining pneumatic tools? (2 marks)

- Outline one standard procedure for loading staples into a pneumatic gun. (2 marks)

- What are two risks of not wearing PPE when using electric foam cutting equipment? (2 marks)

🗣 Section D: Practical / Oral Assessment (3 marks)

Instructor Prompt: Demonstrate or explain the steps involved in:

- Identifying and safely loading nails or staples into a staple gun.

- Checking the safety lock mechanism.

- Storing the tool safely after use.

📖 Section E: Case Study Scenario (7 marks)

Scenario:
Nomvula is working in the frame preparation area. She uses an electric staple gun with worn parts and no functioning safety lock. After use, she stores it without unplugging it. She also neglects to clean the spray booth after applying adhesive to foam seat panels. Later that day, another worker tries to use the same equipment and experiences backflow of adhesive due to a blocked nozzle.

Questions:

- Identify two tools used incorrectly in this scenario. (2 marks)

- What are the safety risks involved in leaving equipment plugged in or uncleaned? (2 marks)

- What PPE should Nomvula have used during the adhesive application? (1 mark)

- What is one standard operating procedure for cleaning spray booths after adhesive application? (2 marks)

Marking Memo

| Section | Marks |
| --- | --- |
| Section A | 6 |
| Section B | 6 |
| Section C | 8 |
| Section D | 3 |
| Section E | 7 |
| Total | 30 |
| Pass Mark (70%) | 21 |

📊 Assessment Rubric

| Criteria | Excellent (5) | Competent (3–4) | Needs Support (1–2) |
| --- | --- | --- | --- |
| Tool identification and purpose (IAC0501, IAC0505, IAC0508, IAC0513) | Accurately identifies tools and explains use in detail | Mostly accurate identification and explanation | Limited or incorrect identification |
| Component recognition and maintenance (IAC0502, IAC0506, IAC0509) | Components and procedures clearly explained | General understanding with some gaps | Incomplete or vague |
| Safety, SOPs, and consequences (IAC0504, IAC0507, IAC0510–IAC0512) | Demonstrates awareness of hazards and applies SOPs | Adequate safety knowledge with minor errors | Poor safety understanding, unclear SOPs |

KM-02-KT07: Upholstered furniture manufacturing processes (5%)

Formative assessment

IAC0701 Workshop processes of producing upholstered items from raw frame to finished product and the implications of design and construction faults on the final product are analysed.

IAC0702 The process of furniture manufacture is briefly outlined.

IAC0703 The operations in furniture manufacture such as machining, assembling and finishing operations are reviewed.

IAC0704 The finishing processes of furniture is outlined.

IAC0705 The importance of productivity and methods to enhance productivity are discussed.

IAC0706 The role of the routing sheet is described.

IAC0707 Job card information such as component sizes and details, shoulder-to-shoulder size and chemicals to use is explained.

IAC0708 Product specifications are understood and their impact on the manufacturing process is discussed in terms of the process flow and methods that will be used.

(Weight 5%)

## Integrated Assessment – KM-02-KT07: Upholstered Furniture Manufacturing Processes

Weight: 5%

SECTION A: Case Study and Scenario-Based Questions

Case Study: Johan and Nandi are assigned to oversee a new batch of high-back chairs for a hospitality client. The routing sheet outlines 11 operations, starting from raw frame inspection to the final finishing stage. However, one frame enters the padding stage without the foam being applied evenly during foaming-up. As a result, the cover is misaligned, and the seat back appears asymmetrical. Upon review, it is found that the job card was missing the shoulder-to-shoulder measurement. The team also realised that the productivity in the foaming section dropped due to overlapping routing sheet tasks.

1. Identify two workshop processes that were not followed properly in this scenario. (IAC0701)

2. Briefly outline the typical upholstered furniture manufacturing process from raw frame to finished product. (IAC0702)

3. What are the three key operations involved in the manufacture of upholstered furniture? (IAC0703)

4. Describe one finishing process used in upholstered furniture production. (IAC0704)

5. Discuss one design or construction fault that may affect the quality of the final product. (IAC0701)

SECTION B: Short Answer Questions

6. Explain the importance of productivity in a furniture workshop. (IAC0705)

7. List and briefly explain two methods used to improve productivity. (IAC0705)

8. What is the function of a routing sheet, and why must it be followed? (IAC0706)

9. What type of information is typically included in a job card? (IAC0707)

10. How do product specifications influence the manufacturing process and workflow? (IAC0708)

SECTION C: Matching Activity

Match the item in Column A with its correct description in Column B:

Column A:
1. Routing Sheet
2. Product Specification
3. Finishing Operation
4. Job Card

Column B:
A. Final stage of manufacturing; includes glides, dust covers, etc.
B. Includes dimensions, material types, and special instructions
C. Lists all steps in the manufacturing process and their sequence
D. Lists required dimensions and materials for each batch or product

Marking Memo

Total Marks: 24
Pass Mark (70%): 17

Assessment Rubric

| Criteria | Excellent (5) | Competent (3–4) | Needs Support (1–2) |
| --- | --- | --- | --- |
| IAC0701: Process application and fault analysis | Identifies and analyses both process and impact | General identification of issue; limited analysis | Partial or inaccurate understanding |
| IAC0702–0704: Process and operation knowledge | Accurately outlines and explains all operations | Some gaps in the process flow or task descriptions | Incomplete or vague process knowledge |
| IAC0705: Productivity awareness | Strong understanding of productivity and methods | Basic ideas but lacks real-world application | Unable to explain concepts clearly |
| IAC0706–0708: Use of production documents | Accurately identifies and uses routing, job card, and specs | Recognises documents but with limited detail | Unclear purpose or relevance of documents |

# KM-02-KT08: Frame preparation: knocking-on principles (5%)

Formative assessment

IAC0801 Frame preparation principles are identified and the effect thereof on quality end-product is analysed.

(Weight 5%)
Integrated Assessment – KM-02-KT08: Frame Preparation – Knocking-On Principles

Weight: 5%

Internal Assessment Criterion: IAC0801 – Frame preparation principles are identified and the effect thereof on quality end-product is analysed

SECTION A: Case Study and Scenario-Based Questions

Case Study: Fatima and Kagiso are working on preparing the seat and back frames for a small batch of armchairs. Fatima carefully applies zig-zag springs and stretches the webbing using a webbing stretcher, keeping the fasteners 10 mm from the working edge. Kagiso, in a hurry, uses staples without marking spacing and positions some too close to the edge. Later, the foaming team notices uneven support and bulging on some chairs. The finishing team also struggles to achieve a smooth edge due to misplaced staples.

1. Identify two frame preparation principles that Fatima followed correctly.

2. Identify two frame preparation errors Kagiso made.

3. How did Kagiso’s errors affect the quality and appearance of the final product?

4. Why is it important to keep the working edge clean in upholstery manufacturing?

SECTION B: Matching Activity

Match each frame preparation principle to its correct description:

A. Tension of Webbing
B. Positioning of Fasteners
C. Spacing of Staples
D. Clean Working Edge

1. Placement of fasteners for grip and visual consistency
2. Prevents uneven compression and sagging
3. Avoids interference with foaming and covering stages
4. Ensures support and comfort without damaging the frame

SECTION C: Short Written Response

5. Explain the relationship between correct frame preparation and final product quality.

6. What steps can be taken during knocking-on to prepare the frame properly for the next operation?

Marking Memo

Total Marks: 18
Pass Mark (70%): 13

Assessment Rubric

| Criteria | Excellent (5) | Competent (3–4) | Needs Support (1–2) |
| --- | --- | --- | --- |
| Understanding of preparation principles | All key principles clearly identified and explained | Most principles identified with minor errors | Limited understanding; key principles missing or misapplied |
| Link to quality of end-product | Effect on quality is analysed in depth with clear examples | General awareness of impact on quality | Vague or no connection to product quality |
| Application in scenario | Scenario analysed accurately with justified recommendations | Partially correct analysis with some practical insight | Incomplete or unclear scenario interpretation |
| Accuracy in matching activity | All matches correct | One incorrect match | Two or more incorrect matches |
| Written response clarity | Written answers are logical, relevant, and clearly structured | Answers mostly relevant but may lack detail or clarity | Unclear, incomplete, or off-topic answers |

# KM-02-KT09: Knocker on techniques (15%)

Formative assessment

IAC0901 Techniques of selecting, measuring, cutting, edge bending, spacing and using hand and power tools to attach zig zag springs securely to a frame with the correct tension.

IAC0902 Techniques of selecting, measuring, cutting allowances, spacing, interweaving and using hand and power tools to attach webbing securely to a frame.

IAC0903 Techniques of spacing and using hand and power tools to attach coil springs securely to a webbing and to the frame.

IAC0904 Identify the types of knots and clips used when securing coil springs and zig zag springs to each other and to the frame.

(Weight 15%)

## Integrated Assessment – KM-02-KT09: Knocker-On Techniques

Weight: 15%

Internal Assessment Criteria Assessed:

IAC0901: Techniques of selecting, measuring, cutting, edge bending, spacing and using hand and power tools to attach zig-zag springs securely to a frame with the correct tension.

IAC0902: Techniques of selecting, measuring, cutting allowances, spacing, interweaving and using hand and power tools to attach webbing securely to a frame.

IAC0903: Techniques of spacing and using hand and power tools to attach coil springs securely to a webbing and to the frame.

IAC0904: Identify the types of knots and clips used when securing coil springs and zig-zag springs to each other and to the frame.

SECTION A: Case Study and Scenario-Based Questions

Case Study:
Tamryn is preparing a set of armchair frames. She begins by installing webbing, using a webbing stretcher and stapler. However, she skips interlacing and does not pre-measure her spacing. She moves on to install zig-zag springs, but fails to curl the ends after cutting. Later, Sipho inspects the coil springs and notices that they are spaced unevenly and tied with standard string instead of approved upholstery twine. During the covering stage, several frames are flagged for loose springs, sagging in the seat, and squeaking noises.

1. Identify at least two errors Tamryn made during the installation of zig-zag springs.

2. What webbing installation techniques were incorrectly applied?

3. How should the coil springs have been secured and spaced correctly?

4. What is the correct type of knot or clip used to tie coil springs?

5. How do these errors affect the overall comfort, durability, and safety of the product?

# KM-02-KT10: Foaming-up or padding-up principles (10%)

Formative assessment

IAC1001 Foaming-up or padding-up principles are identified and described and the effect thereof on quality end-product is analysed.

(Weight 10%)

## Integrated Assessment – KM-02-KT10: Foaming-up or Padding-up Principles

Weight: 10%

SECTION A: Case Study with Scenario-Based Questions

Case Study:
Themba is upholstering a pair of armchairs using a patterned jacquard fabric. He selects medium-density foam for the seat and high-density foam for the armrests. He sprays adhesive directly onto the fabric, presses it to the foam while still wet, and skips trimming the foam edges. Later, during the covering stage, the fabric wrinkles at the corners, the pleats are uneven, and the seams on the backrest do not align with the pattern. When the chairs are inspected, one arm looks slightly fuller than the other, and the left wing’s pleating differs from the right.

1. Identify three principles of foaming-up or padding-up that Themba applied incorrectly.

2. What effect did skipping foam trimming have on the final appearance of the armchairs?

3. Why is it important to wait for adhesive to become tacky before bonding?

4. What steps could Themba have followed to ensure uniformity of both arms and wings?

5. How does poor alignment of pattern and cover placement impact perceived product quality?

SECTION B: Short Answer Questions

6. What is the function of shaping edges and corners correctly during upholstery?

7. Describe the relationship between tension, stretch, and deflection of fabric.

8. Why is the correct selection of foam density important for seating comfort and durability?

9. How can incorrect spacing of tacks or staples affect both appearance and structure?

Marking Memo

Total Marks: 18
Pass Mark (70%): 13

Assessment Rubric

| Criteria | Excellent (5) | Competent (3–4) | Needs Support (1–2) |
| --- | --- | --- | --- |
| Understanding of foaming-up principles | Demonstrates deep understanding across all KT areas | Applies most concepts correctly with minor errors | Misunderstands or omits key KT elements |
| Application to scenario | Applies principles logically with practical, corrective suggestions | Recognises issues with some correct recommendations | Identifies issues but lacks clarity or suggests incorrect actions |
| Fabric and foam alignment | Clearly explains impact on comfort, shape, and visual symmetry | Acknowledges alignment problems but lacks full detail | Does not link misalignment to outcome or customer perception |
| Use of terminology | Correctly uses upholstery terminology throughout | Occasional terminology errors, but meaning is clear | Limited use of relevant terms; unclear explanations |

# KM-02-KT11: Foaming-up or padding-up techniques (10%)

Formative assessment

IAC1101 Techniques of padding and edge rolls when covering coil springs and arms.

IAC1102 Techniques of selecting specified densities of foam, applying adhesive to bond foam-to-foam and using templates and tools to construct and shape the foam and attach it to the frame over the springs and webbing.

IAC1103 Techniques of selecting specified padding / stuffing materials, applying and attaching it by means of tacking, sewing, tying and bonding to the sprung foundations and frame, and shaping it to meet specifications.

IAC1104 Techniques of measuring, cutting and attaching the specified fabric to cover the foam and paddings.

IAC1105 Importance of following workplace procedures handle and stack and store completed prepared frame for the next upholstery procedure and explain the types and consequences of damage to the product.

(Weight 10%)

## Integrated Assessment – KM-02-KT11: Foaming-up or Padding-up Techniques

Weight: 10%

SECTION A: Scenario-Based Questions

Scenario:
Lerato is preparing a curved armchair frame for upholstery. She begins by padding the coil springs on the seat, but forgets to apply the edge rolls. She selects a medium-density foam to build up the arms and bonds the foam directly onto the webbing without shaping it. She cuts the foam roughly by hand and does not use templates. She then measures the fabric quickly, cuts it, and attaches it with uneven tacking. The completed unit is stored leaning against a wet surface in the corner of the workshop.

1. Identify two mistakes Lerato made in the padding and edge roll process. (IAC1101)

2. How should Lerato have selected and applied foam for the curved arms, and what tools should have been used? (IAC1102)

3. What fastening techniques should have been used to attach padding to the sprung foundation? (IAC1103)

4. What could Lerato have done to ensure accuracy when measuring and cutting the fabric? (IAC1104)

5. Describe the potential consequences of storing the completed piece incorrectly. (IAC1105)

SECTION B: Practical Demonstration

Learners are required to demonstrate the following on a frame or simulated structure:

| Task | Criteria Met | Comments |
| --- | --- | --- |
| Apply edge rolls and padding to a coil spring seat correctly. (IAC1101) | ☐ Yes / ☐ No |  |
| Select foam by density and shape foam using template. (IAC1102) | ☐ Yes / ☐ No |  |
| Attach padding to a sprung foundation using correct methods. (IAC1103) | ☐ Yes / ☐ No |  |
| Measure and cut cover fabric accurately. (IAC1104) | ☐ Yes / ☐ No |  |
| Demonstrate correct handling and temporary storage of completed unit. (IAC1105) | ☐ Yes / ☐ No |  |

SECTION C: Short Answer Questions

6. What is the purpose of shaping foam to match the design profile? (IAC1102)

7. List two tools commonly used when cutting and shaping layered foam. (IAC1102)

8. Explain why symmetry is important when attaching padding and fabric. (IAC1104)

9. State two workplace procedures that help prevent product damage during storage. (IAC1105)

Marking Memo

Total Marks: 18
Pass Mark (70%): 13 (plus competency in Section B)

Assessment Rubric

| Criteria | Excellent (5) | Competent (3–4) | Needs Support (1–2) |
| --- | --- | --- | --- |
| Understanding of Techniques | Clearly demonstrates full understanding of all KT elements. | Demonstrates reasonable understanding with minor gaps. | Lacks understanding; confusion about KT concepts. |
| Application of Process | Applies logical, complete process aligned with design sequence. | Applies general process with minor order or detail gaps. | Incorrect or incomplete application of sequence. |
| Use of Tools & Materials | Chooses correct tools and foam types confidently and safely. | Chooses acceptable tools but may need guidance. | Uses inappropriate tools or lacks control/safety. |
| Workplace Practice | Fully adheres to safety and storage procedures. | Mostly compliant with safe handling and stacking. | Fails to apply key workplace practices. |
| Communication | Uses accurate upholstery terminology and explains clearly. | Understands terms, minor language errors. | Limited vocabulary or unclear explanation. |

# KM-02-KT12: Joints and bracings of the frame (5%)

## Facilitator Briefing Note

Integrated Assessment – KM-02-KT12: Joints and Bracings of the Frame (5%)

SAQA ID: 103199

Curriculum Code: 683401000

NQF Level: 2

Weighting: 5%

Internal Assessment Criterion: IAC1201

1. Purpose of the Assessment

This integrated assessment is designed to confirm the learner’s foundational understanding of joints and bracings used in furniture frame construction, with a focus on their types, uses, performance requirements, and manufacturing methods. The assessment includes a variety of instruments to test theoretical knowledge, applied understanding, and basic visual analysis skills required for real-world upholstery work.

2. Structure of the Assessment

The assessment is divided into three sections:

| Section | Type | Method |
| --- | --- | --- |
| Section A | Scenario-based questions | Written (Analytical reasoning) |
| Section B | Practical identification task | Observation checklist |
| Section C | Short answer questions | Written (Knowledge recall) |

3. Instructions for the Facilitator

- Preparation: Ensure learners have completed all content under KM-02-KT12, including KT1201–KT1204.
- Assessment Setting: Provide a quiet space for written and practical components.
- Materials: Use diagrams or physical frame samples.
- Timing: Allocate approximately 1 hour in total.
- Observation: Use the checklist provided to evaluate Section B.
- Feedback: Provide constructive feedback post-assessment.

4. Key Competencies to Observe

Facilitators should ensure learners demonstrate:
- Accurate identification and labelling of joint types.
- Understanding of joint usage and structural function.
- Recognition of faults and reinforcement options.
- Clear, technical language throughout.

5. Remedial and Reassessment Guidance

Learners not meeting the required 70% or failing Section B should:
- Receive targeted feedback based on the rubric.
- Review content with practical support.
- Be reassessed using alternative examples.

## Integrated Assessment – KM-02-KT12: Joints and Bracings of the Frame

Weight: 5%

SECTION A: Scenario-Based Questions

Scenario:
Zama is working in the upholstery workshop and receives three different types of chair frames: one with mortise and tenon joints and wooden corner blocks; another with dowel joints but no additional bracing; and a third with glued butt joints reinforced by metal plates. She is asked to assess the joints and bracings in terms of strength, alignment, and suitability for upholstery before beginning any work.

1. Identify the types of joints present in each of the three chairs and describe the main structural characteristic of each.

2. Which frame(s) may require additional reinforcement before upholstering, and why?

3. Compare the manufacturing techniques that would have been used for the mortise and tenon chair versus the dowelled chair.

4. Suggest a bracing solution to improve the frame with butt joints.

5. Explain how poor alignment or overcut joints can impact the upholstery process and product durability.

SECTION B: Practical Activity

| Task | Criteria Met (Yes/No) | Assessor Comments |
| --- | --- | --- |
| Correctly identify three types of joints used in furniture frames. | ☐ Yes / ☐ No |  |
| Describe one use and one requirement for each joint identified. | ☐ Yes / ☐ No |  |
| Suggest a suitable bracing method for an open-corner or butt-jointed frame. | ☐ Yes / ☐ No |  |
| Identify at least one visible fault in a frame joint (gap, movement, overcut, etc.). | ☐ Yes / ☐ No |  |

SECTION C: Short Answer Questions

6. Name two tools used in the manufacturing of wooden joints in furniture frames.

7. What is the function of bracing in a chair frame?

8. List one advantage and one disadvantage of dowel joints.

9. Why must glue be allowed to cure before the frame is upholstered?

10. What signs indicate that a joint may fail under load?

Marking Memo

Total Marks: 20
Pass Mark (70%): 14 (plus competency in Section B)

Assessment Rubric

| Criteria | Excellent (5) | Competent (3–4) | Needs Support (1–2) |
| --- | --- | --- | --- |
| Understanding of Joint Types | Clearly explains multiple joint types with correct uses and examples | Identifies and describes basic types with some detail | Lists types with minimal explanation or incorrect descriptions |
| Application of Concepts | Matches joints to use, describes impact on frame and upholstery | General application with some understanding of structural role | Misunderstands relationship between joints and furniture performance |
| Manufacturing Knowledge | Shows insight into how joints and bracings are made and evaluated | Describes basic manufacturing processes with clarity | Lacks understanding of process or steps |
| Practical Observation | Confidently identifies and inspects physical frame samples | Identifies most joint types with occasional prompting | Struggles to identify joints or recognise defects |
| Communication | Uses technical vocabulary accurately and clearly | Uses some terminology appropriately | Language vague, lacks specific terms or uses incorrectly |