UPHOLSTERERY FRAME PREPARER

KM02 UPHOLSTERED FURNITURE FRAME PREPARATION

ASSESSMENT GUIDE

TABLE OF CONTENTS

ASSESSMENT REQUIREMENTS	3

TOPIC ELEMENTS TO BE COVERED INCLUDE:	4

KM-02-KT01: Specifications for upholstered furniture (10%)	5

🌟 Integrated Assessment – KM-02-KT01: Specifications for Upholstered Furniture	6

KM-02-KT02: Upholstery raw materials and consumables types and characteristics (15%)	10

Integrated Assessment – KM-02-KT02: Upholstery Raw Materials and Consumables	11

KM-02-KT03: Raw materials and consumables used in the frame padding process (5%)	14

Integrated Assessment – KM-02-KT03: Raw Materials and Consumables Used in the Frame Padding Process	15

KM-02-KT04: Use of chemicals: adhesives and solvents (5%)	18

Integrated Assessment – KM-02-KT04: Use of Chemicals – Adhesives and Solvents	19

KM-02-KT05: Upholstery tools and equipment (10%)	23

🔧 Integrated Assessment – KM-02-KT05: Upholstery Tools and Equipment	25

Integrated Assessment – KM-02-KT06: Upholstery Measurements, Measuring Tools, Techniques and Calculations	30

Integrated Assessment – KM-02-KT07: Upholstered Furniture Manufacturing Processes	34

KM-02-KT08: Frame preparation: knocking-on principles (5%)	37

Integrated Assessment – KM-02-KT08: Frame Preparation – Knocking-On Principles	38

KM-02-KT09: Knocker on techniques (15%)	41

Integrated Assessment – KM-02-KT09: Knocker-On Techniques	42

KM-02-KT10: Foaming-up or padding-up principles (10%)	45

Integrated Assessment – KM-02-KT10: Foaming-up or Padding-up Principles	46

KM-02-KT11: Foaming-up or padding-up techniques (10%)	49

Integrated Assessment for KM-02-KT11: Foaming-up or Padding-up Techniques	50

KM-02-KT12: Joints and bracings of the frame (5%)	53

Integrated Assessment – KM-02-KT12: Joints and Bracings of the Frame	53

# ASSESSMENT REQUIREMENTS

Integrated Formative Assessment:

The skills development provider will use the curriculum to guide them on the stipulated internal assessment criteria and weighting. They will also apply the scope of practical skills and applied knowledge as stipulated by the internal assessment criteria. This formative assessment leads to entrance into the integrated external summative assessment.

Integrated Summative Assessment:

An external integrated summative assessment, conducted through the relevant QCTO Assessment Quality partner is required for the issuing of this qualification. The external integrated summative assessment will focus on the exit level outcomes and associated assessment criteria. The external assessment model requires that the external assessment will be conducted through a combination of a written assessment and practical task at an accredited trade test centre. The written examination will be concluded at an accredited trade test centre and marked by registered assessors. Practical tasks will also be assessed by registered assessors. The combination of the written and practical assessment will be conducted over a period of two working days.

# TOPIC ELEMENTS TO BE COVERED INCLUDE:

Purpose of the Knowledge Modules

The main focus of the learning in this knowledge module is to build an understanding of upholstery design, construction specifications, materials, equipment and production techniques associated with preparing an upholstered furniture frame for covering by hand building the suspension.

The learning will enable learners to demonstrate an understanding of:

KM-02-KT01: Specifications for upholstered furniture (10%)

KM-02-KT02: Upholstery raw materials and consumables types and characteristics (15%)

KM-02-KT03: Raw materials and consumables used in the frame padding process (5%)

KM-02-KT04: Use of chemicals: adhesives and solvents (5%)

KM-02-KT05: Upholstery tools and equipment (10%)

KM-02-KT06: Upholstery measurements, measuring tools, techniques and calculations (5%)

KM-02-KT07: Upholstered furniture manufacturing processes (5%)

KM-02-KT08: Frame preparation: knocking-on principles (5%)

KM-02-KT09: Knocker on techniques (15%)

KM-02-KT10: Foaming-up or padding-up principles (10%)

KM-02-KT11: Foaming-up or padding-up techniques (10%)

KM-02-KT12: Joints and bracings of the frame (5%)

# KM-02-KT01: Specifications for upholstered furniture (10%)

Formative assessment

IAC0101 Various types of product specifications used in the upholstery industry are compared and differentiated.

IAC0102 Various aspects of basic sketches and engineering drawings such as isometric, oblique and orthographic views and projections, sectional views, component parts, dimensioning, scaling, line structures and geometry, drawing labelling, curves and radiuses are identified and each function explained.

IAC0103 Aspects included in industry and workplace product specification concepts such as raw material standards, drawing standards, engineering symbols, abbreviations and conventions are identified.

IAC0104 The concepts of compliance and non-compliance is explained.

IAC0105 The consequences for the upholstery process of prepared and covered frames not meeting the specified product standard are evaluated

(Weight 10%)

## 🌟 Integrated Assessment – KM-02-KT01: Specifications for Upholstered Furniture

Qualification: Furniture Upholsterer – SAQA ID 103199
Knowledge Module: KM-02-KT01
Weight: 10%
Assessment Type: Integrated Formative Assessment using a variety of instruments

📝 Assessment Instruments

| Instrument | Focus IACs | Description |
| --- | --- | --- |
| Multiple Choice Questions (MCQs) | IAC0101, IAC0103 | Assess knowledge of specification types and industry concepts |
| Short Answer Questions | IAC0102, IAC0104 | Assess understanding of drawings and compliance concepts |
| Practical Identification Task | IAC0101, IAC0102, IAC0103 | Hands-on identification of drawing components and symbols |
| Case Study Scenario | IAC0104, IAC0105 | Apply knowledge of compliance and consequences in a real-world context |

📘 Assessment Tasks

Section A: Multiple Choice Questions (10 marks)

Instructions: Select the correct answer. One mark per correct answer.

- Which of the following is not a type of product specification?

  - A) Sketch

  - B) Job card

  - C) Angle grinder

  - D) Technical drawing

- A cutting list is mainly used to:

  - A) Describe upholstery fabrics

  - B) Guide frame measurements and materials

  - C) List employee job roles

  - D) Replace technical drawings

- Engineering symbols are used in:

  - A) Marketing reports

  - B) Upholstery foam grading

  - C) Specification drawings

  - D) Delivery notes

- The abbreviation “Ø” on a drawing refers to:

  - A) Radius

  - B) Diameter

  - C) Depth

  - D) Circumference

- A prototype is best described as:

  - A) A final product ready for sale

  - B) A visual sketch

  - C) A first version of a design for testing

  - D) A material sample

Section B: Short Answer Questions (15 marks)

- Explain the difference between orthographic and isometric views. (5)

- Define “compliance” and provide one example from the upholstery process. (5)

- What is the purpose of drawing standards in the furniture manufacturing industry? (5)

Section C: Practical Identification Task (15 marks)

Instructions: Learners receive a printed technical drawing of a chair frame and must:

- Label the following:

  - An orthographic projection

  - An isometric view

  - A sectional view
(3 × 2 = 6 marks)

- Identify three engineering symbols used in the drawing and explain their meaning:
(3 × 3 = 9 marks)

Section D: Case Study Scenario (10 marks)

Scenario: Kyle, a junior upholsterer, is preparing a set of ottoman frames. The job card clearly specifies the use of 30 mm kiln-dried Saligna. Kyle mistakenly uses 25 mm untreated pine from the offcuts pile. Once upholstered, several ottomans buckle under pressure. The client returns the entire batch, and the company incurs significant rework costs.

Questions:

- Identify whether this scenario reflects compliance or non-compliance. Justify your answer. (2)

- List three possible consequences of Kyle’s mistake. (3)

- Suggest two actions the company could take to prevent this from happening again. (2)

- What specification documents could Kyle have consulted to avoid the error? (3)

✅ Model Answers

Section A (MCQs)

- C

- B

- C

- B

- C

Section B (Short Answers)

- Orthographic views show the object from the front, top, and side in 2D. Isometric views show the object in 3D from a single angle.

- Compliance means following job specifications exactly. Example: Using the correct wood type and thickness.

- Drawing standards ensure uniform interpretation of technical information, improving communication across the team.

Section C (Practical)

- Orthographic = labelled front/side view; Isometric = 3D angled view; Sectional = internal cut view.

- Examples:

  - Ø = diameter

  - ∠ = angle

  - ⌀ = circular hole or bore

Section D (Case Study)

- Non-compliance – Kyle did not follow the job card specification.

- Product failure, client dissatisfaction, cost of rework.

- Staff training; quality control checkpoints.

- Job card, cutting list, technical drawing.

Marking Memo

| Section | Total Marks | Notes |
| --- | --- | --- |
| Section A | 5 × 1 = 5 | One mark per correct answer |
| Section B | 3 × 5 = 15 | Assess understanding and explanation |
| Section C | 15 | 2 marks for correct labelling; 3 per accurate symbol explanation |
| Section D | 10 | Assess real-world application and judgement |

Total: 45 marks
Pass Mark: 31.5 (70%)

📊 Assessment Rubric

| Criteria | Excellent (5) | Good (4) | Satisfactory (3) | Needs Support (1–2) |
| --- | --- | --- | --- | --- |
| Understanding of specification types (IAC0101) | Identifies all types clearly with examples | Identifies most types accurately | Identifies some types; limited explanation | Fails to identify types or confuses concepts |
| Drawing interpretation (IAC0102) | Explains views and symbols clearly | Minor errors in explanation | Basic understanding only | Misinterprets views/symbols |
| Industry concepts (IAC0103) | Identifies drawing standards and symbols comprehensively | Reasonable identification with some omissions | Basic list; lacks detail | Limited or incorrect responses |
| Compliance awareness (IAC0104) | Clear understanding with real-world examples | General understanding | Limited awareness | Confused or no understanding |
| Evaluation of consequences (IAC0105) | Evaluates impacts clearly and suggests valid solutions | Identifies key impacts | Generalised or partial impacts | Fails to link non-compliance to outcomes |

# KM-02-KT02: Upholstery raw materials and consumables types and characteristics (15%)

Formative assessment

IAC0201 The use of specifications and workplace instructions to determine the type, quality, sizes and quantities of raw materials and consumables for the frame preparation process is reasoned.

IAC0202 Various types of frames and their raw materials and type of foundation materials to be used for each component are identified.

IAC0203 Types and gauges of various springs needed for the knocker-on process are identified including coil springs and zig-zag springs.

IAC0204 Types, sizes, quantity and quality of consumables such as clips, nails, staples, edge rolls, twine and cord needed for the springing up process are identified.

IAC0205 Differences between the types of webbing needed for the knocker on process, such as manmade elastic and rubber webbing, and natural fibres like jute webbing are evaluated.

IAC0206 Types, sizes, quantity and quality of consumables such as clips, tacks, and staples needed for the webbing up process are identified.

IAC0207 Methods for identifying density and thickness of various types of foam required for the foaming up process are described.

IAC0208 Consumables required for the foaming up process are listed.

IAC0209 Raw materials required for the padding and covering of coil springs are listed.

IAC0210 Fabrics used for covering webbing, springs, foam and padding including calico, hessian (burlap), polypropylene, Dacron, cambric and flock are listed.

IAC0211 Principles and concepts of safe materials handling and storage methods are reasoned.

(Weight 15%)

## Integrated Assessment – KM-02-KT02: Upholstery Raw Materials and Consumables

Qualification: Furniture Upholsterer (SAQA ID: 103199)
Knowledge Module: KM-02-KT02
Assessment Type: Case Study with Scenario-Based Questions
Weight: 15%
Assessment Instrument: Integrated Formative Assessment

Case Study

Sibongile is a junior upholsterer working at a mid-sized furniture manufacturing workshop. The team has received a large order for ten high-end lounge chairs for a corporate client. Each chair is specified to include:

- Reinforced hardwood frame

- Coil springs for the seating foundation

- Jute webbing on the seat base

- High-density foam for the seat and back

- Internal fabrics such as hessian and Dacron

- Consumables including spray adhesive, staples, clips, edge rolls, and twine

The job card specifies all these requirements and quantities per unit. However, the storeroom is disorganised, poorly labelled, and lacks proper ventilation. Sibongile mistakenly uses:

- 9-gauge zig-zag springs instead of coil springs

- Elastic webbing in place of jute

- Foam without confirming its density or thickness

- Consumables without checking for quality or correct sizing

These issues are only discovered after several chairs have already been partially assembled.

📘 Scenario-Based Questions

- Based on the job card, what steps should Sibongile have taken to ensure she selected the correct type, quality, and quantity of raw materials and consumables? (IAC0201)

- Identify the type of frame specified and explain why a reinforced hardwood frame is suitable for high-end lounge chairs. (IAC0202)

- What is the difference between the 9-gauge zig-zag springs Sibongile used and the coil springs specified? When is each type appropriate? (IAC0203)

- List the consumables Sibongile should have used during the springing-up process. Include notes on expected sizes or quality. (IAC0204)

- Evaluate the impact of using elastic webbing instead of jute webbing in this furniture context. (IAC0205)

- Identify the correct consumables for the webbing-up process. How do they differ from those used in springing-up? (IAC0206)

- How could Sibongile have determined the correct density and thickness of the foam before using it? (IAC0207)

- List the consumables required for the foaming-up process. (IAC0208)

- What raw materials should be used to pad and cover coil springs prior to applying top fabric? (IAC0209)

- Name the fabrics that should be used to cover webbing, springs, foam, and padding. (IAC0210)

- Discuss the risks associated with poor materials handling and storage, as seen in the workshop storeroom. Suggest two improvements. (IAC0211)

✅ Model Answers

- Refer to the job card, check for specifications, confirm quantities using production lists, and identify quality indicators (e.g. batch labels, markings).

- Reinforced hardwood is strong, durable, and stable, ideal for premium seating expected to withstand regular use and support weight comfortably.

- Coil springs are individual, vertical, supportive springs tied in 8-way systems; zig-zag springs are horizontal wire strips. Coil = premium comfort; Zig-zag = budget use.

- Consumables: Clips, 16 mm staples, twine, edge rolls – all should be rust-resistant and match size/spec requirements.

- Elastic webbing may stretch and sag under heavy use, reducing support. Jute is more rigid and suited to structural upholstery.

- Webbing-up consumables: Tacks, longer staples, webbing stretcher. They differ in tool and tension application.

- Use foam labels, check manufacturer specs, or consult the supervisor to confirm density (kg/m³) and thickness (mm).

- Spray adhesive, profile jigs, foam templates, marking chalks, trimming blades.

- Hessian, coconut fibre pad, cotton felt, Dacron – these protect the fabric and smooth the spring surface.

- Calico, hessian, polypropylene, Dacron, cambric, flock – used in different layers depending on application.

- Risks: Material degradation, contamination, trip hazards. Improvements: Labelling system, ventilated shelving or bins.

Marking Memo

- Each question: 5 marks

- Total marks: 55

- Pass mark (70%): 39 marks

📊 Assessment Rubric

| Criteria | Excellent (5 marks) | Competent (3–4 marks) | Needs Support (1–2 marks) |
| --- | --- | --- | --- |
| IAC0201: Specification Use | Clearly identifies all materials, quantities, and quality indicators from job card | Mostly accurate but lacks full depth | Incorrect or incomplete interpretation |
| IAC0202: Frame Types | Correctly identifies reinforced hardwood and justifies its use | Identifies hardwood, vague justification | No understanding of frame suitability |
| IAC0203: Spring Types | Clear comparison with use-case scenarios | Partial distinction or vague examples | Confused or incorrect |
| IAC0204–IAC0206: Consumables | All correct items, sizes, and quality noted | Mostly correct with minor omissions | Misidentified or confused items |
| IAC0207–IAC0208: Foam | Accurately explains foam properties and lists consumables | Acceptable but not detailed | Lacks understanding of density or tools |
| IAC0209–IAC0210: Cover Materials | Lists correct padding and fabric layers | Lists some but not all, or minor errors | Incomplete or incorrect |
| IAC0211: Storage | Clearly identifies risks and suggests valid improvements | General understanding | Inadequate or unsafe suggestions |

# KM-02-KT03: Raw materials and consumables used in the frame padding process (5%)

Formative assessment

IAC0301 Types of natural raw materials used as padding for upholstered furniture are identified and the respective uses and characteristics are evaluated.

IAC0302 Types of man-made raw materials used as padding for upholstered furniture are identified and the respective uses and characteristics are evaluated.

IAC0303 The importance of safe handling and storage of raw materials is justified.

(Weight 5%)

## Integrated Assessment – KM-02-KT03: Raw Materials and Consumables Used in the Frame Padding Process

Qualification: Furniture Upholsterer (SAQA ID 103199)
Knowledge Module: KM-02-KT03
Assessment Type: Integrated Formative Assessment using a variety of instruments
Weight: 5%

📘 Assessment Instruments Overview

| Instrument Type | Focus Area | IAC Alignment |
| --- | --- | --- |
| Multiple Choice Questions (MCQs) | Identification of raw materials | IAC0301 and IAC0302 |
| Matching Activity | Properties and uses | IAC0301 and IAC0302 |
| Short Answer Questions | Handling and storage practices | IAC0303 |
| Case Study and Scenario | Application and justification | IAC0301–IAC0303 |

📝 Section A: Multiple Choice Questions (5 marks)

Choose the correct answer for each question. (1 mark each)

- Which of the following is a natural padding material?
A) Memory foam
B) Polyester fibre
C) Kapok
D) Polyurethane

- Coir is made from:
A) Cotton lint
B) Tree bark
C) Coconut husk fibres
D) Plastic recycling

- Which of the following is a man-made padding material?
A) Dacron
B) Coir
C) Wool
D) Hessian

- Foam density refers to:
A) The colour of the foam
B) Its ability to float
C) The firmness of the foam
D) The mass per cubic metre

- Which of the following is most suitable for lightweight cushion filling?
A) Jute
B) Dacron
C) Kapok
D) Edge roll

🔀 Section B: Matching Activity (6 marks)

Match the padding material in Column A with its correct use in Column B. (1 mark each)

| Column A | Column B |
| --- | --- |
| 1. Coir | A. Final smoothing layer for outer fabric |
| 2. High-density foam | B. Lightweight cushion filling |
| 3. Kapok | C. Used in traditional firm seat foundations |
| 4. Dacron | D. Used for soft armrests or backrests |
| 5. Low-density foam | E. Enhances comfort and reduces fabric friction |
| 6. Cotton felt | F. Smooths spring lines in premium furniture |

✍️ Section C: Short Answer Questions (9 marks)

- Explain two safety practices that should be followed when storing kapok or coir in a workshop. (3 marks)

- List two potential risks of storing foam incorrectly. (2 marks)

- Describe how proper labelling and organisation of raw materials can improve safety and efficiency. (4 marks)

📖 Section D: Case Study Scenario (5 marks)

Scenario: Thabiso is working with a new team and is asked to prepare padding for a batch of ottomans. He selects unlabelled padding from a shelf and uses it across all ten units. The customer later complains that the ottomans feel too soft and do not hold their shape. On inspection, the team realises Thabiso used low-density foam instead of the specified high-density foam and did not apply a Dacron layer to smooth the finish.

Questions:

- Identify two mistakes Thabiso made. (2 marks)

- What effect did these mistakes have on product quality? (2 marks)

- How could these issues have been prevented through safe handling and storage? (1 mark)

✅ Model Answers

Section A (MCQs)

- C

- C

- A

- D

- C

Section B (Matching Activity)

1–C, 2–F, 3–B, 4–E, 5–D, 6–A

Section C (Short Answers)

- Keep kapok sealed to avoid fire risk; store coir in dry, ventilated bins.

- Foam can deform or attract dust if stacked improperly or left uncovered.

- Labelling helps prevent incorrect material use, ensures traceability, and reduces search time.

Section D (Case Study)

- Used the wrong foam density; skipped the Dacron layer.

- The ottomans were too soft and lacked structure.

- Materials should have been labelled, checked, and verified before use.

Marking Memo

| Section | Marks Available |
| --- | --- |
| Section A (MCQ) | 5 |
| Section B (Match) | 6 |
| Section C (Short) | 9 |
| Section D (Case) | 5 |
| Total | 25 |
| Pass mark (70%) | 18 marks |

📊 Assessment Rubric

| Criteria | Excellent (5) | Competent (3–4) | Needs Support (1–2) |
| --- | --- | --- | --- |
| IAC0301: Natural padding materials | Clearly identifies and evaluates natural materials with correct uses | Identifies with minor detail gaps | Limited or incorrect identification |
| IAC0302: Man-made materials | Demonstrates good understanding and use-case knowledge | Generally correct, some confusion | Incorrect or vague response |
| IAC0303: Safe handling and storage | Justifies handling and storage with clear examples | Partially correct, lacks clarity | Misunderstood or omitted safety concerns |

# KM-02-KT04: Use of chemicals: adhesives and solvents (5%)

Formative assessment

IAC0401 The various types of chemicals used in upholstery processes are named.

IAC0402 Characteristics and properties of the chemicals used in upholstery processes are described.

IAC0403 The purpose of each of these chemicals used in the process is explained.

IAC0404 Various types of solvents used in upholstery processes are listed.

IAC0405 Characteristics and properties of these solvents are explained.

IAC0406 The purpose of each solvent used in the process is explained.

IAC0407 The application techniques and sequence used for chemicals and solvents are described.

IAC0408 Safety procedures, hazards and risks associated with handling of chemicals and solvents are described.

IAC0409 Standard operating procedures dealing with spillages, waste and effluent are explained.

IAC0410 Types, usage and care of personal protective equipment used in textile wet product preparation processes are described.

(Weight 5%)

## Integrated Assessment – KM-02-KT04: Use of Chemicals – Adhesives and Solvents

Qualification: Furniture Upholsterer (SAQA ID 103199)
Knowledge Module: KM-02-KT04
Assessment Type: Integrated Formative Assessment using a Case Study
Weight: 5%

Case Study

Lungile is working in the foaming-up section of a busy upholstery workshop. Her task is to bond layers of foam and Dacron for a batch of seat cushions. She selects a solvent-based spray adhesive due to its fast-drying properties. She works in a poorly ventilated corner of the workshop without a respirator or gloves. She places the open adhesive can near a fan heater for warmth.

Later, she moves on to another task and leaves the spray gun and half-empty solvent container open. A co-worker using a heat gun nearby unknowingly ignites the vapour-rich air, causing a flash fire. The fire is quickly contained, but the supervisor initiates an investigation and safety review.

During the review, it is revealed that Lungile was not fully trained on the application sequence for adhesives, the risks associated with solvents, or the safe storage of chemical products. The fire could have been avoided had the correct PPE, ventilation, and storage procedures been followed.

📝 Scenario-Based Questions

- List three types of chemicals commonly used in upholstery and give one example of each. (IAC0401)

- Describe two characteristics or properties of solvent-based adhesives. (IAC0402)

- Explain the purpose of each of the following chemicals:
a) Contact adhesive
b) Spray adhesive
c) Cleaning solvent (IAC0403)

- Identify two types of solvents that might be present in an upholstery workshop. (IAC0404)

- Describe the properties of the two solvents you listed in Question 4. (IAC0405)

- For each solvent above, explain its purpose in the upholstery process. (IAC0406)

- Outline the correct sequence of application when using contact adhesive. (IAC0407)

- Identify three hazards present in the case study and describe how each one could have been avoided. (IAC0408)

- State two standard operating procedures for dealing with spillages, waste, or chemical effluent. (IAC0409)

- List two types of personal protective equipment (PPE) Lungile should have worn. Describe how each item protects her during chemical use. (IAC0410)

✅ Model Answers

- Spray adhesive (e.g. solvent-based aerosol)

- Contact adhesive (e.g. neoprene-based glue)

- Cleaning solvent (e.g. acetone)

- Flammable vapours

- Fast-drying with strong odour requiring ventilation

a) Contact adhesive: Bonds foam to frame with high strength
b) Spray adhesive: Provides an even coat for lamination
c) Cleaning solvent: Removes adhesive residue from tools and surfaces

- Acetone

- Toluene

- Acetone: Fast-evaporating, high flammability

- Toluene: Strong solvent, slower evaporation, high toxicity

- Acetone: Cleans spray gun tips and removes spills

- Toluene: Used in adhesive formulations to enhance bonding performance

- Clean both surfaces

- Apply adhesive to each surface

- Allow to dry until tacky

- Align surfaces and press together firmly

- Open flame near solvent (could have stored solvents away from heat)

- No ventilation (should have used an extractor or open space)

- No PPE (respirator and gloves would reduce exposure)

- Use absorbent materials and dispose of as hazardous waste

- Never pour effluent into sinks or drains; use designated containers

- Respirator: Prevents inhalation of harmful vapours

- Chemical-resistant gloves: Protect skin from burns and irritation

Marking Memo

| Question | Marks |
| --- | --- |
| Q1 | 3 |
| Q2 | 2 |
| Q3 | 3 (1 per item) |
| Q4 | 2 |
| Q5 | 2 |
| Q6 | 2 |
| Q7 | 4 |
| Q8 | 3 |
| Q9 | 2 |
| Q10 | 2 |
| Total | 25 |
| Pass mark (70%) | 18 |

📊 Assessment Rubric

| Criteria | Excellent (5) | Competent (3–4) | Needs Support (1–2) |
| --- | --- | --- | --- |
| Knowledge of chemical types and solvents (IAC0401–IAC0406) | Accurately identifies, explains, and applies knowledge to scenario | Mostly accurate with minor gaps | Limited understanding or vague references |
| Understanding of application techniques (IAC0407) | Demonstrates clear procedural knowledge and reasoning | General steps correct with minor errors | Steps confused or incomplete |
| Hazard awareness and safety (IAC0408–IAC0410) | Risks clearly identified, and appropriate PPE and SOPs provided | Identifies some risks or PPE but lacks full justification | Missing or incorrect safety responses |

# KM-02-KT05: Upholstery tools and equipment (10%)

Formative assessment

IAC0501 Hand tools used by the frame preparing department are identified and their purposes are explained.

IAC0502 Parts of the hand tools are identified.

IAC0503 Maintenance procedures for each tool explained.

IAC0504 The consequences of not handling, cleaning and maintaining hand tools correctly are assessed.

IAC0505 Pneumatic power tools used in the frame preparing department are identified and their purposes explained.

IAC0506 Parts and safety mechanisms of the pneumatic power tools are identified and the maintenance procedures for each tool is explained.

IAC0507 The consequences of not handling, cleaning and maintaining pneumatic power tools correctly are assessed.

IAC0508 Electric power tools used in the frame preparation department are identified and respective purpose is explained.

IAC0509 Parts and safety mechanisms of the electric power tools are identified and maintenance procedures for each tool is assessed.

IAC0510 Consequences of not handling, cleaning and maintaining electric power tools correctly is evaluated.

IAC0511 The identification, selection and loading staples, nails, bits and other consumables into the relevant power tool are explained.

IAC0512 The consequences of not cleaning the spray equipment and spray booth thoroughly after use are assessed.

## 🔧 Integrated Assessment – KM-02-KT05: Upholstery Tools and Equipment

Qualification: Furniture Upholsterer (SAQA ID: 103199)
Knowledge Module: KM-02-KT05
Assessment Type: Integrated Formative Assessment using a Variety of Instruments
Weight: 10%

Assessment Instruments Overview

| Instrument Type | Focus Area |
| --- | --- |
| Multiple Choice Questions (MCQs) | Tool identification, safety, purpose (IAC0501, IAC0505, IAC0508, IAC0513) |
| Matching Activity | Parts and components (IAC0502, IAC0506, IAC0509) |
| Short Answer Questions | Maintenance, fault-finding, SOPs (IAC0503, IAC0507, IAC0510, IAC0512) |
| Practical Simulation / Oral Questions | Loading staples and consumables, safe handling (IAC0511, IAC0504, IAC0507) |
| Case Study Scenario | Risk analysis, safety consequences, PPE, frame preparation (IAC0513, IAC0506) |

📝 Section A: Multiple Choice Questions (6 marks)

Choose the correct answer for each question. (1 mark each)

- Which of the following is a pneumatic tool?
A) Mallet
B) Electric drill
C) Staple gun connected to air supply
D) Hot glue gun

- Which PPE item is essential when using a spray adhesive in a booth?
A) Dust mask
B) Ear protection
C) Welding goggles
D) Respirator

- What is the main consequence of not cleaning spray equipment after use?
A) Increased adhesive strength
B) Equipment overheating
C) Blockages and equipment failure
D) Lower electricity use

- Which hand tool is used to tension jute webbing?
A) Tack hammer
B) Staple remover
C) Webbing stretcher
D) Glue spatula

- Which tool is used for cutting foam accurately?
A) Spray gun
B) Upholstery shears
C) Pneumatic gun
D) Dust mask

- What is a key safety mechanism found in electric staple guns?
A) Trigger lock
B) Temperature gauge
C) Wire loop
D) Rotary switch

🔀 Section B: Matching Activity (6 marks)

Match each tool in Column A to its part or component in Column B. (1 mark each)

| Column A | Column B |
| --- | --- |
| 1. Pneumatic staple gun | A. Nozzle, compressor hose, trigger |
| 2. Upholstery tack hammer | B. Magnetic head, grip handle |
| 3. Foam cutting electric knife | C. Blades, power cord, safety guard |
| 4. Adhesive spray gun | D. Pressure tank, nozzle, trigger |
| 5. Portable power drill | E. Chuck, speed setting dial, battery pack |
| 6. Electric staple gun | F. Power cord, trigger switch, safety tip |

✍️ Section C: Short Answer Questions (8 marks)

- Describe two maintenance procedures for hand tools used in frame preparation. (2 marks)

- What are two consequences of not cleaning or maintaining pneumatic tools? (2 marks)

- Outline one standard procedure for loading staples into a pneumatic gun. (2 marks)

- What are two risks of not wearing PPE when using electric foam cutting equipment? (2 marks)

🗣 Section D: Practical / Oral Assessment (3 marks)

Instructor Prompt: Demonstrate or explain the steps involved in:

- Identifying and safely loading nails or staples into a staple gun.

- Checking the safety lock mechanism.

- Storing the tool safely after use.

📖 Section E: Case Study Scenario (7 marks)

Scenario:
Nomvula is working in the frame preparation area. She uses an electric staple gun with worn parts and no functioning safety lock. After use, she stores it without unplugging it. She also neglects to clean the spray booth after applying adhesive to foam seat panels. Later that day, another worker tries to use the same equipment and experiences backflow of adhesive due to a blocked nozzle.

Questions:

- Identify two tools used incorrectly in this scenario. (2 marks)

- What are the safety risks involved in leaving equipment plugged in or uncleaned? (2 marks)

- What PPE should Nomvula have used during the adhesive application? (1 mark)

- What is one standard operating procedure for cleaning spray booths after adhesive application? (2 marks)

✅ Model Answers

Section A (MCQs)

- C

- D

- C

- C

- B

- A

Section B (Matching Activity)

1–A, 2–B, 3–C, 4–D, 5–E, 6–F

Section C (Short Answers)

- Clean and oil tools after use; inspect handles and heads for wear.

- Jammed parts; potential air pressure hazards or misfiring.

- Unplug or depressurise tool; slide staples into correct chamber; re-lock and test.

- Risk of burns or cuts; inhalation of fumes or debris.

Section D (Practical)

- Learner correctly explains loading staples, checking safety, and storing tools safely.

Section E (Case Study)

- Electric staple gun, spray gun.

- Fire, unintentional activation, injury from blockages.

- Respirator or mask, gloves, goggles.

- Switch off and unplug; clean with correct solvent; wipe surfaces; store cleaning waste safely.

Marking Memo

| Section | Marks |
| --- | --- |
| Section A | 6 |
| Section B | 6 |
| Section C | 8 |
| Section D | 3 |
| Section E | 7 |
| Total | 30 |
| Pass Mark (70%) | 21 |

📊 Assessment Rubric

| Criteria | Excellent (5) | Competent (3–4) | Needs Support (1–2) |
| --- | --- | --- | --- |
| Tool identification and purpose (IAC0501, IAC0505, IAC0508, IAC0513) | Accurately identifies tools and explains use in detail | Mostly accurate identification and explanation | Limited or incorrect identification |
| Component recognition and maintenance (IAC0502, IAC0506, IAC0509) | Components and procedures clearly explained | General understanding with some gaps | Incomplete or vague |
| Safety, SOPs, and consequences (IAC0504, IAC0507, IAC0510–IAC0512) | Demonstrates awareness of hazards and applies SOPs | Adequate safety knowledge with minor errors | Poor safety understanding, unclear SOPs |

Formative assessment

IAC0601 Various units, conversions and terminology for measurements, including quantity, deflection, and compressed air volume and pressure are explained and applied.

IAC0602 Calculations required during the upholstery process are explained and applied.

IAC0603 Measuring tools such as steel tape measure, cloth tape measure, squares, steel rules, vernier calliper, templates and patterns, and pressure gauges and their parts are identified and their functions analysed.

IAC0604 Calculations involving dimensions, spacing, quantities, volume, and air pressure using consistent units and conversions and appropriate mathematical operation are completed, recorded and checked.

IAC0605 Consequences of inaccurate measurements on the upholstery process and the output of the department are assessed.

(Weight 5%)

## Integrated Assessment – KM-02-KT06: Upholstery Measurements, Measuring Tools, Techniques and Calculations

Qualification: Furniture Upholsterer (SAQA ID: 103199)
Assessment Type: Integrated Formative Assessment using a Case Study
Weight: 5%

📘 Case Study: The Misaligned Backrest Batch

Nomfundo is preparing a batch of upholstered backrests for an office chair line. Each frame is slightly angled and requires precise foam blocks cut to 480 mm in width, 540 mm in height, and 50 mm in thickness, with a 5 mm allowance on each edge for compression.

She uses a steel tape measure for the first few units, but when she switches to a cloth tape, she forgets to check that it has not stretched. She also forgets to add the compression allowance to the next five foam blocks. Meanwhile, she uses an outdated pattern that does not account for the 10° incline of the frame top, resulting in misalignment.

Later, a team member asks her to verify air pressure on a pneumatic stapler. Nomfundo estimates the pressure rather than using the gauge and does not realise the pressure is too low. Several staples do not secure properly, and parts must be reassembled.

📝 Scenario-Based Questions

1. List three units of measurement used in upholstery, and convert the following:
a) 1.5 metres = ___ mm
b) 100 psi = ___ kPa
(IAC0601)

2. What is the volume of one foam block Nomfundo is cutting (in mm³)? Include allowance in your answer. (IAC0602)

3. Identify two measuring tools Nomfundo used, and describe their function and one limitation of each. (IAC0603)

4. Calculate the total fabric area needed to cover one foam block face (front and back), if the foam is 490 mm × 550 mm, and the fabric requires 40 mm allowance all around. (IAC0604)

5. What were the consequences of inaccurate measurements and estimation in this case study? List two examples. (IAC0605)

✅ Model Answers

1.
Units: millimetres (mm), centimetres (cm), kilopascals (kPa)
a) 1.5 m = 1500 mm
b) 100 psi = 689.5 kPa

2.
Adjusted foam size: 480 + (2 × 5) = 490 mm
Height: 540 + (2 × 5) = 550 mm
Thickness: 50 mm
Volume = 490 × 550 × 50 = 13 475 000 mm³

3.

- Steel tape measure: Measures large, flat distances; limitation: cannot flex easily for curves.

- Cloth tape: Measures around curves; limitation: can stretch or distort, leading to inaccuracy.

4.
Foam face: 490 mm × 550 mm
Add 40 mm allowance to each side:
Final dimensions = (490 + 80) × (550 + 80) = 570 mm × 630 mm
Area = 570 × 630 = 359 100 mm² per side
Total for front and back = 2 × 359 100 = 718 200 mm²

5.

- Foam blocks did not align with frame angle due to outdated pattern.

- Staples failed to secure due to unmeasured low air pressure, requiring rework.

Marking Memo

| Question | Marks |
| --- | --- |
| Q1 | 3 |
| Q2 | 3 |
| Q3 | 4 (2 each) |
| Q4 | 5 |
| Q5 | 2 |
| Total | 17 |
| Pass Mark (70%) | 12 |

📊 Assessment Rubric

| Criteria | Excellent (5) | Competent (3–4) | Needs Support (1–2) |
| --- | --- | --- | --- |
| IAC0601 Units and conversions | Accurate units with correct conversions | Some conversions correct, minor errors | Units not clearly identified or conversions incorrect |
| IAC0602 Calculations | Fully accurate with correct dimensions and allowance | Minor calculation or allowance error | Incorrect or incomplete calculation |
| IAC0603 Tool identification and analysis | Tools correctly named and function/limitations well explained | General understanding with one omission | Tool or function not clearly explained |
| IAC0604 Applied calculations | Calculations and formula used correctly | Attempted calculation, some inaccuracies | Misapplied or no calculation shown |
| IAC0605 Consequence analysis | Logical explanation of impacts | General impacts mentioned | Minimal or unclear response |

KM-02-KT07: Upholstered furniture manufacturing processes (5%)

Formative assessment

IAC0701 Workshop processes of producing upholstered items from raw frame to finished product and the implications of design and construction faults on the final product are analysed.

IAC0702 The process of furniture manufacture is briefly outlined.

IAC0703 The operations in furniture manufacture such as machining, assembling and finishing operations are reviewed.

IAC0704 The finishing processes of furniture is outlined.

IAC0705 The importance of productivity and methods to enhance productivity are discussed.

IAC0706 The role of the routing sheet is described.

IAC0707 Job card information such as component sizes and details, shoulder-to-shoulder size and chemicals to use is explained.

IAC0708 Product specifications are understood and their impact on the manufacturing process is discussed in terms of the process flow and methods that will be used.

(Weight 5%)

## Integrated Assessment – KM-02-KT07: Upholstered Furniture Manufacturing Processes

SAQA ID: 103199
Curriculum Code: 683401000
Assessment Type: Integrated Formative Assessment Using a Variety of Instruments
Weight: 5%

📝 SECTION A: Case Study and Scenario-Based Questions

Case Study: Johan and Nandi are assigned to oversee a new batch of high-back chairs for a hospitality client. The routing sheet outlines 11 operations, starting from raw frame inspection to the final finishing stage. However, one frame enters the padding stage without the foam being applied evenly during foaming-up. As a result, the cover is misaligned, and the seat back appears asymmetrical. Upon review, it is found that the job card was missing the shoulder-to-shoulder measurement. The team also realised that the productivity in the foaming section dropped due to overlapping routing sheet tasks.

Questions:

- Identify two workshop processes that were not followed properly in this scenario. (IAC0701)

- Briefly outline the typical upholstered furniture manufacturing process from raw frame to finished product. (IAC0702)

- What are the three key operations involved in the manufacture of upholstered furniture? (IAC0703)

- Describe one finishing process used in upholstered furniture production. (IAC0704)

- Discuss one design or construction fault that may affect the quality of the final product. (IAC0701)

✍️ SECTION B: Short Answer Questions

- Explain the importance of productivity in a furniture workshop. (IAC0705)

- List and briefly explain two methods used to improve productivity. (IAC0705)

- What is the function of a routing sheet, and why must it be followed? (IAC0706)

- What type of information is typically included in a job card? (IAC0707)

- How do product specifications influence the manufacturing process and workflow? (IAC0708)

🔁 SECTION C: Matching Activity

Match the item in Column A with its correct description in Column B:

| Column A | Column B |
| --- | --- |
| 1. Routing Sheet | A. Final stage of manufacturing; includes glides, dust covers, etc. |
| 2. Product Specification | B. Includes dimensions, material types, and special instructions |
| 3. Finishing Operation | C. Lists all steps in the manufacturing process and their sequence |
| 4. Job Card | D. Lists required dimensions and materials for each batch or product |

(IAC0706, IAC0707, IAC0708)

✅ Model Answers

Section A: Case Study and Scenario-Based Questions

- Foam was applied unevenly (foaming-up stage); job card lacked proper details (routing sheet/job card not checked).

- Raw frame preparation → Knocker-on → Foaming-up → Padding → Covering → Finishing → Quality Control.

- Machining (frame making), Assembling (springs, foam), Finishing (trimming, inspection).

- Applying fabric protector spray or attaching dust covers.

- Uneven foam cutting can result in misaligned or sagging upholstery.

Section B: Short Answer Questions

- Productivity ensures more output in less time, reducing costs and meeting deadlines.

- (i) Organise tools and materials beforehand; (ii) Use routing sheets to streamline workflow.

- The routing sheet shows the order of operations and helps prevent skipped or repeated steps.

- Job cards include component sizes, quantities, materials, chemicals, and special instructions.

- Specifications determine measurements, finishes, and the sequence of operations — affecting planning, material selection, and quality.

Section C: Matching

1 – C, 2 – B, 3 – A, 4 – D

Marking Memo

| Question | Marks |
| --- | --- |
| Section A | 10 |
| Section B | 10 |
| Section C | 4 |
| Total | 24 |
| Pass Mark (70%) | 17 |

📊 Assessment Rubric

| Criteria | Excellent (5) | Competent (3–4) | Needs Support (1–2) |
| --- | --- | --- | --- |
| IAC0701: Process application and fault analysis | Identifies and analyses both process and impact | General identification of issue; limited analysis | Partial or inaccurate understanding |
| IAC0702–0704: Process and operation knowledge | Accurately outlines and explains all operations | Some gaps in the process flow or task descriptions | Incomplete or vague process knowledge |
| IAC0705: Productivity awareness | Strong understanding of productivity and methods | Basic ideas but lacks real-world application | Unable to explain concepts clearly |
| IAC0706–0708: Use of production documents | Accurately identifies and uses routing, job card, and specs | Recognises documents but with limited detail | Unclear purpose or relevance of documents |

# KM-02-KT08: Frame preparation: knocking-on principles (5%)

Formative assessment

IAC0801 Frame preparation principles are identified and the effect thereof on quality end-product is analysed.

(Weight 5%)

## Integrated Assessment – KM-02-KT08: Frame Preparation – Knocking-On Principles

SAQA ID: 103199
Curriculum Code: 683401000
Assessment Type: Integrated Formative Assessment Using a Variety of Instruments
Weight: 5%
Internal Assessment Criterion:

- IAC0801: Frame preparation principles are identified and the effect thereof on quality end-product is analysed

📝 SECTION A: Case Study and Scenario-Based Questions

Case Study: Fatima and Kagiso are working on preparing the seat and back frames for a small batch of armchairs. Fatima carefully applies zig-zag springs and stretches the webbing using a webbing stretcher, keeping the fasteners 10 mm from the working edge. Kagiso, in a hurry, uses staples without marking spacing and positions some too close to the edge. Later, the foaming team notices uneven support and bulging on some chairs. The finishing team also struggles to achieve a smooth edge due to misplaced staples.

Questions:

- Identify two frame preparation principles that Fatima followed correctly.

- Identify two frame preparation errors Kagiso made.

- How did Kagiso’s errors affect the quality and appearance of the final product?

- Why is it important to keep the working edge clean in upholstery manufacturing?

✍️ SECTION B: Matching Activity

Match each frame preparation principle to its correct description:

| Principle | Description |
| --- | --- |
| A. Tension of Webbing | 1. Placement of fasteners for grip and visual consistency |
| B. Positioning of Fasteners | 2. Prevents uneven compression and sagging |
| C. Spacing of Staples | 3. Avoids interference with foaming and covering stages |
| D. Clean Working Edge | 4. Ensures support and comfort without damaging the frame |

Correctly match A–D to 1–4. (Each correct match = 1 mark)

✏️ SECTION C: Short Written Response

- Explain the relationship between correct frame preparation and final product quality.

- What steps can be taken during knocking-on to prepare the frame properly for the next operation?

✅ Model Answers

Section A: Scenario-Based Questions

- Fatima applied proper tension and maintained a clean working edge.

- Kagiso used inconsistent spacing and placed fasteners too close to the edge.

- The frame showed bulging and poor foam shaping, and finishers struggled to complete the work neatly.

- Keeping the working edge clean ensures a smooth transition to the next step and prevents visible flaws or interference.

Section B: Matching Activity

- A–4

- B–1

- C–2

- D–3

Section C: Written Response

- Proper frame preparation ensures secure attachment, proper foam shape, clean fabric finishes, and structural stability — all of which affect the final product’s comfort, safety, and appearance.

- Use correct tension, align webbing and springs, space fasteners consistently, mark placement in advance, and avoid placing fasteners or excess material near edges.

Marking Memo

| Question | Marks |
| --- | --- |
| Section A (1–4) | 8 |
| Section B (5–6) | 4 |
| Section C (7–8) | 6 |
| Total | 18 |
| Pass Mark (70%) | 13 |

📊 Assessment Rubric

| Criteria | Excellent (5) | Competent (3–4) | Needs Support (1–2) |
| --- | --- | --- | --- |
| Understanding of preparation principles | All key principles clearly identified and explained | Most principles identified with minor errors | Limited understanding; key principles missing or misapplied |
| Link to quality of end-product | Effect on quality is analysed in depth with clear examples | General awareness of impact on quality | Vague or no connection to product quality |
| Application in scenario | Scenario analysed accurately with justified recommendations | Partially correct analysis with some practical insight | Incomplete or unclear scenario interpretation |
| Accuracy in matching activity | All matches correct | One incorrect match | Two or more incorrect matches |
| Written response clarity | Written answers are logical, relevant, and clearly structured | Answers mostly relevant but may lack detail or clarity | Unclear, incomplete, or off-topic answers |

# KM-02-KT09: Knocker on techniques (15%)

Formative assessment

IAC0901 Techniques of selecting, measuring, cutting, edge bending, spacing and using hand and power tools to attach zig zag springs securely to a frame with the correct tension.

IAC0902 Techniques of selecting, measuring, cutting allowances, spacing, interweaving and using hand and power tools to attach webbing securely to a frame.

IAC0903 Techniques of spacing and using hand and power tools to attach coil springs securely to a webbing and to the frame.

IAC0904 Identify the types of knots and clips used when securing coil springs and zig zag springs to each other and to the frame.

(Weight 15%)

## Integrated Assessment – KM-02-KT09: Knocker-On Techniques

SAQA ID: 103199
Curriculum Code: 683401000
Assessment Type: Integrated Formative Assessment Using a Variety of Instruments
Weight: 15%

🔍 Internal Assessment Criteria Assessed:

- IAC0901: Techniques of selecting, measuring, cutting, edge bending, spacing and using hand and power tools to attach zig-zag springs securely to a frame with the correct tension.

- IAC0902: Techniques of selecting, measuring, cutting allowances, spacing, interweaving and using hand and power tools to attach webbing securely to a frame.

- IAC0903: Techniques of spacing and using hand and power tools to attach coil springs securely to a webbing and to the frame.

- IAC0904: Identify the types of knots and clips used when securing coil springs and zig-zag springs to each other and to the frame.

📝 SECTION A: Case Study and Scenario-Based Questions

Case Study:
Tamryn is preparing a set of armchair frames. She begins by installing webbing, using a webbing stretcher and stapler. However, she skips interlacing and does not pre-measure her spacing. She moves on to install zig-zag springs, but fails to curl the ends after cutting. Later, Sipho inspects the coil springs and notices that they are spaced unevenly and tied with standard string instead of approved upholstery twine. During the covering stage, several frames are flagged for loose springs, sagging in the seat, and squeaking noises.

Questions:

- Identify at least two errors Tamryn made during the installation of zig-zag springs. (IAC0901)

- What webbing installation techniques were incorrectly applied? (IAC0902)

- How should the coil springs have been secured and spaced correctly? (IAC0903)

- What is the correct type of knot or clip used to tie coil springs? (IAC0904)

- How do these errors affect the overall comfort, durability, and safety of the product?

✏️ SECTION B: Practical Demonstration Checklist (Simulated or Actual Frame)

Learners are provided with a frame and are required to demonstrate the following:

| Task | Achieved | Not Achieved |
| --- | --- | --- |
| Select and cut zig-zag spring to correct length (IAC0901) | ☐ | ☐ |
| Curl spring ends properly using tools (IAC0901) | ☐ | ☐ |
| Measure, cut and attach interlaced webbing with correct spacing (IAC0902) | ☐ | ☐ |
| Secure coil springs to webbing and frame with correct tools (IAC0903) | ☐ | ☐ |
| Identify and demonstrate tying a clove hitch or bowline knot or apply spring clips (IAC0904) | ☐ | ☐ |

🔁 SECTION C: Short Answer Questions

- List three tools used to attach webbing and describe their function. (IAC0902)

- What safety steps should be taken when cutting and curling zig-zag springs? (IAC0901)

- Explain why using incorrect knots or string to secure springs can be hazardous. (IAC0904)

- What are the consequences of incorrect spacing when installing coil springs? (IAC0903)

✅ Model Answers

Section A – Case Study Answers

- Tamryn did not curl the ends and failed to measure spring spacing.

- Webbing was not interlaced, and spacing was not pre-measured.

- Coil springs should be spaced evenly, clipped or nailed securely, and aligned centrally.

- Correct knots include clove hitch, bowline, or use of certified spring clips.

- Errors lead to sagging, instability, noise, and failure under load.

Section B – Performance Observations
(Marked by assessor during demonstration.)

Section C – Short Answers

- Webbing stretcher (for tension), staple gun (to secure), scissors or webbing knife (to cut to length).

- Wear gloves and goggles, secure spring before cutting, curl ends away from seat contact points.

- Incorrect knots can loosen under pressure, causing springs to shift or detach, posing injury risks.

- Uneven spring spacing leads to pressure points, discomfort, and possible fabric deformation.

🧮 Marking Memo

| Section | Marks Available |
| --- | --- |
| Section A (5 questions) | 10 marks |
| Section B (5 checklist items) | 10 marks |
| Section C (4 questions) | 8 marks |
| Total | 28 marks |
| Pass mark (70%) | 20 marks |

📊 Assessment Rubric

| Criteria | Excellent (5) | Competent (3–4) | Needs Support (1–2) |
| --- | --- | --- | --- |
| Zig-zag spring handling (IAC0901) | Clean cut, correct curl, secure installation | Minor curl errors, spring held adequately | Poor alignment, unsecured ends |
| Webbing installation (IAC0902) | Tensioned, interlaced, neat spacing | Acceptable tension, gaps in interlace | Loose, misaligned, or bunched |
| Coil spring mounting (IAC0903) | Evenly spaced, properly attached | Slight misalignment or irregular spacing | Poor spacing or failed fastening |
| Knots or clip usage (IAC0904) | Appropriate knot or clip, tight and clean | Knot holds but not recommended type | Wrong knot or insecure tie |
| Safety and tool handling | Excellent control, uses PPE | Reasonable handling, needs reminders | Unsafe or incorrect tool use |

# KM-02-KT10: Foaming-up or padding-up principles (10%)

Formative assessment

IAC1001 Foaming-up or padding-up principles are identified and described and the effect thereof on quality end-product is analysed.

(Weight 10%)

## Integrated Assessment – KM-02-KT10: Foaming-up or Padding-up Principles

SAQA ID: 103199
Curriculum Code: 683401000
Internal Assessment Criterion:

- IAC1001: Foaming-up or padding-up principles are identified and described and the effect thereof on quality end-product is analysed
Weight: 10%

📝 SECTION A: Case Study with Scenario-Based Questions

Case Study:
Themba is upholstering a pair of armchairs using a patterned jacquard fabric. He selects medium-density foam for the seat and high-density foam for the armrests. He sprays adhesive directly onto the fabric, presses it to the foam while still wet, and skips trimming the foam edges. Later, during the covering stage, the fabric wrinkles at the corners, the pleats are uneven, and the seams on the backrest do not align with the pattern. When the chairs are inspected, one arm looks slightly fuller than the other, and the left wing’s pleating differs from the right.

Questions:

- Identify three principles of foaming-up or padding-up that Themba applied incorrectly.

- What effect did skipping foam trimming have on the final appearance of the armchairs?

- Why is it important to wait for adhesive to become tacky before bonding?

- What steps could Themba have followed to ensure uniformity of both arms and wings?

- How does poor alignment of pattern and cover placement impact perceived product quality?

✏️ SECTION B: Short Answer Questions

- What is the function of shaping edges and corners correctly during upholstery?

- Describe the relationship between tension, stretch, and deflection of fabric.

- Why is the correct selection of foam density important for seating comfort and durability?

- How can incorrect spacing of tacks or staples affect both appearance and structure?

✅ Model Answers

Section A: Case Study

- Themba applied adhesive while wet, failed to shape edges, and did not ensure pleat uniformity.

- Skipping foam trimming led to bulky corners and uneven folds, causing wrinkling under the fabric.

- Wet adhesive reduces bonding strength and leads to movement or slippage.

- He could have used templates, measured foam and fabric folds, and compared sides visually before final fixing.

- Poor alignment of fabric and seams results in a product that looks rushed or defective, lowering client satisfaction.

Section B: Short Answers

- Correct shaping helps fabric wrap smoothly, prevents bulk, and improves aesthetics.

- Tension is the pulling force, stretch is the material's extension, and deflection is its response to pressure—these must be balanced to avoid sagging or tearing.

- Foam that is too soft wears out quickly; too firm may cause discomfort. Density affects both resilience and structure.

- Wide or uneven spacing can cause fabric to loosen, sag, or tear; tight, consistent spacing ensures a secure and tidy finish.

🧮 Marking Memo

| Section | Marks Available |
| --- | --- |
| Section A (5 questions) | 10 marks |
| Section B (4 questions) | 8 marks |
| Total | 18 marks |
| Pass mark (70%) | 13 marks |

📊 Assessment Rubric

| Criteria | Excellent (5) | Competent (3–4) | Needs Support (1–2) |
| --- | --- | --- | --- |
| Understanding of foaming-up principles | Demonstrates deep understanding across all KT areas | Applies most concepts correctly with minor errors | Misunderstands or omits key KT elements |
| Application to scenario | Applies principles logically with practical, corrective suggestions | Recognises issues with some correct recommendations | Identifies issues but lacks clarity or suggests incorrect actions |
| Fabric and foam alignment | Clearly explains impact on comfort, shape, and visual symmetry | Acknowledges alignment problems but lacks full detail | Does not link misalignment to outcome or customer perception |
| Use of terminology | Correctly uses upholstery terminology throughout | Occasional terminology errors, but meaning is clear | Limited use of relevant terms; unclear explanations |

# KM-02-KT11: Foaming-up or padding-up techniques (10%)

Formative assessment

IAC1101 Techniques of padding and edge rolls when covering coil springs and arms.

IAC1102 Techniques of selecting specified densities of foam, applying adhesive to bond foam-to-foam and using templates and tools to construct and shape the foam and attach it to the frame over the springs and webbing.

IAC1103 Techniques of selecting specified padding / stuffing materials, applying and attaching it by means of tacking, sewing, tying and bonding to the sprung foundations and frame, and shaping it to meet specifications.

IAC1104 Techniques of measuring, cutting and attaching the specified fabric to cover the foam and paddings.

IAC1105 Importance of following workplace procedures handle and stack and store completed prepared frame for the next upholstery procedure and explain the types and consequences of damage to the product.

(Weight 10%)

## Integrated Assessment for KM-02-KT11: Foaming-up or Padding-up Techniques

- SAQA ID: 103199

- Curriculum Code: 683401000

- Weight: 10%

- Internal Assessment Criteria: IAC1101 to IAC1105

This assessment includes a variety of instruments, model answers, a marking memo, and a rubric.

🧪 Integrated Assessment: KM-02-KT11 – Foaming-up or Padding-up Techniques

Assessment Structure and Instruments

| Section | Assessment Type | Assessment Method |
| --- | --- | --- |
| A | Scenario-Based Questions | Written Analysis |
| B | Practical Demonstration | Observation Checklist |
| C | Short Answer Questions | Written Test |

📝 SECTION A: Scenario-Based Questions (Written – Analytical)

Scenario:
Lerato is preparing a curved armchair frame for upholstery. She begins by padding the coil springs on the seat, but forgets to apply the edge rolls. She selects a medium-density foam to build up the arms and bonds the foam directly onto the webbing without shaping it. She cuts the foam roughly by hand and does not use templates. She then measures the fabric quickly, cuts it, and attaches it with uneven tacking. The completed unit is stored leaning against a wet surface in the corner of the workshop.

Answer the following questions:

- Identify two mistakes Lerato made in the padding and edge roll process. (IAC1101)

- How should Lerato have selected and applied foam for the curved arms, and what tools should have been used? (IAC1102)

- What fastening techniques should have been used to attach padding to the sprung foundation? (IAC1103)

- What could Lerato have done to ensure accuracy when measuring and cutting the fabric? (IAC1104)

- Describe the potential consequences of storing the completed piece incorrectly. (IAC1105)

🔧 SECTION B: Practical Demonstration (Assessor Observation)

Learners are required to demonstrate the following on a frame or simulated structure:

| Task | Criteria Met | Comments |
| --- | --- | --- |
| Apply edge rolls and padding to a coil spring seat correctly. (IAC1101) | ☐ Yes / ☐ No |  |
| Select foam by density and shape foam using template. (IAC1102) | ☐ Yes / ☐ No |  |
| Attach padding to a sprung foundation using correct methods. (IAC1103) | ☐ Yes / ☐ No |  |
| Measure and cut cover fabric accurately. (IAC1104) | ☐ Yes / ☐ No |  |
| Demonstrate correct handling and temporary storage of completed unit. (IAC1105) | ☐ Yes / ☐ No |  |

✏️ SECTION C: Short Answer Questions (Written Knowledge Test)

- What is the purpose of shaping foam to match the design profile? (IAC1102)

- List two tools commonly used when cutting and shaping layered foam. (IAC1102)

- Explain why symmetry is important when attaching padding and fabric. (IAC1104)

- State two workplace procedures that help prevent product damage during storage. (IAC1105)

✅ Model Answers

Section A

- She omitted the edge rolls and padded directly over unprepared springs.

- She should have selected high-density foam for structure, used a template to shape, and applied adhesive correctly.

- Tying and tacking techniques should have been used for secure attachment to the coil foundation.

- Use a flexible tape, mark lines with chalk, and double-check measurements before cutting.

- Moisture and pressure can deform or damage foam, loosen adhesive, and stain or mildew the fabric.

Section C

- To create ergonomic comfort and maintain visual design accuracy.

- Foam knife, sanding block, electric foam cutter.

- To ensure balanced appearance and consistent comfort.

- Stack frames flat on dry, padded surfaces; avoid placing against sharp or damp objects.

🧮 Marking Memo

| Section | Marks |
| --- | --- |
| Section A | 10 |
| Section B | Practical – Competent / Not Yet Competent |
| Section C | 8 |
| Total | 18 (Written component) |
| Pass mark (70%) | 13 marks minimum, plus competency in Section B |

📊 Assessment Rubric

| Criteria | Excellent (5) | Competent (3–4) | Needs Support (1–2) |
| --- | --- | --- | --- |
| Understanding of Techniques | Clearly demonstrates full understanding of all KT elements. | Demonstrates reasonable understanding with minor gaps. | Lacks understanding; confusion about KT concepts. |
| Application of Process | Applies logical, complete process aligned with design sequence. | Applies general process with minor order or detail gaps. | Incorrect or incomplete application of sequence. |
| Use of Tools and Materials | Chooses correct tools and foam types confidently and safely. | Chooses acceptable tools but may need guidance. | Uses inappropriate tools or lacks control/safety. |
| Workplace Practice | Fully adheres to safety and storage procedures. | Mostly compliant with safe handling and stacking. | Fails to apply key workplace practices. |
| Communication | Uses accurate upholstery terminology and explains clearly. | Understands terms, minor language errors. | Limited vocabulary or unclear explanation. |

# KM-02-KT12: Joints and bracings of the frame (5%)

## Integrated Assessment – KM-02-KT12: Joints and Bracings of the Frame

SECTION A: Scenario-Based Questions (Written – Analytical)

Scenario:
Zama is working in the upholstery workshop and receives three different types of chair frames: one with mortise and tenon joints and wooden corner blocks; another with dowel joints but no additional bracing; and a third with glued butt joints reinforced by metal plates. She is asked to assess the joints and bracings in terms of strength, alignment, and suitability for upholstery before beginning any work.

Questions:

- Identify the types of joints present in each of the three chairs and describe the main structural characteristic of each.

- Which frame(s) may require additional reinforcement before upholstering, and why?

- Compare the manufacturing techniques that would have been used for the mortise and tenon chair versus the dowelled chair.

- Suggest a bracing solution to improve the frame with butt joints.

- Explain how poor alignment or overcut joints can impact the upholstery process and product durability.

SECTION B: Practical Activity (Assessor Observation)

Learners are given physical samples or diagrams of frame components. They must:

| Task | Criteria Met (Yes/No) | Assessor Comments |
| --- | --- | --- |
| Correctly identify three types of joints used in furniture frames | ☐ Yes / ☐ No |  |
| Describe one use and one requirement for each joint identified | ☐ Yes / ☐ No |  |
| Suggest a suitable bracing method for an open-corner or butt-jointed frame | ☐ Yes / ☐ No |  |
| Identify at least one visible fault in a frame joint (gap, movement, overcut, etc.) | ☐ Yes / ☐ No |  |

SECTION C: Short Answer Questions (Written – Theoretical)

- Name two tools used in the manufacturing of wooden joints in furniture frames.

- What is the function of bracing in a chair frame?

- List one advantage and one disadvantage of dowel joints.

- Why must glue be allowed to cure before the frame is upholstered?

- What signs indicate that a joint may fail under load?

✅ Model Answers

Section A

- Mortise and tenon (strong interlocking), dowel (moderate strength, precision required), butt joint (weak, needs reinforcement).

- The dowel and butt-jointed frames may require additional bracing due to lower load capacity.

- Mortise and tenon is typically hand-cut or machined with a mortiser and tenoner; dowels require drill jigs and dowel pins with adhesive.

- Add wooden corner blocks or a steel plate at the joint for reinforcement.

- Misaligned joints affect frame levelness and cause uneven padding and cover tension, which can shorten the furniture’s lifespan.

Section C

- Mortiser, dowel jig, table saw.

- Bracing prevents movement and maintains structural integrity under pressure.

- Advantage: easy and fast to manufacture; Disadvantage: less strong than interlocking joints.

- If not cured, glue joints may slip or fail under pressure, affecting frame strength.

- Gaps, movement when pressed, cracking sounds, or visible misalignment.

🧮 Marking Memo

| Section | Marks Available |
| --- | --- |
| Section A | 10 |
| Section B | Competency-based |
| Section C | 10 |
| Total | 20 marks |
| Pass Mark (70%) | 14 marks + competency in Section B |

📊 Assessment Rubric

| Criteria | Excellent (5) | Competent (3–4) | Needs Support (1–2) |
| --- | --- | --- | --- |
| Understanding of Joint Types | Clearly explains multiple joint types with correct uses and examples | Identifies and describes basic types with some detail | Lists types with minimal explanation or incorrect descriptions |
| Application of Concepts | Matches joints to use, describes impact on frame and upholstery | General application with some understanding of structural role | Misunderstands relationship between joints and furniture performance |
| Manufacturing Knowledge | Shows insight into how joints and bracings are made and evaluated | Describes basic manufacturing processes with clarity | Lacks understanding of process or steps |
| Practical Observation | Confidently identifies and inspects physical frame samples | Identifies most joint types with occasional prompting | Struggles to identify joints or recognise defects |
| Communication | Uses technical vocabulary accurately and clearly | Uses some terminology appropriately | Language vague, lacks specific terms or uses incorrectly |