UPHOLSTERERY 	FRAME PREPARER

KM01 BASIC PRINCIPLES OF MANUFACTURING OF UPHOLSTERED FURNITURE

FACILITATOR GUIDE

Table of Contents

KM-01: Basic Principles for Manufacturing of Upholstered Furniture	5

KM-01-KT01: Furniture Types, Uses and Styles	7

KT0101 – Furniture Types and Products and Their Uses	9

KT0102 – Styles of Upholstered Furniture, Including Other Soft Furnishings	12

KT0103 – Historical Factors and Technology Influencing Furniture Design	15

Integrated Formative Assessment: KM-01-KT01	18

KM-01-KT02: Upholstered Furniture Construction	22

KT0201 – Terminology Used in the Upholstery Industry	24

KT0202 – Standard Dimensions of Upholstered Furniture	27

KT0203 – Furniture Frame Construction Including Parts Identification, Joints and Assembly Techniques	30

KT0204 – Furniture Frame Construction Materials	33

KT0205 – Traditional and Modern Foundations Including Coil Springs, Animal and Vegetable Fibre Paddings, and Foam	36

KT0206 – Furniture Cover Construction Including Parts Identification, Joints and Assembly Techniques	39

KT0207 – Furniture Upholstery Materials Including Natural and Man-Made Fabric and Finishings	42

KT0208 – Impact of Design and Construction Faults	45

🌟 Integrated Formative Assessment: KM-01-KT02	48

KM-01-KT03: Ergonomics Related to Furniture Manufacturing	52

KT0301 – Definition of Ergonomics	54

KT0302 – Principles of Ergonomics	57

KT0303 – Purpose of Ergonomics in Furniture Manufacturing	60

KT0304 – Concepts of Ergonomics Pertaining to Upholstered Furniture	63

KT0305 – Standard Measurements for Furniture	66

KT0306 – Lifting and Stacking	69

KT0307 – Moving Materials and Equipment	72

🌟 Integrated Formative Assessment: KM-01-KT03 – Ergonomics Related to Furniture Manufacturing	75

🌟 Integrated Formative Assessment: KM-01-KT03 – Ergonomics Related to Furniture Manufacturing	78

KM-01-KT04: Quality in the Upholstered Furniture Manufacturing Process	83

KT0401 – Definition and Importance of Quality, Quality Control and Quality Assurance	85

KT0402 – Principles and Practices of Quality Control	88

KT0403 – Principles and Practices of Quality Assurance	91

KT0404 – Types and Causes of Quality Problems	95

KT0405 – Consequences of Poor Quality Management	98

KT0406 – Production Routines, Product Inspections and Quality Control	101

KT0407 – Concepts of Product Defects and Faults and Their Causes	104

KT0408 – Principles and Concepts of Final Quality Check for Product Defects and Faults	107

KT0409 – Reporting Procedures for the Final Prepared Frame	110

KT0410 – Materials Handling and Product Stacking and Storing for the Next Process	113

KT0411 – Storage and Dispatch of Upholstered Frame Components and Complete Products	116

Integrated Formative Assessment (Case Study Based)	119

KM-01-KT05: Compressors and Compressed Air	123

KT0501 – Compressed Air	125

KT0502 – Compressed Air Generation	127

KT0503 – Compressed Air Properties	130

KT0504 – Compressed Air Uses	133

KT0505 – Problem Identification and Solving	136

KT0506 – Hazards and Risks	139

KT0507 – Pneumatic Tools	142

Integrated Formative Assessment KM-01-KT05: Compressors and Compressed Air	145

KM-01-KT06: Drawings for Furniture Manufacturing	148

KT0601 – Sketches and Engineering Drawings	149

KT0602 – Isometric Views	152

KT0603 – Lines Used	154

KT0604 – Hidden Detail	157

KT0605 – Legends and Symbols	160

Integrated Formative Assessment – KM-01-KT06	163

KM-01-KT07: Productivity and Efficiency in the Production of Upholstered Furniture	167

KT0701 – Productivity	168

KT0702 – Efficiency	170

KT0703 – Waste Management	172

KT0704 – Quality of Raw Material and Consumables	175

KT0705 – Raw Material and Consumables Faults and Defects	177

KT0706 – Product Quality of the Frame Preparation and Upholstery Processes and Defects	180

Integrated Formative Assessment – KM-01-KT07	183

KM-01-KT08: Upholstery Related Health and Safety Principles and Practices	186

KT0801 – Occupational Health and Safety Act, Regulations and Responsibilities of Employer and Employee	188

KT0802 – Hazards, Safety, Hazardous Substances, Incidents	190

KT0803 – Hazard Identification and Safe Work Procedures	192

KT0804 – Incidents and Incident Investigation	194

KT0805 – Safety and Information Signs, Location of First Aid and Firefighting Equipment and Personnel, Emergency Evacuation Procedures and Meeting Points	196

KT0806 – Hazardous Substances, MSDS and Storage	199

KT0807 – Personal Protective Equipment (PPE)	202

KT0808 – First Aid and First Aiders in the Workplace	204

KT0809 – Fire Extinguishers and Fire Fighters in the Workplace	206

KT0810 – Emergency Procedures: Evacuation and Emergency Meeting Points	209

Integrated Formative Assessment – KM-01-KT08	211

KM-01-KT09: Housekeeping, Cleaning and Record Keeping	215

KT0901 – Housekeeping Principles and Requirements	217

KT0902 – Cleaning Agents and Equipment	219

KT0903 – Routine Cleaning	221

KT0904 – Product Change Cleaning	223

KT0905 – Contamination	225

Integrated Formative Assessment – KM-01-KT09: Housekeeping, Cleaning and Record Keeping	227

# KM-01: Basic Principles for Manufacturing of Upholstered Furniture

NQF Level: 2 | Credits: 4

The purpose of Knowledge Module KM-01 is to provide learners with a foundational understanding of the principles that underpin the manufacture of upholstered furniture. Positioned at NQF Level 2, this module serves as an essential entry point into the upholstery profession by introducing key concepts, processes, and practices that support the initial stages of furniture production.

At the heart of this module is the aim to develop a learner's ability to recognise and apply basic principles related to upholstery design, frame construction, material selection, and production preparation. The learning content is specifically curated to enable learners to understand how furniture is conceived, planned, and prepared for the upholstering process within a manufacturing environment.

Through this module, learners will explore the diverse types and styles of furniture, and how these relate to both form and function in various contexts. They will develop an understanding of the structural elements involved in upholstered furniture construction and the importance of ergonomic considerations in the design and manufacture of functional and comfortable products. The module also introduces learners to the critical role of quality in production, equipping them to identify and apply quality standards in manufacturing processes.

Practical knowledge of tools and systems is embedded in the learning outcomes, including an introduction to compressors and compressed air, which are frequently used in upholstery workshops. Learners will also gain a basic understanding of how to read and interpret furniture manufacturing drawings, which serve as visual guides for design and assembly.

A strong emphasis is placed on the concepts of productivity and efficiency in the production process, encouraging learners to adopt practices that contribute to streamlined operations and optimal output. Equally important are the topics of health and safety, as well as housekeeping and record keeping, which instil safe, clean, and organised working habits aligned with industry standards.

The module is structured around nine topic areas, each contributing a specific weighting to the overall content:

- KM-01-KT01: Furniture types, uses and styles (5%)

- KM-01-KT02: Upholstered furniture construction (15%)

- KM-01-KT03: Ergonomics related to furniture manufacturing (10%)

- KM-01-KT04: Quality in the upholstered furniture manufacturing process (20%)

- KM-01-KT05: Compressors and compressed air (10%)

- KM-01-KT06: Drawings for furniture manufacturing (5%)

- KM-01-KT07: Productivity and efficiency in the production of upholstered furniture (10%)

- KM-01-KT08: Upholstery-related health and safety principles and practices (20%)

- KM-01-KT09: Housekeeping, cleaning and record keeping (5%)

By completing this knowledge module, learners will be equipped with the essential theoretical grounding to participate meaningfully in the initial phases of upholstered furniture production, while also fostering safe, efficient, and quality-driven work practices.

## KM-01-KT01: Furniture Types, Uses and Styles

Weight: 5%

This knowledge topic introduces learners to the foundational understanding of furniture types, their intended uses, and the diverse styles of upholstered furniture, including associated soft furnishings. It sets the stage for appreciating the wide variety of furniture products that exist in both contemporary and traditional environments, while also tracing the historical and technological developments that have influenced furniture design over time.

The learning begins with an exploration of different types of furniture—ranging from seating and tables to storage and beds—alongside an explanation of their practical uses in domestic, commercial, and specialised settings. Learners are encouraged to recognise the functional purpose of each type of furniture and how form is influenced by its intended use.

The topic then delves into styles of upholstered furniture, including visual characteristics, form, and finish. Learners will be introduced to a range of aesthetic styles such as traditional, classic, modern, minimalist, and eclectic, as well as the influence of culture and fashion in furniture trends. This extends to soft furnishings that complement upholstered furniture, such as cushions, loose covers, and bolsters.

A critical part of this topic is the historical progression of furniture design. Learners will explore how furniture has evolved through the ages, particularly noting the impact of historical events and technological change. This includes examining how the Industrial Revolution, world wars, and the invention of new materials and manufacturing techniques reshaped both the design and the accessibility of furniture. Traditional foundations, such as the use of coil springs, animal hair, vegetable fibre padding, and foam, will be discussed in context to understand the shift in materials and comfort expectations over time.

This knowledge topic aligns with the following topic elements:

- KT0101: Furniture types and products and their uses

- KT0102: Styles of upholstered furniture, including other soft furnishings

- KT0103: Historical factors and technology influencing the design and development of furniture, including traditional foundations like coil springs, animal and vegetable fibre paddings, and foam

Learning will be assessed through the following Internal Assessment Criteria:

- IAC0101: Different types of furniture are identified and the respective uses are explained

- IAC0102: The main historical factors and eras that influenced the development of furniture design, including the impact of the industrial revolution, world wars, development of new materials, and technology are outlined

- IAC0103: Various types and styles of upholstered furniture, including other soft furnishings, are identified and respective uses are explained

By the end of this topic, learners will have a well-rounded understanding of the broad categories of furniture, the ways in which they serve human needs, and how style, history, and innovation have collectively shaped the upholstered furniture that surrounds us today.

### KT0101 – Furniture Types and Products and Their Uses

Purpose of this Topic Element

This topic introduces learners to the different types of furniture commonly found in domestic, commercial, institutional, and specialised environments. It supports the development of a foundational vocabulary and contextual awareness in the field of upholstered furniture. Understanding furniture by type and function allows learners to make informed choices during design, manufacturing, and client consultation processes.

Key Content to Cover

- Definitions and Classifications of Furniture Types

  - Seating: Sofas, armchairs, dining chairs, office chairs, ottomans

  - Sleeping: Beds, sleeper couches, bunk beds, headboards

  - Tables: Coffee tables, side tables, dining tables, office desks

  - Storage: Wardrobes, chests, credenzas, filing cabinets

  - Specialised Furniture: Recliners, medical chairs, theatre seating, hospitality booths

- Functional and Ergonomic Considerations

  - Intended use (e.g., comfort, support, posture)

  - Duration and context of use (e.g., long-term office seating vs. occasional guest chairs)

- Application of Upholstery in Different Furniture Types

  - Where and why upholstery is applied: seating surfaces, arms, backs, panels

  - Material considerations: foam density, textile durability, ergonomic shaping

- Domestic vs Commercial Use

  - Domestic: aesthetics, personal comfort, style preferences

  - Commercial: durability, compliance with safety standards, ease of maintenance

Practical Examples

| Furniture Type | Example Product | Primary Use |
| --- | --- | --- |
| Sofa | 3-seater upholstered sofa | Domestic seating for comfort |
| Office Chair | High-back swivel chair with foam cushioning | Prolonged seated work environments |
| Ottoman | Upholstered storage ottoman | Footrest, seat, and storage unit |
| Theatre Chair | Reclining padded seat with cup holder | Auditorium seating |
| Sleeper Couch | Converts to a bed with fold-out mechanism | Small-space living or guest seating |

South African Case Study: Hospitality Sector

Case: A Cape Town-based upholstery company was commissioned to provide custom seating for a boutique hotel in Stellenbosch. The brief required them to design furniture that offered comfort, durability, and aesthetic alignment with the historic building’s interiors.

Discussion Points:

- What types of furniture were needed?

- How did the use (e.g., lobby vs. dining area) affect the upholstery choices?

- What materials were most appropriate for high-use public environments?

- How might the company ensure their pieces remained functional and attractive over time?

Global Case Study: Sustainable Furniture Design

Case: A Scandinavian company has developed modular sofas made from sustainable frames and recycled textile upholstery. The modularity allows users to reconfigure furniture depending on need.

Discussion Points:

- What types of users might benefit from this?

- How could this concept be adapted in a South African urban context?

- What implications does modularity have on manufacturing and upholstering?

Facilitator-Led Activities

- Furniture Walkaround
Visit different departments or use images to identify furniture by type. Ask learners to record:

  - Function

  - Upholstery areas

  - Observed materials

  - User profile (e.g., adult, child, formal guest)

- Matching Activity
Provide cards with furniture names and a separate set with their primary functions. Learners must match them and explain why.

- Material Analysis Table
Provide swatches of various foam and fabric materials. Let learners assign appropriate matches to a list of furniture items based on function and user needs.

Critical Thinking Questions

- How does the intended use of a furniture piece influence the type and style of upholstery applied?

- In what ways does upholstered furniture differ between domestic and commercial settings? Consider both function and aesthetic.

- Why might an upholstered dining chair in a restaurant require different design and material decisions compared to one in a private home?

- How could advances in technology or material science change the types of upholstered furniture produced in the future?

- What ethical or sustainability issues should furniture manufacturers consider when designing upholstered furniture?

### KT0102 – Styles of Upholstered Furniture, Including Other Soft Furnishings

Purpose of this Topic Element

This topic explores the stylistic diversity of upholstered furniture and its associated soft furnishings. Learners will develop an understanding of how aesthetic choices, historical influences, materials, and user needs inform the design and classification of styles. They will also learn to distinguish between traditional, contemporary, and emerging styles within various furniture categories, and to appreciate the role of soft furnishings in enhancing function and appearance.

Key Content to Cover

- Definition of Style in Furniture

  - Style refers to the visual design, form, detailing, and materials that characterise a particular era, movement, or trend.

  - Includes cultural, architectural, and functional influences.

- Major Upholstered Furniture Styles

  - Traditional/Classical: Victorian, Edwardian, Georgian

  - Mid-century Modern: Minimal, organic shapes, wooden legs

  - Contemporary/Modern: Clean lines, metal/foam materials

  - Rustic/Farmhouse: Earth tones, natural textiles, tufted upholstery

  - Eclectic/Fusion: Mixed design influences and bold upholstery

- Common Soft Furnishings

  - Cushions, bolsters, throws, curtains, and loose covers

  - Their roles: decorative, comfort-enhancing, protective, seasonal update

- Influencing Factors

  - Design movements, fashion, architecture

  - Cultural heritage and regional preferences

  - Available materials and technologies

- Style Identification and Interpretation

  - Recognising key visual and structural elements

  - Matching furniture and furnishings to interior themes

Practical Examples

| Style | Visual Features | Common Soft Furnishings |
| --- | --- | --- |
| Victorian | Tufted backs, carved wood, velvet upholstery | Tasselled cushions, heavy curtains |
| Mid-Century Modern | Low profiles, tapered legs, smooth leather or textured fabric | Bolsters, geometric-patterned cushions |
| Industrial | Exposed framework, neutral tones, leather and metal combo | Simple leather cushions, exposed zips |
| Contemporary | Minimalist, modular, monochrome palettes | Plain cushions, removable covers |
| African-Inspired | Bold prints, carved hardwood frames | Shweshwe or wax-print cushions, grass-texture |

South African Case Study: Blending Heritage with Modern Design

Case: A Johannesburg-based design firm was commissioned to furnish a luxury game lodge. The client requested pieces that reflect South African cultural identity while maintaining a modern aesthetic.

Discussion Points:

- What upholstered furniture styles would suit this context?

- How might African motifs or materials be incorporated without compromising comfort or durability?

- What role do soft furnishings play in expressing cultural identity?

International Case Study: Scandinavian Style in Global Markets

Case: A Cape Town furniture retailer imports and locally reproduces Scandinavian-style sofas for young urban professionals. The demand is high due to their affordability, clean lines, and adaptability in small apartments.

Discussion Points:

- What defines Scandinavian design in upholstered furniture?

- Why might this style appeal to the South African market?

- How can local materials be adapted to reproduce this style affordably?

Facilitator-Led Activities

- Style Identification Gallery

  - Display images of various upholstered furniture styles. Ask learners to:

    - Identify the style

    - Describe key features

    - Suggest suitable soft furnishings

- Design Challenge

  - Provide a room theme (e.g., heritage lounge, corporate reception). Learners must:

    - Choose an appropriate furniture style

    - Recommend complementary soft furnishings

    - Justify their choices

- Style Timeline Activity

  - Construct a historical timeline of furniture styles. Mark key events that influenced each (e.g., industrial revolution, Bauhaus movement, digital age).

Critical Thinking Questions

- How do furniture styles reflect the social and cultural values of a specific time or place?

- In what ways do soft furnishings influence the overall perception and usability of a furniture piece?

- Can a single piece of furniture blend multiple styles effectively? Why or why not?

- What challenges might a manufacturer face in producing traditional-style furniture using modern materials and machinery?

- How can South African furniture designers create contemporary products that preserve cultural heritage and local craftsmanship?

### KT0103 – Historical Factors and Technology Influencing Furniture Design

Purpose of this Topic Element

This topic provides learners with an understanding of how furniture design and development have been shaped by significant historical events and technological progress. By tracing the evolution of materials, tools, and design philosophies, learners can contextualise modern furniture production and appreciate how past innovations continue to influence current industry practices. This knowledge is essential for recognising traditional methods and components still present in upholstery today, and for understanding how new technologies create new possibilities for design and manufacture.

Key Content to Cover

- Historical Eras and Influences

  - Pre-industrial period: Handmade furniture, craftsmanship, use of natural fibres (e.g., straw, horsehair)

  - Industrial Revolution: Mechanisation, mass production, standardisation of components

  - World War periods: Scarcity of materials, introduction of alternative textiles and fillers (e.g., latex, synthetic fibres), modular design

  - Post-war consumer boom: Expansion of furniture markets, demand for affordable, comfortable home furnishings

  - Digital era: CAD design, CNC cutting, smart fabrics, modular and flat-pack furniture

- Traditional Upholstery Foundations

  - Use of coil springs, jute webbing, animal hair, vegetable fibre padding, and early foam types

  - Stitching techniques and layer structuring in heritage furniture

- Material and Technological Advancements

  - Transition from natural to synthetic padding materials

  - Introduction of foams (e.g., polyurethane), composites, high-resilience materials

  - Rise of engineered woods, adhesives, automated cutting machines

- Influence on Design Aesthetics

  - Shifts in visual and functional design based on material and tool availability

  - The simplification of forms due to production efficiency

  - Emphasis on ergonomics, sustainability, and modularity in recent decades

Practical Examples

| Period/Event | Impact on Furniture Design |
| --- | --- |
| Industrial Revolution | Standardised dimensions, turned wooden legs, expansion of the middle-class market |
| World War II | Metal frames, tubular steel, simple and compact designs |
| 1950s – Mid-century Modern | Foam padding, bright colours, focus on form and comfort |
| 1980s – Technological Rise | CNC routers, mass-produced plastic moulded seats, new textiles |
| Present Day | Smart furniture, flat-pack design, recycled and sustainable materials |

South African Case Study: Heritage Furniture and Traditional Methods

Case: A small upholstery business in KwaZulu-Natal specialises in restoring antique colonial-era furniture. The workshop uses traditional materials such as horsehair padding and coil springs. While modern foam is available, clients request authentic restoration.

Discussion Points:

- What historical construction methods are still in use?

- How do traditional materials differ in durability and comfort?

- What challenges arise in sourcing heritage materials today?

International Case Study: Foam Innovation Post-WWII

Case: After WWII, foam became the primary cushioning material in Western countries, replacing traditional fillings due to affordability and mass production needs. Designers began experimenting with organic shapes and colour, leading to the rise of mid-century modern style.

Discussion Points:

- How did new materials influence design innovation?

- What limitations might foam present that natural materials do not?

- How do current environmental concerns impact foam usage today?

Facilitator-Led Activities

- Material Comparison Table

  - Learners compare traditional and modern padding materials (coil springs, horsehair, foam, polyester) in terms of cost, comfort, durability, and sustainability.

- Timeline Building Exercise

  - Groups map out major historical periods and technological innovations in furniture, linking them to examples and material use.

- “Then and Now” Display

  - Use images of heritage and modern upholstered furniture. Learners analyse:

    - Materials used

    - Construction methods

    - Visual changes

Critical Thinking Questions

- How did the Industrial Revolution alter the role of the furniture maker and upholsterer?

- In what ways has the introduction of synthetic materials changed consumer expectations for comfort and affordability?

- Can traditional upholstery techniques coexist with modern production methods? Why or why not?

- What ethical or environmental considerations should guide the future selection of upholstery materials?

- How does understanding historical context support the work of a modern upholsterer?

## Integrated Formative Assessment: KM-01-KT01

Topic: Furniture Types, Uses and Styles
Assessment Weight: 5%
Internal Assessment Criteria Assessed:

- IAC0101: Different types of furniture are identified and the respective uses are explained

- IAC0102: The main historical factors and eras that influenced the development of furniture design including the impact of the industrial revolution, world wars, development of new materials and technology are outlined

- IAC0103: Various types and styles of upholstered furniture, including other soft furnishings are identified and respective uses are explained

📝 Assessment Task

Instructions to Learners
You are required to complete all three sections of this assessment. Use full sentences where applicable. You may refer to learning materials and case studies discussed during class activities.

Section A: Matching (Furniture Types and Uses)

(Instrument Type: Matching | Aligned to IAC0101)
Total: 6 marks

Match each type of furniture in Column A with its correct use in Column B. Write only the letter of the correct match next to the number.

| Column A: Furniture Type | Column B: Use |
| --- | --- |
| 1. Ottoman | A. Sleeping furniture for small spaces |
| 2. Dining chair | B. Seating in reception and lounges |
| 3. Sleeper couch | C. Informal seating and footrest |
| 4. Work desk | D. Eating or dining support seating |
| 5. Armchair | E. Workstation for office or home |
| 6. Headboard | F. Decorative and supportive back panel for bed |

Section B: Short Answer (Historical Factors and Design)

(Instrument Type: Short Answer | Aligned to IAC0102)
Total: 9 marks

Answer the following questions:

- Name three historical events or eras that influenced furniture design and briefly describe how each impacted the design or production of furniture. (6 marks)

- List two ways in which the Industrial Revolution changed how furniture was made. (3 marks)

Section C: Source-Based Activity (Furniture Styles and Soft Furnishings)

(Instrument Type: Visual Source-Based | Aligned to IAC0103)
Total: 10 marks

Study the image of the upholstered lounge furniture below (provided in your workbook or projected in class). Then answer the questions that follow.

(Note to facilitator: Provide an image showing a Victorian-style tufted armchair and a minimalist modern sofa with scatter cushions and a throw.)

Questions:

- Identify the style of each furniture item shown. (2 marks)

- Name one soft furnishing visible in each furniture item and explain its use. (4 marks)

- Explain how the design of the two items differs in terms of style and materials. (4 marks)

✅ Model Answers and Marking Memo

Section A: Matching

(1 mark each)

| Number | Correct Answer |
| --- | --- |
| 1 | C |
| 2 | D |
| 3 | A |
| 4 | E |
| 5 | B |
| 6 | F |

Total: 6 marks

Section B: Short Answer

- (2 marks per event and explanation – max 6 marks)

  - Industrial Revolution: Introduced machinery, enabling mass production and reduced need for handcrafting.

  - World War II: Material shortages led to the use of synthetic textiles and simplified designs.

  - Post-war boom (1950s–60s): Focus shifted to comfort and affordability, leading to widespread use of foam and standardised components.

- (1½ marks per answer – max 3 marks)

  - Furniture was mass-produced using machines.

  - Use of new materials like steam-bent wood and later metal frames.

Section C: Source-Based Activity

- (1 mark each – max 2 marks)

  - Armchair: Victorian style

  - Sofa: Contemporary / minimalist

- (2 marks per furnishing – name + use – max 4 marks)

  - Armchair: Cushion with tassels – for comfort and decoration

  - Sofa: Throw – for warmth and to protect upholstery

- (2 marks for style comparison + 2 marks for material use – max 4 marks)

  - Style: Victorian furniture is ornate with curves and tufting; minimalist furniture is sleek with clean lines.

  - Materials: Victorian uses velvet and wood; contemporary may use cotton blend and chrome or steel.

📊 Assessment Rubric: KM-01-KT01 Integrated Assessment

| Assessment Criteria | Outstanding (5) | Competent (4) | Partially Competent (3) | Not Yet Competent (1–2) |
| --- | --- | --- | --- | --- |
| IAC0101: Furniture types and uses | Accurately matches all types with detailed justifications | Correctly identifies and matches most types | Identifies some types; limited reasoning | Confused or incorrect matches; no reasoning |
| IAC0102: Historical factors and influence on design | Clearly outlines multiple historical factors with relevance | Outlines most factors with fair relevance | Lists a few general events | Few or no relevant historical references |
| IAC0103: Styles and soft furnishings – identification and use | Distinguishes styles with excellent clarity and application | Identifies most features and uses | Basic style recognition; limited depth | No or incorrect identification of styles or uses |

## KM-01-KT02: Upholstered Furniture Construction

Weight: 15%

This knowledge topic introduces learners to the fundamental construction principles and technical terminology associated with the manufacture of upholstered furniture. As the core of the furniture upholstery trade, a clear understanding of how frames and upholstery materials interact is essential for producing structurally sound, comfortable, and visually appealing products.

Learners begin by familiarising themselves with key terminology commonly used in the upholstery industry. This shared technical language enables clear communication between design, production, and quality assurance teams. The topic also introduces standard dimensions used in furniture manufacturing and how these measurements relate to comfort, function, and ergonomics. Understanding these standards forms the basis for accurate and consistent product development.

A critical part of this topic is dedicated to the construction of furniture frames. Learners explore various types of joints, structural parts, and assembly techniques, as well as the wide range of materials used for frame construction. These include hardwoods and softwoods, composite wood products, plastics, fibreglass, and metals. Special attention is paid to how material characteristics influence the strength, weight, and durability of the final product, as well as how they affect the upholstery process.

The module also covers both traditional and modern furniture foundations. Learners investigate historical methods such as the use of coil springs and animal or vegetable padding, alongside modern practices involving foam, webbing, and synthetic fill materials. This comparative approach helps learners understand how advancements in materials and tools have shaped current manufacturing methods.

In addition to structural aspects, learners will study the construction of furniture covers, including identification of key components, seams, joins, and finishing techniques. The topic concludes by addressing common construction and design faults. Learners evaluate how poor construction practices or defective materials can compromise the cover preparation process, the visual quality of the product, and overall production efficiency.

The topic content is structured around the following eight elements:

- KT0201: Terminology used in the upholstery industry

- KT0202: Standard dimensions of upholstered furniture

- KT0203: Furniture frame construction including parts identification, joints and assembly techniques

- KT0204: Furniture frame construction materials including hardwood, soft wood, composite wood products, plastic, fibreglass and metal frames

- KT0205: Traditional and modern foundations including coil springs, animal and vegetable fibre paddings, and foam

- KT0206: Furniture cover construction including parts identification, joints and assembly techniques

- KT0207: Furniture upholstery materials including natural and man-made fabric and finishings

- KT0208: Impact of design and construction faults

Assessment of this topic is based on the following Internal Assessment Criteria:

- IAC0201: The influence of standard dimensions and the science of ergonomics on the design of upholstered furniture is reasoned

- IAC0202: Parts of various types of upholstered furniture are identified using the correct abbreviations and the respective functions are explained

- IAC0203: Types of traditional and modern materials used to produce seat frames are identified and the effect of their characteristics on the upholstery process is reasoned

- IAC0204: Components used for various upholstered furniture products are identified

- IAC0205: The consequences of poor frame construction and raw material faults and defects on the cover preparation process, final product, and the production output of the department are evaluated

By the end of this topic, learners will have developed the competence to identify and reason through construction principles, apply technical vocabulary, recognise and classify core materials, and assess the impact of quality and design faults on the upholstery process. This foundational knowledge supports accurate decision-making and collaboration across the upholstery production workflow.

### KT0201 – Terminology Used in the Upholstery Industry

Purpose of this Topic Element

The aim of this topic is to introduce learners to the technical language and standard terminology used within the furniture upholstery sector. Effective communication is a key competency in the workplace, particularly when interpreting work instructions, collaborating with colleagues, understanding technical drawings, and reporting on production quality or faults. Mastery of correct terms builds professional credibility, prevents errors, and supports occupational health and safety.

Key Content to Cover

- Importance of Industry Terminology

  - Promotes clarity in communication across production teams

  - Minimises errors caused by misinterpretation

  - Ensures consistency in documentation and labelling

- Basic Upholstery Terms

  - Frame – the base structure of a furniture piece

  - Webbing – strong woven bands used for seat support

  - Padding – materials like foam or fibre used to create comfort

  - Tack strip – a strip used to secure fabric edges tightly

  - Piping (or Welt) – a fabric-covered cord sewn into seams for decorative finish

  - Tufting – a technique of stitching through upholstery to form indentations

  - Skirting – fabric at the bottom of upholstered furniture used for decorative or concealment purposes

- Measurement and Finishing Terms

  - Pitch – the angle of the seat back

  - Crown – the curve or height at the centre of the cushion

  - Rail – the horizontal part of the frame under the seat or back

  - Selvage – the edge of fabric that prevents fraying

- Abbreviations and Codes (where applicable)

  - Use of abbreviations in technical drawings or materials lists (e.g., FR = Fire Retardant, HR = High Resilience foam)

Practical Examples

| Term | Definition | Contextual Use |
| --- | --- | --- |
| Webbing | Woven elastic or jute straps attached across the seat base | "Ensure the jute webbing is stretched tightly for adequate seat support." |
| Coil Spring | A helical spring sewn or tied into the frame for support | "The seat has eight-way hand-tied coil springs for added resilience." |
| Foam Density | The weight per cubic metre of foam, indicating firmness and support | "Use a 30/200 foam for the backrest to maintain structure and comfort." |
| Tufting | Decorative stitching technique creating dimples or depressions | "The Chesterfield design features deep button tufting throughout the backrest." |
| Skirting | Fabric that covers the legs or bottom edge of furniture | "The sofa has box-pleated skirting to match the drapery in the lounge." |

South African Case Study: Miscommunication on the Workshop Floor

Case: At a furniture manufacturing facility in Durban, a junior upholsterer misunderstood a production supervisor’s instruction to install “decking fabric” and used a top fabric instead. This resulted in material waste and delayed delivery.

Discussion Points:

- What terminology confusion occurred?

- What could have prevented the misunderstanding?

- How does correct use of terminology affect time, cost, and quality?

International Reference Case: Quality Control Failure Due to Mislabelled Foam

Case: A European manufacturer received an order for high-resilience (HR) foam cushions but mistakenly installed regular density foam. The client returned the full batch due to comfort issues.

Discussion Points:

- What role did incorrect labelling or interpretation play?

- How important is terminology when ordering or storing materials?

- How can standardisation of terms support global supply chains?

Facilitator-Led Activities

- Flash Card Activity

  - Create cards with upholstery terms on one side and definitions on the reverse. Learners quiz each other in pairs or small groups.

- Tool and Material Labelling Exercise

  - Display a range of tools and components (physical or images). Learners must label each item using correct terms and abbreviations.

- Role-Play: Supervisor Instructions

  - In pairs, one learner gives a technical instruction using correct terminology (e.g., “Install HR foam with Dacron wrap”), and the other must identify or act out the correct response. Debrief on any errors and clarify terminology.

Critical Thinking Questions

- Why is the correct use of terminology important for health and safety in the upholstery environment?

- How could incorrect or inconsistent terminology impact quality assurance or product returns?

- In what ways can terminology improve teamwork and reduce waste in a production line?

- Should South African furniture manufacturers adopt international terminology standards, or develop their own? Why?

- How might technology (e.g., digital work orders, mobile apps) help reduce terminology-related errors on the production floor?

### KT0202 – Standard Dimensions of Upholstered Furniture

Purpose of this Topic Element

This topic introduces learners to the standard dimensions used in the design and construction of upholstered furniture. Understanding and applying these dimensions ensures that furniture is functional, comfortable, safe, and aligned with industry benchmarks. Knowledge of standardised measurements supports effective communication across design, manufacturing, and quality control functions and ensures that finished products meet user expectations and ergonomic requirements.

Key Content to Cover

- Importance of Standard Dimensions

  - Promote comfort, usability, and safety

  - Support consistency across product ranges

  - Ensure compatibility with other furniture and spatial layouts

- Key Measurement Areas in Upholstered Furniture

  - Seat height: Distance from the floor to the top of the seat (typically 450 mm for domestic chairs)

  - Seat depth: Distance from the front edge to the back cushion (typically 500–550 mm)

  - Seat width: Varies based on chair type (500–600 mm per seat unit)

  - Backrest height: From the seat base to the top of the back (ranging 400–700 mm)

  - Armrest height: Relative to the seat and user’s elbow level (typically 180–250 mm above seat)

  - Overall height, width, and depth: Total footprint of furniture

- Use of Anthropometric Data

  - Application of human body measurements in furniture design

  - Consideration for target users: adults, children, elderly, or differently-abled persons

- Adjusting for Function and Context

  - Higher seat height for dining chairs vs. lower for lounge chairs

  - Softer vs. firmer foam depending on depth and user interaction

  - Commercial/public seating: standardisation for accessibility, cleaning, and compliance

- Ergonomic Considerations

  - Relationship between dimensions and postural comfort

  - Avoiding strain by supporting correct sitting posture

  - Design for long-term seating (e.g., office chairs) vs. short-term use (e.g., waiting room)

Practical Examples

| Furniture Type | Common Standard Dimensions |
| --- | --- |
| Lounge Chair | Seat height: 400–450 mm; Seat depth: 550 mm; Back height: 600 mm |
| Dining Chair | Seat height: 450–480 mm; No arms preferred for close table access |
| Office Task Chair | Adjustable seat height: 420–520 mm; Lumbar support built-in |
| Sofa (2-seater) | Total width: 1400–1600 mm; Seat height: 450 mm |
| Headboard (Queen Bed) | Width: 1500–1600 mm; Height above bed: 800–1000 mm |

South African Case Study: Seating Complaint at a Community Health Clinic

Case: A furniture manufacturer in the Eastern Cape was contracted to supply seating for a rural clinic. The chairs were built using decorative measurements intended for home use. Within two weeks, complaints were received that elderly patients found the seating too low and difficult to stand up from. Several replacements had to be made.

Discussion Points:

- Which standard dimensions were likely incorrect?

- How does user profile affect design specifications?

- What role does ergonomics play in healthcare and public seating environments?

Facilitator-Led Activities

- Measurement Practice Exercise

  - Provide a selection of upholstered chairs and sofas. Learners must:

    - Measure seat height, depth, width, and backrest height

    - Record results and compare against industry standard tables

    - Identify any ergonomic risks or deviations

- Anthropometric Comparison Activity

  - Use printed anthropometric charts to match body measurements with seat dimensions

  - Discuss how these would differ for children’s furniture or bariatric seating

- Design Brief Task

  - Learners design an upholstered chair for a public library space. They must select appropriate dimensions based on function, user profile, and comfort needs. Include sketch and notes.

Critical Thinking Questions

- Why is it important to standardise furniture dimensions within a manufacturing facility?

- How does the function of furniture influence the measurements that should be used?

- What are the risks of ignoring anthropometric data when designing seating products?

- Should standard dimensions be adjusted for cultural or regional preferences in South Africa?

- In what ways do ergonomic standards improve the overall usability and safety of upholstered furniture?

### KT0203 – Furniture Frame Construction Including Parts Identification, Joints and Assembly Techniques

Purpose of this Topic Element

This topic provides learners with foundational knowledge of how furniture frames are constructed, focusing on the identification of frame parts, the use of appropriate joints, and the application of standard assembly techniques. A solid understanding of frame construction is essential in upholstery, as the frame supports all additional components such as springs, padding, and covers. Poor construction compromises comfort, durability, and overall quality.

Key Content to Cover

- Parts of Furniture Frames

  - Rails: Top, bottom, side, front, and back rails

  - Legs: Integral or separately attached

  - Stiles and posts: Vertical structural supports (especially in chairs)

  - Arms and arm supports: Often curved or shaped, connected to frame sides

  - Back support: Framework holding the backrest and upholstery

  - Webbing support frame: Part of the seat base structure

- Common Types of Joints

  - Mortise and tenon: Strong traditional joint used in frame corners

  - Dowel joints: Cylindrical pegs glued into matching holes

  - Butt joints: Simple end-to-end joint, often reinforced

  - Lap joints: Overlapping components for broader contact surface

  - Corner blocks (cleats): Added blocks for strength at frame corners

  - Stapled or nailed joints: Used in mass production, often reinforced with glue

- Assembly Techniques

  - Dry fitting before gluing

  - Use of pneumatic staplers or nailers

  - Clamp time and glue curing procedures

  - Use of jigs for consistency in large-scale manufacturing

- Durability and Quality Considerations

  - Load-bearing areas must be reinforced

  - Material compatibility (e.g., softwood vs hardwood in joint areas)

  - Moisture content in timber and how it affects stability

  - Safety compliance and long-term integrity

Practical Examples

| Furniture Type | Frame Part | Joinery Used |
| --- | --- | --- |
| Lounge chair | Side rail to front rail | Dowel joints and corner blocks |
| Dining room chair | Back legs to backrest | Mortise and tenon |
| Ottoman | Base support for webbing | Butt joint with glued cleat |
| Recliner | Back to seat connection | Bolt-in bracket and reinforced rail |
| Upholstered headboard | Legs and panel frame | Lap joints with staples and glue |

South African Case Study: Frame Failure in Institutional Seating

Case: A manufacturer in Gauteng supplied a batch of lounge chairs to a university residence. Within three months, several units had failed structurally, particularly at the front rail. Investigation revealed that softwood butt joints had been used without reinforcement in load-bearing areas.

Discussion Points:

- What joints should have been used for this application?

- How does the intended use environment affect frame design?

- What preventative quality control processes could have avoided this failure?

Facilitator-Led Activities

- Parts Labelling Exercise

  - Provide images or samples of chair or sofa frames. Learners must label all visible parts using correct industry terms.

- Joint Identification Workshop

  - Present physical samples or diagrams of different joints. Learners:

    - Identify joint type

    - Discuss advantages, disadvantages, and appropriate applications

- Assembly Demonstration

  - Demonstrate a basic frame assembly using pre-cut wooden components. Emphasise joint types, glue application, clamping, and squaring. Allow learners to practise assembly in groups.

Critical Thinking Questions

- Why is it important to match the joint type to the function and stress load of a furniture component?

- How does poor joint selection impact not only product quality but also workplace efficiency and client trust?

- What are the challenges of balancing cost-efficiency and structural integrity in large-scale furniture manufacturing?

- Should traditional joinery techniques be preserved in modern mass production? Why or why not?

- What role does quality assurance play in the inspection of frame assembly before upholstery begins?

### KT0204 – Furniture Frame Construction Materials

(Including hardwood, softwood, composite wood products, plastic, fibreglass, and metal frames)

Purpose of this Topic Element

This topic introduces learners to the various materials used in the construction of furniture frames. The choice of material significantly affects the structural strength, weight, appearance, cost, and durability of the final upholstered furniture product. Learners will explore the properties, benefits, and limitations of traditional and modern materials used in frame-making, equipping them with the knowledge needed to make informed material selections in different design and production contexts.

Key Content to Cover

- Overview of Frame Materials

  - Materials used must support structural integrity, ease of manufacturing, upholstery compatibility, and longevity.

- Hardwoods

  - Examples: Kiaat, Oak, Beech, Mahogany

  - Characteristics: Dense, durable, stable

  - Common uses: High-end or load-bearing furniture (e.g., chair legs, frames for heirloom sofas)

- Softwoods

  - Examples: Pine, Spruce

  - Characteristics: Lightweight, easier to work with, more cost-effective

  - Common uses: Inner framing, cost-sensitive production, bulk manufacturing

- Composite Wood Products

  - Types: Plywood, MDF (Medium-Density Fibreboard), Chipboard

  - Characteristics: Engineered from wood fibres or particles bonded with resin

  - Common uses: Back panels, arm inserts, structural components in modern manufacturing

- Plastics

  - Thermoplastics and injection moulded parts

  - Advantages: Lightweight, moisture-resistant, flexible designs

  - Limitations: Susceptible to UV damage, lower structural strength

- Fibreglass

  - Reinforced polymer commonly used in curved and moulded frames

  - Advantages: Durable, corrosion-resistant, suitable for complex shapes

  - Limitations: More expensive, limited repairability

- Metals

  - Examples: Steel, aluminium

  - Common in modern or commercial furniture

  - Advantages: High structural strength, slim profiles, longevity

  - Limitations: Heavy, may require welding or specialised assembly methods

- Factors Influencing Material Selection

  - Cost, availability, aesthetics, ease of machining

  - Upholstery method compatibility (stapling, gluing, screwing)

  - Sustainability and recyclability

Practical Examples

| Material | Typical Furniture Use | Key Properties |
| --- | --- | --- |
| Solid Pine | Frame bases for low-cost sofas | Lightweight, affordable, needs reinforcement |
| Beechwood | Dining chairs, premium lounge frames | Strong, fine-grained, ideal for joinery |
| Plywood | Arm panels, seat decks | Stable, multi-directional strength |
| MDF | Back panels, curved profiles for upholstery | Smooth surface, uniform density |
| Fibreglass | Moulded shells in modern recliners | Light, non-corrosive, sleek design |
| Mild Steel | Institutional furniture (e.g., school seating frames) | Durable, weldable, powder-coatable |

South African Case Study: Pine vs Hardwood in Budget Sofa Manufacturing

Case: A small furniture factory in George received a large retail order for an affordable range of family sofas. To reduce costs, they used pine for all frame components instead of the usual combination of pine and hardwood. Within six months, customer returns increased due to warping and loose joints.

Discussion Points:

- What characteristics of pine contributed to the structural failure?

- What are the cost vs. quality implications in material selection?

- How could better material selection have reduced warranty claims?

Facilitator-Led Activities

- Material Sample Comparison

  - Provide learners with physical samples or labelled images of hardwood, softwood, MDF, chipboard, and metal. Learners identify each material and list suitable frame applications.

- Strength and Weight Discussion

  - Pass around foam mock-ups or labelled materials. Learners rate each on weight, expected durability, and suitability for different furniture items.

- Design Brief Exercise

  - In pairs, learners choose a furniture product (e.g., hotel headboard, sleeper couch) and recommend frame materials. They must justify their selection based on cost, function, and longevity.

Critical Thinking Questions

- How does the choice of frame material influence the lifespan and performance of upholstered furniture?

- What are the potential risks of using low-cost materials in furniture frames intended for heavy use?

- In which situations might composite materials be more suitable than solid wood for frame construction?

- How can furniture manufacturers in South Africa balance sustainability with commercial viability in selecting frame materials?

- What quality control measures should be taken to ensure frame materials meet structural and safety standards?

### KT0205 – Traditional and Modern Foundations Including Coil Springs, Animal and Vegetable Fibre Paddings, and Foam

Purpose of this Topic Element

This topic provides learners with a comprehensive understanding of both traditional and modern foundational systems used in upholstered furniture. Foundations form the supportive base beneath upholstery materials and significantly influence the comfort, durability, and structure of the final product. Learners will explore how the evolution of materials and techniques has impacted the construction of upholstered furniture and how each foundation type suits different design, aesthetic, and functional requirements.

Key Content to Cover

- Definition of Foundations in Upholstery

  - The underlying systems that support padding and fabric in furniture

  - Critical to comfort, structure, and longevity

- Traditional Foundations

  - Coil Springs

    - Typically hand-tied with twine in 8-way or diagonal arrangements

    - Provide resilience and distribute weight evenly

    - Still used in high-end or restored antique furniture

  - Animal Hair Padding

    - Usually horsehair, used for its resilience and breathability

    - Common in antique and heritage restoration

  - Vegetable Fibre Padding

    - Coconut fibre (coir), kapok, and cotton wadding

    - Biodegradable and natural alternatives

    - Used in traditional and eco-conscious designs

- Modern Foundations

  - Foam (Polyurethane, High Resilience (HR), Memory Foam)

    - Vary in density and firmness

    - Lightweight, cost-effective, widely used

    - Available in sheet form or moulded shapes

  - Webbing

    - Jute, rubber, or synthetic strapping used to support seat cushions

    - Often used in combination with foam

  - No-sag or Zig-zag Springs

    - Sinuous wire springs fastened to the frame

    - Faster installation, common in mass-produced furniture

- Comparative Analysis

  - Comfort level, cost, longevity, labour intensity, environmental impact

  - Choosing the right foundation based on design, budget, and use environment

Practical Examples

| Foundation Type | Use Context | Key Characteristics |
| --- | --- | --- |
| Coil Springs (8-way tied) | High-end armchairs, antique reproductions | Labour intensive, durable, premium comfort |
| Coconut Fibre Padding | Eco-conscious lounge chairs | Natural, biodegradable, firm support |
| Foam (30/200 HR) | Budget sofas, office seating | Economical, lightweight, good resilience |
| No-sag Springs | Modern sofas, recliners | Fast installation, good support, less durability |
| Memory Foam | Premium mattresses, custom headrests | Moulds to body, heat-sensitive comfort |

South African Case Study: Combining Old and New Foundations in Custom Upholstery

Case: A Cape Town furniture studio was commissioned to create custom sofas for a boutique hotel. The brief requested a heritage appearance with modern comfort. The upholsterer combined traditional coil springs with high-resilience foam overlays to meet the aesthetic and ergonomic expectations.

Discussion Points:

- Why was it important to combine traditional and modern foundations in this context?

- What impact did this have on cost, production time, and durability?

- How does material selection influence customer satisfaction in high-end settings?

Facilitator-Led Activities

- Material Handling Station

  - Provide samples of traditional and modern foundation materials: coil springs, coconut fibre, horsehair, foam slabs, webbing

  - Learners inspect, handle, and describe the texture, flexibility, weight, and expected comfort of each

- Foundation Mapping Exercise

  - Present learners with images of cutaway upholstered seats

  - Learners identify which foundation materials were used and justify their conclusions

- Debate: Tradition vs. Innovation

  - Divide learners into two groups:

    - One defends the use of traditional foundation techniques

    - The other argues for modern methods

  - Discuss which is most suitable for the South African market today

Critical Thinking Questions

- In what types of furniture or client contexts would traditional foundation techniques still be preferable today?

- What are the advantages of using foam over natural fibre fillings in mass production?

- How do different foundation types affect customer satisfaction and product longevity?

- Should sustainability influence the selection of upholstery foundations? Why or why not?

- What role do cost and labour availability play in selecting between traditional and modern foundation methods in a South African manufacturing context?

### KT0206 – Furniture Cover Construction Including Parts Identification, Joints and Assembly Techniques

Purpose of this Topic Element

This topic introduces learners to the process of constructing upholstered furniture covers, including the identification of parts, seam types, and joining methods used to ensure proper fit, durability, and aesthetic quality. Understanding how covers are designed, constructed, and assembled is critical in producing high-quality upholstered furniture that meets client specifications and withstands use in domestic and commercial settings.

Key Content to Cover

- Overview of Furniture Cover Construction

  - Covers are the visible, tactile surface of upholstered furniture

  - Serve both aesthetic and protective functions

  - Must be accurately fitted to the furniture’s frame and padding

- Parts of a Furniture Cover

  - Seat cushion cover: Top panel, boxing, zip or Velcro closure

  - Backrest cover: Back panel, front panel, piping or plain seams

  - Armrest cover: Curved or angled panels, often with visible seams

  - Base or apron: Conceals lower frame; may include pleats or skirting

  - Side panels and wings (e.g., wingback chairs)

- Joints and Seams in Cover Construction

  - Plain seam: Basic joining of two fabric pieces, common in internal joins

  - Double-stitched seam: Reinforced seam for durability in stress points

  - French seam: Decorative and strong; used on visible external joins

  - Piping (or welt seam): Decorative cord enclosed in fabric for edge definition

  - Zips, Velcro, ties: Used in removable or washable covers

- Assembly Techniques

  - Measuring and cutting using patterns or templates

  - Sewing sequence for panels and assembly order

  - Fitting to foam or padding before final tacking or stapling to the frame

  - Aligning patterns and maintaining fabric grain direction

  - Techniques for tight-fit vs loose-fit slipcovers

- Finishing Considerations

  - Trim alignment, symmetry, and concealed seams

  - Upholstery-specific sewing machines and their settings

  - Stretch behaviour of fabrics and impact on fitting

Practical Examples

| Cover Part | Typical Construction Details | Common Use Context |
| --- | --- | --- |
| Seat Cushion Cover | Top and bottom panels, boxed sides, zip on rear | Sofas, dining chair seats |
| Skirt or Apron | Pleated or straight, sewn to base edge | Traditional sofas and lounge chairs |
| Piping Detail | Applied to seams for definition and structure | High-end or classic designs |
| Velcro Closure | Used in loose covers for ease of removal | Children's furniture, hospitality use |
| Stretch Panel Inserts | Sewn to aid fitting on complex curves or recliner mechanisms | Modern recliners and ergonomic chairs |

South African Case Study: Pattern Mismatch in Commercial Sofa Order

Case: A Durban-based upholstery business completed a large order of restaurant sofas using striped fabric. Upon delivery, it was discovered that the stripe patterns on seat and back panels were misaligned across units. The client rejected the entire batch.

Discussion Points:

- What went wrong in the cover assembly process?

- How can careful measurement, cutting, and alignment prevent this issue?

- What systems should be in place to ensure visual quality control?

Facilitator-Led Activities

- Cover Part Identification

  - Display images or fabric templates of common cover parts. Learners must:

    - Name the part

    - Describe how it is attached

    - Indicate its function

- Fabric Alignment Exercise

  - Provide striped or patterned fabrics and shaped foam cut-outs. Learners must mark and plan the correct layout to align patterns during cover construction.

- Stitching Techniques Demonstration

  - Demonstrate plain, double-stitched, and piping seams on an industrial sewing machine

  - Allow learners to practise stitching on upholstery-grade fabric

Critical Thinking Questions

- How does the quality of cover construction affect the overall perception and durability of a furniture piece?

- What are the consequences of poor measuring, seam placement, or fitting in upholstered furniture?

- How can upholstery businesses ensure consistency in cover assembly across large orders?

- When might it be better to use removable covers instead of fixed ones? What factors would you consider?

- How should fabric stretch, texture, and pattern influence the choice of seam types and assembly methods?

### KT0207 – Furniture Upholstery Materials Including Natural and Man-Made Fabric and Finishings

Purpose of this Topic Element

This topic equips learners with knowledge of the materials used to upholster furniture, focusing on the characteristics, applications, and performance of both natural and man-made fabrics and finishings. Upholstery materials influence not only the visual and tactile appeal of furniture but also its functionality, durability, maintenance, and compliance with safety and quality standards. A well-informed upholsterer must be able to select appropriate materials to suit a product’s use, environment, and client expectations.

Key Content to Cover

- Classification of Upholstery Fabrics

  - Natural fibres:

    - Examples: Cotton, linen, wool, leather

    - Characteristics: Breathable, biodegradable, generally softer

    - Limitations: May fade, shrink, or wear faster; often more expensive

  - Man-made fibres:

    - Examples: Polyester, acrylic, nylon, vinyl, PU leather

    - Characteristics: Durable, colourfast, wrinkle-resistant, more affordable

    - Limitations: May trap heat, can feel less natural to the touch

- Blended Fabrics

  - Combining natural and synthetic fibres (e.g., polycotton)

  - Enhanced performance and cost efficiency

- Backing Materials and Reinforcements

  - Woven and non-woven backings for added strength

  - Fire-retardant coatings or chemical treatments

  - Water-resistant or stain-resistant layers (Scotchgard, PVC backing)

- Surface Finishes and Textures

  - Embossing, tufting, printing, coating, quilting

  - Decorative vs. functional finishes

- Key Material Performance Indicators

  - Martindale rating: Abrasion resistance

  - Pilling resistance: Surface wear

  - Colour fastness: Resistance to fading

  - Tear strength and seam slippage resistance

- Environmental and Client Considerations

  - Flame-retardant regulations in public spaces

  - Hypoallergenic materials for hospitals or children’s furniture

  - Eco-friendly, recycled, or sustainably produced fabrics

Practical Examples

| Material Type | Use Case | Key Characteristics |
| --- | --- | --- |
| Leather (natural) | Executive chairs, heritage sofas | Luxurious, long-lasting, requires care |
| Polyester (synthetic) | Mass-market sofas, budget lounge suites | Durable, colourfast, widely available |
| Polycotton (blend) | Headboards, decorative cushions | Soft feel with better wear resistance |
| Vinyl (synthetic) | Restaurant seating, hospital furniture | Easy to clean, waterproof, resistant to stains |
| Wool (natural) | Premium lounge chairs | Warm, breathable, natural elasticity |

South African Case Study: Upholstery Failure in Coastal Environment

Case: A furniture retailer in Durban sold a range of upholstered patio seating made with cotton-linen blend covers. Within two months, customer complaints emerged about mould and fading. Investigation revealed that the fabric was not suited to high-humidity, UV-exposed environments.

Discussion Points:

- Why was the selected fabric inappropriate for this environment?

- What properties should have been prioritised in material selection?

- How does geographic location affect upholstery material decisions?

Facilitator-Led Activities

- Fabric Handling and Comparison

  - Provide swatches of natural and synthetic fabrics.

  - Learners evaluate texture, weight, stretch, and surface finish.

  - Discuss appropriate applications for each material.

- Material Specification Table

  - Learners complete a worksheet comparing Martindale ratings, composition, and ideal use for different materials provided.

- Client Brief Role-Play

  - Pairs of learners act as upholsterer and client.

  - Based on a room description (e.g., medical waiting area, high-end lounge), they must discuss suitable upholstery options.

Critical Thinking Questions

- How does the choice of upholstery material affect a furniture item’s maintenance requirements and lifespan?

- Why is it important to understand the environment in which upholstered furniture will be used when selecting fabric?

- How can upholstery professionals balance aesthetics, function, and cost when recommending materials to clients?

- What role do safety standards (e.g., fire retardancy) play in the selection of upholstery fabrics for public spaces?

- In what ways can South African furniture manufacturers incorporate sustainable upholstery materials without compromising performance or cost?

### KT0208 – Impact of Design and Construction Faults

Purpose of this Topic Element

This topic prepares learners to identify and understand the implications of design and construction faults in the upholstery manufacturing process. It emphasises how early-stage design decisions, frame or cover construction issues, and material selection errors can affect product performance, visual quality, client satisfaction, and production efficiency. Learners will be equipped to recognise common faults and reason through their causes and consequences—key skills in quality assurance and preventative maintenance in the upholstery environment.

Key Content to Cover

- Types of Design Faults

  - Poor ergonomic planning (e.g., incorrect seat depth or height)

  - Inadequate material selection (e.g., fabric not suitable for use context)

  - Overly complex or inconsistent design features

  - Aesthetic mismatches (e.g., pattern clashing, uneven proportions)

- Types of Construction Faults

  - Poor joint alignment or weak fastening

  - Frame not squared, leading to distortion

  - Cover stitching errors: loose seams, mismatched patterns, exposed threads

  - Uneven padding distribution causing surface lumps

  - Incorrect spring or foam installation leading to sagging or noise

- Consequences of Design and Construction Faults

  - Cover preparation issues: difficulty in fitting, distortion, early wear

  - Final product quality: poor comfort, misalignment, poor aesthetic appeal

  - Production output: increased rework time, waste of materials, production delays

  - Client satisfaction: increased returns, reputational damage, financial loss

- Root Cause Analysis

  - Identifying the stage (design, frame build, cover stitching, final assembly) where fault occurred

  - Use of checklists, inspection protocols, and communication feedback loops

- Preventative Measures

  - Adherence to technical drawings and standard procedures

  - Proper material checks before assembly

  - Skilled supervision and quality control at each stage

Practical Examples

| Fault Type | Example | Resulting Problem |
| --- | --- | --- |
| Incorrect foam density | Low-density foam in seat intended for daily use | Premature sagging, loss of comfort |
| Uneven leg lengths | One leg slightly shorter than others | Furniture wobbles, unsafe for users |
| Weak corner joints | Butt joint without reinforcement on chair frame | Loosening and instability over time |
| Pattern misalignment | Stripes not matching at seams in sofa upholstery | Visually unprofessional, client dissatisfaction |
| Misplaced staples | Staples too close to edge of webbing | Webbing tears under tension |

South African Case Study: Warranty Claim from Retail Chain

Case: A Johannesburg-based manufacturer supplied a chain of furniture retailers with an affordable lounge suite range. Within four months, complaints included sagging cushions, noise when seated, and frayed stitching. Investigation revealed that:

- Foam density was below required standard

- Sinuous springs were not properly fixed with clips

- Stitching was completed using general sewing machines instead of upholstery-grade equipment

Discussion Points:

- At what stages did faults occur in the manufacturing process?

- What could have prevented each issue?

- How do multiple small faults accumulate to affect overall customer experience?

Facilitator-Led Activities

- Fault Identification Gallery

  - Display photos of furniture items with faults (e.g., collapsed cushions, visible seams, skew arms)

  - Learners identify the fault and suggest a probable cause

- Root Cause Analysis Role-Play

  - In small groups, learners role-play a scenario in which a faulty product is returned

  - Teams investigate design drawings, material records, and assembly steps to identify the fault source

- Preventative Checklist Exercise

  - Learners develop a checklist for avoiding the most common faults in design and construction

  - Compare and discuss across groups

Critical Thinking Questions

- Why is it important to identify design and construction faults before a product reaches the client?

- How can communication between departments help reduce the occurrence of faults in furniture production?

- What are the long-term business implications of recurring product faults in a competitive market?

- How does the cost of correcting a fault after production compare to the cost of preventing it during production?

- In what ways can quality control systems be improved to support both production efficiency and product excellence?

## 🌟 Integrated Formative Assessment: KM-01-KT02

Topic: Upholstered Furniture Construction
Assessment Type: Case Study-Based Integrated Formative Assessment
Assessment Weight: 15%
Internal Assessment Criteria Covered:

- IAC0201

- IAC0202

- IAC0203

- IAC0204

- IAC0205

Case Study Scenario: Production Faults at Comfort Line Upholstery

Comfort Line Upholstery is a small furniture manufacturing company based in Port Elizabeth. The team recently received a contract to produce 150 lounge chairs for a university student residence. The chairs needed to be durable, comfortable, and visually appealing.

However, shortly after the delivery, the client reported several issues:

- Some chairs were unsteady and wobbled when sat on.

- The seat cushions sagged quickly and were uncomfortable.

- The arms of several chairs were misaligned.

- Seams on the covers were pulling open after limited use.

Upon investigation, the following findings were made:

- The production team used softwood without reinforcement in the chair frames.

- The webbing was secured using only staples and no corner blocks.

- Incorrect foam density was used in the seat pads.

- Frame dimensions varied slightly due to inconsistent cutting.

- Cover components were stitched using non-upholstery thread.

❓ Assessment Questions

Q1. Based on the case study, explain how incorrect frame dimensions and poor ergonomic planning affected the functionality and comfort of the chairs. (IAC0201 – 5 marks)

Q2. Identify four parts of an upholstered chair mentioned or implied in the case study, provide their correct abbreviations, and explain their functions. (IAC0202 – 8 marks)

Q3. Identify two traditional and two modern materials used in furniture frame construction. For each, explain how their characteristics can affect the upholstery process. (IAC0203 – 8 marks)

Q4. List four components commonly found in upholstered furniture, as suggested in the case, and briefly describe their purpose in the product. (IAC0204 – 4 marks)

Q5. Evaluate how the faults in raw materials and poor construction practices impacted the cover preparation process, the final product, and overall production efficiency at Comfort Line Upholstery. (IAC0205 – 10 marks)

✅ Model Answers and Marking Memo

Q1. Influence of dimensions and ergonomics (5 marks)

- Incorrect frame dimensions led to discomfort (e.g., seat height too low or depth too short)

- Poor ergonomic planning resulted in poor posture support and reduced usability

- Students may have struggled to get in/out of chairs due to incorrect proportions

- Structural imbalance (e.g., misaligned arms) affected the user experience
(1 mark per valid point, max 5 marks)

Q2. Parts, abbreviations and functions (8 marks)

Any 4 of the following:

| Part | Abbreviation | Function |
| --- | --- | --- |
| Front Rail | FR | Supports the front of the seat frame |
| Back Post | BP | Holds the backrest and supports user’s back |
| Webbing | WB | Distributes weight and supports the seat cushion |
| Arm Frame | AF | Provides structure and alignment for the armrest |

(2 marks per correct set: 1 mark for abbreviation + 1 mark for function)

Q3. Frame materials and effects (8 marks)

Traditional Materials:

- Hardwood (e.g., beech): Strong and durable, ideal for load-bearing; may be difficult to staple

- Coil Springs: Provide comfort and resilience but require skilled installation

Modern Materials:

- Softwood (e.g., pine): Lightweight and affordable; may weaken under stress

- Plywood: Stable and multi-directional strength; suitable for staple or screw attachment

(1 mark for identifying each material + 1 mark for explaining its effect, max 8 marks)

Q4. Upholstered furniture components (4 marks)

- Foam Padding: Provides softness and support

- Webbing: Supports the seat structure

- Cover Fabric: Aesthetic finish, covers foam and structure

- Thread: Joins panels and must withstand tension

(1 mark per valid component and description)

Q5. Evaluation of consequences (10 marks)

- Frame instability caused poor fit for covers and reduced comfort

- Low-density foam led to sagging, affecting the visual and functional quality

- Misaligned components resulted in time-consuming rework

- Stitching faults impacted durability and customer satisfaction

- Production delays and material wastage reduced profitability
(2 marks per well-reasoned point, max 10 marks)

📊 Assessment Rubric: KM-01-KT02 Integrated Formative Assessment

| Criteria | 5 – Outstanding | 4 – Competent | 3 – Developing | 1–2 – Not Yet Competent |
| --- | --- | --- | --- | --- |
| IAC0201: Reasoning around ergonomics and dimensions | Clear, relevant explanation with correct examples | Good reasoning with some relevant points | Basic link between ergonomics and design | Little to no understanding shown |
| IAC0202: Parts identified, correct abbreviations and purpose | 4+ correct with full detail | 3 correct with some clarity | 2 correct, limited explanation | 1 or fewer, incorrect or vague |
| IAC0203: Traditional and modern material properties | All 4 correct with accurate effects on process | 3 correct with basic reasoning | 2 correct with limited explanation | 1 or none correct |
| IAC0204: Components of upholstered furniture | 4 clearly and accurately identified | 3 relevant items identified | 2 relevant items identified | 1 or fewer, vague |
| IAC0205: Evaluation of faults and impact on process | Comprehensive and well-structured evaluation | Mostly complete with relevant insights | Some evaluation attempted | No clear reasoning or impact explained |

## KM-01-KT03: Ergonomics Related to Furniture Manufacturing

NQF Level 2 | Credits: 5 | Weight: 10%

This knowledge topic introduces the foundational concepts of ergonomics and their practical application in furniture manufacturing environments. It equips learners with the ability to recognise how ergonomic principles impact not only the comfort and usability of upholstered furniture, but also the safety, efficiency, and well-being of workers involved in production processes.

Ergonomics in furniture manufacturing refers to designing furniture and planning tasks in ways that suit the physical capabilities and limitations of users and workers. In the context of upholstery production, ergonomic considerations extend from designing seating that supports the human body correctly, to handling, moving, and storing materials in ways that prevent injury and reduce fatigue on the factory floor.

The learning begins with a clear definition and purpose of ergonomics in manufacturing settings. Learners will explore the guiding principles and best practices that ensure both product quality and human health are optimised. They will also be introduced to key ergonomic concepts specific to upholstered furniture, such as seat height, backrest angle, armrest placement, and support surfaces.

This topic also includes practical applications of ergonomics in relation to the design and use of tools and equipment for lifting, moving, and stacking materials. Emphasis is placed on using the correct equipment for different tasks, such as selecting ladders of appropriate height and using mechanical aids where manual lifting is not suitable. The importance of standard measurements and sizes is considered, not only from the perspective of customer comfort but also in relation to streamlining production processes and minimising risk to workers.

By understanding and applying ergonomic principles in both design and workshop practices, learners will be able to contribute to safer, more efficient, and more user-centred furniture manufacturing.

This knowledge topic is structured across the following seven elements:

- KT0301: Definition

- KT0302: Principles

- KT0303: Purpose

- KT0304: Concepts of ergonomics pertaining to upholstered furniture

- KT0305: Standard measurements for furniture

- KT0306: Lifting and stacking

- KT0307: Moving materials and equipment

Assessment of this topic is based on the following Internal Assessment Criteria:

- IAC0301: Ergonomics is defined in terms of its applications and impact in furniture manufacturing processes

- IAC0302: The principles and best practices of ergonomics in the industry are outlined

- IAC0303: The purpose of ergonomics is described in terms of the effect on the finished product and the production processes

- IAC0304: The need for standard sizes in furniture manufacture is motivated with reference to ergonomics

- IAC0305: Lifting and moving equipment are listed and their uses described

- IAC0306: The lifting and moving equipment’s impact on ergonomics during production is outlined

- IAC0307: Material storage (such as stacking) is described with reference to the different sizes, the use of spacers, and the avoidance of damage

- IAC0308: The correct equipment is used based on the situation; such as ladders of sufficient lengths for the heights involved

- IAC0309: The principles of ergonomics are applied to lifting to identify correct lifting procedures and minimise the risk of injury to self

Learners who successfully complete this topic will understand how ergonomic design improves product functionality and enhances health and safety in the workplace. They will be able to apply best practices to lifting, handling, and workstation design, contributing to improved productivity and worker well-being in upholstery manufacturing environments.

Bottom of Form

### KT0301 – Definition of Ergonomics

Purpose of this Topic Element

The purpose of this topic element is to introduce learners to the term ergonomics and its meaning within the context of furniture manufacturing. At NQF Level 2, the focus is on understanding what ergonomics is, why it is important, and how it affects the way we design furniture and plan tasks in upholstery workshops. Learners will gain the ability to define ergonomics in their own words and explain its relevance to both product users and production workers.

Key Content to Cover

- Definition of Ergonomics

  - Ergonomics is the science of designing workspaces, products, and tasks to fit the capabilities and limitations of the human body.

  - In furniture manufacturing, ergonomics aims to ensure that:

    - Furniture is comfortable and supportive for the user

    - Work tasks are safe and efficient for production workers

- Basic Explanation for NQF Level 2

  - Ergonomics means making things fit people, not the other way around.

  - This includes how a chair supports your back or how a worker lifts a heavy fabric roll without injury.

- Why Ergonomics Matters in Furniture Manufacturing

  - Prevents injury and fatigue in workers

  - Improves comfort and safety of the final furniture product

  - Increases productivity by reducing strain and errors

- Key Words and Concepts

  - “Fit-for-purpose”

  - “Comfortable”

  - “Safe”

  - “Efficient”

  - “User-friendly”

  - “Body posture”

Practical Examples

| Situation | Ergonomic Design Feature |
| --- | --- |
| Dining chair used for long meals | Seat height matches average leg length for comfort |
| Worker assembling frames all day | Adjustable workbench to reduce bending or stretching |
| Upholsterer using a hand staple gun repeatedly | Ergonomic tool grip to reduce hand strain |
| Lounge furniture for elderly users | Slightly higher seat height for ease of getting up |

South African Case Study: Preventable Injury in a Workshop

Case: At a small upholstery workshop in East London, a worker was asked to carry heavy plywood sheets from one end of the factory to the other. There were no trolleys available, and the material was stacked above shoulder height. The worker injured his back and required time off work. The workshop had not considered ergonomic factors in material handling.

Discussion Points:

- What went wrong?

- How could ergonomics have helped prevent the injury?

- What low-cost changes could improve safety in this scenario?

Facilitator-Led Activities

- Definition Brainstorm

  - Ask learners: “What does the word ergonomics sound like to you?”

  - Allow learners to guess or describe before introducing the formal definition.

- Furniture Check

  - Learners sit on different chairs or sofas (or view images).

  - They describe how the seat height, backrest, or armrest feels and if it supports them well.

- Ergonomics Word Match

  - Create flashcards with keywords (e.g., posture, comfort, fit, height).

  - Learners match words to descriptions or pictures of ergonomic vs non-ergonomic designs.

Critical Thinking Questions

- In your own words, what is ergonomics?

- Why is it important to think about people when designing furniture or tasks in a workshop?

- How could poor ergonomics cause a problem in your job or at home?

- What does a “comfortable chair” mean to you, and how is that related to ergonomics?

- Can you think of one thing in your home or workspace that could be more ergonomic? Why?

### KT0302 – Principles of Ergonomics

Purpose of this Topic Element

The purpose of this topic is to introduce learners to the basic principles of ergonomics and how these principles are applied in both the design of upholstered furniture and in workplace safety and productivity. At NQF Level 2, learners are expected to understand simple, clear guidelines that help ensure furniture is comfortable and functional, and that workplace activities are safe and efficient.

By learning these principles, learners will be able to contribute to improved user experience for clients and healthier working conditions for colleagues on the production floor.

Key Content to Cover

- What Are the Principles of Ergonomics?

| Ergonomic Principle | Explanation (Level-appropriate) |
| --- | --- |
| Fit the task to the person | Tools, tasks, and furniture should match the size and strength of people |
| Encourage neutral posture | Sitting or standing positions should not strain joints or muscles |
| Reduce force and repetition | Avoid tasks that need too much strength or repeating the same movement |
| Avoid awkward positions | Work surfaces and furniture should prevent twisting, bending, or reaching |
| Allow movement and rest | People should not sit or stand still for too long |

- Applying Ergonomic Principles in Furniture Manufacturing

  - Designing chairs that support natural sitting posture

  - Using lifting trolleys to reduce back strain

  - Placing tools within easy reach on the assembly line

  - Keeping materials at waist level to avoid bending

- Benefits of Following Ergonomic Principles

  - Increased comfort for furniture users

  - Fewer injuries and muscle strain in workers

  - Better productivity and fewer sick days

  - Improved product quality and client satisfaction

Practical Examples

| Context | Ergonomic Principle Applied |
| --- | --- |
| Upholstering a lounge chair | Backrest supports natural spine curve |
| Cutting fabric at a workbench | Bench height adjusted to elbow level to avoid hunching |
| Carrying foam sheets | Use of a trolley to reduce lifting force and fatigue |
| Workshop layout | Tools arranged to reduce walking and twisting movements |

South African Case Study: Repetitive Strain in a Cutting Section

Case: In a furniture factory in Gauteng, a fabric cutter complained of wrist and shoulder pain. He was using heavy manual scissors repeatedly throughout the day and the cutting table was too low. Management replaced the scissors with lightweight electric shears and raised the table to hip height.

Discussion Points:

- Which ergonomic principles were not followed in the original setup?

- How did the solution reduce strain and increase productivity?

- What other departments might benefit from similar changes?

Facilitator-Led Activities

- Ergonomic Spot the Risk

  - Show learners photos of a workshop with good and poor ergonomic setups.

  - Learners identify which principles are followed or violated in each image.

- Principle-to-Practice Match

  - Give learners cards with ergonomic principles and another set with real-life workshop examples.

  - Learners match the principle to the correct application.

- Small Group Brainstorm

  - Ask learners to list everyday tasks (e.g., lifting foam, tacking fabric).

  - For each task, discuss how ergonomics can improve safety and efficiency.

Critical Thinking Questions

- Why is it important for the task to be adapted to the person and not the other way around?

- What could happen if someone works in an awkward posture every day?

- How do ergonomic principles improve both worker health and the quality of the final product?

- Can ergonomic improvements be made without expensive equipment? Give an example.

- Think of a task you do often—how could you change it to make it more comfortable or safe?

### KT0303 – Purpose of Ergonomics in Furniture Manufacturing

Purpose of this Topic Element

This topic introduces learners to the purpose of ergonomics and its relevance to both the finished furniture product and the production process. At NQF Level 2, learners are expected to understand how ergonomics improves comfort and safety for furniture users, while also reducing injury, fatigue, and mistakes among workers in manufacturing environments.

By exploring the dual impact of ergonomics—on both the user and the maker—learners will develop the ability to explain why ergonomics is essential at all stages of the upholstery manufacturing process.

Key Content to Cover

- Purpose of Ergonomics in Furniture Design

  - To ensure furniture supports the natural shape and movement of the human body

  - To provide comfort and safety for users

  - To reduce physical stress during regular use

- Purpose of Ergonomics in the Manufacturing Process

  - To reduce worker fatigue and prevent injuries

  - To improve the speed and accuracy of work tasks

  - To minimise errors and material wastage

  - To ensure tools, equipment, and workstations suit worker needs

- Impact of Good Ergonomics

  - Increased productivity and fewer work disruptions

  - Fewer injury-related absences or compensation claims

  - Greater customer satisfaction due to quality and comfort

  - Higher staff morale and less turnover in factory teams

Practical Examples

| Area | Purpose of Ergonomics |
| --- | --- |
| Sofa Design | Comfortable posture, correct seat height, and support for back |
| Cutting Workstation | Prevent bending or over-reaching to reduce fatigue and mistakes |
| Stapling Tools | Comfortable grip and correct trigger tension to reduce hand strain |
| Packing Station | Work height suitable for standing or sitting to reduce back strain |

South African Case Study: Poor Comfort and High Return Rate

Case: A furniture company in Limpopo designed a new line of lounge chairs for elderly clients. Although the style was attractive, the chairs were too low and deep, making it difficult for users to sit or rise comfortably. The company received numerous complaints, resulting in product returns and loss of sales. The chairs had been designed without considering the ergonomic needs of the target market.

Discussion Points:

- How did ignoring ergonomics affect customer satisfaction?

- What design changes could have solved the issue?

- What could be the financial impact of poor ergonomic planning?

Facilitator-Led Activities

- Furniture Evaluation Activity

  - Learners sit on a variety of chairs or assess pictures of seating.

  - They evaluate the comfort, support, and ease of use.

  - Discuss what makes each chair “more ergonomic.”

- Workshop Walkabout (or Virtual Tour)

  - Learners identify workstation features that support comfort and safety.

  - They note any risks or improvements needed.

- Dual-Impact Mapping

  - Create a two-column table:

    - Column A: Benefits of ergonomics for the user

    - Column B: Benefits for the worker

  - Learners fill in examples discussed during class.

Critical Thinking Questions

- Why should furniture be designed with the user’s body in mind?

- How can ergonomics make workers faster and safer at their jobs?

- What are the risks to a business that ignores ergonomics in its product design?

- If you were the manager of a factory, how would you explain the importance of ergonomics to your team?

- How does ergonomics contribute to the reputation and success of a furniture brand?

### KT0304 – Concepts of Ergonomics Pertaining to Upholstered Furniture

Purpose of this Topic Element

This topic introduces learners to specific ergonomic concepts as they relate to the design and construction of upholstered furniture. At NQF Level 2, learners are expected to identify and apply basic ergonomic features that affect user comfort and physical support. They will explore how details such as seat height, backrest angle, armrest placement, and cushion firmness are directly influenced by ergonomic thinking.

Understanding these concepts ensures that learners not only recognise what makes a chair or sofa comfortable and supportive but also begin to apply this knowledge when assembling, upholstering, or assessing furniture in a real work environment.

Key Content to Cover

- Basic Ergonomic Concepts for Upholstered Furniture

  - Seat height: Should allow the user’s feet to rest flat on the floor

  - Seat depth: Should support the thighs without pressing behind the knees

  - Backrest angle: Should encourage a relaxed but supported sitting posture (100–110 degrees)

  - Lumbar support: Lower back must be supported to reduce fatigue

  - Armrest height: Should support elbows comfortably without lifting shoulders

  - Cushion firmness: Must balance comfort and support, preventing bottoming-out or excessive softness

- Human Factors in Furniture Use

  - Anthropometric measurements: Human body sizes and shapes vary

  - Users include children, elderly people, differently-abled individuals, or people of different body weights

  - Furniture must cater to a target group, not a generic average

- Application in Upholstered Products

  - High-back chairs for posture support

  - Firmer cushions in waiting rooms vs softer ones in lounges

  - Recliner chairs designed for relaxed postures with leg elevation

  - Sofas with integrated lumbar contours or memory foam

- Quality vs Comfort Considerations

  - Furniture may look visually appealing but be ergonomically poor

  - Functional comfort is a key quality indicator in premium upholstered goods

Practical Examples

| Feature | Ergonomic Consideration | Application |
| --- | --- | --- |
| Seat height | 450 mm standard, adjusted for children or elderly | Dining chairs, sofas |
| Armrest height | 180–250 mm above seat level | Lounge chairs, office chairs |
| Backrest support | Curved to match spine’s natural shape | Ergonomic task chairs, reception chairs |
| Cushion firmness | Medium-firm density for balanced comfort | Public seating, office lounges |
| Seat width | 500–600 mm per person minimum | Modular sofas, single seats |

South African Case Study: Inaccessible Clinic Seating

Case: A healthcare facility in Mpumalanga ordered benches for their waiting area. The furniture appeared sleek and modern but had very low seats and no armrests. Elderly patients and pregnant women struggled to use them. Complaints led to replacement orders with corrected ergonomic features.

Discussion Points:

- Which ergonomic concepts were ignored?

- What changes were needed?

- How might the situation have been avoided during the design phase?

Facilitator-Led Activities

- Furniture Fit Test

  - Learners sit on various chairs or observe images

  - They evaluate the ergonomics: is the height correct? Is the back supported?

  - Complete a checklist for each chair based on ergonomic principles

- Role-Play Scenario

  - Learners act as design team members asked to create chairs for a specific user group (e.g., nursery school, retirement home)

  - They must decide on seat height, backrest angle, arm support, and padding type

- Upholstery Foam Comparison

  - Provide foam samples of various densities

  - Learners compare support and comfort by touch and pressure

Critical Thinking Questions

- Why is it important to design upholstered furniture for a specific user group?

- How does a poorly designed seat affect someone’s body after long-term use?

- Can a piece of furniture be beautiful but uncomfortable? Why might this happen?

- What would you change in a chair if it was going to be used in a public hospital instead of a lounge at home?

- How do small changes in measurements (like seat height) make a big difference in comfort and safety?

### KT0305 – Standard Measurements for Furniture

Purpose of this Topic Element

This topic focuses on teaching learners the standard measurements used in the manufacture of upholstered furniture. Understanding these measurements is essential to ensure that furniture items are comfortable, functional, and safe for end users. At NQF Level 2, learners are expected to know common industry dimensions and understand how these are influenced by ergonomics and human body measurements (anthropometry).

Accurate and consistent measurement ensures repeatability in production, reduces errors, and supports the creation of products that meet client needs and expectations.

Key Content to Cover

- What Are Standard Measurements?

  - Industry-agreed or commonly used dimensions for comfort and functionality

  - Based on average human body sizes and movements

  - Used for seating, tables, beds, headboards, and more

- Common Upholstered Furniture Measurements

| Furniture Part | Standard Measurement | Purpose |
| --- | --- | --- |
| Seat height | 430–480 mm | Allows feet to rest flat on the floor |
| Seat depth | 450–550 mm | Supports thighs without pressing behind knees |
| Seat width | 500–600 mm per person | Provides enough room for comfort |
| Backrest height | 600–800 mm (or higher for full-back support) | Ensures posture and shoulder support |
| Armrest height | 180–250 mm above the seat | Keeps shoulders relaxed and elbows supported |
| Table-to-seat gap | 250–300 mm | Allows leg clearance under the table |

- Why Standard Sizes Matter

  - Consistency across production batches

  - Reduces returns and client complaints

  - Makes furniture usable and comfortable for most people

  - Supports easy replacement of cushions and slipcovers

- Factors That May Influence Adjustment of Measurements

  - Age group (e.g., children, elderly)

  - Target users (e.g., hospitality, healthcare)

  - Purpose of furniture (e.g., lounging, dining, working)

  - Regional variations or custom client briefs

Practical Examples

| Use Case | Measurement Consideration |
| --- | --- |
| Office waiting area | Medium-firm cushions, seat height 450 mm |
| Retirement home lounge chair | Higher seat (480 mm), wider armrests for stability |
| Preschool classroom chairs | Seat height adjusted to 300 mm for children |
| Hotel bedroom headboard | Height: 800–1000 mm above mattress for visual balance |

South African Case Study: Incorrect Sizing Leads to Redesign

Case: A small upholstery company in the Free State supplied armless lounge chairs to a hotel in Bloemfontein. The chairs were stylish but had a seat height of only 390 mm, which was too low for most guests. As a result, many complaints were received from older patrons who struggled to sit down and stand up. The hotel requested a redesign and replacement.

Discussion Points:

- What standard measurement was ignored or incorrectly applied?

- What impact did this have on the user?

- How can manufacturers avoid this kind of error?

Facilitator-Led Activities

- Measurement Practice

  - Use a tape measure to measure existing chairs or benches in the workshop or classroom.

  - Learners compare their measurements to the industry standards and record their findings.

- Client Brief Simulation

  - Provide a fictional client brief (e.g., chairs for a community centre).

  - Learners select appropriate measurements based on the users’ needs.

- Furniture Design Table

  - Learners create a basic chart that shows each furniture part and the recommended standard dimensions.

Critical Thinking Questions

- Why is it important to follow standard measurements in furniture production?

- How do standard sizes relate to user comfort and safety?

- What might happen if a manufacturer ignores or guesses furniture dimensions?

- Should furniture always be made to standard measurements? When might they need to be adjusted?

- How can understanding measurements reduce mistakes and material waste in production?

### KT0306 – Lifting and Stacking

Purpose of this Topic Element

This topic introduces learners to safe and ergonomic practices for lifting and stacking materials and components in a furniture manufacturing environment. Lifting and stacking are common tasks in upholstery workshops and factories. However, if done incorrectly, they can cause injuries, damage to materials, and production delays.

Learners will develop a basic understanding of how to lift correctly, how to stack materials safely, and why using the correct method matters for their own safety and the protection of products.

Key Content to Cover

- The Importance of Safe Lifting and Stacking

  - Prevents injury (especially to back, shoulders, and knees)

  - Protects raw materials from damage

  - Improves workflow efficiency

  - Reduces absenteeism and compensation claims

- Ergonomic Lifting Principles

  - Keep the back straight and bend the knees

  - Keep the load close to the body

  - Avoid twisting while lifting or carrying

  - Lift smoothly; do not jerk the load

  - Get help for heavy or awkward items

- Safe Stacking Principles

  - Heavier items at the bottom

  - Do not stack higher than chest height

  - Use spacers between layers to avoid pressure damage

  - Ensure the stack is stable and level

  - Allow access aisles and do not block fire exits

- Common Materials That Require Careful Lifting and Stacking

  - Foam sheets (light but bulky)

  - Timber frame components (varying sizes and weights)

  - Fabric rolls (long and difficult to handle alone)

  - Upholstered cushions (soft but may deform or attract dirt)

- Consequences of Unsafe Practices

  - Personal injury

  - Damaged goods

  - Increased waste

  - Loss of production time

Practical Examples

| Item | Recommended Lifting/Stacking Practice |
| --- | --- |
| Foam cushions | Stack gently, not too high; use clean, dry surface |
| Wooden frame parts | Lift in pairs if large; stack on pallet with spacers |
| Fabric rolls | Use a trolley or carry with two hands; store upright if possible |
| Upholstered panels | Stack face-to-face using protective cloth between items |

South African Case Study: Injury and Stock Damage in Loading Area

Case: At a small upholstery business in Gqeberha, a worker tried to lift multiple timber frame parts at once. While lifting, he twisted his upper body, resulting in a lower back strain. In the process, the panels were dropped and two were damaged beyond repair. Production was delayed by a day, and medical costs were incurred.

Discussion Points:

- What went wrong with the lifting method?

- How could the stacking of panels have prevented the drop?

- What tools or training might have improved the outcome?

Facilitator-Led Activities

- Lifting Demonstration

  - Demonstrate correct and incorrect lifting methods using foam blocks or lightweight boxes

  - Learners practise correct posture under supervision

- Stacking Challenge

  - Learners stack different mock components using spacers and check for balance and safety

  - Discuss what worked and what did not

- Workshop Observation (or Simulation)

  - Learners identify unsafe stacking or lifting practices in images or a real workspace

  - Provide recommendations for improvement

Critical Thinking Questions

- Why is it dangerous to twist your body while lifting a heavy object?

- What could happen if items are stacked too high or without spacers?

- How can lifting injuries affect production and the business as a whole?

- What should a worker do if they are unsure whether an item is too heavy to lift alone?

- How can good stacking practices reduce damage and material waste?

### KT0307 – Moving Materials and Equipment

Purpose of this Topic Element

This topic teaches learners the safe and ergonomic methods for moving materials and equipment in a furniture manufacturing or upholstery environment. It emphasises the correct use of tools, trolleys, and techniques that reduce injury risk and protect materials from damage during handling and transportation.

At NQF Level 2, learners are expected to understand basic handling procedures, select the right equipment for the task, and explain how these practices improve both safety and productivity on the workshop floor.

Key Content to Cover

- Why Safe Movement Matters

  - Prevents injury (strains, slips, impact injuries)

  - Avoids damage to materials and tools

  - Ensures clear, hazard-free workspaces

  - Promotes efficient workflow and reduces fatigue

- Common Materials and Equipment That Must Be Moved

  - Timber, plywood sheets, and frame components

  - Foam blocks, cushions, and padding

  - Upholstery fabric rolls and tools

  - Pneumatic staplers, compressors, and scissors

- Tools and Equipment for Moving

  - Trolleys: For moving foam, fabric, or panels safely across distances

  - Pallet jacks: For shifting stacked frame parts or heavier loads

  - Wheelbarrows or push carts: For offloading and site delivery

  - Roller stands: For feeding long items into cutting stations

  - Adjustable workbenches: Can double as moving stations for work in progress

- Best Practices When Moving Materials

  - Always assess the weight and size before moving an item

  - Use both hands and keep the load balanced

  - Avoid rushing; plan your path and clear the area

  - Use ramps or lifts for vertical movement, not manual lifting

  - Communicate clearly when moving items in shared workspaces

Practical Examples

| Material/Equipment | Correct Movement Method |
| --- | --- |
| Foam blocks | Use a trolley or carry one at a time to avoid obstruction |
| Fabric rolls | Use two hands or team lift; avoid dragging |
| Frame components | Use a cart or carry in pairs if large or long |
| Air compressor | Wheel into position using handle; ensure hoses are secured |

South African Case Study: Collision in Workshop Aisle

Case: At a factory in Pretoria, a worker was moving two frame panels by hand down a narrow aisle while another team member was pushing a trolley in the opposite direction. Neither announced their movement, and a collision occurred. The trolley damaged the corner of one frame, and the worker suffered a wrist sprain from the impact.

Discussion Points:

- What safety and communication procedures were missing?

- How could equipment or layout planning have helped?

- What equipment should have been used to transport the panels?

Facilitator-Led Activities

- Equipment Identification Exercise

  - Display images of trolleys, carts, roller stands, and pallet jacks

  - Learners name the equipment and describe appropriate uses

- Route Planning Simulation

  - Learners map out a path for moving a large frame from the cutting area to the upholstery station

  - Identify obstacles and safe practices along the way

- Practical Demo: Moving Safely

  - Demonstrate safe pushing, pulling, and turning techniques using light materials

  - Allow learners to practise under supervision

Critical Thinking Questions

- What is the safest way to move large or awkward items in a busy factory?

- How does poor material handling affect both worker safety and product quality?

- What types of injuries can happen if equipment is used incorrectly or not used at all?

- Why is communication important when moving materials in a shared space?

- What would you do if the equipment you needed to move something was not available?

## 🌟 Integrated Formative Assessment: KM-01-KT03 – Ergonomics Related to Furniture Manufacturing

Weight: 10% | Level: NQF 2
Internal Assessment Criteria Covered: IAC0301 to IAC0309

🧾 Case Study Scenario: SafeMove Upholstery Factory

SafeMove Upholstery is a small manufacturer in KwaZulu-Natal that supplies sofas, armchairs, and headboards to local furniture outlets. Recently, several issues have been reported:

- One of the workers was injured when lifting a stack of heavy frame panels incorrectly.

- A fabric roll was damaged due to poor storage.

- Some of the chairs delivered were returned because they were uncomfortable and difficult to get in and out of. The production manager has asked you to assist with identifying ergonomic risks and making suggestions to improve safety and comfort in the factory.

📝 Assessment Tasks

Task 1: Short Answer Questions

(Aligned to IAC0301, IAC0302, IAC0303, IAC0304 – 20 marks)

Q1. Define ergonomics and explain its importance in a furniture manufacturing environment. (IAC0301 – 5 marks)
Q2. List and briefly explain four ergonomic principles that should be followed in a furniture factory. (IAC0302 – 5 marks)
Q3. Describe the purpose of ergonomics and how it affects the quality of both the finished product and the production process. (IAC0303 – 5 marks)
Q4. Motivate why standard furniture measurements are important in terms of ergonomics. Provide two examples. (IAC0304 – 5 marks)

Task 2: Matching Activity

(Aligned to IAC0305 and IAC0306 – 10 marks)

Instructions: Match each piece of lifting or moving equipment in Column A with its correct use and ergonomic benefit in Column B.

| Column A: Equipment | Column B: Use and Benefit |
| --- | --- |
| A. Trolley | 1. Prevents bending when handling sheets of foam |
| B. Pallet jack | 2. Allows moving large materials over a distance with little effort |
| C. Adjustable workbench | 3. Raises work height to prevent stooping |
| D. Roller stand | 4. Helps shift stacked materials safely |
| E. Push cart | 5. Suitable for tools and small frames to avoid carrying by hand |

Task 3: Diagram Labelling

(Aligned to IAC0307 and IAC0308 – 10 marks)

Instructions: Label the following parts of a safe stacking and storage diagram using the terms listed below:

- Spacers

- Height limit

- Flat base

- Correct equipment

- Access aisle

Bonus: Describe how incorrect stacking could cause product damage or injury. (2 marks)

Task 4: Practical Observation – Lifting Technique Role Play

(Aligned to IAC0309 – 10 marks)

Instructions: In pairs, demonstrate the correct lifting procedure using a foam block or equivalent safe item. Your assessor will use a checklist to evaluate:

| Observation Checklist | Yes / No |
| --- | --- |
| 1. Keeps back straight |  |
| 2. Bends knees to lift |  |
| 3. Keeps load close to the body |  |
| 4. Lifts smoothly without jerking |  |
| 5. Avoids twisting during lifting |  |

✅ Model Answers and Marking Memo

Task 1: Short Answer (20 marks)

- Q1: Ergonomics is the science of designing tasks, tools, and furniture to fit the human body. It reduces injury, increases comfort, and improves productivity. (5 marks)

- Q2:

  - Fit the task to the person – use appropriate tools

  - Avoid awkward posture – work at proper height

  - Reduce force and repetition – use mechanical aids

  - Encourage movement and rest – avoid long static positions (1.25 each)

- Q3: Ergonomics improves worker health, reduces injuries, and improves product comfort. Chairs that are too low or soft may be hard to use. Ergonomic production prevents errors. (5 marks)

- Q4: Standard sizes ensure comfort and safety. Example 1: Seat height of 450 mm allows feet to rest flat. Example 2: Armrest height of 200 mm reduces shoulder strain. (5 marks)

Task 2: Matching (10 marks)

- A → 2

- B → 4

- C → 3

- D → 1

- E → 5
(2 marks each)

Task 3: Diagram Labelling (10 marks)

- Correct labels: spacers, flat base, height limit, access aisle, correct equipment (1 mark each)

- Bonus: Incorrect stacking can cause material damage, slipping hazards, or collapsing stacks. (2 marks)

## 🌟 Integrated Formative Assessment: KM-01-KT03 – Ergonomics Related to Furniture Manufacturing

Weight: 10% | NQF Level: 2
Internal Assessment Criteria Covered: IAC0301 – IAC0309

Instrument 1: Short Answer Questions

Targeted IACs: IAC0301, IAC0302, IAC0303
Total Marks: 15

- Define ergonomics and explain how it applies to furniture manufacturing processes. (IAC0301 – 5 marks)

- List and explain three basic principles of ergonomics used in the furniture manufacturing industry. (IAC0302 – 5 marks)

- Explain the purpose of ergonomics in terms of how it affects the finished furniture product and the production process. (IAC0303 – 5 marks)

Instrument 2: Matching Activity

Targeted IAC: IAC0305
Total Marks: 5

Instructions: Match the lifting or moving equipment in Column A with its use in Column B.

| Column A | Column B |
| --- | --- |
| A. Pallet jack | 1. Raises work surface to reduce bending and improve posture |
| B. Trolley | 2. Moves small frame parts across factory with little effort |
| C. Adjustable workbench | 3. Lifts and moves heavy stacked items like wood panels |
| D. Roller stand | 4. Supports long materials being cut or processed |
| E. Push cart | 5. Used to move large, soft items like foam cushions safely |

Instrument 3: Diagram Labelling

Targeted IACs: IAC0304, IAC0307, IAC0308
Total Marks: 9

Instructions: Label the following items on the stacking and equipment layout diagram using the terms provided below. (1 mark each)

- Spacers

- Access aisle

- Stack height limit

- Flat, level base

- Upright ladder

- Trolley

- Storage rack

- Protective wrap

- Fire exit clearance

Instrument 4: Scenario-Based Multiple Choice Questions

Targeted IAC: IAC0306
Total Marks: 5

Instructions: Circle the correct answer.

- What is the main ergonomic benefit of using a trolley to move fabric rolls?
a) Speeds up production only
b) Prevents shoulder and wrist injuries
c) Reduces cost of transport
d) Keeps the workshop neat

- A pallet jack improves ergonomics by:
a) Helping workers store tools
b) Eliminating the need for cutting tools
c) Reducing the need for manual lifting of stacked goods
d) Increasing the number of staff on duty

(3 more MCQs to be added if needed for balance – currently 2 = 2 marks; suggested balance for 5 marks)

Instrument 5: Practical Observation Checklist

Targeted IAC: IAC0309
Total Marks: 10

Instructions: The learner demonstrates correct lifting of a foam block or equivalent item. Assessor uses the checklist below.

| Ergonomic Lifting Practice | Yes (2) | No (0) |
| --- | --- | --- |
| 1. Approaches object squarely with feet shoulder-width apart |  |  |
| 2. Bends at the knees, not the back |  |  |
| 3. Keeps back straight throughout the lift |  |  |
| 4. Holds object close to the body |  |  |
| 5. Lifts using legs, not back |  |  |

✅ Model Answers and Marking Memo

Instrument 1: Short Answers

- Q1: Ergonomics is the study of fitting tasks, tools, and work environments to people. In furniture manufacturing, it ensures workers do not get injured and furniture is comfortable and supportive. (5 marks)

- Q2:

  - Fit the task to the person (use the right height workbench)

  - Avoid awkward postures (do not twist when lifting)

  - Reduce repetitive tasks (use tools to assist) (5 marks – 1.5 per point + 0.5 for clarity)

- Q3: Ergonomics improves comfort in final products (e.g., correct seat height) and reduces injuries during production (e.g., using lifting aids). (5 marks)

Instrument 2: Matching (5 marks)

- A → 3

- B → 5

- C → 1

- D → 4

- E → 2
(1 mark each)

Instrument 3: Diagram Labelling (9 marks)

Each correctly placed label = 1 mark

Instrument 4: Multiple Choice (5 marks)

- b) Prevents shoulder and wrist injuries

- c) Reduces the need for manual lifting of stacked goods
(Additional MCQs can be added for 3 more marks)

Instrument 5: Observation Checklist (10 marks)

Each correct technique observed = 2 marks
Total = 10 marks

📊 Assessment Rubric: KM-01-KT03 – Ergonomics

| Criteria | 5 – Excellent | 4 – Competent | 3 – Developing | 1–2 – Not Yet Competent |
| --- | --- | --- | --- | --- |
| Understanding of ergonomic definitions and principles | Accurate, clear, and well-reasoned | Mostly correct, shows basic understanding | Partial understanding, some confusion | Incorrect or no understanding shown |
| Knowledge of equipment and their correct applications | Correctly matches and explains purpose | Mostly correct with minor errors | Some incorrect matches | Incorrect or no identification |
| Application of lifting/stacking procedures | Fully correct technique with safety awareness | Mostly correct with minor technique flaws | Needs guidance; steps missed | Unsafe or incorrect technique |
| Labelling of safety and storage components | All labels correct and well-placed | Minor errors, mostly accurate | Some labels correct, others missing | Incorrect or not attempted |

Task 4: Observation Checklist (10 marks)

- 2 marks per correct technique

- All five observed techniques = 10 marks

📊 Assessment Rubric: KM-01-KT03 – Ergonomics

| Criteria | 5 – Excellent | 4 – Competent | 3 – Developing | 1–2 – Not Yet Competent |
| --- | --- | --- | --- | --- |
| Demonstrates understanding of ergonomic principles | Defines and applies concepts correctly and in context | Mostly accurate, minor misunderstandings | Limited understanding or application | Misunderstands basic concepts |
| Can explain purpose and outcomes of ergonomics | Clear link to comfort, productivity, and safety | Reasonable link to user and workplace wellbeing | Vague or incomplete explanations | No link between ergonomics and product/process |
| Identifies and uses equipment appropriately | Correct matches and real-life application examples | Mostly correct matches and uses | Some errors in matching or function description | Unable to identify or match equipment correctly |
| Applies correct lifting and stacking techniques | Demonstrates full understanding in role-play | Follows most steps correctly | Needs assistance or forgets key steps | Unsafe or incorrect handling |

## KM-01-KT04: Quality in the Upholstered Furniture Manufacturing Process

NQF Level 2 | Credits: [TBC by curriculum] | Weight: 20%

This knowledge topic provides learners with the essential foundation to understand and apply the concepts and practices related to quality in the manufacture of upholstered furniture. At this level, quality is not only about the appearance or comfort of the final product, but also about the processes, materials, tools, teamwork, and systems that support consistent and efficient production.

Learners will be introduced to the core definitions of quality, quality control, and quality assurance, and will learn to distinguish between these terms in practical terms. They will gain insight into how quality is monitored throughout the production process — from the inspection of raw materials and the preparation of frames, to the final covering and dispatch of finished components.

In addition to understanding quality systems, learners will explore common causes of quality problems, including faults in materials, breakdowns in equipment, poor handling, and human factors such as absenteeism and ineffective teamwork. They will be taught how to assess the implications of these issues, and how to contribute positively to the quality management process through consistent checks, safe storage, accurate reporting, and effective handling procedures.

This topic also covers production routines, inspection procedures, and post-production controls, with a strong focus on identifying and addressing product defects and faults. Learners will be expected to analyse how these defects occur, why final quality checks are essential, and how proper materials handling and stacking influence the integrity of products as they move to the next stage in manufacturing or to the client.

On successful completion of this topic, learners will understand how quality systems operate in real upholstery workshops, and how their own attention to detail, teamwork, and procedural adherence directly contribute to the reputation and success of the organisation and the industry as a whole.

The topic consists of the following elements:

- KT0401: Definition and importance of quality, quality control and quality assurance

- KT0402: Principles and practices of quality control

- KT0403: Principles and practices of quality assurance

- KT0404: Types and causes of quality problems, including raw material faults and defects, tools and equipment faults and defects

- KT0405: Consequences of poor quality management for the frame preparing and frame covering department, the company and the industry

- KT0406: Production routines, product inspections and quality control during upholstery frame covering processes

- KT0407: Concepts of product defects and faults and their causes

- KT0408: Principles and concepts of final quality check for product defects and faults

- KT0409: Reporting procedures for the final prepared frame

- KT0410: Materials handling and product stacking and storing for the next process

- KT0411: Storage and dispatch of upholstered frame components and complete products

Assessment of this topic is based on the following Internal Assessment Criteria:

- IAC0401: Concepts of quality, quality control and quality assurance are defined and differentiated

- IAC0402: Causes of quality problems in the frame preparation and frame covering department are identified and their implications assessed

- IAC0403: Consequences of poor or no quality management systems are evaluated

- IAC0404: Pre-production, production and post-production routines and quality control checks are described and evaluated

- IAC0405: Defects and faults in furniture are assessed and their causes analysed

- IAC0406: Reasons for final quality check procedures are justified

- IAC0407: Consequences of poor materials handling and unsafe product stacking and storing are analysed

Learners who master this topic will be better prepared to maintain high standards in every stage of the upholstery process, helping to ensure customer satisfaction, reduced waste, and efficient production operations.

### KT0401 – Definition and Importance of Quality, Quality Control and Quality Assurance

Purpose of this Topic Element

This topic introduces learners to the fundamental definitions of quality, quality control (QC), and quality assurance (QA) within the upholstered furniture manufacturing environment. Understanding these concepts helps learners recognise their role in maintaining standards throughout production, from raw material selection to the dispatch of completed products.

At NQF Level 2, learners are expected to define each term clearly, describe how they differ, and explain why they matter at every stage of the frame preparation and upholstery process.

Key Content to Cover

- Definition of Quality

  - Quality is the degree to which a product meets customer expectations and production standards.

  - In furniture: it includes comfort, durability, aesthetic appeal, and workmanship.

- Definition of Quality Control (QC)

  - QC is the inspection and testing of materials and products at different stages of production to identify and correct defects.

  - Involves checks during frame preparation, material cutting, upholstery fitting, and finishing.

- Definition of Quality Assurance (QA)

  - QA refers to the systems and processes in place to prevent problems before they occur.

  - Examples include setting standards, training workers, creating procedures, and using checklists.

- Difference Between QC and QA

  - QC is reactive: it checks for problems after they occur.

  - QA is proactive: it sets up systems to prevent problems.

- Importance of Quality in Upholstered Furniture

  - Satisfied customers and fewer returns

  - Better company reputation and more business

  - Less rework, waste, and downtime

  - Increased safety and usability of the product

  - Compliance with client specifications and industry standards

Practical Examples

| Concept | Example in Upholstery Workshop |
| --- | --- |
| Quality | A sofa is well-finished, comfortable, with no loose threads or dents |
| Quality Control | Checking each frame for cracks before fitting the upholstery |
| Quality Assurance | Using a checklist to guide each step of the frame preparation |

South African Case Study: Missed Quality Checks Lead to Rework

Case: In a Johannesburg-based upholstery company, a batch of armchairs was delivered to a hotel. The chairs were aesthetically pleasing but several had wobbly legs, exposed staples, and uneven stitching. The hotel returned half the order. Investigation showed that quality control was skipped due to production pressure, and staff were not trained to follow the QA procedures.

Discussion Points:

- Which quality control and assurance steps were missed?

- What was the impact on the business?

- How could better QA have prevented the problem?

Facilitator-Led Activities

- Definitions Sorting Activity

  - Provide learners with a list of statements; they categorise them as relating to Quality, Quality Control, or Quality Assurance.

- Role-Play: Quality Walkthrough

  - Learners act as QC inspectors in a simulated upholstery line.

  - They identify whether a product needs rework or meets the required standard.

- Pair and Share

  - Learners work in pairs to list five examples of poor quality they have seen or experienced (home, shops, or workshops).

  - Then identify whether it was a QC or QA issue.

Critical Thinking Questions

- Why is it important to understand the difference between quality control and quality assurance?

- What are the possible results of poor-quality products reaching the customer?

- How can workers on the factory floor help prevent quality problems?

- What would happen if there were no quality control checks in the workshop?

- How does teamwork support the achievement of quality in furniture production?

### KT0402 – Principles and Practices of Quality Control

Purpose of this Topic Element

This topic helps learners understand the principles and practices of quality control (QC) within the upholstered furniture manufacturing environment. QC is an essential process that ensures the furniture produced is free of defects, meets customer expectations, and conforms to production specifications.

At NQF Level 2, learners are expected to describe QC in practical terms, explain its purpose at various stages of production, and understand how to perform basic quality checks as part of their day-to-day duties.

Key Content to Cover

- Definition of Quality Control (QC)

  - QC is the process of monitoring, inspecting, and testing materials, components, and finished products to detect and correct faults.

- Principles of Quality Control

  - Inspection at key stages: Pre-production, in-process, and final inspection.

  - Preventing defects from progressing to the next stage of production.

  - Consistency in product quality, regardless of who performs the task.

  - Feedback and improvement: Using QC findings to prevent future problems.

  - Documentation: Recording inspection results and actions taken.

- Quality Control Practices in Upholstery

  - Measuring timber for straightness and cracks before cutting

  - Checking frame joints for looseness or splitting before assembly

  - Ensuring fabric pieces are cut according to pattern sizes

  - Inspecting stitched seams for strength and neatness

  - Checking for correct cushion fit, tension in webbing, and clean surfaces

  - Ensuring labels, tags, or branding elements are attached correctly

- Who Performs Quality Control?

  - QC may be performed by dedicated inspectors or line workers.

  - Everyone in the production process contributes by checking their own work.

Practical Examples

| QC Activity | When Performed | Why It Matters |
| --- | --- | --- |
| Inspecting frame joints | After frame assembly | Prevents weak furniture and reduces returns |
| Checking fabric panels before sewing | Before upholstery fitting | Ensures correct size, avoids waste |
| Testing seat cushion firmness | Final product inspection | Ensures customer comfort and product performance |
| Verifying measurements of cut components | During pre-production | Maintains accuracy and consistency |

South African Case Study: Overlooked QC at Stitching Station

Case: In a Cape Town upholstery factory, a large order of dining chairs was prepared for a new restaurant. Due to pressure to meet the deadline, the stitching quality of seat covers was not inspected. As a result, many covers had loose threads and weak seams. After installation, the customer complained about tearing within a week, resulting in a costly rework and reputational damage.

Discussion Points:

- Which QC steps were ignored?

- What could the inspectors or line workers have done differently?

- What part of the production routine should have included this inspection?

Facilitator-Led Activities

- QC Checklist Creation

  - Learners work in small groups to create a basic quality control checklist for a product of their choice (e.g., ottoman, armchair).

  - They then role-play using this checklist to inspect a sample item or drawing.

- QC Observation Practice

  - Using real or mock samples of furniture components (foam, fabric, wooden frame pieces), learners perform a basic inspection.

  - They record defects and recommend corrective actions.

- Group Brainstorm

  - Learners identify three benefits and three risks of ignoring QC at different stages of production.

Critical Thinking Questions

- What is the difference between checking your work and performing a full quality control inspection?

- Why should quality control happen at different stages of production and not just at the end?

- What might happen if a frame with a small crack is not detected during QC?

- How can quality control help reduce material waste?

- Who is responsible for quality control in a workshop, and why should every worker be involved?

### KT0403 – Principles and Practices of Quality Assurance

Purpose of this Topic Element

This topic introduces learners to the principles and practices of quality assurance (QA) in the upholstered furniture manufacturing process. While quality control focuses on identifying and correcting problems after they occur, quality assurance is about preventing problems before they happen through systems, planning, and continuous improvement.

At NQF Level 2, learners will be expected to describe QA in simple, operational terms and identify how systems such as checklists, standard procedures, team training, and good communication contribute to consistent product quality and workplace efficiency.

Key Content to Cover

- Definition of Quality Assurance (QA)

  - QA is the system of planned activities that ensure quality is built into every stage of the manufacturing process.

  - It is proactive, meaning it aims to prevent defects or errors before they occur.

- Principles of Quality Assurance

  - Consistency: Following the same processes every time to get reliable results.

  - Documentation: Recording procedures and expected outcomes (e.g., SOPs).

  - Prevention: Stopping problems by identifying risks early.

  - Responsibility: QA is a shared responsibility; everyone follows agreed procedures.

  - Continuous Improvement: Using feedback to improve methods and reduce waste.

- Quality Assurance Practices in Upholstered Furniture Manufacturing

  - Standard operating procedures (SOPs) for each workstation

  - Training workers on product specifications and acceptable tolerances

  - Using templates for cutting and marking fabric

  - Planning inspections and assigning roles

  - Encouraging reporting of issues before they become serious

  - Keeping production areas clean and well-organised

- Difference Between QC and QA

  - QA is built into the process (e.g., staff training, process planning)

  - QC checks the result of the process (e.g., product inspection)

Practical Examples

| Quality Assurance Activity | Purpose |
| --- | --- |
| Training all staff on stitching standards | Prevents weak seams before they occur |
| Using fabric-cutting templates | Ensures parts are consistent in size and fit |
| Labelling raw materials before use | Prevents mix-ups that could lead to incorrect items |
| Keeping checklists for frame inspections | Ensures no step is missed during production |

South African Case Study: No QA, High Rework Costs

Case: In a Durban-based factory, a new team was hired to assist with increased production. The company did not provide clear guidelines, and workers followed their own methods. Several completed headboards had mismatched fabric panels and incorrect heights. These were only noticed at final inspection. Rework took two full days and delayed delivery.

Discussion Points:

- What QA practices could have prevented these errors?

- Was the problem with the product, the process, or the people?

- How does a lack of documentation affect production quality?

Facilitator-Led Activities

- Quality Assurance vs Quality Control Sorting

  - Learners are given cards with statements or examples and must sort them under QA or QC.

- Create a QA Flow

  - In small groups, learners develop a flowchart showing the steps needed to ensure quality in preparing an upholstered armchair, from frame inspection to final check.

- Team Audit Simulation

  - Learners conduct a mock QA “audit” using a checklist to assess whether all QA practices are in place for a selected task (e.g., fabric cutting station).

Critical Thinking Questions

- Why is quality assurance more than just checking the final product?

- How do training and standard procedures help prevent mistakes in upholstery production?

- What could happen if a business has no QA system in place?

- Who is responsible for ensuring QA is followed on the production floor?

- What QA practice could you apply in your current or future workplace to improve consistency?

### KT0404 – Types and Causes of Quality Problems

Purpose of this Topic Element

This topic focuses on helping learners identify and understand the types and causes of quality problems that can occur in the upholstered furniture manufacturing process. These problems may relate to raw materials, tools and equipment, incorrect use of materials, or human factors such as absenteeism or poor teamwork.

At NQF Level 2, learners are expected to recognise common production faults and understand the impact these faults have on product quality, production efficiency, and customer satisfaction. The goal is to develop a proactive awareness of quality risks in both the frame preparation and frame covering departments.

Key Content to Cover

- Types of Quality Problems

  - Material faults: wood knots, warping, foam inconsistencies, faded or torn fabric

  - Incorrect material use: using the wrong fabric, glue, thread, or foam density

  - Workmanship defects: uneven stitching, poorly fitted fabric, wobbly frames

  - Machine or tool faults: blunt blades, loose staplers, malfunctioning sewing machines

  - Finishing defects: poor alignment, exposed staples, or uneven padding

- Common Causes of Quality Problems

  - Use of low-grade or damaged raw materials

  - Lack of knowledge or training on correct processes

  - Unavailable or poorly maintained equipment

  - Power interruptions during production

  - Absenteeism or lack of supervision

  - Poor teamwork or communication on the production floor

  - Rushing production to meet deadlines without proper checks

- Departments Affected

  - Frame preparation department: wood selection, joining, sanding, frame stability

  - Frame covering department: foam cutting, fabric application, stitching, finish

- Impact of Quality Problems

  - Customer complaints and product returns

  - Production delays and financial losses

  - Waste of materials and resources

  - Loss of reputation and client trust

Practical Examples

| Cause of Problem | Resulting Fault |
| --- | --- |
| Using warped wood | Chair legs are uneven or unstable |
| Incorrect foam type used | Cushion is too soft or too firm, affecting comfort |
| Loose stitching on upholstery | Seams split after short use |
| Dull cutting blade | Frayed fabric edges or inconsistent shapes |
| Absent worker not replaced | Delay in production or task rushed by others |
| Team not communicating sizes | Cover pieces do not fit frames correctly |

South African Case Study: Tool Fault Leads to Product Failure

Case: In a workshop in Polokwane, a staple gun used in the frame covering department was not firing consistently. Instead of reporting the issue, the operator continued using it, leading to loose upholstery in several sofas. The defect was only picked up during delivery, resulting in returned products and an unhappy client.

Discussion Points:

- What was the cause of the fault?

- Could this have been avoided?

- How can proper tool maintenance reduce quality problems?

Facilitator-Led Activities

- Fault Identification Table

  - Provide photos or real examples of upholstery faults (e.g., torn seam, uneven legs).

  - Learners list the possible causes of each and match them to the production stage.

- Root Cause Brainstorm

  - Learners choose one common defect and list all possible causes using a cause-and-effect (“fishbone”) diagram.

- Workstation Walkabout (if applicable)

  - Learners observe current production or simulated areas and identify any risks that could lead to quality problems.

Critical Thinking Questions

- Why is it important to trace a fault back to its cause, not just fix the product?

- What could happen if small faults are ignored during production?

- How can team communication help prevent quality problems?

- What should you do if you notice a defect in your materials or tools before starting work?

- How can preventing quality problems save the company money and time?

### KT0405 – Consequences of Poor Quality Management

Purpose of this Topic Element

This topic helps learners understand the short-term and long-term consequences of poor or absent quality management systems in the upholstered furniture manufacturing process. Learners will consider the impact not only on the frame preparation and frame covering departments, but also on the company and the broader industry.

At NQF Level 2, learners are expected to explain in simple terms how poor quality systems can lead to increased costs, customer dissatisfaction, safety risks, wasted materials, and loss of business. They will also be encouraged to identify their own role in supporting a strong quality culture in the workplace.

Key Content to Cover

- What is Poor Quality Management?

  - Lack of standards, systems, or procedures to ensure quality

  - Failure to inspect, monitor, or respond to faults during production

  - Inconsistent application of quality checks

  - Absence of team communication about product issues

- Consequences for the Frame Preparation Department

  - Unstable frames, weak joints, incorrect dimensions

  - Need for rework or complete replacement

  - Slowing down the next stage of production (covering)

- Consequences for the Frame Covering Department

  - Incorrect fitting of foam and fabric

  - Product defects passed to final inspection or customer

  - Wasted material due to rework or poor handling

- Consequences for the Company

  - Customer complaints, product returns, and negative reviews

  - Financial losses due to rework, warranty claims, and downtime

  - Missed deadlines and reputational damage

  - Increased pressure on staff, low morale, and high turnover

- Consequences for the Industry

  - Reduced customer trust in locally made products

  - Lower demand and competitiveness in the market

  - Difficulty in maintaining skilled labour due to job insecurity

Practical Examples

| Consequence | Example in Practice |
| --- | --- |
| Customer dissatisfaction | Client returns a lounge suite due to fabric wrinkling and poor fit |
| Production delays | Stapling tools fail during covering stage, pausing all work |
| Increased waste | 10 metres of fabric cut incorrectly due to absence of templates |
| Brand damage | Hotel contract cancelled after repeated delivery of faulty goods |

South African Case Study: Failed Tender Due to Quality Failures

Case: A medium-sized upholstery company in the Western Cape was awarded a government school tender to supply dormitory beds. However, the company delivered items with visible defects: misaligned headboards, uneven padding, and weak frame joints. Due to poor quality assurance and inspection systems, the company lost the contract renewal, and another supplier was selected for future orders.

Discussion Points:

- Which departments contributed to the quality failure?

- What business consequences followed?

- What quality systems could have prevented this?

Facilitator-Led Activities

- Group Discussion: Chain Reaction

  - Learners create a visual chain showing how one small fault (e.g. no fabric inspection) can lead to larger company or industry consequences.

- Reputation Role Play

  - In small groups, learners play the roles of customer, supervisor, and worker. They discuss how quality faults impact each stakeholder.

- Case Review Comparison

  - Present learners with two project examples: one with strong quality management, and one without. Learners identify key differences in outcome.

Critical Thinking Questions

- How can a small quality mistake become a big problem for a company?

- Why does every worker play a part in quality management, not just the inspector?

- What happens to team morale when the same quality problems keep happening?

- How can a poor-quality product affect how people view South African-made furniture?

- What is one thing you can do every day to help maintain quality standards in your department?

### KT0406 – Production Routines, Product Inspections and Quality Control

Purpose of this Topic Element

This topic provides learners with a practical understanding of the production routines, inspection points, and quality control (QC) activities that take place during the frame covering process in an upholstery workshop. It highlights the importance of consistency, attention to detail, and routine checks to maintain quality throughout production.

At NQF Level 2, learners are expected to follow defined routines, apply basic inspection steps, and participate in quality control activities as part of their daily work responsibilities.

Key Content to Cover

- What Are Production Routines?

  - Step-by-step processes followed to complete tasks in the same way every time

  - Designed to ensure consistency, efficiency, and predictable outcomes

  - Includes preparing tools and materials, following work instructions, cleaning stations, and checking work

- Common Upholstery Frame Covering Routines

  - Preparing frames (dusting, sanding, checking joints)

  - Cutting and applying foam and padding

  - Cutting and fitting fabric panels using templates

  - Attaching covers using staples or tacks

  - Final smoothing, trimming, and presentation

- Inspection During Production

  - Visual checks for fabric alignment, seam quality, foam firmness

  - Physical testing (e.g. pressing cushions, checking joint strength)

  - Verifying correct materials and components were used

  - Ensuring templates and measurements were followed

  - Spot-checking by supervisors or quality inspectors

- Final Quality Control Checks

  - Ensuring the product matches the work order or client brief

  - Checking for visible faults, uneven surfaces, exposed fasteners

  - Verifying proper labelling, batch marking, or product ID

  - Signing off on checklists or quality documents

- Why Routines and Inspections Matter

  - Helps identify problems before they reach the next stage

  - Builds habits of consistency and pride in workmanship

  - Saves time and reduces waste from rework or returned products

Practical Examples

| Routine or Inspection Step | Purpose |
| --- | --- |
| Dusting frame before covering | Prevents trapped dust under fabric or glue issues |
| Using a cutting template for side panels | Ensures correct fit and prevents uneven seams |
| Pressing cushion after stuffing | Checks for sagging or overfilling |
| Checking staple placement on fabric edge | Prevents loose fabric or visible fasteners |
| Final checklist review | Confirms product meets all client and production standards |

South African Case Study: Missing a Step, Losing Time

Case: At a workshop in Limpopo, a team was rushing to complete an order of upholstered footstools. One operator skipped the foam adhesion routine, leading to poorly cushioned tops that shifted under the fabric. The entire batch had to be reopened and redone, delaying delivery by two days.

Discussion Points:

- What routine step was skipped?

- How could inspections have prevented the issue?

- What was the cost in terms of time and resources?

Facilitator-Led Activities

- Routine Mapping

  - Learners work in pairs to map the production routine for a simple upholstered product (e.g. footstool, armrest).

  - Identify where inspections and quality checks should occur.

- Inspection Station Simulation

  - Set up a sample station with various upholstery faults (e.g. misaligned fabric, under-filled cushion).

  - Learners inspect and complete a QC checklist.

- Spot-the-Step Game

  - Show images of correct and incorrect production steps.

  - Learners must identify what step was missed or done incorrectly.

Critical Thinking Questions

- Why is it important to follow the same routine every time you complete a task?

- What could happen if one inspection step is skipped during production?

- How does each worker’s routine help the next stage of production?

- What role does the final quality control check play before dispatching a product?

- How can simple inspection routines save time and money in the long run?

### KT0407 – Concepts of Product Defects and Faults and Their Causes

Purpose of this Topic Element

This topic enables learners to recognise and differentiate between product defects and faults in upholstered furniture, and to identify the causes of these issues during the frame preparation and covering stages. Understanding the difference between a defect and a fault, as well as their root causes, helps prevent rework, reduce material waste, and ensure that quality standards are met.

At NQF Level 2, learners are expected to describe common problems seen in upholstered furniture and explain how these can result from incorrect processes, poor material handling, or equipment issues.

Key Content to Cover

- What is a Product Defect?

  - A defect is a flaw in the final product that reduces its functionality, appearance, or safety.

  - It is usually visible or felt by the end-user and may lead to customer dissatisfaction or returns.

- What is a Product Fault?

  - A fault is often a technical error in construction or assembly that may not be immediately visible, but will affect the product’s performance or lifespan over time.

  - Faults are sometimes identified during inspections or through customer complaints.

- Examples of Product Defects

  - Uneven or sagging cushions

  - Fabric wrinkles, visible staples, or loose seams

  - Exposed wood on frame edges

  - Mismatched fabric panels

  - Surface indentations or stains

- Examples of Product Faults

  - Weak or misaligned frame joints

  - Incorrect webbing tension leading to poor seat support

  - Loose or broken spring systems

  - Substandard foam density used in seat padding

- Common Causes of Defects and Faults

  - Poor quality or damaged raw materials

  - Incorrect cutting or measuring of fabric or foam

  - Inaccurate template use or faulty equipment

  - Lack of skill or inattention to detail

  - Rushing production or skipping inspection steps

  - Lack of communication between departments

Practical Examples

| Defect/Fault | Possible Cause |
| --- | --- |
| Seat cushion sags within a week | Low-density foam used or insufficient webbing |
| Fabric visibly misaligned | Template not used or rushed application |
| Uneven legs | Frame not checked for squareness or level |
| Loose seams | Stitching too wide or incorrect tension on machine |
| Cracks in frame visible under fabric | Damaged timber used or over-stapling during covering |

South African Case Study: Rejected Stock Due to Hidden Faults

Case: In a Gauteng-based factory, a large retail client rejected a batch of lounge chairs after delivery. Although the fabric and stitching appeared correct, the chairs had a hidden fault: the seat springs were poorly installed and collapsed under normal use. The fault was traced to a rushed installation process and the absence of a mid-production inspection.

Discussion Points:

- Was the problem a defect or a fault?

- Could the issue have been prevented? At which stage?

- How could this affect the business’s long-term relationship with the client?

Facilitator-Led Activities

- Defect vs Fault Sorting

  - Provide examples or photos of furniture issues.

  - Learners categorise them as defects or faults and list likely causes.

- Root Cause Ladder

  - Learners are given a fault (e.g. loose armrest).

  - In groups, they build a “ladder” identifying all the steps and choices that may have caused it (e.g. incorrect screw length, no QC check, wrong drill bit).

- Sample Inspection Practice

  - Use real or mock-up pieces with planted defects or faults.

  - Learners perform a basic inspection and report findings using an observation sheet.

Critical Thinking Questions

- Why is it important to know the difference between a defect and a fault?

- What is more dangerous: a visible defect or a hidden fault? Why?

- How can skipping a simple quality check lead to a product fault?

- What can you do if you find a defect at your workstation that you did not cause?

- What would you say to a team member who tells you to ignore a small defect to save time?

### KT0408 – Principles and Concepts of Final Quality Check for Product Defects and Faults

Purpose of this Topic Element

This topic introduces learners to the principles and practices involved in conducting a final quality check on upholstered furniture before it moves to dispatch or the next production stage. Final quality checks are a vital last opportunity to identify and resolve defects or faults that may have been missed during production. At NQF Level 2, learners are expected to understand the role of the final inspection in maintaining customer satisfaction and upholding product standards.

Key Content to Cover

- What Is a Final Quality Check?

  - A systematic inspection of the completed product before storage, dispatch, or handover

  - Ensures the item meets all design, functional, and aesthetic standards

  - Involves checking appearance, function, stability, finish, and customer specifications

- Purpose of Final Quality Checks

  - To prevent defective or faulty products from reaching customers

  - To protect the reputation of the company and reduce returns or complaints

  - To ensure compliance with production, safety, and ergonomic standards

  - To act as the final opportunity to catch any overlooked issues

- What to Inspect During Final Quality Check

  - Fabric: no wrinkles, stains, or mismatched seams

  - Frame: stable and square, no movement or gaps in joints

  - Upholstery: even cushioning, no sagging, tight and smooth finish

  - Finishes: no visible staples or rough edges

  - Labels, tags, or markings present and correct

- Who Performs the Final Quality Check?

  - Often done by team leaders, supervisors, or designated quality controllers

  - However, all staff are expected to report any visible problems

- Recording and Reporting

  - Findings are logged in a quality control sheet or checklist

  - Defective items are sent for rework with clear notes

  - Approved items are signed off for dispatch or storage

Practical Examples

| Item Inspected | What to Look For | Action If Problem Found |
| --- | --- | --- |
| Chair leg joints | Movement, cracks, misalignment | Return to workshop for frame repair |
| Backrest upholstery | Sagging, uneven finish, loose corners | Sent to be re-stretched and secured |
| Fabric panels | Fading, stains, mismatched grain direction | Replace fabric before dispatch |
| Seat cushion | Too soft, uneven shape, foam shifting | Adjust foam or replace component |

South African Case Study: Delivery Delayed by Final Inspection

Case: At an upholstery firm in East London, a delivery of high-end ottomans was halted during final inspection. Although the products appeared complete, a final check revealed inconsistencies in foam firmness and visible staple heads on several pieces. As a result, the items were flagged, returned for minor rework, and delivered two days later—but with zero returns from the client.

Discussion Points:

- What issues were caught in the final check?

- How did this inspection protect the company?

- Would it have been better to deliver on time but with small defects?

Facilitator-Led Activities

- Mock Final Inspection Exercise

  - Learners receive checklists and inspect prepared mock items with embedded defects

  - They complete QC reports and classify items as “pass”, “rework”, or “reject”

- Checklist Creation Task

  - In groups, learners design a final inspection checklist for a specific product (e.g. headboard, dining chair)

- Video Demonstration and Reflection

  - Watch a short demonstration of a final QC process (live or recorded)

  - Learners reflect on what was done well and what was missed

Critical Thinking Questions

- Why is the final quality check considered the most important one before dispatch?

- How does a final check help reduce customer complaints?

- What should a worker do if they are unsure whether a product meets the quality standard?

- How would you feel as a customer receiving furniture with visible defects?

- What are the risks of skipping or rushing through final quality inspections?

### KT0409 – Reporting Procedures for the Final Prepared Frame

Purpose of this Topic Element

This topic introduces learners to the standard reporting procedures that must be followed after a furniture frame has been fully prepared and covered. The correct reporting of quality outcomes, faults, and completion status helps ensure traceability, supports continuous improvement, and prepares the product for the next phase, whether storage, dispatch, or assembly.

At NQF Level 2, learners should understand the importance of accurate and timely reporting, be able to identify what information is required, and demonstrate basic completion of quality and production records.

Key Content to Cover

- What Is a Final Frame Report?

  - A documented summary of the product’s condition after the final preparation stage

  - Includes results of quality checks, identification of defects or rework, and confirmation of readiness for next process or delivery

- Purpose of Reporting

  - Provides evidence that checks were completed

  - Helps with traceability (who did what and when)

  - Alerts supervisors or QA teams to any recurring issues

  - Ensures all products are signed off before progressing

- What to Include in Reporting

  - Date and time of completion

  - Product or batch ID number

  - Inspector’s name and/or preparer’s name

  - Results of inspection (Pass / Rework / Reject)

  - Noted defects or faults (if any)

  - Action taken (e.g. repaired, sent to rework, approved for dispatch)

  - Comments or recommendations

- Types of Reporting Tools

  - Paper-based checklists or job cards

  - Daily production logs

  - Quality control forms

  - Electronic systems (where applicable)

- Reporting Flow

  - Completed by the person responsible for final preparation or inspection

  - Handed to the supervisor or QA officer for review

  - Stored with batch files or uploaded to system if required

Practical Examples

| Section of Report | Example Entry |
| --- | --- |
| Product ID | Chair-MDL/245-23 |
| Date/Time | 2025/04/06 – 14h30 |
| Inspected By | J. Dlamini |
| Result | Rework |
| Fault Noted | Fabric seam not aligned at backrest |
| Action Taken | Returned to covering department for adjustment |
| Comments | Review template position for consistency |

South African Case Study: Lost Time Due to Missing Report

Case: At a furniture manufacturer in Mpumalanga, a batch of armchairs was stacked for dispatch without proper completion of the final frame preparation reports. During loading, one chair collapsed due to a loose joint that had not been recorded or flagged. The entire load had to be unpacked and rechecked, costing the company hours in lost time and delivery penalties.

Discussion Points:

- What reporting step was skipped?

- How could the issue have been prevented?

- Why is documentation critical in production environments?

Facilitator-Led Activities

- Report Form Completion Task

  - Learners are given a mock inspection scenario and must complete a quality control form for the final prepared frame.

- Job Card Role Play

  - In pairs, one learner acts as the final preparer, and the other as the supervisor. They review a job card and discuss what further actions are needed.

- Sample Audit Simulation

  - Provide a set of fictitious frame reports (some incomplete, some accurate). Learners identify which ones meet requirements and justify their decisions.

Critical Thinking Questions

- Why is it important to record inspection findings, even when there are no defects?

- What might happen if reporting is delayed or not completed at all?

- Who should be responsible for signing off the final preparation report?

- How can accurate reporting improve communication between departments?

- What could a report tell you about a recurring problem on the production line?

### KT0410 – Materials Handling and Product Stacking and Storing for the Next Process

Purpose of this Topic Element

This topic introduces learners to correct practices for handling, stacking, and storing prepared upholstered frames and materials to prevent damage and maintain quality as items move from one production stage to the next. At NQF Level 2, learners must understand how proper handling and storage methods support a safe, efficient, and quality-driven production environment.

Key Content to Cover

- What Is Materials Handling in Upholstery?

  - The movement and positioning of items (frames, foam, fabric, tools) in the workshop

  - Includes lifting, carrying, stacking, pushing, pulling, and placing materials

- Principles of Good Materials Handling

  - Use correct lifting techniques to protect workers and products

  - Choose the right tools and equipment (e.g. trolleys, straps, foam carriers)

  - Plan routes to avoid congestion and accidents

  - Avoid dragging or throwing materials

  - Handle components with care to avoid scratches, tears, or deformation

- Stacking and Storing Products

  - Stack items according to size, shape, and weight

  - Use spacers or protective padding to prevent contact damage

  - Never place heavy items on soft or finished components

  - Keep stacks stable and below safe height limits

  - Store items in dry, clean, and designated areas

- Transition to the Next Process

  - Label and organise products clearly for the receiving department

  - Record movement in stock or production logs

  - Communicate readiness or defects to relevant teams

- Risks of Poor Handling and Stacking

  - Damage to prepared frames (dents, scratches, warping)

  - Misplacement or loss of labelled items

  - Workplace injuries due to falls or unstable stacks

  - Rework due to unnoticed storage damage

Practical Examples

| Handling/Storage Task | Good Practice | Risk If Done Incorrectly |
| --- | --- | --- |
| Storing covered chair frames | Use padding between items and stack 3 high max | Torn fabric, collapsed legs |
| Moving foam blocks | Use a trolley or carry flat one at a time | Deformed foam or trip hazard |
| Lifting headboards | Use two workers and place against a wall with spacing | Personal injury, scuffed fabric |
| Labelling batch items for next stage | Use waterproof tags with product code | Incorrect order assembly or dispatch delays |

South African Case Study: Delivery Delay Due to Poor Storage

Case: In a Cape Town factory, several completed lounge chairs were stacked on top of each other without spacers. Overnight, one stack fell, damaging the upholstery and frame structure. The batch had to be reworked, delaying delivery by three days and requiring urgent overtime labour.

Discussion Points:

- What handling or stacking practices were missed?

- How could these items have been stored differently?

- What systems or tools might have prevented the damage?

Facilitator-Led Activities

- Stacking Do’s and Don’ts

  - Show images or physical examples of good and poor stacking

  - Learners explain what is correct or incorrect and suggest improvements

- Storage Zone Design

  - Learners sketch a simple layout of a safe and efficient storage area for upholstered frames and materials

- Manual Handling Demonstration

  - Teach and practise safe lifting techniques using foam, fabric rolls, or frames

  - Include discussion on why technique and teamwork matter

Critical Thinking Questions

- Why should soft or finished components never be placed under heavier items?

- What problems can happen if products are stored too high or without support?

- How does proper labelling help during product movement between departments?

- Who is responsible for making sure items are stored correctly after preparation?

- How could you redesign your workstation to make handling and storage safer and faster?

### KT0411 – Storage and Dispatch of Upholstered Frame Components and Complete Products

Purpose of this Topic Element

This topic introduces learners to the best practices for storing and dispatching upholstered frame components and finished furniture products. It focuses on maintaining product quality during the final stage of production, ensuring correct labelling, safe handling, and readiness for transportation or assembly.

At NQF Level 2, learners are expected to understand the importance of organised, clean, and damage-free storage areas, as well as procedures to prepare items for dispatch while protecting the integrity and presentation of the product.

Key Content to Cover

- What Is Storage in the Final Stage?

  - The temporary placement of completed or semi-completed furniture components in designated areas before delivery or assembly

  - Should protect items from dust, impact, moisture, and deformation

- Storage Practices for Upholstered Components

  - Use protective covers (e.g. plastic wrapping, blankets)

  - Store vertically or horizontally depending on shape and weight

  - Use racking systems or spacers to prevent surface contact

  - Keep items labelled with clear identification: product code, batch number, destination

- What Is Dispatch?

  - The preparation and movement of goods for delivery to the customer or next department

  - Includes packing, loading, and transport documentation

- Dispatch Best Practices

  - Conduct final visual inspection before loading

  - Pack items to avoid movement during transit (straps, cushioning, edge guards)

  - Load heavier items first, lighter ones on top or securely upright

  - Complete delivery notes, checklists, or electronic dispatch records

- Risks Associated with Poor Storage and Dispatch

  - Surface damage (scratches, dents, stains)

  - Loss of parts due to incorrect labelling or grouping

  - Delays due to missing items or miscommunication

  - Customer complaints due to damaged products on arrival

Practical Examples

| Task | Correct Action | Potential Risk If Ignored |
| --- | --- | --- |
| Storing covered chair backs | Stand upright with padding between | Fabric may stretch, tear, or gather moisture |
| Preparing set of dining chairs for delivery | Wrap in breathable plastic, label, group securely | Set may be incomplete or arrive with marks |
| Loading into delivery vehicle | Heaviest items at base, secured with straps | Items may fall or shift, causing damage |
| Dispatch documentation | Match checklist to loaded goods and get signature | Confusion or loss during delivery process |

South African Case Study: Incorrect Dispatch Leads to Customer Complaint

Case: In a Pretoria-based upholstery business, a customer ordered ten custom upholstered headboards. During dispatch, two headboards were loaded without labels and sent to the wrong address. Additionally, two others were stacked without sufficient padding and arrived with visible fabric scuffs. The customer returned the items and requested a discount on the order.

Discussion Points:

- What storage and dispatch errors occurred?

- How could proper labelling and handling have prevented this?

- What long-term effect could this have on the business?

Facilitator-Led Activities

- Storage Zone Setup Simulation

  - Learners group and label mock components using appropriate protection and placement techniques

- Dispatch Checklist Role Play

  - In pairs, learners pack and check off a list of items for dispatch using a mock order and product samples

- Label and Match Exercise

  - Learners match unlabelled products to job cards and explore the importance of consistent ID marking and tracking

Critical Thinking Questions

- Why is protective wrapping important for upholstered items in storage?

- What could happen if items are stored too long in an unventilated area?

- How does poor dispatch documentation lead to delivery delays?

- What can you do if you see a finished item being stored incorrectly?

- Why is it important to inspect each item before it is loaded for dispatch?

## Integrated Formative Assessment (Case Study Based)

KM-01-KT04: Quality in the Upholstered Furniture Manufacturing Process
Weight: 20% | NQF Level 2

Case Study: Qhala Upholstery Co.

Qhala Upholstery Co. is a small furniture manufacturer in the Eastern Cape that recently received a large order of customised armchairs for a national hotel chain. During production, several challenges arose:

- Workers in the frame preparation department reported a shortage of quality timber, resulting in the use of some warped and cracked wood.

- One team member in the frame covering area was absent for three days without replacement, leading to rushed upholstery work to meet the delivery deadline.

- Equipment faults were noticed, including a faulty staple gun and a broken foam cutter, but no maintenance request was logged.

- No mid-production inspection was done due to time pressure.

- On final inspection, several chairs had loose arms, sagging cushions, visible fabric creases, and missing product labels. Ten of the thirty units were flagged for rework, delaying dispatch.

- During stacking, three chairs were damaged when piled incorrectly without spacers. This was not reported immediately.

📘 Assessment Tasks

1. Short Answer Questions
(Aligned with IAC0401, IAC0406)
a) Define and differentiate between quality, quality control, and quality assurance.
b) Explain why the final quality check is important before dispatch.

2. Scenario-Based Question
(Aligned with IAC0402, IAC0405)
a) Identify three causes of the quality problems in the case study.
b) For each cause, state one defect or fault it resulted in, and explain how it affected the quality of the product.

3. Extended Response
(Aligned with IAC0403)
a) Discuss the consequences that poor or missing quality management systems had on:

- The frame covering department

- The company

- The wider industry

4. Checklist Activity
(Aligned with IAC0404)
a) List one activity that should have been completed in each of the following stages:

- Pre-production

- Production

- Post-production
b) For each activity, explain how it contributes to maintaining product quality.

5. Risk Reflection Task
(Aligned with IAC0407)
a) Describe two risks that resulted from poor product handling and stacking.
b) Suggest how Qhala Upholstery could prevent this from happening in future.

✅ Model Answers and Marking Memo (Sample)

1a)

- Quality: The standard of a finished product meeting expectations

- Quality Control: Checks done during and after production to find defects

- Quality Assurance: Systems put in place to prevent defects from occurring
(3 marks)

1b)

- The final quality check ensures that only defect-free, approved items are dispatched. It is the last opportunity to catch issues that may have been missed earlier.
(2 marks)

2a)

- Use of warped timber

- Absenteeism with no replacement

- Faulty staple gun not reported
(3 marks)

2b)

- Warped timber → loose frame joints

- Rushed work → sagging cushions

- Faulty stapler → loose or visible staples
(3 marks for linkages, 3 for reasoning)

3a)

- Frame covering department: Increased rework, pressure, low morale

- Company: Delivery delays, additional costs, potential contract loss

- Industry: Harm to reputation of local manufacturing quality
(6 marks)

4a)

- Pre-production: Check wood for warping

- Production: Mid-point inspection of staple alignment

- Post-production: Final visual and function test
(3 marks)

4b)

- These actions help detect faults early, ensure consistency, and reduce customer complaints.
(3 marks)

5a)

- Damage to fabric from stacking without spacers

- Items fell, risking injury and delaying delivery
(2 marks)

5b)

- Use labelled, stable racks; train workers in safe storage methods
(2 marks)

Rubric

| Criteria | Excellent (5) | Competent (3–4) | Not Yet Competent (1–2) |
| --- | --- | --- | --- |
| Defines and differentiates quality, quality control, and quality assurance | Clear, accurate definitions with thorough differentiation and examples | Mostly accurate definitions with some differentiation | Inaccurate or missing definitions and poor differentiation |
| Identifies causes of quality problems and assesses implications | Comprehensive identification with detailed implications for each cause | Most causes identified with general implications provided | Few causes identified, little to no assessment of implications |
| Evaluates consequences of poor/no quality management systems | Insightful evaluation at department, company, and industry levels | Adequate evaluation with limited reference to wider industry | Weak or missing evaluation of consequences |
| Describes and evaluates QC routines and contributions to quality | Detailed description with clear evaluation of each QC stage | Basic description and some evaluation of QC stages | Limited or no reference to QC routines and their impact |
| Assesses defects/faults and analyses causes | Multiple valid defects/faults identified with correct root causes | Some defects/faults identified with partially correct causes | Few or no valid defects/faults identified or analysed |
| Justifies reasons for final quality check procedures | Well-reasoned justification with relevant procedural examples | Justification present but lacks depth or examples | Poor or no justification of final quality check procedures |
| Analyses consequences of poor handling and stacking | Accurate and detailed analysis of risks and potential outcomes | General analysis with minor inaccuracies or omissions | Little understanding of handling and stacking consequences |

## KM-01-KT05: Compressors and Compressed Air

NQF Level 2 | Weight: 10%

Introduction

This knowledge topic introduces learners to the basic concepts, applications, and safety principles of compressors and compressed air systems in the upholstered furniture manufacturing environment. Compressed air is widely used to power tools, clean surfaces, and automate functions in many production facilities. A foundational understanding of how compressors work, how to operate pneumatic equipment safely, and how to maintain air systems is essential for all workers who interact with these systems.

Learners will begin by exploring what compressed air is, how it is generated, and the typical properties and behaviours that make it useful for furniture production. The topic further unpacks common uses of compressed air, such as powering pneumatic staplers, spray guns, and other workshop tools.

A significant focus is placed on identifying and controlling the hazards and risks associated with compressed air use. These include noise, vibration, high pressure, hose failure, and improper tool use. Learners will become familiar with safety protocols, such as setting correct pressure, draining water from airlines, and understanding colour coding systems used in workshops. The correct start-up and shut-down procedures of pneumatic tools will be taught to promote safe, responsible equipment handling.

Through this module, learners will also gain insight into the components of a compressor unit—such as the receiver tank, motor, dryer, and filters—and the importance of regular maintenance. They will learn how to identify common faults and the proper channels for reporting problems, contributing to a proactive and safe workshop culture.

By mastering these skills and concepts, learners will be better equipped to support a productive and hazard-aware work environment where air-powered tools are used safely and efficiently.

Topic Elements Covered

- KT0501: Compressed air

- KT0502: Compressed air generation

- KT0503: Compressed air properties

- KT0504: Compressed air uses

- KT0505: Problem identification and solving

- KT0506: Hazards and risks

- KT0507: Pneumatic tools

Internal Assessment Criteria (IAC) and Weight

- IAC0501: The concept of compressed air is described

- IAC0502: The process of compressed air generation is outlined

- IAC0503: Different compressors are identified and their advantages and disadvantages are outlined

- IAC0504: The properties of compressed air are listed and linked to its uses

- IAC0505: Hazards arising from various air tools, including noise, vibration, fumes, hoses and connectors, are explained

- IAC0506: Safety requirements and procedures are outlined and linked to the hazards they address

- IAC0507: Start-up and shut-down procedures for pneumatic devices are outlined

- IAC0508: Common and critical faults of equipment are listed and described; fault reporting channels are identified

- IAC0509: The importance of setting the correct pressure on pneumatic tools and machines is explained

- IAC0510: The importance of draining water from airlines is explained

- IAC0511: The proper colour coding for airlines is described

- IAC0512: Components of a compressor are identified and the consequences of incorrect air pressure, hose leaks, and water in hoses are assessed

### KT0501 – Compressed Air

Purpose of this Topic Element

This topic introduces learners to the concept of compressed air, its definition, basic behaviour, and role in the furniture upholstery workshop. Compressed air is a crucial energy source used to power pneumatic tools such as staplers, nail guns, and spray systems. Learners must understand what compressed air is, how it is created, and why it is used in manufacturing.

At NQF Level 2, learners are expected to explain the concept of compressed air in simple terms, identify where it is used in their work environment, and recognise why it is valuable in production processes.

Key Content to Cover

- What Is Compressed Air?

  - Compressed air is air that has been forced into a smaller space, increasing its pressure.

  - It is stored under pressure in a compressor tank and used as a source of energy to operate pneumatic tools.

- How Is Compressed Air Used in Upholstery Workshops?

  - It powers tools such as:

    - Pneumatic staplers

    - Air blowers for cleaning

    - Spray guns for finishes

    - Air wrenches or drills (where applicable)

  - It supports efficiency, consistency, and precision in production.

- Why Use Compressed Air Instead of Electricity or Manual Tools?

  - Pneumatic tools are lighter, faster, and often safer than electric equivalents.

  - Compressed air systems can power multiple tools from one source.

  - Air-powered tools are less likely to overheat and offer more control.

- Basic Properties to Emphasise

  - Compressed air stores energy and must be treated with care.

  - It can be dangerous if misused (e.g. blown into skin, used to clean clothing).

  - Air expands rapidly when released, and this can cause injury or equipment damage if not handled correctly.

Practical Examples

| Compressed Air Use | Purpose in Upholstery Workshop |
| --- | --- |
| Pneumatic stapler | Attaches fabric or webbing to the frame quickly |
| Spray gun | Applies paint or finishes evenly across surfaces |
| Air blower | Cleans work surfaces or tools after sanding or cutting |

South African Case Study: Understanding Compressed Air in a Workshop

Case: Sipho is a junior upholsterer at a workshop in Gqeberha. He notices his co-worker using compressed air to blow dust off his clothes, which causes a sudden noise and stinging in his ear. The supervisor immediately stops the worker and explains that compressed air should never be used directly on the body because of its high pressure and potential for internal injury. Sipho learns that even though compressed air feels like wind, it is a powerful tool that requires careful handling.

Discussion Points:

- What mistake did the worker make?

- Why is it dangerous to misuse compressed air?

- What should Sipho do if he sees this happen again?

Facilitator-Led Activities

- Compressed Air Around Us

  - Learners brainstorm common tools or machines in the workshop that use compressed air.

  - Discuss what happens if the pressure is too high or too low.

- Concept Match-Up

  - Learners match terms (e.g. compressed air, pneumatic tool, compressor tank) with definitions or images.

- Safety Scenario Review

  - Present a short incident description involving misuse of compressed air. Ask learners what safety rules were broken and what should have been done.

### KT0502 – Compressed Air Generation

Purpose of this Topic Element

This topic introduces learners to the basic process of how compressed air is generated in an upholstery workshop. Understanding how air is drawn in, compressed, stored, and distributed gives learners foundational knowledge for working safely and effectively with pneumatic systems.

At NQF Level 2, learners are expected to describe how compressed air is produced in simple, practical terms, and recognise the main components of a compressor system, such as the intake, motor, receiver tank, and air lines.

Key Content to Cover

- The Basic Process of Air Compression

  - Air is drawn in from the atmosphere by the compressor.

  - The air is compressed by reducing its volume using a piston or rotary screw.

  - The compressed air is stored in a receiver tank under high pressure.

  - It is then distributed through a network of airlines and hoses to the required tools.

- Key Components of a Basic Compressor System

  - Air Intake: Where outside air enters the compressor

  - Motor: Powers the compression process (electric or petrol)

  - Compressor Pump: Increases the pressure of the air

  - Receiver Tank: Stores the compressed air for use

  - Pressure Gauge: Monitors the air pressure in the tank

  - Drain Valve: Removes water that collects in the tank

  - Hoses and Connectors: Deliver compressed air to the tools

- Single-Stage vs Two-Stage Compression (Simplified)

  - Single-stage compressors compress air once and are suitable for small-scale tasks.

  - Two-stage compressors compress air twice, allowing for higher pressures and larger-scale applications.

- Power Sources

  - Most workshop compressors use electric motors, although some portable compressors use petrol engines.

Practical Examples

| Compressor Part | Function |
| --- | --- |
| Motor | Converts electrical energy to mechanical motion |
| Receiver tank | Holds compressed air until needed |
| Drain valve | Releases built-up moisture inside the tank |
| Air filter | Removes dust and particles from intake air |
| Pressure switch | Turns motor on and off based on tank pressure |

South African Case Study: Faulty Compressor Causes Downtime

Case: At a small upholstery firm in Soweto, the workshop’s compressor frequently shuts off. A technician discovers the air filter is clogged and the drain valve is never opened, causing water build-up. The compressed air supply becomes unreliable, delaying frame covering work. The maintenance team explains that understanding how the compressor works helps workers prevent downtime and maintain consistent production.

Discussion Points:

- What parts of the compressor were not maintained?

- What problems did this cause?

- What basic actions could the workers have taken to avoid this situation?

Facilitator-Led Activities

- Compressor Diagram Labelling

  - Provide a diagram of a basic compressor. Learners label each part and explain its function.

- Process Flow Drawing

  - In pairs, learners draw the steps of how air moves from the atmosphere into the tank and out through the hose.

- Component Matching Game

  - Learners match cards with component names (e.g., pressure gauge, receiver tank) to their functions.

Critical Thinking Questions

- Why is it important to understand how a compressor generates air?

- What could happen if too much moisture builds up in the air tank?

- How does the pressure gauge help keep the system safe?

- What steps should be part of a basic compressor maintenance routine?

- If your air tools lose pressure during use, what compressor parts would you check first?

### KT0503 – Compressed Air Properties

Purpose of this Topic Element

This topic introduces learners to the physical properties of compressed air that influence how it behaves and is used in a workshop setting. Understanding these properties helps learners to work more safely and efficiently with air-powered tools and systems in the furniture manufacturing environment.

At NQF Level 2, learners are expected to list and describe the key properties of compressed air and explain how these properties influence its storage, delivery, and application in pneumatic tools.

Key Content to Cover

- What Are the Properties of Compressed Air?

  - Pressure: Compressed air is stored at high pressure, which allows it to perform work (e.g., driving staples or spraying finishes).

  - Elasticity: Compressed air expands rapidly when released, making it suitable for fast and repeated motion (e.g., in stapling).

  - Temperature: Compressing air generates heat. As air is stored or used, it may cool and form condensation (water vapour).

  - Moisture Content: Atmospheric air contains water vapour, which condenses during compression and must be removed to avoid tool damage.

  - Cleanliness: Compressed air must be clean and free from particles or oil to prevent damage to precision pneumatic tools.

- Why These Properties Matter in Upholstery Workshops

  - High pressure powers fast and efficient stapling or spraying

  - Moisture can cause rusting, hose blockages, or tool malfunction

  - Clean air ensures longer tool life and better product finish

  - Rapid expansion means hoses must be securely connected to avoid accidents

- Impact on Production

  - The performance of pneumatic equipment depends on consistent air pressure

  - Variations in pressure or moisture can lead to inconsistent results, product damage, or downtime

Practical Examples

| Property | Effect in Workshop |
| --- | --- |
| Pressure | Drives staples deeply and accurately into frames |
| Moisture content | Causes internal rusting in air tools if not drained from tank |
| Temperature | Warm air cools and condenses – must be filtered to avoid damage |
| Elasticity | Enables air tools to operate with quick start-stop movements |
| Cleanliness | Prevents contamination of spray finishes and protects tool seals |

South African Case Study: Water in the Airline Causes Spray Gun Failure

Case: At a furniture workshop in Port Elizabeth, a newly painted headboard shows bubbles and surface streaks. Investigation reveals that water had built up in the airline feeding the spray gun. The moisture was not drained from the compressor tank, and the inline dryer was never checked. The painter had no idea that compressed air can carry moisture that affects product quality.

Discussion Points:

- Which property of compressed air was misunderstood?

- What could have prevented this issue?

- How do compressed air properties affect final product quality?

Facilitator-Led Activities

- Properties and Effects Matching

  - Learners match each compressed air property (e.g. pressure, moisture) with its effect on tools or processes.

- Pressure Test Demonstration

  - If safe and possible, show learners how different pressure levels affect the operation of a pneumatic stapler.

- Air System Fault Hunt

  - Using a diagram or image of an air system, learners identify where problems can occur if properties like moisture or pressure are not controlled.

Critical Thinking Questions

- Why must compressed air be drained regularly from the tank?

- How does moisture in compressed air affect upholstery finishes and tools?

- What happens if the air pressure drops below the tool’s required operating level?

- Why is clean, dry air essential for operating a spray gun effectively?

- How do the properties of compressed air make it useful but also risky if mishandled?

Critical Thinking Questions

- What makes compressed air different from normal air?

- Why is compressed air used in furniture production instead of electric tools in some cases?

- What are the risks of treating compressed air like a harmless gas?

- How can we use compressed air responsibly in a shared workshop?

- What would you do if you saw someone using compressed air incorrectly?

### KT0504 – Compressed Air Uses

Purpose of this Topic Element

This topic helps learners identify and understand the various uses of compressed air in the furniture upholstery workshop. By recognising where and how compressed air is used, learners can appreciate its role in improving efficiency, precision, and safety during different production processes.

At NQF Level 2, learners are expected to name tools and tasks that rely on compressed air and explain how air pressure contributes to each activity.

Key Content to Cover

- Common Uses of Compressed Air in Upholstery Workshops

  - Pneumatic staplers and nailers: Used to fasten fabric, webbing, or components to wooden frames

  - Spray guns: Used to apply paint, finishes, or adhesives smoothly and evenly

  - Air blowers: Used to clean dust and debris from workstations, tools, or frame surfaces

  - Inflation tools: Occasionally used for cushions or testing foam pressure in special applications

  - Sanders or drills (in advanced or multi-functional upholstery shops): Powered by air for surface preparation or repair

- Benefits of Using Compressed Air Tools

  - Speed and efficiency: Increases productivity and reduces fatigue

  - Precision: Tools operate consistently at the set pressure

  - Cleaner finish: Especially with spray applications

  - Lower maintenance: Fewer moving electrical parts means reduced wear and overheating

- Considerations When Using Compressed Air

  - Ensure the correct pressure is set for each tool

  - Tools should be connected securely to prevent air leaks

  - Never use air directly on the skin or clothing for cleaning

  - Follow standard operating procedures to avoid accidents

Practical Examples

| Tool or Equipment | Compressed Air Function |
| --- | --- |
| Pneumatic stapler | Drives staples quickly and evenly into the wooden frame |
| Spray gun | Applies smooth finishes to headboards or side panels |
| Air blower | Removes dust before applying fabric or adhesive |
| Upholstery tack gun | Shoots tacks into padding or decorative details |

South African Case Study: Choosing the Right Tool for the Task

Case: In a Johannesburg workshop, a junior upholsterer attempts to use a manual stapler to fasten thick fabric to a dense hardwood frame. The task is slow and uneven, resulting in loose and poorly aligned edges. The supervisor replaces the tool with a pneumatic stapler, powered by compressed air, which completes the job quickly and with clean, professional results. The team then discusses which tools are most effective in different scenarios.

Discussion Points:

- Why was the manual stapler not suitable for the task?

- How did the pneumatic tool improve the outcome?

- What other tools in the workshop benefit from compressed air?

Facilitator-Led Activities

- Tool Identification Chart

  - Learners complete a table showing common air-powered tools, their uses, and required safety checks.

- Demonstration and Reflection

  - Where safe and appropriate, demonstrate the difference between pneumatic and manual stapling.

  - Learners reflect on speed, quality, and physical effort.

- Tool Matching Exercise

  - Provide images of workshop tools and a list of tasks. Learners match each tool to its appropriate function.

Critical Thinking Questions

- What advantages does compressed air offer over electric or manual tools in upholstery?

- How can compressed air help improve product quality?

- What might happen if the wrong pressure is used with a pneumatic stapler?

- Why is it important to use each air tool only for its intended purpose?

- How do you ensure safety when using compressed air tools in the workshop?

### KT0505 – Problem Identification and Solving

Purpose of this Topic Element

This topic equips learners with the ability to identify common faults and solve basic problems related to the use of compressed air and pneumatic tools in the upholstery workshop. It promotes proactive maintenance awareness and encourages responsible tool usage to maintain safety, productivity, and product quality.

At NQF Level 2, learners are expected to observe signs of malfunction, understand the likely causes, and know the correct procedures to report or assist in resolving issues without attempting unauthorised repairs.

Key Content to Cover

- Typical Problems in Compressed Air Systems

  - Air leaks from hoses or fittings

  - Loss of pressure affecting tool performance

  - Blocked air filters or moisture build-up

  - Overheating compressors

  - Oil or water contamination in lines

  - Tools not firing correctly or jamming

- Indicators of Problems

  - Hissing sounds near hose connections (air leak)

  - Reduced stapler or spray gun power

  - Visible water droplets in hoses or around fittings

  - Tools working intermittently or not at all

  - Unusual noises or vibrations from compressor

- Common Causes

  - Loose or damaged fittings and connectors

  - Dirty or clogged air filters

  - Failure to drain moisture from compressor tank

  - Incorrect air pressure settings

  - Worn or damaged tool parts

- Solving the Problems

  - Visual inspection of hose and tool connections

  - Checking the compressor pressure gauge

  - Draining water from the tank and lines

  - Notifying a supervisor or technician for further action

  - Using fault reporting channels as per workshop procedure

- Why Early Identification Matters

  - Prevents equipment damage

  - Maintains safety and prevents accidents

  - Avoids production delays and rework

  - Reduces long-term maintenance costs

Practical Examples

| Problem Observed | Likely Cause | Action to Take |
| --- | --- | --- |
| Stapler fires weakly | Low air pressure or hose leak | Check pressure, inspect hose for damage |
| Spray gun spits paint | Moisture in airline | Drain compressor tank and inspect filters |
| Loud hissing at hose joint | Loose or worn fitting | Turn off air and notify supervisor immediately |
| Compressor cuts off frequently | Overheating or blocked intake | Clean filters, ensure adequate ventilation |

South African Case Study: Missed Warning Signs Lead to Workshop Delay

Case: In an upholstery factory in Durban, the afternoon shift reports that the air stapler has been “acting weak” all day. No one checked the pressure or reported it. By 15h00, the compressor shuts down completely due to overheating caused by clogged filters and moisture build-up. Production halts for the rest of the day. A technician later confirms the issue could have been prevented with early fault identification and timely reporting.

Discussion Points:

- What signs were ignored?

- What simple actions could have prevented the breakdown?

- Why is it important for all workers to report problems immediately?

Facilitator-Led Activities

- Fault Identification Walkthrough

  - Learners examine sample tool setups or images and identify potential issues.

- Problem–Cause–Action Matching

  - Provide cards with problems, causes, and actions. Learners match sets correctly.

- Reporting Simulation

  - Learners role-play reporting a tool fault using a mock form or verbal report to a supervisor.

Critical Thinking Questions

- What should you do first if your pneumatic tool stops working properly?

- Why is it dangerous to continue using a tool with a suspected fault?

- What problems can moisture in the airline cause?

- How does proper fault reporting help protect your teammates?

- What small maintenance habit could help you avoid bigger air system issues?

### KT0506 – Hazards and Risks

Purpose of this Topic Element

This topic introduces learners to the hazards and risks that arise when working with compressed air and pneumatic tools. These include physical risks such as pressure-related injuries, noise exposure, hose failure, vibration, and improper use of tools. Understanding these risks is essential for maintaining a safe and productive working environment.

At NQF Level 2, learners should be able to identify common compressed air hazards and explain the potential risks they pose to themselves, others, and the quality of work.

Key Content to Cover

- Hazards Associated with Compressed Air

  - High-pressure air: Can penetrate skin or cause internal injury if directed at the body

  - Air hoses and connectors: Can whip dangerously if disconnected or damaged

  - Vibration: Long-term use of vibrating tools may cause strain or fatigue

  - Noise: Many pneumatic tools are noisy and can cause hearing damage over time

  - Flying debris: Air blowers can spread dust, staples, or particles at high speed

  - Fumes: Some compressed air systems power spray guns that release paint fumes

  - Overheating compressors: Can become a fire hazard if not ventilated or maintained

- Risks to Safety and Health

  - Personal injury (eye, skin, hearing, respiratory)

  - Workplace accidents due to slipping on hoses or sudden hose release

  - Damage to equipment from incorrect use or pressure settings

  - Long-term health risks from exposure to noise and vibration

- Unsafe Practices to Avoid

  - Blowing air onto clothing or skin

  - Using damaged hoses or connectors

  - Using tools at incorrect pressure

  - Bypassing start-up or shut-down procedures

  - Ignoring strange sounds, leaks, or performance changes

- Legal and Workshop Responsibilities

  - Complying with Occupational Health and Safety (OHS) procedures

  - Wearing PPE: ear protection, goggles, gloves

  - Reporting hazards to a supervisor or technician

  - Participating in safety briefings and training

Practical Examples

| Hazard | Risk | Prevention |
| --- | --- | --- |
| Loose air hose | Whipping action if under pressure | Check fittings and secure before use |
| Loud stapler gun | Hearing damage over time | Wear ear protection (earmuffs or plugs) |
| Air blown at skin | Can cause internal injuries or burst eardrums | Never use compressed air for body cleaning |
| Faulty tool with leak | Air loss, poor performance, or unexpected movement | Report tool immediately for repair or replacement |

South African Case Study: Preventable Accident in the Workshop

Case: In an upholstery plant in Bloemfontein, a new worker uses an air blower to clean fabric dust from his overalls. The high-pressure air causes pain and swelling in his hand. An investigation reveals that he had never been trained in compressed air safety. The workshop then introduces a safety awareness campaign and updates its PPE policy.

Discussion Points:

- What hazard was involved in this incident?

- What risks could have been avoided?

- What role does training and supervision play in accident prevention?

Facilitator-Led Activities

- Hazard Identification Exercise

  - Learners are shown photos or diagrams of compressed air tools. They list potential hazards and explain how to avoid them.

- Risk Scenarios

  - Groups are given short hazard scenarios and must identify:

    - What the hazard is

    - Who is at risk

    - How the risk can be eliminated or reduced

- Safety Poster Project

  - Learners design a simple safety poster that communicates one key risk and how to avoid it using words and visuals.

Critical Thinking Questions

- Why is it dangerous to use compressed air to clean clothing or skin?

- What PPE should you wear when using compressed air tools?

- What could happen if a hose is damaged but still used?

- How can long-term exposure to air tool vibration or noise affect your health?

- What should you do if you notice someone misusing compressed air in the workshop?

### KT0507 – Pneumatic Tools

Purpose of this Topic Element

This topic introduces learners to the most commonly used pneumatic tools in the upholstered furniture industry and provides an overview of their functions, safe handling, and maintenance. Pneumatic tools are powered by compressed air and form a critical part of the production process, contributing to speed, consistency, and accuracy.

At NQF Level 2, learners must be able to identify basic pneumatic tools, describe their use, follow safe operating procedures, and understand the importance of correct pressure settings and regular maintenance.

Key Content to Cover

- Definition of Pneumatic Tools

  - Tools powered by compressed air instead of electricity or manual effort.

  - Receive air through a hose connected to a compressor.

- Common Pneumatic Tools in Upholstery Workshops

  - Staplers and nailers: For attaching fabric, foam, or webbing to frames.

  - Spray guns: For applying paint, glue, or protective finishes.

  - Air blowers: For clearing dust from workspaces or products.

  - Air wrenches or drills: Occasionally used for frame assembly or repairs.

- Advantages of Pneumatic Tools

  - Lightweight and easy to handle

  - Consistent and high-speed operation

  - Reduced user fatigue compared to manual tools

  - Less risk of overheating compared to electric tools

- Safe Use of Pneumatic Tools

  - Always check hose connections before use

  - Do not exceed recommended pressure levels

  - Always wear PPE: gloves, goggles, ear protection

  - Disconnect tools before cleaning or changing parts

  - Use tools only for their intended purpose

  - Follow correct start-up and shut-down procedures

- Basic Maintenance Responsibilities

  - Clean tools regularly to prevent dust build-up

  - Store in dry, safe conditions

  - Report faults or poor performance immediately

  - Drain moisture from lines to prevent damage

Practical Examples

| Tool | Use | Safety Tip |
| --- | --- | --- |
| Pneumatic stapler | Fastens fabric to wooden frame | Keep fingers clear of the stapling area |
| Pneumatic spray gun | Applies smooth paint or finish coatings | Use in ventilated area with face protection |
| Air blower | Clears dust or debris from frame before upholstering | Never aim at people or clothing |
| Air tack gun | Drives tacks into padding or trim | Always check pressure and trigger safety |

South African Case Study: Incorrect Tool Usage Results in Rework

Case: At an upholstery company in Limpopo, a new worker uses a pneumatic spray gun to apply glue to fabric but fails to adjust the pressure. The glue spits out unevenly, causing bubbles and poor adhesion. Several panels need to be redone. The supervisor uses this incident to retrain all staff on tool-specific pressure settings and maintenance.

Discussion Points:

- What caused the tool to perform poorly?

- What safety or operational procedures were missed?

- What lessons can be learned from this mistake?

Facilitator-Led Activities

- Tool Identification and Demonstration

  - Show examples or images of common pneumatic tools. Learners name them and describe their function and safety requirements.

- Operating Procedure Role-Play

  - Learners simulate connecting, using, and disconnecting pneumatic tools safely using real tools (disconnected) or replicas.

- Tool Maintenance Checklist

  - Learners create a simple checklist for daily inspection and maintenance of a selected pneumatic tool.

Critical Thinking Questions

- What are the advantages of using pneumatic staplers over manual staplers in production?

- What could happen if a pneumatic tool is used at the wrong pressure setting?

- Why must tools be disconnected from the air supply before maintenance?

- How do air-powered tools improve both safety and productivity?

- What is your responsibility if you notice a pneumatic tool is leaking or not working properly?

## Integrated Formative Assessment KM-01-KT05: Compressors and Compressed Air

(Weight 10%)

Internal Assessment Criteria: IAC0501 – IAC0512

Case Study: Compressed Air System Incident at Langa Upholstery Works

Langa Upholstery Works in Cape Town has recently expanded its production line to meet new demand. A second-hand air compressor was installed and connected to the existing pneumatic network. Within a week, workers began reporting issues. One team member, Themba, noticed that his air stapler would fire unevenly, sometimes with reduced pressure. Another employee used an air blower to clean his overalls, resulting in a minor skin injury due to high pressure. Meanwhile, moisture in the lines affected the finish of a batch of headboards, leading to customer complaints.

The supervisor discovered that the compressor had not been drained, the filters were clogged, and no colour coding was used on the airlines. The team was not trained in start-up or shut-down procedures, and faults were not being reported correctly.

📘 Assessment Questions (Scenario-Based)

Question 1 – IAC0501
In your own words, describe what compressed air is and how it is used in this scenario.

Question 2 – IAC0502
Outline how compressed air is generated in a standard workshop compressor.

Question 3 – IAC0503
Name two types of compressors and briefly state one advantage and one disadvantage of each.

Question 4 – IAC0504
List two properties of compressed air and explain how each property affected the workplace in the case study.

Question 5 – IAC0505
Identify three hazards that occurred in the case study, related to the use of compressed air or air tools.

Question 6 – IAC0506
Describe two safety procedures that should be followed when working with compressed air, and link each to one of the hazards above.

Question 7 – IAC0507
Outline the correct start-up and shut-down procedure for a pneumatic tool such as a stapler.

Question 8 – IAC0508
List two common faults found in the case study and explain how they should have been reported according to good workplace practice.

Question 9 – IAC0509
Explain why setting the correct pressure on pneumatic tools is important, using Themba’s experience as an example.

Question 10 – IAC0510
Why is it important to drain water out of the compressor airline regularly?

Question 11 – IAC0511
What is airline colour coding, and how could it have prevented confusion in the Langa Upholstery Works scenario?

Question 12 – IAC0512
List four components of a compressor system and describe one possible consequence if each is not maintained properly.

✅ Model Answers

Q1: Compressed air is air that has been squeezed into a smaller space, increasing its pressure. It is used to power tools like staplers and spray guns in the workshop.

Q2: A compressor draws in air through an intake, compresses it with a motor and piston, and stores it in a tank under pressure for use via hoses.

Q3:

- Piston compressor: Advantage – suitable for small workshops. Disadvantage – noisy and slower.

- Rotary screw compressor: Advantage – runs continuously and quietly. Disadvantage – more expensive.

Q4:

- Pressure: Affected the stapler, which lost power when pressure dropped.

- Moisture: Caused finish defects due to water in the lines.

Q5:

- Using air on clothing (injury)

- Moisture in lines (product defect)

- No hose colour coding (confusion and risk)

Q6:

- Always wear PPE (prevents skin injury)

- Drain the tank daily (prevents moisture build-up and product issues)

Q7:

- Start-up: Connect hose, check pressure, test trigger before use

- Shut-down: Turn off compressor, disconnect hose, bleed pressure

Q8:

- Faults: Moisture contamination, inconsistent stapling

- Report: Fill out a fault log or inform supervisor immediately with tool details

Q9: Without correct pressure, tools may misfire or perform poorly, as seen in Themba’s weak stapler.

Q10: Water builds up in the tank due to condensation. If not drained, it can damage tools and affect finishes.

Q11: Colour coding helps identify which hose supplies which pressure level or purpose. It avoids misuse or confusion.

Q12:

- Motor: Overheating if dirty

- Pressure gauge: Unsafe operation if faulty

- Drain valve: Moisture build-up if not opened

- Air filter: Blockage if clogged

📝 Marking Memo

- Allocate 1–2 marks per question depending on complexity.

- Award full marks for correct, complete answers that show understanding in context.

- Partial marks for basic or partially correct answers.

- Use the rubric above to assess the quality of learner responses.

## KM-01-KT06: Drawings for Furniture Manufacturing

NQF Level 2 | Credits: Included in KM-01 | Weight: 5%

Introduction

This knowledge topic introduces learners to the basic principles of reading, interpreting, and applying technical drawings within the upholstered furniture manufacturing process. Drawings are the visual language of production and are critical for ensuring that design intentions are accurately translated into physical furniture components. At NQF Level 2, learners are required to develop the ability to identify different types of drawings, understand the information conveyed through various drawing features, and apply that information correctly to guide the manufacturing process.

The topic begins with an overview of sketches and engineering drawings, highlighting their respective uses and purposes. Learners will explore how isometric views are used to represent three-dimensional shapes and how different lines and line structures are used to convey critical information, such as visible edges, centre lines, and folds. They will also be introduced to hidden detail, which includes features not visible in standard views but essential to accurate construction, such as internal reinforcements or concealed joints.

In addition, learners will gain familiarity with the legends and symbols commonly used in furniture manufacturing drawings, including material identifiers, fastener types, and direction markers. Interpreting these correctly ensures that each work piece is selected and prepared according to the requirements outlined by the technical team or designer.

By the end of this topic, learners will be able to identify work pieces based on technical drawings, interpret essential features, and ensure that the manufacturing process aligns with the intended specifications. They will also understand how to incorporate both visible and hidden details into their work, which is essential for quality, consistency, and safety in the furniture production environment.

### KT0601 – Sketches and Engineering Drawings

Purpose of this Topic Element

This topic introduces learners to sketches and engineering drawings, helping them understand the purpose, differences, and application of each in the context of furniture manufacturing. Learners will explore how both hand-drawn and computer-generated drawings communicate the technical requirements of a product. These skills ensure that products are manufactured to specification, minimise errors, and support efficiency in the workshop.

At NQF Level 2, learners are expected to identify different drawing types, describe their purpose, and use basic interpretation skills to understand what actions or production steps are required.

Key Content to Cover

- What Is a Sketch?

  - A sketch is a freehand or simplified drawing that conveys basic ideas or dimensions.

  - Often used in early design stages or when communicating with team members on the factory floor.

- What Is an Engineering Drawing?

  - A technical drawing created to exact scale using precise dimensions and symbols.

  - Produced either by hand (with instruments) or using CAD software.

  - Used for manufacturing, assembly, and quality control.

  - May include multiple views (front, side, top), exploded views, isometric views, and section views.

- Differences Between Sketches and Engineering Drawings

| Feature | Sketch | Engineering Drawing |
| --- | --- | --- |
| Accuracy | Rough / approximate | Exact measurements and scale |
| Purpose | Idea sharing, concept testing | Manufacturing and construction |
| Format | Hand-drawn, informal | Structured, standardised |
| Use in workshop | Initial planning or annotation | Main reference for production |

- Why They Are Important in Furniture Upholstery

  - Enable upholsterers to understand frame construction, component placement, and overall structure.

  - Help identify the correct parts and materials to be used.

  - Reduce errors and promote quality and consistency.

Practical Examples

- Sketch: A supervisor draws a quick diagram of a curved armrest on the whiteboard to show where extra padding should be added.

- Engineering Drawing: A CAD drawing shows all parts of a lounge chair with exact measurements, joinery techniques, and upholstery sections.

South African Case Study: Misinterpreting a Drawing at Mzansi Lounge Works

Case: At Mzansi Lounge Works in East London, a worker mistook a rough sketch from a designer for a final technical drawing and cut foam to the wrong dimensions. The error delayed the project and led to material waste. After this incident, the factory introduced a visual reference board with examples of approved engineering drawings and marked all sketches clearly as ‘not for cutting’. Workers were trained to understand the differences.

Discussion Questions:

- What misunderstanding led to the error?

- How can clearly labelling sketches vs. drawings prevent confusion?

- Why is it important to check if a drawing is final and approved?

Facilitator-Led Activities

- Drawing Type Sorting Task

  - Learners are given a mix of hand sketches and technical drawings.

  - They must sort them into two groups: sketches and engineering drawings.

  - Discuss the purpose of each and when they are used.

- Label the Drawing

  - Provide a simple engineering drawing.

  - Learners label the title block, scale, views, and measurements.

- Group Discussion

  - Pose the question: What could happen if an upholsterer follows a sketch instead of the official drawing?

- Interpret & Identify

  - Use a simple chair engineering drawing.

  - Learners identify the number of frame parts and any visible fasteners.

Critical Thinking Questions

- When is it appropriate to use a sketch in furniture manufacturing?

- What key information is usually missing in a sketch that is found in an engineering drawing?

- Why should all workshop staff be trained in reading engineering drawings?

- How can confusion between sketches and formal drawings be prevented?

- If you receive a drawing with missing dimensions, what should you do?

### KT0602 – Isometric Views

Purpose of this Topic Element

This topic introduces learners to the concept of isometric views in technical drawing. Isometric views provide a three-dimensional representation of an object that helps the viewer understand its shape, structure, and proportions. These drawings are particularly useful in furniture manufacturing, where components are assembled in three dimensions and spatial awareness is essential.

At NQF Level 2, learners are expected to recognise isometric drawings, understand their purpose, and interpret basic shapes and features represented from an isometric perspective.

Key Content to Cover

- What Is an Isometric View?

  - A method of drawing a 3D object on a 2D surface using three axes: vertical, 30° to the left, and 30° to the right.

  - All dimensions are drawn to scale, and no lines converge (unlike in perspective drawing).

  - Used to show height, width, and depth clearly in one drawing.

- Purpose of Isometric Views in Furniture Manufacturing

  - To visualise how different parts of the furniture fit together.

  - To assist with assembly by showing the orientation of parts.

  - To complement orthographic views (top, front, side) in engineering drawings.

  - Often used in assembly instructions and component breakdown diagrams.

- Key Characteristics

  - Shows three sides of an object simultaneously.

  - All lines follow three main axes: vertical and two 30° angles from the horizontal.

  - All dimensions remain true to scale.

  - Often shaded or labelled for clarity.

Practical Examples

| Furniture Example | Use of Isometric View |
| --- | --- |
| Upholstered ottoman | Shows cushion, base, legs, and fabric wrapping in one drawing |
| Headboard with storage cavity | Reveals internal shelf design and access panel placement |
| Sectional lounge unit | Illustrates how multiple seating elements connect at angles |

South African Case Study: Using Isometric Views for Accurate Assembly

Case: At Bayview Upholstery in Gqeberha, an apprentice was asked to assemble a lounge chair from parts. The engineering drawing provided included only front and side views. The apprentice misaligned the legs and support braces. A revised drawing with an isometric view was issued, and the assembly was completed correctly. The supervisor then updated workshop protocols to always include isometric views in part assembly packages.

Discussion Points:

- What information did the isometric view add that the 2D views did not?

- How can 3D visualisation improve accuracy in assembly?

- Should isometric drawings be included with all product plans?

Facilitator-Led Activities

- Identify the View

  - Provide learners with a set of mixed drawings (front view, top view, isometric).

  - Learners must identify the isometric view and explain what it shows.

- Complete the Drawing

  - Provide a simple orthographic drawing of a rectangular box.

  - Learners draw an isometric version using grid paper.

- Isometric Spot-the-Difference

  - Learners are shown two similar isometric views with a slight variation in dimensions or parts.

  - They identify and explain the differences.

### KT0603 – Lines Used

Purpose of this Topic Element

This topic introduces learners to the types of lines used in technical and engineering drawings and how these lines convey meaning. Lines are not drawn at random — each line has a specific purpose and standard interpretation in technical communication. Understanding these visual conventions is essential for accurately reading and interpreting drawings in the upholstered furniture manufacturing process.

At NQF Level 2, learners should be able to identify different line types, understand what each line represents, and interpret line-based information correctly when preparing workpieces or components.

Key Content to Cover

- Types of Lines and Their Uses

| Line Type | Description | Purpose in Furniture Drawing |
| --- | --- | --- |
| Continuous thick | Solid, thick line | Represents visible edges and outlines of objects |
| Continuous thin | Solid, thin line | Used for dimension lines, projection lines, leader lines |
| Dashed (short dashes) | Evenly spaced short dashes | Shows hidden features not visible in the current view |
| Chain (long dash–short dash) | Alternating long and short dashes | Represents centre lines or symmetry |
| Freehand lines | Often used in sketches | Indicates irregular shapes or design concepts |

- Interpreting Line Meaning

  - Learners should understand that different lines communicate different types of information.

  - Incorrect interpretation may lead to cutting the wrong size, ignoring hidden components, or misaligning joints.

- Standards in Technical Drawing

  - Line styles follow international drawing standards, such as SANS or ISO, to ensure clarity.

  - These conventions allow drawings to be read by anyone, regardless of who created them.

- Application to Upholstered Furniture

  - Learners may encounter continuous thick lines for frame outlines, dashed lines for internal braces, and thin lines for cutting or folding instructions.

  - Reading line types correctly ensures precise cutting, accurate assembly, and safe component integration.

Practical Examples

- Dashed lines on a chair seat drawing show where internal wooden supports are located.

- Centre lines are used on circular armrest drawings to mark symmetry before drilling.

- Thin lines with arrows represent dimensions for foam thickness and panel lengths.

South African Case Study: Misreading a Line at a Cape Town Workshop

Case: In a factory in Cape Town, an upholsterer misread a dashed line on an armrest drawing as a cut line. The result was a structural failure when the chair was assembled. Upon review, it was found that the line indicated a hidden support, not an external cut. The workshop responded by reinforcing training on line types and their meanings to prevent such incidents in future.

Discussion Points:

- What mistake occurred and why?

- How could this error have been prevented?

- Why is line type knowledge important for workshop communication?

Facilitator-Led Activities

- Line Type Identification Chart

  - Learners are given a chart with different lines and must match each to its name and function.

- Mark the Line Exercise

  - On a basic furniture drawing, learners are asked to identify or trace different line types using coloured pencils.

- Workshop Drawing Walkthrough

  - Learners bring an actual drawing (or are provided one) and identify each type of line used, explaining what it represents.

Critical Thinking Questions

- Why is it important to use different lines to show different information on a drawing?

- What could happen if a dashed line is mistaken for a cutting line?

- How do standard line types improve safety and accuracy in the workplace?

- When reviewing a drawing, what should you do if you are unsure what a line represents?

- How can understanding line types help you communicate better with others in the workshop?

Bottom of Form

Critical Thinking Questions

- What advantages do isometric views provide that sketches or flat drawings do not?

- How would a missing isometric view affect your understanding of how to assemble a piece of furniture?

- Why do you think most manufacturers include isometric views in assembly manuals?

- How can developing the ability to visualise objects in 3D help you in your work as an upholsterer?

- What is the difference between an isometric view and a perspective drawing?

### KT0604 – Hidden Detail

Purpose of this Topic Element

This topic equips learners with the ability to identify and interpret hidden details in technical drawings. Hidden details refer to internal features or components of a product that are not visible in a standard external view but are crucial for accurate construction and assembly. In furniture manufacturing, understanding hidden detail ensures that structural integrity, functionality, and aesthetics are maintained throughout the production process.

At NQF Level 2, learners are expected to recognise hidden detail indicators, understand their purpose, and describe the correct manufacturing actions that must be taken based on this information.

Key Content to Cover

- Definition of Hidden Detail

  - Refers to parts of the object that are not visible from the outside but must be accounted for during construction.

  - These details are represented using dashed lines in engineering drawings.

- Examples of Hidden Details in Upholstered Furniture

  - Internal wooden or metal braces

  - Foam cavities or shaped cut-outs

  - Stapled joint reinforcements not visible once covered

  - Slots for dowels or fittings inside solid components

- How Hidden Detail Is Represented

  - Dashed lines (short, evenly spaced) are used to show features such as internal holes, grooves, or supports.

  - The drawing may also include sectional views to reveal internal components more clearly.

- Importance of Recognising Hidden Detail

  - Prevents incorrect cutting or assembly

  - Ensures internal support is in place before upholstery begins

  - Avoids costly rework or safety failures

  - Supports compliance with quality standards

- Link to Manufacturing Actions

  - Correct tools must be used (e.g., drilling in the correct hidden location)

  - Material preparation must account for internal joinery

  - Assembly processes must follow the drawing before covering or finishing

Practical Examples

| Furniture Item | Hidden Detail Example | Action Required |
| --- | --- | --- |
| Upholstered bench | Internal plywood cross-brace shown as dashed lines | Drill and attach brace before foam is applied |
| Armrest | Dowel joint location shown in hidden detail | Ensure holes are drilled accurately before glue |
| Padded headboard | Concealed MDF board support frame | Frame must be built to spec before padding |

South African Case Study: Ignoring Hidden Detail Leads to Frame Collapse

Case: At a Johannesburg upholstery firm, a frame assembler ignored the dashed lines on a technical drawing, assuming they were decorative. The frame was constructed without the internal diagonal braces. After the chair was upholstered, it collapsed during final inspection. The supervisor revised the training manual and reminded staff that hidden details are functional, not optional.

Discussion Points:

- Why did the frame collapse?

- How could the assembler have recognised the importance of dashed lines?

- What steps could have prevented this error?

Facilitator-Led Activities

- Hidden Detail Spotting

  - Provide learners with sample drawings. Ask them to identify all hidden features and label what each might represent.

- Drawing-to-Action Role Play

  - Learners are given a drawing showing hidden internal joints and must describe the manufacturing steps they would follow to prepare the part.

- Compare and Reflect

  - Show two furniture frame drawings: one with hidden detail, one without. Discuss how each would affect the final product.

Critical Thinking Questions

- Why is it important to include hidden detail in a furniture drawing?

- What might happen if hidden braces or joints are left out during assembly?

- How do dashed lines differ from visible outline lines in meaning?

- What actions should you take when a drawing shows hidden details inside a component?

- What should you do if a hidden detail is unclear or seems missing in a drawing?

### KT0605 – Legends and Symbols

Purpose of this Topic Element

This topic introduces learners to legends and symbols used in furniture manufacturing drawings. Symbols serve as a universal visual language that communicates complex technical information in a simplified way. The legend is the drawing’s reference table, explaining the meaning of each symbol and abbreviation. Understanding these elements allows workers to correctly identify materials, fasteners, finishes, and actions, without needing extensive written instructions.

At NQF Level 2, learners are expected to read and interpret basic symbols and legends, and use that information to prepare materials or perform tasks as outlined in the drawing.

Key Content to Cover

- What Is a Legend?

  - A table or key located on the drawing sheet.

  - Contains explanations of all the symbols, abbreviations, or codes used.

  - Often includes information such as:

    - Material codes (e.g., MDF, HR foam)

    - Fastener types (e.g., S = staples, N = nails)

    - Finish specifications

    - Tool references

- What Are Symbols in Technical Drawings?

  - Simplified icons or graphic indicators representing specific elements.

  - Used to save space and ensure clarity.

  - Examples in upholstery:

    - Arrows showing direction of grain or fabric

    - Staples or fasteners

    - Foam thickness indicators

    - Upholstery seam types

    - Labels for cutting guides or edge finishing

- Standard Drawing Symbols

  - ISO and SANS standards may influence symbol usage.

  - Workshops may also develop custom symbols relevant to their internal processes.

- Why Legends and Symbols Are Important

  - Prevent misinterpretation of details

  - Speed up reading and understanding

  - Ensure standardisation across departments

  - Allow for accurate material preparation and assembly

Practical Examples

| Symbol or Abbreviation | Meaning | Action |
| --- | --- | --- |
| MDF | Medium-density fibreboard | Select the correct board from stock |
| ➤ | Direction of staple or tack | Align the gun to fire in that direction |
| 25mm HRF | 25 mm high-resilience foam | Cut foam of correct thickness |
| ✖ | Indicates drilling or punching point | Prepare tool to create hole at that mark |
| CL | Centre line | Align materials symmetrically |

South African Case Study: Misreading a Legend Causes Incorrect Material Use

Case: At an upholstery plant in George, a new employee read “25mm HRF” in a drawing legend but cut 25mm low-density foam instead. The finished batch of cushions sagged under pressure, leading to product returns. The manager implemented a legend reference chart in the foam cutting section to avoid confusion in future.

Discussion Points:

- What was misunderstood in this case?

- How could referring to the legend have helped the employee?

- How does a standardised system reduce errors?

Facilitator-Led Activities

- Symbol Matching Exercise

  - Provide a list of symbols or abbreviations and ask learners to match each to its meaning and appropriate manufacturing action.

- Legend Building Activity

  - Learners are given a sample furniture drawing with missing legend. They must construct a legend using symbols seen on the drawing.

- Reading and Reacting

  - Present a full drawing with a legend. Ask learners to interpret it and list the tools, materials, and processes they will need.

Critical Thinking Questions

- What could happen if a worker ignores or misreads the legend on a drawing?

- How do symbols make communication more efficient in a multi-language workshop environment?

- Why should legends be kept consistent across all drawings in a company?

- What would you do if you encounter a symbol you do not recognise on a drawing?

- How can you ensure you are using the correct materials and tools based on the drawing legend?

Top of Form

## Integrated Formative Assessment – KM-01-KT06

Assessment Instruments Used

- Oral questioning

- Short written answers

- Drawing analysis activity

- Symbol and legend matching exercise

- Practical interpretation scenario

Scenario: A New Drawing Arrives in the Upholstery Workshop

You are a trainee upholsterer at Thoko's Upholstery Studio. Your supervisor gives you a new engineering drawing of an ottoman that includes multiple views, a legend, and symbols. You must use this drawing to prepare your workpieces accurately before assembly and upholstery begin.

Assessment Questions by Internal Assessment Criteria

1. IAC0601 – Sketches and engineering drawings are identified according to type and use

Question: Look at the two drawings provided (one freehand and one with dimensions and title block).
Task: Identify which is a sketch and which is an engineering drawing. Explain one use for each.

2. IAC0602 – Engineering drawings are correctly interpreted and the relevant actions and processes are identified

Question: Based on the engineering drawing of the ottoman, list three tasks you need to complete before starting upholstery.

3. IAC0603 – Line structure and dimensions are correctly identified and their meanings described

Task: Look at the line types on the drawing (solid, dashed, centre line).
Activity: Match each line type to its meaning and purpose.

4. IAC0604 – The interpretation is done methodically to ensure that all the important details are incorporated into the manufacturing process

Question: Using the full engineering drawing, describe the steps you would follow to make sure no important detail is missed before you cut or assemble any part.

5. IAC0605 – Work pieces to be used are identified based on the engineering drawing

Task: Refer to the bill of materials and views. List all wood, foam, and fasteners required for the ottoman base.

6. IAC0606 – Hidden details are listed and the actions to be taken are outlined

Question: Identify two hidden details shown in the drawing using dashed lines. What actions must you take based on this information?

✅ Model Answers

Q1 – IAC0601

- Sketch: The freehand drawing is a sketch, used for initial idea communication.

- Engineering Drawing: The second is an engineering drawing, used for manufacturing with precise dimensions and technical information.

Q2 – IAC0602

- Mark out frame dimensions on wood stock

- Cut foam padding to match drawing sizes

- Pre-drill holes for corner joints as shown in plan

Q3 – IAC0603

- Solid line = visible outer edges

- Dashed line = hidden detail (internal braces)

- Centre line = marks symmetry or centre of holes

Q4 – IAC0604

- Review all views: top, side, front

- Read the legend to understand symbols

- Check for any dashed lines showing hidden details

- List materials needed based on drawing

- Confirm dimensions before marking or cutting

Q5 – IAC0605

- Wood: 18mm MDF for frame

- Foam: 25mm high-resilience foam for seat top

- Fasteners: Staples (S) and wood screws (WS)

Q6 – IAC0606

- Hidden detail 1: Internal cross brace – action: must be installed before foam is glued

- Hidden detail 2: Recessed screw fixing – action: drill correct depth hole before assembly

📝 Marking Memo

| Question | IAC | Max Marks | Key Answer Points |
| --- | --- | --- | --- |
| Q1 | IAC0601 | 4 | Correct identification and basic purpose of each |
| Q2 | IAC0602 | 4 | Lists at least three correct processes |
| Q3 | IAC0603 | 4 | Correct matching of each line with its function |
| Q4 | IAC0604 | 4 | Logical, methodical steps showing planning and awareness |
| Q5 | IAC0605 | 4 | Identifies correct components based on drawing |
| Q6 | IAC0606 | 4 | Recognises hidden detail and links it to action required |
| Total |  | 24 marks |  |

📊 Rubric – Integrated Formative Assessment: KM-01-KT06

| Performance Level | Descriptor |
| --- | --- |
| Excellent (5) | Accurately identifies all drawing types, line meanings, hidden details, and material requirements. Follows correct steps methodically with insight. |
| Competent (3–4) | Identifies most drawing features and actions with minor errors. Demonstrates general understanding of drawing interpretation and its applications. |
| Not Yet Competent (1–2) | Struggles to differentiate drawing types or interpret symbols. Misses key steps or misreads drawing content, leading to incorrect conclusions. |

## KM-01-KT07: Productivity and Efficiency in the Production of Upholstered Furniture

NQF Level 2 | Weight: 10%

Introduction

This knowledge topic provides learners with a foundational understanding of productivity and efficiency in the context of upholstered furniture manufacturing. In a competitive and quality-driven industry, productivity does not simply refer to working faster, but to working smarter, consistently, and accurately, while using resources responsibly. This includes managing time, tools, materials, and tasks in a manner that contributes to high-quality outputs and minimises waste.

Learners are introduced to the key principles that govern efficient production in a workshop environment. These include understanding the flow of work, the impact of individual and team performance, and the need for maintaining consistent output without compromising quality. Emphasis is also placed on recognising time-wasters and productivity barriers, such as disorganised workspaces, equipment failures, miscommunication, or absenteeism.

A major focus of the topic is on the role of the individual worker in achieving overall production goals. Learners are encouraged to reflect on how their behaviour, punctuality, and attention to detail can influence the efficiency of the entire production line. They will also explore strategies for improving workflow, including following standard operating procedures, preparing tools and materials in advance, and reporting faults or delays promptly.

By the end of this topic, learners will understand how productivity and efficiency directly affect cost, quality, delivery timelines, and team morale, and will be equipped with the knowledge to contribute meaningfully to a smooth and successful production process within the upholstered furniture industry.

### KT0701 – Productivity

(Aligned to KM-01-KT07: Productivity and Efficiency in the Production of Upholstered Furniture | Weight: 10%)

Purpose of this Topic Element

This topic introduces learners to the concept of productivity within the upholstered furniture manufacturing environment. It focuses on understanding productivity as a measurable output and a critical factor in workshop and business performance. Learners are expected to understand how productivity impacts operations and how it relates to their role in the production process.

At NQF Level 2, learners must be able to define productivity and identify its significance in relation to the work they perform, the expectations set by supervisors, and the delivery requirements of the production team.

Key Content to Cover

- Definition of Productivity

  - Productivity refers to the amount of output (products made, tasks completed) relative to the input (time, labour, and resources used).

  - Productivity is measured by how efficiently materials, time, and skills are used in a manufacturing environment.

- Importance of Productivity

  - Ensures that production targets are met.

  - Contributes to on-time delivery of finished goods.

  - Helps control costs and reduce waste.

  - Supports the reputation of the company through consistent performance.

- How Productivity Is Affected

  - By absenteeism or tardiness

  - By equipment failures or lack of maintenance

  - By poor material handling or disorganisation

  - By miscommunication or unclear instructions

- Worker’s Role in Maintaining Productivity

  - Arriving on time and ready to work

  - Preparing tools and materials before starting

  - Following instructions and standard procedures

  - Reporting any delays or defects immediately

Practical Examples

| Scenario | Productivity Impact |
| --- | --- |
| A worker finishes 10 chairs instead of 8 | Positive – output increased without errors or quality loss |
| A stapler breaks and no replacement is found | Negative – production halts, impacting output and timelines |
| A team member is absent for two days | Negative – more strain on others, fewer tasks completed |

Facilitator-Led Activities

- Definition Matching

  - Provide learners with definitions and key terms related to productivity. They must match the correct terms to definitions (e.g. productivity, input, output).

- Case Comparison

  - Present two short case examples: one showing high productivity and one showing low productivity. Learners identify the differences and suggest causes.

- Workshop Reflection

  - Learners reflect on one task they perform and discuss how they could maintain or improve productivity.

Critical Thinking Questions

- Why is productivity important in furniture manufacturing?

- How can poor planning affect productivity in the workshop?

- What small actions can you take each day to help maintain or improve productivity?

- What should you do if a delay or fault is slowing down your work?

- How does teamwork influence productivity in a production environment?

### KT0702 – Efficiency

(Aligned to KM-01-KT07 | Weight: 10%)

Purpose of this Topic Element

This topic introduces learners to the concept of efficiency in the upholstered furniture manufacturing process. Efficiency refers to the effective use of resources, time, and effort to complete a task or produce an item without unnecessary waste or duplication. Understanding efficiency supports the learner in contributing to streamlined operations and achieving workplace targets.

At NQF Level 2, learners are expected to understand what efficiency means in their day-to-day work and how their actions can either support or undermine efficient workflow in a production setting.

Key Content to Cover

- Definition of Efficiency

  - Efficiency is the ability to complete tasks using the least amount of resources and time, while still meeting quality and production standards.

  - It involves doing things right the first time and avoiding unnecessary rework, delays, or wastage.

- Examples of Efficiency in Upholstery Workshops

  - Using time effectively by preparing tools and materials before starting work

  - Working in a tidy and organised space

  - Following standard operating procedures

  - Maintaining tools and equipment to avoid breakdowns

- Factors That Affect Efficiency

  - Lack of planning or preparation

  - Poor communication

  - Untidy or cluttered work areas

  - Repeating work due to avoidable mistakes

- Improving Efficiency

  - Follow instructions carefully the first time

  - Ask for clarification when unsure

  - Keep workstations clean and organised

  - Use tools and materials as directed

Practical Examples

| Behaviour | Efficiency Outcome |
| --- | --- |
| Worker lays out all materials before start | Saves time and avoids delays |
| Cutting is done based on clear measurements | Minimises rework and wastage |
| Worker looks for tools repeatedly | Wastes time, reduces output |

Facilitator-Led Activities

- Efficiency Spot Check

  - Learners are presented with two workshop scenarios: one efficient and one inefficient. They identify the efficient actions and explain their benefit.

- Personal Efficiency Planning

  - Learners describe one daily task and list two ways to make it more efficient.

- Team Efficiency Game

  - Small teams are timed to perform a basic setup task (e.g. laying out materials for a job). The facilitator debriefs the group on what actions improved or reduced efficiency.

Critical Thinking Questions

- What is the difference between being productive and being efficient?

- What are some common habits that reduce efficiency in the workshop?

- How can keeping your workstation organised help improve efficiency?

- Why is it important to follow instructions the first time?

- What should you do if you notice a co-worker doing something inefficiently?

### KT0703 – Waste Management

(Aligned to KM-01-KT07 | Weight: 10%)

Purpose of this Topic Element

This topic introduces learners to the importance of waste management in the upholstered furniture manufacturing environment. It highlights how effective waste management practices contribute to productivity, cost reduction, environmental responsibility, and overall workplace efficiency.

At NQF Level 2, learners are expected to identify different types of waste, understand their sources, and apply appropriate methods for handling, reducing, and disposing of waste materials in the upholstery workshop.

Key Content to Cover

- Definition of Waste in Manufacturing

  - Any material, time, or resource that does not add value to the finished product.

  - Includes off-cuts, damaged materials, overproduction, packaging waste, and inefficient use of supplies.

- Types of Waste in Upholstery Workshops

  - Material waste: fabric, foam, wood, staples, adhesives, plastic coverings

  - Time waste: delays due to poor planning, searching for tools or materials

  - Energy waste: leaving compressors running, misusing machinery

  - Environmental waste: excessive packaging, poor disposal of chemicals or off-cuts

- Common Causes of Waste

  - Inaccurate measurements or poor cutting

  - Lack of planning or supervision

  - Poor training or incorrect use of tools

  - Over ordering or incorrect stock handling

- Principles of Waste Management

  - Reduce: minimise waste generation by accurate planning and use of resources

  - Reuse: repurpose suitable off-cuts or packaging for other tasks

  - Recycle: separate and send eligible waste materials for recycling

  - Dispose responsibly: ensure hazardous materials are disposed of in line with workplace policy

- Workplace Responsibilities

  - Store materials correctly to avoid damage

  - Use cutting plans and templates

  - Report and record waste

  - Follow workshop procedures for disposal

Practical Examples

| Scenario | Waste Management Action |
| --- | --- |
| Foam off-cuts are scattered on the floor | Collect and store for reuse or proper disposal |
| A worker cuts fabric without using a template | Increases material waste – use templates or cutting guides |
| Spray adhesive cans disposed in general waste | Hazardous – must follow approved disposal procedure |

Facilitator-Led Activities

- Waste Spotting Walkabout

  - Learners visit a mock or real workshop and identify areas where waste is being generated or unmanaged.

- Waste Sorting Task

  - Present different types of workshop waste (wood, fabric, glue cans, plastic). Learners sort into reduce, reuse, recycle, or dispose categories.

- Waste Impact Brainstorm

  - Learners discuss what could happen if waste is not managed: cost impact, health risks, environmental concerns, and lost productivity.

Critical Thinking Questions

- What types of waste have you seen in a furniture manufacturing workshop?

- Why is it important to reduce waste in the upholstery production process?

- How can poor waste management affect productivity and efficiency?

- What are some ways to reduce material waste when cutting fabric or foam?

- How can you take responsibility for managing waste in your workstation?

### KT0704 – Quality of Raw Material and Consumables

(Aligned to KM-01-KT07: Productivity and Efficiency in the Production of Upholstered Furniture | Weight: 10%)

Purpose of this Topic Element

This topic addresses the importance of ensuring that raw materials and consumables used in the upholstered furniture manufacturing process are of acceptable quality standards. Learners will understand how the quality of these inputs affects productivity, efficiency, final product quality, and customer satisfaction.

At NQF Level 2, learners are expected to identify different types of raw materials and consumables, assess their suitability, and follow procedures for reporting faults or defects.

Key Content to Cover

- Definition of Raw Materials and Consumables

  - Raw materials: Timber, foam, fabric, webbing, springs, frames.

  - Consumables: Glue, staples, tacks, thread, dust covers.

- Characteristics of Quality Materials

  - Free from visible damage, contamination, moisture, or deformities.

  - Match the specifications on the job card or design.

  - Compatible with the tools, adhesives, and finishes being used.

- Consequences of Poor-Quality Materials

  - Increased waste due to rework or rejection.

  - Reduced lifespan or performance of the finished product.

  - Delays in production due to sorting or replacing materials.

  - Negative impact on customer satisfaction and brand reputation.

- Quality Checks

  - Visual inspection for damage, defects, and contamination.

  - Checking foam density, fabric batch numbers, and frame dimensions.

  - Verifying that the correct type and quantity have been issued.

- Reporting Faulty Materials

  - Follow workplace procedures to log and isolate defective materials.

  - Notify supervisors immediately.

  - Prevent use of unsuitable items in production.

Practical Examples

| Material/Consumable | Potential Fault | Required Action |
| --- | --- | --- |
| Foam block | Crushed edges or uneven density | Return to supplier or isolate from production |
| Fabric roll | Faded patch or misprint | Cut around if minor or report if significant |
| Glue canister | Expired or incorrect type | Remove from use and request replacement |
| Wood frame | Cracks, warping, loose joints | Tag for repair or replacement |

Facilitator-Led Activities

- Material Inspection Table

  - Learners examine physical samples of raw materials and identify quality vs. faulty examples.

- Fault Reporting Role-Play

  - Learners practise describing a material fault and reporting it according to a sample procedure.

- Checklist Comparison

  - Compare a defective foam batch against a quality checklist and highlight discrepancies.

Critical Thinking Questions

- Why is it important to check raw materials before using them in production?

- What could happen if poor-quality materials are used in a finished furniture product?

- How can you tell if a consumable is no longer suitable for use?

- What are the correct steps to take if you find a defect in a batch of materials?

- How does quality control of materials link to productivity and efficiency in the workshop?

### KT0705 – Raw Material and Consumables Faults and Defects

(Aligned to KM-01-KT07 | NQF Level 2 | Weight: 10%)

Purpose of this Topic Element

This topic introduces learners to the importance of identifying and responding to faults and defects in raw materials and consumables used in the upholstered furniture manufacturing process. Detecting problems early helps to maintain production quality, reduce waste, and ensure that materials meet job requirements.

At NQF Level 2, learners are expected to recognise visible and functional faults in common materials and consumables, follow correct procedures for reporting them, and understand how such faults affect productivity and efficiency.

Key Content to Cover

- Definition of Faults and Defects

  - Faults: Errors or imperfections that affect the performance or appearance of materials (e.g. discolouration, tears, cracks).

  - Defects: Structural or manufacturing problems that make a material or item unfit for use (e.g. warped wood, expired glue, loose fabric weave).

- Common Raw Material Faults

  - Warped or cracked timber frames

  - Damaged or misaligned foam

  - Faded, stained, or torn upholstery fabric

  - Contaminated or inconsistent fibre padding

- Common Consumable Defects

  - Blocked or dried-out glue nozzles

  - Bent or broken staples or tacks

  - Expired or separated adhesive contents

  - Faulty stitching threads or jammed reels

- Impact on Productivity and Efficiency

  - Causes delays due to replacement or rework

  - Leads to poor-quality outputs and customer complaints

  - Increases material waste and costs

  - Slows down workflow and affects delivery timelines

- Procedures for Managing Faults and Defects

  - Inspect all materials before use

  - Follow company procedure for tagging and isolating faulty items

  - Notify supervisor or quality control immediately

  - Record defects where required for stock or supplier follow-up

Practical Examples

| Material or Consumable | Example Fault or Defect | Impact | Action |
| --- | --- | --- | --- |
| Foam sheet | Torn edges or collapsed area | Uneven cushioning in final product | Report and request replacement |
| Fabric roll | Dye variation or print misalignment | Inconsistent appearance in upholstery | Cut around defect or replace roll |
| Glue canister | Expired or hardened glue | Bonding failure during assembly | Remove from use and replace |
| Staples | Rusted or bent | Jamming staple gun; weak fastening | Discard and select new batch |

Facilitator-Led Activities

- Material Inspection Exercise
Learners examine samples of raw materials and consumables to identify visible faults and defects, and describe their effect on the final product.

- Fault Reporting Role-Play
Learners practise reporting a discovered defect using a simulated company procedure (verbal and written).

- Sorting Task
Learners sort a mixed batch of acceptable and defective items into “use,” “rework,” and “reject” categories, justifying their choices.

Critical Thinking Questions

- Why is it important to inspect materials and consumables before starting production?

- What could happen if a damaged fabric roll is used without checking?

- How do defects in consumables such as glue or staples affect production flow?

- What is the correct action to take when you find a defect in a foam block?

- How does identifying faults early contribute to overall efficiency in the workshop?

### KT0706 – Product Quality of the Frame Preparation and Upholstery Processes and Defects

(Aligned to KM-01-KT07 | NQF Level 2 | Weight: 10%)

Purpose of this Topic Element

This topic introduces learners to the concept of product quality in the frame preparation and upholstery processes, and how defects arising during these stages can impact the finished furniture product. Learners will develop an understanding of how to identify quality issues, their causes, and how to prevent or address them as part of maintaining high production standards.

At NQF Level 2, learners are expected to recognise typical defects that can occur during frame preparation and upholstery, understand how these affect product quality, and apply workplace procedures to deal with such defects appropriately.

Key Content to Cover

- Definition of Product Quality

  - Meeting the design specifications, finish standards, and structural requirements of the upholstered item.

  - Ensuring uniformity, durability, and safety in the final product.

- Common Defects in Frame Preparation

  - Misaligned joints

  - Incomplete fastening or gluing

  - Uneven surfaces or sharp edges

  - Warped or cracked frame elements

- Common Defects in Upholstery Processes

  - Wrinkled or loose fabric covers

  - Uneven padding or poor foam alignment

  - Exposed staples or tacks

  - Poor seam finishes or visible stitching errors

- Causes of Defects

  - Use of faulty or unsuitable materials

  - Incorrect measurements or cutting

  - Inadequate training or lack of attention to detail

  - Poor maintenance of equipment or incorrect tools used

- Effects of Defects on Product Quality

  - Compromised structural integrity

  - Negative customer perception and reduced satisfaction

  - Increased rework or product rejection

  - Delay in delivery and higher production costs

- Preventing and Addressing Defects

  - Quality checks at each production stage

  - Following standard operating procedures and job specifications

  - Reporting defects as per workshop policy

  - Using templates and guides to ensure accuracy

Practical Examples

| Stage | Possible Defect | Impact | Preventive Action |
| --- | --- | --- | --- |
| Frame preparation | Loose corner joints | Weak product structure | Ensure joints are clamped, glued, and fixed |
| Upholstery fitting | Sagging fabric or uneven seams | Poor aesthetic quality | Use tension guides and inspect before stapling |
| Foam application | Bulges or flat spots | Discomfort and visual unevenness | Check foam fit before fixing cover |

Facilitator-Led Activities

- Defect Identification Table

  - Present learners with images or physical samples of defective frames and upholstery work. Learners identify the defect and link it to its production stage.

- Quality Check Role-Play

  - Learners simulate inspecting an item for defects before it moves to the next department and practise reporting findings clearly.

- Cause-and-Effect Mapping

  - Learners list common defects and brainstorm potential causes and prevention methods using a simplified fishbone diagram.

Critical Thinking Questions

- Why is it important to check the quality of a product at each stage of production?

- What could happen if a small defect in frame preparation is ignored?

- What are some signs of poor-quality workmanship during upholstery?

- How can following procedures help reduce the chance of defects?

- What should you do if you find a defect in a product during final inspection?

## Integrated Formative Assessment – KM-01-KT07

Weight: 10%
Assessment Type: Integrated Formative Assessment
Instruments Used:

- Structured Questions

- Matching Task

- Scenario-Based Questions

- Practical Interpretation Task

- Short Motivation

Assessment Task 1: Structured Questions

Instrument: Written Response
Covers: IAC0701, IAC0702, IAC0705
Instructions: Answer the questions in full sentences.

- Explain why planning a job in the upholstery production process is important in avoiding delays.

- Discuss what productivity means in a manufacturing environment.

- Why is minimising waste important for both production efficiency and cost saving?

Assessment Task 2: Matching Activity

Instrument: Matching Task
Covers: IAC0703
Instructions: Match the factors influencing productivity with their correct impact.

| Factor | Impact on Manufacturing Process |
| --- | --- |
| A. Late material deliveries | 1. Slows down the workflow |
| B. Equipment malfunction | 2. Causes downtime and missed deadlines |
| C. Poor communication | 3. Leads to errors and rework |
| D. Incomplete cutting lists | 4. Causes delays in material preparation |

Assessment Task 3: Scenario-Based Questions

Instrument: Case Scenario
Covers: IAC0704, IAC0708
Scenario:
You are working in a team preparing upholstered seat covers. The supervisor notices that production has stalled due to a missing cutting list. Later in the day, a power outage interrupts the production process.

Questions:
a. What role does a cutting list, specification sheet, and routing sheet play in ensuring uninterrupted production?
b. What steps should the team take to deal with the unplanned power outage?

Assessment Task 4: Practical Interpretation and Motivation

Instrument: Written Task
Covers: IAC0706, IAC0707
Instructions: Answer in your own words.

a. Why is it important to store reusable raw materials after a job is completed?
b. Explain how planned interruptions (such as scheduled maintenance or training) can improve overall productivity.

Model Answers

Task 1 – Structured Questions

- Job planning is important to avoid delays because it ensures that all resources, materials, and tools are available when needed, preventing stoppages.

- Productivity is the ability to complete tasks within timeframes using resources efficiently while maintaining quality standards.

- Minimising waste reduces costs, supports sustainable practices, and helps maintain clean and efficient workspaces.

Task 2 – Matching

- A → 2

- B → 1

- C → 3

- D → 4

Task 3 – Scenario-Based
a. These documents guide material preparation, cutting, and process routing, which help teams avoid confusion and delays.
b. The team should report the issue to the supervisor, follow the unplanned interruption procedure, secure workstations, and resume work safely once the power returns.

Task 4 – Practical Interpretation
a. Storing reusable raw materials reduces future costs and limits waste. It ensures that usable items are not discarded unnecessarily.
b. Planned interruptions allow time for maintenance, staff briefings, and training, which can prevent breakdowns and improve long-term productivity.

Marking Memo

| Question/Task | Assessment Criteria | Marks |
| --- | --- | --- |
| Task 1 Q1 | Explains importance of job planning clearly | 5 |
| Task 1 Q2 | Defines productivity and links it to manufacturing | 5 |
| Task 1 Q3 | Discusses waste minimisation and its effect | 5 |
| Task 2 | Correctly matches all 4 pairs | 5 |
| Task 3a | Describes functions of cutting/spec/routing sheets | 5 |
| Task 3b | Outlines procedure for unplanned interruptions | 5 |
| Task 4a | Explains value of reusing raw materials | 5 |
| Task 4b | Motivates benefit of planned interruptions | 5 |
| Total |  | 40 |

Assessment Rubric (Out of 40 Marks)

| Level | Description | Marks Range |
| --- | --- | --- |
| Excellent | Demonstrates full understanding, uses clear examples, answers all tasks accurately | 36–40 |
| Competent | Meets most expectations, explains key points clearly, minor errors only | 30–35 |
| Developing | Partial understanding, some key points missed or unclear | 24–29 |
| Not Yet Competent | Limited or incorrect responses, does not meet expected outcomes | 0–23 |

## KM-01-KT08: Upholstery Related Health and Safety Principles and Practices

NQF Level: 2  |  Credits: As per curriculum structure  |  Weight: 20%

1. Purpose of the Knowledge Module

The purpose of this Knowledge Module is to provide learners with a clear and practical understanding of health and safety responsibilities, risks, and procedures specific to the upholstered furniture manufacturing environment. This module focuses on the principles of occupational health and safety, legal compliance, risk identification, safe work practices, emergency protocols, and the responsible use and storage of hazardous materials.

By the end of this module, learners will be able to demonstrate an understanding of workplace hazards and their control, apply safety procedures, recognise and interpret safety signage, and comply with emergency and first aid protocols relevant to the upholstery department.

2. Key Knowledge Areas

The topic elements to be covered in this module include:

- KT0801: Occupational Health and Safety Act, regulations, and responsibilities of employer and employee

- KT0802: Hazards, safety, hazardous substances, incidents

- KT0803: Hazard identification and safe work procedures

- KT0804: Incidents and incident investigation

- KT0805: Safety and information signs, location of first aid and firefighting equipment and personnel, emergency evacuation procedures and meeting points

- KT0806: Hazardous substances, Material Safety Data Sheets (MSDS), and storage

- KT0807: Personal Protective Equipment (PPE)

- KT0808: First aid and first aiders in the workplace

- KT0809: Fire extinguishers and fire fighters in the workplace

- KT0810: Emergency procedures: evacuation and emergency meeting points

3. Internal Assessment Criteria and Weighting

This module carries a total weight of 20% and is assessed against the following Internal Assessment Criteria:

- IAC0801: The responsibilities of the employer and employee in terms of the OHSA are listed and explained

- IAC0802: Terms hazard, safe work practices, hazardous substance, incident are defined and discussed

- IAC0803: Hazards and potential hazards in the upholstery department are identified and discussed

- IAC0804: Safety and information signs in the workplace are defined

- IAC0805: Hazardous substances in the frame preparation department are defined and the interpretation of MSDS is discussed

- IAC0806: Safe storage of hazardous substances is argued

- IAC0807: Various types of fire extinguishers in the workplace are identified and the safe usage is discussed

- IAC0808: Legal compliance aspects of working with compressors and compressed air are evaluated

- IAC0809: The use and function of appropriate PPE in the upholstery covering department are analysed, and consequences of non-compliance are reasoned

- IAC0810: The importance of emergency procedures for workplace incidents is justified

4. Application in Upholstered Furniture Manufacturing

Learners completing this module will be able to apply key health and safety practices specifically suited to the upholstery production environment, which includes the use of pneumatic tools, adhesives, flammable materials, and heavy lifting. They will also develop workplace readiness by understanding how to respond effectively in emergencies, prevent injuries, and contribute to a legally compliant and hazard-conscious workplace.

### KT0801 – Occupational Health and Safety Act, Regulations and Responsibilities of Employer and Employee

Purpose of this Topic Element

This topic introduces learners to the legal framework governing health and safety in the workplace, with a focus on the Occupational Health and Safety Act (OHSA). It highlights the respective roles and responsibilities of both employers and employees within an upholstered furniture manufacturing environment. At NQF Level 2, learners are expected to understand the basic provisions of the Act and how they apply in their daily work.

Key Content to Cover

- Overview of the Occupational Health and Safety Act (OHSA)

  - Purpose: To provide for the health and safety of persons at work.

  - Key focus: Prevent workplace injuries, illnesses, and fatalities.

- Responsibilities of the Employer

  - Provide and maintain a working environment that is safe and without risk to health.

  - Ensure all machinery and equipment are safe to use.

  - Provide information, instruction, training, and supervision.

  - Ensure that the workplace has proper safety signage, PPE, and emergency procedures.

  - Keep records of incidents and ensure compliance with OHSA regulations.

- Responsibilities of the Employee

  - Take reasonable care for their own health and safety.

  - Co-operate with the employer on matters of health and safety.

  - Follow workplace safety rules and procedures.

  - Report hazards, unsafe conditions, or incidents immediately.

  - Use equipment and PPE as instructed.

- Consequences of Non-Compliance

  - Disciplinary action for employees.

  - Legal liability and penalties for employers.

  - Increased risk of injury, accidents, and workplace disruption.

Examples Relevant to the Upholstery Workplace

| Role | Responsibility |
| --- | --- |
| Employer | Ensuring guards are fitted on staple guns and pneumatic tools. |
| Employee | Reporting an air hose with a leak instead of continuing to use it unsafely. |
| Employer | Providing PPE such as gloves, goggles, and dust masks in the cutting area. |
| Employee | Wearing goggles and a dust mask when working with foam cutting equipment. |

Facilitator-Led Activities

- Group Brainstorm

  - Ask learners to list all the ways their employer promotes a safe workplace. Then ask how they, as employees, contribute to safety.

- Regulation Matching Exercise

  - Provide learners with short descriptions of scenarios (e.g., “The employer did not provide fire extinguishers”). Learners identify whether the responsibility lies with the employer or the employee.

- OHSA Poster Creation

  - In small groups, learners create a visual poster listing at least three responsibilities each for employers and employees. Display in the workshop as a visual reminder.

Critical Thinking Questions

- Why do you think it is important for both employers and employees to take responsibility for health and safety?

- What might happen if an employee ignores a known hazard in the workplace?

- How can a company show that it takes the OHSA seriously?

- What are some examples of your responsibilities as an employee in terms of health and safety?

### KT0802 – Hazards, Safety, Hazardous Substances, Incidents

Purpose of this Topic Element

This topic provides learners with a foundational understanding of key terms related to workplace health and safety. At NQF Level 2, learners are expected to accurately define and discuss the terms hazard, safety, hazardous substances, and incident in the context of the upholstered furniture production environment.

Understanding these concepts is essential for learners to recognise risks, apply safe work practices, and respond appropriately to incidents in the workplace.

Key Content to Cover

- Definition of a Hazard

  - Any source or situation with the potential to cause harm, injury, or ill health.

  - Examples: exposed wires, unguarded tools, flammable adhesives, unstable foam stacks.

- Definition of Safety

  - The condition of being protected from harm or other non-desirable outcomes.

  - Achieved by following safe work procedures, using PPE, and maintaining awareness of the environment.

- Definition of Hazardous Substances

  - Materials that may cause harm to people, property, or the environment through exposure.

  - Common examples in upholstery: adhesives, solvents, cleaning agents, foam dust.

- Definition of an Incident

  - An unplanned event that could or does result in injury, illness, damage, or other loss.

  - Includes both accidents (with consequences) and near misses (no immediate harm but potential).

Examples Relevant to the Upholstery Workshop

| Term | Example in the Workplace |
| --- | --- |
| Hazard | A leaking glue container near a heat source |
| Safety | Ensuring ventilation when using adhesives |
| Hazardous Substance | Foam off-cuts generating fine particles that may be inhaled |
| Incident | A worker slipping on an unmarked wet floor |

Facilitator-Led Activities

- Sorting Activity

  - Provide learners with a list of workplace situations. They must classify each as a hazard, hazardous substance, or incident.

- Workstation Risk Walkthrough

  - In groups, learners list potential hazards around a workstation and propose simple ways to reduce or eliminate the risks.

- Terminology Match

  - Learners match each term (hazard, safety, hazardous substance, incident) to its correct definition and an example.

Critical Thinking Questions

- What is the difference between a hazard and an incident?

- Why is it important to identify hazardous substances in the workshop?

- How can safety be improved by understanding what incidents have occurred before?

- What should you do if you discover a hazardous substance spill in your area?

### KT0803 – Hazard Identification and Safe Work Procedures

Purpose of this Topic Element

This topic equips learners with the ability to identify potential hazards in their upholstery working environment and apply the appropriate safe work procedures to manage or eliminate those hazards. At NQF Level 2, learners must demonstrate basic awareness of typical workplace risks and show that they can follow health and safety guidelines designed to prevent injury, illness, or damage.

Key Content to Cover

- Understanding Hazard Identification

  - Identifying sources of potential harm in the workplace.

  - Recognising physical, chemical, mechanical, ergonomic, and environmental hazards.

  - Regular hazard inspections as part of routine operations.

- Common Upholstery Hazards

  - Sharp tools (scissors, knives, staple guns)

  - Heavy lifting of frames or rolls of fabric

  - Flammable adhesives or solvents

  - Compressed air and pneumatic tools

  - Loose wires or blocked exits

- Safe Work Procedures

  - Steps outlined by the employer to reduce or eliminate risk.

  - Following instructions for tool use and machine operation.

  - Wearing PPE such as gloves, masks, and eye protection.

  - Storing hazardous materials safely.

  - Reporting identified hazards to a supervisor.

- Benefits of Following Safe Work Procedures

  - Reduces workplace injuries and incidents.

  - Supports productivity and team morale.

  - Ensures compliance with OHSA regulations.

Examples from the Upholstery Workplace

| Hazard | Safe Work Procedure |
| --- | --- |
| Exposed blade on fabric cutter | Use safety cover and cut away from the body |
| Heavy rolls of fabric | Use correct lifting techniques or ask for assistance |
| Adhesive fumes | Work in ventilated area and wear a respirator |
| Compressed air line loose | Tag the equipment and report to supervisor |

Facilitator-Led Activities

- Hazard Spotting Poster

  - Learners work in groups to create a poster illustrating hazards in an upholstery workshop and recommended safe procedures.

- Workshop Walkabout

  - In a real or simulated environment, learners identify five potential hazards and suggest a safe work procedure for each.

- Scenario Analysis

  - Present a workplace scenario with multiple hazards. Learners identify them and describe how to respond safely.

Critical Thinking Questions

- Why is it important to identify hazards before starting work?

- What could happen if safe work procedures are ignored?

- How does hazard identification contribute to a safer workplace?

- Who is responsible for reporting hazards — and why?

### KT0804 – Incidents and Incident Investigation

Purpose of this Topic Element

This topic introduces learners to the processes of recognising, reporting, and investigating incidents in the upholstered furniture manufacturing environment. At NQF Level 2, learners are expected to understand what constitutes an incident and how proper investigation helps prevent recurrence, supports legal compliance, and improves workplace safety.

Key Content to Cover

- Definition of an Incident

  - Any unplanned event that results in or could result in injury, illness, damage to property, or interruption of production.

  - Includes both accidents and near misses.

- Importance of Reporting Incidents

  - Legal requirement under OHSA.

  - Helps identify patterns or risks in the workplace.

  - Enables corrective action to be taken.

- Steps in Incident Investigation

  - Immediate response: secure the area and attend to injuries.

  - Report to supervisor or responsible person.

  - Collect details: who, what, where, when, how.

  - Identify root cause (e.g., faulty equipment, unsafe behaviour).

  - Document findings and recommend corrective actions.

- Roles in the Investigation

  - Employee: report accurately and honestly.

  - Supervisor: lead investigation, gather information, recommend solutions.

  - Health and Safety Representative (if applicable): support and monitor process.

- Benefits of Effective Incident Investigation

  - Prevents similar incidents.

  - Improves training and awareness.

  - Supports compliance and improves the safety culture.

Examples Relevant to the Upholstery Workshop

| Type of Incident | Response and Investigation Step |
| --- | --- |
| Worker trips over uncoiled air hose | Report the hazard, document the event, and enforce tool storage SOPs |
| Fire starts in foam off-cut bin | Evacuate, extinguish if safe, report, and investigate storage method |
| Pneumatic staple gun misfires | Isolate equipment, report fault, investigate tool condition and usage |

Facilitator-Led Activities

- Incident Simulation Exercise

  - Learners are presented with a simple incident scenario and work in groups to complete a basic investigation report (template provided).

- Root Cause Brainstorm

  - Using case examples, learners identify potential root causes (unsafe act, unsafe condition, lack of training) and propose prevention strategies.

- Incident vs. Near Miss Sorting

  - Learners categorise a list of workplace events into “incident” or “near miss,” then discuss why both must be reported.

Critical Thinking Questions

- Why should near misses be reported, even if no one was injured?

- What could happen if an incident is not properly investigated?

- Who is responsible for reporting and investigating an incident in your workplace?

- How can regular investigations improve safety procedures over time?

### KT0805 – Safety and Information Signs, Location of First Aid and Firefighting Equipment and Personnel, Emergency Evacuation Procedures and Meeting Points

Purpose of this Topic Element

This topic provides learners with the knowledge and ability to identify and interpret safety and information signs, locate emergency equipment and personnel, and follow evacuation procedures in the upholstered furniture production environment. At NQF Level 2, learners are expected to understand and apply safety signage and emergency response protocols in line with workplace policies and legal requirements.

Key Content to Cover

- Safety and Information Signs

  - Purpose: Communicate safety instructions and information quickly and clearly.

  - Types of signs used in the upholstery workplace include:

    - Hearing protection required

    - Respirator area

    - Dust mask area

    - Exit signs

    - First aid location signs

    - Fire extinguisher and hose reel signs

    - Eye wash station location

    - Emergency alarm signs

    - First aider on duty

    - Fire fighter on duty

    - Emergency numbers

    - Emergency meeting points

    - Colour coding of pneumatic air pipes

    - Demarcation lines for aisles and work zones

- Location and Identification of Safety Resources

  - First aid kits and designated first aiders

  - Fire extinguishers, hose reels, and trained fire fighters

  - Eye wash stations and emergency exits

  - Location maps showing safety equipment and assembly points

- Emergency Procedures

  - Evacuation protocol (stop work, follow exit signs, report to meeting point)

  - Role of fire wardens and first aiders

  - Procedure for fire, chemical spills, and injury incidents

  - Importance of knowing emergency numbers and alarm signals

Examples Relevant to the Upholstery Workplace

| Sign/Procedure | Purpose |
| --- | --- |
| Fire extinguisher location sign | Indicates the placement of fire-fighting equipment |
| Demarcated aisle lines | Keeps walkways clear for safe movement |
| First aider on duty sign | Identifies responsible person in case of injury |
| Emergency evacuation map | Guides workers to exits and assembly points in case of fire or emergency |

Facilitator-Led Activities

- Sign Identification Walkabout

  - Learners identify and record safety signs and equipment in a real or simulated workshop, explaining each one's meaning and purpose.

- Emergency Drill Simulation

  - Conduct a mock evacuation. Learners practise following procedures, assembling at the correct meeting point, and identifying fire wardens and first aiders.

- Sign Design Activity

  - In pairs, learners design a missing safety sign for the workshop and explain where and why it should be placed.

Critical Thinking Questions

- Why is it important to follow evacuation procedures exactly as trained?

- How can knowing the location of fire extinguishers and first aid kits reduce injury during an incident?

- What could happen if safety signs are ignored or not maintained?

- Why must all staff know who the designated first aiders and fire fighters are?

### KT0806 – Hazardous Substances, MSDS and Storage

Purpose of this Topic Element

This topic equips learners with foundational knowledge about hazardous substances used in the upholstery workshop, how to interpret Material Safety Data Sheets (MSDS), and the importance of following safe storage practices. At NQF Level 2, learners must recognise common hazardous materials, understand the information provided in MSDS, and apply correct procedures for safe handling and storage.

Key Content to Cover

- Definition of Hazardous Substances

  - Any material or chemical that poses a risk to health, safety, or the environment.

  - Common hazardous substances in upholstery include:

    - Adhesives (solvent- or water-based)

    - Cleaning agents

    - Lubricants

    - Foam and related materials

- Purpose and Contents of MSDS (Material Safety Data Sheets)

  - Identifies hazards associated with a substance.

  - Lists handling, storage, first aid, and emergency response instructions.

  - Information includes:

    - Product identification and ingredients

    - Health hazard data

    - Fire and explosion hazard data

    - Safe handling and storage instructions

    - Personal protective equipment requirements

    - Emergency procedures

- Storage Requirements

  - Substances must be:

    - Clearly labelled

    - Stored in original or approved containers

    - Separated by compatibility (e.g., flammable, corrosive)

    - Kept in well-ventilated areas

    - Locked away if hazardous to untrained persons

- Consequences of Unsafe Storage

  - Increased fire risk or toxic exposure

  - Environmental damage or workplace contamination

  - Violation of safety regulations and potential fines

Examples from the Upholstery Workshop

| Hazardous Substance | Risk | Safe Storage Practice |
| --- | --- | --- |
| Contact adhesive | Highly flammable, toxic fumes | Store in cool, ventilated area; away from heat or sparks |
| Solvent cleaner | Skin and eye irritant, vapour hazard | Keep in sealed containers with hazard labels |
| Lubricants | Slippery spill risk | Store upright in marked cabinet with spill trays |
| Foam off-cuts | Fire hazard when stored improperly | Compress and store in fire-resistant bins away from ignition |

Facilitator-Led Activities

- MSDS Interpretation Task

  - Provide learners with a sample MSDS. Ask them to identify three safety risks, the PPE required, and the correct storage method.

- Storage Labelling Simulation

  - Learners receive a list of chemicals and must assign correct hazard labels and suggest a compliant storage area.

- Group Discussion

  - Pose the question: “What might happen if adhesives are stored near a heat source?” Discuss legal and safety consequences.

Critical Thinking Questions

- Why must MSDS be available and accessible in the workshop?

- What can go wrong if substances are not stored in the correct containers?

- How can knowing how to read an MSDS improve workplace safety?

- What is your responsibility if you notice an unlabelled or leaking chemical container?

### KT0807 – Personal Protective Equipment (PPE)

Purpose of this Topic Element

This topic enables learners to identify and correctly use Personal Protective Equipment (PPE) specific to upholstery work. At NQF Level 2, learners must understand the purpose and proper use of various types of PPE, particularly those used in the upholstery covering department, and recognise the consequences of non-compliance with health and safety requirements.

Key Content to Cover

- Definition of PPE

  - Equipment worn to minimise exposure to workplace hazards that can cause injuries or illness.

  - PPE does not eliminate the hazard but reduces the risk of injury.

- Types of PPE Used in Upholstery

  - Eye protection: Safety goggles to guard against flying staples, glue spray, or dust.

  - Respiratory protection: Dust masks or respirators for use with foam dust or adhesives.

  - Hand protection: Gloves to prevent burns, cuts, or skin contact with chemicals.

  - Hearing protection: Earplugs or earmuffs near compressors or staple guns.

  - Foot protection: Closed, slip-resistant shoes to protect against dropped tools and spills.

  - Protective clothing: Aprons or overalls to shield from glue, dust, and debris.

- Responsibilities of Employers and Employees

  - Employers must provide PPE, train workers in its correct use, and maintain it in good condition.

  - Employees must wear PPE as required and report damaged or missing equipment.

- Consequences of Not Using PPE

  - Increased risk of injury or long-term health issues.

  - Legal non-compliance and potential disciplinary action.

  - Disruption to production due to accidents.

Examples Relevant to the Upholstery Workshop

| Task | Hazard | Recommended PPE |
| --- | --- | --- |
| Spraying adhesive | Inhalation of fumes | Respirator, gloves, safety goggles |
| Using pneumatic staple gun | Flying fasteners | Safety goggles, hearing protection |
| Cutting foam | Fine dust and skin irritation | Dust mask, gloves |
| Lifting fabric rolls | Crush injuries to feet | Safety footwear |

Facilitator-Led Activities

- PPE Identification Station

  - Display different PPE items. Learners label them, describe their purpose, and match them to upholstery tasks.

- PPE Violation Role-Play

  - Learners role-play scenarios where PPE is incorrectly or not used. Group discusses what went wrong and how it should have been handled.

- Condition Check Exercise

  - Learners inspect PPE samples for damage or wear and determine if they are safe for continued use.

Critical Thinking Questions

- Why is it important to wear PPE even when you are experienced in a task?

- What should you do if the PPE provided is damaged or does not fit properly?

- What risks could arise if you work with adhesives or foam dust without a mask?

- How can PPE compliance improve safety for the entire team?

### KT0808 – First Aid and First Aiders in the Workplace

Purpose of this Topic Element

This topic introduces learners to the role and importance of first aid and designated first aiders in the upholstered furniture manufacturing environment. At NQF Level 2, learners must be able to recognise basic first aid concepts, identify where first aid resources and personnel are located, and understand how first aid contributes to a safe and responsive workplace.

Key Content to Cover

- Definition of First Aid

  - Immediate assistance given to a person suffering from injury or illness before professional medical help is available.

  - Aims to preserve life, prevent the condition from worsening, and promote recovery.

- Role of the First Aider

  - A trained individual designated to provide initial care during an emergency.

  - Responsibilities include:

    - Assessing the situation

    - Ensuring the scene is safe

    - Administering appropriate first aid

    - Contacting emergency services if needed

    - Recording the incident as per workplace protocol

- First Aid Equipment in the Workplace

  - First aid box or cabinet stocked with required items (e.g., bandages, antiseptic, gloves, scissors).

  - Clearly marked and accessible location.

  - Regularly checked and maintained.

- Legal Requirements

  - OHSA requires employers to ensure that first aid facilities and trained personnel are available.

  - First aiders must be certified and display their identification in the workplace.

Examples Relevant to the Upholstery Workshop

| Scenario | First Aider’s Action |
| --- | --- |
| Worker cuts hand while trimming foam | Clean wound, apply pressure, dress the wound |
| Inhalation of strong adhesive fumes | Move person to fresh air, monitor breathing, report incident |
| Burn from hot glue gun | Cool burn with water, apply dressing, refer if necessary |
| Slip and fall near a wet area | Check for injuries, keep worker still, call for assistance |

Facilitator-Led Activities

- First Aid Box Exploration

  - Learners examine the contents of a first aid kit and identify the purpose of each item.

- Role Identification

  - Learners locate and list the names and contact details of designated first aiders displayed in the workplace.

- First Response Simulation

  - In small groups, learners simulate basic first aid scenarios and describe how they would react and whom they would inform.

Critical Thinking Questions

- Why is it important to know where the first aid kit and first aiders are located?

- What might happen if a first aid incident is not recorded properly?

- Why must first aiders be trained and certified?

- How does immediate first aid response help reduce the severity of an injury?

### KT0809 – Fire Extinguishers and Fire Fighters in the Workplace

Purpose of this Topic Element

This topic introduces learners to the types and safe use of fire extinguishers and the role of trained fire fighters in the workplace. In the upholstery production environment—where flammable materials such as adhesives, foam, and solvents are present—basic fire safety awareness is essential. At NQF Level 2, learners are expected to identify fire extinguishers, describe their use, and understand who the designated fire fighters are and what their roles include.

Key Content to Cover

- Types of Fire Extinguishers and Their Uses

  - Water (Red): For Class A fires – paper, wood, textiles (not for electrical or flammable liquids)

  - Foam (Cream): For Class A and B fires – solids and flammable liquids

  - CO₂ (Black): For electrical fires and flammable liquids

  - Dry Powder (Blue): Multipurpose – used for Class A, B, and C fires

  - Wet Chemical (Yellow): For cooking oils and fats (Class F fires)

- Reading and Interpreting Fire Extinguisher Labels

  - Colour codes

  - Instructions for use

  - Expiry dates and inspection tags

- Safe Use of Fire Extinguishers (PASS Technique)

  - Pull the pin

  - Aim the nozzle at the base of the fire

  - Squeeze the handle

  - Sweep side to side

- Workplace Fire Fighters

  - Trained staff members designated to respond in the event of a fire

  - Responsibilities include:

    - Coordinating evacuation

    - Using fire-fighting equipment if safe

    - Assisting emergency services

    - Conducting fire drills and safety checks

- Legal and Workplace Requirements

  - Fire extinguishers must be serviced and clearly marked

  - Fire fighters must receive formal training and be identifiable in the workplace

Examples Relevant to the Upholstery Workshop

| Fire Hazard | Correct Fire Extinguisher | Role of Fire Fighter |
| --- | --- | --- |
| Electrical fire at compressor unit | CO₂ extinguisher | Ensure area is evacuated, isolate power, use CO₂ unit |
| Flammable liquid spill ignites | Foam or dry powder extinguisher | Contain spread, alert emergency team, use extinguisher |
| Ignition of foam offcuts | Water (if no electrics), dry powder | Evacuate area, initiate fire drill procedure |

Facilitator-Led Activities

- Fire Extinguisher Identification

  - Learners are shown different types of extinguishers (real or images) and match them to the correct fire class and hazard.

- PASS Technique Demonstration

  - Facilitator demonstrates the PASS method. Learners practise with empty or training units.

- Fire Response Role-Play

  - Learners simulate a fire incident. Roles are assigned: fire fighter, evacuee, and observer. Afterward, the group reflects on the actions taken.

Critical Thinking Questions

- Why is it important to use the correct fire extinguisher for the specific type of fire?

- What could go wrong if an untrained person tries to put out a fire?

- Why must fire extinguishers be regularly serviced and clearly marked?

- How can designated fire fighters help reduce the risk of injury during a fire emergency?

### KT0810 – Emergency Procedures: Evacuation and Emergency Meeting Points

Purpose of this Topic Element

This topic focuses on the emergency procedures necessary during workplace incidents, with emphasis on the correct actions for evacuation and the identification of emergency meeting points. In the upholstery environment, where flammable materials and pneumatic tools are present, a well-structured emergency response plan is critical. At NQF Level 2, learners are expected to recognise alarm signals, follow evacuation procedures, and report to designated meeting points without hesitation or confusion.

Key Content to Cover

- Definition of an Emergency

  - Any unplanned event that poses a risk to life, health, property, or the environment.

  - Examples: fire, explosion, chemical spill, structural collapse, or medical emergency.

- Evacuation Procedures

  - Stop work immediately when the alarm is sounded or instructed.

  - Use designated escape routes, avoiding lifts and unsafe areas.

  - Do not collect personal belongings that may delay evacuation.

  - Assist others if safe to do so, particularly visitors or persons with limited mobility.

  - Close doors behind you to slow fire or smoke spread.

  - Do not re-enter the building until declared safe.

- Emergency Meeting Points

  - Pre-identified safe areas outside the building where employees must gather after evacuation.

  - Purpose:

    - Account for all staff.

    - Prevent re-entry.

    - Receive further instructions from fire fighters or safety officers.

- Roles During Emergency

  - Evacuee (everyone): Exit quickly and safely, report to the meeting point.

  - Fire wardens and safety officers: Assist in evacuation, guide people to safety, ensure full clearance.

- Importance of Emergency Preparedness

  - Saves lives and reduces injuries.

  - Limits property and equipment damage.

  - Supports legal compliance with OHSA.

Examples Relevant to the Upholstery Workshop

| Scenario | Expected Response |
| --- | --- |
| Fire breaks out near glue station | Sound alarm, evacuate via marked route, gather at meeting point |
| Power outage affects air compressors | Stop operations, report issue, wait for safety clearance |
| Chemical spill of solvent adhesive | Alert supervisor, evacuate area if instructed, do not touch substance |

Facilitator-Led Activities

- Evacuation Drill Simulation

  - Learners practise responding to a mock emergency alarm. They follow evacuation routes and assemble at the emergency meeting point.

- Mapping Exercise

  - Learners draw a basic floor plan of the workshop and mark the nearest fire exits, extinguishers, first aid kits, and meeting points.

- Procedure Sequencing Task

  - Learners are given mixed-up steps of an evacuation procedure. They must arrange them in the correct order.

Critical Thinking Questions

- Why is it dangerous to ignore an emergency evacuation alarm?

- How do emergency meeting points help keep everyone safe?

- What could happen if a person re-enters the building before it is cleared?

- Why is it important to practise emergency drills regularly?

## Integrated Formative Assessment – KM-01-KT08

Module Title: Upholstery Related Health and Safety Principles and Practices
Assessment Weight: 20%
NQF Level: 2
Assessment Type: Integrated, using multiple instruments
Assessment Conditions: Individual tasks and group activities under simulated or actual workshop conditions

🔧 Assessment Instrument 1: Short Answer Questions

Covers: IAC0801, IAC0802, IAC0809
Instructions: Answer each question clearly in your own words.

- List two responsibilities of an employer and two responsibilities of an employee under the Occupational Health and Safety Act.

- Define the following terms and give an upholstery-related example for each:
a) Hazard
b) Hazardous substance
c) Incident
d) Safe work practice

- What PPE should be worn when spraying adhesive, and what could happen if it is not used?

🔄 Assessment Instrument 2: Matching Exercise

Covers: IAC0804
Instructions: Match each safety/information sign to its purpose.

| Sign | Purpose |
| --- | --- |
| A. First aid location sign | 1. Indicates hearing protection is mandatory |
| B. Hearing protection sign | 2. Shows where to find fire-fighting equipment |
| C. Fire extinguisher sign | 3. Guides to emergency response equipment |
| D. Exit sign | 4. Identifies nearest safe route out of the building |

Answer Key:
A → 3
B → 1
C → 2
D → 4

Assessment Instrument 3: MSDS Interpretation Task

Covers: IAC0805, IAC0806
Instructions: Use the supplied Material Safety Data Sheet for a common solvent-based adhesive.

- Identify two health hazards associated with this substance.

- Describe where and how this adhesive should be safely stored.

- List two PPE items required when handling this substance.

Assessment Instrument 4: Fire Safety Scenario-Based Question

Covers: IAC0807, IAC0808
Scenario:
You are working near the foam cutting area when a small electrical fire breaks out at the compressor switchboard. The nearest fire extinguisher is a red water-based unit.

Questions:
a. Should you use this fire extinguisher? Why or why not?
b. Which type of extinguisher should be used?
c. What is the role of the trained workplace fire fighter in this situation?
d. What legal precautions must be followed when working with compressors?

🚨 Assessment Instrument 5: Practical Evacuation Reflection

Covers: IAC0810
Instructions: After participating in a simulated evacuation drill:

- Describe the steps you followed during the evacuation.

- Where was your emergency meeting point located?

- Why is it important to report to the meeting point and not re-enter the building?

✅ Model Answers

Instrument 1: Short Answers

- Employer: provide PPE, train staff. Employee: use PPE, report hazards.

- a) Hazard: exposed wire; b) Hazardous substance: solvent glue; c) Incident: slip on wet floor; d) Safe work practice: turning off tools before unplugging.

- Goggles and respirator. Without them, you could suffer eye irritation or inhale toxic fumes.

Instrument 2: Matching
A → 3, B → 1, C → 2, D → 4

Instrument 3: MSDS

- Inhalation risk, skin irritation.

- Store in a cool, ventilated area, away from flames.

- Gloves and mask.

Instrument 4: Scenario
a. No, water should not be used on electrical fires.
b. CO₂ extinguisher.
c. Evacuate area, use extinguisher if trained, inform emergency services.
d. Maintain pressure levels, drain water, check hoses regularly.

Instrument 5: Evacuation

- Stopped work, followed exit signs, gathered at meeting point.

- Located behind workshop storeroom.

- To account for everyone and avoid re-entry danger.

📊 Marking Memo

| Task | Max Marks |
| --- | --- |
| Instrument 1: Short Answers | 10 marks |
| Instrument 2: Matching | 4 marks |
| Instrument 3: MSDS Interpretation | 6 marks |
| Instrument 4: Fire Safety Scenario | 10 marks |
| Instrument 5: Evacuation Reflection | 10 marks |
| Total | 40 marks |

Assessment Rubric

| Level | Criteria Description | Score Range |
| --- | --- | --- |
| Excellent | Fully meets outcomes, answers are clear, correct, and well explained | 36–40 |
| Competent | Most outcomes met, minor errors present | 30–35 |
| Developing | Partial understanding, key outcomes missing | 24–29 |
| Not Yet Competent | Limited understanding, incorrect or missing responses | 0–23 |

## KM-01-KT09: Housekeeping, Cleaning and Record Keeping

NQF Level: 2  |  Credits: As per curriculum  |  Weight: 5%

1. Purpose of the Knowledge Module

The purpose of this Knowledge Module is to provide learners with essential knowledge regarding housekeeping principles, appropriate use of cleaning agents and equipment, and the importance of maintaining cleanliness to prevent contamination in the upholstery production environment. This includes routine cleaning and cleaning required when products change, ensuring that learners can contribute to a hygienic, efficient, and safety-conscious workplace.

By mastering this content, learners will be able to identify and apply proper cleaning procedures and explain the impact of poor housekeeping and contamination on production quality and compliance.

2. Key Knowledge Areas

The topic elements to be covered in this module include:

- KT0901: Housekeeping principles and requirements

- KT0902: Cleaning agents and equipment

- KT0903: Routine cleaning

- KT0904: Product change cleaning

- KT0905: Contamination

3. Internal Assessment Criteria and Weighting

This Knowledge Module carries a total weighting of 5% and is assessed against the following Internal Assessment Criteria:

- IAC0901: Housekeeping principles and requirements are explained

- IAC0902: Cleaning requirements are defined

- IAC0903: The prevention of product contamination is evaluated

4. Application in Upholstered Furniture Manufacturing

Clean and orderly workspaces are essential for the quality, safety, and productivity of upholstery manufacturing. Through this module, learners will understand how proper cleaning and record keeping help maintain standards and reduce risks. These practices ensure compliance with health and safety requirements and help maintain the lifespan of equipment and quality of final products.

### KT0901 – Housekeeping Principles and Requirements

Purpose of this Topic Element

This topic introduces learners to the fundamental principles of good housekeeping in the context of upholstered furniture manufacturing. At NQF Level 2, learners are expected to understand the standards and responsibilities associated with maintaining a clean, organised, and safe workspace, particularly in frame preparation and upholstery areas. Effective housekeeping is not just about cleanliness, but about reducing hazards, improving workflow, and supporting quality output.

Key Content to Cover

- Definition of Housekeeping in Manufacturing

  - Systematic process of keeping the work environment clean, organised, and hazard-free.

  - Includes physical cleaning, disposal of waste, proper material and tool storage, and clear floor space.

- Key Housekeeping Principles

  - Clean as you go: Continuous tidying during work shifts.

  - Everything in its place: Tools, materials, and equipment must be stored correctly.

  - Daily cleaning routines: Set times and responsibilities for end-of-shift or shift-change cleaning.

  - Clear walkways: Aisles and emergency exits must remain unobstructed.

  - Labelling and storage: Clear labels and designated areas for materials and waste.

- Housekeeping Requirements in the Upholstery Environment

  - Off-cuts of foam and fabric must be regularly cleared to prevent fire hazards.

  - Adhesive containers and solvent bottles must be sealed and stored properly.

  - Cleaning of air filters and dust extractors in machinery.

  - Neat storage of staple guns, pneumatic hoses, and safety gear.

- Benefits of Good Housekeeping

  - Enhances safety by reducing slips, trips, and fire risks.

  - Supports efficiency and productivity.

  - Contributes to quality control and equipment maintenance.

  - Encourages a culture of responsibility and teamwork.

Examples Relevant to the Upholstery Workplace

| Area | Housekeeping Action |
| --- | --- |
| Frame preparation bench | Clean glue residue, remove wood dust, return tools to racks |
| Foam cutting station | Clear scraps, vacuum dust, store rolls off the floor |
| Tool storage cabinet | Organise by tool type, label drawers, inspect for cleanliness |
| Shared walkways | Keep free from obstacles, check for spills, mark damaged flooring |

Facilitator-Led Activities

- Housekeeping Spot-the-Hazard Exercise

  - Show learners a photo of a messy workstation. Ask them to list what housekeeping violations they see and suggest corrections.

- Daily Cleaning Checklist Development

  - Learners work in groups to create a checklist for cleaning duties in a specific workstation (e.g., foam cutting or frame preparation).

- Workstation Walkabout (or Simulated)

  - Learners observe a real or model workstation and report on the state of housekeeping using a simple checklist.

Critical Thinking Questions

- How does poor housekeeping contribute to workplace injuries or accidents?

- What might be the impact on productivity if tools are not returned to their correct places?

- Why should everyone in the team participate in daily housekeeping routines?

- How can good housekeeping improve the overall quality of the final upholstered product?

### KT0902 – Cleaning Agents and Equipment

Purpose of this Topic Element

This topic introduces learners to the correct identification, selection, and use of cleaning agents and equipment within the upholstery production environment. At NQF Level 2, learners are expected to recognise commonly used cleaning products and tools, understand their correct applications, and apply basic safety practices during cleaning tasks to ensure an organised, safe, and hygienic workspace.

Key Content to Cover

- Types of Cleaning Agents in Upholstery Workshops

  - Multipurpose cleaners – used for wiping benches and surfaces.

  - Solvent-based degreasers – used to remove glue residue or oil from machinery.

  - Foam-safe dust removers – used in areas where foam dust is present.

  - Sanitisers – for communal areas or after spills involving bodily fluids.

- Types of Cleaning Equipment

  - Manual tools: brooms, dustpans, rags, mops, brushes.

  - Mechanical tools: industrial vacuum cleaners, dust extractors, spray bottles.

  - Storage equipment: labelled buckets, PPE for cleaning tasks, caddies for chemicals.

- Safe Use and Handling of Cleaning Agents

  - Always follow the instructions and use PPE as required (gloves, goggles).

  - Never mix cleaning chemicals unless approved.

  - Use only approved containers and store agents in clearly labelled locations.

- Maintenance of Cleaning Equipment

  - Cleaning tools should be rinsed and stored properly after use.

  - Check filters on vacuum systems regularly.

  - Replace worn-out brushes, mops, or cloths.

Examples Relevant to the Upholstery Workplace

| Cleaning Task | Agent or Equipment | Safety Measure |
| --- | --- | --- |
| Removing glue from the workbench | Solvent-based cleaner, rag | Use gloves, ventilate area |
| Clearing foam dust from machinery | Vacuum cleaner with foam filter | Wear dust mask, clean filter afterward |
| General floor cleaning | Mop, bucket with detergent | Mark wet floor, wear closed shoes |
| Wiping tools and equipment | Lint-free cloth and general-purpose cleaner | Store cleaning cloths separately by task |

Facilitator-Led Activities

- Toolbox Talk – Cleaning Materials Review

  - Display a table of common cleaning agents and equipment used in the workshop. Learners describe their correct use and any hazards.

- Sorting Task: Agent or Equipment?

  - Learners receive flashcards and must sort them into categories (agent, equipment, or both), explaining their purpose.

- Cleaning Safety Role-Play

  - Simulate an incorrect cleaning practice (e.g., spraying solvent without PPE). Learners identify the mistake and demonstrate the correct approach.

Critical Thinking Questions

- Why is it important to use the right cleaning agent for the right surface?

- What might happen if cleaning chemicals are not stored in their original containers?

- How does properly maintained cleaning equipment contribute to workplace safety?

- What personal protective equipment should be worn when using strong cleaning products?

### KT0903 – Routine Cleaning

Purpose of this Topic Element

This topic provides learners with the essential knowledge and understanding of routine cleaning practices in the upholstery manufacturing environment. At NQF Level 2, learners are expected to participate in daily, scheduled cleaning tasks that support workplace safety, minimise risks of contamination or injury, and uphold production standards. Routine cleaning is an essential part of housekeeping that ensures a clean, orderly, and hazard-free working environment.

Key Content to Cover

- What is Routine Cleaning?

  - Scheduled cleaning tasks that are performed at regular intervals—daily, after each shift, or weekly.

  - Includes cleaning tools, surfaces, walkways, and waste disposal points.

- Components of Routine Cleaning in Upholstery

  - Work surfaces: benches, tool areas, and cutting tables.

  - Floors: sweeping, vacuuming, or mopping foam dust, wood chips, or debris.

  - Equipment: wiping down pneumatic tools, vacuuming machine filters, cleaning air hoses.

  - Waste areas: emptying bins, separating recyclables and hazardous waste.

  - Material storage: tidying and wiping shelves, removing expired or damaged items.

- Roles and Responsibilities

  - Learners should know which tasks are their responsibility and follow the workshop's cleaning schedule.

  - Cleaning should be documented in cleaning logs or checklists as per workplace procedures.

- Importance of Routine Cleaning

  - Reduces fire hazards (e.g., from foam or fabric off-cuts).

  - Prevents injury from cluttered or slippery floors.

  - Extends equipment lifespan through regular maintenance.

  - Enhances productivity by maintaining an organised space.

Examples Relevant to the Upholstery Workshop

| Cleaning Task | Frequency | Responsible Person |
| --- | --- | --- |
| Sweep around workstations | After each shift | Upholstery team |
| Vacuum foam cutter machine | Weekly | Machine operator |
| Clean adhesive spray nozzles | Daily | Assigned technician |
| Empty glue and fabric waste bins | Twice a day | Cleaning staff or operator |

Facilitator-Led Activities

- Daily Cleaning Plan Exercise

  - Learners work in groups to create a routine cleaning schedule for an upholstery workshop area (e.g., the cutting station or the spray booth).

- Cleaning Task Observation

  - Learners observe a routine cleaning activity (real or simulated) and provide feedback on safety, completeness, and efficiency.

- Cleaning Checklist Completion

  - Provide learners with a blank cleaning checklist and a list of tasks. They must match each task with the correct frequency and responsible role.

Critical Thinking Questions

- What are the risks of not following a routine cleaning schedule in the upholstery workshop?

- How can routine cleaning contribute to improved quality control?

- Why should cleaning responsibilities be clearly assigned and documented?

- What are some signs that routine cleaning is not being carried out effectively?

### KT0904 – Product Change Cleaning

Purpose of this Topic Element

This topic introduces learners to the cleaning processes specifically required when changing from one product to another in the upholstery production environment. At NQF Level 2, learners must recognise the importance of preventing cross-contamination between product lines, especially when switching materials, colours, or adhesives. Proper product change cleaning ensures that production quality and health and safety standards are upheld.

Key Content to Cover

- Definition of Product Change Cleaning

  - Cleaning conducted between the production of different product types or batches to ensure no residue, material, or waste contaminates the next product.

- When Product Change Cleaning Is Required

  - Changing from one fabric or foam colour to another.

  - Switching adhesives (e.g., from water-based to solvent-based).

  - Moving from standard to custom orders with unique components.

  - Reconfiguring tools or machines for new product specifications.

- Areas Typically Cleaned During Product Change

  - Work surfaces: tables, benches, and measuring tools.

  - Tools and machines: spray guns, cutting tools, staplers, and sewing machines.

  - Storage and handling areas: clear out leftover materials from previous product run.

  - Waste containers: empty bins to avoid mixing materials.

- Purpose and Benefits

  - Prevents staining, material mismatching, and quality defects.

  - Reduces risk of customer complaints or returns.

  - Helps maintain clear inventory and traceability.

  - Supports efficient transitions between jobs.

Examples Relevant to the Upholstery Workshop

| Scenario | Required Cleaning Action |
| --- | --- |
| Switching from blue to white fabric | Clean all surfaces and machines to remove coloured lint or fibres |
| Changing from solvent-based to water-based glue | Flush spray guns and replace nozzles to avoid chemical reaction |
| Moving from commercial to domestic furniture | Remove and store all unused commercial-grade components |

Facilitator-Led Activities

- Case Sorting Activity

  - Present learners with several production scenarios. Learners decide whether routine or product change cleaning is needed and explain their choice.

- Step-by-Step Cleaning Plan

  - Learners develop a checklist for product change cleaning between two sample upholstered items with different materials.

- Role-Play: Changeover Meeting

  - Learners simulate a team meeting at the end of a production run to plan cleaning before starting a new product line.

Critical Thinking Questions

- What could happen if leftover material from one product contaminates another during manufacturing?

- How does product change cleaning contribute to customer satisfaction?

- Why must tools be checked for residue during a product changeover?

- How can a team ensure that no step is missed during product change cleaning?

### KT0905 – Contamination

Purpose of this Topic Element

This topic introduces learners to the concept of contamination in the upholstered furniture manufacturing environment. At NQF Level 2, learners must understand what contamination is, how it occurs, and how to prevent it. Contamination can result in reduced product quality, customer dissatisfaction, health and safety risks, and regulatory non-compliance. Learners will explore both material and process-based causes of contamination and methods to eliminate or reduce them through good housekeeping and cleaning practices.

Key Content to Cover

- Definition of Contamination

  - The presence of unwanted substances, materials, or residues in, on, or around upholstery components, work surfaces, or equipment.

  - Can be physical (dust, fibres), chemical (cleaning agents, adhesives), or biological (mould, bacteria).

- Common Sources of Contamination

  - Residue from previous products (foam dust, fabric threads, adhesives).

  - Dirty tools or equipment.

  - Poor waste separation or overflowing bins.

  - Inadequate personal hygiene practices.

  - Improper storage of materials.

- Risks and Consequences of Contamination

  - Damaged or stained products.

  - Health and safety risks to workers and end users.

  - Increased rework, delays, and waste.

  - Loss of customer confidence and possible product returns.

- Preventative Measures

  - Implement proper cleaning and storage routines.

  - Clean work areas and tools thoroughly during and after production.

  - Use colour-coded containers or work zones to avoid material mixing.

  - Maintain well-ventilated and dry storage areas.

  - Handle materials with clean hands or gloves when required.

Examples Relevant to the Upholstery Workshop

| Contamination Type | Source | Impact |
| --- | --- | --- |
| Foam dust | Leftover from cutting station | Stains fabric, affects adhesive bonding |
| Solvent residue | Uncleaned spray gun | Damages material or discolours the fabric surface |
| Mould contamination | Improperly stored foam in damp areas | Health hazard and product rejection |
| Cross-material mixing | Fabric from previous job left in workstation | Incorrect assembly or aesthetic defects |

Facilitator-Led Activities

- Contamination Identification Game

  - Learners are shown pictures of contaminated vs. clean workspaces. They must identify the contaminant and its possible cause.

- Contamination Prevention Plan

  - In small groups, learners draft a contamination prevention plan for a specific production stage (e.g., foam storage or fabric handling).

- Fault Diagnosis Exercise

  - Learners are presented with a defective product and asked to trace back the potential source of contamination.

Critical Thinking Questions

- How could contamination affect the final quality of an upholstered furniture product?

- What are some examples of contamination that may not be visible immediately?

- What responsibility do individual workers have in preventing contamination?

- How can poor contamination control lead to increased costs in production?

## Integrated Formative Assessment – KM-01-KT09: Housekeeping, Cleaning and Record Keeping

NQF Level: 2  |  Weight: 5%
Assessment Approach: Multiple instruments (short-answer, classification, practical task)
Assessment Context: Upholstery workshop (frame preparation and covering department)

🔧 Assessment Instrument 1: Short-Answer Questions

Covers: IAC0901
Instructions: Answer each question clearly in your own words.

- What is the purpose of housekeeping in the upholstery workshop?

- List three housekeeping practices that help maintain a safe and productive workplace.

- Why is it important to keep walkways and exits clear at all times?

Assessment Instrument 2: Classification Task – Cleaning Agents and Equipment

Covers: IAC0902
Instructions: Classify each of the following items as either a cleaning agent or cleaning equipment. Place each item into the correct column.

| Item | Cleaning Agent or Cleaning Equipment? |
| --- | --- |
| Multipurpose surface spray |  |
| Mop |  |
| Solvent adhesive remover |  |
| Dust extractor |  |
| Foam-safe cleaner |  |
| Scrubbing brush |  |

Assessment Instrument 3: Practical Application – Cleaning Schedule Design

Covers: IAC0902, IAC0903
Instructions: Design a simple cleaning schedule for your workstation that includes:

- Two routine cleaning tasks

- One task required during a product change

- The cleaning agents or equipment needed

- Who is responsible for each task

- The frequency of the task

You may use a table format for clarity.

⚠️ Assessment Instrument 4: Contamination Risk Identification

Covers: IAC0903
Instructions: Read each situation below. Identify if there is a contamination risk, and briefly explain how to prevent it.

| Situation | Contamination Risk? (Yes/No) | How to Prevent It |
| --- | --- | --- |
| A worker stores blue foam offcuts on a bench where white fabric is cut. |  |  |
| A stapler is left with glue residue on the handle and used on a new product. |  |  |
| Used cloths and new fabric are stored together in the same bin. |  |  |

✅ Model Answers

Instrument 1: Short Answers

- To maintain a clean, safe, and organised work environment.

- Clean as you go, proper waste disposal, tool storage.

- To prevent accidents and ensure emergency access.

Instrument 2: Classification

- Cleaning Agents: Multipurpose surface spray, Solvent adhesive remover, Foam-safe cleaner

- Cleaning Equipment: Mop, Dust extractor, Scrubbing brush

Instrument 3: Example Schedule Table

| Task | When | Tools/Agents | Responsible |
| --- | --- | --- | --- |
| Sweep floor | End of each shift | Broom, dustpan | Operator |
| Wipe benches | Daily | Surface cleaner, cloth | Operator |
| Clean spray gun (change) | On product change | Solvent, brush | Technician |

Instrument 4: Risk Identification

| Situation | Risk? | Prevention |
| --- | --- | --- |
| Blue foam offcuts near white fabric | Yes | Remove offcuts and clean surface before use |
| Stapler with glue residue used again | Yes | Clean tool properly before next job |
| Used cloths and new fabric in same bin | Yes | Use separate, labelled storage containers |

📊 Marking Memo (Total: 20 marks)

| Instrument | Max Marks |
| --- | --- |
| Instrument 1 – Short Answers | 5 |
| Instrument 2 – Classification | 3 |
| Instrument 3 – Schedule | 6 |
| Instrument 4 – Risk Task | 6 |
| Total | 20 marks |

Assessment Rubric

| Level | Criteria Description | Score Range |
| --- | --- | --- |
| Excellent | All tasks completed with clear, correct responses and practical insight | 17–20 |
| Competent | Most answers accurate with minor errors or omissions | 14–16 |
| Developing | Partial understanding; several errors; requires support | 10–13 |
| Not Yet Competent | Incomplete or incorrect responses; lacks understanding | 0–9 |