FURNITURE UPHOLSTERER

KM03 UPHOLSTERED FURNITURE BASIC COVER FITTING

FACILITATOR GUIDE

Table of Contents

Facilitator Guide Introduction	6

KM-03: Upholstered Furniture Basic Cover Fitting	6

KM-03-KT01: Specifications for Upholstered Furniture	8

KT0101: The Function of Product Specifications and Standards in the Upholstery Manufacturing Process	10

KT0102: Types of Product Specifications and Their Uses	13

KT0103: Principles and Concepts of Interpreting Specifications and Workplace Instructions	16

KT0104: Frame and Upholstery Components Construction and Parts Identification	19

KT0105: Compliance and Non-Compliance	22

KT0106: Consequences of Prepared Frames and Covered Furniture Not Meeting Specified Standards	25

📝 Integrated Assessment: KM-03-KT01	28

Facilitator Assessment Briefing	32

KM-03-KT02: Upholstery Raw Materials and Consumables Types and Characteristics	35

KT0201: Furniture Raw Materials with Reference to Natural and Manmade Fabrics and Finishings	38

KT0202: Location of Raw Materials and Consumables	41

KT0203: Main Upholstery Material Types	44

KT0204: Types, Sizes, Quantity and Quality of Consumables Needed for Cover Fitting, Finishing and Trimming	48

KT0205: Density, Thickness and Types of Foam Required for the Foaming Up Process	52

KT0206: Types of Padding for Covering Coil Springs	56

KT0207: Consumables Required for the Foaming Up Process, Including Adhesives, Solvents, Profile Jigs and Templates	60

KT0208: Using Profile Jigs and Templates	63

KT0209: Fabrics Used for Covering Webbing, Springs, Foam and Padding Including Calico, Hessian, Polypropylene, Dacron and Flock	66

KT0210: Types and Gauges of Various Springs Needed for the Knocker-On Process, Including Coil Springs and Zig-Zag Springs	70

KT0211: Types, Sizes, Quantity and Quality of Consumables Including Clips, Nails, Staples, Edge Rolls, Twine and Cord Needed for the Springing Up Process	73

KT0212: Types of Webbing Needed for the Knocker-On Process, Including Manmade Elastic and Rubber Webbing, and Those Made from Natural Fibres like Jute Webbing and Hessian	77

KT0213: Labels, Codes and the Coding System and Traceability	84

KT0214: Principles and Concepts of Safe Materials Handling and Storage Methods	87

Integrated Assessment – KM-03-KT02	90

KM-03-KT03: Use of Chemicals – Adhesives and Solvents	94

KT0301: Chemical Types, Characteristics, Properties and Purpose	96

KT0302: Adhesives and Solvents – Characteristics and Uses, Flammable and Hazardous, Safe Handling and Storage	99

KT0303: Application Equipment, Tools and Dispensers	103

KT0304: Measuring and Mixing Techniques and Procedures	106

KT0305: Preparation and Application Techniques	109

KT0306: Safety Hazards and Risks	113

Integrated Assessment – KM-03-KT03	116

Facilitator Assessment Briefing	119

KM-03-KT04: Upholstery Tools and Equipment	122

KT0401: Upholstery Hand Tools for the Furniture Covering Processes – Operation, Components and Parts, Uses, Safe Storage and Handling	124

KT0402: Pneumatic Tools – Settings and Operation, Components and Parts, Relationship Between Parts, Uses, Safe Storage and Handling, Supply Systems	128

KT0403: Upholstery Electric Portable Power Tools – Settings and Operation, Components and Parts, Uses, Safe Storage and Handling	132

KT0404: Upholstery Covering Equipment	135

KT0405: Personal Protective Equipment and Materials Handling Equipment	138

KT0406: Tools and Equipment Maintenance, Fault Finding and Standard Operating Procedures	141

KT0407: Safety Measures and Mechanisms	145

Integrated Assessment – KM-03-KT04	148

Facilitator Assessment Briefing	151

KM-03-KT05: Upholstery Measurements, Measuring Tools, Techniques and Calculations	153

KT0501: Principles of Accurate Measurements During the Frame Preparation Process	155

KT0502: Units of Measurement and Measuring Tools and Equipment Required for the Frame Preparation Process	158

KT0503: Measuring Techniques	161

KT0504: Calculations and Angles	164

KT0505: Calculating Allowances	167

KT0506: Problem Solving	170

Integrated Assessment – KM-03-KT05	173

Facilitator Assessment Briefing	176

KM-03-KT06: Upholstered Furniture Manufacturing Processes	178

KT0601: Upholstered Furniture Manufacturing Process from Raw Frame to Finished Product	180

KT0602: Process Flow and Productivity	183

KT0603: Operations in the Process Flow – Knocker-On Operations, Foaming-Up Operations, Padding Sprung Foundations and Frames, and Covering Operations	186

KT0604: Routing Sheets	189

KT0605: Cutting Lists	192

KT0606: Product Specifications	195

KT0607: Finishing Aids	198

Integrated Assessment – KM-03-KT06	201

Facilitator Assessment Briefing	204

KM-03-KT07: Upholstery Covering Principles	206

KT0701: Stretch, Tension and Deflection of Fabric	208

KT0702: Spacing of Tacks, Nails and Staples	211

KT0703: Uniformity of a Pair of Arms and Wings, Pleating and Folding and Buttoning	214

KT0704: Positioning of the Cover and Alignments	217

KT0705: Pattern Matching (Fabric)	220

KT0706: Principles and Concepts of Preparing and Stacking Covered Frames for the Next Step in the Production Process	223

Integrated Assessment – KM-03-KT07	226

Facilitator Assessment Briefing	229

KM-03-KT08: Upholstered Furniture Covering Operations and Techniques	231

KT0801: Correct Sequence and Procedure	233

KT0802: Slip Stitching with Circular Needle and Piping Slip Stitching	236

KT0803: Buttoning	239

KT0804: Tufting	242

KT0805: Tacking, Stapling, Nailing	245

KT0806: Pleating and Folding Fabric	248

KT0807: Pulling, Tensioning and Stretching	251

KT0808: Preventing Tack Ties	254

KT0809: Shaping – Edges and Corners by Using Hand Skills	257

KT0810: Measuring – Centralising the Cover, Spacing of Buttons and Studs, Front Borders of Chairs	260

KT0811: Attaching Decorative Trimmings	263

KT0812: Cutting In of Covers	266

KT0813: Foam Marking	269

Integrated Assessment – KM-03-KT08	272

Facilitator Assessment Briefing	275

# Facilitator Guide Introduction

## KM-03: Upholstered Furniture Basic Cover Fitting

NQF Level: 2
Credits: 8

1. Purpose of the Knowledge Module

The purpose of this Knowledge Module is to introduce learners to the foundational knowledge required to understand and carry out basic upholstery covering procedures on prepared frames. It focuses on the theoretical principles underpinning upholstery specifications, materials, equipment, techniques, and production processes associated with the basic cover fitting stage in upholstered furniture manufacturing.

Through this module, learners will acquire essential knowledge that complements the practical skills and workplace experience required to operate effectively as an Upholstery Cover Fitter. This foundational understanding is critical to ensure precision, quality, and efficiency in the application of covers using various materials, tools, and techniques.

2. Key Knowledge Areas

The module consists of eight Knowledge Topics (KTs), each aligned with specific learning outcomes and internal assessment criteria:

- KM-03-KT01: Specifications for upholstered furniture (10%)

- KM-03-KT02: Upholstery raw materials and consumables types and characteristics (20%)

- KM-03-KT03: Use of chemicals: adhesives and solvents (10%)

- KM-03-KT04: Upholstery tools and equipment (10%)

- KM-03-KT05: Upholstery measurements, measuring tools, techniques and calculations (10%)

- KM-03-KT06: Upholstered furniture manufacturing processes (10%)

- KM-03-KT07: Upholstery covering principles (10%)

- KM-03-KT08: Upholstered furniture covering operations and techniques (20%)

Each topic builds progressively to equip learners with a coherent understanding of upholstery covering practices, from interpreting specifications to identifying materials, handling adhesives, measuring accurately, and applying basic covering operations using appropriate techniques.

3. Internal Assessment Criteria and Weighting

The total weighting of the Knowledge Module is distributed as follows, guiding the development of assessments and ensuring alignment with occupational competence:

- IAC0101 – IAC0105: Understand and interpret product specifications (10%)

- IAC0201 – IAC0214: Identify and evaluate raw materials and consumables (20%)

- IAC0301 – IAC0310: Describe adhesives, solvents and their safe use (10%)

- IAC0401 – IAC0410: Identify and maintain upholstery tools and equipment (10%)

- IAC0501 – IAC0505: Apply measuring and calculation techniques (10%)

- IAC0601 – IAC0606: Understand upholstery processes and workflows (10%)

- IAC0701: Describe and apply covering principles (10%)

- IAC0801 – IAC0809: Apply upholstery covering techniques in sequence and identify faults (20%)

These criteria ensure learners are assessed on knowledge that directly supports performance in the workplace, fostering occupational readiness.

4. Application in Upholstered Furniture Production

This module forms a critical link in the broader upholstery production value chain. The knowledge gained is applied in covering operations where the prepared upholstery frame is transformed into a visually and functionally complete product.

Understanding specifications ensures the accurate interpretation of job cards and technical drawings. Familiarity with raw materials and adhesives contributes to correct material selection and application. Proficiency in tools, equipment, and techniques helps learners avoid common covering faults and achieve the desired aesthetic and structural outcomes. The module thus supports both craftsmanship and productivity in upholstered furniture production.

## KM-03-KT01: Specifications for Upholstered Furniture

NQF Level: 2
Credit Weight: 10%

1. Purpose of the Knowledge Topic

The purpose of this Knowledge Topic is to equip learners with the foundational knowledge required to understand, interpret, and apply product specifications in the context of upholstered furniture manufacturing. In the upholstery production process, precise specifications are essential to ensure quality, consistency, and compliance with client requirements and industry standards. This topic enables learners to develop the ability to recognise different types of specifications, identify key construction elements of upholstery components, and apply these insights to real-world tasks.

Understanding how to interpret technical documents such as drawings, sketches, prototypes, and job cards is a critical skill for an Upholstery Cover Fitter. It empowers learners to work confidently with industry-standard documentation and avoid costly errors that may arise from non-compliance with given specifications.

2. Key Knowledge Areas

The topic is structured around six key knowledge elements:

- KT0101: The function of product specifications and standards in the upholstery manufacturing process

- KT0102: Types of product specifications, including sketches, technical drawings, samples, prototypes, cutting lists, and job cards, and their respective uses

- KT0103: Principles and concepts for interpreting specifications and workplace instructions to determine appropriate types, quantities, sizes, and qualities of materials and consumables

- KT0104: Frame and upholstery components construction and identification of various parts

- KT0105: The concepts of compliance and non-compliance with specifications

- KT0106: Consequences of failing to meet required product standards during frame preparation and covering

These knowledge components lay the groundwork for accurate material usage, quality assurance, and a deep understanding of the production process from an operational and compliance perspective.

3. Internal Assessment Criteria and Weighting

This topic carries a total weighting of 10% in the overall Knowledge Module and is assessed through the following Internal Assessment Criteria:

- IAC0101: Differentiation and comparison of various types of product specifications used in the upholstery industry

- IAC0102: Identification and explanation of aspects in sketches and engineering drawings, including types of views (isometric, oblique, orthographic), dimensions, geometry, labelling, and features like curves and radiuses

- IAC0103: Identification of concepts included in workplace and industry product specification documents such as drawing standards, engineering symbols, material standards, and conventions

- IAC0104: Explanation of the concepts of compliance and non-compliance in product manufacturing

- IAC0105: Evaluation of consequences for the upholstery process when frames or covers do not meet specified product standards

These assessment criteria guide the development of lesson plans and formative assessments to ensure that learners achieve the required level of competence in reading and applying product specifications.

4. Application in Upholstered Furniture Manufacturing

This knowledge topic directly supports the learner’s role in ensuring that all upholstery work adheres to precise specifications, which is critical in both mass production and bespoke furniture manufacture. The learner will engage with real production documents and use this knowledge to plan covering procedures, select appropriate materials, and execute cover fitting tasks with accuracy.

The ability to identify non-compliance early in the process can reduce waste, rework, and client dissatisfaction, while also supporting quality assurance practices in the upholstery value chain. Ultimately, this topic reinforces the learner’s capacity to contribute to efficient and quality-driven upholstery manufacturing operations.

### KT0101: The Function of Product Specifications and Standards in the Upholstery Manufacturing Process

Purpose of the Topic

This topic introduces learners to the purpose and importance of product specifications and standards within the upholstery manufacturing environment. Learners will understand that specifications are not merely paperwork, but essential guides to ensure correct construction, material selection, quality assurance, and legal compliance.

By mastering this topic, learners will be able to:

- Describe what product specifications are.

- Understand the role of standards in ensuring consistent quality and safety.

- Appreciate the importance of compliance in meeting client requirements.

Content Breakdown

- Definition of Product Specifications

  - A product specification is a detailed, written description of a product’s design, materials, dimensions, and performance criteria. It acts as a blueprint for production.

- Types of Standards and Their Role

  - Internal Standards: Specific to the company; may include house style guides, internal quality benchmarks.

  - Industry Standards: E.g., SABS codes (South African Bureau of Standards), ISO standards where applicable.

  - Health and Safety Standards: Ensure that materials (e.g., adhesives, foams) comply with occupational safety requirements.

- Why Specifications and Standards Matter

  - To ensure consistency in product output.

  - To guide accurate material usage and reduce wastage.

  - To meet client and regulatory requirements.

  - To prevent errors that can result in costly rework or unsafe furniture.

Facilitation Strategies

- Use physical samples of job cards, cutting lists, and sketches to show how specifications are communicated on the factory floor.

- Break learners into pairs to discuss what might go wrong in the absence of a proper specification.

- Use a visual diagram to show how specifications flow from the design team to the upholstery floor.

Example

A factory in Durban receives an order for 100 restaurant booth seats. The product specification lists the type of foam (28/200 density), fabric (vinyl, flame-retardant), leg height (120 mm), and backrest angle (15 degrees). If the upholsterer ignores the angle specification and builds them at 90 degrees, the restaurant may reject the entire batch for being uncomfortable, leading to financial loss.

Case Study: Non-Compliance at Harmony Designs (Cape Town)

Background:
Harmony Designs manufactures bespoke headboards. One of their clients, a boutique hotel chain, ordered 50 upholstered headboards in navy blue velvet with brass nail detailing. The design specification included a curved top profile and a height of 1.2 metres.

Problem:
The upholsterer, in a rush to meet the deadline, used straight-top frames by mistake, and the quality inspector failed to compare the final product to the original specification.

Result:

- The hotel rejected the delivery.

- Harmony Designs had to remake the order at their own cost.

- A new internal quality check procedure was implemented to verify compliance against specs at each stage.

Teaching Point:
This demonstrates how even small deviations from specifications can have serious consequences for client relationships and profitability.

Critical Thinking Questions

- Why is it important for the upholsterer to understand the specification document before starting work?

- What could happen if incorrect materials are used due to a misinterpretation of a specification?

- What systems can be introduced on the workshop floor to ensure compliance with specifications?

- In your own experience, have you seen an example where not following a specification led to a problem? How was it resolved?

- Do you think product specifications should be flexible or strict? Why?

Practical Activity

Provide learners with two specification sheets:

- One for a standard dining chair.

- One for a lounge chair with piping and buttoning.

Task:
Ask learners to identify:

- Key dimensions

- Materials to be used

- Cover type and pattern

- Trims or embellishments
Then, in groups, ask learners to role-play a quality control check comparing the finished product to the specification.

### KT0102: Types of Product Specifications and Their Uses

Purpose of the Topic

This topic introduces learners to the various types of product specifications used in the upholstery manufacturing process. Learners will gain the ability to identify and differentiate between sketches, technical drawings, samples, prototypes, cutting lists, and job cards, and understand their roles in guiding production, maintaining quality, and ensuring that client expectations are met.

Understanding the purpose and application of each specification type enhances the learner’s ability to interpret instructions accurately, avoid mistakes, and contribute meaningfully to production outcomes.

Content Breakdown

- Sketches

  - Informal or conceptual drawings used during the early design phase.

  - Typically used for initial communication of design intent.

  - May not contain full measurements or detailing.

- Technical Drawings

  - Detailed, scaled, and dimensioned engineering drawings.

  - Include multiple views (top, side, sectional).

  - Used for precision during frame construction, cutting, and upholstery.

  - Can be isometric, oblique, or orthographic.

- Samples

  - Material swatches, hardware pieces, or mini versions of full designs.

  - Used to approve fabric types, colours, stitching, foam density, or decorative trims.

  - Helpful in pre-production approval processes.

- Prototypes

  - One-off or trial product made to full scale.

  - Tests feasibility, design, comfort, ergonomics, and construction before mass production.

  - Used for client presentation and final approval.

- Cutting Lists

  - Tabulated sheets listing all parts to be cut from materials, including dimensions, quantities, and materials.

  - Ensure that all required frame and padding components are prepared efficiently and consistently.

- Job Cards

  - Instructional documents accompanying each production job.

  - Include task allocations, due dates, product descriptions, materials needed, and client notes.

  - Central to daily operations and traceability.

Facilitation Strategies

- Provide printed examples of each document type.

- Display side-by-side comparisons of a sketch vs. a technical drawing.

- Conduct a matching activity where learners must link document types to their uses.

- Highlight differences between conceptual, preparatory, and production-focused documents.

Example

A Cape Town furniture workshop receives an order for ten armchairs.

- The client provides a sketch with a curved arm design.

- The design team creates a technical drawing detailing dimensions, joinery, and component parts.

- A sample swatch of blue chenille is submitted and approved.

- A prototype is made and modified after client feedback.

- A cutting list is created to prepare frame components.

- Job cards are issued to the upholsterer and seamstress for tracking.

Case Study: The Missing Job Card at Makhaya Furniture Works

Background:
Makhaya Furniture Works, based in Gqeberha, was commissioned to produce 30 ottomans for a hotel lobby renovation. All production materials were delivered on time, and sketches were approved.

Problem:
The job cards were not issued, resulting in staff not knowing who was assigned to which tasks. No records were available to track the foam type required, and the wrong foam was used. The order was delayed, and the foam had to be replaced.

Teaching Point:
Even when designs and materials are in place, the absence of a single document—like a job card—can cause major operational setbacks.

Critical Thinking Questions

- What is the difference between a sketch and a technical drawing? Why is each important?

- What role does a prototype play in the quality assurance process?

- Why is a cutting list critical before production starts?

- How do job cards contribute to time management and accountability in a factory?

- Can you think of a real-life example where missing or unclear specifications caused confusion? How could it have been prevented?

Practical Activity

Provide learners with:

- A conceptual sketch

- A technical drawing

- A cutting list

- A job card

Task:
Ask learners to:

- Identify what each document communicates.

- Highlight potential problems that could arise if that document were missing.

- Present in groups how these documents work together to ensure smooth production.

### KT0103: Principles and Concepts of Interpreting Specifications and Workplace Instructions

Purpose of the Topic

The purpose of this topic is to develop the learner’s ability to interpret workplace specifications and instructions correctly and confidently. This skill is essential in determining the type, quality, size, and quantity of raw materials and consumables required during the frame preparation process in upholstery manufacturing.

This topic builds the foundation for learners to transition from reading technical documents to making operational decisions, thus improving efficiency, preventing material wastage, and ensuring compliance with product standards.

Content Breakdown

- Understanding Specifications and Workplace Instructions

  - Specifications: Detail-oriented documents such as technical drawings, cutting lists, and bill of materials.

  - Workplace Instructions: Verbal or written instructions provided by supervisors or on job cards, including special requests or changes to standard procedures.

- Key Concepts to Interpret

  - Type of material: e.g. high-density foam, birch ply, hessian cloth, elastic webbing.

  - Quality: material grade, standard vs. premium, recycled vs. virgin materials.

  - Size: thickness, length, width – based on measurement from drawings or job cards.

  - Quantity: amount of material required per unit and for the full production run.

- Reading and Extracting Information

  - Identifying materials from codes or descriptions.

  - Cross-referencing dimensions and parts with cutting lists.

  - Using symbols and abbreviations used in upholstery industry documentation (e.g. mm, m², qty, dia).

- Linking Instructions to Action

  - Planning material preparation and workflow.

  - Communicating clarifications when unclear or inconsistent information appears.

  - Maintaining traceability and stock control.

Facilitation Strategies

- Use actual job cards and cutting lists from local upholstery factories.

- Walk learners through labelled technical drawings and demonstrate how to read part numbers and dimension markers.

- Set up a simulation station where learners “pull” materials from a storeroom based on given specifications.

Example

A learner receives a job card indicating: “Back panel: 15 mm plywood, 420 mm x 670 mm – 3 units per chair.”

- The learner interprets this to mean that 3 pieces of 15 mm thick plywood, each measuring 420 mm by 670 mm, are required for each chair.

- For a batch of 10 chairs, the learner calculates: 3 x 10 = 30 panels total.

If the learner fails to interpret the quantity correctly and cuts only 20 panels, production will halt during assembly.

Case Study: The Misread Drawing at Sizwe Upholstery Co.

Background:
Sizwe Upholstery Co. in Pretoria received an order for a set of modular couches. The frame preparation team misread the cutting list and used foam of 75 mm thickness instead of the specified 50 mm.

Impact:

- The overall height of the couch increased, altering the intended ergonomic design.

- The client rejected the prototype during inspection.

- Additional foam had to be purchased, and the incorrect pieces were discarded, resulting in a financial loss.

Teaching Point:
Interpreting specifications accurately is not only a skill but a critical responsibility that prevents wastage, redesigns, and client dissatisfaction.

Critical Thinking Questions

- How can a small error in interpreting size or quantity affect the entire upholstery process?

- What systems can be implemented to double-check interpretation of job instructions before cutting materials?

- How does understanding materials (e.g. foam density or fabric stretch) help in interpreting workplace specifications effectively?

- What should you do if you do not understand part of the drawing or instruction?

Practical Activity

Materials Required:

- Sample job card

- Cutting list

- Technical drawing with part dimensions

- Store inventory list

Task:

- In small groups, learners must calculate how many of each material is required for a batch of 5 chairs.

- They must identify the correct type and grade of each item and complete a materials requisition form.

Reflection:
Ask learners to reflect on why accuracy in interpretation leads to efficiency and reduced costs.

### KT0104: Frame and Upholstery Components Construction and Parts Identification

Purpose of the Topic

This topic introduces learners to the construction and identification of the internal frame structure and upholstery components of a furniture product. It builds foundational knowledge for understanding how different parts work together to provide support, shape, comfort, and aesthetics in upholstered furniture. Learners will be introduced to common frame types, joinery, reinforcement methods, and upholstery layering—essential for assembling, covering, and inspecting furniture items.

This topic strengthens the learner’s ability to communicate accurately using industry terminology, follow production instructions, and visually inspect parts during and after assembly.

Content Breakdown

- Frame Construction Basics

  - Frame materials: Hardwood (e.g. meranti, beech), softwood (e.g. pine), plywood, MDF.

  - Types of frames: Armchairs, sofas, dining chairs, ottomans.

  - Joinery methods: Butt joints, dowel joints, mortise and tenon, corner blocks, and metal brackets.

  - Reinforcements: Webbing, corner blocks, glue application.

- Main Upholstery Components

  - Arms – curved or square, padded or unpadded.

  - Seat base – springs (zig-zag, coil), tensioned webbing, foam base.

  - Backrest – loose cushion or fixed frame, vertical or inclined.

  - Support components – stretcher rails, legs, and feet (fixed or detachable).

  - Cover materials – calico, foam, batting, outer fabric or leather.

- Component Labelling and Identification

  - Understanding diagrams and exploded views.

  - Recognising part names and functions.

  - Interpreting location codes (e.g. "LF" for left front leg).

  - Common abbreviations (e.g. "BkPnl" for back panel, "SdFrm" for side frame).

- Why Part Identification Matters

  - Facilitates correct assembly order.

  - Prevents mismatch of left/right or front/back components.

  - Helps quality control teams trace assembly errors.

Facilitation Strategies

- Display physical examples or mock-up frame sections showing joints and labels.

- Use cutaway diagrams to show inside layers (frame, webbing, foam, fabric).

- Provide learners with blank component diagrams and ask them to label each part.

- Facilitate a hands-on matching game: learners match names of components with physical parts.

Example

In the construction of a tub chair:

- The frame includes a curved backrest connected to side arms using dowel joints.

- The seat is fitted with elastic webbing and medium-density foam.

- The legs are bolted onto corner blocks at the base.

- The cover includes a Dacron layer under a printed linen fabric, stapled to the inside base.

Identifying each component correctly ensures accurate construction and reduces assembly errors.

Case Study: Component Mislabelling at Ikhaya Interiors

Background:
Ikhaya Interiors, based in Johannesburg, outsourced the cutting of frames for a 50-unit sofa order. The parts were delivered unlabelled.

Problem:
During assembly, staff struggled to differentiate between left and right arms and inner vs. outer rails. Several frames were assembled incorrectly and needed to be disassembled, causing delays and additional labour costs.

Resolution:
The company introduced a colour-coded labelling system and trained staff on part identification from component drawings.

Teaching Point:
Correct labelling and identification of frame and upholstery components prevents costly errors and supports smooth workflow.

Critical Thinking Questions

- Why is it important to know the names and positions of all parts in a frame?

- How can incorrect identification of parts affect the structural integrity of a furniture piece?

- What strategies can be used in your workplace to improve part identification?

- How do labelling systems and diagrams support new team members during production?

Practical Activity

Task:
Provide learners with:

- A diagram of an unlabelled sofa frame.

- A list of parts: seat rail, backrest, arm support, cross brace, leg, and seat panel.

Activity:

- Learners must correctly label each part on the diagram.

- Then, match each part to a brief written function (e.g. “supports back cushion”).

Group Extension:
In pairs, learners take turns describing the function of each component and how it contributes to the structure or comfort of the final product.

### KT0105: Compliance and Non-Compliance

Purpose of the Topic

This topic enables learners to understand the meaning, importance, and implications of compliance and non-compliance in the upholstery manufacturing process. Learners will explore how adherence to specifications, quality standards, safety guidelines, and workplace procedures impacts product quality, customer satisfaction, legal accountability, and workplace efficiency.

By mastering this topic, learners will be able to explain what it means for a frame or upholstered product to be compliant or non-compliant, how to identify non-compliance, and what corrective steps should be taken.

Content Breakdown

- Definition of Compliance

  - Conformance to specifications, technical drawings, safety standards, materials requirements, and client instructions.

  - Meeting internal company quality benchmarks and external regulatory standards.

- Definition of Non-Compliance

  - Any deviation from approved specifications or procedures.

  - Use of incorrect materials, poor workmanship, incorrect assembly, or failure to meet safety and design standards.

- Areas Where Compliance is Required

  - Frame dimensions and construction.

  - Material selection and usage (e.g. foam density, fabric type).

  - Safety (e.g. flame retardancy, structural integrity).

  - Aesthetic standards (e.g. symmetry, fabric alignment, seam quality).

  - Labelling and documentation.

- Causes of Non-Compliance

  - Misreading of specifications or drawings.

  - Lack of training or experience.

  - Use of incorrect or poor-quality materials.

  - Time pressure or negligence.

  - Absence of quality control checks.

- Compliance Systems

  - Standard Operating Procedures (SOPs).

  - Quality inspection points.

  - Feedback loops and non-conformance reports.

Facilitation Strategies

- Facilitate a gallery walk showing compliant vs. non-compliant furniture images.

- Use checklists and mock inspections to simulate quality control activities.

- Invite a local manufacturer or QC supervisor to share real examples of compliance issues.

Example

During an internal quality audit at a Cape Town workshop, a batch of armchairs was flagged for non-compliance. The issue: the seat height was 470 mm instead of the required 450 mm. Investigation revealed that the incorrect leg length was used due to a supplier substitution error that was not verified against the job card.

Consequence: Client return, rework, cost implications, and reputational damage.

Case Study: Compliance Failure at Coastal Comforts

Background:
Coastal Comforts, a mid-sized upholstery factory in Gqeberha, won a contract to supply custom recliners to a wellness spa chain. The spa specified water-resistant, fire-retardant vinyl and foam of 30/200 density.

Issue:
A supplier delivered standard vinyl and medium-density foam. The production team, unaware of the specification, used the incorrect materials.

Impact:

- The spa noticed the difference during delivery.

- The entire batch was rejected due to non-compliance with safety regulations.

- Coastal Comforts had to absorb the costs and lost the client contract.

Resolution:
The company instituted a pre-production compliance checklist and trained staff on interpreting and cross-verifying material specs.

Critical Thinking Questions

- What are the possible consequences of non-compliance in furniture upholstery?

- How can a team ensure that the materials used meet the client and safety specifications?

- What role does each employee play in maintaining compliance?

- What steps should be taken when non-compliance is discovered during production?

- What are the benefits of building a culture of compliance in the workplace?

Practical Activity

Task:
Provide learners with a compliant specification for a small ottoman, including materials, dimensions, and covering details.

Scenario:
They are told the final product was made using a different foam and a fabric not listed in the job card.

Activity:

- Identify all points of non-compliance.

- Suggest how the team should have caught the issue.

- Draft a Non-Conformance Report (NCR) and propose corrective actions.

Extension:
Divide learners into two groups: “Compliance Control” and “Production Team”. Role-play a quality assurance meeting where issues are reported and solutions are discussed.

### KT0106: Consequences of Prepared Frames and Covered Furniture Not Meeting Specified Standards

Purpose of the Topic

This topic addresses the impact of non-compliance on the upholstery process and the broader business environment. Learners will explore what happens when prepared frames or upholstered furniture fail to meet technical, aesthetic, safety, or client-specified standards. They will understand the implications for product quality, client satisfaction, production timelines, cost, and company reputation.

The aim is to encourage learners to adopt a quality-focused mindset, understand their role in upholding standards, and learn to identify and correct issues before they escalate.

Content Breakdown

- What Constitutes Failure to Meet Standards

  - Incorrect frame dimensions or joinery.

  - Inconsistent foam thickness or incorrect density.

  - Poor fabric alignment, seam puckering, or sagging padding.

  - Use of incorrect materials or deviations from job card instructions.

- Immediate Consequences

  - Product rejection by quality control.

  - Delays in production timelines.

  - Rework required, causing material and time wastage.

- Client-Facing Consequences

  - Order rejection or returns.

  - Dissatisfaction and damage to customer relationships.

  - Loss of repeat business or cancellation of future orders.

- Operational and Financial Impact

  - Increased production costs due to rework and waste.

  - Resource shortages for other jobs.

  - Lower staff morale and productivity.

- Legal and Regulatory Consequences

  - Fines or legal action if furniture fails safety tests.

  - Non-compliance with flame-retardant or structural regulations may breach contractual agreements or national safety codes.

Facilitation Strategies

- Create a timeline map of how one small error affects subsequent departments.

- Facilitate a group discussion around quality vs. speed in manufacturing.

- Use role-play to simulate a meeting with a dissatisfied client or quality control manager.

Example

A lounge chair is covered using the wrong batch of velvet, which has a slightly different sheen and texture. Although the difference is subtle, the client notices it during delivery. The batch is rejected, and the factory must reorder material, delay delivery, and redo all covers—costing R6,000 in materials and two days in lost time.

Case Study: Failure to Meet Aesthetic Standards at Imvelo Furniture

Background:
Imvelo Furniture, based in Polokwane, delivered a batch of upholstered bench seats to a retail store. The store’s designer had approved a sample showing neatly aligned vertical stripes on the fabric.

Issue:
The production team did not use pattern-matching techniques. Stripes were misaligned across seams and panels.

Impact:

- The retail store refused to display the benches.

- Imvelo had to recall the products, remove the covers, and reupholster all units.

- The store published negative feedback on social media.

Teaching Point:
This scenario highlights the reputational and financial risks of not meeting aesthetic and technical standards.

Critical Thinking Questions

- What could happen if the wrong fabric is used, but the furniture is still functional? Should it still be considered non-compliant?

- Who should be held accountable when a product does not meet the required specifications?

- How can errors be detected earlier in the upholstery process?

- What checks can be implemented between departments to prevent non-compliance from reaching the client?

- Why is it important to balance quality standards with production deadlines?

Practical Activity

Scenario:
You are part of a team that has completed a batch of ottomans. A client returns two items, citing the following:

- Uneven leg height.

- Stitching coming undone at corners.

- Foam compressing too quickly.

Task:

- As a group, categorise these into technical, aesthetic, and functional non-compliance.

- Discuss what may have caused each issue.

- Propose corrective and preventive actions.

Extension:
Draft a Quality Improvement Report based on the case and present it as if to management.

## 📝 Integrated Assessment: KM-03-KT01

Knowledge Module: Upholstered Furniture Basic Cover Fitting
Topic: Product Specifications
Total Marks: 50
Duration: 60 Minutes
Assessment Weighting: 10%
Level: NQF Level 2

Section A: Multiple Choice Questions (1 mark each × 5 = 5 marks)

Instructions: Choose the correct answer and write only the letter (A–D) next to the question number.

- Which of the following is not a type of drawing used in upholstery specifications?
A. Isometric
B. Oblique
C. Botanical
D. Orthographic

- A job card is mainly used to:
A. Represent the final furniture finish
B. Allocate tasks, materials and deadlines
C. Sketch a design concept
D. Measure fabric stretch

- A typical raw material standard specifies:
A. Company logo placement
B. Wood grain direction
C. Foam density and thickness
D. Upholstery theme

- Which symbol is typically found on engineering drawings?
A. Musical note
B. Welding seam
C. Zigzag stitch
D. Tension loop

- Non-compliance in a finished chair may lead to:
A. Faster delivery
B. Increased client trust
C. Product rejection
D. Creative appreciation

Section B: Short Answer Questions (3 marks each × 5 = 15 marks)

- Name three types of upholstery product specifications and briefly describe how each is used in the production process.

- List three standard symbols or conventions commonly found on upholstery engineering drawings and explain what they represent.

- What is meant by "compliance" in the upholstery manufacturing process? Provide one example.

- Define "non-compliance" and explain how it could affect the covering phase of a furniture product.

- Why is it important to follow drawing standards and material specifications in frame preparation?

Section C: Practical Interpretation Task (10 marks)

Instructions:
Study the simplified sketch below (provided by the assessor in class), which includes dimensions, part labels, and a materials list. Then answer the questions:

- Identify and describe the view used in the sketch (e.g. isometric, orthographic).

- Based on the dimensions, calculate how many metres of fabric would be needed to cover five identical seat panels, each measuring 400 mm × 450 mm.

- On the drawing, the foam is marked as "28/200". Explain what this specification means in terms of quality and comfort.

- A sectional view shows part of the internal frame. What is the purpose of a sectional drawing in this context?

- The sketch uses the abbreviation “TYP.” next to corner blocks. What does this mean, and why is it important in interpreting the drawing?

Section D: Case Study (20 marks)

Scenario:
At Mhlobo Upholstery, a team was tasked to manufacture a batch of ottomans. The specification required high-resilience foam (32/200) and an oblique frame design. During the final inspection, it was discovered that the foam used was standard (25/200), and the frame corners were not aligned to the specified angles. The customer rejected the full batch.

Answer the following:

- Identify two types of non-compliance in this case (4 marks).

- What could be the possible reasons for the team failing to meet the specification? (4 marks)

- What are the functional and aesthetic consequences of incorrect foam and frame angles in this scenario? (4 marks)

- What corrective steps should be taken in the workshop to avoid such errors in future? (4 marks)

- Why is comparing different types of specifications important in avoiding such a failure? (4 marks)

✅ Model Answers and Marking Memo

| Question | Model Answer Summary | Marks |
| --- | --- | --- |
| 1–5 | C, B, C, B, C | 1 mark each |
| 6 | Sketches (early design), Technical Drawings (detailed construction), Cutting Lists (material use) | 3 |
| 7 | E.g. ⌀ (diameter), ⊥ (perpendicular), R (radius) | 3 |
| 8 | Adhering to specified requirements – e.g. correct foam density used | 3 |
| 9 | Deviating from required standards – e.g. wrong materials cause product rejection | 3 |
| 10 | Ensures accuracy and avoids rework or waste | 3 |
| 11 | Oblique view | 2 |
| 12 | 5 panels × (0.4 × 0.45) = 0.9 m² → at least 1 linear metre (allowing for waste) | 2 |
| 13 | Density = 28 kg/m³, hardness = 200N → Medium support | 2 |
| 14 | Shows internal components for clarity and assembly | 2 |
| 15 | TYP. = typical (used in all similar parts) | 2 |
| 16 | Incorrect foam, wrong frame angles | 4 |
| 17 | Misinterpretation of drawings, lack of QC checks | 4 |
| 18 | Foam too soft → sinks quickly; frame angle → misalignment | 4 |
| 19 | Train staff, verify materials, cross-check job cards | 4 |
| 20 | Prevents material mismatch and production errors | 4 |

📊 Assessment Rubric (Summary)

| Criteria | Outstanding (5) | Competent (3–4) | Partially Achieved (1–2) | Not Achieved (0) |
| --- | --- | --- | --- | --- |
| Knowledge of Specification Types | Clearly identifies and explains all with examples | Identifies most with correct uses | Limited recall; few relevant uses | No understanding shown |
| Interpretation of Drawings | Fully correct, identifies views, labels, calculations | Mostly accurate, minor errors | Incomplete or vague responses | Cannot interpret at all |
| Understanding of Compliance Concepts | Clear, real-world examples; shows insight | Basic understanding with example | General idea only | No understanding |
| Application in Case Study | Insightful analysis and recommendations | Reasonable suggestions with some depth | Generic answers only | Off-topic or no answer |

### Facilitator Assessment Briefing

Knowledge Module: KM-03 – Upholstered Furniture Basic Cover Fitting
Topic: KT0101–KT0106 – Product Specifications
Assessment Instrument Type: Integrated Assessment using a variety of instruments
Qualification: Furniture Upholsterer (SAQA ID 103199)
NQF Level: 2
Credits: 8
Weighting: 10%

1. Purpose of the Assessment

This integrated assessment is designed to evaluate learners’ ability to:

- Identify and interpret various types of product specifications used in the upholstery industry.

- Understand basic principles of engineering drawings and compliance concepts.

- Apply theoretical knowledge to practical and scenario-based tasks.

The assessment addresses Internal Assessment Criteria (IAC0101–IAC0105) and integrates knowledge across visual interpretation, materials understanding, and compliance evaluation.

2. Assessment Duration and Format

- Duration: 60 minutes

- Total Marks: 50

- Assessment Sections:

  - Section A: Multiple Choice (5 marks)

  - Section B: Short Answer (15 marks)

  - Section C: Practical Interpretation (10 marks)

  - Section D: Case Study Scenario (20 marks)

3. Assessment Preparation

Before the Assessment:

- Prepare printed copies of the assessment for each learner.

- Prepare sample sketches or simplified engineering drawings for Section C.

- Ensure the assessment room is quiet, well-lit, and free from distractions.

- Brief learners on:

  - Assessment purpose and duration

  - Instructions for each section

  - Permissible materials (e.g. calculators, rulers for Section C)

  - Rules regarding academic honesty

4. Assessment Administration

During the Assessment:

- Monitor time and progress.

- Clarify instructions if learners request support (no content clarification permitted).

- Ensure learners complete all sections independently.

5. Marking Guidelines

Use the model answers and marking memo provided. For each section:

- Section A: Mark strictly using the answer key (1 mark per correct answer).

- Section B: Allocate 3 marks per question using the model responses.

- Section C: Award partial marks where appropriate. For calculation or reasoning questions, show mark allocation per step.

- Section D: Use the full rubric to assess depth of analysis, realism of proposed actions, and alignment with compliance knowledge.

6. Assessment Rubric Application

Apply the rubric as follows:

| Criteria | Outstanding (5) | Competent (3–4) | Partially Achieved (1–2) | Not Achieved (0) |
| --- | --- | --- | --- | --- |
| Knowledge of Specification Types | Clearly identifies, explains, and applies all | Identifies most types with correct context | Incomplete recall or misapplication | Unable to name or describe any |
| Interpretation of Drawings | Accurate reading, view ID, dimensions, symbols | Mostly correct, with minor errors | Partially correct, lacks clarity | Cannot interpret sketches |
| Understanding of Compliance Concepts | Applies with examples and understanding | Understands with minor gaps | General awareness without clear example | No awareness or examples |
| Case Study Application | Logical, contextually grounded, realistic | Provides adequate solutions with minor gaps | Generic, lacks relevance or depth | Off-topic or absent |

7. Post-Assessment Feedback

- Return scripts and provide individual feedback within 3 working days.

- Highlight:

  - Strengths demonstrated (e.g. drawing interpretation)

  - Areas for improvement (e.g. explanation of compliance)

- Provide remediation opportunities if learners scored below 50%.

8. Recording and Moderation

- Store assessment scripts and completed rubrics in learners’ folders

- Submit assessment summary sheets to the internal moderator.

- The internal moderator must verify:

  - Fairness and consistency of marking

  - Use of appropriate model answers and rubrics

  - Adherence to assessment policy

## KM-03-KT02: Upholstery Raw Materials and Consumables Types and Characteristics

Weighting: 20% | NQF Level: 2

1. Purpose of the Knowledge Topic

The purpose of this Knowledge Topic is to equip learners with foundational and applied knowledge of the raw materials and consumables commonly used in upholstery processes. Learners will gain the ability to identify, evaluate, and correctly apply materials and consumables for various stages of upholstery production—such as foaming, springing, webbing, and cover fitting—while considering material properties, standards, quantities, and workplace instructions.

This knowledge is essential for maintaining quality standards, ensuring production efficiency, and contributing to health and safety in the upholstery environment. It forms a critical link between interpreting design specifications and executing the practical upholstery processes.

2. Key Knowledge Areas (Topic Elements)

This Knowledge Topic comprises the following learning elements:

- KT0201: Furniture raw materials with reference to natural and manmade fabrics and finishings

- KT0202: Location of raw materials and consumables

- KT0203: Main upholstery material types including woven plain, woven patterned, printed cretonne, pile, knitted jersey, coated fabrics, and animal skins

- KT0204: Types, sizes, quantity and quality of consumables for cover fitting, finishing and trimming

- KT0205: Density, thickness and types of foam required for the foaming up process

- KT0206: Types of padding used for covering coil springs

- KT0207: Consumables required for the foaming up process, including adhesives, solvents, profile jigs and templates

- KT0208: Using profile jigs and templates

- KT0209: Fabrics for covering webbing, springs, foam and padding including calico, hessian, polypropylene, Dacron, and flock

- KT0210: Types and gauges of springs for the knocker-on process, including coil and zig-zag springs

- KT0211: Consumables for the springing up process including clips, nails, staples, edge rolls, twine and cord

- KT0212: Types of webbing for the knocker-on process, including manmade elastic, rubber webbing, and natural fibres like jute and hessian

- KT0213: Labels, codes, and traceability systems in upholstery materials

- KT0214: Principles and concepts of safe materials handling and storage methods

3. Internal Assessment Criteria and Weighting

This Knowledge Topic carries a total weighting of 20% and will be assessed using the following Internal Assessment Criteria (IACs):

- IAC0201: Reasoning the use of specifications and workplace instructions to determine the type, quality, size and quantity of raw materials and consumables

- IAC0202: Listing and defining characteristics of raw materials used in furniture making

- IAC0203: Identifying consumables used in covering processes and calculating required quantities

- IAC0204: Describing typical defects and faults in covering materials and analysing the reporting and rectification process

- IAC0205: Identifying various frame types and foundation materials

- IAC0206: Identifying types and gauges of springs for the knocker-on process

- IAC0207: Identifying consumables for the springing up process

- IAC0208: Evaluating types of webbing used for the knocker-on process

- IAC0209: Identifying consumables for the webbing-up process

- IAC0210: Describing methods for identifying foam density and thickness

- IAC0211: Listing consumables for the foaming up process

- IAC0212: Listing raw materials used in padding and covering coil springs

- IAC0213: Listing fabrics used for covering webbing, springs, foam and padding

- IAC0214: Reasoning principles and concepts of safe materials handling and storage methods

4. Application in Upholstered Furniture Manufacturing

This topic lays the groundwork for practical upholstery tasks by establishing knowledge of what materials are used, how they are selected, and how they are managed on the workshop floor. By understanding the properties, handling requirements, and usage specifications of both raw materials and consumables, learners can make informed decisions that affect product quality, workplace safety, and operational efficiency.

Correct application of this knowledge will ensure that each stage of the upholstery process—whether foaming, springing, webbing, or trimming—is completed with the correct tools and materials in alignment with the job specification. It also contributes to reducing waste, avoiding material faults, and achieving desired aesthetics and functionality in upholstered products.

### KT0201: Furniture Raw Materials with Reference to Natural and Manmade Fabrics and Finishings

Purpose of the Topic

This topic introduces learners to the various raw materials used in furniture upholstery, specifically focusing on the characteristics, advantages, limitations, and applications of natural and manmade fabrics and finishings. Learners will be able to differentiate between material types, understand their typical uses in upholstery, and explain how these materials impact product aesthetics, durability, comfort, and cost.

Key Content Areas

- Definition of Raw Materials in Upholstery

  - Upholstery raw materials refer to the textiles and coverings applied to furniture surfaces.

  - These materials are chosen based on function, design brief, durability, and client specification.

- Types of Raw Materials

  - Natural Fabrics

    - Cotton, linen, wool, silk, jute, hemp, leather

    - Biodegradable, breathable, absorbent

    - Often more expensive; affected by humidity and sunlight

  - Manmade Fabrics

    - Polyester, acrylic, nylon, rayon, polypropylene, vinyl, faux leather

    - Durable, stain-resistant, often more cost-effective

    - Suitable for high-traffic or commercial settings

- Typical Finishings

  - Flame retardant treatments

  - Water and stain resistance coatings

  - Anti-microbial and anti-fungal treatments

  - UV protection for outdoor applications

- Factors Influencing Fabric Selection

  - End-use environment (e.g. home, hospitality, healthcare)

  - Comfort and tactile quality

  - Wear resistance and rub count

  - Colour fastness and shrinkage

  - Budget and availability

Examples of Fabric Applications

- Natural Fabric Example:
A high-end lounge suite at a boutique hotel may be upholstered in linen or full-grain leather, which offers comfort and natural elegance.

- Manmade Fabric Example:
A waiting room in a government office might use polyester or vinyl, which is easy to clean and resistant to staining and wear.

Case Study: Fabric Selection at Ubuntu Interiors

Background:
Ubuntu Interiors, a Johannesburg-based upholstery studio, received a contract to refurbish 40 restaurant chairs. The designer proposed using jute for its rustic appearance.

Problem:
Jute frayed under frequent use and stained easily when exposed to wine or sauces.

Solution:
The team recommended a polypropylene-blend fabric with a jute-like weave. It met the aesthetic brief while offering increased durability, stain resistance, and ease of maintenance.

Learning Point:
Understanding the functional properties of raw materials helps prevent costly mistakes and ensures client satisfaction.

Critical Thinking Questions

- Why might a client prefer natural fabrics over manmade ones in a residential setting?

- What are some risks of using untreated natural fibres in high-traffic commercial spaces?

- How can the choice of raw material impact the durability and maintenance of the final furniture product?

- What questions would you ask a supplier to confirm if a fabric meets fire retardancy requirements?

- How does the source of the material (local vs imported) affect production planning and costing?

Practical Classroom Activity

Materials Needed:
Fabric swatches (natural and manmade), two sample chairs (one upholstered in leather or cotton, another in vinyl or polyester)

Instructions:

- Learners must identify whether each swatch is natural or manmade and justify their answer.

- Learners inspect both chairs and list the benefits and drawbacks of the materials used in each.

- As a group, discuss which chair would be more suitable for a school library and why.

### KT0202: Location of Raw Materials and Consumables

Purpose of the Topic

The purpose of this topic is to ensure that learners understand how raw materials and consumables are stored, accessed, and managed within an upholstery manufacturing environment. Correctly identifying and locating materials not only supports production efficiency but also contributes to safety, cost control, traceability, and workflow continuity.

By the end of this topic, learners should be able to describe how upholstery workshops organise and label materials, how to read storage maps or stock references, and what practices are followed to retrieve and return items to designated storage areas.

Key Content Areas

- Definition of Raw Materials and Consumables

  - Raw materials: Primary materials used in the manufacture of furniture, such as fabric rolls, foams, wood panels, and padding materials.

  - Consumables: Supporting items that are used up during the process, such as staples, adhesives, nails, trims, labels, and twine.

- Importance of Organised Material Location

  - Saves time and prevents production delays.

  - Reduces loss and wastage through better accountability.

  - Enhances workplace safety by keeping paths and surfaces clear.

  - Facilitates accurate inventory control and reordering.

- Common Storage Systems

  - Open Racking: For large fabric rolls, foam sheets, or timber boards.

  - Bins or Cubbies: For small items like clips, nails, and fasteners.

  - Secured Cabinets: For solvents, adhesives, and flammable materials.

  - Labelled Shelving: Using item codes or barcode systems.

- Location Indicators

  - Physical signs and labels (e.g., “Rack A – Foam | Rack B – Adhesives”).

  - Job cards or picking slips with reference numbers.

  - Warehouse layout diagrams for larger workshops.

  - Stock management software in modern or high-volume upholstery firms.

- Link to Traceability

  - Correct location tracking supports traceability (e.g., in identifying fabric batches or foam densities).

  - Essential for quality control and client assurance.

Example of Material Organisation

A local upholstery business stores raw materials in zones:

- Zone A: Rolled fabrics sorted by type (e.g., vinyl, cotton, linen)

- Zone B: Foam sheets and cut-offs, stacked and wrapped

- Zone C: Small consumables in bins, labelled with size and quantity (e.g., “Staples – 12 mm”)

- Zone D: Adhesives and solvents, stored in a ventilated, locked cabinet

- Items are tracked using a simple spreadsheet updated daily

Case Study: Inefficiency at Khanyisa Upholstery

Background:
Khanyisa Upholstery in Gqeberha struggled to meet delivery timelines despite having sufficient stock on hand.

Issue:
The workshop had no fixed storage layout. Workers spent up to 30 minutes searching for the correct fabric or running to the store room multiple times during a single task.

Resolution:
Management created a zoned layout for raw materials and introduced colour-coded labels. A laminated map was placed in the main production area, and staff were trained to return materials to correct locations after use.

Result:
Production time improved, wastage dropped, and reordering was more accurate due to improved inventory records.

Critical Thinking Questions

- Why is it important to store foam and fabric in separate areas?

- How can poor organisation of consumables affect production costs?

- What are the risks of storing adhesives and flammable solvents improperly?

- In your opinion, should storage areas be accessible to all workers or only certain team members? Why?

- How would you explain the importance of returning materials to their original location to a new team member?

Practical Classroom Activity

Task: Simulate a storage environment using labelled boxes and dummy materials.

Setup:
Prepare 5–6 labelled “storage zones” using cardboard boxes:

- Fabric Rolls

- Foams

- Small Tools and Trims

- Adhesives and Chemicals

- Threads and Fasteners

Instructions:

- Divide learners into teams and provide them with a “job card” listing materials needed.

- Learners must locate and “collect” materials from the correct boxes in the correct sequence.

- Learners return unused items to their original box and explain why this step is important.

Reflection:
Facilitate a discussion about the benefits of organised material storage and lessons learned during the activity.

### KT0203: Main Upholstery Material Types

Focus: Woven plain, woven patterned, printed cretonne, pile, knitted jersey, coated fabrics, and animal skins

Purpose of the Topic

This topic introduces learners to the main categories of materials used to upholster furniture. Learners will develop the ability to distinguish between different material types based on appearance, texture, weave structure, durability, and appropriateness for specific furniture applications.

Understanding these materials is vital for recommending suitable upholstery options to clients, aligning material selection with production needs, and ensuring the functionality and aesthetic appeal of upholstered products.

Key Content Areas

- Woven Plain Fabrics

  - Made by weaving threads in a crisscross pattern (e.g. canvas, calico)

  - Strong and durable; often used as linings or base cloths

  - Uniform texture and appearance

- Woven Patterned Fabrics

  - Woven with decorative patterns or motifs (e.g. jacquard, brocade)

  - Provide visual interest and luxurious finishes

  - Used for feature chairs, headboards, and high-end applications

- Printed Cretonne

  - Cotton or cotton-blend fabric with printed floral or graphic designs

  - Lightweight, breathable, and cost-effective

  - Common in residential furniture and décor pieces

- Pile Fabrics

  - Characterised by a raised surface or “pile” (e.g. velvet, corduroy)

  - Rich texture and visual depth

  - Requires directional application due to shading and nap

- Knitted Jersey

  - Stretchy fabric produced by knitting loops (e.g. polycotton jersey)

  - Soft and flexible, but less durable for high-traffic furniture

  - Typically used for loose covers or temporary upholstery

- Coated Fabrics

  - Fabrics treated with plastic coatings such as PVC or PU

  - Water-resistant, easy to clean, and flame-retardant options available

  - Ideal for healthcare, hospitality, and children’s furniture

- Animal Skins

  - Includes genuine leather and sheepskin

  - Durable, high-value, with natural aesthetic and comfort

  - Requires specific storage and care due to sensitivity to humidity and light

Example Applications by Type

| Material Type | Application Example |
| --- | --- |
| Woven plain | Base cloth in deep-buttoned headboards |
| Woven patterned | Feature chairs in hotel lobbies |
| Printed cretonne | Scatter cushions and bedroom occasional chairs |
| Pile fabrics | Classic velvet armchairs in formal lounges |
| Knitted jersey | Removable slipcovers for children’s furniture |
| Coated fabrics | Clinic waiting room benches, restaurant banquettes |
| Animal skins | Executive leather office chairs, luxury recliners |

Case Study: Inappropriate Fabric Selection at Ntombi Upholstery

Background:
Ntombi Upholstery in Durban was commissioned to upholster a set of dining chairs for a family home. The client chose a pile velvet for its rich appearance.

Issue:
Within two months, the chairs showed signs of pile crushing and shading where users sat most often. The family was unhappy with the worn appearance.

Resolution:
The workshop replaced the pile fabric with a woven patterned poly-blend, better suited to high-frequency use and easier to maintain.

Learning Point:
Material type must be matched not only to appearance, but also to usage expectations and environmental exposure.

Critical Thinking Questions

- What are the advantages of using coated fabrics in a hospital or clinic setting?

- Why might a pile fabric be unsuitable for heavy-use furniture like restaurant seating?

- How does the directionality of a fabric (e.g. nap in velvet) influence how it should be cut and applied?

- In what context would a knitted jersey be an appropriate material for upholstery?

- What special considerations must be taken into account when using natural leather for upholstery?

Practical Classroom Activity

Task: Material Type Identification and Application Matching

Materials Needed: Swatches or samples (or high-quality images if physical samples unavailable) of:

- Canvas (plain woven)

- Jacquard (patterned woven)

- Printed cretonne

- Velvet (pile)

- Knitted jersey

- Vinyl (coated)

- Leather (animal skin)

Instructions:

- Learners must group the materials by type (woven, pile, coated, etc.).

- Each learner selects one material and describes:

  - Its construction

  - A suitable furniture application

  - One advantage and one limitation

- Facilitate a group discussion on which materials would be best suited for:

  - A family sofa

  - A waiting room bench

  - A decorative occasional chair

  - A recliner in a luxury home

### KT0204: Types, Sizes, Quantity and Quality of Consumables Needed for Cover Fitting, Finishing and Trimming

Purpose of the Topic

The purpose of this topic is to enable learners to identify the common consumables used in the upholstery processes of cover fitting, finishing, and trimming. Learners will explore the different types, sizes, and qualities of materials and develop the ability to determine appropriate quantities based on the work specification.

This knowledge is crucial for ensuring that upholstered furniture is finished professionally, securely, and to the desired aesthetic standard.

Key Content Areas

- Definition of Consumables

  - Items that are used up during the production process and require regular restocking.

  - Not reused or recovered once applied.

- Common Consumables for Cover Fitting

  - Staples (e.g. 10 mm, 12 mm, 14 mm) – used to secure fabric to the frame.

  - Tacks and Nails – for areas requiring extra grip or reinforcement.

  - Upholstery Twine – for hand stitching or securing springs during cover application.

  - Tack Strips (metal or cardboard) – to create clean edges without visible fasteners.

  - Glue and Adhesives – for foam or fabric fixing, especially on curves or difficult areas.

  - Edge Rolls – protect and soften the front edge of arms and seat frames.

  - Cord – used for piping detail.

- Consumables for Finishing and Trimming

  - Piping cord – gives a decorative edge and conceals stitching.

  - Gimps and Braids – applied over staples and tacks for decoration.

  - Decorative Nails – single or continuous strips; enhance the design and hide fastenings.

  - Dust Covers and Bottom Cloths – lightweight fabrics like black cambric used to finish the underside.

  - Velcro or Zip Closures – used in removable covers or panelled applications.

- Selecting the Correct Size and Quantity

  - Based on:

    - Type of furniture (e.g. ottoman vs. high-back armchair).

    - Type of material being applied.

    - Number of units to be completed.

    - Reinforcement or finishing needs.

  - Excess use of staples or low-quality tacks can damage frames or lead to poor appearance.

- Quality Considerations

  - Staples and nails should be rust-resistant.

  - Trims must match the design specification in colour, size, and texture.

  - Adhesives must be appropriate for upholstery and compliant with safety regulations.

Example

| Consumable | Size | Application |
| --- | --- | --- |
| Staples | 12 mm | Covering seat panel corners |
| Upholstery tacks | 15 mm | Fixing calico to seat base |
| Decorative nail strips | 15 mm head size | Finishing arm fronts |
| Piping cord | 4 mm | Cushion edge trim |
| Dust cover fabric | 80 gsm | Underneath lounge chairs |
| Tack strip | 1.2 m metal | Edge finishing for visible seams |

Case Study: Quality Trimming at Thando Seating Solutions

Background:
Thando Seating Solutions in Pretoria was awarded a contract to produce 30 decorative armchairs for a boutique hotel. The job card required antique brass nail strips for finishing arm fronts and black bottom cloth underneath each chair.

Issue:
The production assistant mistakenly ordered chrome-finished nail heads and non-woven polypropylene bottom cloth.

Resolution:
The procurement team corrected the order. Workers were retrained to cross-reference the materials with the design brief before starting any trimming or finishing.

Outcome:
Consistency across the batch was restored. The client commended the improved attention to finish detail.

Critical Thinking Questions

- What problems can arise if the wrong size staples or tacks are used during cover fitting?

- How can low-quality edge rolls affect the comfort and appearance of upholstered furniture?

- Why is it important for trims like braids and decorative nails to match the client’s specification?

- What factors should be considered when calculating how many metres of dust cover fabric are needed?

- What role do consumables play in the perceived quality and value of the final product?

Practical Classroom Activity

Task: Calculate and match consumables to a furniture specification

Setup: Provide learners with:

- A furniture profile (e.g. “High-back chair with piped seat cushion and dust cover”)

- A materials list

- Sample consumables (or images)

Instructions:

- Learners list the consumables they would use and explain why.

- Estimate quantities based on:

  - Piping: cushion perimeter = 2.1 m

  - Staples: 1 staple per 3 cm around frame = approx. 120 staples

  - Bottom cloth: seat base = 0.5 m²

- Groups compare answers and justify their selections.

### KT0205: Density, Thickness and Types of Foam Required for the Foaming Up Process

Purpose of the Topic

This topic introduces learners to the physical characteristics and classifications of foam used in upholstered furniture. Learners will understand how to identify and select foam by density, thickness, and type, ensuring that each application supports the correct level of comfort, durability, and design intent.

The topic provides foundational knowledge required for working with foam in practical tasks such as cushion construction, seat builds, backrests, and padding layers during the foaming-up stage of production.

Key Content Areas

- Definition and Role of Foam

  - A lightweight, resilient padding material used to provide comfort, support, and shape in furniture.

  - Used in seat cushions, backs, arms, and various contouring applications.

- Foam Density

  - Measured in kilograms per cubic metre (kg/m³).

  - Indicates the mass of foam per unit volume – not directly related to softness or firmness.

  - Common densities:

    - 20–24 kg/m³: Light use, occasional furniture

    - 25–28 kg/m³: Medium use, general seating

    - 30–35+ kg/m³: High use, commercial seating or premium applications

- Foam Thickness

  - Refers to the vertical height of the foam sheet or block.

  - Selection depends on intended use (e.g. 50 mm for backrest vs. 100 mm for seating).

  - May be layered with other materials for contour or firmness variation.

- Foam Firmness (Indentation Force Deflection – IFD)

  - Often denoted in formats such as 28/200, where:

    - 28 = density

    - 200 = Newtons of pressure to compress by 40%

  - Higher values indicate firmer foams.

- Common Foam Types in Upholstery

  - Flexible Polyurethane (PU) Foam – widely used; available in many grades

  - Rebonded Foam – made from shredded offcuts, bonded with adhesive; very firm

  - Memory Foam – expensive; used in high-end seating or mattresses

  - High Resilience Foam (HR) – better recovery and longevity

  - Soft/Convoluted Foam – used in padding layers for comfort zones

- Foam Selection Criteria

  - Weight-bearing use (e.g. seat vs. armrest)

  - Client preference (soft vs. firm)

  - Longevity expectations

  - Cost constraints

  - Flame-retardancy or allergen resistance

Example Applications by Foam Type and Density

| Foam Grade | Density (kg/m³) | Firmness (N) | Typical Use |
| --- | --- | --- | --- |
| 20/100 | Low density | Soft | Scatter cushions, backrests (decorative) |
| 28/200 | Medium density | Medium firm | Lounge seat cushions |
| 32/250 | High density | Firm | Public seating, restaurant banquettes |
| Rebonded Foam | N/A (composite) | Very firm | Springing bases, ottoman tops |

Case Study: Foam Failure at Marula Upholstery

Background:
Marula Upholstery in Limpopo supplied a batch of armchairs for a guest lodge. The seats used 20/100 foam to save on material cost.

Problem:
Within weeks, the foam began sagging and compressing permanently, leading to discomfort and early replacement requests.

Resolution:
The workshop replaced all seat cushions with 28/200 medium-density foam, suitable for daily use. The team revised their foam selection checklist for all future projects.

Learning Point:
Incorrect foam selection, especially where comfort and resilience are essential, can damage client satisfaction and require costly rectification.

Critical Thinking Questions

- How does foam density differ from foam firmness, and why is the distinction important?

- Why is rebonded foam not suitable for backrests, even though it is very firm?

- In what situations would high resilience foam be preferred over standard PU foam?

- What are the risks of using low-density foam in commercial seating?

- How can learners test the density and firmness of a foam sample without equipment?

Practical Classroom Activity

Task: Foam Evaluation and Matching Exercise

Materials:

- Foam samples of different densities (labelled and unlabelled)

- A list of furniture components (e.g. seat cushion, backrest, ottoman top)

Instructions:

- Learners examine and physically test each foam sample for firmness and flexibility.

- Match each foam to the most suitable application from the list.

- Discuss:

  - Why certain foams were matched to specific components

  - What might happen if the wrong foam is used

- Conclude with a demonstration on how to read foam labels like "28/200" and how to measure foam thickness.

### KT0206: Types of Padding for Covering Coil Springs

Purpose of the Topic

This topic introduces learners to the various padding materials used specifically in the process of covering coil springs during upholstered furniture construction. Learners will develop an understanding of the functions of padding, the types available, and their suitability for different components of furniture that include spring-based structures.

This knowledge is essential for ensuring comfort, safety, and durability of the furniture, particularly where coil springs are used in traditional upholstery builds.

Key Content Areas

- Purpose of Padding Over Coil Springs

  - Provides a protective barrier between the coil spring system and the layers above.

  - Helps distribute weight evenly and prevents wear of foam or fabric over time.

  - Contributes to comfort and shape retention.

- Types of Padding Materials

  - Fibre Padding (e.g. polyester fibre):

    - Soft, lightweight, and adds loft.

    - Often used in backs or arms.

  - Cotton Felt (Needled Felt or Flax Felt):

    - Dense and natural; offers firm support.

    - Traditionally used over springs for durability.

  - Rubberised Hair (Coir):

    - Made from coconut fibre and natural latex.

    - Breathable, firm, and durable.

    - Common in classic and heritage upholstery.

  - Foam Sheets:

    - Often layered above padding for smoother contour.

    - Vary in thickness and density.

  - Dacron (Polyester Batting):

    - Used to smooth foam or felt edges and add softness.

  - Hessian or Burlap:

    - Used as a foundation layer to cover springs before padding.

- Layering Sequence Example

  - Coil Springs

  - Hessian base

  - Felt or coir padding

  - Foam

  - Dacron

  - Final fabric covering

- Considerations for Material Choice

  - Client comfort requirements

  - Furniture type (e.g. lounge chair vs. dining seat)

  - Budget and durability needs

  - Upholstery style (modern vs. traditional)

Example

| Padding Material | Key Property | Used For |
| --- | --- | --- |
| Cotton Felt | Dense, firm | Traditional seating over springs |
| Rubberised Hair | Breathable, durable | Heritage-style armchairs and sofas |
| Fibre Padding | Soft, light | Additional loft in arms and back cushions |
| Foam | Smooth contouring | Top layer above felt or hair |
| Dacron | Soft finish, smooth edges | Between foam and fabric for a rounded appearance |

Case Study: Padding Choices at Strand Design Studio

Background:
Strand Design Studio, a family-run upholstery business in Cape Town, was commissioned to restore a set of mid-century armchairs. The brief required the use of sustainable materials with a modern comfort upgrade.

Solution:

- The team used rubberised coir over the re-tied coil springs for structure.

- A thin foam layer was added for comfort.

- Dacron was applied before the final velvet fabric.

Client Outcome:
The result retained the traditional build but offered a smoother, more comfortable seat.

Learning Point:
Padding materials can be combined and adapted to meet both aesthetic and ergonomic goals.

Critical Thinking Questions

- Why is it important to use padding over coil springs before applying foam?

- In what types of furniture might rubberised hair be more suitable than cotton felt?

- What are the risks of using only foam directly over springs without a base layer?

- How might client preference and budget influence padding selection?

- How does padding contribute to the longevity of upholstered furniture?

Practical Classroom Activity

Task: Padding Identification and Layering Simulation

Materials:

- Samples or images of: felt, coir, foam, Dacron, hessian

- Diagrams of cross-sections of upholstered seat bases

Instructions:

- Learners examine samples and describe properties (e.g. firm, flexible, breathable).

- In groups, learners design a padding sequence for:

  - A vintage lounge chair

  - A modern ottoman

- Justify their choices based on durability, comfort, and design.

Discussion Prompt:
Compare traditional and modern layering techniques. When is a hybrid approach best?

### KT0207: Consumables Required for the Foaming Up Process, Including Adhesives, Solvents, Profile Jigs and Templates

Purpose of the Topic

This topic enables learners to identify and understand the correct consumables and tools used during the foaming up process in furniture upholstery. Learners will become familiar with the application of adhesives and solvents, and the function of profile jigs and templates in achieving accuracy, consistency, and safety during foam application.

Correct usage and understanding of these materials are essential for maintaining quality standards, minimising waste, and upholding health and safety regulations in the workshop environment.

Key Content Areas

- Definition of the Foaming Up Process

  - Involves the application and attachment of foam to prepared frames using adhesives.

  - May include shaping and trimming foam using jigs or templates before covering.

- Adhesives

  - Spray Contact Adhesive:

    - Most commonly used; allows repositioning before bonding.

    - Applied via compressor and spray gun or aerosol can.

  - Solvent-based Adhesives:

    - Stronger grip but requires ventilation due to fumes.

    - Often used in structural foam applications.

- Solvents

  - Used to clean surfaces or thin adhesive.

  - Require strict adherence to safety data sheets and PPE use.

  - Examples: acetone, mineral spirits (only in ventilated areas).

- Profile Jigs

  - Templates used to shape foam consistently according to design.

  - Prevent variation across units or batches.

  - Often made from MDF or plastic.

- Templates

  - Guide the cutting and placement of foam.

  - Particularly useful for arms, back curves, or custom-shaped cushions.

- Safety and Handling

  - Always follow manufacturer’s instructions for adhesives and solvents.

  - Store flammable materials in approved safety cabinets.

  - Use gloves, respirators, and eye protection as needed.

Example Applications

| Item | Function | Application Context |
| --- | --- | --- |
| Spray adhesive | Secures foam to wood frame | Armrests, backs, and seats |
| Solvent | Cleans overspray or nozzle blockages | Between jobs or for maintenance |
| Profile jig | Ensures identical foam shapes | Arm curves on tub chairs |
| Template | Assists with foam layout for batch cutting | Custom mattress toppers or modular cushions |

Case Study: Inconsistency in Foam Placement at Mabuya Manufacturing

Background:

At Mabuya Manufacturing in Bloemfontein, a batch of upholstered benches had visible irregularities in seat foam shapes. Customer complaints cited uneven firmness and lumpy seat contours.

Investigation:
It was discovered that different workers were manually trimming foam without using jigs or templates, leading to inconsistencies.

Resolution:
The team introduced pre-cut templates and required the use of jigs for all foam trimming. Foam adhesives were also applied using measured spray patterns and distance guides.

Outcome:
Product uniformity improved, customer satisfaction increased, and material wastage was significantly reduced.

Critical Thinking Questions

- Why is it important to use profile jigs or templates during the foaming process?

- What are the risks associated with using adhesives or solvents without proper PPE?

- How can the workshop environment influence the performance of foam adhesives?

- In what way can poorly applied foam affect the final upholstered product?

- Why is it important to control spray pattern and coverage when applying adhesive?

Practical Classroom Activity

Task: Demonstrate Safe and Accurate Use of Foam Adhesives and Templates

Materials:

- Demonstration foam panels

- Wooden boards (or simulated frames)

- Pre-cut MDF templates

- Spray adhesive cans (non-toxic demo if indoors)

Instructions:

- Demonstrate how to apply spray adhesive from a safe distance.

- Show the use of a template to trace or cut foam consistently.

- Learners practice pressing foam into position using an adhesive-safe method.

- Emphasise correct PPE and material handling protocols.

Debrief:
Ask learners to reflect on the risks of inconsistency and the importance of repeatability in production.

### KT0208: Using Profile Jigs and Templates

Purpose of the Topic

This topic teaches learners how to correctly use profile jigs and templates to achieve accurate, repeatable shapes and placements in upholstery production processes. These tools are especially important when applying foam, shaping frames, or preparing materials for mass production.

By the end of this topic, learners should be able to explain the purpose of jigs and templates, distinguish between them, and demonstrate how to use them safely and effectively in a practical upholstery environment.

Key Content Areas

- Definition of Profile Jigs and Templates

  - Profile Jig: A guiding tool used to shape or align material consistently.

  - Template: A pattern or outline used to trace or cut materials to the same dimensions.

- Functions and Applications

  - Ensures accuracy and repeatability of cuts and shapes.

  - Speeds up production time and reduces errors.

  - Improves aesthetic consistency, especially in curved or contoured components.

  - Used in foam cutting, fabric layout, wood shaping, and trimming details.

- Materials Used to Make Jigs and Templates

  - Medium-Density Fibreboard (MDF)

  - Plywood

  - Perspex or plastic sheets

  - Cardboard (for temporary or disposable templates)

- Examples of Use in Upholstery

  - Creating symmetrical arm shapes for tub chairs

  - Cutting foam for back cushions to match frame contours

  - Outlining seat bases for cutting dust covers

  - Repeating edge trim spacing or tack strip positions

- Storage and Maintenance

  - Store flat and dry to avoid warping.

  - Label clearly to match components and jobs.

  - Inspect regularly for damage that may compromise accuracy.

- Health and Safety

  - Use templates with the correct tools (e.g. foam knives, hot wire cutters) under supervision.

  - Keep hands clear of cutting paths.

  - Avoid loose clothing near jigs or mounted cutters.

Example Applications

| Tool Type | Used For | Material |
| --- | --- | --- |
| Profile jig | Contouring foam arms for lounge chairs | MDF |
| Fabric template | Tracing fabric for repeat cushions | Cardboard or plastic |
| Base board guide | Aligning springing-up clips consistently | Plywood |
| Marking template | Even spacing for decorative nails on front arms | Perspex |

Case Study: Template Standardisation at Bester Brothers Upholstery

Background:
Bester Brothers Upholstery in Port Elizabeth scaled up production for a national furniture retailer. They noticed slight dimensional variation across batches of ottoman foam and fabric panels.

Investigation:
Different team members were cutting without uniform guides, resulting in quality control rejections.

Solution:
The team leader, Lisa, introduced clearly labelled templates for each cushion, cover, and trim element. A wall rack was installed to store all templates by product line.

Result:
Error rates dropped, and team productivity improved due to reduced decision-making time during material preparation.

Critical Thinking Questions

- Why is it important to differentiate between a jig and a template in upholstery production?

- What are the benefits of using templates in both small batch and mass production?

- How can poorly stored templates affect the quality of finished components?

- What safety considerations should learners be aware of when using cutting tools with jigs?

- How might technology (e.g. CNC cutting or digital templates) complement or replace physical templates in future?

Practical Classroom Activity

Task: Create and Use a Simple Template

Materials:

- Cardboard, MDF offcuts, or thick paper

- Pencils, rulers, scissors or utility knives

- Sample furniture shape (e.g. curved arm or round seat base)

Instructions:

- Learners create a reusable template for one upholstery component (e.g. seat base).

- Trace the template onto a piece of foam or fabric (scrap material).

- Discuss accuracy, ease of use, and how to label and store the template.

Reflection:
Facilitate a discussion on how using jigs and templates contributes to quality control and workshop efficiency.

### KT0209: Fabrics Used for Covering Webbing, Springs, Foam and Padding Including Calico, Hessian, Polypropylene, Dacron and Flock

Purpose of the Topic

This topic enables learners to identify and explain the use of specialist upholstery fabrics that serve as interior support or finishing layers within an upholstered furniture item. These fabrics are not visible in the final product but play a critical role in maintaining the structural integrity, comfort, and quality of the furniture.

By the end of the session, learners should be able to distinguish between different types of foundation and support fabrics, describe their characteristics, and explain their application during the upholstery process.

Key Content Areas

- Purpose of Underlying Cover Fabrics

  - Provide protection between materials (e.g. between springs and foam).

  - Prevent foam degradation by reducing friction or exposure to dust and debris.

  - Help maintain shape and structure during use.

  - Contribute to hygiene, breathability, and even fire-retardant properties in some cases.

- Common Types of Cover Fabrics

  - Calico

    - Plain-woven cotton

    - Breathable, light-coloured

    - Used to cover foam before the outer fabric is applied

    - Allows for smooth fitting of the top cover

  - Hessian (Burlap)

    - Coarse woven jute fibre

    - Used over springs as a foundational barrier

    - Often tacked down before applying felt or foam

  - Polypropylene (Non-woven Fabric)

    - Lightweight, synthetic

    - Resistant to moisture and mildew

    - Used under cushions, behind backs, or in low-contact areas

    - Commonly black or grey

  - Dacron (Polyester Fibre Batting)

    - Soft and fluffy texture

    - Applied over foam to soften edges and create rounded contours

    - Aesthetic enhancement under the final fabric

  - Flock

    - Loose textile fibres, often recycled

    - Used to fill voids or create a padded texture in fixed upholstery

    - Sometimes bonded with adhesive for specific applications

- Application and Layering Techniques

  - Order of use typically follows:

    - Hessian → Felt or Foam → Dacron → Calico → Top Cover

  - Must be secured with staples or tacks and stretched evenly.

- Selection Criteria

  - Fabric strength and resistance to tearing

  - Breathability

  - Cost-effectiveness

  - Compliance with safety regulations

  - Compatibility with foam and padding types used

Example Applications

| Fabric | Function | Applied Over |
| --- | --- | --- |
| Hessian | Base layer over springs | Coil springs |
| Dacron | Rounds and softens edges | Foam seat cushions |
| Calico | Holds inner layers together | Dacron, foam |
| Polypropylene | Bottom cloth, dust cover | Seat bases, frames |
| Flock | Filling compound for arm panels | Small cavities |

Case Study: Correct Fabric Selection at Olwethu Interiors

Background:

Olwethu Interiors, operating in East London, received a complaint from a corporate client that their new office reception chairs were losing shape within weeks. Upon inspection, the foam had shifted, and the corners looked flattened.

Root Cause:
The workshop skipped the Dacron and calico layers, using only foam and outer fabric. This resulted in sharp edges and insufficient hold of the internal padding.

Solution:
The production team was retrained to follow the proper layering sequence, including Dacron and calico. New chairs were supplied, and the client was satisfied with the improved result.

Learning Point:
Even though not visible, internal upholstery fabrics are essential to comfort, longevity, and professional finish.

Critical Thinking Questions

- What might happen if hessian is omitted when covering coil springs?

- Why is Dacron considered essential for aesthetic finishing even though it is hidden?

- What are the risks of using low-quality polypropylene fabric in bottom cloth applications?

- How does the combination of calico and Dacron improve the lifespan of a seat cushion?

- In what circumstances might flock be a better choice than foam?

Practical Classroom Activity

Task: Material Identification and Layer Planning Exercise

Materials:

- Physical or image samples of: calico, hessian, polypropylene, Dacron, and flock

- A scenario card describing a furniture item (e.g. “Deep-buttoned armchair with coil spring base”)

Instructions:

- Learners identify each fabric sample by touch and appearance.

- In groups, decide on a suitable layering sequence for the scenario provided.

- Each group presents their material plan and explains their choice and placement of each fabric.

Extension:
Ask learners to sketch a cross-section of their chair build-up, labelling each fabric layer.

### KT0210: Types and Gauges of Various Springs Needed for the Knocker-On Process, Including Coil Springs and Zig-Zag Springs

Purpose of the Topic

This topic introduces learners to the main types of springs used in upholstered furniture, with a focus on their application in the knocker-on process. Learners will understand how spring types and wire gauges influence the strength, comfort, and durability of furniture, and how to identify the correct spring type and gauge for different upholstery applications.

The topic also highlights the importance of correct installation procedures and the role of springs in supporting seating structures.

Key Content Areas

- The Knocker-On Process

  - A traditional upholstery method where springs are attached (or “knocked on”) to the seat or back frame of the furniture.

  - Springs form the foundation support before padding and covering are applied.

- Main Types of Upholstery Springs

  - Coil Springs (Double Cone Springs):

    - Traditional, vertically positioned.

    - Provide deep, resilient support.

    - Common in high-quality, hand-tied furniture.

  - Zig-Zag Springs (No-Sag or Sinuous Springs):

    - S-shaped wire springs fixed horizontally across seat or back frames.

    - Provide uniform support and easier installation.

    - Widely used in modern manufacturing.

- Spring Gauges

  - Refers to the thickness of the wire used to make the spring.

  - Measured in Standard Wire Gauge (SWG) or American Wire Gauge (AWG).

  - Lower gauge number = thicker wire = firmer support.

  - Typical range:

    - Coil springs: 8–11 gauge

    - Zig-zag springs: 9–13 gauge

  - Selection depends on:

    - Type of furniture (e.g. lounge chair vs. dining seat)

    - Weight load expectations

    - Desired firmness

- Other Spring Variants

  - Arc Springs: Pre-formed in a curved shape; used in some automotive or recliner seating.

  - Grid Springs: A network of interconnected coils or wire grids used in bases.

- Spring Layout and Fixing

  - Even spacing and consistent alignment are essential.

  - Springs are fixed using clips, twine, or spring nails.

  - Incorrect gauge or placement can lead to creaking, sagging, or frame damage.

Example Applications

| Spring Type | Typical Gauge | Used In |
| --- | --- | --- |
| Coil Spring | 9 gauge | Traditional armchairs and sofas |
| Zig-Zag Spring | 11–13 gauge | Modern lounge suites and office chairs |
| Arc Spring | 12 gauge | Recliner backs and motion chairs |
| Coil Grid System | 8 gauge | Premium mattresses and integrated bases |

Case Study: Spring Gauge Misuse at Naidoo Upholstery Co.

Background:
Naidoo Upholstery Co. in Durban produced a series of reception sofas for a corporate client. The furniture used 11-gauge zig-zag springs for seats.

Problem:
After three months, seats sagged and creaked under daily use.

Root Cause:
The team had used too light a gauge for the intended weight load. The springs flexed excessively and deformed.

Resolution:
They upgraded to 9-gauge springs and spaced them closer together. The improved support was well received by the client.

Learning Point:
Gauge selection directly affects functionality, longevity, and client satisfaction.

Critical Thinking Questions

- How does the spring gauge affect the firmness and support of upholstered seating?

- What might happen if coil springs are installed too far apart or at an angle?

- Why would a workshop choose zig-zag springs over coil springs?

- How could incorrect spring installation impact the covering and trimming processes?

- What questions should you ask a client when choosing the type of spring for a new design?

Practical Classroom Activity

Task: Identify and Compare Upholstery Spring Types and Gauges

Materials:

- Samples of coil springs (different gauges)

- Zig-zag springs (pre-cut segments)

- Measuring tools (calliper or gauge chart)

- A small wooden seat frame mock-up

Instructions:

- Learners inspect and classify springs by type and thickness.

- Measure the diameter of spring wire and compare to gauge charts.

- In pairs, design a basic spring layout plan for:

  - A dining seat base (zig-zag)

  - A vintage armchair (coil spring grid)

- Discuss how gauge and spring type influence comfort and durability.

### KT0211: Types, Sizes, Quantity and Quality of Consumables Including Clips, Nails, Staples, Edge Rolls, Twine and Cord Needed for the Springing Up Process

Purpose of the Topic

This topic equips learners with the knowledge and skills required to correctly identify and select consumables used in the springing up process. This process involves fixing springs (typically coil or zig-zag) to the frame to provide the necessary base support for upholstered furniture.

By understanding the types, sizes, quantities, and quality standards of these consumables, learners will be able to support efficient, safe, and structurally sound spring installation.

Key Content Areas

- Definition of Springing Up

  - The process of attaching and securing springs to the seat or back frame of upholstered furniture using consumables.

  - Involves anchoring, tensioning, and positioning springs according to the required support layout.

- Key Consumables Used

  - Clips (Spring Clips or Staples-on Clips):

    - Secure zig-zag springs to wooden frames.

    - Available in single or double-prong variants.

  - Nails and Tacks:

    - Used for tying coil springs and securing twine.

    - Typically round-headed or domed upholstery nails.

  - Staples:

    - Used to attach twine, webbing, or edge rolls.

    - Must be rust-resistant and suited to the staple gun in use.

  - Edge Rolls:

    - Applied to front seat rails and arms to soften edges.

    - Made from paper, plastic, or felt-wrapped forms.

  - Twine (Upholstery Twine or Spring Twine):

    - Used to lash coil springs together in an 8-way or cross-tie format.

    - Must be strong, slightly stretchable, and knot-secure.

  - Cord:

    - Used in piping or for decorative edge applications.

    - May also act as reinforcement in some areas.

- Determining the Correct Size and Quantity

  - Based on:

    - Spring size and type (coil or zig-zag)

    - Frame dimensions

    - Number of attachment points

    - Layout of spring grid or sequence

  - Example: A standard 3-spring seat may require 8 clips, 12 nails, 3 m of twine, and 2 m of edge roll.

- Quality Considerations

  - Rust-resistant metal parts (especially in coastal or humid areas)

  - Non-fraying, high-tensile twine

  - Uniform edge rolls with consistent diameter

  - Staples that penetrate without splitting the wood

- Impact of Poor Consumable Selection

  - Loose springs, noisy operation

  - Sagging or misalignment

  - Premature failure of seat support

  - Aesthetic compromise

Example Table: Consumables for Springing Up Process

| Consumable | Typical Size | Function |
| --- | --- | --- |
| Spring Clips | 25 mm | Anchor zig-zag springs to frame |
| Twine | 3 mm (waxed) | Tie coil springs securely to frame |
| Staples | 12–14 mm | Secure webbing and edge rolls |
| Upholstery Nails | 16 mm | Fix twine and secure base materials |
| Edge Roll | 12 mm diameter | Protect edge and shape of seat rail |
| Cord | 4 mm | Piping support or edge strengthening |

Case Study: Spring Failure at Van der Merwe Upholstery

Background:
Van der Merwe Upholstery, based in George, supplied a batch of occasional chairs to a residential development. After six months, multiple chairs developed noisy, sagging seats.

Findings:
Low-quality spring clips had failed due to corrosion. Staples had pulled out due to incorrect size, and the twine used was general string, not proper upholstery twine.

Solution:
The team replaced all affected consumables with specification-approved hardware and reinforced their purchasing standards.

Learning Point:
The strength and durability of the spring foundation depend heavily on the quality and suitability of consumables used.

Critical Thinking Questions

- Why is it important to use rust-resistant staples or nails in upholstery construction?

- What consequences can arise from using incorrect twine in the springing process?

- In what ways does edge roll improve both safety and comfort in a finished seat?

- What factors must be considered when choosing staple length for fixing webbing?

- How can a workshop monitor the quality of consumables received from suppliers?

Practical Classroom Activity

Task: Plan and Assemble a Springing-Up Kit for a Seat Frame

Materials:

- Sample seat frame (or diagram)

- Coil or zig-zag spring layout plan

- Consumables (labels or real items)

Instructions:

- Learners identify which consumables are required for the frame shown.

- Estimate quantities for each item based on the plan.

- Present a list and explain:

  - The purpose of each consumable

  - The reason for size and quantity selection

- Optionally, assemble a mock frame (if practical materials available).

### KT0212: Types of Webbing Needed for the Knocker-On Process, Including Manmade Elastic and Rubber Webbing, and Those Made from Natural Fibres like Jute Webbing and Hessian

Purpose of the Topic

This topic provides learners with an understanding of the types of webbing used in upholstered furniture, specifically in the knocker-on process. It highlights the differences between manmade and natural fibre webbings, their properties, applications, and implications for comfort and durability.

By the end of the topic, learners will be able to select appropriate webbing types for different furniture applications and explain how webbing contributes to the foundational support system of an upholstered product.

Key Content Areas

- What is Webbing in Upholstery?

  - A woven strap or band used to support the seating and back structure in furniture.

  - Attached directly to the frame using staples or tacks.

  - Forms the base upon which foam, padding, and covering are applied.

- Types of Webbing

  - Manmade Webbing:

    - Elastic Webbing:

      - Contains rubber threads covered with polyester or synthetic fibres.

      - Provides stretch and resilience.

      - Ideal for backs and seats requiring flexibility.

    - Rubber Webbing:

      - Thicker, heavier-duty elastic bands.

      - Long-lasting and often used in high-traffic seating.

      - Requires tensioning and sometimes metal clips.

  - Natural Fibre Webbing:

    - Jute Webbing:

      - Made from woven jute fibre.

      - Non-elastic, traditional, strong, and sustainable.

      - Often used in vintage or hand-crafted upholstery.

    - Hessian Webbing (Burlap Strapping):

      - Similar to jute but less refined.

      - Coarser texture, commonly used in base layers or for economy applications.

- Comparison of Manmade vs Natural Webbing

| Feature | Manmade (Elastic/Rubber) | Natural (Jute/Hessian) |
| --- | --- | --- |
| Stretch | High | None |
| Durability | Excellent (if high quality) | Good, but degrades if exposed to moisture |
| Comfort | Softer feel, adds bounce | Firmer seat, classic support feel |
| Appearance | Hidden under layers | Hidden under layers |
| Cost | Medium to high | Low to medium |

- Application Considerations

  - Type of furniture (e.g. dining chair vs. recliner).

  - Expected weight load and use frequency.

  - Desired comfort level (firm vs. flexible).

  - Alignment with traditional or modern build styles.

- Installation and Tensioning

  - Webbing must be stretched to specific tension (varies by type).

  - Requires a webbing stretcher or tension tool.

  - Must be fixed evenly across the frame for consistent support.

Example Applications

| Furniture Type | Recommended Webbing | Reason |
| --- | --- | --- |
| Lounge Chair (Modern) | Elastic webbing | Comfortable flexibility |
| Dining Chair (Classic) | Jute webbing | Firm, structured seating |
| Recliner Back | Rubber webbing | Flexible, durable |
| Footstool | Jute or hessian | Cost-effective, low-impact use |

Case Study: Misapplied Webbing at Langley Seating Studio

Background:
Langley Seating Studio in Pietermaritzburg produced a series of ottomans using elastic webbing as a base layer. Within months, the ottomans sagged and lost shape.

Root Cause:
The selected webbing was too flexible for the purpose. The design called for firm, non-stretch jute webbing.

Resolution:
The production team was trained to differentiate webbing types by material and application suitability. They replaced the elastic webbing with pre-stretched jute webbing and reinforced frame spacing.

Learning Point:
Understanding the functional differences between webbing types is critical to ensuring performance and longevity.

Critical Thinking Questions

- Why would a lounge chair require elastic webbing while a dining chair would not?

- What risks are involved in under-tensioning webbing during installation?

- How does webbing choice affect the durability of seating furniture?

- Why is hessian webbing considered more appropriate for base layers than seat support?

- What signs might indicate that incorrect webbing was used during production?

Practical Classroom Activity

Task: Webbing Type Evaluation and Application Match

Materials:

- Samples of elastic, rubber, jute, and hessian webbing

- Description cards for various furniture pieces (e.g. "Formal dining chair", "Children’s lounge seat", "Vintage armchair")

Instructions:

- Learners examine the texture, stretch, and weave of each sample.

- In small groups, match each webbing sample to a furniture description.

- Present choices to the class, explaining their reasoning based on application context.

Extension:
Demonstrate basic tensioning technique using a webbing stretcher and staple gun (with instructor supervision).

KT0213: Labels, Codes and the Coding System and Traceability

Purpose of the Topic

This topic introduces learners to the systems and procedures involved in labelling, coding, and traceability of raw materials and consumables in the upholstery manufacturing environment. These systems ensure that materials can be correctly identified, tracked, and verified throughout the production process, supporting quality control, compliance, stock management, and customer satisfaction.

By the end of this topic, learners should be able to describe the purpose and function of labelling and coding systems and explain how traceability contributes to accountability and operational efficiency.

Key Content Areas

- Definition of Labelling and Coding

  - Labels: Printed or written tags attached to materials for identification.

  - Codes: Alphanumeric or symbolic systems used to categorise, reference, or track materials.

- Purpose of Coding Systems

  - Prevent material mix-ups during production.

  - Allow quick access to material information (e.g. density of foam, fabric batch).

  - Facilitate communication between departments (e.g. stockroom and workshop).

  - Support internal systems (ERP, stock control, procurement).

- Types of Labels and Codes Used in Upholstery

  - Foam Codes (e.g. 28/200 = 28 kg/m³ density, 200 N firmness).

  - Fabric Rolls: Batch number, material type, colour code, fire rating.

  - Adhesives and Chemicals: Safety and expiry labelling, usage codes.

  - Staples and Fasteners: Size code and box count labelling.

  - Barcodes or QR Codes: Linked to inventory or tracking systems.

- Traceability in Upholstery Production

  - Enables identification of source, batch, and movement of materials.

  - Supports quality assurance and client follow-up.

  - Important for resolving defects or complaints—e.g. tracking back to faulty foam or mismatched fabric.

- Common Practice in a Workshop

  - Use of colour-coded stickers (e.g. green = ready for use, red = quarantined).

  - Labelling storage bins and shelves clearly.

  - Recording material movement on stock sheets or digital platforms.

Example Applications

| Material | Label/Code | Purpose |
| --- | --- | --- |
| Foam Sheet | 30/180 | Indicates density and firmness |
| Fabric Roll | B233 - Poly/Charcoal | Batch, type, and colour code |
| Adhesive Can | PU-FAST / 04-2025 | Product code and expiry date |
| Staple Box | STPL12-5K | Staple length (12 mm), quantity (5 000) |

Case Study: Traceability Failure at Elize Upholstery Works

Background:

Elize Upholstery Works, a Johannesburg-based company, received a client complaint about deteriorating foam in a recently delivered sofa batch.

Issue:
The foam used had no label or code, and records of which batch had been used were missing. The supplier refused to accept accountability due to lack of traceability.

Resolution:
Elize Upholstery implemented a coded labelling system for all foam and fabric rolls. Material usage logs were introduced for each production batch.

Outcome:
Future defects could be tracked to specific suppliers or storage conditions, improving accountability and production reliability.

Critical Thinking Questions

- Why is it risky to use materials that are not clearly labelled in a manufacturing environment?

- How can coding systems help identify the cause of a product defect?

- What types of materials require traceability most urgently in upholstery, and why?

- How might a client benefit from your ability to trace the materials used in their furniture?

- What is the difference between a label, a batch number, and a stock code?

Practical Classroom Activity

Task: Design and Implement a Simple Labelling and Coding System

Materials:

- Labels, stickers, markers, sample materials (or images thereof)

- Templates for coding (e.g. [Item Type] – [Material] – [Batch No.])

Instructions:

- Divide learners into groups and assign them 4–5 sample materials.

- Each group creates a label and coding system for:

  - Foam

  - Fabric

  - Fasteners

  - Adhesives

- Present and explain:

  - The purpose of their coding format

  - How traceability would work in a real workshop

Extension:
Introduce a simple form where learners “check out” material for a job card, simulating traceability in action.

### KT0213: Labels, Codes and the Coding System and Traceability

Purpose of the Topic

This topic introduces learners to the systems and procedures involved in labelling, coding, and traceability of raw materials and consumables in the upholstery manufacturing environment. These systems ensure that materials can be correctly identified, tracked, and verified throughout the production process, supporting quality control, compliance, stock management, and customer satisfaction.

By the end of this topic, learners should be able to describe the purpose and function of labelling and coding systems and explain how traceability contributes to accountability and operational efficiency.

Key Content Areas

- Definition of Labelling and Coding

  - Labels: Printed or written tags attached to materials for identification.

  - Codes: Alphanumeric or symbolic systems used to categorise, reference, or track materials.

- Purpose of Coding Systems

  - Prevent material mix-ups during production.

  - Allow quick access to material information (e.g. density of foam, fabric batch).

  - Facilitate communication between departments (e.g. stockroom and workshop).

  - Support internal systems (ERP, stock control, procurement).

- Types of Labels and Codes Used in Upholstery

  - Foam Codes (e.g. 28/200 = 28 kg/m³ density, 200 N firmness).

  - Fabric Rolls: Batch number, material type, colour code, fire rating.

  - Adhesives and Chemicals: Safety and expiry labelling, usage codes.

  - Staples and Fasteners: Size code and box count labelling.

  - Barcodes or QR Codes: Linked to inventory or tracking systems.

- Traceability in Upholstery Production

  - Enables identification of source, batch, and movement of materials.

  - Supports quality assurance and client follow-up.

  - Important for resolving defects or complaints—e.g. tracking back to faulty foam or mismatched fabric.

- Common Practice in a Workshop

  - Use of colour-coded stickers (e.g. green = ready for use, red = quarantined).

  - Labelling storage bins and shelves clearly.

  - Recording material movement on stock sheets or digital platforms.

Example Applications

| Material | Label/Code | Purpose |
| --- | --- | --- |
| Foam Sheet | 30/180 | Indicates density and firmness |
| Fabric Roll | B233 - Poly/Charcoal | Batch, type, and colour code |
| Adhesive Can | PU-FAST / 04-2025 | Product code and expiry date |
| Staple Box | STPL12-5K | Staple length (12 mm), quantity (5 000) |

Case Study: Traceability Failure at Elize Upholstery Works

Background:
Elize Upholstery Works, a Johannesburg-based company, received a client complaint about deteriorating foam in a recently delivered sofa batch.

Issue:
The foam used had no label or code, and records of which batch had been used were missing. The supplier refused to accept accountability due to lack of traceability.

Resolution:
Elize Upholstery implemented a coded labelling system for all foam and fabric rolls. Material usage logs were introduced for each production batch.

Outcome:
Future defects could be tracked to specific suppliers or storage conditions, improving accountability and production reliability.

Critical Thinking Questions

- Why is it risky to use materials that are not clearly labelled in a manufacturing environment?

- How can coding systems help identify the cause of a product defect?

- What types of materials require traceability most urgently in upholstery, and why?

- How might a client benefit from your ability to trace the materials used in their furniture?

- What is the difference between a label, a batch number, and a stock code?

Practical Classroom Activity

Task: Design and Implement a Simple Labelling and Coding System

Materials:

- Labels, stickers, markers, sample materials (or images thereof)

- Templates for coding (e.g. [Item Type] – [Material] – [Batch No.])

Instructions:

- Divide learners into groups and assign them 4–5 sample materials.

- Each group creates a label and coding system for:

  - Foam

  - Fabric

  - Fasteners

  - Adhesives

- Present and explain:

  - The purpose of their coding format

  - How traceability would work in a real workshop

Extension:
Introduce a simple form where learners “check out” material for a job card, simulating traceability in action.

### KT0214: Principles and Concepts of Safe Materials Handling and Storage Methods

Purpose of the Topic

This topic introduces learners to the basic principles and best practices for the safe handling and storage of raw materials and consumables used in upholstery. Learners will explore how to prevent accidents, protect materials from damage or contamination, and maintain an efficient and legally compliant workshop environment.

By the end of this topic, learners will be able to demonstrate an understanding of proper lifting techniques, storage protocols, labelling practices, and handling precautions for hazardous or sensitive materials.

Key Content Areas

- Importance of Safe Handling and Storage

  - Prevents workplace injuries (e.g. back strain, cuts, chemical exposure).

  - Protects materials from degradation (e.g. moisture, sunlight, pests).

  - Supports organisation, traceability, and workflow.

  - Aligns with health, safety, and environmental (HSE) regulations.

- Safe Manual Handling Practices

  - Use correct lifting technique: back straight, knees bent, load close to body.

  - Do not lift heavy rolls or foam alone – use trolleys or team lifting.

  - Avoid twisting when carrying loads.

  - Wear protective gloves when handling sharp materials (e.g. springs, metal trims).

- Storage Principles

  - Store foam and fabric off the ground, away from walls, and covered.

  - Stack rolls and panels in a stable, accessible configuration.

  - Label all materials with content, batch, and expiry information where applicable.

  - Keep chemicals in locked, ventilated cabinets, away from heat sources.

- Hazardous Material Handling

  - Read Safety Data Sheets (SDS) for all adhesives, solvents, and cleaning products.

  - Wear PPE (gloves, eye protection, masks) when using sprays or liquid adhesives.

  - Ensure proper ventilation in application areas.

  - Store flammable materials in compliance with local safety regulations.

- Clean and Safe Workspaces

  - Maintain clear walkways.

  - Dispose of offcuts and waste responsibly.

  - Conduct regular safety checks of storage areas.

- Emergency Preparedness

  - Know the location of:

    - First aid kits

    - Fire extinguishers

    - Emergency exits

  - Understand workshop emergency procedures for spills or fires.

Example Applications

| Material | Safe Handling/Storage Practice |
| --- | --- |
| Foam sheets | Store flat on clean surfaces; avoid direct sunlight |
| Fabric rolls | Upright or horizontal racking; avoid damp or dust |
| Adhesives | Store in original containers, away from ignition sources |
| Staples and nails | Keep in sealed containers to prevent rust or injury |

Case Study: Safety Audit at Fourways Furniture Co.

Background:
During a health and safety audit, Fourways Furniture Co. in Johannesburg received warnings about blocked walkways and open containers of spray adhesive stored near electrical outlets.

Corrective Action:
The team reorganised storage to ensure clear access and moved flammable materials into a compliant storage cabinet. They also retrained staff on safe usage and introduced a daily cleanup schedule.

Result:
The workshop passed a follow-up inspection and reported improved safety awareness and productivity.

Critical Thinking Questions

- What are the dangers of stacking foam or fabric too high in the storage area?

- Why should flammable adhesives be stored separately from general consumables?

- How does safe storage contribute to productivity in a busy upholstery workshop?

- What actions should you take if you witness a co-worker using a solvent without PPE?

- How can a well-labelled storage system prevent workplace injuries?

Practical Classroom Activity

Task: Evaluate and Improve Workshop Safety Layout (Simulated)

Materials:

- Diagram or physical mock-up of a workshop

- Labels for storage zones, hazard warnings, and PPE areas

Instructions:

- Learners are presented with a layout that contains safety violations (e.g. blocked exits, unlabelled chemicals).

- In groups, learners identify the risks and recommend corrective measures.

- Present proposed safety plan including:

  - Safe storage layout

  - PPE allocation

  - Emergency readiness steps

Extension:
Have learners role-play responding to a small spill or identifying unsafe storage practices in a real or simulated setting.

## Integrated Assessment – KM-03-KT02

Qualification: Furniture Upholsterer (SAQA ID 103199)
Knowledge Module: KM-03-KT02
Topic: Upholstery Raw Materials and Consumables Types and Characteristics
NQF Level: 2 | Credits: 8 | Weighting: 20%
Assessment Duration: 90 minutes
Total Marks: 60

🔹 Section A: Multiple Choice (1 mark each × 10 = 10 marks)

Write only the letter (A–D) next to the question number.

- Which of the following is a natural fibre used in upholstery?
A. Polypropylene
B. Polyester
C. Jute
D. PVC

- Which material is commonly used to round off edges over foam?
A. Hessian
B. Felt
C. Dacron
D. Flock

- Which foam grade indicates a firm density?
A. 20/100
B. 28/150
C. 28/200
D. 18/80

- What does the number “28” in 28/200 foam stand for?
A. Spring count
B. Firmness
C. Thickness
D. Density

- What type of webbing is best suited to classic dining chairs?
A. Elastic webbing
B. Rubber webbing
C. Jute webbing
D. Plastic strapping

- Profile jigs are primarily used to:
A. Apply solvent
B. Cut material consistently
C. Fix springs
D. Staple bottom cloths

- Which of the following is a function of polypropylene in upholstery?
A. Decorative trim
B. Structural padding
C. Bottom cloth cover
D. Top fabric layer

- What does “traceability” in upholstery material refer to?
A. Reusing materials
B. Matching fabric to colour charts
C. Tracking batch and material sources
D. Reordering from stock

- A 9-gauge zig-zag spring is:
A. Lighter than 13 gauge
B. Stronger than 13 gauge
C. More flexible than 13 gauge
D. Only used for backs

- Which safety measure is essential when using spray adhesives?
A. Wearing heavy-duty boots
B. Covering all windows
C. Using them near electrical heaters
D. Ensuring ventilation and PPE

🔹 Section B: Short Answer Questions (4 marks each × 5 = 20 marks)

- Name and describe four common consumables used during the cover fitting or trimming process.

- List and explain the purpose of any three under-cover fabrics used in upholstery.

- Explain the difference between natural and manmade webbing, giving one example of each.

- Describe two safety rules for storing upholstery foam and two for storing adhesives.

- Explain the role of a template in the upholstery process and give one example of its use.

🔹 Section C: Practical Scenario (3 marks each × 5 = 15 marks)

Scenario:
You are preparing to upholster a high-use restaurant chair using zig-zag springs, firm foam, and a durable fabric. The frame has a flat seat base and a gently curved back.

- Which type and gauge of spring would you use?

- What foam grade would you recommend, and why?

- Which under-fabric would you use to cover the springs before applying foam?

- What safety precautions should be taken when applying spray adhesive to this chair?

- Identify two consumables needed for trimming and describe their function.

🔹 Section D: Case Study – Scenario-Based Questions (3 marks each × 5 = 15 marks)

Case Study:

Mbali Interiors was tasked with producing 20 high-end armchairs for a client. The team used memory foam but omitted Dacron. They also applied adhesive in a closed room, stored rolls of fabric on the floor near windows, and used mismatched staples.

- Identify three problems with the team’s handling or storage of materials.

- How could the omission of Dacron affect the chair’s appearance or performance?

- What are the risks of using mismatched staple sizes in production?

- Suggest two improvements to Mbali Interiors’ workshop storage methods.

- What steps could be taken to ensure consistent foam cutting across all 20 chairs?

✅ Model Answers and Marking Memo

Section A – Answers

- C 2. C 3. C 4. D 5. C 6. B 7. C 8. C 9. B 10. D

Section B – Marking Notes

- Staples (fasten covers), Decorative nails (aesthetic trim), Edge rolls (soften front rail), Twine (tie spring units).

- Hessian (over springs), Calico (smooth layer), Polypropylene (dust cover).

- Natural = jute (firm, no stretch), Manmade = elastic webbing (flexible).

- Foam: store flat, avoid direct sun. Adhesives: lockable storage, good ventilation.

- Template ensures repeated accuracy – e.g. cutting identical arm shapes.

Section C – Marking Notes

- Zig-zag spring, 9-gauge (stronger for high use).

- 28/200 or 32/250 foam – firm, durable, retains shape.

- Hessian – protects foam from spring abrasion.

- Spray away from ignition, wear PPE, ventilate.

- Gimp (decorative cover for staples), piping cord (finishing seams).

Section D – Marking Notes

- Fabric exposed to sunlight, no ventilation, incorrect staple size.

- Fabric may show sharp corners, loss of softness.

- May pull out or not grip properly, resulting in cover failure.

- Elevate rolls, introduce labelled storage racks.

- Use jigs or templates, label foam per chair.

📊 Assessment Rubric

| Criteria | Outstanding (5) | Competent (3–4) | Partially Achieved (1–2) | Not Achieved (0) |
| --- | --- | --- | --- | --- |
| Knowledge of Materials | Clearly identifies and describes full range with context | Understands most with minor errors | Some relevant info, limited scope | Off-topic or no response |
| Understanding of Application | Matches correct materials to context | Mostly accurate with a few mismatches | Vague or partially correct usage | No understanding of application |
| Safety and Handling Awareness | Demonstrates complete awareness | Reasonable awareness, some gaps | Minimal safety points recognised | No reference to safety |
| Critical Thinking and Justification | Thoughtful, clearly justified responses | Some insight, with support | Basic or shallow reasoning | No explanation or unsupported ideas |

## KM-03-KT03: Use of Chemicals – Adhesives and Solvents

Weighting: 10% | NQF Level: 2

1. Purpose of the Knowledge Topic

The purpose of this Knowledge Topic is to provide learners with a foundational understanding of the types, functions, application, and safe handling of chemicals used in upholstery processes, specifically adhesives and solvents. Learners will be introduced to the chemical properties, tools, equipment, and techniques involved in the preparation and application of these substances in a manufacturing context.

This knowledge is critical for ensuring the quality, consistency, and safety of upholstery processes where wet products are applied. It also introduces learners to the responsibilities associated with health and environmental compliance, including spill control, waste disposal, and personal protective equipment.

2. Key Knowledge Areas (Topic Elements)

The topic comprises the following elements:

- KT0301: Chemical types, characteristics, properties and purpose

- KT0302: Adhesives and solvents – characteristics, uses, flammability and safe handling

- KT0303: Application equipment, tools and dispensers

- KT0304: Measuring and mixing techniques and procedures

- KT0305: Preparation and application techniques

- KT0306: Safety hazards and risks

3. Internal Assessment Criteria and Weighting

This Knowledge Topic carries a weighting of 10% and is assessed according to the following Internal Assessment Criteria:

- IAC0301: The various types of chemicals used in upholstery processes are named

- IAC0302: Characteristics and properties of the chemicals used in upholstery processes are described

- IAC0303: The purpose of each of these chemicals used in the process is explained

- IAC0304: Various types of solvents used in upholstery processes are listed

- IAC0305: Characteristics and properties of these solvents are explained

- IAC0306: The purpose of each solvent used in the process is explained

- IAC0307: The application techniques and sequence used for chemicals and solvents are described

- IAC0308: Safety procedures, hazards and risks associated with handling of chemicals and solvents are described

- IAC0309: Standard operating procedures dealing with spillages, waste and effluent are explained

- IAC0310: Types, usage and care of personal protective equipment (PPE) used in textile wet product preparation processes are described

4. Application in Upholstered Furniture Manufacturing

Adhesives and solvents are essential to modern upholstery processes, particularly in securing foams, attaching fabrics, and preparing surfaces. Understanding their correct use ensures durable and aesthetically sound finishes, while improper use can compromise product integrity and workplace safety.

This Knowledge Topic supports learners in making informed decisions regarding chemical selection, understanding how to use application tools and dispensers, and ensuring that all tasks are performed in compliance with occupational health and safety standards. It is also key to maintaining environmental responsibility in the handling and disposal of chemicals and associated waste.

### KT0301: Chemical Types, Characteristics, Properties and Purpose

Purpose of the Topic

This topic introduces learners to the different types of chemicals commonly used in upholstery, focusing on their purpose, characteristics, and functional properties. It provides a foundation for understanding how chemicals support the manufacturing process and how to identify them based on physical and functional criteria.

By mastering this content, learners will be better prepared to handle chemicals safely, select the correct product for the task, and understand how chemical properties influence performance in upholstery applications.

Key Content Areas

- Definition of Chemicals in Upholstery Context

  - Substances used to bond, clean, prepare, or finish upholstered components.

  - Includes adhesives, solvents, and other auxiliary chemical agents.

- Main Chemical Types Used in Upholstery

  - Adhesives

    - Bonding agents used to attach foam, fabric, or other soft components to frames.

  - Solvents

    - Liquids used to dissolve adhesives, clean surfaces or tools, or thin viscous chemicals.

  - Cleaning Agents

    - Occasionally used to prepare surfaces (e.g. degreasers).

  - Primers and Activators

    - Occasionally used to improve bonding on difficult surfaces.

- Characteristics and Properties

  - Viscosity – Thickness and flow rate of the liquid.

  - Volatility – How quickly the chemical evaporates (important for drying time and safety).

  - Flammability – Whether it can catch fire; highly relevant for solvents and adhesives.

  - Toxicity – Potential harm through inhalation, ingestion, or skin contact.

  - Bond Strength – Adhesives are chosen based on the load-bearing requirement.

  - Open Time – Time between applying the chemical and achieving bond.

- Purpose and Application

  - Adhesives are applied to join foam to wood, padding to backing, or layers of fabric.

  - Solvents clean overspray or remove dried adhesive from tools.

  - Understanding characteristics helps avoid product failure, such as peeling, foam separation, or surface damage.

Example Table: Chemical Types and Functions

| Chemical Type | Characteristic | Purpose |
| --- | --- | --- |
| Contact adhesive | High tack, quick bond | Foam to foam or foam to frame bonding |
| Solvent (acetone) | Fast evaporation, strong | Cleaning adhesive spray guns |
| Water-based glue | Low odour, longer drying | Low-impact applications, eco-conscious products |
| Spray adhesive | Aerosol or compressor-applied | Large surface bonding in seat cushions |

Case Study: Misuse of Chemical Products at Caledon Upholstery

Background:
Caledon Upholstery took on a bulk order for office chairs and used a general-purpose craft adhesive to speed up production. After six weeks, some clients reported seat covers loosening and foam detachment.

Investigation:
It was discovered that the glue used had low tack and low bond strength, and was not designed for upholstery or high-compression areas.

Outcome:
The company recalled the affected chairs and switched to a contact adhesive formulated for foam and textile use. Employees were retrained to check product specifications and data sheets before use.

Learning Point:
Using the wrong chemical, even if more affordable or available, can lead to poor workmanship, increased rework, and reputational damage.

Critical Thinking Questions

- What is the risk of using an adhesive with incorrect bond strength in high-load furniture areas?

- Why is it important to understand the drying time or open time of a chemical product before application?

- What could happen if a solvent is used on a surface it is not intended for?

- How would you decide between using a spray adhesive and a brush-on adhesive?

- In your view, should workshop staff be allowed to substitute chemical products if the specified one is unavailable? Why or why not?

Practical Classroom Activity

Task: Chemical Matching and Application Purpose

Materials:

- Product labels or data sheets (simulated or real) for 3–5 chemical products (e.g. contact adhesive, cleaning solvent, spray adhesive, water-based glue)

- Description cards listing upholstery applications (e.g. “Attaching foam to seat frame”, “Cleaning nozzle”, “Low-odour adhesive needed”)

Instructions:

- Learners read the product characteristics (viscosity, flammability, drying time).

- Match each product to one or more of the listed applications.

- Present findings, explaining why certain products are suited (or not) for each use.

Extension Discussion:
Facilitate discussion about alternative products and risks of substitution in the absence of proper technical knowledge.

### KT0302: Adhesives and Solvents – Characteristics and Uses, Flammable and Hazardous, Safe Handling and Storage

Purpose of the Topic

This topic builds on the introduction to chemicals by focusing specifically on adhesives and solvents used in upholstery. Learners will explore their working properties, hazards, and the correct methods of safe handling and storage.

This foundational knowledge helps reduce the risk of accidents, supports correct product usage, and ensures compliance with workplace health and safety protocols in upholstery environments.

Key Content Areas

- Common Upholstery Adhesives

  - Contact Adhesive

    - Applied to both surfaces, allowed to dry, then pressed together.

    - High initial tack and strong bond.

  - Spray Adhesive

    - Aerosol or compressor-based; allows for even coverage.

    - Quick drying and efficient for covering large areas.

  - Water-Based Adhesive

    - Low odour, environmentally friendly, but longer curing time.

    - Used in eco-sensitive or indoor environments.

- Common Upholstery Solvents

  - Acetone

    - Strong solvent used to clean tools or remove adhesive residue.

    - Highly flammable, fast-evaporating.

  - Mineral Spirits / Turpentine

    - Less aggressive, used for mild cleaning.

    - Often has an odour and requires good ventilation.

  - Alcohol-Based Solvents

    - Quick-drying and often used for surface preparation.

- Hazards and Risk Characteristics

  - Flammability

    - Many solvents and some adhesives are highly flammable.

  - Inhalation Risk

    - Fumes may cause dizziness, headaches, or long-term health issues.

  - Skin and Eye Contact

    - Can lead to irritation, burns, or allergic reactions.

  - Reactivity and Storage

    - Must not be stored near open flames or heat sources.

- Safe Handling and Storage

  - Always refer to Safety Data Sheets (SDS) for each product.

  - Store flammable chemicals in ventilated, fire-safe cabinets.

  - Ensure lids are tightly sealed when not in use.

  - Never use near open flames or hot equipment.

  - Wear appropriate Personal Protective Equipment (PPE):

    - Gloves, masks or respirators, and safety glasses.

  - Use adhesives and solvents only in well-ventilated areas.

- Spill and Disposal Procedures

  - Absorb spills using appropriate material (e.g. sand or commercial spill pads).

  - Dispose of chemicals via approved disposal channels.

  - Never pour solvents into sinks or stormwater systems.

Example Table: Properties and Safety of Adhesives and Solvents

| Product | Type | Flammable | PPE Required | Storage Advice |
| --- | --- | --- | --- | --- |
| Contact Adhesive | Solvent-based | Yes | Gloves, mask, ventilation | Steel cabinet, labelled container |
| Spray Adhesive | Aerosol | Yes | Respirator, goggles | Keep upright, away from sunlight |
| Acetone | Solvent | Highly | Gloves, mask | Cool, dry place with ventilation |
| Water-Based Adhesive | Non-flammable | No | Minimal | Avoid freezing; reseal after use |

Case Study: Fire Hazard at Kofi Designs

Background:
At Kofi Designs in Mafikeng, a worker used a solvent-based adhesive next to a portable heater. A sudden spark caused the adhesive vapours to ignite, resulting in minor burns and equipment damage.

Investigation:
The staff had not been trained in the flammability of chemical adhesives, and the room lacked proper ventilation.

Action Taken:

- All staff were retrained on SDS interpretation and chemical hazards.

- A flammable storage cabinet was installed.

- Adhesive use was restricted to a designated ventilated glue station.

Learning Point:
Even common products like spray glue can pose serious fire risks if used incorrectly.

Critical Thinking Questions

- What are the potential consequences of storing flammable adhesives near a heat source?

- Why is ventilation important when applying solvent-based adhesives?

- How do you determine the appropriate PPE for a particular chemical?

- What are the risks of not following spill response procedures for solvents?

- What information on a Safety Data Sheet would help you handle a solvent safely?

Practical Classroom Activity

Task: Chemical Safety Assessment Simulation

Materials:

- Printed Safety Data Sheets for 3 sample products

- Hazard labels and PPE samples

- Storage cabinet diagram (poster or drawing)

Instructions:

- In pairs, learners read through each SDS and identify:

  - Flammability rating

  - Required PPE

  - First aid steps

- Learners sort items into:

  - Safe vs unsafe storage locations

  - Suitable vs unsuitable working environments

- Facilitate a classroom discussion on the importance of chemical literacy in upholstery.

### KT0303: Application Equipment, Tools and Dispensers

Purpose of the Topic

This topic introduces learners to the equipment and tools used to apply adhesives and solvents in upholstery. Understanding these tools is essential for accurate, safe, and efficient chemical application in a production environment.

By the end of this session, learners will be able to identify common tools and dispensers, explain their purpose, and demonstrate an understanding of how improper equipment use can affect quality and safety.

Key Content Areas

- Common Equipment and Tools

  - Spray Guns (Compressor-Attached):

    - Used with contact adhesives in workshop settings.

    - Allow controlled spray pattern and efficient application across large areas.

  - Aerosol Cans:

    - Pre-filled with adhesive or cleaning agent.

    - Convenient but limited to small jobs and offer less control.

  - Brushes and Rollers:

    - Used to apply water-based adhesives or edge-specific contact glue.

    - Ideal for precise or small-area applications.

  - Solvent Dispensers (Bottles with Nozzles):

    - Used to apply or control solvents in cleaning or dissolving tasks.

    - Prevent overuse and minimise fumes.

  - Measuring Jugs and Mixing Sticks:

    - Used for mixing two-part adhesives or diluting chemicals.

- Features of Effective Equipment

  - Allows uniform application with minimal waste.

  - Ensures accurate control over flow or spray.

  - Easy to clean and maintain.

  - Made of non-reactive materials to resist corrosion or blockage.

- Maintenance and Cleaning

  - Adhesive spray guns must be flushed with solvent after each use.

  - Brushes and rollers should be cleaned immediately to avoid hardening.

  - Always follow the manufacturer’s instructions for tool care.

- Health and Safety Considerations

  - All equipment used with chemicals should be operated in ventilated areas.

  - Store tools safely to prevent contamination and spills.

  - Wear PPE when cleaning applicators, especially with solvents.

Example Table: Tools and Their Application

| Tool/Equipment | Used For | Notes |
| --- | --- | --- |
| Spray gun (compressor) | Applying contact adhesive | Requires regular maintenance and nozzle care |
| Aerosol adhesive can | Small-area bonding | Quick use, no clean-up, flammable |
| Brush or roller | Water-based adhesive application | Easy to control, must clean before hardening |
| Solvent dispenser | Spot cleaning and tool maintenance | Prevents spills and excessive solvent usage |

Case Study: Equipment Maintenance Failure at Daniels Upholstery

Background:
Daniels Upholstery in Kimberley received multiple complaints about uneven fabric adhesion and peeling foam layers in their newly delivered lounge chairs.

Investigation:
The spray adhesive guns had not been cleaned properly. The nozzles were partially blocked, causing irregular spray patterns and adhesive gaps.

Solution:
The workshop created a cleaning checklist for all glue equipment, with supervision from the team leader after each use. Usage logs were introduced to monitor tool maintenance.

Outcome:
Application quality improved, rework decreased, and staff became more aware of the importance of equipment care.

Critical Thinking Questions

- Why is it important to clean adhesive spray guns after use, even if they appear to be functioning?

- What risks might occur if a solvent dispenser leaks during application?

- How would you decide whether to use a brush or spray adhesive for a particular task?

- What are the possible outcomes of applying adhesive unevenly?

- How could poor tool maintenance affect the health and safety of workers?

Practical Classroom Activity

Task: Tool Identification and Usage Guide

Materials:

- Samples or visuals of spray guns, aerosol cans, brushes, rollers, measuring jugs

- Task cards (e.g. "Apply glue to foam seat base", "Clean dried glue from nozzle")

Instructions:

- Learners match each task card to the most appropriate tool.

- In pairs, they describe:

  - Why that tool is suitable

  - One maintenance step required for the tool

  - A potential risk if misused

- Groups present findings and receive peer feedback.

Extension:
Simulate a quality check: Inspect a mock "applied" panel (using paper) to spot issues caused by poor application techniques.

### KT0304: Measuring and Mixing Techniques and Procedures

Purpose of the Topic

This topic introduces learners to the correct measuring and mixing techniques required for preparing chemical substances used in upholstery, particularly adhesives and two-part systems. Learners will gain an understanding of how accuracy and procedural discipline influence the effectiveness, safety, and quality of chemical use in the workplace.

By mastering this topic, learners will be able to follow standard procedures for measuring, diluting, or mixing adhesives and solvents safely and correctly.

Key Content Areas

- Importance of Accurate Measurement

  - Ensures correct chemical properties are maintained.

  - Prevents underperformance (e.g. weak bonding or curing failure).

  - Reduces waste and cost due to overuse or errors.

  - Maintains compliance with product instructions and health regulations.

- Common Measuring Tools

  - Graduated jugs – for measuring liquids accurately by volume.

  - Digital or manual scales – for solid or resin-based substances.

  - Measuring syringes – for small volume solvents or activators.

  - Mixing sticks and containers – for blending components thoroughly.

- Mixing Procedures

  - Single-component adhesives: Stir before use if required.

  - Two-part systems (e.g. epoxy-based adhesives):

    - Must be mixed in precise ratios (e.g. 1:1 or 2:1).

    - Mix thoroughly until uniform in colour and consistency.

    - Use within specified working time or pot life.

  - Always follow the manufacturer’s instructions printed on packaging or data sheets.

- Mixing Environments and Safety

  - Use in ventilated areas with flat, clean surfaces.

  - Ensure containers are labelled and not cross-contaminated.

  - Wear PPE (gloves, goggles, aprons as applicable).

  - Avoid mixing near heat sources or open flames.

- Storage of Mixed Chemicals

  - Most mixed adhesives must be used immediately.

  - Unused mixed product often cannot be stored or reused.

  - Dispose of remaining mixed chemicals via approved channels.

Example: Mixing Instructions from a Product Label

Product: FoamFix PU 2-Part Adhesive
Mix Ratio: 2:1 by weight (Resin: Hardener)
Pot Life: 30 minutes at 20°C
Application: Spread with notched spatula; press surfaces together after 5 minutes
Curing Time: 12 hours minimum before load

Case Study: Ratio Error at Tshiamo Seating Projects

Background:
Tshiamo Seating Projects in Rustenburg mixed a batch of two-part adhesive by eye rather than measuring it. They used an approximate 1:1 ratio instead of the specified 2:1.

Result:
The mixture remained tacky and failed to cure properly, resulting in multiple product failures during quality inspection.

Action Taken:
Staff were retrained to use digital scales and refer to data sheets. Laminated mixing guides were posted in the glue station.

Learning Point:
Precision in measuring and mixing is not optional. Even small deviations can result in total product failure.

Critical Thinking Questions

- Why is the pot life of a mixed adhesive important, and how does it affect planning in production?

- What risks are associated with using incorrect ratios in two-part chemical systems?

- How would you handle a situation where only one component of a two-part adhesive is available?

- Why is it necessary to mix adhesives in a separate, labelled container?

- What are the environmental risks of incorrect disposal of unused mixed chemicals?

Practical Classroom Activity

Task: Simulate the Measurement and Mixing of Two-Part Adhesive

Materials:

- Measuring jugs, plastic containers, dummy chemicals (coloured water or gel)

- Product label mock-ups with mix ratio instructions

- Mixing sticks and labels

Instructions:

- Learners calculate required quantities using the given ratios (e.g. 200 g total at 2:1).

- Measure both parts using jugs or scales and mix in a container.

- Observe colour change or consistency (if using simulated products).

- Discuss what would happen if the incorrect ratio was used.

Extension:
Introduce a timing element to simulate pot life limits and decision-making under pressure.

### KT0305: Preparation and Application Techniques

Purpose of the Topic

This topic introduces learners to the practical steps and best practices for preparing surfaces and applying adhesives or solvents in upholstery processes. Proper preparation and application ensure the bond strength, finish quality, and durability of upholstered components.

Learners will explore the importance of surface readiness, application sequencing, and correct usage of tools and PPE during the process.

Key Content Areas

- Preparation Techniques

  - Surface Cleaning: Remove dust, grease, and debris before applying adhesives.

  - Dryness: Ensure all surfaces are dry to allow for full adhesive performance.

  - Material Alignment: Dry-fit foam, fabric, or padding before applying chemicals.

  - Ventilation: Set up in a well-ventilated space with adequate airflow.

  - Tool Preparation: Ensure all tools (spray guns, rollers, brushes) are clean and ready.

- Application Techniques

  - Spray Adhesive:

    - Apply at consistent distance (e.g. 20–30 cm) in a crosshatch pattern.

    - Allow required open time before bonding.

  - Contact Adhesive:

    - Apply thin layers to both surfaces.

    - Wait for tackiness before pressing together.

  - Brush-On Adhesive:

    - Use for smaller or more precise areas.

    - Avoid excess pooling or streaking.

  - Solvent Use:

    - Spot-apply using cloth or dispenser.

    - Avoid saturation or dripping onto sensitive materials.

- Sequencing and Workflow

  - Work from back to front or inside to outside where applicable.

  - Apply adhesive in sections to maintain control and accuracy.

  - Always wear gloves and a mask to avoid direct contact or inhalation.

  - Press and hold bonded surfaces with even pressure.

- Common Application Mistakes

  - Skipping surface prep → weak bonding.

  - Over-application → adhesive seepage and staining.

  - Poor alignment → wrinkling or foam shifting.

  - Insufficient drying time → long-term failures.

Example Table: Application Technique by Material

| Material Bonded | Technique | Tool/Method | Notes |
| --- | --- | --- | --- |
| Foam to wood frame | Contact adhesive | Spray gun or brush | Apply to both surfaces evenly |
| Fabric to foam | Water-based adhesive | Roller or brush | Allow full drying before stretching |
| Foam layering | Spray adhesive | Compressor spray gun | Light, even coats with 1 min open time |
| Solvent for cleaning gun | Spot wipe | Cloth + solvent dispenser | Use in ventilated area with gloves |

Case Study: Application Failure at Luthando & Sons Upholstery

Background:

Luthando & Sons Upholstery in Queenstown received a complaint from a long-time client. The newly upholstered daybeds had visible bubbles and wrinkles in the seat surface after just a few days.

Investigation:
It was discovered that adhesives had been applied too heavily and unevenly, and that the team bonded the materials before the open time had passed.

Solution:
The company introduced a visual step-by-step application chart above the glue station and scheduled a practical workshop with their supplier for all staff.

Learning Point:
Following preparation and application protocols precisely avoids rework and maintains client trust.

Critical Thinking Questions

- Why is it important to allow the full open time before bonding surfaces with contact adhesive?

- What are the risks of over-applying adhesive to soft materials like foam?

- How can misalignment during application affect the final quality and longevity of a product?

- What strategies can a workshop use to ensure all workers follow correct application sequences?

- What would you do if you noticed that a team member regularly skips surface preparation steps?

Practical Classroom Activity

Task: Simulated Adhesive Application Sequence

Materials:

- Scrap foam and board pieces

- Simulated adhesives (or labelled water for practice)

- Clean rags, spray bottles, brushes, gloves

- Pre-labelled job cards with instructions (e.g. “Foam to board – spray adhesive”)

Instructions:

- Learners work in pairs to simulate each step of the preparation and application sequence.

- Emphasise correct tool choice, surface prep, and waiting time.

- Groups rotate roles (applicator and observer) and provide peer feedback on technique.

Debrief Discussion:

- What step was most often rushed or overlooked?

- How would this impact a real-world job outcome?

### KT0306: Safety Hazards and Risks

Purpose of the Topic

This topic provides learners with an understanding of the health and safety hazards associated with the use of adhesives, solvents, and chemical application processes in upholstery. It equips learners with the knowledge to identify, mitigate, and respond to risks in a safe and responsible manner.

By the end of the session, learners should be able to apply workplace safety procedures confidently and understand how chemical hazards relate to personal and environmental health.

Key Content Areas

- Types of Chemical Hazards in Upholstery

  - Flammability: High risk from solvents and spray adhesives near heat sources.

  - Toxic Fumes: Inhalation of VOCs (volatile organic compounds) can cause respiratory issues, dizziness, or long-term harm.

  - Skin and Eye Irritation: Direct contact can result in burns, rashes, or sensitivity reactions.

  - Reactivity: Some chemicals may react with others or degrade under heat or sunlight.

- Typical Risk Situations

  - Using adhesives near naked flames or power tools with sparks.

  - Applying spray adhesive in poorly ventilated rooms.

  - Handling solvents without gloves or eye protection.

  - Storing incompatible chemicals together.

- Preventative Measures

  - PPE Use: Gloves, safety goggles, aprons, and respirators or masks.

  - Ventilation: Use of extractor fans, open windows, or dedicated glue stations.

  - Training: Reading and following the Safety Data Sheet (SDS) for each chemical product.

  - Storage: Clearly labelled, flammable materials stored in approved safety cabinets.

- Emergency Response Procedures

  - Know the location of first aid kits, eye wash stations, and emergency exits.

  - In the event of inhalation, move to fresh air immediately.

  - For spills, use absorbent material (never water on reactive substances).

  - Report incidents immediately to a supervisor or safety officer.

Example Table: Hazard and Preventative Action

| Hazard | Example | Preventative Action |
| --- | --- | --- |
| Flammable vapours | Using solvent near heat gun | Keep all flammable items away from heat |
| Inhalation of fumes | Spraying glue in a closed room | Use extractor fan and wear mask |
| Skin irritation | Handling solvent with bare hands | Wear gloves and wash hands immediately |
| Eye contact | Solvent splash while decanting | Wear safety goggles and face away from splash |

Case Study: Incident at Zanele Upholstery Works

Background:
At Zanele Upholstery Works in Soweto, a worker attempted to decant solvent into a smaller container while another staff member was welding a metal frame nearby. The solvent vapours ignited, resulting in a flash fire.

Investigation:
The workshop had no designated chemical area, and the worker had not received training on the dangers of flammable vapours.

Corrective Action:

- A flammable material zone was created, far from heat-producing equipment.

- All staff received training on chemical risks and PPE.

- SDSs were printed and posted on the walls near chemical storage and application areas.

Learning Point:
Lack of awareness of flammable vapour risks can have serious safety consequences.

Critical Thinking Questions

- Why must flammable adhesives and solvents be stored in a metal safety cabinet?

- What could happen if a spray adhesive is used near a heater or spotlight?

- How can you tell if a workshop is adequately ventilated for chemical use?

- What steps would you take if a co-worker accidentally spills solvent on their skin?

- Why is it important for all staff to be trained in emergency procedures, not just supervisors?

Practical Classroom Activity

Task: Hazard Identification and Prevention Simulation

Materials:

- Workshop layout poster or drawing

- Red/green hazard markers

- Cards with safety scenarios (e.g. “Using solvent with no gloves”, “Blocked fire exit”)

Instructions:

- Learners work in small groups to identify safety risks in a sample workshop layout.

- Use red markers to highlight hazard zones and green to mark safe practice zones.

- Each group selects one scenario card and acts out:

  - What went wrong

  - What should have been done

  - How the situation could be prevented next time

Debrief:
Discuss how easily routine actions can become dangerous without consistent safety practices.

## Integrated Assessment – KM-03-KT03

Qualification: Furniture Upholsterer (SAQA ID 103199)
Knowledge Module: KM-03-KT03 – Use of Chemicals: Adhesives and Solvents
NQF Level: 2 | Credits: 8 | Weighting: 10%
Assessment Duration: 60 minutes | Total Marks: 40

🔹 Section A: Matching Terms (1 mark each × 5 = 5 marks)

Match the chemical or safety concept in Column A with the correct description in Column B.

| Column A | Column B |
| --- | --- |
| A. Acetone | 1. A material that softens or dissolves another substance |
| B. Contact Adhesive | 2. High-tack adhesive applied to both surfaces |
| C. Pot Life | 3. Time available to use a mixed product before it sets |
| D. Solvent | 4. Commonly used cleaning agent; evaporates quickly |
| E. SDS | 5. A document outlining handling and safety information |

🔹 Section B: Short Answer Questions (3 marks each × 5 = 15 marks)

- List three types of chemicals used in upholstery and briefly state their purpose.

- Describe two characteristics of solvent-based adhesives that make them suitable for upholstery use.

- What does the term flammable mean in the context of chemical safety, and how should flammable materials be stored?

- List two key points to follow when applying adhesives using a spray gun.

- Identify two standard operating procedures for dealing with chemical spillages or waste in a workshop.

🔹 Section C: Practical Scenario – Application and Safety (2 marks each × 5 = 10 marks)

Scenario:
You are assigned to bond foam to a curved plywood backrest for a set of office chairs. The workshop is equipped with spray adhesives, water-based glue, acetone, and all standard PPE.

- Which adhesive would you choose and why?

- What preparation should you do before applying the adhesive?

- What PPE must be used during this task?

- Describe how you would safely clean the nozzle of a blocked spray gun using acetone.

- What is the correct sequence for applying contact adhesive to both surfaces?

🔹 Section D: Case Study – Scenario-Based Questions (2 marks each × 5 = 10 marks)

Case Study:

Jenna and Yusuf work at Coastal Upholstery. Jenna used solvent-based adhesive in a poorly ventilated corner of the workshop, while Yusuf was working nearby with a glue roller. Jenna forgot to wear gloves and got adhesive on her skin. There was no PPE signage in the area.

- Identify two safety risks presented in the case study.

- What actions should have been taken to prevent the incident?

- How can Jenna safely clean adhesive off her skin?

- Why is PPE signage important in a shared chemical area?

- Suggest two improvements to Coastal Upholstery’s chemical safety procedures.

✅ Model Answers and Marking Memo

Section A: Matching

A–4, B–2, C–3, D–1, E–5

Section B: Sample Answers

- Adhesives (for bonding), solvents (for cleaning), primers (for surface prep).

- Quick drying, strong bond, high tack.

- Easily catches fire; store in metal cabinet, labelled, away from heat.

- Maintain 20–30 cm distance, crosshatch spray pattern.

- Absorb spills, report to supervisor, use correct disposal method.

Section C: Sample Answers

- Spray adhesive – easier to apply to curved surfaces.

- Clean and dry both surfaces, align materials.

- Gloves, respirator or mask, goggles.

- Use cloth dampened with acetone, wear gloves, avoid ignition sources.

- Apply to both surfaces, wait until tacky, then press together.

Section D: Sample Answers

- Poor ventilation and lack of gloves.

- Use PPE, work in ventilated area.

- Wipe with approved solvent or hand cleaner, rinse skin.

- Reminds staff of PPE required and improves safety compliance.

- Install signage, conduct regular training, enforce PPE usage.

📊 Assessment Rubric

| Criteria | Outstanding (5) | Competent (3–4) | Partially Achieved (1–2) | Not Achieved (0) |
| --- | --- | --- | --- | --- |
| Knowledge of Chemicals | Full range identified with accurate properties and uses | Most chemicals understood with minor gaps | Limited recall; general or vague answers | No understanding of terms |
| Application Technique Knowledge | Accurate sequence and prep clearly explained | Basic understanding with some detail | Gaps in sequence or vague procedure | Incorrect or missing sequence |
| Safety Awareness | Risks well identified and mitigated in scenario-based tasks | Most risks identified; partial risk control | One risk or incorrect response | No risk identification |
| SOP and PPE Understanding | SOPs and PPE described with full relevance to chemical work | General understanding of procedures and gear | Incomplete or vague understanding | Does not reference SOP or PPE |

### Facilitator Assessment Briefing

Knowledge Module: KM-03-KT03 – Use of Chemicals: Adhesives and Solvents
Qualification: Furniture Upholsterer (SAQA ID 103199)
NQF Level: 2, Credits: 8
Assessment Duration: 60 minutes
Weighting: 10%
Assessment Type: Integrated Assessment using a Variety of Instruments

🎯 Purpose of the Assessment

This integrated assessment evaluates learners' ability to:

- Identify and describe various chemical types and their purposes in upholstery (IAC0301–IAC0303).

- Demonstrate knowledge of solvents, including their properties and applications (IAC0304–IAC0306).

- Explain and apply correct preparation, mixing, and application techniques (IAC0307).

- Recognise and respond to chemical safety hazards, procedures, and the correct use of PPE (IAC0308–IAC0310).

The assessment uses a variety of instruments to reflect theory, understanding, application, and judgement.

📝 Instruments Used

| Section | Instrument Type | Marks |
| --- | --- | --- |
| Section A | Matching | 5 |
| Section B | Short Answer | 15 |
| Section C | Scenario-Based Application | 10 |
| Section D | Case Study with Analysis | 10 |
| Total |  | 40 |

📌 Facilitator Preparation Checklist

- Print sufficient copies of the assessment paper and answer sheets.

- Ensure access to SDS documents, sample PPE, and examples of adhesives and solvents (real or simulated).

- Prepare any props for the scenario activities (e.g. labelled containers, tool visuals).

- Brief learners on assessment duration and format.

- Ensure the venue is quiet, well-lit, and ventilated, with a visible clock and emergency plan.

- Ensure a first aid kit is on hand and fire safety requirements are in place (for chemical-related scenarios or discussion).

✅ Instructions to be Read to Learners

- You will be assessed on your knowledge and understanding of the use of chemicals, adhesives, and solvents in the upholstery process.

- The assessment consists of four sections. Follow the instructions for each carefully.

- Use the answer sheet provided and write clearly.

- You will be given 60 minutes to complete the paper.

- Ask if you do not understand any question — the facilitator can clarify wording but will not provide answers.

- This is an individual assessment. No communication or assistance is allowed.

- Ensure your name is clearly written on all pages.

🧾 Marking and Moderation Guidelines

- Use the model answers and memo provided to allocate marks.

- Allow some flexibility in phrasing, provided core content is accurate and relevant.

- Ensure consistency in applying the rubric descriptors to evaluate competency.

- Retain completed scripts, signed marking checklists, and moderation evidence for external verification.

🧪 Special Considerations

- Where practical demonstrations are included (if adapted for classroom simulation), ensure safe handling of materials.

- Learners with barriers to learning should be accommodated per institutional policy and QCTO guidelines.

🔒 Assessment Integrity

- Keep assessment scripts in a secure place until marked.

- Record marks on the learner assessment record and back them up digitally.

- Any cases of misconduct must be documented and reported per the provider's assessment policy.

## KM-03-KT04: Upholstery Tools and Equipment

Weighting: 10% | NQF Level: 2

1. Purpose of the Knowledge Topic

The purpose of this Knowledge Topic is to equip learners with the essential knowledge and understanding of the tools and equipment used in upholstery covering processes. Learners will be introduced to hand tools, pneumatic tools, electric power tools, and covering equipment, along with their components, functions, safety mechanisms, and standard maintenance practices.

This topic further develops the learner’s awareness of safe handling procedures, equipment fault identification, and the selection of the appropriate tool and consumables for the task. In addition, learners will explore the importance of personal protective equipment (PPE) and materials handling equipment as part of a safe and productive working environment.

2. Key Knowledge Areas (Topic Elements)

The following topic elements are covered in this Knowledge Topic:

- KT0401: Upholstery hand tools for the furniture covering processes – operation, components and parts, uses, safe storage and handling

- KT0402: Pneumatic tools – settings and operation, components and parts, uses, safe storage and handling, and supply systems

- KT0403: Upholstery electric portable power tools – settings and operation, components and parts, uses, safe storage and handling

- KT0404: Upholstery covering equipment

- KT0405: Personal protective equipment and materials handling equipment

- KT0406: Tools and equipment maintenance, fault finding, and standard operating procedures

- KT0407: Safety measures and mechanisms

3. Internal Assessment Criteria and Weighting

This Knowledge Topic carries a total weighting of 10% and will be assessed using the following Internal Assessment Criteria:

- IAC0401: Hand tools used by the cover fitter are identified and respective purposes are explained

- IAC0402: Parts of hand tools are identified and the maintenance procedures for each tool are described

- IAC0403: The consequences of not handling, cleaning, and maintaining hand tools correctly are assessed

- IAC0404: Pneumatic power tools used by the cover fitter are identified and respective purposes explained

- IAC0405: Parts and safety mechanisms of pneumatic power tools are identified and maintenance procedures for each tool are explained

- IAC0406: Consequences of not handling, cleaning, and maintaining pneumatic power tools correctly are assessed

- IAC0407: Electric power tools used by the cover fitter are identified and respective purposes described

- IAC0408: Parts and safety mechanisms of electric power tools are identified and maintenance procedures for each tool are explained

- IAC0409: Consequences of not handling, cleaning, and maintaining electric power tools correctly are assessed

- IAC0410: The selection and loading of staples, nails, bits, and other consumables into the relevant power tool are reasoned

4. Application in Upholstered Furniture Manufacturing

The correct use of tools and equipment is essential for producing high-quality upholstered furniture efficiently and safely. This topic enables learners to understand not only how to use each tool but also how to choose the most appropriate tool for a specific function. A strong emphasis is placed on safety mechanisms, routine maintenance, and preventative care, to ensure tool longevity and worker safety.

In addition, learners will be prepared to apply standard operating procedures in real workshop environments, make informed decisions about consumables, and comply with relevant workplace safety protocols.

### KT0401: Upholstery Hand Tools for the Furniture Covering Processes – Operation, Components and Parts, Uses, Safe Storage and Handling

Purpose of the Topic

This topic introduces learners to the manual hand tools used during the furniture covering process. Learners will gain knowledge of the operation, parts, correct handling, and storage of tools used by a cover fitter in an upholstery workshop. Understanding the correct use and care of hand tools supports efficient work, reduces waste, and promotes safety.

By the end of the topic, learners will be able to identify key tools, describe their purpose, and explain how to store and handle each safely.

Key Content Areas

- Common Upholstery Hand Tools

  - Tack Hammer: Used for inserting tacks or nails into the frame or webbing.

  - Staple Lifter: Used to remove staples or tacks without damaging the frame.

  - Regulator Needle: Smooths out lumps under the fabric by repositioning stuffing.

  - Upholstery Shears: Extra-sharp scissors used to cut heavy fabric or lining.

  - Webbing Stretcher: Applies tension to webbing before tacking it down.

  - Rubber Mallet: Used to drive tacks or manipulate frame joints without damage.

  - Ripping Chisel: Used to pry off fabric and dismantle old upholstery.

- Components and Parts

  - Handles (wooden or plastic): Ensure good grip and user control.

  - Heads or blades: The active part of the tool, often hardened for durability.

  - Joints (in shears): Enable movement and precise control.

- Uses in Covering Processes

  - Tacking down upholstery fabric.

  - Cutting material to fit curved or angled surfaces.

  - Removing old staples or recovering a frame.

  - Re-adjusting inner layers like padding or foam.

- Safe Handling and Usage

  - Use tools for their intended purpose only.

  - Always cut away from the body with sharp tools.

  - Use a stable work surface and maintain good posture when applying pressure.

  - Do not force dull tools – this increases the risk of injury.

- Safe Storage of Hand Tools

  - Store in toolboxes or tool boards with designated compartments.

  - Keep sharp edges covered or sheathed.

  - Clean tools before storing to prevent corrosion or contamination.

  - Store regulators and needles in protective sleeves to avoid punctures.

- Consequences of Poor Handling or Maintenance

  - Blunt scissors or shears may tear fabric, increasing material costs.

  - Dropping tools may damage the cutting edge or pose a safety hazard.

  - Improper storage leads to rust, dullness, or injury.

  - Not removing dust and adhesives may shorten tool lifespan.

Example: Tool Identification Table

| Tool Name | Function | Safe Storage Tip |
| --- | --- | --- |
| Webbing Stretcher | Pulls webbing tight before tacking | Hang vertically to avoid handle warping |
| Regulator Needle | Adjusts stuffing inside fabric | Store with tip guard or in roll-up pouch |
| Tack Hammer | Drives tacks into frame | Keep separate from other heavy metal tools |
| Upholstery Shears | Cuts upholstery fabric | Sheath blade and avoid moisture |

Case Study: Improper Tool Use at Marais Upholstery Studio

Background:

Marais Upholstery Studio in Pretoria experienced multiple delays and fabric waste in a batch of custom lounge chairs. The apprentice, Tanveer, had used household scissors instead of upholstery shears, and the shears in the workshop were blunt and rusted.

Findings:
The incorrect cutting tools caused frayed edges, resulting in misaligned fabric pieces and significant rework.

Resolution:
The workshop supervisor purchased a set of professional shears and implemented a monthly sharpening schedule. All apprentices were trained in tool identification and care.

Learning Point:
Using the correct, well-maintained tool for the task saves time, improves product quality, and prevents wastage.

Critical Thinking Questions

- Why is it important to use a webbing stretcher instead of pulling webbing by hand?

- What are the risks of using blunt scissors or dull shears on upholstery fabric?

- How would you teach a new apprentice to store and maintain hand tools safely?

- What steps can be taken to prevent regulators and needles from causing injury in the toolbox?

- If a staple lifter breaks mid-use, what is the appropriate course of action?

Practical Classroom Activity

Task: Tool Identification and Safe Handling Demonstration

Materials:

- Real or image samples of hand tools

- Toolboxes, sleeves, or foam blocks for storage

- Labelling cards

Instructions:

- Learners identify each hand tool, its function, and the part(s) most vulnerable to damage.

- In groups, learners demonstrate correct handling of one tool.

- Groups develop and present a tool care checklist for safe storage and use.

Extension:
Conduct a short quiz where learners match tools to a list of upholstery activities (e.g. “Remove staples from dining chair frame”).

### KT0402: Pneumatic Tools – Settings and Operation, Components and Parts, Relationship Between Parts, Uses, Safe Storage and Handling, Supply Systems

Purpose of the Topic

This topic provides learners with a comprehensive understanding of pneumatic tools used in furniture covering. Pneumatic tools are powered by compressed air, and are commonly used in upholstery workshops for their speed, efficiency, and reduced physical strain on the user.

Learners will explore the parts and functioning of pneumatic tools, how to set and use them safely, and the requirements of compressed air systems in a workshop. Safe storage, maintenance, and risk awareness are emphasised throughout.

Key Content Areas

- What Are Pneumatic Tools?

  - Tools powered by compressed air, typically connected to a central air compressor via hoses.

  - Commonly used in upholstery for repetitive tasks like stapling, nailing, or tacking.

- Common Pneumatic Tools in Upholstery

  - Pneumatic Stapler: Fires staples into wood frames with high precision.

  - Pneumatic Nailer: Used to attach decorative nails or fasten backing boards.

  - Pneumatic Tack Gun: Drives upholstery tacks consistently and cleanly.

- Main Components and Relationships Between Parts

  - Air Compressor: Generates and stores pressurised air.

  - Hoses and Connectors: Deliver compressed air from the compressor to the tool.

  - Trigger and Safety Lock: Activates the firing mechanism.

  - Magazine: Holds staples, nails, or other consumables.

  - Exhaust Vent and Pressure Regulator: Manages airflow and pressure output.

- Operation and Settings

  - Check compressor pressure is within the tool’s operating range (usually 80–120 PSI).

  - Load correct size and type of staples or nails.

  - Connect the hose and test on a scrap material first.

  - Always engage the safety lock when not in use.

  - Adjust depth-of-drive setting to control how deep the fastener goes.

- Safe Handling and Storage

  - Disconnect tools from air supply when not in use.

  - Store in a dry area, in a padded toolbox or wall-mounted rack.

  - Never point the tool at anyone or carry it by the hose.

  - Release air pressure and drain compressor tank daily to prevent moisture build-up.

- Air Supply Systems in a Workshop

  - Central compressor system with air regulators, moisture traps, and pressure gauges.

  - Routine inspection of hoses, seals, and filters prevents failure and injury.

  - Air lines must be labelled and securely mounted to avoid trip hazards.

Example Table: Pneumatic Tool Comparison

| Tool Type | Typical Use | Consumable | Pressure Range (PSI) | Safety Feature |
| --- | --- | --- | --- | --- |
| Pneumatic Stapler | Attaching fabric to wooden frame | Staples | 80–100 | Safety nose/trigger lock |
| Pneumatic Nailer | Securing trim, boards, or frame parts | Nails | 90–110 | Anti-dry fire mechanism |
| Pneumatic Tack Gun | Applying decorative tacks | Tacks | 85–100 | Trigger guard |

Case Study: Safety Oversight at De Klerk Interiors

Background:
At De Klerk Interiors in Port Elizabeth, an employee, Megan, failed to adjust the air pressure before using a pneumatic stapler on a thin birch frame. The staples over-penetrated the wood, damaging the frame and emerging through the fabric.

Investigation:
The compressor pressure was set too high (135 PSI), and Megan had not conducted a test fire or reviewed the material thickness.

Resolution:

- A standard air pressure checklist was introduced.

- Employees were retrained to adjust PSI and test on scrap first.

- Colour-coded pressure settings were added to tool stations.

Learning Point:
Incorrect pneumatic tool settings can result in material damage, injury, and time lost to rework.

Critical Thinking Questions

- Why is it important to check the air pressure before using a pneumatic stapler?

- What might happen if the wrong size staples are loaded into a pneumatic tool?

- How does the relationship between the compressor and the tool affect performance?

- What routine maintenance should be carried out on air hoses and connectors?

- What steps should be taken if a pneumatic tool jams during use?

Practical Classroom Activity

Task: Pneumatic Tool Safety Simulation and Component Labelling

Materials:

- A pneumatic stapler or tack gun (non-pressurised demo or visual aids)

- Compressor unit diagram or sample hoses

- PPE (gloves, goggles)

- Labels or markers

Instructions:

- Learners identify and label parts of a pneumatic tool (e.g. magazine, trigger, hose connector).

- Watch or review a demonstration on connecting a pneumatic tool to a compressor safely.

- In groups, learners list the steps to take before and after using a pneumatic tool.

- Discuss what each group would do in the event of an air hose leak or tool malfunction.

Extension:
Have learners record a “safety briefing” script they could deliver to a new team member about using pneumatic tools.

### KT0403: Upholstery Electric Portable Power Tools – Settings and Operation, Components and Parts, Uses, Safe Storage and Handling

Purpose of the Topic

This topic introduces learners to portable electric power tools used in the upholstery covering process. These tools increase speed and precision in operations such as cutting fabric, trimming foam, and preparing frame elements. Learners will explore how electric tools function, how to operate them safely, and the importance of regular care, correct settings, and secure storage.

Mastery of this topic will enable learners to safely and efficiently integrate power tools into their workflow, while protecting both the tool and the user.

Key Content Areas

- What Are Electric Portable Power Tools?

  - Handheld, motorised tools powered by electricity (mains or battery).

  - Used for cutting, drilling, sanding, grinding, and other upholstery-related tasks.

- Common Electric Tools in Upholstery

  - Electric Foam Cutter: Used to cut foam with precision, including angled or curved cuts.

  - Electric Staple Gun: Less common than pneumatic but used where air compressors are not available.

  - Fabric Shears (Electric): High-speed scissors for long cuts on heavy upholstery fabric.

  - Cordless Drills/Screwdrivers: For frame preparation or fastening components.

  - Angle Grinders (with sanding attachments): Sometimes used in frame preparation or smoothing components.

- Key Components and Settings

  - Trigger Switch: Powers the tool on/off.

  - Speed Control: Adjusts rotation or cutting speed.

  - Blades/Heads: Must be compatible and properly secured.

  - Battery Pack (Cordless Tools): Should be charged and checked before use.

  - Power Cable and Plug (Corded Tools): Must be inspected for wear.

- Operational Safety

  - Always inspect tool and cord before use.

  - Secure materials before cutting or drilling.

  - Do not force the tool – let it do the work.

  - Disconnect from power when changing blades or cleaning.

- Safe Storage and Handling

  - Coil cords neatly and store tools in dry, clean environments.

  - Keep blades protected or retracted.

  - Store in lockable tool cabinets or dedicated shelves.

  - Charge batteries away from flammable materials and follow manufacturer instructions.

- Hazards and Risk Prevention

  - Electric shock from damaged cords or exposed wiring.

  - Injuries from incorrect cutting technique or unsecured materials.

  - Eye injuries from flying particles – always wear goggles.

  - Fire hazard from overheated components or improper use near upholstery foam.

Example Table: Electric Tool Use and Precautions

| Tool | Function | Safety Tip |
| --- | --- | --- |
| Electric Foam Cutter | Cut foam precisely and quickly | Keep blade cool; wear heat-resistant gloves |
| Fabric Shears | Cut long lengths of fabric | Unplug before changing blades |
| Cordless Drill | Install screws into frame | Check battery charge and chuck tightness |
| Angle Grinder | Smooth wood or metal components | Always wear eye protection and gloves |

Case Study: Electric Cutter Incident at Smit & Pillay Upholstery

Background:
At Smit & Pillay Upholstery in Benoni, an apprentice, Neo, used an electric foam cutter to trim seat cushions. He did not wait for the blade to cool before placing it down on a vinyl offcut.

Incident:
The hot blade melted through the vinyl, damaging both the floor mat and the cutter. No one was injured, but the tool was permanently damaged.

Resolution:
A cooling stand was introduced for hot tools, and a brief refresher course on power tool safety was conducted.

Learning Point:
Electric tools retain heat and can cause damage or injury even after they are switched off.

Critical Thinking Questions

- What are the risks of using electric tools in a damp or poorly ventilated area?

- Why is it important to match the tool blade or bit to the task at hand?

- How can you tell if a corded tool is safe to use?

- What steps should be taken if a battery becomes hot during use?

- Why is it important to store electric tools in designated, dry spaces?

Practical Classroom Activity

Task: Electric Power Tool Safety and Setup Simulation

Materials:

- Demonstration tools (non-live or unplugged) such as foam cutters, drills, and shears

- Sample materials (foam, fabric, wood blocks)

- Safety goggles, gloves, and dummy power cords

Instructions:

- Learners identify tool components and explain their functions.

- Each learner demonstrates (with a non-powered tool) how to inspect, set up, and prepare the tool for use.

- Class discusses what to do before, during, and after using each tool.

- Each group presents one potential hazard and one solution for a selected electric tool.

Top of Form

### KT0404: Upholstery Covering Equipment

Purpose of the Topic

This topic introduces learners to the larger-scale and specialised equipment used in upholstery covering processes, beyond hand and power tools. It includes equipment that assists with fabric handling, pressing, shaping, steaming, and assembling padded or covered sections of furniture. This knowledge ensures learners are familiar with equipment setup, safe operation, and care, and can identify the correct machine or apparatus for a specific upholstery covering task.

Key Content Areas

- Definition of Upholstery Covering Equipment

  - Equipment that supports the finishing stages of upholstery: shaping, smoothing, steaming, pressing, or attaching layers of fabric and padding.

  - Typically includes workshop-based machinery or larger portable units used in tandem with hand tools.

- Examples of Upholstery Covering Equipment

  - Industrial Steamers: Used to soften and relax fabric for stretching over frames.

  - Pressing Tables or Pads: Used to press fabric flat or shape contours during application.

  - Padding Machines: Compress and hold layers of padding or foam during assembly.

  - Staple/Trim Presses: Used in high-volume settings to drive multiple fasteners in one movement.

  - Work Benches with Upholstery Fixtures: Adjustable tables with clamps and supports for holding frames in place.

  - Upholstery Hoists or Lifters: Assist in lifting or positioning large pieces safely.

- Functions and Benefits

  - Speeds up the covering process.

  - Improves consistency and finish quality.

  - Reduces manual strain on workers.

  - Enhances safety when handling large or curved pieces.

- Operational Safety

  - Ensure machines are turned off when adjusting settings.

  - Use heat-resistant gloves when operating steamers or presses.

  - Keep cords, hoses, and attachments tidy and clear of walkways.

  - Use equipment only after training or demonstration.

- Storage and Maintenance

  - Drain and clean steamers daily to prevent mineral build-up.

  - Cover pressing tables when not in use.

  - Inspect all mechanical fixtures for looseness or damage.

  - Follow manufacturer maintenance schedules for each piece of equipment.

Example Table: Covering Equipment Overview

| Equipment | Function | Handling Precaution |
| --- | --- | --- |
| Industrial Steamer | Softens fabric for easier stretching | Avoid direct skin contact with steam |
| Upholstery Press Table | Shapes and flattens padded sections | Ensure work area is heat resistant |
| Staple Press | Drives multiple fasteners at once | Use both hands and check pressure settings |
| Adjustable Work Bench | Holds frames during covering | Lock wheels or clamps before use |

Case Study: Equipment Handling at Langston Upholstery Co.

Background:
Langston Upholstery Co. in Bloemfontein installed a new steam press table to assist with deep-buttoned ottoman production. Zanele, a junior upholsterer, attempted to operate the press without gloves or instruction, resulting in a minor hand burn.

Findings:

- No safety signage was present.

- Zanele had not been shown the proper cool-down sequence.

Corrective Action:

- Staff received refresher training on operating heated and pressurised equipment.

- Safety signage and laminated quick-use guides were attached to all equipment.

Learning Point:
Introducing new equipment requires training, supervision, and clearly posted instructions.

Critical Thinking Questions

- What are the benefits of using a steamer during the fabric stretching process?

- Why should padding machines be adjusted before power is switched on?

- What risks are associated with pressing equipment in a shared workshop?

- How can upholstery lifters or hoists prevent injuries?

- What maintenance step is essential after using a steam-based tool?

Practical Classroom Activity

Task: Equipment Identification and Handling Planning

Materials:

- Photographs or diagrams of 3–4 upholstery covering equipment types

- Handling guideline cards (e.g. “Apply heat-resistant gloves”, “Drain after use”)

- Table or whiteboard to group items

Instructions:

- Learners match each equipment image with its function and key handling rule.

- Groups discuss what can go wrong if the equipment is used improperly.

- Learners design a safety checklist poster for one machine.

Extension:
Invite learners to demonstrate a safe mock-use of equipment (if equipment is unavailable, simulate the action and discuss each safety step aloud).

### KT0405: Personal Protective Equipment and Materials Handling Equipment

Purpose of the Topic

This topic introduces learners to the essential Personal Protective Equipment (PPE) and materials handling equipment used in upholstery workshops. Learners will explore how PPE contributes to personal safety and how handling equipment supports safe, efficient movement and positioning of tools, materials, and furniture components.

By understanding the correct use, maintenance, and storage of these items, learners will be better equipped to work responsibly and prevent workplace injuries.

Key Content Areas

- Definition of PPE and Handling Equipment

  - PPE: Gear worn by workers to protect against health or injury risks.

  - Materials Handling Equipment: Tools or machinery used to lift, move, or position heavy or bulky items.

- Types of PPE Used in Upholstery

  - Gloves: Protect hands from adhesives, solvents, cuts, or abrasions.

  - Safety Glasses or Goggles: Prevent eye injuries from flying particles, staples, or splashes.

  - Respirators or Masks: Protect against dust and inhalation of chemical fumes.

  - Aprons or Overalls: Shield clothing and skin from dirt, adhesives, or sparks.

  - Ear Protection: Used in areas with loud staple guns or cutting machines (less common but important).

- Types of Materials Handling Equipment

  - Trolleys and Carts: For moving tools, materials, or furniture frames safely.

  - Hoists or Lifters: Mechanised or manual devices for lifting heavier objects.

  - Padded Slings or Straps: Help to manoeuvre curved or padded items.

  - Roll Racks or Fabric Rollers: Make it easier to dispense and store fabric rolls.

- Safe Use and Storage

  - PPE must be worn consistently, even for short tasks.

  - Items like goggles and gloves must be cleaned and stored in labelled lockers or drawers.

  - Handling equipment must be checked regularly for damage (e.g. broken wheels or loose straps).

  - Do not overload trolleys; always balance loads properly.

- Workplace Integration

  - PPE requirements must be clearly posted in all workshop zones.

  - Learners should know when and where each type of PPE is required.

  - All staff should be trained in manual handling techniques to avoid strain injuries.

Example Table: PPE and Handling Equipment in Use

| Item | Purpose | Key Instruction |
| --- | --- | --- |
| Chemical-resistant gloves | Prevent skin contact with solvents or glue | Replace when torn or sticky |
| Respirator mask | Avoid inhaling spray fumes or sanding dust | Fit securely and check filters regularly |
| Trolley | Move frames and foam between stations | Do not drag heavy loads across uneven floors |
| Safety goggles | Protect eyes from staple or solvent splash | Store in protective case when not in use |

Case Study: Missed PPE Use at Bartlett Design Works

Background:
At Bartlett Design Works in George, Brent was using a staple gun on a padded armrest without goggles. A staple deflected and bounced off the armrest, hitting him in the cheek. Fortunately, the injury was superficial.

Investigation:

- No signage was visible above the stapling station.

- Goggles were stored in a locked cupboard, making them less accessible.

Resolution:

- Clear signage was added to all stapling and cutting stations.

- Goggles and gloves were moved to open-access safety shelves.

- Brent was given first aid and required to attend a safety refresher.

Learning Point:
Even familiar, everyday tasks require full PPE compliance, and equipment should be easy to access at all times.

Critical Thinking Questions

- Why is it important to wear PPE even for short or “quick” tasks?

- What injuries might result from lifting a large ottoman frame alone?

- How can workshop layout influence safe materials handling?

- What are the consequences of using a damaged trolley in the workshop?

- Why should PPE like gloves or masks be personal to each worker?

Practical Classroom Activity

Task: PPE and Equipment Matching and Demonstration

Materials:

- Physical PPE items or visuals (gloves, goggles, mask, apron)

- Sample handling equipment (trolley, fabric roll dispenser – visual or actual)

- Station signs with hazards (e.g. “Solvent Station”, “Cutting Bench”, “Stapling Area”)

Instructions:

- Learners match PPE to station type and explain their choices.

- Each learner practises how to correctly wear and remove PPE.

- In pairs, simulate a safe lifting technique using a mock object (e.g. foam block).

- Groups identify potential hazards if PPE is not used and present brief risk statements.

Extension:
Have learners design a PPE reminder poster for a specific upholstery process and present it to the class.

### KT0406: Tools and Equipment Maintenance, Fault Finding and Standard Operating Procedures

Purpose of the Topic

This topic focuses on the care, maintenance, and fault identification procedures for upholstery tools and equipment. Learners will develop an understanding of how to follow standard operating procedures (SOPs) for cleaning, servicing, and troubleshooting common issues in upholstery tools — from manual tools to power-driven and pneumatic equipment.

Proper tool care extends lifespan, prevents costly damage, and ensures safe, efficient operation within the workshop.

Key Content Areas

- Why Maintenance Matters

  - Prevents downtime due to breakdowns.

  - Improves tool safety and performance.

  - Ensures consistency in quality of workmanship.

  - Complies with manufacturer warranties and workshop policy.

- Basic Maintenance Tasks

  - Daily Cleaning: Wipe down tools, remove dust, adhesive residue, or fabric fibres.

  - Lubrication: Apply suitable oil to moving parts (e.g. scissors pivot, drill chucks).

  - Sharpening: Regularly sharpen blades and scissors using appropriate techniques or services.

  - Air Tool Draining: Release moisture from pneumatic hoses and tanks.

  - Battery Maintenance: Charge only on designated charging stations and check for swelling or overheating.

- Fault Finding Techniques

  - Visual Inspection: Check for cracks, fraying cords, loose screws, or missing parts.

  - Functional Check: Run tool briefly (if safe) to check for abnormal sounds or jerky movements.

  - Troubleshooting Common Problems:

    - Pneumatic stapler misfiring → check air pressure or staple alignment.

    - Electric cutter overheating → blade may be dull or motor clogged.

    - Shears not closing smoothly → hinge may need lubrication or cleaning.

- Standard Operating Procedures (SOPs)

  - Each tool type should have a step-by-step checklist for cleaning, use, and storage.

  - SOPs should be:

    - Visible and accessible in the work area.

    - Updated regularly in line with manufacturer guidelines.

    - Taught during onboarding and reviewed quarterly.

- Recording and Reporting

  - Use maintenance logs to track repairs, replacements, and scheduled servicing.

  - Any fault or damage must be reported to the supervisor immediately.

  - Only qualified personnel should conduct complex repairs.

Example Table: Maintenance Responsibilities

| Tool/Equipment | Maintenance Task | Frequency | SOP Reminder |
| --- | --- | --- | --- |
| Pneumatic Stapler | Clean magazine and check pressure | Daily | Disconnect air before cleaning |
| Foam Cutter | Check blade tension and sharpness | Weekly | Cool down before storing |
| Fabric Shears | Oil hinge, check alignment | Weekly | Store in dry, padded sleeve |
| Electric Drill | Inspect cord and bit tightness | Before each use | Unplug before changing bit |

Case Study: Fault Ignored at Creative Furnishings

Background:
At Creative Furnishings in East London, Chantelle noticed that the electric staple gun was sparking slightly when used. She continued to use it because she was behind schedule.

Incident:
Later that afternoon, the staple gun failed entirely, and the power board short-circuited, causing a delay across the production line and damaging a nearby foam cutter.

Investigation & Resolution:

- The tool had a frayed power cable, which had not been reported.

- A formal tool check-in/check-out system was introduced.

- Staff received refresher training on identifying faults and escalation procedures.

Learning Point:
Minor faults must be taken seriously. Early reporting and inspection prevent safety hazards and productivity loss.

Critical Thinking Questions

- What might happen if a blunt foam cutter is not reported and continues to be used?

- Why should all tools be cleaned before storing, even if they will be used the next day?

- How can a workshop prevent misunderstandings about who is responsible for maintaining a tool?

- Why is it important for each tool to have its own SOP posted?

- What system could your team introduce to track tool maintenance effectively?

Practical Classroom Activity

Task: Tool Maintenance SOP Simulation

Materials:

- Sample tools (or images), maintenance checklists, cleaning cloths, oil, dummy logbooks

- Flipchart paper or printed SOP templates

Instructions:

- Learners select a tool and list daily, weekly, and monthly maintenance tasks.

- In pairs, they simulate checking and cleaning the tool using the SOP.

- Each group fills in a mock maintenance log entry.

- Groups present their SOP poster or checklist and explain one common fault and how to prevent it.

Extension:
Facilitator introduces a mystery fault (e.g. tool jammed, cord frayed), and learners must inspect and identify the issue using a structured fault-finding form.

### KT0407: Safety Measures and Mechanisms

Purpose of the Topic

This topic introduces learners to the safety systems, mechanisms, and practices that are essential when working with upholstery tools and equipment. It builds on previous knowledge by focusing on preventative safety, built-in protective features, and how to act in unsafe situations.

Learners will be equipped to identify and apply safety mechanisms in tools and workshop environments, helping to create a culture of risk awareness and responsible tool use.

Key Content Areas

- Types of Safety Mechanisms in Tools and Equipment

  - Trigger Locks: Prevent accidental firing of staple and nail guns.

  - Pressure Release Valves: Found on compressors to avoid over-pressurisation.

  - Blade Guards and Shields: On electric shears, foam cutters, and saws.

  - Dead-Man Switches: Tools only operate while actively held or pressed.

  - Automatic Shut-Off Features: Prevent overheating or tool burnout.

- Personal and Environmental Safety Measures

  - Use of correct PPE (goggles, gloves, masks).

  - Clearly marked safety zones and designated equipment areas.

  - Good housekeeping – no cords, tools, or clutter on the floor.

  - First Aid Station clearly marked and stocked.

  - Emergency Procedures – fire evacuation plans, chemical spill protocols.

- Safe Working Practices

  - Always follow manufacturer instructions and SOPs.

  - Inspect tools before use for faults or wear.

  - Never bypass safety features for speed.

  - Only use tools for their intended purpose.

- Behavioural Safety Culture

  - Report unsafe practices immediately.

  - Support and remind peers to wear PPE and follow safety rules.

  - Keep incident logs and hold regular safety talks or tool-box briefings.

Example Table: Tool Safety Features and Functions

| Tool or Equipment | Safety Feature | Function |
| --- | --- | --- |
| Pneumatic Stapler | Nose safety lock | Prevents firing unless nose is pressed against surface |
| Electric Shears | Blade guard | Protects user from accidental contact with blade |
| Air Compressor | Pressure relief valve | Releases excess air to avoid explosion |
| Cordless Drill | Trigger lock | Prevents accidental activation during transport |

Case Study: Safety Mechanism Bypassed at Somerset Seating

Background:
At Somerset Seating in Paarl, Ridwaan disabled the nose safety lock on his pneumatic stapler to “save time” while attaching dust covers. During use, the stapler fired unintentionally, grazing his leg.

Investigation:

- Ridwaan had not reported a previous issue with the safety lock.

- The workshop culture discouraged taking time for tool inspections.

- There was no supervisor present during the shift.

Corrective Action:

- Immediate tool servicing was mandated for all faulty safety mechanisms.

- A “no modification” policy was introduced.

- Safety champions were appointed per workshop team.

Learning Point:
Bypassing or ignoring safety mechanisms undermines the protection they are designed to provide.

Critical Thinking Questions

- What is the purpose of a dead-man switch, and when might it be used in upholstery tools?

- Why should learners never override safety features, even if it “saves time”?

- What simple workshop behaviours help prevent injuries before they occur?

- How can a team encourage each other to consistently follow safety protocols?

- What should you do if you notice a colleague using a faulty tool?

Practical Classroom Activity

Task: Safety Walkthrough and Hazard Identification

Materials:

- Sample safety signs, PPE, tool visuals or actual tools

- Clipboards and safety checklists

- Hazard cards (e.g. “Staple gun on bench with cord tangled”, “No goggles at trimming station”)

Instructions:

- Learners use a safety checklist to perform a simulated “walkthrough” of a model workspace (can be real or diagram-based).

- They identify at least five hazards or missing safety mechanisms.

- In groups, learners present:

  - What they found

  - Why it was unsafe

  - What should be done to resolve it

- Facilitator leads a discussion on how to apply these principles in a real workplace.

## Integrated Assessment – KM-03-KT04

Qualification: Furniture Upholsterer (SAQA ID 103199)
Knowledge Module: KM-03-KT04 – Upholstery Tools and Equipment
NQF Level: 2 | Credits: 8 | Weighting: 10%
Assessment Type: Case Study with Scenario-Based Questions
Duration: 60 minutes
Total Marks: 40

📘 Case Study: Tool Management at Mathews Upholstery Studio

Background:
Mathews Upholstery Studio in Gqeberha is a busy workshop specialising in lounge suites and office seating. The team includes Nomvula (a senior upholsterer), Pieter (a junior technician), and Ayanda (an apprentice).

During a recent week, several incidents occurred in the tool and equipment station:

- Pieter used household scissors to cut upholstery fabric. He explained that the proper shears were blunt and had not been sharpened in weeks.

- Ayanda was using a pneumatic stapler with the safety lock disabled to “save time”. She did not drain the compressor hose at the end of the day.

- Nomvula used an electric foam cutter that had a frayed power cord. It sparked during use, and she unplugged it and placed it back in the drawer without tagging it for inspection.

- In the storeroom, multiple toolboxes were disorganised. Tools were sticky, dusty, and scattered across benches.

- A new delivery of staples and nails arrived, but Pieter mistakenly loaded long frame nails into the staple gun, which caused a jam and delayed the day’s work.

Management conducted a review and issued a notice to the team, requiring immediate retraining on tool identification, maintenance procedures, and safety mechanisms.

❓ Scenario-Based Questions (4 marks each × 10 = 40 marks)

- Identify two hand tools that should have been used instead of household scissors and explain their correct purpose.

- What parts of a pneumatic stapler must be checked before use, and what maintenance step did Ayanda neglect?

- Describe two consequences of using a pneumatic stapler with a disabled safety mechanism.

- List two key safety features on an electric foam cutter and explain how they help prevent injury.

- Nomvula failed to report a tool fault. What procedure should she have followed after noticing the sparking power cord?

- Describe two maintenance procedures that should be routinely followed for hand tools like shears or staple lifters.

- What is the purpose of draining a compressor hose, and what could happen if this is neglected over time?

- How should tools be stored to avoid damage and promote safety in a shared upholstery workspace?

- Pieter loaded incorrect consumables into a pneumatic tool. Explain the correct process for selecting and loading staples or nails.

- Based on the events in the case study, identify three workshop rules or SOPs that could prevent these incidents from recurring.

✅ Model Answers and Marking Memo

- Upholstery shears (for clean cuts through thick fabric), fabric chalk and measuring tools (to plan and guide cuts).

- Check magazine, air hose connection, safety lock, and trigger. Ayanda failed to drain the air hose, which prevents moisture build-up.

- Risk of accidental firing, leading to injury or damage to materials.

- Blade guard and dead-man switch – blade is covered and stops when user lets go.

- She should have tagged the tool as faulty, reported it immediately, and removed it from service.

- Clean after each use; sharpen or oil hinges; store in dry place.

- Prevents moisture damage, rust, and tool misfires.

- Use labelled drawers, padded holders, and wall-mounted racks. Keep blades and bits covered.

- Match staple/nail type to tool specification; check length, thickness, and fit. Always load with power disconnected.

- Mandatory tool check-in/check-out, daily maintenance schedule, compulsory PPE and tool safety training.

📊 Assessment Rubric

| Criteria | Outstanding (4) | Competent (3) | Partially Achieved (1–2) | Not Achieved (0) |
| --- | --- | --- | --- | --- |
| Tool Knowledge | Tools clearly identified and used correctly | Mostly correct tools and functions | Vague or limited understanding | Incorrect or no reference |
| Maintenance and Safety Practices | Describes thorough SOPs and rationale | Some good practices described | Incomplete or unclear explanation | No maintenance knowledge shown |
| Risk Identification | Clear link to potential consequences and risks | General understanding of risks | Superficial or incomplete risk awareness | Risks not identified |
| Application of SOPs | Accurate and appropriate procedures explained | Understands general process | Inconsistent or partially relevant steps | No SOP or procedural understanding |

### Facilitator Assessment Briefing

Knowledge Module: KM-03-KT04 – Upholstery Tools and Equipment
Qualification: Furniture Upholsterer (SAQA ID 103199)
NQF Level: 2
Credits: 8
Assessment Type: Case Study with Scenario-Based Questions
Assessment Duration: 60 minutes
Total Marks: 40
Assessment Weighting: 10%

🎯 Purpose of the Assessment

This integrated assessment evaluates the learner’s ability to:

- Identify and describe the function of upholstery hand tools, pneumatic tools, and electric power tools (IAC0401, IAC0404, IAC0407).

- Identify tool components and safety mechanisms, and describe maintenance procedures (IAC0402, IAC0405, IAC0408).

- Assess the consequences of incorrect handling, cleaning, and maintenance of tools (IAC0403, IAC0406, IAC0409).

- Apply knowledge to select and load consumables (e.g. staples, nails) into relevant tools (IAC0410).

🛠 Assessment Instrument

A single case study serves as the foundation for ten scenario-based questions that assess both technical knowledge and practical reasoning. The instrument addresses all ten Internal Assessment Criteria (IAC0401–IAC0410) and requires learners to reflect on a real-world workplace context.

✅ Facilitator Preparation Checklist

- Print copies of the case study and questions for each learner.

- Prepare model answers and the marking rubric.

- Brief learners on duration, expected conduct, and assessment structure.

- Ensure the assessment room is quiet, clean, and well-ventilated.

- Ensure learners have access to pens, paper, and answer sheets (if separate).

- Arrange any visual aids or sample tools if required for learner reference prior to assessment.

📢 Instructions to Be Read to Learners

- You are about to complete an assessment based on a case study.

- You will answer ten scenario-based questions.

- Your responses must show that you understand how to work with upholstery tools and equipment safely and responsibly.

- Write clearly and answer in full sentences.

- You will have 60 minutes to complete the paper.

- If you do not understand the wording of a question, you may raise your hand for clarification — no answers will be given.

- This is an individual task — no communication is allowed.

🧾 Marking and Moderation Guidelines

- Use the model answers and rubric provided.

- Allocate full marks only when the learner has given a complete and accurate answer.

- Accept varied phrasing if the content reflects correct understanding.

- Clearly record marks and ensure a second marker or moderator reviews a sample of the assessments.

- Store all assessment evidence securely for internal and external moderation.

🔒 Assessment Integrity and Record-Keeping

- The facilitator must actively invigilate the session.

- Prevent the use of phones, internet, or collaboration during the assessment.

- Collect all assessment documents immediately at the end of the session.

- Ensure that all scripts are signed, dated, and filed with the appropriate assessment register.

- Record the results on the learner achievement spreadsheet.

## KM-03-KT05: Upholstery Measurements, Measuring Tools, Techniques and Calculations

Weighting: 10% | NQF Level: 2

1. Purpose of the Knowledge Topic

The purpose of this Knowledge Topic is to equip learners with the foundational knowledge and practical skills required to perform accurate measurement, conversion, and basic calculations during the upholstery process. Accurate measurement is a critical function in upholstery — influencing material planning, quality control, and the overall fit and finish of upholstered products.

Learners will develop the ability to use a range of measuring tools, apply mathematical operations to real workshop tasks, and solve common measurement problems encountered during frame preparation and covering. Emphasis is placed on accuracy, consistency of units, and verification techniques, ensuring high standards in production.

2. Key Knowledge Areas (Topic Elements)

The following topic elements are covered in this Knowledge Topic:

- KT0501: Principles of accurate measurements during the frame preparation process

- KT0502: Units of measurement and measuring tools and equipment required for the frame preparation process

- KT0503: Measuring techniques

- KT0504: Calculations and angles

- KT0505: Calculating allowances

- KT0506: Problem solving

3. Internal Assessment Criteria and Weighting

This Knowledge Topic carries a total weighting of 10% and will be assessed according to the following Internal Assessment Criteria:

- IAC0501: Various units, conversions and terminology for measurements, including quantity, deflection, and compressed air volume and pressure are explained and applied

- IAC0502: Calculations required during the upholstery process are explained and applied

- IAC0503: Measuring tools such as steel tape measure, cloth tape measure, squares, steel rules, Vernier calliper, templates and patterns, and pressure gauges and their parts are identified and their functions analysed

- IAC0504: Calculations involving dimensions, spacing, quantities, volume, and air pressure using consistent units and conversions and appropriate mathematical operations are completed, recorded and checked

- IAC0505: Consequences of inaccurate measurements on the upholstery process and the output of the department are assessed

4. Application in Upholstered Furniture Manufacturing

Measurement plays a critical role in frame preparation, material cutting, foam fitting, fabric alignment, and final assembly. Mistakes in units, incorrect use of tools, or failure to check calculations can result in wasted materials, delays, and compromised product quality.

This topic prepares learners to work competently and confidently with measurement tools, carry out necessary calculations, and apply problem-solving techniques to measurement-related issues in the upholstery environment.

### KT0501: Principles of Accurate Measurements During the Frame Preparation Process

Purpose of the Topic

This topic introduces learners to the fundamental principles of accurate measurement in the context of frame preparation for upholstered furniture. Learners will understand why precision is essential, what factors influence measurement accuracy, and how errors at this stage can affect the final product. The focus is on developing measurement discipline, consistency, and attention to detail.

Key Content Areas

- Why Accurate Measurement Matters

  - Ensures fit and alignment of foam, padding, and fabric.

  - Reduces material wastage.

  - Supports standardisation and repeatability across product batches.

  - Affects the structural integrity and aesthetic quality of the final piece.

- Core Principles of Accurate Measurement

  - Start from a fixed reference point (zero point).

  - Keep measuring tools flat, aligned, and taut.

  - Always double-check measurements before marking or cutting.

  - Use appropriate tools for the specific material and purpose.

  - Ensure tools are clean and undamaged before use.

- Environmental and Human Factors Affecting Accuracy

  - Poor lighting may cause misreading of measurements.

  - Working in a rush increases likelihood of error.

  - Using blunt pencils, smudged rulers, or worn-out tapes can distort results.

  - Fatigue, inattention, and incorrect assumptions also lead to miscalculation.

- Workshop Best Practices

  - Use clearly marked workstations with accessible measuring tools.

  - Integrate checking steps into the workflow (e.g. cut-check-confirm).

  - Maintain a habit of recording key dimensions and checking against specifications or cutting lists.

Example Table: Measurement Checks in Frame Preparation

| Task | Required Measurement | Common Error | Correction Principle |
| --- | --- | --- | --- |
| Cutting a wooden leg component | Length in mm or cm | Measuring from incorrect end | Align zero mark and hold firmly |
| Marking foam for seat panel | Width and thickness | Ruler lifted at midpoint | Lay ruler flat and press gently |
| Positioning mounting brackets | Centre-to-centre spacing | Forgetting to allow for fastener size | Add bracket offset before drilling |

Case Study: Measurement Mistake at Dlamini & Sons Upholstery

Background:
At Dlamini & Sons Upholstery in Polokwane, the frame team cut 20 ottoman bases 2 cm too narrow because the apprentice, Musa, started measuring from the 2 cm mark on the steel rule instead of the true zero point.

Consequences:

- The foam pieces had to be re-cut.

- Fabric covers no longer aligned with the base structure.

- The delivery deadline was missed, costing the company goodwill and additional material expenses.

Resolution:
The supervisor introduced a “measure-mark-confirm” checklist and held a toolbox talk on reading and using steel rules correctly.

Learning Point:
Consistent measuring discipline is critical to ensure quality and prevent costly rework.

Critical Thinking Questions

- Why is it important to double-check your measurement before marking or cutting?

- What are the consequences of cutting all parts to the wrong length, even if the mistake is consistent?

- How would you correct a habit of using the wrong zero point on a measuring tape?

- In your experience, what contributes most to measurement errors — tools, people, or the environment?

- How could a team system be developed to catch errors before parts are passed on to the next stage?

Practical Classroom Activity

Task: Measurement Accuracy Challenge

Materials:

- Steel rule, cloth tape, square, pencil, off-cut timber, or foam board

- Marking templates with specific dimensions (e.g. “cut a square 250 mm × 250 mm”)

Instructions:

- Learners measure and mark a specific shape or length on a given material.

- Facilitator checks the accuracy (within 1 mm tolerance).

- Learners compare results and identify where their technique may have led to small inaccuracies.

Extension:
Facilitator introduces deliberate “errors” (e.g. worn-out tape, misaligned edge) and asks learners to identify what went wrong and how to correct it.

### KT0502: Units of Measurement and Measuring Tools and Equipment Required for the Frame Preparation Process

Purpose of the Topic

This topic introduces learners to the standard units of measurement used in upholstery, and the range of tools and equipment used to take accurate measurements during the frame preparation process. Emphasis is placed on choosing the right measuring tool for the task and understanding how to apply units consistently and correctly to avoid costly errors in material and fit.

By mastering this topic, learners will be able to interpret, apply, and convert between relevant units of measurement and use each tool according to its intended purpose in the upholstery context.

Key Content Areas

- Common Units of Measurement

  - Length and Width: millimetres (mm), centimetres (cm), metres (m)

  - Volume: cubic centimetres (cm³), litres (L)

  - Angle: degrees (°)

  - Air Pressure: kilopascals (kPa), pounds per square inch (PSI)

  - Weight: kilograms (kg), grams (g)

- Conversions and Terminology

  - 1 m = 100 cm = 1 000 mm

  - 1 inch = 25.4 mm (important for imported tools and specifications)

  - PSI to kPa conversion: 1 PSI ≈ 6.895 kPa

  - Terminology: deflection, tolerance, radius, diameter, gauge, compressed air volume

- Measuring Tools and Their Applications

  - Steel Tape Measure: For general long measurements on frames or foam sheets.

  - Cloth Tape Measure: For curved or flexible surfaces (e.g. padding).

  - Squares (Try Square, Framing Square): For right-angle checks.

  - Steel Rule: For precision edge measuring and straight lines.

  - Vernier Calliper: For internal and external diameter checks or thickness.

  - Templates and Patterns: Reused shapes or cut-outs for repeat accuracy.

  - Pressure Gauge: To measure compressed air pressure (for pneumatic tools or testing foam compression).

- Tool Selection Considerations

  - Material type (rigid, soft, curved)

  - Accuracy required

  - Shape of the surface

  - Measurement environment (e.g. confined space, low light)

Example Table: Tool and Unit Application

| Tool | Used To Measure | Unit of Measurement | Key Consideration |
| --- | --- | --- | --- |
| Steel Tape Measure | Frame dimensions | mm, cm | Hold flat and align from true zero |
| Cloth Tape | Curved cushion length | mm, cm | Avoid stretching tape during use |
| Vernier Calliper | Foam or rod diameter | mm | Read digital or dial face correctly |
| Pressure Gauge | Air compressor tank | PSI or kPa | Calibrate regularly for accuracy |

Case Study: Misreading at Tessa Furniture Works

Background:
At Tessa Furniture Works in Welkom, the production team received cutting instructions from a client in inches, but the apprentice, Shaun, misread the order and measured in centimetres. As a result, 15 chair backs were too short by nearly 5 cm each.

Consequences:

- The entire batch had to be reworked.

- Materials were wasted, and time was lost.

- The client raised concerns about professionalism.

Resolution:

- Metric/Imperial conversion charts were posted on workshop walls.

- Measuring tools were marked clearly with unit labels.

- Learners were trained to ask when unclear about measurement units.

Learning Point:
Always clarify the unit system before starting and use tools that clearly reflect that system.

Critical Thinking Questions

- Why is it important to standardise the use of measurement units in a workshop?

- What problems could occur if a steel rule and a cloth tape are used interchangeably without checking accuracy?

- How would you explain the purpose of a pressure gauge to someone new to upholstery?

- Why must learners be able to convert between millimetres, centimetres, and metres?

- What strategies can be put in place to avoid unit conversion errors?

Practical Classroom Activity

Task: Tool and Unit Matching Exercise

Materials:

- Variety of measuring tools (or images): steel tape, cloth tape, calliper, pressure gauge, template

- Sample objects: wooden block, foam roll, curved padding

- Unit cards: mm, cm, PSI, kPa, ° (angle), m²

Instructions:

- Learners work in pairs to match each tool with its appropriate use and unit.

- Each pair must explain how they would take a specific measurement using that tool.

- Conduct a conversion exercise: e.g. convert 2.5 m to mm, or 50 PSI to kPa.

Extension:
Ask learners to design a visual unit conversion chart for the upholstery workshop and present it to the class.

### KT0503: Measuring Techniques

Purpose of the Topic

This topic equips learners with knowledge of basic and reliable measuring techniques used in upholstery, particularly during frame preparation and component assembly. Learners will practise how to apply these techniques to ensure accuracy, consistency, and safety when measuring different surfaces and materials in the workshop.

By the end of this topic, learners will demonstrate the ability to apply practical measuring methods that support precise cutting, alignment, spacing, and assembly tasks in upholstery production.

Key Content Areas

- General Measuring Techniques

  - Aligning from the Zero Mark: Always begin from the true zero point on a tape or rule.

  - Holding the Tool Steady: Keep the tool flat and taut, with no bends or slack.

  - Reading at Eye Level: Avoid parallax errors by reading measurements directly from above.

  - Marking Clearly: Use sharp pencils, fine markers, or chalk for visible, accurate lines.

  - Double-Checking: Measure twice, cut once — verify critical dimensions before cutting or assembly.

- Surface-Specific Techniques

  - Flat Surfaces: Use rigid tools like steel rules or squares.

  - Curved or Flexible Surfaces: Use cloth tapes or flexible rulers.

  - Corners and Angles: Use squares and protractors to check right angles or specific angles.

- Working with Patterns and Templates

  - Trace outlines carefully using controlled, even pressure.

  - Ensure templates are labelled and reused consistently for repeat items.

  - Secure templates to prevent movement while marking.

- Techniques for Measuring Irregular Shapes

  - Take reference measurements (e.g. overall length, width, depth).

  - Use cross-measuring to confirm squareness (e.g. measuring diagonally).

  - Use mock-ups or trial fittings if unsure.

Example Table: Measurement Technique by Task

| Task | Best Measuring Tool | Technique Tip |
| --- | --- | --- |
| Measuring chair seat width | Steel tape or rule | Start from fixed edge and pull tape taut |
| Measuring arm curve | Cloth tape | Keep tape snug but not stretched |
| Marking leg bracket holes | Square and steel rule | Mark from both sides to confirm alignment |
| Measuring foam thickness | Vernier calliper or ruler | Ensure tool is perpendicular to foam surface |

Case Study: The “Almost Right” Measurements at Maluleka Upholstery

Background:
At Maluleka Upholstery in Kimberley, a team was tasked with covering a new range of armchairs. Susan used a steel tape to measure the arm width, but read the value from an angle, resulting in each foam arm piece being 4 mm too thick.

Result:
When assembled, the arms appeared bulky, and the fabric did not fit correctly, requiring foam trimming and replacement of fabric panels.

Intervention:
The facilitator reviewed measuring techniques and demonstrated how reading at an angle causes parallax errors. A mirror was installed on the measurement station to support correct eye-level reading.

Learning Point:
Even small measurement errors, when repeated, can lead to increased labour and material waste.

Critical Thinking Questions

- Why is it important to use different measuring tools for flat and curved surfaces?

- What is parallax error, and how can it be prevented?

- Why should templates be stored and labelled properly after use?

- How can you ensure your marking lines are consistent and do not smudge?

- What would you do if your first and second measurements differ slightly?

Practical Classroom Activity

Task: Measuring Technique Workshop Rotation

Materials:

- Steel tape, cloth tape, steel rule, square, sample foam and wood pieces

- Pencils, chalk, markers

- Templates or pattern cards

Instructions:

- Set up measuring stations for different surfaces and tools.

- Learners rotate in groups, practising:

  - Measuring and marking with accuracy

  - Recording measurements

  - Comparing tool results on the same object

- Facilitator checks for technique, tool handling, and clarity of markings.

Extension:
Ask learners to bring in a simple household object (e.g. a cushion, chair leg) and measure it using at least two different techniques. They present the method and justify their tool choice.

### KT0504: Calculations and Angles

Purpose of the Topic

This topic introduces learners to the basic calculations and angle measurements required in the upholstery workshop, particularly during frame preparation and fabric or foam cutting tasks. Learners will develop the ability to perform mathematical operations using consistent units, as well as measure and work with angles to ensure precise joins, folds, and component placement.

Competence in this area supports improved planning, waste reduction, and higher quality finished products.

Key Content Areas

- Basic Calculations in Upholstery

  - Length × Width for surface area (e.g. fabric and padding coverage).

  - Addition and subtraction for allowance calculations.

  - Multiplication and division for repeating patterns, spacing fasteners, or dividing symmetrical parts.

  - Volume (Length × Width × Height) for foam and filling.

  - Air pressure settings for pneumatic tools.

- Working with Angles

  - Most upholstery frames use 90° angles (right angles), but certain backs, arms, or legs may require:

    - 45° cuts for mitre joints

    - 30° or 60° angles for tapered shapes

  - Tools: protractor, adjustable square, bevel gauge, or angle finder

- Common Upholstery Calculations

  - Calculating the number of staples needed per 100 mm.

  - Measuring equal spacing for button placement or tufting.

  - Estimating fabric length needed based on pattern repeats.

  - Calculating air pressure change when splitting hose systems.

- Converting Measurements

  - Converting mm to cm, or metres to mm (e.g. 1.2 m = 1 200 mm)

  - Converting angles: e.g. combining two 45° pieces to form a 90° corner.

Example Table: Upholstery Calculations in Practice

| Calculation Task | Formula/Technique | Example |
| --- | --- | --- |
| Fabric required for seat top | Length × Width (in mm or cm) | 600 mm × 500 mm = 300 000 mm² or 3 000 cm² |
| Foam volume | Length × Width × Thickness | 50 × 40 × 10 cm = 20 000 cm³ |
| Button spacing (3 buttons evenly) | Total width ÷ (Number + 1) | 900 mm ÷ (3 + 1) = 225 mm spacing |
| Staple count for edge | Length ÷ spacing interval | 1 200 mm ÷ 50 mm = 24 staples |

Case Study: Miscalculated Angles at Tembe Upholstery

Background:
At Tembe Upholstery in Rustenburg, a team was tasked with assembling custom frame corners for an angled armrest design. Lerato measured and cut all internal frame pieces assuming the corner was a perfect 90°, when it was actually 120°.

Result:
The pieces did not align, causing gaps and instability. Several timber components had to be discarded and recut, resulting in time loss and increased material usage.

Resolution:

- A bevel gauge and angle finder were added to the measurement station.

- The workshop implemented a check-before-cut protocol for all non-standard designs.

Learning Point:
Incorrect angle assumptions can have costly structural and visual consequences in furniture construction.

Critical Thinking Questions

- Why is it important to understand angles when preparing frames or cutting mitres?

- How would you ensure accurate spacing when placing staples along an armrest?

- What are the risks of using incorrect units in upholstery calculations?

- Why is it necessary to round up material measurements during the planning phase?

- How could calculation errors affect both quality and budget in an upholstery workshop?

Practical Classroom Activity

Task: Angle and Upholstery Calculation Circuit

Materials:

- Worksheets with calculation tasks

- Angle tools: protractor, bevel square, sample frame corners

- Rulers, calculators, scrap foam or fabric for mock planning

Instructions:

- Set up 3–4 stations:

  - Station 1: Area and volume calculations

  - Station 2: Fastener spacing problems

  - Station 3: Angle identification on mock frames

  - Station 4: Air pressure settings for dual-tool use

- Learners rotate through each station, completing a calculation or measuring task.

- Facilitator reviews and provides group feedback on most common challenges.

Extension:
Introduce a real client scenario where learners must prepare a cutting and measurement plan for an ottoman, calculating all dimensions and allowances.

### KT0505: Calculating Allowances

Purpose of the Topic

This topic introduces learners to the practice of calculating allowances during upholstery preparation. Allowances refer to extra measurements added to actual dimensions to ensure proper fitting, joining, stretching, folding, or covering of materials. Calculating allowances correctly is vital for preventing shortages, misfits, and excessive waste during frame assembly and fabric cutting.

By the end of this topic, learners will be able to determine the correct allowances needed for a variety of upholstery tasks and apply these consistently to cutting plans and measuring templates.

Key Content Areas

- What Are Allowances?

  - Allowances are additional lengths or margins included in a measurement to accommodate:

    - Folding, tucking, or stapling fabric and padding.

    - Joining seams or matching patterns.

    - Foam compression or contouring.

    - Shrinkage in steaming or stretching processes.

- Types of Upholstery Allowances

  - Fabric Allowance: Extra material added for wrapping and securing (typically 20 mm to 50 mm depending on design).

  - Foam Allowance: Added when the foam needs to compress into a tight fit.

  - Joining or Seam Allowance: Required for sewing or joining materials together.

  - Edge Clearance: Space needed to allow staples or trims without obstruction.

- How to Calculate Allowances

  - Total Dimension = Actual Measurement + Allowance

  - Example:
Seat width = 500 mm
Add 30 mm each side for wrapping → Total fabric width = 560 mm

  - Record allowances clearly on cutting lists, templates, and work instructions.

- Factors Affecting Allowance Calculations

  - Type and thickness of material.

  - Type of join or attachment method.

  - Design detail (e.g. deep buttoning, contour lines, piping).

  - Degree of stretch in fabric.

Example Table: Upholstery Allowance Guidelines

| Material or Task | Typical Allowance | Purpose |
| --- | --- | --- |
| Upholstery fabric for frame | 30–50 mm per edge | Folding over edges and stapling |
| Foam in fitted cavity | +5 mm to actual size | Compression for snug fit |
| Fabric with piping detail | +10–20 mm | For seam and turning space |
| Patterned fabric | Varies (full repeat) | Pattern alignment and symmetry |

Case Study: Insufficient Allowance at SewRight Upholstery

Background:
At SewRight Upholstery in Mthatha, Elmarie cut all the backrest panels to the exact width of the wooden frame (450 mm), forgetting to include wrapping and stapling allowances. When she began fitting the fabric, she realised the panels were too small, and visible gaps formed at the sides.

Outcome:

- All pieces had to be re-cut.

- Extra fabric was ordered, and project deadlines were missed.

- The team updated their cutting list templates to include a column for allowance entries.

Learning Point:
Neglecting allowances leads to material waste, rework, and delays — even if the base measurements are correct.

Critical Thinking Questions

- Why should allowances vary depending on the material used (e.g. vinyl vs cotton)?

- What could happen if a foam panel is cut with no allowance for compression?

- How would you calculate an allowance for a seat cushion that requires fabric to wrap around the underside?

- Why is it important to record allowances on cutting lists or job cards?

- What is the risk of including too much allowance in a fabric panel?

Practical Classroom Activity

Task: Allowance Calculation Challenge

Materials:

- Sample job card with frame measurements

- Fabric swatches with varying stretch

- Cutting list templates

Instructions:

- Learners receive 3 frame measurements (e.g. seat, backrest, arm).

- They must calculate the total fabric size required for each, adding appropriate allowances.

- Discuss how the type of material or design feature influences their decisions.

Extension:
Have learners present a brief explanation of their calculations and how they would communicate these to a team member cutting the fabric.

### KT0506: Problem Solving

Purpose of the Topic

This topic introduces learners to basic problem-solving techniques used in the context of measurement errors, tool selection, and calculation issues during the upholstery process. Learners will explore how to identify the root causes of common problems, make appropriate decisions to correct them, and apply logical reasoning to avoid repeated mistakes.

Developing these problem-solving skills equips learners to work independently, troubleshoot effectively, and support continuous improvement in the workplace.

Key Content Areas

- What Is Problem Solving in Upholstery?

  - Identifying a problem or inconsistency during the frame preparation or covering process.

  - Analysing the cause (e.g. incorrect measurement, wrong tool, inaccurate calculation).

  - Selecting and applying an appropriate solution.

  - Recording the problem or informing the team if needed.

- Common Measurement-Related Problems

  - Inaccurate measurements causing mismatched parts.

  - Incorrect units (e.g. cm vs mm).

  - Material shortages due to underestimating allowances.

  - Tool misreading or misalignment.

  - Angle miscalculations resulting in poor joins.

- Problem-Solving Process

  - Step 1: Identify what is wrong (e.g. part does not fit).

  - Step 2: Gather information (measure again, inspect tool or list).

  - Step 3: Determine the cause (e.g. tape stretched, incorrect allowance).

  - Step 4: Choose the best solution (e.g. recut, re-measure, alert supervisor).

  - Step 5: Take corrective action and reflect for improvement.

- When to Involve Others

  - If the issue affects the whole batch or requires reordering materials.

  - If the cause is unclear or may involve equipment malfunction.

  - If safety or delivery timelines are at risk.

Example Table: Problem-Solving Scenarios

| Problem Identified | Possible Cause | Solution |
| --- | --- | --- |
| Backrest panel too wide for frame | Allowance added on both sides | Recheck measurement and cut new panel |
| Foam not compressing into cavity | Foam cut too large, low density | Adjust foam dimensions or select higher density |
| Fabric cut short by 50 mm | Tape read from 5 cm mark | Mark correct zero point, use new tape |
| Bracket holes do not align | Wrong spacing calculation | Measure bracket centres and re-drill |

Case Study: Reactive vs Preventative Thinking at J&N Upholstery

Background:
At J&N Upholstery in Durban, Kabelo noticed that the side panels of the ottoman he was assembling were consistently 10 mm too short. Instead of reporting the issue, he added filler blocks to extend the base. The problem repeated in five more units before the foreman intervened.

Investigation:
The measurement template had been updated, but the old one was still in circulation. Kabelo did not cross-check the measurements against the job card.

Outcome:
The team introduced a job card verification step and trained all staff on preventative problem-solving.

Learning Point:
Addressing a symptom (short panel) without fixing the cause (outdated template) leads to more waste and rework.

Critical Thinking Questions

- What is the difference between a quick fix and a true solution in upholstery problem solving?

- How can good measurement habits help reduce the need for problem solving?

- Why is it important to report recurring problems, even if you can work around them?

- What would you do if your measurements are consistently different from your co-worker’s?

- How can problem-solving skills support teamwork and quality control in the upholstery process?

Practical Classroom Activity

Task: Measurement Problem Solving Simulation

Materials:

- Sample job card with known error or missing data

- Measuring tools, foam pieces, and cut fabric swatches

- Problem scenario cards (e.g. “foam too large”, “fabric cut short”, “tape unreadable”)

Instructions:

- Learners work in pairs to identify and solve the problem on their scenario card.

- Each team presents the problem, their analysis, and their solution.

- Discuss how similar problems could be prevented in future.

Extension:
Facilitator introduces a real-life scenario from the workshop and asks learners to propose multiple solution options and evaluate the pros and cons of each.

## Integrated Assessment – KM-03-KT05

Qualification: Furniture Upholsterer (SAQA ID 103199)
Knowledge Module: KM-03-KT05 – Upholstery Measurements, Measuring Tools, Techniques and Calculations
NQF Level: 2 | Credits: 8 | Weighting: 10%
Duration: 60 minutes | Total Marks: 40
Assessment Type: Variety of Instruments (Short Answer, Practical Scenario, Matching, and Application)

🔹 Section A: Matching – Measuring Tools and Units (5 marks)

Match each tool in Column A with the most appropriate description in Column B.

| Column A | Column B |
| --- | --- |
| A. Vernier calliper | 1. Used to measure compressed air in PSI or kPa |
| B. Steel tape measure | 2. Provides curved measurements for foam edges |
| C. Cloth tape | 3. Measures diameter or internal dimensions of rods or foam tubes |
| D. Pressure gauge | 4. Best for long, straight surface measurements |
| E. Template | 5. Used to trace and repeat consistent shapes |

🔹 Section B: Short Answer Questions (3 marks each × 5 = 15 marks)

- Why is it important to use the correct measuring tool for curved versus flat surfaces?

- List any three units of measurement used in upholstery and explain what each one is used for.

- Describe the difference between an actual measurement and a measurement with allowance.

- How can you avoid parallax errors when using a steel rule?

- What are the potential consequences of incorrect unit conversions in an upholstery job?

🔹 Section C: Scenario-Based Application (2 marks each × 5 = 10 marks)

Scenario:
You are preparing to cut fabric for a curved armrest cover. The actual armrest width is 450 mm. You need to allow 40 mm for wrapping and stapling on each side.

- What is the total fabric width you need to cut?

- Which tool would be best for measuring the curve of the armrest?

- How would you record your final measurement on the cutting list?

- What could go wrong if you did not include an allowance?

- How would you double-check your measurement before marking the fabric?

🔹 Section D: Problem Solving and Calculation (2 marks each × 5 = 10 marks)

- A seat panel is 550 mm long and 400 mm wide. What is the surface area in mm²?

- You need to cut 8 foam blocks, each 300 mm wide. How much foam (in mm) do you need in total?

- You are spacing 4 buttons evenly across a 1 000 mm panel. What is the spacing between each button?

- A pressure gauge shows 85 PSI. Convert this to kilopascals (kPa), using the formula:
    1 PSI ≈ 6.895 kPa

- A measurement is written as 1.2 metres. Convert this to millimetres.

✅ Model Answers and Marking Memo

Section A:
A–3, B–4, C–2, D–1, E–5

Section B Sample Answers:
6. Curved surfaces require flexible tapes; flat surfaces need rigid tools.
7. mm (foam length), PSI (air pressure), cm (fabric width).
8. Actual is exact dimension; allowance includes extra margin for folding, seams, etc.
9. Hold tool flat and read measurement at eye level.
10. Material shortages, poor fit, increased waste.

Section C Sample Answers:
11. 450 mm + 40 mm + 40 mm = 530 mm
12. Cloth tape measure
13. Write “530 mm (incl. 2 × 40 mm allowance)”
14. Cover will be too small; visible gaps
15. Measure again and verify with another team member

Section D Sample Answers:
16. 550 × 400 = 220 000 mm²
17. 300 × 8 = 2 400 mm
18. 1 000 ÷ (4 + 1) = 200 mm spacing
19. 85 × 6.895 = 586.075 kPa
20. 1.2 m = 1 200 mm

📊 Assessment Rubric

| Criteria | Outstanding (4) | Competent (3) | Partially Achieved (1–2) | Not Achieved (0) |
| --- | --- | --- | --- | --- |
| Tool Identification and Use | Accurately matches tool to function and surface type | Mostly accurate with minor errors | Some tools misused or misunderstood | No understanding of tool usage |
| Measurement and Unit Conversions | Converts and records with precision and consistency | Minor conversion or labelling issues | Incomplete or inaccurate conversions | Cannot perform basic conversions |
| Calculations and Accuracy | All calculations are correct and checked | Small errors with good working shown | Basic setup attempted but errors present | No attempt or incorrect working |
| Problem Solving and Justification | Reasoned, realistic and safe responses to scenarios | Acceptable logic and problem recognition | Responses vague or incomplete | No understanding of the problem |

### Facilitator Assessment Briefing

Knowledge Module: KM-03-KT05 – Upholstery Measurements, Measuring Tools, Techniques and Calculations
Qualification: Furniture Upholsterer (SAQA ID 103199)
NQF Level: 2 | Credits: 8 | Weighting: 10%
Assessment Duration: 60 minutes
Assessment Type: Variety of Instruments

🎯 Purpose of the Assessment

This assessment is designed to evaluate whether learners can:

- Identify and correctly use a variety of measuring tools in different upholstery contexts.

- Apply accurate units of measurement, interpret and perform basic calculations, and identify angles.

- Understand and apply allowances in fabric and foam cutting.

- Solve common measurement-related problems in the frame preparation and upholstery process.

Assessment tasks are directly aligned with IAC0501–IAC0505.

🛠 Instruments Used

| Section | Type | Marks |
| --- | --- | --- |
| Section A | Matching | 5 |
| Section B | Short Answer Questions | 15 |
| Section C | Scenario Application | 10 |
| Section D | Calculation & Problem Solving | 10 |
| Total |  | 40 |

✅ Facilitator Preparation Checklist

- Print sufficient copies of the assessment.

- Ensure measuring tools (or visual aids) are available if referenced during review.

- Prepare the marking memo and rubric.

- Brief learners on the structure, rules, and time limits.

- Set up a well-lit, distraction-free assessment space.

📢 Instructions to be Read to Learners

- This assessment is divided into four sections using different types of questions.

- Read each instruction carefully.

- Use full sentences where required, and ensure all units and labels are correct.

- You have 60 minutes to complete the assessment.

- If a question is unclear, raise your hand for clarification – no answers will be given.

- This is an individual task. Talking or sharing is not permitted.

📊 Marking and Moderation Guidelines

- Use the model answers and rubric to guide consistent marking.

- Partial marks may be awarded for correct processes with minor errors.

- Check mathematical operations, logic, and clarity of explanations.

- Scripts must be signed, scored, and safely filed for moderation.

## KM-03-KT06: Upholstered Furniture Manufacturing Processes

Weighting: 10% | NQF Level: 2

1. Purpose of the Knowledge Topic

The purpose of this Knowledge Topic is to provide learners with a foundational understanding of the end-to-end upholstered furniture manufacturing process, from the preparation of the raw frame to the final finishing stage. The topic introduces learners to the flow of operations, documentation used in production, and the way design specifications influence methods and outcomes on the workshop floor.

By developing a structured understanding of the manufacturing process, learners will gain the knowledge required to contribute efficiently to their role, interpret key production documents such as routing sheets and cutting lists, and understand the sequence and logic of operations that make up the furniture production cycle.

2. Key Knowledge Areas (Topic Elements)

This Knowledge Topic consists of the following topic elements:

- KT0601: Upholstered furniture manufacturing process from raw frame to finished product

- KT0602: Process flow and productivity

- KT0603: Operations in the process flow: knocker-on operations, foaming-up operations, padding sprung foundations and frames, and covering operations

- KT0604: Routing sheets

- KT0605: Cutting lists

- KT0606: Product specifications

- KT0607: Finishing aids

3. Internal Assessment Criteria and Weighting

This Knowledge Topic carries a total weighting of 10% and will be assessed according to the following Internal Assessment Criteria:

- IAC0601: Workshop processes of producing upholstered items from raw frame to finished product and the implications of design and construction faults on the final product are analysed

- IAC0602: The process of furniture manufacture is briefly outlined

- IAC0603: The operations in furniture manufacture such as knocking-on operations, foaming-up operations, padding and sprung foundations and frames, and covering operations are reviewed

- IAC0604: The role of the routing sheet is described

- IAC0605: Job card information such as component sizes and details, shoulder-to-shoulder size, and chemicals to use is explained

- IAC0606: Product specifications are understood and their impact on the manufacturing process is discussed in terms of the process flow and methods that will be used

4. Application in Upholstered Furniture Manufacturing

A clear understanding of the manufacturing process allows learners to work efficiently, accurately, and with foresight. They will recognise the importance of process flow, documentation, and correct execution of each operation, as well as how errors at one stage can affect the quality of the final product. This topic also enhances learners' ability to work collaboratively across departments by understanding how their tasks fit into the bigger picture of production.

### KT0601: Upholstered Furniture Manufacturing Process from Raw Frame to Finished Product

Purpose of the Topic

This topic introduces learners to the complete sequence of operations involved in manufacturing an upholstered furniture item, beginning with the raw frame and concluding with the final finished product. The learner will gain an overview of how each stage builds on the previous one, and how precision, timing, and quality control at every step determine the success of the final output.

By understanding the entire process, learners will be able to work more effectively in their assigned role, understand upstream and downstream operations, and anticipate how their work contributes to the overall production flow.

Key Content Areas

- Overview of the Manufacturing Stages

  - Raw frame preparation: Sanding, reinforcement, mounting support components.

  - Springing-up and webbing: Fitting of springs and base supports.

  - Padding: Laying and attaching foam or padding material.

  - Foaming-up: Shaping and securing foam components to frame surfaces.

  - Covering: Cutting, fitting, and stapling upholstery fabric or leather.

  - Finishing: Attaching legs, applying trims or piping, cleaning, and packaging.

- Purpose and Function of Each Stage

  - Each stage ensures a specific structural, functional, or aesthetic requirement is met.

  - For example, the padding stage ensures comfort, while covering enhances the look and texture of the product.

- Quality Requirements at Each Stage

  - Use of correct tools and materials.

  - Following measurements and product specifications.

  - Inspection and correction before passing to the next stage.

- Interdependence of Stages

  - A mistake in the frame (e.g. misaligned corners) can cause problems during padding or covering.

  - Delays or poor workmanship at any stage affect the overall productivity and quality.

Example Table: Upholstery Manufacturing Stages

| Stage | Key Actions | Critical Consideration |
| --- | --- | --- |
| Raw Frame Prep | Sand, drill, brace | Structural soundness, squareness |
| Springing-Up | Attach springs, tension webbing | Centre alignment, consistent pressure |
| Padding | Cut and glue foam, layer padding | Matching contours and sizes |
| Foaming-Up | Shape, position, and secure foam | No gaps, even adhesion |
| Covering | Cut, align, staple fabric | Pattern alignment, wrinkle-free finish |
| Finishing | Apply trims, inspect, clean, and package | No visible defects, clean presentation |

Case Study: The Domino Effect at Eastline Upholstery

Background:

At Eastline Upholstery in Polokwane, a batch of armchairs was delayed due to errors in the raw frame stage. The internal bracing was not secured properly, which led to loose arms and uneven foam surfaces during the foaming-up stage. As a result, fabric stretched unevenly during covering and ripples appeared on the outer edges.

Outcome:
Three armchairs had to be reworked, and the batch delivery was delayed by two days. The team traced the fault to one of the frame jigs being out of square.

Learning Point:
A problem early in the process can affect multiple stages and compromise the final product’s integrity and appearance.

Critical Thinking Questions

- Why is it important for a learner to understand the full furniture manufacturing process even if they are only working on one stage?

- What could happen if the foam is not aligned correctly during the padding stage?

- How can workers in later stages help identify and report problems from earlier steps?

- Why is the finishing stage as important as the structural stages?

- How does teamwork and communication improve the flow from raw frame to final product?

Practical Classroom Activity

Task: Manufacturing Sequence Mapping

Materials:

- Printed step-by-step visuals or cards representing each stage

- Blank chart paper and markers

Instructions:

- Learners work in small groups to arrange the manufacturing process steps in the correct sequence.

- For each step, they list the main task, tools/materials used, and key quality checks.

- Groups present their completed map and explain the importance of each step.

Extension:
Ask learners to identify one thing that could go wrong at each stage and what the downstream effect might be.

### KT0602: Process Flow and Productivity

Purpose of the Topic

This topic introduces learners to the concept of process flow in the upholstery workshop and how it directly influences productivity, efficiency, and quality control. Learners will explore the value of structured workflow, communication between stages, and the importance of adhering to sequence and timing.

By the end of this topic, learners will be able to identify the key stages in process flow, understand how tasks are scheduled and coordinated, and recognise their own role in maintaining smooth production.

Key Content Areas

- Definition of Process Flow

  - A process flow is the organised sequence of operations in manufacturing — from raw material intake to finished product output.

  - Each workstation represents a step in this flow.

- Importance of Process Flow in Upholstery

  - Reduces bottlenecks and idle time.

  - Prevents backtracking or duplication of effort.

  - Ensures each product moves smoothly from one station to the next.

  - Supports quality assurance by building in checkpoints.

- Link Between Process Flow and Productivity

  - Productivity is increased when tools, materials, and tasks are organised and predictable.

  - Interruptions (e.g. missing materials, unclear routing, tool failure) reduce workshop output.

  - Clean and structured environments lead to fewer mistakes and faster completion.

- Elements That Support Efficient Process Flow

  - Routing sheets and job cards

  - Labelling systems for parts and frames

  - Standardised workstations

  - Clear communication between departments

  - Regular feedback loops from quality control

Example Table: Productivity Blockers and Solutions

| Process Flow Disruption | Effect on Productivity | Possible Solution |
| --- | --- | --- |
| Missing foam pieces at padding | Padding team delayed | Check job card and request from storeroom |
| Frame arrives late to next station | Idle time for covering team | Rebalance schedule or flag in routing sheet |
| Covering team waits for specifications | Incorrect fabric cut and time wasted | Supervisor confirms job card at start of shift |
| Poor communication on batch status | Misaligned delivery with client deadline | Introduce daily production meeting |

Case Study: Disconnected Flow at Mokoena Interiors

Background:
At Mokoena Interiors in Bloemfontein, a batch of dining chairs was scheduled for delivery in five days. The covering team started early, but discovered that the frames were not yet foamed, even though the routing sheet marked them as “ready”.

Investigation:
The foaming team had been delayed by a tooling issue, but the update was not communicated to the routing clerk. The mistake created a bottleneck and the batch missed its dispatch window.

Resolution:

- A simple daily task progress sheet was introduced.

- The routing sheet was updated only after visual confirmation.

- The teams now hold a 10-minute shift briefing at the start of each day.

Learning Point:
Accurate process tracking and team communication are essential to maintain productivity.

Critical Thinking Questions

- Why does poor process flow result in lower productivity in an upholstery workshop?

- What can a team do if they notice a delay in the previous operation?

- How does your role affect other stations in the process flow?

- Why is it important to update routing sheets or job cards regularly?

- What are some signs that process flow in a workshop needs improvement?

Practical Classroom Activity

Task: Process Flow Simulation

Materials:

- Pre-written cards with different manufacturing steps

- A floor plan or layout of a sample upholstery workshop

- Coloured sticky notes or markers for disruptions

Instructions:

- Learners arrange the steps of upholstered furniture production in order along a mock “flow line”.

- The facilitator introduces disruptions (e.g. “foam not delivered”, “missing job card”) and learners discuss how it affects the downstream tasks.

- Each group presents their strategy for maintaining productivity despite disruptions.

Extension:
Ask learners to develop a simple checklist for process flow monitoring to use in a real upholstery setting.

### KT0603: Operations in the Process Flow – Knocker-On Operations, Foaming-Up Operations, Padding Sprung Foundations and Frames, and Covering Operations

Purpose of the Topic

This topic introduces learners to the key technical operations involved in the upholstery manufacturing process, namely: knocker-on operations, foaming-up operations, padding of sprung foundations and frames, and covering operations. Learners will understand the purpose, tools, sequence, and expected outcomes of each stage, and how accuracy and care at each point ensure the success of the final product.

By the end of this topic, learners will recognise how these operations interlink, how errors in one can affect the next, and how they contribute to overall efficiency, comfort, and quality.

Key Content Areas

- Knocker-On Operations

  - Securing webbing, frames, or brackets using hammers or pneumatic tools.

  - Usually the first assembly step after raw frame inspection.

  - Accuracy is crucial to ensure square, aligned joints.

- Foaming-Up Operations

  - Cutting, shaping, and adhering foam to frame surfaces.

  - May involve multiple foam layers of varying densities.

  - Requires precise alignment and clean adhesive application.

- Padding Sprung Foundations and Frames

  - Laying materials such as calico, Dacron, and padding over springs and foam.

  - Ensures even comfort and prepares the surface for fabric covering.

  - Important to stretch evenly and avoid bumps or wrinkles.

- Covering Operations

  - Cutting, fitting, and stapling upholstery fabric to the frame.

  - Pattern matching, grain alignment, and fabric tension control are vital.

  - May include finishing touches like piping, buttons, or trim.

- Sequence and Interdependence

  - These operations must follow a set order to maintain productivity and structural integrity.

  - Errors (e.g. foam misalignment) will affect all following tasks.

  - Clear job cards and routing sheets help guide the flow correctly.

Example Table: Operational Summary

| Operation | Tools Used | Key Consideration |
| --- | --- | --- |
| Knocker-On | Rubber mallet, tack hammer, stapler | Ensure frame is secure and square |
| Foaming-Up | Spray adhesive, utility knife, foam cutter | Foam cut to size, aligned and pressed evenly |
| Padding Sprung Base | Calico, Dacron, webbing stretcher | No lumps or folds, even layers |
| Covering | Upholstery fabric, stapler, scissors | Proper tension, aligned grain, wrinkle-free |

Case Study: Missed Operation Check at Legacy Designs

Background:
At Legacy Designs in Kimberley, a new team member, Brandon, began the foaming-up process on a batch of headboards before the knocker-on stage had been confirmed. The headboards lacked critical support brackets, and once the foam and padding were applied, it was too late to access the frame for bracket installation.

Outcome:

- 12 headboards had to be stripped and redone.

- The team added a pre-operation checklist to confirm that each step was signed off before continuing.

Learning Point:
Following the operational sequence avoids rework and ensures the product remains structurally sound.

Critical Thinking Questions

- Why must the knocker-on operation be completed before foaming-up begins?

- What would be the impact of loose padding on customer satisfaction?

- How does tension in the fabric affect the durability of the finished product?

- Why is it important to record operations correctly on routing sheets?

- What tools are essential during covering operations to ensure a neat result?

Practical Classroom Activity

Task: Operation Simulation and Process Board

Materials:

- Visual aids or samples of each operation stage

- Foam pieces, webbing, calico, covering fabric (for demo only)

- Process flow chart or blank board

Instructions:

- Learners are divided into groups and assigned one operation.

- Each group prepares a mini-presentation describing the operation, tools, purpose, and quality indicators.

- The facilitator assembles their input into a complete process board showing the upholstery production sequence.

Extension:
Use foam off-cuts and fabric to simulate a hands-on demo of correct vs incorrect foaming and padding techniques, then discuss outcomes.

### KT0604: Routing Sheets

Purpose of the Topic

This topic introduces learners to the purpose, format, and function of routing sheets in the upholstery manufacturing process. Routing sheets are essential for tracking a furniture item’s progress through production, ensuring all steps are completed in sequence, and maintaining quality and accountability.

By the end of this topic, learners will be able to identify the components of a routing sheet, describe its use in the workshop, and understand how it supports workflow management and traceability.

Key Content Areas

- What Is a Routing Sheet?

  - A document that outlines the sequence of operations required to manufacture a specific product.

  - Includes information on which workstations the item must pass through, in what order, and who is responsible at each stage.

- Key Features of a Routing Sheet

  - Product code or name

  - Operation steps, listed in manufacturing sequence

  - Assigned departments or workstations

  - Responsible persons or signatures

  - Timeframes or completion dates

  - Space for notes or deviations

- Functions of a Routing Sheet

  - Acts as a checklist to ensure all tasks are completed.

  - Helps track who performed each operation.

  - Supports time management and delivery scheduling.

  - Forms part of the quality control and traceability system.

- Importance in Upholstery Workflow

  - Prevents steps from being skipped or performed out of order.

  - Provides clarity to team members working across different shifts.

  - Allows supervisors to monitor progress and identify bottlenecks.

  - Essential for reporting on productivity, efficiency, and issues.

Example Table: Routing Sheet Excerpt

| Step | Operation | Workstation | Employee | Date Completed | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | Frame inspection | Assembly area | Nomvula | 2024-03-01 | Frame passed with minor chip |
| 2 | Knocker-on operation | Fastening station | Pieter | 2024-03-02 | All joints secure |
| 3 | Padding | Padding station | Sipho | 2024-03-03 | Foam fitted |
| 4 | Covering | Upholstery area | Zanele | 2024-03-04 | Awaiting final trim |

Case Study: Lost Routing Sheet at Elite Covers

Background:
Elite Covers in Pietermaritzburg lost a routing sheet for a batch of headboards midway through production. Without the sheet, the covering team could not confirm whether foam density and placement had been approved, and work was paused.

Outcome:

- The batch missed its deadline due to rechecking and clarification delays.

- The workshop instituted a digital routing system with backups.

- Team leads were trained on the importance of routing sheet integrity.

Learning Point:
Routing sheets are not optional – they are central to accountability, traceability, and production flow.

Critical Thinking Questions

- Why is it important to record each operation on the routing sheet as it is completed?

- What risks arise if routing sheets are incomplete, incorrect, or missing?

- How does the routing sheet contribute to teamwork in a multi-stage process?

- What is the benefit of using routing sheets in both small and large batch production?

- What can be done to ensure routing sheets are kept secure and up to date?

Practical Classroom Activity

Task: Routing Sheet Role-play and Completion Exercise

Materials:

- Sample routing sheets

- Role cards (e.g. frame builder, foam technician, upholsterer, finisher)

- Workshop scenario cards

Instructions:

- Learners simulate a mini upholstery production line using role cards.

- As each step is completed, learners sign and timestamp their section of the routing sheet.

- Discuss how the routing sheet helped track progress and identify any slow points or confusion.

Extension:
Introduce a situation where one routing sheet is missing or has an error — learners must explain how they would resolve the issue and prevent recurrence.

### KT0605: Cutting Lists

Purpose of the Topic

This topic introduces learners to the cutting list, a key production document used in the manufacturing of upholstered furniture. Cutting lists ensure that all frame components, foam pieces, fabric panels, and other materials are cut to the correct dimensions before assembly or application begins.

Learners will develop an understanding of the format, use, and function of cutting lists and the importance of accuracy and clarity when reading or referring to this document. Cutting lists support material control, job preparation, and workshop efficiency.

Key Content Areas

- Definition and Purpose of a Cutting List

  - A cutting list details all the parts or components that need to be cut for a particular product or job.

  - It includes dimensions, quantities, material types, and sometimes notes about shape, angle, or allowance.

- What a Cutting List Contains

  - Component name or description (e.g. seat foam, back frame)

  - Quantity required

  - Dimensions (length × width × thickness)

  - Material type (e.g. chipboard, high-density foam, fabric type)

  - Additional notes (e.g. grain direction, mitred corners, allowance added)

- Role of the Cutting List in Production

  - Ensures that all materials are prepared before assembly begins.

  - Helps prevent material wastage or incorrect cuts.

  - Links closely with job cards, routing sheets, and product specifications.

  - Can be used as a checklist during the preparation stage.

- Accuracy and Communication

  - Errors on a cutting list can lead to incorrect part sizing, misalignment, and production delays.

  - All dimensions should be checked against the product specification before cutting begins.

  - Where unclear, the learner should ask for clarification from a supervisor or team lead.

Example Table: Excerpt from a Cutting List

| Item No. | Component | Qty | Dimensions (mm) | Material | Notes |
| --- | --- | --- | --- | --- | --- |
| 001 | Seat base panel | 2 | 550 × 500 × 18 | Chipboard | Square edges |
| 002 | Seat foam block | 2 | 540 × 490 × 50 | High-density foam | Allow 5 mm compression |
| 003 | Outer back cover | 2 | 600 × 650 | Upholstery fabric | Include 30 mm allowance each side |

Case Study: Mismatched Cuts at Bayside Upholstery

Background:
At Bayside Upholstery in Port Shepstone, the cutting team received a job card for six custom armchairs. The cutting list specified fabric panel widths as 450 mm, but the product specification listed the actual frame width as 480 mm. None of the pieces had added allowance for stapling.

Outcome:

- All fabric panels were too narrow and had to be recut.

- Several metres of fabric were wasted, and the delivery was delayed.

- A new protocol was introduced requiring double-verification of cutting list dimensions against specifications.

Learning Point:
Even minor errors in a cutting list can lead to significant waste and rework. Cross-checking is essential.

Critical Thinking Questions

- Why should every learner check a cutting list against the product specification before cutting begins?

- What would be the risk of not including allowances in the listed dimensions?

- How does a cutting list help improve workshop productivity and reduce material waste?

- What should a learner do if a dimension or item in the cutting list is unclear?

- Why is it important to record quantities accurately on the cutting list?

Practical Classroom Activity

Task: Cutting List Analysis and Correction Exercise

Materials:

- Sample cutting lists with minor errors or incomplete data

- Product specifications for the same item

- Colour-coded pens or markers

Instructions:

- Learners compare the cutting list with the product specification.

- Identify errors or missing information (e.g. forgotten allowance, mismatched foam size).

- Suggest corrected dimensions or notes.

- Discuss how these changes prevent waste and promote quality.

Extension:
Ask learners to prepare a simple cutting list based on a sketch or frame diagram provided by the facilitator.

### KT0606: Product Specifications

Purpose of the Topic

This topic introduces learners to the concept and application of product specifications in the upholstered furniture manufacturing process. Product specifications describe the technical and visual expectations for the finished item and inform decisions throughout the process flow, including cutting, assembling, padding, covering, and finishing.

By the end of this topic, learners will understand how to read, interpret, and apply product specifications, and how design and construction requirements directly influence the materials, methods, and sequencing used in production.

Key Content Areas

- What Are Product Specifications?

  - A set of detailed instructions and criteria that define:

    - Dimensions

    - Shape and structure

    - Materials and components

    - Colour, texture, and finishes

    - Comfort and functional expectations

  - May be presented as a technical drawing, a spec sheet, or client brief.

- Purpose and Role in the Manufacturing Process

  - Guide the selection of tools, materials, and procedures.

  - Ensure consistency across multiple units or production runs.

  - Help workers understand the end goal and maintain quality standards.

  - Allow for cost control by planning material usage precisely.

- How Specifications Influence Process Flow

  - A design with curves requires more flexible materials and cutting precision.

  - A product with buttons or piping alters the covering operation.

  - Foam thickness, firmness, and fabric choice depend on performance requirements.

  - Knocker-on and padding methods change if the frame shape is non-standard.

- Responding to Specification Changes

  - Changes must be confirmed in writing via a revised specification.

  - Teams must communicate updates to prevent errors.

  - Production must pause if specifications are unclear or missing.

Example Table: Specification-to-Process Impact

| Specification Detail | Impact on Manufacturing Process |
| --- | --- |
| Foam density: 32/200 | Choose correct foam grade during padding/foaming |
| Seat width: 550 mm + 30 mm wrap | Cut fabric to include allowance, confirm foam dimension |
| Upholstery: Faux leather, black | Use sharp tools for cutting, avoid creasing during cover |
| Trim: Double piping | Add piping to cutting list, include trim allowance |

Case Study: Misread Specification at GreenEdge Upholstery

Background:
At GreenEdge Upholstery in Port Elizabeth, the team produced a set of 10 ottomans based on an outdated version of the product specification. The client had changed the covering material from woven fabric to faux leather, which required different cutting and stapling techniques. The leather was damaged due to excessive stretching and incorrect tools.

Outcome:

- All ottoman covers had to be replaced.

- The cutting and covering teams were retrained on reading and confirming specification updates.

- Specifications were moved to a central folder with date-stamped versions.

Learning Point:
Specifications must be read carefully and consistently — they drive the entire manufacturing process and are not optional.

Critical Thinking Questions

- Why must product specifications be treated as part of the manufacturing process, not just a design document?

- What could happen if padding materials are chosen without checking the specifications?

- How should changes to product specifications be communicated across the production team?

- What is the risk of working with an outdated version of a specification sheet?

- How can workers ensure they are using the correct specification at the start of each batch?

Practical Classroom Activity

Task: Specification Interpretation and Process Planning

Materials:

- Sample product specification (including drawing and detail)

- A blank process flow sheet

- Coloured pens or markers

Instructions:

- Learners read the product specification and highlight key manufacturing details.

- They complete a mini process flow, identifying how each specification affects each stage of production (cutting, padding, covering).

- Present their completed plans to the group for feedback.

Extension:
Include a deliberate specification change (e.g. fabric type or foam thickness) and ask learners to revise their plan accordingly.

### KT0607: Finishing Aids

Purpose of the Topic

This topic introduces learners to the finishing aids used during the final stages of upholstered furniture manufacturing. These include the tools, products, and materials that help ensure the furniture is presented in a clean, professional, and client-ready condition.

Learners will understand what finishing aids are, why they are important, and how to use them safely and effectively to enhance the appearance, comfort, and quality of the final product.

Key Content Areas

- Definition of Finishing Aids

  - Items used at the end of the manufacturing process to enhance presentation or protect the product.

  - Can be tools, materials, or chemical products applied during or after the final inspection.

- Types of Finishing Aids

  - Spray bottles with water or anti-wrinkle solution (for fabric smoothing).

  - Steamers for wrinkle removal and fabric stretching.

  - Scissors or snips for trimming threads or loose fibres.

  - Vacuum cleaners or air blowers to remove dust and debris.

  - Cleaning cloths or furniture polish for wiping hard surfaces.

  - Protective coverings (plastic wrap, fabric bags) for shipping or display.

- Purpose and Use of Finishing Aids

  - Improve aesthetic quality (smoothness, cleanliness).

  - Enhance customer satisfaction by presenting a polished product.

  - Maintain product hygiene and integrity during storage and transport.

  - Ensure safety by removing foreign objects (e.g. staples, pins).

- Precautions and Safe Use

  - Avoid harsh chemicals that could damage upholstery.

  - Follow safety guidelines for heat-based tools like steamers.

  - Test products on hidden areas before full application.

  - Store finishing aids separately and label clearly.

Example Table: Common Finishing Aids

| Finishing Aid | Function | Usage Tip |
| --- | --- | --- |
| Hand steamer | Removes fabric wrinkles | Keep at safe distance to prevent scorching |
| Lint roller | Removes hair, dust, or loose threads | Roll gently to avoid pulling fabric |
| Fabric protector spray | Adds stain resistance | Use in well-ventilated area |
| Snips or thread trimmer | Cuts loose ends or errant stitches | Use before final inspection or photography |
| Polishing cloth | Buffs and shines wooden or metal legs | Use soft, clean cloth to avoid scratches |

Case Study: Final Touch Failures at DecoDesign

Background:
At DecoDesign in Gqeberha, a batch of chairs passed all production checks but was delivered to the client with visible chalk marks, lint, and loose threads. The team had skipped the finishing stage to save time before dispatch.

Outcome:

- The client returned four units and requested a quality audit.

- A “final touch checklist” was introduced for all dispatches.

- Staff were retrained on the purpose and safe use of finishing aids.

Learning Point:
Skipping finishing aids undermines the value of good manufacturing – quality must be maintained to the last stage.

Critical Thinking Questions

- Why is the finishing stage as important as cutting or padding?

- What could happen if finishing aids like steamers are misused?

- How do finishing aids improve the perceived value of furniture?

- Why is it important to test sprays or solutions on hidden areas first?

- What system can be used to ensure no product leaves the factory without proper finishing?

Practical Classroom Activity

Task: Finishing Station Setup and Demonstration

Materials:

- Sample upholstered item (real or mock-up)

- Finishing aids: spray bottle, lint roller, snips, polishing cloth

- Final inspection checklist

Instructions:

- Learners practise using each finishing aid on a sample product.

- Each learner completes a finishing checklist, verifying the product is ready for dispatch.

- Discuss the importance of visual presentation and how it reflects on brand quality.

Extension:
Create a finishing station layout diagram and label the purpose and position of each aid/tool.

## Integrated Assessment – KM-03-KT06

Qualification: Furniture Upholsterer (SAQA ID 103199)
Knowledge Module: KM-03-KT06 – Upholstered Furniture Manufacturing Processes
NQF Level: 2 | Credits: 8 | Weighting: 10%
Assessment Type: Variety of Instruments
Duration: 60 minutes
Total Marks: 40

🔹 Section A: Matching (5 marks)

Match the process flow term in Column A with its correct description in Column B.

| Column A | Column B |
| --- | --- |
| A. Knocker-on operation | 1. Used to track progress through each manufacturing stage |
| B. Routing sheet | 2. Process of shaping and attaching foam to the frame |
| C. Covering operation | 3. Attaching webbing, frames or brackets |
| D. Foaming-up | 4. Final layer applied for aesthetic and function |
| E. Finishing aids | 5. Tools used to clean, trim, and prepare for dispatch |

🔹 Section B: Short Answer (3 marks each × 5 = 15 marks)

- What is the consequence of skipping a step in the upholstered furniture process flow?

- Name three common finishing aids and explain how each one is used.

- What information is found on a routing sheet, and why is it important?

- Explain how the cutting list and job card are used together in production.

- How do product specifications influence the way a piece of furniture is manufactured?

🔹 Section C: Scenario-Based Questions (2 marks each × 5 = 10 marks)

Scenario:
Thando is assembling a custom ottoman. He completes the foaming-up process and proceeds directly to covering. He later discovers that the padding stage was skipped, which causes the fabric to sit unevenly. Meanwhile, his routing sheet was not updated, and the covering team assumed the item was ready.

- What was the key mistake made by Thando?

- What could have been done to prevent this error?

- How does skipping padding affect the quality of the finished product?

- What step should be taken before beginning the covering stage?

- How can accurate use of the routing sheet improve communication in the workshop?

🔹 Section D: Application (2 marks each × 5 = 10 marks)

- Identify the four main operational stages in the upholstery process.

- What information would you expect to see on a cutting list for a backrest panel?

- What is the purpose of using finishing aids after the covering operation?

- Why is it important to confirm all materials and chemicals on a job card?

- What are two possible consequences of using outdated or incorrect product specifications?

✅ Model Answers and Marking Memo

Section A:
A–3, B–1, C–4, D–2, E–5

Section B Sample Answers:
6. Workflow is disrupted; quality may suffer or items may need to be redone.
7. Lint roller (removes hair), steamer (removes wrinkles), snips (trims loose threads).
8. Product ID, steps, assigned employees, dates. Tracks item’s manufacturing journey.
9. Cutting list gives materials/dimensions; job card explains sizes, notes, and chemicals.
10. They determine materials, sequence, tools, and tolerances for manufacturing.

Section C Sample Answers:
11. Thando skipped the padding stage.
12. He should have reviewed the routing sheet and confirmed all previous steps.
13. Fabric may wrinkle, appear lumpy, or fail to meet comfort expectations.
14. Confirm that all previous stages are signed off and properly completed.
15. It helps the team track what is done, by whom, and what comes next.

Section D Sample Answers:
16. Knocker-on, foaming-up, padding, covering.
17. Dimensions, quantity, material, allowance, shape/angle note.
18. Improve visual quality, remove debris, ensure professional finish.
19. Prevents misuse of chemicals or wrong sizes/materials.
20. Incorrect dimensions, unsuitable materials, customer complaints.

📊 Assessment Rubric

| Criteria | Outstanding (4) | Competent (3) | Partially Achieved (1–2) | Not Achieved (0) |
| --- | --- | --- | --- | --- |
| Process Understanding | Thorough understanding of all operations and sequences | General understanding, minor gaps | Incomplete or vague understanding | Incorrect or missing knowledge |
| Document Interpretation | Accurately explains routing sheet, job card, spec usage | Mostly correct; minor errors | Misinterprets key documents | Cannot identify purpose of documents |
| Application of Knowledge | Correct responses to scenarios and tools | Minor application errors | Unclear link between theory and application | Incorrect or impractical responses |
| Quality and Clarity of Answers | Precise, well-structured, uses terminology accurately | Mostly clear with few gaps | Vague explanations or poor structure | Unreadable or missing answers |

### Facilitator Assessment Briefing

Knowledge Module: KM-03-KT06 – Upholstered Furniture Manufacturing Processes
Qualification: Furniture Upholsterer (SAQA ID 103199)
NQF Level: 2 | Credits: 8 | Weighting: 10%
Assessment Type: Variety of Instruments
Duration: 60 minutes
Total Marks: 40

🎯 Purpose of the Assessment

This assessment aims to evaluate learners’ understanding of:

- The full sequence of operations in upholstered furniture production.

- The role and impact of process documents such as routing sheets, job cards, cutting lists, and product specifications.

- The importance of each stage from frame preparation to finishing, and how errors or miscommunication affect the final output.

All questions are aligned with IAC0601 to IAC0606.

✅ Assessment Instrument Breakdown

| Section | Format | Marks |
| --- | --- | --- |
| A | Matching | 5 |
| B | Short Answer | 15 |
| C | Scenario-Based Questions | 10 |
| D | Application | 10 |
| Total |  | 40 |

🛠 Facilitator Preparation Checklist

- Ensure copies of the assessment are printed and collated.

- Have the marking memo and rubric ready.

- Confirm that learners have pens, rulers, and calculators if needed.

- Prepare sample routing sheets, job cards, and cutting lists (for reference).

- Arrange a clean, quiet space for the assessment.

📢 Instructions to be Read to Learners

- This assessment tests your understanding of furniture manufacturing processes and related documents.

- Answer all questions clearly and show calculations where needed.

- Use full sentences and technical terms where appropriate.

- You have 60 minutes to complete this assessment.

- Raise your hand if you need help understanding the question — no answers will be given.

- This is an individual task. Do not communicate with others during the session.

📊 Marking and Moderation Guidelines

- Use the model answers to guide marking.

- Accept varied but relevant phrasing that shows clear understanding.

- Assign partial marks for steps shown in calculations or logical reasoning.

- Ensure marking is fair, consistent, and complete, and prepare files for moderation.

## KM-03-KT07: Upholstery Covering Principles

Weighting: 10% | NQF Level: 2

1. Purpose of the Knowledge Topic

The purpose of this Knowledge Topic is to introduce learners to the fundamental principles and practices involved in applying fabric or material to upholstered furniture during the covering stage of the manufacturing process. This includes how to manage fabric tension, alignment, symmetry, fastening methods, and visual uniformity.

Covering is one of the most visible aspects of upholstered furniture and is often a key indicator of quality in the final product. Learners will be taught how to ensure both functional durability and aesthetic appeal, and how to prepare covered items for the next stage in the production process.

2. Key Knowledge Areas (Topic Elements)

The following topic elements are covered in this Knowledge Topic:

- KT0701: Stretch, tension and deflection of fabric

- KT0702: Spacing of tacks, nails and staples

- KT0703: Uniformity of a pair of arms and wings, pleating and folding and buttoning

- KT0704: Positioning of the cover and alignments

- KT0705: Pattern matching (fabric)

- KT0706: Principles and concepts of preparing and stacking covered frames for the next step in the production process

3. Internal Assessment Criteria and Weighting

This Knowledge Topic carries a total weighting of 10% and will be assessed according to the following Internal Assessment Criterion:

- IAC0701: Covering principles are identified and the effect thereof on quality end-product is analysed.

4. Application in Upholstered Furniture Manufacturing

In the upholstery workshop, a poorly executed covering stage can render an otherwise well-constructed product unacceptable to clients. Accurate alignment, consistent fastening, correct tension, and attention to symmetrical design features are all essential. This topic empowers learners to work confidently in a high-precision task area, while also understanding how their workmanship impacts the next stage of production and the overall customer experience.

### KT0701: Stretch, Tension and Deflection of Fabric

Purpose of the Topic

This topic introduces learners to the concepts of stretch, tension, and deflection when applying fabric or material to upholstered furniture. These physical properties determine how the cover will behave during application and how it will perform over time. Improper control of these elements can lead to wrinkling, sagging, tearing, or misalignment.

By mastering this topic, learners will be able to apply upholstery fabric correctly with consistent tension, ensuring both durability and visual uniformity in the final product.

Key Content Areas

- Fabric Stretch

  - The natural ability of fabric to expand under force.

  - Varies depending on fabric type, weave, and direction (warp vs weft).

  - Excessive stretch during application may lead to distortion after use.

- Tension

  - The force applied when pulling fabric over a frame.

  - Must be even and appropriate – not too tight (which causes puckering) and not too loose (which causes sagging).

  - Tension should be monitored continuously during stapling or tacking.

- Deflection

  - The change in position or shape of fabric or frame components under tension.

  - Can occur when frames are not reinforced or when fabric pulls too hard on lightweight components.

  - Managing deflection ensures that the fabric remains smooth and secure.

- Correct Application Techniques

  - Always apply tension from the centre outwards, balancing each side.

  - Use both hands or tensioning tools where required.

  - Secure fabric temporarily first, inspect, then fasten permanently.

  - Constantly check for evenness across the surface.

Example Table: Fabric Behaviour by Type

| Fabric Type | Stretch Behaviour | Tension Handling Tip |
| --- | --- | --- |
| Cotton Twill | Minimal stretch | Tension must be firm but evenly distributed |
| Faux Leather | Moderate stretch (heat-sensitive) | Avoid over-pulling to prevent tears |
| Knitted Upholstery | High stretch | Use light, gradual tension and more fasteners |
| Woven Polyester Mix | Moderate with firm weave | Test stretch direction before covering |

Case Study: Over-Tensioning at Nash Interiors

Background:
At Nash Interiors in Bloemfontein, Nontando was covering a lounge chair with a two-way stretch polyester fabric. In an effort to achieve a perfectly smooth finish, she applied too much tension, especially across the front corners. Within days, the client reported visible fabric tearing at the staple points and slight frame warping.

Intervention:
The supervisor demonstrated how to balance tension gradually, test the stretch of fabric beforehand, and avoid over-tightening across unsupported sections of the frame.

Learning Point:
Excessive tension causes fabric failure and long-term distortion. Fabric properties must be understood and respected in application.

Critical Thinking Questions

- Why should fabric be applied from the centre outwards when covering a seat or panel?

- How does the direction of the fabric grain influence tension?

- What could happen if a highly stretchable fabric is pulled too tightly?

- How can a worker check for even tension during the covering process?

- What steps would you take if the frame begins to deflect during fabric application?

Practical Classroom Activity

Task: Tension Testing and Stretch Comparison

Materials:

- Sample squares of various upholstery fabrics

- Small wooden frame mock-ups

- Stapler, tacking hammer, chalk or markers

Instructions:

- Learners stretch each fabric sample across a mock frame to observe its stretch properties.

- Measure how much the fabric stretches under light, medium, and heavy pull.

- Practise securing each fabric type at correct tension and inspect the results.

- Compare the effect of under-tensioning and over-tensioning on each fabric type.

Extension:
Facilitator simulates a poor tensioning job (e.g. too loose on one side) and learners must identify the problem and explain how they would fix it.

### KT0702: Spacing of Tacks, Nails and Staples

Purpose of the Topic

This topic introduces learners to the principles of correct spacing when fastening upholstery materials using tacks, nails, or staples. The position and spacing of fasteners directly affect both the appearance and durability of the upholstered piece. Learners will understand how to apply fasteners consistently and securely, in line with both visual standards and structural requirements.

By the end of this topic, learners will be able to identify correct spacing intervals for various applications and recognise the impact of spacing errors on quality, safety, and production efficiency.

Key Content Areas

- Purpose of Correct Spacing

  - Provides even grip to hold the fabric securely.

  - Maintains smooth, wrinkle-free appearance.

  - Prevents gaps, looseness, or fraying around edges.

  - Supports structural integrity of the frame where applicable.

- General Spacing Guidelines

  - Standard spacing ranges between 10 mm and 50 mm, depending on the task:

    - Tightly curved or tensioned areas: closer spacing (10–20 mm)

    - Flat, stable areas: wider spacing (30–50 mm)

  - Uniform spacing improves aesthetic finish and reduces risk of fabric shifting.

- Tool-Specific Considerations

  - Staple Guns: Ensure staples are fully driven in, placed in a straight line.

  - Tacks: Apply with consistent hammer pressure; spacing must follow contour.

  - Upholstery Nails: Often used as decorative elements – alignment and equal spacing are essential.

- Factors That Influence Spacing

  - Fabric type and thickness.

  - Amount of tension applied during covering.

  - Curves and corners, which may require denser spacing.

  - Use of piping, buttons, or trims, which can obscure fastener lines.

Example Table: Fastener Spacing by Application

| Application Area | Recommended Spacing | Reason |
| --- | --- | --- |
| Straight seat base panel | 30–40 mm | Standard support with minimal tension |
| Inside arm curve | 10–15 mm | High tension and curved area |
| Outer back with faux leather | 20–30 mm | Avoid sagging and tearing |
| Decorative nail row | Precisely measured gaps | Visual uniformity and client expectations |

Case Study: Uneven Staple Line at Zinzi Seating

Background:
At Zinzi Seating in Cape Town, Simphiwe applied fabric to a lounge chair backrest using staples spaced irregularly – ranging from 15 mm to 60 mm. The areas with wider spacing showed visible puckering, while some staples were misaligned and overdriven, causing indentation marks on the surface.

Outcome:
The piece was returned to the workshop for correction. Simphiwe received coaching on staple spacing techniques, including how to use a spacing gauge and start from the centre line outward.

Learning Point:
Inconsistent spacing reduces both aesthetic value and product quality. Precision is essential.

Critical Thinking Questions

- Why does closer spacing improve performance in curved or high-tension areas?

- What might happen if fasteners are spaced too far apart on faux leather?

- How does consistent spacing affect the appearance of decorative nails?

- What is the benefit of using a guide or measurement tool when spacing fasteners?

- How could incorrect fastener spacing affect the longevity of the upholstery?

Practical Classroom Activity

Task: Spacing Practice and Visual Inspection

Materials:

- Fabric offcuts, foam, and small wooden boards

- Manual tacking tools and pneumatic staple guns

- Rulers, chalk lines, spacing templates

Instructions:

- Learners practise applying fasteners at various spacing intervals on mock upholstery setups.

- Peer review to check for alignment, uniformity, and surface appearance.

- Each learner creates a small “sample board” showing 3 different spacing styles and explains when each would be used.

Extension:
Use a decorative nail spacing jig, and challenge learners to achieve visual perfection over a set distance.

### KT0703: Uniformity of a Pair of Arms and Wings, Pleating and Folding and Buttoning

Purpose of the Topic

This topic equips learners with the knowledge and practical understanding required to achieve uniformity and consistency in visible upholstery details, specifically in pairs of arms and wings, as well as pleating, folding, and buttoning techniques. These elements are key contributors to the aesthetic value and symmetry of upholstered furniture.

By mastering these skills, learners will ensure that their work meets professional visual standards, delivers functional comfort, and reflects high-quality craftsmanship.

Key Content Areas

- Uniformity in Pairs of Arms and Wings

  - Both sides must be identical in height, width, angle, and tension.

  - Use mirroring techniques during covering to ensure consistency.

  - Always compare against drawings, templates, or the completed opposite side.

- Pleating and Folding

  - Used to shape fabric smoothly around curves or corners.

  - Should be even, symmetrical, and firmly secured with consistent spacing.

  - Improves appearance, especially in rolled arms, wings, and corners.

  - Requires practice and attention to grain direction and fabric behaviour.

- Buttoning (e.g. deep buttoning or decorative)

  - Must follow the correct grid or pattern (e.g. diamond, square).

  - Buttons must be aligned, evenly spaced, and secured with even depth.

  - Incorrect buttoning can cause distortion or discomfort.

- Common Errors and How to Avoid Them

  - Misaligned pairs: caused by rushing or failing to measure consistently.

  - Uneven pleats: result from poor marking or inconsistent pulling.

  - Crooked buttoning rows: due to inaccurate spacing or lack of templates.

Example Table: Techniques for Symmetrical Features

| Feature | Uniformity Technique | Common Check |
| --- | --- | --- |
| Armrests (left and right) | Measure height and distance from centreline | Use spirit level or template |
| Wingback curves | Mirror covering sequence | Compare one side against the other |
| Pleats on rolled arms | Count folds, space equally | Use pins to mark and visual inspection |
| Button rows | Use chalk grid, start from centre | Measure spacing with string or ruler |

Case Study: Inconsistent Detailing at Thulani Designs

Background:
At Thulani Designs in East London, the team produced a set of six wingback chairs. During final inspection, it was noted that the pleating on the right arm of each chair had two fewer folds than the left arm, and the top button on several chairs was off-centre by 15 mm.

Outcome:

- The batch was returned for correction.

- The team introduced a pleating and buttoning checklist and assigned peer checks before items moved to finishing.

- Templates were standardised for pleat spacing and button grid placement.

Learning Point:
Lack of attention to uniformity undermines visual appeal and affects client satisfaction — especially in paired and repeated details.

Critical Thinking Questions

- Why is it important that both arms of a chair appear identical?

- What are the risks of uneven pleats or folds in terms of aesthetics and function?

- How does buttoning technique affect the comfort of the seat or backrest?

- What tools or methods can help ensure perfect symmetry in repeated details?

- Why is visual inspection not enough when checking uniformity — what else should be done?

Practical Classroom Activity

Task: Paired Symmetry Practice and Buttoning Grid

Materials:

- Upholstered mock arms or wings (foam or board), pins, chalk, tape measures

- Fabric samples, buttons, thread, foam or cardboard boards

Instructions:

- Learners work in pairs to cover a set of matching “arm” forms.

- They must mirror the pleating or fold pattern exactly between left and right.

- Each learner also completes a buttoning layout on a mock board using a pre-drawn grid.

- Peer review and discussion to assess uniformity and offer suggestions for improvement.

Extension:

Challenge learners to identify uniformity errors in sample photos of professional upholstery and propose corrective techniques.

### KT0704: Positioning of the Cover and Alignments

Purpose of the Topic

This topic focuses on the importance of accurately positioning the fabric or cover on an upholstered furniture piece and ensuring correct alignment across all surfaces and edges. Learners will understand how to centre fabric features, align covers to structural components, and avoid visual or structural distortions that result from misalignment.

Correct positioning and alignment contribute directly to the professional appearance, comfort, and customer satisfaction of the finished product.

Key Content Areas

- Correct Positioning of Covers

  - Centre the fabric both horizontally and vertically before stapling.

  - Begin tacking from the centre outward to distribute tension evenly.

  - Use reference points on the frame (e.g. corners, centre marks) to guide placement.

- Alignment of Edges and Patterns

  - Ensure the fabric grain runs in the correct direction (warp and weft alignment).

  - Visual elements (e.g. stripes, patterns, seams) must align across parts.

  - Check alignment before fixing permanently — minor shifts can create major defects.

- Common Issues with Poor Positioning

  - Twisting or warping of fabric lines or patterns.

  - Misaligned seams that appear unprofessional.

  - Uneven tension causing fabric to bunch or sag.

  - Covers that pull away from corners or shift under pressure.

- Tools and Techniques

  - Use of chalk lines, centre marks, or templates.

  - Temporary pinning or light stapling to test positioning.

  - Visual inspection from multiple angles before final fastening.

Example Table: Positioning Errors and Their Impact

| Positioning Error | Likely Impact on Product | Preventive Measure |
| --- | --- | --- |
| Cover not centred on seat base | Uneven seams and tension | Mark centre lines and align before fastening |
| Pattern off-centre or skewed | Visual imbalance and customer dissatisfaction | Lay fabric flat and align using pattern lines |
| Fabric twisted on backrest | Misalignment of grain, visible tension lines | Check vertical and horizontal symmetry |

Case Study: Misalignment at CustomEdge Upholstery

Background:
At CustomEdge Upholstery in George, a batch of dining chairs with striped fabric was delivered to a client. Upon unpacking, the client noted that the stripe pattern was visibly off-centre on many chairs, and some seats had lines that tilted diagonally across the surface.

Investigation:
The covering team had not used chalk lines or templates and relied on visual judgement alone. No alignment check was performed before stapling.

Outcome:

- The entire batch had to be redone.

- A centre-marking procedure was introduced.

- Mandatory visual verification was added to the covering SOP.

Learning Point:
Small positioning errors during covering can cause large aesthetic failures in the final product.

Critical Thinking Questions

- Why is it important to begin stapling or tacking fabric from the centre outwards?

- What visual cues can help learners identify misalignment before the fabric is fixed?

- How can a learner make sure a patterned fabric remains straight and symmetrical during covering?

- What are the consequences of misaligned seams or grain direction in client perception?

- What steps can be taken to correct a cover that has been attached incorrectly?

Practical Classroom Activity

Task: Centre Marking and Pattern Alignment Exercise

Materials:

- Upholstery fabric (plain and patterned)

- Mock seat bases or backrest boards

- Chalk, pins, and measuring tape

Instructions:

- Learners mark the centre of both the fabric and the frame component.

- Align the fabric to the component and test positioning with light pinning.

- Staple lightly, inspect for alignment, then complete fastening.

- Practise this process with both plain and patterned fabrics and compare results.

Extension:
Present learners with pre-covered items (some aligned, some misaligned) and ask them to identify the issues and suggest corrective steps.

### KT0705: Pattern Matching (Fabric)

Purpose of the Topic

This topic introduces learners to the principles and techniques of fabric pattern matching in upholstery. Pattern matching refers to aligning the design elements of the fabric—such as stripes, checks, florals, or motifs—across different parts of a furniture piece so that the appearance is visually continuous and symmetrical.

Mastering pattern matching is essential for achieving professional-level aesthetics, especially on premium or custom furniture where clients expect perfect alignment of all visual features.

Key Content Areas

- What Is Pattern Matching?

  - Aligning repeating patterns on upholstery fabric so that the design flows seamlessly across adjoining sections.

  - Matching is especially important for visible areas like front panels, arms, backs, and cushions.

- Types of Patterns

  - Directional (e.g. florals with stems or motifs that face a direction).

  - Geometric (e.g. stripes, checks, diamonds).

  - Random or non-directional (e.g. textures with no fixed orientation).

- Pattern Matching Techniques

  - Mark the pattern repeat before cutting the fabric.

  - Always allow extra length and width to align repeats during cutting.

  - Use templates, chalk lines, and measurement tools to mark where the design should meet.

  - Cut all pieces with pattern direction in mind—consistency is key.

- Impact of Incorrect Pattern Matching

  - Results in a product that appears misaligned or chaotic.

  - Gives a poor visual impression and may lead to customer complaints or returns.

  - Often leads to fabric wastage due to re-cutting or rejection.

Example Table: Pattern Matching Considerations

| Pattern Type | Matching Challenge | Upholstery Consideration |
| --- | --- | --- |
| Stripes | Must run straight and match at seams | Use centreline and mark carefully before cut |
| Floral repeat | Needs even vertical alignment | Centre large motifs on key visual areas |
| Checks | Match horizontally and vertically | Align both grain and repeat from base up |
| Abstract | Less visible alignment needed | Still check for symmetry and orientation |

Case Study: Broken Stripes at Prestige Furniture

Background:
At Prestige Furniture in Nelspruit, a customer ordered a set of armchairs using a striped linen-blend fabric. When delivered, several chairs had visibly misaligned stripes between the arm panels and the seat base. Some stripes even angled downward, making the product look untidy.

Investigation:
The cutter did not adjust the cutting layout to accommodate the stripe pattern. No extra fabric had been allowed for repeat matching, and pieces were cut strictly to size.

Outcome:
The items had to be reupholstered at the manufacturer’s expense. The team revised its fabric planning system and introduced a compulsory pattern-matching briefing during cutting.

Learning Point:
Pattern matching begins at the cutting stage. Proper planning prevents costly rework.

Critical Thinking Questions

- Why is it necessary to allow extra fabric when working with patterned upholstery materials?

- What is the difference between matching stripes and matching floral prints?

- How can incorrect pattern matching affect customer perception?

- What are some tools that can assist in achieving accurate pattern alignment?

- What steps would you follow to ensure that all patterned panels of a chair are correctly matched?

Practical Classroom Activity

Task: Pattern Matching and Layout Planning

Materials:

- Upholstery fabric samples (striped, floral, geometric)

- Paper templates for seat and back panels

- Scissors, chalk, measuring tapes

Instructions:

- Learners identify the repeat pattern on each fabric sample.

- Using templates, they lay out how they would cut matching pieces.

- Mark matching points and note direction and grain.

- Discuss how much extra fabric is required and how to minimise waste while achieving alignment.

Extension:
Set a challenge to plan pattern matching across three connected panels (e.g. seat, inside arm, and backrest) using minimal fabric.

### KT0706: Principles and Concepts of Preparing and Stacking Covered Frames for the Next Step in the Production Process

Purpose of the Topic

This topic introduces learners to the correct procedures for preparing, handling, and stacking covered frames at the end of the covering stage. Once a furniture item has been covered, it must be safely and efficiently transferred to the next department (e.g. finishing, quality control, or packing) without being damaged, dirtied, or mislabelled.

Learners will understand the importance of clean handling practices, the correct use of protective materials, and how to organise stacked items to support smooth workflow and product integrity.

Key Content Areas

- Preparation Before Stacking

  - Inspect the frame to ensure covering is complete and meets quality standards.

  - Remove loose threads, dust, or residue.

  - Attach required labels or routing documentation.

- Safe Handling Practices

  - Use clean hands or gloves to prevent marking fabric or leather.

  - Avoid dragging or stacking on dirty or rough surfaces.

  - Work in pairs for large or bulky items to prevent damage from drops or twisting.

- Stacking Principles

  - Stack items vertically or side-by-side, not directly on top of one another unless designed to nest.

  - Use soft padding, felt, or blankets between items.

  - Leave space for ventilation and access.

  - Keep stacking areas clear, labelled, and designated.

- Production Flow Considerations

  - Stack in the order in which the next department will receive items.

  - Notify the receiving team or update the routing system when items are ready.

  - Keep stacking areas close to the point of completion to minimise movement.

Example Table: Stacking Practices and Risk Prevention

| Action | Potential Risk Prevented | Best Practice |
| --- | --- | --- |
| Use of blankets between items | Prevents fabric friction and creasing | Place a soft cloth between every covered frame |
| Labelling before stacking | Prevents mix-ups or delays | Attach label to the underside or back with job number |
| Clear pathway to storage area | Prevents collisions and damage | Keep route open and free of sharp objects |
| Gloves worn during handling | Prevents marks and stains on fabric | Require gloves for final product handling |

Case Study: Scratched Covers at Mavuso Interiors

Background:
At Mavuso Interiors in Durban, a team stacked completed dining chairs directly on top of one another on a wooden pallet. The top three chairs arrived in the finishing section with visible marks on the outer back panels due to frame-to-fabric contact.

Outcome:

- The chairs were sent back for repair, causing delays.

- A new protocol was introduced: all covered items must be separated with padded cloths, and no items may be double-stacked without approval.

- The team leader was tasked with daily inspection of the stacking area.

Learning Point:
Proper stacking is a protective measure—not just a space-saving method. Prevention saves time and cost.

Critical Thinking Questions

- Why is it important to clean and inspect a covered frame before stacking?

- What risks are introduced when items are stacked directly on one another without padding?

- How can you organise stacked items to support efficiency in the next department?

- What are some of the responsibilities of the covering team before the product moves on?

- Why is it important to communicate with the next team after stacking is complete?

Practical Classroom Activity

Task: Stacking Simulation and Handling Safety

Materials:

- Covered sample boards or mock frame models

- Blankets, labels, gloves, hand truck (if available)

- Floor area marked as “stacking zone”

Instructions:

- Learners inspect and prepare a sample item for handover.

- Practise correct lifting and handling using gloves.

- Stack items safely using padding, labels, and clear spacing.

- Each learner explains their stacking method and reasoning.

Extension:
Introduce a timed relay in which learners must safely move and stack items while avoiding simulated obstacles, maintaining professional standards.

## Integrated Assessment – KM-03-KT07

Qualification: Furniture Upholsterer (SAQA ID 103199)
Knowledge Module: KM-03-KT07 – Upholstery Covering Principles
NQF Level: 2 | Credits: 8 | Weighting: 10%
Assessment Type: Variety of Instruments
Duration: 60 minutes
Total Marks: 40

🔹 Section A: Matching (5 marks)

Match the concept in Column A with its correct description in Column B.

| Column A | Column B |
| --- | --- |
| A. Pleating | 1. Ensures alignment of fabric design across adjacent sections |
| B. Pattern matching | 2. Decorative or functional pulling of fabric into uniform depth |
| C. Buttoning | 3. Aligning left and right frame features in appearance and tension |
| D. Fabric tension | 4. Creates smooth folds around curved areas |
| E. Symmetry of arms/wings | 5. Balance of stretch applied during covering |

🔹 Section B: Short Answer (3 marks each × 5 = 15 marks)

- Why is it important to centre and align the fabric before beginning the covering process?

- Explain how incorrect spacing of staples or nails affects product quality.

- List three visual features that must be symmetrical on a pair of chair arms or wings.

- Describe the risks of not preparing and stacking covered frames correctly.

- What must be checked before applying finishing aids or dispatching a covered item?

🔹 Section C: Scenario-Based Application (2 marks each × 5 = 10 marks)

Scenario:
Karabo is covering the outer back of a wingback chair using striped fabric. He forgets to mark the centre of the fabric and staples the cover from one edge across to the other. When finished, the stripes run slightly diagonally, and the pattern is misaligned with the arms.

- What key mistake did Karabo make?

- How could he have avoided the alignment issue?

- What customer reaction might this fault cause?

- If the chair needs to be recovered, what cost implications could arise?

- What tool or technique could have helped maintain proper alignment?

🔹 Section D: Practical Reasoning and Process Flow (2 marks each × 5 = 10 marks)

- What is the ideal spacing (in mm) for staples along a curved edge under tension?

- How do pleating and folding enhance both visual appeal and fabric fit?

- What is the function of chalk lines in the covering process?

- Name two precautions when handling covered frames before stacking.

- Why should pattern alignment be checked during both the cutting and covering stages?

✅ Model Answers and Marking Memo

Section A:
A–4, B–1, C–2, D–5, E–3

Section B Sample Answers:
6. To prevent twisting, uneven tension, and misalignment.
7. Loose areas or fabric bunching; visible irregularities.
8. Height, angle, and pleating or seam alignment.
9. Creasing, soiling, damage to fabric or frame structure.
10. Final inspection, label attached, routing confirmed.

Section C Sample Answers:
11. He did not mark and align the centre of the fabric.
12. Marked both fabric and frame centres before tacking.
13. Disappointment, returns, or perception of poor quality.
14. Labour and material waste due to rework.
15. Chalk line, centre pinning, or alignment jig.

Section D Sample Answers:
16. 10–20 mm depending on curve and fabric tension.
17. Allows smooth shaping of fabric and creates symmetry.
18. To guide placement, tension, and alignment of fabric.
19. Use gloves, avoid double-stacking or direct contact.
20. To ensure design elements match and avoid misalignment later.

📊 Assessment Rubric

| Criteria | Outstanding (4) | Competent (3) | Partially Achieved (1–2) | Not Achieved (0) |
| --- | --- | --- | --- | --- |
| Covering Principles | Demonstrates complete understanding and accurate use | General understanding with minor gaps | Limited explanation and incomplete examples | Cannot explain or identify key terms |
| Application of Knowledge | Clear application with appropriate examples | Mostly correct reasoning and linkage | Inconsistent or vague explanations | No understanding demonstrated |
| Process Insight | Reflects awareness of sequence and visual expectations | Shows process steps but lacks clarity | Misunderstands key stages | Process not understood |
| Communication and Clarity | Clear, concise responses in own words | Mostly clear, some technical errors | Poor structure or unclear terminology | Responses incomplete or irrelevant |

### Facilitator Assessment Briefing

Knowledge Module: KM-03-KT07 – Upholstery Covering Principles
Qualification: Furniture Upholsterer (SAQA ID 103199)
NQF Level: 2 | Credits: 8 | Weighting: 10%
Assessment Duration: 60 minutes
Assessment Type: Variety of Instruments
Total Marks: 40

🎯 Purpose of the Assessment

This assessment evaluates the learner’s understanding and ability to:

- Apply principles of tension, symmetry, pleating, folding, and buttoning in upholstery.

- Identify correct fastener spacing, alignment, and positioning of fabric.

- Recognise and resolve common covering errors.

- Understand how to prepare and stack covered frames correctly for the next step in production.

✅ Assessment Instrument Overview

| Section | Format | Marks |
| --- | --- | --- |
| A | Matching | 5 |
| B | Short Answer | 15 |
| C | Scenario-Based Application | 10 |
| D | Practical Reasoning & Process Flow | 10 |
| Total |  | 40 |

🛠 Facilitator Preparation Checklist

- Print and collate the assessment papers and model answers.

- Prepare fabric swatches or visual aids (optional).

- Ensure learners have writing materials and sufficient space.

- Brief learners on assessment format, time allocation, and rules.

- Confirm readiness for internal moderation if applicable.

📢 Instructions to Be Read to Learners

- This assessment focuses on your understanding of how to apply upholstery covering principles.

- Answer all questions clearly and in full sentences.

- Where a scenario is provided, apply your reasoning and knowledge to solve the problem.

- Use correct terminology where possible.

- You have 60 minutes to complete the assessment.

- You may raise your hand for clarification of the question wording — no answers will be given.

- This is an individual assessment. No communication with peers is permitted.

📊 Marking and Moderation Guidelines

- Use the model answers to ensure consistency and fairness.

- Award full marks only for complete and correct answers.

- Consider alternative wording if technically correct.

- Keep all assessment records for internal or external moderation.

## KM-03-KT08: Upholstered Furniture Covering Operations and Techniques

Weighting: 20% | NQF Level: 2

1. Purpose of the Knowledge Topic

The purpose of this Knowledge Topic is to provide learners with a comprehensive understanding of the practical operations and techniques involved in covering upholstered furniture. This includes the correct sequence of application, use of appropriate hand and power tools, and mastery of specialised skills such as tufting, buttoning, slip stitching, pleating, and the accurate positioning of decorative elements.

Learners will also develop the ability to measure, cut, attach, and shape fabric around a variety of frame types using both traditional and modern methods. A strong emphasis is placed on workplace procedures, product protection, and the identification and correction of faults that may affect the functionality or aesthetic quality of the final product.

2. Key Knowledge Areas (Topic Elements)

The following topic elements are covered in this Knowledge Topic:

- KT0801: Correct sequence and procedure

- KT0802: Slip stitching with circular needle and piping slip stitching

- KT0803: Buttoning

- KT0804: Tufting

- KT0805: Tacking, stapling, nailing

- KT0806: Pleating and folding fabric

- KT0807: Pulling, tensioning and stretching

- KT0808: Preventing tack ties

- KT0809: Shaping – edges and corners by using hand skills

- KT0810: Measuring – centralising the cover, spacing of buttons and studs, front borders of chairs

- KT0811: Attaching decorative trimmings

- KT0812: Cutting in of covers

- KT0813: Foam marking

3. Internal Assessment Criteria and Weighting

This Knowledge Topic carries a total weighting of 20% and will be assessed according to the following Internal Assessment Criteria:

- IAC0801: Upholstery covering techniques are defined and the application thereof is described

- IAC0802: Using the correct terminology, explain the difference in the sequence of fitting covers to a drop-on and a drop-in dining chair seat

- IAC0803: Using the correct terminology, explain the difference in the sequence of fitting covers to a fully covered and a show wood chair

- IAC0804: Using the correct terminology, explain the difference in the sequence of fitting covers to various prepared frames, including a round pouffe with padded sides, fully upholstered sofa, dining room chair with upholstered back, and an arm chair with a loose seat cushion

- IAC0805: Explain the principles and concepts of stretching techniques when using hand tools, power tools, and tacking strips to attach covers to the frame

- IAC0806: Explain the principles and concepts of blind stitching and blind tacking when attaching covers to the frame

- IAC0807: Identify covering faults in covering materials, consumables and techniques and explain the causes and how to rectify and prevent them

- IAC0808: Explain the principles and concepts of measuring, cutting and attaching the specified fabric to cover the foam and paddings

- IAC0809: Identify the principles of following workplace procedures to handle, stack and store completed furniture for the next procedure and explain the types and consequences of damage to the product

4. Application in Upholstered Furniture Manufacturing

This topic prepares learners to operate confidently within a production environment, applying specialised skills in the correct order, and adhering to workplace safety and quality standards. Whether producing a drop-in seat or a fully upholstered sofa, the learner must understand how to adapt techniques to suit frame types, fabric characteristics, and design specifications. Proper execution of these techniques contributes directly to the functionality, longevity, and visual appeal of upholstered furniture in commercial and domestic markets.

### KT0801: Correct Sequence and Procedure

Purpose of the Topic

This topic introduces learners to the correct sequence and procedures followed when covering various types of upholstered furniture frames. Each type of product—whether a drop-on seat, a show wood chair, or a fully upholstered sofa—requires a logical and structured series of steps to ensure high-quality outcomes.

Learners will be guided through the systematic order in which covering operations must occur, including surface preparation, placement of padding, attachment of fabric, trimming, and finishing. Following the correct sequence prevents errors, supports time management, and promotes uniform results across production batches.

Key Content Areas

- Why Sequence Matters

  - Ensures materials are applied in a logical and efficient manner.

  - Minimises handling of completed sections, reducing the risk of damage or dirt.

  - Enables consistent tension, symmetry, and finish.

  - Supports troubleshooting if problems arise.

- Typical Upholstery Covering Sequence

  - Step 1: Review specification and confirm measurements.

  - Step 2: Prepare and inspect the frame.

  - Step 3: Apply foam and padding as required.

  - Step 4: Align and centre the fabric.

  - Step 5: Start tacking or stapling from centre outward.

  - Step 6: Complete detailing such as buttoning or pleating.

  - Step 7: Trim excess fabric and finish edges.

  - Step 8: Apply decorative trims (if specified).

  - Step 9: Final check and prepare for stacking or dispatch.

- Adaptation by Product Type

  - A drop-in seat may follow a shorter, simpler sequence than a fully upholstered sofa.

  - Items with exposed wood elements (e.g. show wood chairs) require additional care to avoid damage.

  - Multi-part products (e.g. a sofa with loose cushions) must be sequenced for individual components and then assembled correctly.

- Consequences of Incorrect Sequencing

  - Poor finish or wrinkling of fabric.

  - Fabric misalignment or tension imbalance.

  - Damage to previously completed areas due to unnecessary handling.

  - Lost time and increased material wastage due to rework.

Example Table: Comparison of Covering Procedures

| Furniture Type | Key Sequence Consideration | Unique Step |
| --- | --- | --- |
| Drop-in seat | Start with webbing/foam, then fabric | Secure from centre and fold edges under neatly |
| Show wood dining chair | Protect wood surface, cover backrest first | Mask wood edges during covering |
| Armchair with loose seat cushion | Cover frame first, then fit and align loose cushion | Ensure cushion matches pattern and tension |
| Round pouffe with padded sides | Cover circular top first, then wrap side evenly | Apply fabric evenly around circumference |

Case Study: Out-of-Order Steps at Hlatshwayo Upholstery

Background:
At Hlatshwayo Upholstery in Pretoria, Sipho began covering a series of show wood chairs by securing the seat fabric first. He realised too late that the inside back panel was now difficult to access due to the angle and tension of the seat cover.

Outcome:

- Fabric had to be removed and reapplied.

- Time was lost, and materials were wasted.

- The team was retrained on proper covering sequences based on product type.

Learning Point:
Starting at the wrong point can make the rest of the job more difficult or even impossible to complete to standard.

Critical Thinking Questions

- Why does the sequence of covering steps differ between a drop-in seat and a fully upholstered chair?

- What risks are introduced if foam or padding is not applied before the fabric?

- How does following a correct sequence save time and reduce waste?

- Why must a worker consult the specification sheet before starting the covering process?

- What are the possible consequences of reversing steps in the covering process?

Practical Classroom Activity

Task: Sequence Planning and Simulation

Materials:

- Step cards or flashcards showing covering operations (mixed up)

- Sample product descriptions (e.g. dining chair, armchair, pouffe)

Instructions:

- Learners receive a product scenario and a set of step cards.

- They must organise the cards into the correct covering sequence.

- Present and justify the order to the class.

- Facilitator leads a reflection on how incorrect steps could affect the outcome.

Extension:
Learners write their own covering sequence for a sample item, including unique product-specific adaptations.

### KT0802: Slip Stitching with Circular Needle and Piping Slip Stitching

Purpose of the Topic

This topic introduces learners to the technique of slip stitching used in upholstery to secure fabric edges and finish coverings with minimal visibility of the stitches. It focuses specifically on the use of a circular needle and on piping slip stitching, which is used to attach or reinforce decorative piping or welting along seams and edges.

Slip stitching is commonly used where a clean, hand-finished appearance is required, such as in hidden seams, cushion closures, or final detailing on high-end upholstered furniture.

Key Content Areas

- What Is Slip Stitching?

  - A hand-sewing method that creates a nearly invisible seam by anchoring fabric edges together.

  - Typically used where machine stitching is not practical or visible stitching is undesirable.

- Slip Stitching with a Circular Needle

  - Circular needles are curved hand needles that allow stitching into tight or awkward spaces.

  - Useful for joining internal seams or applying fabric inside corners or curves.

  - Requires small, even stitches that catch the inner fold of both fabric layers.

- Piping Slip Stitching

  - Involves sewing fabric tightly against pre-made piping or corded welting.

  - Stitching follows the edge of the piping, securing it cleanly to the frame or cushion.

  - Often used to finish off edges, borders, or as a decorative join between panels.

- Application and Technique

  - Use waxed upholstery thread or strong polyester for durability.

  - Stitches should be tight but not pulling, to avoid puckering.

  - Secure thread ends firmly and bury knots inside the fabric to keep them hidden.

- When to Use Slip Stitching

  - To close hand-stuffed cushions.

  - To attach facing fabric over stapled seams or tacking points.

  - To finish piping along curves, inside corners, or on visible outer seams.

Example Table: Comparison of Stitching Techniques

| Stitching Technique | Tool Used | Application Area | Key Benefit |
| --- | --- | --- | --- |
| Slip stitch (straight) | Straight upholstery needle | Closing cushion seams, hemming fabric edges | Invisible finish on straight seams |
| Slip stitch (circular needle) | Curved needle | Inner corners, rounded edges, fixed seat covers | Access to tight or recessed areas |
| Piping slip stitching | Curved or straight needle | Along piping lines, corners, cushions | Neat join between fabric and decorative piping |

Case Study: Visible Seams at Ekaya Upholstery

Background:
At Ekaya Upholstery in Mahikeng, a client returned two ottomans, complaining that the seams around the piping were bumpy and visible. Investigation revealed that the final hand stitching had been done using a straight needle, resulting in awkward angles and visible thread along the curves.

Outcome:

- Items were re-stitched using a circular needle for tighter, neater finishing.

- The team was retrained on slip stitching with correct tools for different product types.

- A sample stitching standard board was introduced for reference.

Learning Point:
Using the correct needle and method is essential when working with detailed finishes like piping.

Critical Thinking Questions

- Why is a circular needle better than a straight needle in certain upholstery applications?

- What risks are involved if slip stitching is not secured properly?

- How does slip stitching contribute to the overall appearance of upholstered furniture?

- In what scenarios would piping slip stitching be preferred over stapling?

- What might happen if tension is applied unevenly when slip stitching along piping?

Practical Classroom Activity

Task: Slip Stitching and Piping Practice

Materials:

- Fabric swatches, foam blocks or cushions

- Circular needles, straight upholstery needles

- Piping cord, thread, scissors

Instructions:

- Learners practise both regular and piping slip stitching on prepared fabric samples.

- First attempt the stitch on a straight edge using a straight needle.

- Then perform the same on a curve or piping with a circular needle.

- Compare neatness, accessibility, and finish between the two methods.

Extension:
Ask learners to create a small sample board with a labelled example of each stitching method for reference and evaluation.

### KT0803: Buttoning

Purpose of the Topic

This topic introduces learners to the technique of buttoning, a specialised upholstery skill used to attach fabric to padded surfaces in a way that is both decorative and functional. Buttoning secures layers of fabric and padding while creating distinct visual patterns such as tufting, often seen in high-end or traditional furniture.

Learners will understand the types of buttoning, the tools required, and the step-by-step process for executing neat, symmetrical, and secure buttoning layouts across various types of furniture frames.

Key Content Areas

- Purpose and Function of Buttoning

  - Provides both aesthetic value and structural function.

  - Keeps padding in place and helps maintain shape and comfort.

  - Common in headboards, sofas, ottomans, and traditional armchairs.

- Types of Buttoning

  - Shallow buttoning: Decorative, with slight indentation.

  - Deep buttoning (tufting): Creates diamond or square patterns; pulls fabric deep into the padding.

  - Single-line buttoning: Found on backrests or cushions.

- Buttoning Tools and Materials

  - Upholstery buttons (covered or decorative).

  - Buttoning twine, tufting needles, or buttoning wire.

  - Measuring tools (e.g. chalk, ruler, template) for even spacing.

  - Foam or padding, pre-marked with layout grid.

- Steps in Buttoning Process

  - Step 1: Mark layout grid (e.g. diamond or square) on the fabric and foam.

  - Step 2: Drill holes in the frame (for deep buttoning).

  - Step 3: Thread the button using twine or wire.

  - Step 4: Pull through the foam and tie off securely at the back of the frame.

  - Step 5: Adjust tension to create consistent depth and form.

Example Table: Buttoning Layouts

| Buttoning Type | Visual Effect | Application Area |
| --- | --- | --- |
| Single-line | Straight row across a panel | Dining chair backs, cushions |
| Diamond tufting | Classic and deep, luxury feel | Headboards, ottomans, traditional chairs |
| Square tufting | Modern and structured look | Sofas, modular seating |

Case Study: Uneven Buttoning at Reba Furniture

Background:
At Reba Furniture in Bloemfontein, a learner upholsterer buttoned a set of headboards using a diamond tufting pattern. The first row of buttons was 30 mm off-centre, and some buttons were sunk deeper than others. The result was a misaligned, uneven surface.

Outcome:

- The batch had to be re-buttoned using a template grid and depth guide.

- The workshop implemented a buttoning checklist, requiring a peer check before final fixing.

Learning Point:
Accurate measurement, alignment, and tension are critical to achieve professional-looking buttoning.

Critical Thinking Questions

- What is the functional benefit of deep buttoning beyond visual design?

- How does incorrect spacing of buttons affect the furniture’s appearance?

- What are the risks of uneven button depth?

- Why is it important to mark a layout before starting the buttoning process?

- What tools or techniques could help maintain equal tension in each button?

Practical Classroom Activity

Task: Buttoning Pattern Layout and Application

Materials:

- Padded boards or foam blocks with pre-attached fabric

- Buttons, buttoning twine or wire, tufting needles

- Rulers, chalk, templates for square and diamond layouts

Instructions:

- Learners begin by marking a layout grid on fabric.

- Practice threading and securing buttons at even spacing and consistent depth.

- Each learner compares their buttoning pattern with a peer and discusses areas for improvement.

- Facilitator demonstrates how to adjust tension to create uniform buttoning.

Extension:
Challenge learners to re-mark and re-button a misaligned sample using only measurement and inspection techniques.

### KT0804: Tufting

Purpose of the Topic

This topic introduces learners to the technique of tufting, a classic upholstery method that involves pulling fabric down into the padding and securing it in a patterned layout, typically using buttons or stitching. Tufting adds depth, texture, and elegance to upholstered furniture and also contributes to shape retention and durability.

By the end of this session, learners will be able to describe the different types of tufting, understand the layout and preparation process, and demonstrate how to tuft evenly and securely using appropriate tools and materials.

Key Content Areas

- What Is Tufting?

  - Tufting is the process of creating indentations or folds in upholstered fabric by pulling it down into padding and securing it at set intervals, often with buttons.

  - Common in luxury, traditional, and feature furniture (e.g. headboards, ottomans, chaise lounges).

- Types of Tufting

  - Diamond tufting – most traditional, creates a quilted look.

  - Square tufting – modern and structured, often found on minimalist designs.

  - Blind tufting – tufting without visible buttons.

  - Button tufting – adds decorative buttons at each tuft point.

- Tufting Process and Preparation

  - Step 1: Mark a precise tufting grid on both the foam and fabric.

  - Step 2: Drill holes through the baseboard (if required for deep tufting).

  - Step 3: Thread buttons or fasteners with twine or strong thread.

  - Step 4: Pull through foam and secure at the back of the frame.

  - Step 5: Form consistent folds or pleats between tuft points.

- Key Tufting Considerations

  - Use high-density foam that can support tufting without tearing.

  - Tufting should be symmetrical, with evenly spaced rows and consistent depth.

  - Pleats or folds should form naturally between points to enhance the visual effect.

Example Table: Tufting Types and Application

| Tufting Style | Appearance | Common Applications |
| --- | --- | --- |
| Diamond Tufting | Traditional, curved folds | Chesterfield sofas, classic headboards |
| Square Tufting | Straight lines, modern look | Modular ottomans, sleek backrests |
| Button Tufting | Adds texture and decoration | Accent chairs, luxury cushions |
| Blind Tufting | Subtle indents, no button | Contemporary furniture, minimalist sets |

Case Study: Collapsed Tufting at Tshiamo Upholstery

Background:
Tshiamo Upholstery in Rustenburg completed a tufted ottoman using square tufting. After three weeks of use, several tuft points flattened out, and pleats were no longer visible.

Investigation:
The team had used low-density foam and did not drill holes for button twine to anchor into the base. Additionally, no tension checks were performed during tufting.

Outcome:

- The foam was replaced and re-tufted using correct methods.

- A foam selection and tufting checklist was introduced.

- Learners received a refresher workshop on tufting grid marking and tension control.

Learning Point:
Tufting requires precision, preparation, and proper material selection to ensure durability and visual appeal.

Critical Thinking Questions

- Why is a marked tufting grid essential before beginning the tufting process?

- What could happen if tufting is attempted on foam that is too soft?

- How does tufting contribute to the comfort and shape retention of upholstered furniture?

- Why is it important to maintain equal spacing and depth during tufting?

- How would you decide whether to use buttons in a tufted design?

Practical Classroom Activity

Task: Tufting Grid and Technique Demonstration

Materials:

- Foam blocks with backing board

- Upholstery fabric, chalk, buttons, twine or thread

- Tufting needles, rulers, staplers

Instructions:

- Learners draw a diamond or square tufting grid on sample fabric and foam.

- Thread buttons and practise pulling through and securing tuft points.

- Adjust pleats between points to form clean lines or folds.

- Compare results in groups and identify how to correct uneven tufting.

Extension:
Provide a sample of incorrect tufting and challenge learners to diagnose the problem and propose a step-by-step fix.

### KT0805: Tacking, Stapling, Nailing

Purpose of the Topic

This topic introduces learners to the core upholstery techniques of tacking, stapling, and nailing, which are essential for securing fabric, padding, and decorative elements to furniture frames. Each fastening method has specific tools, spacing requirements, and safety considerations that must be followed to ensure a secure and aesthetically pleasing result.

Learners will understand when to use each method, how to apply them correctly, and how poor fastening can affect the appearance, durability, and safety of the upholstered product.

Key Content Areas

- Tacking

  - Traditionally involves hammering upholstery tacks into wooden frames.

  - Still used in heritage furniture restoration and where staple guns are unsuitable.

  - Requires precision and uniform spacing.

  - Often replaced with modern stapling in mass production.

- Stapling

  - Most common method in upholstery workshops.

  - Uses pneumatic or manual staple guns to quickly and securely attach materials.

  - Staple spacing should typically range between 10 mm and 50 mm, depending on fabric tension and curve.

  - Staples should be driven flush without tearing the fabric.

- Nailing

  - Includes both decorative nails (nailhead trim) and functional nails.

  - Used to attach fabrics where stapling is not appropriate or for visual effect.

  - Decorative nails require consistent spacing and gentle hammering with a magnetic or domed hammer to avoid damage.

- Application Tips

  - Start fastening from the centre and work outward to ensure even fabric tension.

  - Use edge folding techniques to prevent fraying under fasteners.

  - Always inspect tools for wear and check alignment for accurate application.

  - Maintain consistent pressure for stapling and nailing to prevent visible indentations or misfires.

Example Table: Fastening Methods and Applications

| Method | Tool Required | Application Area | Best Used For |
| --- | --- | --- | --- |
| Tacking | Upholstery hammer, tacks | Traditional frame joints, inner corners | Heritage pieces or fine detailing |
| Stapling | Pneumatic or manual staple gun | General upholstery, arms, seats | Speed, strong hold, curved or padded areas |
| Nailing | Nail set, upholstery nails | Decorative borders, headboards | Decorative finishing, high-end visual detailing |

Case Study: Misapplied Fasteners at Kwezi Upholstery

Background:
At Kwezi Upholstery in Gqeberha, Thuli used a staple gun to cover the inside arm of a curved chair. She placed the staples too far apart and pulled the fabric unevenly. Over time, customers reported that the cover started to sag and loosen, especially around the inside curve.

Outcome:

- The item was returned for rework, requiring fabric replacement.

- The facilitator used this example in training on spacing and tension techniques.

- Learners now use spacing templates and practice edge-folding for staple placement.

Learning Point:
Inconsistent or improper fastening reduces both functional quality and aesthetic value of upholstery.

Critical Thinking Questions

- Why is stapling generally preferred over tacking in modern upholstery workshops?

- What could happen if decorative nails are not spaced evenly?

- When might a learner choose tacking over stapling?

- What are the key safety concerns when using pneumatic staple guns?

- How does the spacing of fasteners relate to the type of fabric being applied?

Practical Classroom Activity

Task: Fastening Practice with Staple and Nail Tools

Materials:

- Scrap wood boards, various fabrics, foam offcuts

- Manual and pneumatic staplers, tacking hammers, decorative nails

Instructions:

- Learners practise stapling at correct intervals on straight and curved edges.

- Create a decorative nail line using a template for even spacing.

- Compare visual finish and holding strength across fastening types.

- Evaluate safety handling and correct tool use through peer feedback.

Extension:
Introduce a time challenge: secure a square panel using correct spacing, folding, and finish—then peer-assess for neatness and consistency.

### KT0806: Pleating and Folding Fabric

Purpose of the Topic

This topic introduces learners to the techniques of pleating and folding fabric during the upholstery process. These techniques are used to manage excess material, create aesthetic details, and ensure that fabric fits smoothly around curved surfaces, corners, and recesses.

Proper pleating and folding enhance the visual appeal, symmetry, and professional finish of upholstered furniture. Learners will practise how to create even, clean folds, and understand where and when to apply these methods.

Key Content Areas

- What Is Pleating?

  - A technique used to fold and secure fabric to manage bulk or create decorative effects.

  - Common on rolled arms, outer corners, skirted bases, and classic furniture styles.

  - Pleats can be symmetrical (e.g. box pleats) or directional (e.g. knife pleats).

- What Is Folding?

  - The process of turning under raw fabric edges to hide staples or seams and create a clean finish.

  - Often applied at corners, borders, and transition points (e.g. seat to arm, cushion to base).

  - Involves double-folding and tacking down with precision.

- Techniques and Best Practices

  - Pleats should be evenly spaced and mirrored on left and right sides.

  - Use chalk lines or pins to mark pleat positions before securing.

  - Folding should be tight but smooth, avoiding puckering or bunching.

  - Secure pleats and folds with temporary tacks first to check appearance before final fastening.

- Tools Used

  - Chalk, pins, stapler, tacking hammer, creasing tool or folding bone.

  - Templates can assist in consistent pleat size and alignment.

Example Table: Comparison of Pleating and Folding Techniques

| Technique | Function | Application Area |
| --- | --- | --- |
| Knife Pleats | Create clean, directional folds | Outer arms, base skirts |
| Box Pleats | Balanced folds for symmetry | Front panels, back skirts |
| Simple Folding | Clean finish at frame edges | Front borders, seams |
| Corner Folding | Tucks excess fabric smoothly at corners | Inside and outside back corners |

Case Study: Uneven Pleating at Dineo Upholstery

Background:
At Dineo Upholstery in Johannesburg, Nathi was assigned to pleat the rolled arms of a vintage sofa. He completed the right side with six even pleats but folded the left side with only four, using a different pleat size.

Outcome:

- The mismatch was immediately visible and the arm had to be reupholstered.

- Nathi was trained to use mirror marking and pleating templates.

- The team developed a standard visual check for symmetrical pleating before final fixing.

Learning Point:
Visual symmetry is critical—pleats must match in spacing, size, and direction across all sides.

Critical Thinking Questions

- What is the visual and functional purpose of pleating in upholstery?

- How can uneven pleats affect the appearance of upholstered arms or panels?

- Why should folds be made before securing fabric with staples or tacks?

- What tools can help maintain consistency in pleat size?

- In what situations might folding be more appropriate than pleating?

Practical Classroom Activity

Task: Pleating and Folding Demonstration and Replication

Materials:

- Fabric offcuts, small upholstered forms or foam corners

- Pins, chalk, staplers, tacking hammers, template guides

Instructions:

- Learners practise forming pleats on both flat and curved surfaces using chalk lines.

- Create mirror-image pleats on left and right sides of a mock arm or panel.

- Practise folding fabric along frame borders for clean finishes.

- Review each other’s work for symmetry, neatness, and tension.

Extension:
Provide fabric with visible grain or patterns and ask learners to align folds with design features for a premium finish.

### KT0807: Pulling, Tensioning and Stretching

Purpose of the Topic

This topic equips learners with the knowledge and practical understanding of how to pull, tension, and stretch upholstery fabric correctly during the covering process. These techniques are critical to ensure that the fabric lies smoothly, evenly, and securely over the padding and frame.

Incorrect pulling or uneven tension may result in wrinkling, puckering, sagging, or frame distortion. Learners will gain insight into the balance required between fabric behaviour, tension control, and structural alignment.

Key Content Areas

- What Is Pulling, Tensioning and Stretching?

  - Pulling: Manually guiding fabric over the frame or foam to create initial shape.

  - Tensioning: Applying force to maintain firmness and alignment of the cover.

  - Stretching: Carefully extending the fabric so that it lies flat, tight, and wrinkle-free.

- Why These Techniques Are Important

  - Ensures aesthetic quality: no puckers or sagging.

  - Supports functional durability: prevents movement or wear over time.

  - Maintains symmetry and alignment across matching parts.

- Fabric-Specific Considerations

  - Different fabrics (e.g. woven, vinyl, leather) have varying elasticity and behaviour.

  - Stretch direction (warp and weft) affects how the material is tensioned.

  - Overstretching can distort patterns or damage the material.

- Practical Tips for Applying Tension

  - Always pull from centre outward, alternating sides for balance.

  - Use tensioning tools or grips if hands alone are not sufficient.

  - Check for even pressure along edges and corners.

  - Temporarily secure fabric with a few staples or tacks, inspect, and adjust before final fastening.

Example Table: Tensioning Challenges and Solutions

| Challenge | Cause | Solution |
| --- | --- | --- |
| Wrinkles along outer panel | Uneven tension | Re-stretch from centre, alternate directions |
| Fabric pulls loose over time | Under-tensioning or weak fasteners | Increase staple density and pull tighter |
| Distorted pattern | Over-pulling in one direction | Re-check alignment and stretch evenly |
| Staple pulls through fabric | Fabric too delicate or over-tensioned | Reduce force, use backing or tape reinforcement |

Case Study: Over-Tensioning at Motsamai Custom Upholstery

Background:

At Motsamai Custom Upholstery in Mbombela, a team member covered a curved headboard using a tightly woven linen fabric. In an effort to achieve a smooth finish, he applied excessive tension, particularly around the top corners.

Result:

- The fabric tore at the staple points, and the curve appeared pinched and uneven.

- The client rejected the unit, citing a lack of finish quality.

Corrective Action:

- Re-training on tension balance using low-stretch fabrics.

- Staff introduced fabric tension tests before starting the covering process.

Learning Point:
Tension must be even, controlled, and suitable to the fabric type—more is not always better.

Critical Thinking Questions

- Why is it important to apply tension from the centre outward during the covering process?

- What signs can you look for to confirm that tension is even across the fabric?

- How can the type of fabric influence how much tension should be applied?

- What are the consequences of over-tensioning vs under-tensioning?

- Why is it necessary to inspect fabric alignment before final stapling?

Practical Classroom Activity

Task: Tensioning and Pulling Technique Workshop

Materials:

- Foam blocks, fabric swatches of different types

- Staple boards or small wooden frames

- Tensioning tools, chalk, and staplers

Instructions:

- Learners practise pulling and tensioning different fabrics onto mock panels.

- Apply centre-outward tension, secure temporarily, and evaluate for evenness.

- Compare effects of over-pulling and under-tensioning.

- Document how different fabric types respond to tension and how they must be handled.

Extension:
Each learner presents one panel with correct tension and one with a simulated error—peers must identify and explain the issue and suggest improvements.

### KT0808: Preventing Tack Ties

Purpose of the Topic

This topic introduces learners to the concept of tack ties—unintended and visible indentations, tension lines, or fabric distortions that occur when fasteners such as tacks, staples, or nails are applied incorrectly during the upholstery covering process.

Tack ties affect both the visual finish and structural integrity of the product. Learners will explore the causes, preventative methods, and corrective techniques to ensure a clean, professional covering outcome.

Key Content Areas

- What Are Tack Ties?

  - Unwanted pull lines, dimples, or tension distortions that appear on fabric surfaces after tacking, stapling, or nailing.

  - Typically occur when fasteners are applied with uneven tension, misaligned fabric, or excessive pulling.

- Common Causes

  - Fastening fabric while under unbalanced tension.

  - Applying fasteners too close to the fabric edge.

  - Skipping pre-inspection of fabric placement before final tacking.

  - Inadequate temporary fastening before committing to final positions.

- Impact of Tack Ties

  - Reduces aesthetic value—creases, puckers, or stretch marks are visible.

  - May compromise fabric durability due to strain at fastening points.

  - Can result in product rejection, rework, or client dissatisfaction.

- Prevention Techniques

  - Pre-tension evenly from centre outward.

  - Use temporary tacks to test placement before finalising.

  - Fold and smooth fabric edges before fastening.

  - Fasten with a consistent rhythm and spacing, avoiding over-pulling.

  - For delicate fabrics, use backing cloth or tape for reinforcement.

Example Table: Tack Tie Prevention Techniques

| Problem Identified | Preventative Action |
| --- | --- |
| Fabric dimpled around tacks | Apply even tension and avoid overstretching |
| Edge puckering | Fold fabric edge and staple with smooth motion |
| Visible diagonal tension lines | Balance pulls on opposite sides before securing |
| Pull-through on lightweight fabric | Use backing tape or double-fold the fabric edge |

Case Study: Tack Tie Troubles at Bapela Upholstery

Background:
At Bapela Upholstery in Polokwane, a set of bar stools upholstered in a light vinyl arrived at quality control with visible puckering around the base. The staples had been applied unevenly, and the material had pulled tightly from two anchor points.

Outcome:

- The vinyl covers had to be removed and replaced.

- Learners were taught to apply pre-staple inspections, use chalk lines, and test for smoothness before tacking.

- A new checklist was introduced for final visual inspection post-tacking.

Learning Point:
The quality of finish begins with proper preparation and even fastening. Prevention of tack ties protects both product appearance and workshop efficiency.

Critical Thinking Questions

- What are tack ties, and how can they affect a finished upholstered product?

- Why is it important to apply fasteners from the centre outward when securing fabric?

- What preventative steps can a worker take before placing permanent tacks or staples?

- How do different fabric types influence the likelihood of tack ties?

- What could happen if tack ties are not identified and corrected before delivery?

Practical Classroom Activity

Task: Tack Tie Identification and Correction Exercise

Materials:

- Fabric-covered sample boards (some intentionally tacked with visible tack ties)

- Staplers, tacking tools, fabric offcuts

- Chalk, measuring tools

Instructions:

- Learners examine provided samples to identify visible tack ties.

- Mark the location and explain the probable cause.

- Practise removing, repositioning, and re-securing fabric using the correct method.

- Before and after comparisons are made and discussed in groups.

Extension:
Ask learners to demonstrate a tack tie-free finish using a lightweight fabric and curved surface, under facilitator observation.

### KT0809: Shaping – Edges and Corners by Using Hand Skills

Purpose of the Topic

This topic focuses on the manual techniques required to shape fabric neatly around edges and corners of upholstered furniture using hand skills. These areas demand precision, control, and practice to ensure the final product has a smooth, symmetrical, and professional appearance.

Learners will explore how different types of corners—inside, outside, rounded, square—require unique folding and tensioning techniques. They will also understand how to use hand tools effectively to tuck, pinch, fold, and smooth fabric into complex shapes without creating bulk, wrinkles, or inconsistencies.

Key Content Areas

- Types of Corners and Edges

  - Inside corners: typically found where the back meets the seat or inside arms.

  - Outside corners: commonly seen on box cushions and square frames.

  - Rounded corners: require stretching and pleating simultaneously.

  - Sharp edges: need folding and tensioning without distortion.

- Techniques for Shaping

  - Use fingertips, folding tools, and smooth tensioning to shape fabric around transitions.

  - Fold under excess material into a triangle tuck or waterfall fold.

  - Use temporary tacks before finalising placement.

  - Avoid bulk by trimming excess fabric (with caution) and forming even folds.

- Tools Used

  - Upholstery hammer, mallet, regulator, folding spatula or creasing tool.

  - Chalk or pencils for marking edges.

  - Safety scissors or duckbill shears for trimming fabric in tight corners.

- Fabric-Specific Considerations

  - Thicker fabrics like leather or vinyl may require pre-softening or heat treatment.

  - Lightweight fabrics may need reinforcement to avoid fraying when folded tightly.

  - Patterned or striped fabrics must be aligned precisely at the edge.

Example Table: Common Corner Types and Techniques

| Corner Type | Technique | Common Application |
| --- | --- | --- |
| Inside corner | Fold-in and tuck under, smooth evenly | Armchair back corners |
| Outside corner | Triangle fold and secure neatly | Ottoman top corners |
| Rounded corner | Pleat gently around curve | Seat cushions, padded tops |
| Box edge | Double fold with edge tap | Storage benches, modern frames |

Case Study: Shaping Inconsistencies at Everly Design Studio

Background:
At Everly Design Studio in Paarl, a team of apprentices including Lerato, Stefanus, and Raeesa worked on a bespoke collection of corner chairs. Each learner was assigned a corner to shape on the same unit. Upon inspection, the trainer noticed that the edges varied in size, fold style, and smoothness—some had sharp, elegant folds, while others were bulky or uneven.

Outcome:

- The team reviewed each corner and discussed what techniques had been used.

- They practised shaping on identical mock corners using the same tools and fabric.

- A visual reference guide with photographs of correct folds was added to the workshop wall.

Learning Point:
Shaping edges and corners requires consistency and a system—repetition alone is not enough without guidance and self-checking.

Critical Thinking Questions

- What makes corners and edges more challenging to upholster than flat surfaces?

- How can excess bulk at a corner be reduced without cutting too much fabric?

- Why is it important to match the fold technique across all corners of a product?

- What risks could arise from poorly shaped edges or misaligned corner folds?

- How would you approach shaping a rounded corner using a thick upholstery fabric?

Practical Classroom Activity

Task: Edge and Corner Shaping Drill

Materials:

- Corner frame mock-ups with inside, outside, and rounded shapes

- Fabric offcuts (in varying thicknesses)

- Folding tools, hammers, chalk, scissors

Instructions:

- Each learner shapes three different corner types using correct folding and tacking techniques.

- Compare each fold for neatness, smoothness, and alignment.

- Document which techniques worked best for each type and reflect on common errors.

- Peer evaluation to reinforce visual standards.

Extension:
Set up a “corner challenge” where learners shape all four corners of a sample in under 20 minutes and are scored on neatness, symmetry, and finish.

### KT0810: Measuring – Centralising the Cover, Spacing of Buttons and Studs, Front Borders of Chairs

Purpose of the Topic

This topic introduces learners to essential measuring skills used during the upholstery process to ensure that fabric placement, buttoning, and detailing are symmetrical, neat, and aligned with design specifications. Centralising the cover, calculating equal spacing between studs and buttons, and measuring across the front border of chairs are critical for producing furniture that is both aesthetically pleasing and functionally balanced.

Learners will develop confidence in applying measuring tools, marking methods, and layout techniques to achieve precision and uniformity throughout their upholstery work.

Key Content Areas

- Centralising the Cover

  - Identify the centre of the frame and fabric before starting.

  - Use chalk lines or fold marks to guide alignment.

  - Begin tacking from the centre outward to maintain balance.

  - Prevents distortion of patterns and uneven seams.

- Spacing of Buttons and Studs

  - Use rulers, templates, or marking strings to calculate even spacing.

  - Spacing should be symmetrical, consistent, and based on product design.

  - Button and stud placement affects both appearance and comfort.

  - Precision is essential in tufted and trimmed designs.

- Measuring Front Borders of Chairs

  - Accurate measuring prevents skewed upholstery and uneven corners.

  - Ensure alignment with internal framing and legs.

  - Mark both vertical and horizontal reference points to guide tension and folding.

  - Common area for visual inspection and client scrutiny.

Example Table: Upholstery Measurement Applications

| Measurement Area | Tools Used | Purpose |
| --- | --- | --- |
| Centre of frame/fabric | Measuring tape, chalk | Aligns fabric with frame for symmetry |
| Button or stud spacing | Ruler, paper template | Ensures even layout and consistent depth |
| Front border alignment | Tape measure, square ruler | Guides fold placement and visual balance |

Case Study: Inconsistent Spacing at Bester & Co.

Background:
At Bester & Co. Upholsterers in Stellenbosch, a team led by Thuli, Johan, and Anusha produced a set of six headboards featuring button rows and studded borders. Upon inspection, Johan’s button placement was neat and symmetrical, but Anusha’s row had inconsistent spacing between buttons, and Thuli’s studs followed a slightly tilted line across the front.

Outcome:

- Only Johan’s headboards passed quality control.

- The team was retrained on using button templates and measuring tools before final fastening.

- A measuring checklist was introduced, and peer-checks were added to the final inspection step.

Learning Point:
Visual details like button rows and border alignment are easily noticed by clients—accuracy in measuring is vital for workshop reputation.

Critical Thinking Questions

- Why should centralising the fabric be the first step before tacking or buttoning?

- What are the consequences of uneven spacing between buttons or studs?

- How can measuring tools assist in reducing upholstery errors?

- Why is the front border of a chair considered a focal point in design?

- What steps can be taken to ensure measuring remains accurate throughout a long production run?

Practical Classroom Activity

Task: Measuring and Marking Demonstration

Materials:

- Upholstered sample boards or foam panels

- Tape measures, chalk, rulers, templates

- Buttons, studs, marking pins

Instructions:

- Learners identify and mark the centre of both fabric and frame.

- Lay out a button or stud grid using measuring and spacing tools.

- Secure markings with pins and verify symmetry before fastening.

- Evaluate peer work for accuracy and visual consistency.

Extension:
Learners create their own reusable button spacing template using card or paper to assist with future layout tasks.

### KT0811: Attaching Decorative Trimmings

Purpose of the Topic

This topic introduces learners to the process of attaching decorative trimmings to upholstered furniture. These trimmings include items such as piping, braiding, nailhead trim, fringe, and other embellishments used to enhance the appearance and style of the furniture piece.

Learners will understand the types of trimmings used in different design contexts, how to apply them neatly and securely, and the importance of aligning them to design specifications to meet aesthetic and functional standards.

Key Content Areas

- Types of Decorative Trimmings

  - Double piping or single piping

  - Braiding, gimp braid, cord

  - Nailhead trim (decorative studs)

  - Fringe, tassels, fabric borders

  - Contrasting welts or inlays

- Purpose and Application

  - Enhances the visual detail and contour of a piece.

  - Conceals seams, tacking lines, or transitions.

  - Provides stylistic definition (e.g. traditional vs contemporary finishes).

  - Reinforces branding or design themes in commercial upholstery.

- Techniques for Attachment

  - Glue application (e.g. for braiding and fringe).

  - Sewing (e.g. piping inserted into seams or stitched around edges).

  - Nailing or stapling (e.g. decorative studs placed evenly along borders).

  - Use of templates, guides, and chalk lines to maintain straightness and symmetry.

- Best Practices

  - Always test the trimming for colourfastness and fray risk.

  - Secure ends neatly by folding under or tucking.

  - Begin and end at discreet points (e.g. under chair arms or behind legs).

  - Double-check alignment before final fastening.

Example Table: Trimming Types and Usage

| Trimming Type | Attachment Method | Common Application |
| --- | --- | --- |
| Piping | Sewn or slip-stitched | Seat cushions, borders, edge definition |
| Braiding | Glued or sewn | Covering staples, skirt lines |
| Nailhead trim | Hammered using magnetic hammer | Arm outlines, front panels |
| Fringe | Glued or stitched | Base of traditional chairs and sofas |

Case Study: Trim Alignment at Qhama and De Waal Interiors

Background:

At Qhama and De Waal Interiors in Port Elizabeth, Sipho, Janine, and Devlin were upholstering a series of occasional chairs with velvet fabric and braided trim. Sipho’s trim followed a clean line, while Janine’s braid veered downward at one side, and Devlin left a frayed edge exposed behind the leg.

Outcome:

- The quality controller returned Janine and Devlin’s chairs for correction.

- A team review was held on trimming techniques and alignment marking.

- Each learner created a practice board displaying three trimming types correctly attached.

Learning Point:
Decorative trimmings require the same level of precision and planning as structural upholstery elements.

Critical Thinking Questions

- Why is the alignment of decorative trimmings important in upholstery presentation?

- How can a worker ensure that nailhead trim follows a straight and consistent line?

- What safety precautions must be followed when hammering decorative nails?

- What happens if trimming ends are not secured or folded under?

- How would you choose between piping and braid when finishing a modern-style sofa?

Practical Classroom Activity

Task: Trimming Application Practice

Materials:

- Upholstered mock boards or corners

- Braiding, fringe, piping, decorative nails

- Glue guns, upholstery needles, nail hammers, chalk

Instructions:

- Learners apply at least two types of decorative trimming to mock boards.

- Use marking tools and guides to ensure alignment.

- Present work for peer and facilitator feedback on neatness and accuracy.

- Record the trimming type, method used, and reflections on technique challenges.

Extension:
Create a challenge in which learners match a trim to a style brief (e.g. “formal dining chair”, “vintage ottoman”) and justify their design choice.

### KT0812: Cutting In of Covers

Purpose of the Topic

This topic introduces learners to the practice of cutting in covers—a method used to trim fabric on the frame after it has been loosely positioned. This technique is often applied in custom or precision upholstery, where fabric is first attached at key points, and then carefully trimmed to fit tightly along frame edges, corners, or contours.

Learners will understand how to execute cutting in accurately and safely, avoiding overcutting, fraying, or under-trimming. This skill supports a clean finish, material control, and the ability to work on complex or non-standard furniture shapes.

Key Content Areas

- Definition and Purpose

  - Cutting in is the controlled trimming of excess fabric after it has been lightly fixed to the frame.

  - Ensures perfect fit around frame shapes and details without pre-cutting errors.

  - Used in areas where fabric needs to wrap around components, such as legs, curves, or recesses.

- Steps for Cutting In

  - Step 1: Loosely staple or tack the fabric in place, ensuring correct positioning.

  - Step 2: Gently mark the fold or corner where trimming will occur.

  - Step 3: Use sharp upholstery scissors or duckbill shears to trim away excess.

  - Step 4: Finish folding and fix permanently with tacks, staples, or stitching.

- Tools and Safety

  - Always use sharp, clean scissors or shears to avoid jagged cuts.

  - Keep hands clear of cutting path and work on a stable surface.

  - For precision, use chalk markings and cut slowly in stages.

- Common Challenges

  - Overcutting: leaves gaps or reveals raw edges.

  - Under-trimming: causes bulges or excess folds at corners.

  - Fraying: some fabrics require backing tape or sealing to prevent wear after cutting.

Example Table: Cutting In Situations and Considerations

| Cutting In Scenario | Consideration | Preventative Technique |
| --- | --- | --- |
| Cutting around chair legs | Avoid cutting too close to wood | Use centre notch, then trim evenly around |
| Cutting inside corners | Maintain fold direction and avoid fraying | Use sharp corner fold with minimal cut |
| Trimming thick fabric on curves | May create bulk if not reduced correctly | Feather edges, cut in layers if necessary |
| Cutting inside back frame border | Edge must be symmetrical and tensioned | Mark centre and work outward with care |

Case Study: Overcut Panel at Kriel Furniture Works

Background:
At Kriel Furniture Works in Mpumalanga, Bianca, Ayanda, and Naeem were working on upholstered benches for a boutique hotel. While cutting in fabric around the curved inside frame, Bianca accidentally overcut the edge, leaving a gap between the fold and the wood. The piece was returned to the covering table and required a replacement fabric panel.

Outcome:

- All team members were trained to test fit using fold lines before making cuts.

- Learners were paired in teams to practise “cut and check” sequences.

- A checklist was introduced to include “confirm fit before cut” in the process flow.

Learning Point:
Cutting in is a skill that requires patience, attention to detail, and preparation—not just scissor handling.

Critical Thinking Questions

- Why is it better to temporarily tack fabric in place before cutting it in?

- What are the risks of overcutting a fabric when shaping it around a leg or curve?

- How can different types of fabric affect the way you cut in and finish the edge?

- What visual cues might indicate that more trimming is needed before final fixing?

- What safety practices should learners follow when using scissors or duckbill shears?

Practical Classroom Activity

Task: Cutting In Demonstration and Practice

Materials:

- Upholstered mock frames or scrap corners

- Offcut upholstery fabrics of varying thickness

- Chalk, measuring tape, sharp scissors or duckbill shears

Instructions:

- Learners practise attaching fabric loosely to a corner and mark key cutting points.

- Apply the cut-in technique around a mock leg or curve.

- Peer review for overcuts, smooth finish, and correct edge control.

- Reflect on how each learner adjusted their approach depending on the fabric.

Extension:
Introduce patterned fabric and require learners to maintain pattern alignment while performing a cut-in, reinforcing multi-skill application.

### KT0813: Foam Marking

Purpose of the Topic

This topic introduces learners to the fundamental skill of foam marking, which involves the accurate layout and marking of foam blocks before cutting, shaping, or assembling them into upholstered furniture components. Foam marking is an essential step in ensuring that cushions, padding, and shaped inserts fit precisely into frames or covers.

Through this topic, learners will understand how to read measurements, apply templates or specifications, and use marking tools to ensure efficiency, material accuracy, and waste minimisation.

Key Content Areas

- Why Foam Marking Is Important

  - Ensures correct fit into frames and covers.

  - Supports consistency across production batches.

  - Reduces foam wastage and avoids incorrect cutting.

  - Provides a reference for shaping (e.g. bevels, chamfers, or profile cuts).

- Tools and Materials

  - Chalk, felt-tip markers, ballpoint pens (for denser foams).

  - Templates or cutting patterns for repetitive shapes.

  - Rulers, measuring tapes, squares, and straight edges.

  - Spray adhesives or notes to mark orientation (top, bottom, front).

- Steps for Foam Marking

  - Step 1: Review the specification or cutting list.

  - Step 2: Measure foam dimensions using correct units (e.g. mm or cm).

  - Step 3: Mark the outline clearly on the foam surface.

  - Step 4: Label special cuts (e.g. angles, recesses) where required.

  - Step 5: Double-check all markings before cutting begins.

- Common Foam Types in Upholstery

  - High-density foam for seat cushions.

  - Medium-density foam for backs and arms.

  - Reticulated or soft foam for layered padding.

  - Each may require different marking and cutting approaches.

Example Table: Foam Marking Applications

| Foam Application | Required Accuracy | Marking Tool | Notes |
| --- | --- | --- | --- |
| Seat base cushion | High | Felt-tip marker | Must match cover dimensions exactly |
| Curved arm padding | Medium | Chalk or marker | Allow shaping tolerance |
| Backrest overlay | Medium | Pen or chalk | Mark fold or bend points |
| Tufted insert (with buttons) | Very High | Template and pen | Ensure hole positions align with layout |

Case Study: Mislabelled Foam at Langa Interiors

Background:

At Langa Interiors in Kempton Park, Morgan, Zinhle, and Morne were preparing foam inserts for a sofa with contoured back cushions. Morne marked two blocks using the wrong orientation, and as a result, the cutting followed the width instead of the height, making the foam unusable.

Outcome:

- The foam had to be reordered, delaying the production by a day.

- The workshop implemented a system requiring all foam blocks to be marked with arrows showing orientation and initials of the marker.

- Learners now double-check dimensions and placement before cutting.

Learning Point:
Even a small error in marking foam can result in material waste, delivery delays, and added cost.

Critical Thinking Questions

- Why is it necessary to mark foam before cutting instead of estimating the shape?

- What could happen if foam is cut using incorrect orientation?

- How can templates assist when marking foam for repetitive furniture components?

- Why should markings be double-checked before making the first cut?

- What are some best practices to keep marking accurate during busy production runs?

Practical Classroom Activity

Task: Foam Marking and Layout Planning

Materials:

- Foam blocks or foam sheets of various thicknesses

- Rulers, chalk, measuring tapes, pens, and templates

- Sample cutting list

Instructions:

- Learners review a mock cutting list and identify foam block dimensions.

- Mark out several foam pieces for different furniture components.

- Compare marked areas to ensure clear, legible, and accurate layout.

- Present marked foam to peers or facilitator for feedback before simulated cutting.

Extension:
Ask learners to create a labelling system for tracking orientation and component allocation on foam pieces in a multi-item order.

## Integrated Assessment – KM-03-KT08

Qualification: Furniture Upholsterer (SAQA ID 103199)
Knowledge Module: KM-03-KT08 – Upholstered Furniture Covering Operations and Techniques
NQF Level: 2 | Credits: 8 | Weighting: 20%
Assessment Type: Variety of Instruments
Duration: 90 minutes
Total Marks: 50

🔹 Section A: Short Answer (4 marks each × 5 = 20 marks)

- Define what is meant by upholstery covering techniques and describe how they are applied in professional practice. (IAC0801)

- Using the correct terminology, explain the difference in covering sequence between a drop-on and a drop-in dining chair seat. (IAC0802)

- Compare the sequence of fitting covers to a fully covered chair versus a show wood chair, using industry-appropriate terms. (IAC0803)

- Describe the step-by-step fitting process for each of the following:

  - A round pouffe with padded sides

  - A fully upholstered sofa

  - A dining chair with an upholstered back

  - An armchair with a loose seat cushion
(IAC0804)

- Explain the principles of measuring, cutting, and attaching fabric to cover foam and padding. (IAC0808)

🔹 Section B: Scenario-Based Questions (3 marks each × 5 = 15 marks)

Scenario:
At Riana Interiors, Dumisani and Marcia are upholstering a new range of high-back armchairs. Marcia uses a staple gun and tacking strip to apply the outer back, while Dumisani uses blind stitching along the seat edge. During quality control, one chair shows wrinkles across the backrest and visible thread along the seat front.

- What stretching techniques using hand or power tools should Marcia have used to achieve a smooth finish? (IAC0805)

- What is blind stitching and why is it important in concealed upholstery joins? (IAC0806)

- Identify the covering faults on both chairs and explain how they could be prevented. (IAC0807)

- Describe how Marcia and Dumisani should have followed workplace procedures to avoid damage during storage or transfer to finishing. (IAC0809)

- What steps should be followed to ensure even fabric tension and alignment during final fastening? (IAC0805 + IAC0808)

🔹 Section C: Comparative Table (2 marks per row × 5 rows = 10 marks)

Complete the following table by comparing key aspects of covering across different furniture types:

| Furniture Item | Main Covering Sequence Detail | Special Consideration |
| --- | --- | --- |
| Drop-on dining chair seat |  |  |
| Show wood lounge chair |  |  |
| Armchair with loose seat cushion |  |  |
| Round pouffe |  |  |
| Upholstered back dining chair |  |  |

(IAC0802, IAC0803, IAC0804)

✅ Model Answers and Marking Memo (Summary)

- Section A: Marks awarded for completeness, clarity, correct use of terminology, and alignment with covering processes and terminology.

- Section B: Assesses application of knowledge to real-world scenario; partial marks may be awarded for logical reasoning with minor omissions.

- Section C: Assesses comparison skills and ability to transfer knowledge across different furniture types.

📊 Assessment Rubric

| Criteria | Outstanding (4) | Competent (3) | Partially Achieved (1–2) | Not Achieved (0) |
| --- | --- | --- | --- | --- |
| Understanding of Covering Techniques | Clear, accurate, and applied in all answers | Mostly accurate, minor misunderstandings | Inconsistent, lacks detail or structure | Incorrect, off-topic, or missing |
| Use of Technical Terminology | All terms used accurately and appropriately | Most terms used appropriately | Some terms used incorrectly or not at all | No understanding of key terminology |
| Scenario Application | Practical, realistic and insightful solutions | Solutions generally relevant | Solutions vague or missing key principles | Misunderstood or did not address it |
| Clarity and Completeness | Responses well organised, legible, and logically sequenced | Generally clear with occasional confusion | Unclear structure or incomplete | No logical structure or coherence |

### Facilitator Assessment Briefing

Knowledge Module: KM-03-KT08 – Upholstered Furniture Covering Operations and Techniques
Qualification: Furniture Upholsterer (SAQA ID 103199)
NQF Level: 2 | Credits: 8 | Weighting: 20%
Assessment Duration: 90 minutes
Assessment Type: Variety of Instruments
Total Marks: 50

🎯 Purpose of the Assessment

This assessment aims to evaluate the learner’s ability to:

- Describe and apply covering techniques including pleating, buttoning, blind stitching, and tensioning.

- Compare covering sequences for various frame types and chair designs.

- Interpret and apply measurement and cutting principles related to upholstery.

- Identify and rectify covering faults, and describe how to store and protect finished items.

✅ Assessment Overview

| Section | Type | Marks |
| --- | --- | --- |
| A | Short Answer | 20 |
| B | Scenario-Based Questions | 15 |
| C | Comparative Table | 10 |
| Total |  | 45 |

Note: Section weighting aligns with Internal Assessment Criteria IAC0801–IAC0809.

🛠 Facilitator Preparation Checklist

- Prepare printed copies of the assessment and marking rubric.

- Set up a quiet, well-lit space for learners.

- Confirm learners have pens, rulers, and other necessary tools.

- Display visuals of different chair types for Section C (optional).

- Prepare templates or samples of foam, fabric, and frames for reference (optional).

📢 Instructions to Read Aloud to Learners

- This assessment is designed to test your understanding of upholstery covering operations and techniques.

- Read all questions carefully and answer in full sentences where appropriate.

- Refer to correct upholstery terminology when describing tools, techniques, or processes.

- Use diagrams or bullet points only where the question permits.

- You have 90 minutes to complete the paper.

- Do not speak to other learners during the assessment. Raise your hand if you need clarification.

📊 Marking Guidelines

- Refer to the model answers for content accuracy, not just word-for-word responses.

- Accept variations in terminology if technically accurate.

- Award partial marks for reasonable attempts or explanations.

- Record learner marks for internal moderation and verification.

Bottom of Form