FURNITURE UPHOLSTERER

KM04 ADVANCED UPHOLSTERY FURNITURE TECHNOLOGY

FACILITATOR GUIDE

Table of Contents

KM-04: Advanced Upholstery Furniture Technology	6

KM-04-KT01: Historical and Technological Factors Influencing Furniture Manufacturing Processes (10%)	8

KT0101: Types, Styles and Uses of Furniture Including Upholstered Furniture	10

KT0102: The Main Historical Factors and Eras That Influenced the Designs of Different Types of Furniture	12

KT0103: The Industrial Revolution and the Development of New Technologies and Furniture Making and Upholstery Materials	15

KT0104: The Impact of Factories on the Human Resources and Their Skills	18

Integrated Assessment: KM-04-KT01	20

Facilitator Briefing Note	24

KM-04-KT02: Furniture Types, Styles, and Construction (15%)	26

KT0201: Types and Styles of Furniture, Including Upholstery, and Their Uses	28

KT0202: Concepts of Ergonomic Design and Standard Dimensions of Upholstered Furniture	31

KT0203: Main Furniture Construction Principles (Stability, Squareness, Ergonomics, Measurements, Conversions, etc.)	34

KT0204: Furniture Manufacturing Processes Flow Including Materials, Parts Identification, Machining, Joints and Assembly, Frame Preparing, Upholstery, Using the Correct Terminology	37

KT0205: Furniture Manufacturing Process from Raw Wood to Finished Product	40

KT0206: Concepts of Furniture Construction Including Frame Construction, Construction of Padding and Upholstery Components	44

KT0207: Impact of Design and Construction Faults	47

KT0208: Problem Solving	50

Integrated Assessment – KM-04-KT02	53

Facilitator Assessment Briefing	58

KM-04-KT03: Equipment and Tools Used in Advanced Upholstery of Furniture (15%)	61

KT0301: Types and Uses of Hand Tools	63

KT0302: Types and Uses of Power Tools	66

KT0303: Types and Uses of Pneumatic Tools	68

KT0304: How to Use the Tools Correctly and Safely	71

KT0305: Settings Including Pressure Settings	74

KT0306: Maintenance of Different Tools (Sharpening, Lubrication, Tension, Pressure, etc.)	77

KT0307: Correct Grease is Used on the Tools	80

KT0308: Correct Size Staples and Nails Are Used on the Machine	83

KT0309: Storage of Tools	86

KT0310: Reading Labels and MSDS	89

KT0311: Manufacturer Specifications and Instructions	92

KT0312: Lockout Devices and Safety Guards	95

Integrated Assessment – KM-04-KT03	98

Facilitator Assessment Briefing	102

KM-04-KT04: Health and Safety in the Advanced Upholstery Department (15%)	105

KT0401: BCOE Act	107

KT0402: Occupational Health and Safety (OHS) Act	110

KT0403: General Hazards in the Workshop	113

KT0404: Fire Extinguishers and Uses	116

KT0405: Evacuation Plans and Signage in the Workshop	119

KT0406: Demarcation Lines	122

KT0407: Safe Working Procedures in the Various Machining Operations	124

KT0408: Machine Safety (Isolate the Machine)	127

KT0409: Personal Safety (PPC and PPE)	130

KT0410: Hazards and Incidents	133

KT0411: Lock-Out Devices and Procedure	136

Integrated Assessment – KM-04-KT04	139

Facilitator Assessment Briefing	143

KM-04-KT05: Concepts and Principles of Measuring and Calculations Used in the Furniture Upholstery Processes (15%)	146

KT0501: Taking Accurate Measures	148

KT0502: Units of Measurement	151

KT0503: Conversions	154

KT0504: Formulae and Calculations	156

KT0505: Angles and Curves	158

KT0506: Standard Sizes	161

KT0507: Allowances and Tolerances	164

KT0508: Measuring Instruments – Tape Measure, Vernier, Callipers, Square	167

KT0509: Calibrations	170

KT0510: Accuracy and Faults	173

Integrated Assessment – KM-04-KT05	176

Facilitator Assessment Briefing	180

KM-04-KT06: Engineering Drawings (15%)	183

KT0601: Read and Interpret Furniture Specifications	185

KT0602: Sketches and Engineering Drawings	188

KT0603: Layout of Drawings	190

KT0604: Legends and Symbols	193

KT0605: Dimensions and Labelling	196

KT0606: Isometric Views	199

KT0607: Line Types	201

KT0608: Hidden Details	204

KT0609: Solid Lines	206

KT0610: Projections	208

KT0611: To Scale Drawing	210

KT0612: Cutting Lists	212

Integrated Assessment – KM-04-KT06	215

Facilitator Assessment Briefing	217

KM-04-KT07: Samples, Prototypes and Customer Specifications (5%)	221

KT0701: Definition of Product Specifications, Samples, Prototypes and Customer Specifications	223

KT0702: Content of Specification Sheets	226

KT0703: Interpretation of Product Specifications, Sample or Prototype	229

KT0704: Uses of Product Specifications, Sample or Prototype	231

KT0705: Types of Product Specifications, Sample or Prototype	234

Integrated Assessment – KM-04-KT07	237

Facilitator Assessment Briefing	241

KM-04-KT08: Efficiencies – Output, Scores and Targets (5%)	244

KT0801: Targets and Productivity	246

KT0802: Productivity Principles and Practices	249

KT0803: Organising Self and Own Targets and Own Time	252

KT0804: Recording Score and Daily Output	255

Integrated Assessment – KM-04-KT08	258

Facilitator Assessment Briefing	262

KM-04-KT09: Inspection and Disposal of Finished Upholstery Products (5%)	264

KT0901: Inspection Principles	266

KT0902: Techniques of Disposing According to Product Type	268

KT0903: Quality in Manufacturing Processes and Products	270

Integrated Assessment – KM-04-KT09	273

Facilitator Assessment Briefing	277

# KM-04: Advanced Upholstery Furniture Technology

NQF Level 3 | Credits 2

Purpose of the Knowledge Modules

The purpose of this knowledge module is to build an in-depth understanding of advanced upholstery furniture technology within the context of a manufacturing environment. It equips learners with knowledge of historical developments, ergonomic and technical considerations, safety, specifications, and quality control procedures. Learners will gain theoretical competence in interpreting engineering drawings, understanding product specifications and quality systems, as well as calculating measurements, which are critical to ensuring precision and efficiency in the upholstery process.

This module supports the development of essential theoretical knowledge that underpins the practical and workplace components of the occupational qualification. It enables learners to integrate knowledge of equipment, tools, construction techniques, and productivity principles in advanced upholstery settings. This integration will contribute to improved product quality, efficiency, and compliance with industry standards.

4.2 Key Knowledge Areas

Learners will demonstrate an understanding of the following knowledge topics:

- KM-04-KT01: Historical and technological factors influencing furniture manufacturing processes (10%)

- KM-04-KT02: Furniture types, styles, and construction (15%)

- KM-04-KT03: Equipment and tools used in advanced upholstery of furniture (15%)

- KM-04-KT04: Health and safety in the advanced upholstery department (15%)

- KM-04-KT05: Concepts and principles of measuring and calculations used in the furniture upholstery processes (15%)

- KM-04-KT06: Engineering drawings (15%)

- KM-04-KT07: Samples, prototypes and customer specifications (5%)

- KM-04-KT08: Efficiencies: output, scores and targets (5%)

- KM-04-KT09: Inspection and disposal of finished upholstery products (5%)

4.3 Internal Assessment Criteria and Weighting

Learner competence will be assessed against the following internal assessment criteria:

- IAC0101 – IAC0104: Historical influences, industrialisation, production methods, and socio-economic impacts (10%)

- IAC0201 – IAC0205: Furniture construction principles, terminology, and production processes (15%)

- IAC0301 – IAC0306: Tool and equipment use, maintenance, safety, and manufacturer specifications (15%)

- IAC0401 – IAC0411: Health and safety routines, hazard management, fire safety, PPE, and incident protocols (15%)

- IAC0501 – IAC0507: Application of formulae, measuring accuracy, calibration, and equipment selection (15%)

- IAC0601 – IAC0605: Engineering drawing interpretation, projections, scales, and labels (15%)

- IAC0701 – IAC0707: Interpretation of specifications, sample and prototype analysis (5%)

- IAC0801 – IAC0805: Productivity calculations, non-conformance analysis, and waste reduction (5%)

- IAC0901 – IAC0905: Inspection techniques, non-conformance reporting, and disposal procedures (5%)

4.4 Application in Furniture Design

Knowledge acquired in this module is essential to the effective planning, design, and manufacture of high-quality upholstered furniture. By mastering the principles covered, learners will be able to interpret technical documentation, follow design specifications, select and maintain tools, monitor quality, and ensure production efficiency. This competency directly supports the learner's role in reducing waste, ensuring workplace safety, and meeting customer and organisational quality standards in an advanced upholstery manufacturing environment​

## KM-04-KT01: Historical and Technological Factors Influencing Furniture Manufacturing Processes (10%)

4.1 Purpose of the Knowledge Modules

The purpose of this knowledge module is to build an in-depth understanding of advanced upholstery furniture technology within the context of a manufacturing environment. It equips learners with knowledge of historical developments, ergonomic and technical considerations, safety, specifications, and quality control procedures. Learners will gain theoretical competence in interpreting engineering drawings, understanding product specifications and quality systems, as well as calculating measurements, which are critical to ensuring precision and efficiency in the upholstery process.

This module supports the development of essential theoretical knowledge that underpins the practical and workplace components of the occupational qualification. It enables learners to integrate knowledge of equipment, tools, construction techniques, and productivity principles in advanced upholstery settings. This integration will contribute to improved product quality, efficiency, and compliance with industry standards.

4.2 Key Knowledge Areas

Learners will demonstrate an understanding of the following knowledge topics:

- KM-04-KT01: Historical and technological factors influencing furniture manufacturing processes (10%)

- KM-04-KT02: Furniture types, styles, and construction (15%)

- KM-04-KT03: Equipment and tools used in advanced upholstery of furniture (15%)

- KM-04-KT04: Health and safety in the advanced upholstery department (15%)

- KM-04-KT05: Concepts and principles of measuring and calculations used in the furniture upholstery processes (15%)

- KM-04-KT06: Engineering drawings (15%)

- KM-04-KT07: Samples, prototypes and customer specifications (5%)

- KM-04-KT08: Efficiencies: output, scores and targets (5%)

- KM-04-KT09: Inspection and disposal of finished upholstery products (5%)

4.3 Internal Assessment Criteria and Weighting

Learner competence will be assessed against the following internal assessment criteria:

- IAC0101 – IAC0104: Historical influences, industrialisation, production methods, and socio-economic impacts (10%)

- IAC0201 – IAC0205: Furniture construction principles, terminology, and production processes (15%)

- IAC0301 – IAC0306: Tool and equipment use, maintenance, safety, and manufacturer specifications (15%)

- IAC0401 – IAC0411: Health and safety routines, hazard management, fire safety, PPE, and incident protocols (15%)

- IAC0501 – IAC0507: Application of formulae, measuring accuracy, calibration, and equipment selection (15%)

- IAC0601 – IAC0605: Engineering drawing interpretation, projections, scales, and labels (15%)

- IAC0701 – IAC0707: Interpretation of specifications, sample and prototype analysis (5%)

- IAC0801 – IAC0805: Productivity calculations, non-conformance analysis, and waste reduction (5%)

- IAC0901 – IAC0905: Inspection techniques, non-conformance reporting, and disposal procedures (5%)

4.4 Application in Furniture Design

Knowledge acquired in this module is essential to the effective planning, design, and manufacture of high-quality upholstered furniture. By mastering the principles covered, learners will be able to interpret technical documentation, follow design specifications, select and maintain tools, monitor quality, and ensure production efficiency. This competency directly supports the learner's role in reducing waste, ensuring workplace safety, and meeting customer and organisational quality standards in an advanced upholstery manufacturing environment

### KT0101: Types, Styles and Uses of Furniture Including Upholstered Furniture

Theoretical Learning Content

Furniture is designed to support human activities such as seating, sleeping, eating, and storage. Upholstered furniture, in particular, adds comfort and aesthetic value through padding, springs, webbing, and fabric or leather covers. The types, styles, and uses of furniture are influenced by cultural trends, technological advancements, material availability, and ergonomics.

1. Furniture Types

Furniture can be categorised by function and form. Examples include:

- Seating: Chairs, sofas, armchairs, pouffes, benches

- Sleeping: Beds, daybeds, sleeper couches

- Storage: Wardrobes, chests, cabinets

- Work surfaces: Desks, tables, dressing tables

Upholstered furniture is mostly found in seating and sleeping categories, but may include elements in others (e.g. upholstered headboards on beds).

2. Furniture Styles

Furniture styles have evolved over centuries. Some of the most influential styles include:

- Baroque (17th Century) – Ornate and dramatic, with heavy carving and gilding

- Rococo (18th Century) – Lighter and more playful than Baroque; featured asymmetry and floral motifs

- Victorian (19th Century) – Dark woods, intricate upholstery, tufting (e.g. deep buttoning)

- Art Deco (1920s–1930s) – Geometric patterns, chrome finishes, rich colours

- Mid-Century Modern (1940s–1960s) – Clean lines, minimalism, use of plywood and bold textiles

- Contemporary and Sustainable Styles (1990s–Present) – Functionality, modularity, eco-conscious materials

3. Uses of Furniture

Furniture serves both functional and aesthetic purposes:

- Residential: Living rooms, bedrooms, dining areas, patios

- Commercial: Hotels, restaurants, waiting rooms, office lounges

- Institutional: Schools, hospitals, government facilities

Upholstered furniture enhances comfort, acoustic insulation, thermal comfort, and visual appeal. In commercial settings, it may also reinforce brand identity through custom design.

Examples

- A Victorian chaise lounge with deep buttoning demonstrates advanced upholstery techniques and reflects historical opulence.

- A modular sofa in a contemporary home with interchangeable fabric panels reflects modern needs for flexibility and sustainability.

- Restaurant booths upholstered in vinyl illustrate the use of wipeable, durable materials for high-use environments.

Case Study

Case Study: Comfort Lounge Furniture Co. – Balancing Tradition and Innovation

Comfort Lounge Furniture Co., a South African SME, blends traditional craftsmanship with modern design. The company produces luxury armchairs using techniques derived from Victorian deep-buttoning but applies contemporary fabrics such as eco-friendly faux leather. The challenge lies in catering to a niche market that values both heritage design and environmental consciousness.

Key questions:

- How can traditional upholstery techniques remain relevant in a market dominated by mass production?

- What are the benefits of combining historical styles with modern materials?

Critical Thinking Questions

- What distinguishes an upholstered chair from a non-upholstered one, and why is this distinction important in furniture design?

- How do furniture styles reflect the social and economic conditions of the time in which they were popular?

- Why might a hotel opt for Art Deco upholstered furniture in its lobby rather than a modern minimalist style?

- Discuss how upholstery design in public transport (e.g. buses or trains) balances style, cost, and durability.

- How can upholstery manufacturers in South Africa incorporate indigenous or local influences into modern furniture design?

### KT0102: The Main Historical Factors and Eras That Influenced the Designs of Different Types of Furniture

Theoretical Learning Content

Furniture design does not exist in isolation; it is shaped by the cultural, economic, and technological contexts of its time. Major historical periods have influenced furniture aesthetics, functionality, materials, and production techniques. A basic understanding of these eras allows upholstery professionals to recognise design elements, honour traditional styles, and contribute to restoration, reproduction, or reinterpretation of furniture pieces.

1. Ancient and Classical Eras

- Egyptian: Used wood, ivory, and gilding. Furniture was symbolic of status (e.g. thrones and ceremonial beds).

- Greek and Roman: Introduced symmetry, proportion, and early forms of upholstery (e.g. cushions and low couches).

2. Medieval Period (500–1500 AD)

- Characterised by heavy, sturdy furniture made mostly of oak.

- Functional and often austere due to religious and feudal influences.

- Limited upholstery; comfort was not a major consideration.

3. Renaissance (14th–17th Century)

- Inspired by classical antiquity; saw a revival of decorative arts.

- Ornate furniture with carved details, inlay, and use of velvet and brocade fabrics.

- Upholstery became more refined and integrated with woodwork.

4. Baroque and Rococo (17th–18th Century)

- Baroque: Dramatic, richly decorated furniture, often gilded and monumental in size.

- Rococo: More delicate and decorative with curved lines and pastel colours.

- Upholstery often featured floral motifs and elaborate trimming.

5. Neoclassical and Regency Periods (Late 18th – Early 19th Century)

- Return to straight lines, symmetry, and classical influences.

- More restrained in ornamentation compared to Rococo.

- Common fabrics: silks, damask, and striped cottons.

6. Victorian Era (1837–1901)

- Industrialisation enabled mass production, leading to ornate but often over-decorated styles.

- Deep-buttoned upholstery became a hallmark (e.g. Chesterfield sofas).

- Comfort and visual extravagance were prioritised.

7. 20th Century Movements

- Art Nouveau (1890–1910): Organic, flowing lines; integration of art and furniture.

- Art Deco (1920s–1930s): Bold geometric shapes, use of chrome, leather, and exotic woods.

- Mid-Century Modern (1940s–1960s): Simplicity, functionality, and new materials such as fibreglass and moulded plywood.

- Postmodern and Contemporary: Fusion of past styles with innovation, sustainability, and user customisation.

Examples

- The Chippendale style (mid-1700s) combined Gothic and Rococo influences, with elaborately carved legs and backs.

- A 1920s Art Deco armchair upholstered in velvet demonstrates the era’s luxury and geometric design.

- Post-apartheid South African furniture design often draws on indigenous crafts and motifs, integrating traditional skills with modern aesthetics.

Case Study

Case Study: Heritage Interiors – Interpreting History Through Modern Upholstery

Heritage Interiors is a Cape Town-based firm that restores and reinterprets historical furniture. Their recent project involved recreating a Victorian parlour suite for a heritage hotel, using original techniques such as deep buttoning, coil spring tying, and hand-stitched horsehair padding. They complemented this with updated fire-retardant materials and ethically sourced fabrics. The challenge lay in maintaining historical authenticity while complying with modern regulations.

Discussion points:

- Why is historical accuracy important in restoration projects?

- How do contemporary demands such as fire safety and sustainability influence reinterpretations of historical designs?

Critical Thinking Questions

- How did the Industrial Revolution change furniture design and production methods?

- In what ways did global exploration (voyages of discovery) influence the materials and motifs used in European furniture?

- Which furniture era most influenced the development of upholstery as a craft, and why?

- What are the differences between Rococo and Neoclassical styles in terms of furniture design and upholstery?

- How can understanding historical furniture styles benefit a modern furniture upholsterer?

### KT0103: The Industrial Revolution and the Development of New Technologies and Furniture Making and Upholstery Materials

Theoretical Learning Content

The Industrial Revolution, which began in the late 18th century, marked a fundamental turning point in manufacturing processes, including furniture and upholstery production. It brought about a shift from hand-crafted items to mass production, enabled by the development of machinery, new materials, and factory-based work systems. This era significantly influenced not only the quantity and affordability of furniture but also the design, construction, and upholstery methods.

1. Technological Advancements in Furniture Making

The following innovations were particularly influential:

- Mechanised wood cutting and shaping tools: Bandsaws, lathes, and routers reduced the need for hand carving.

- Steam-powered and later electric machinery: Increased efficiency and enabled the mass replication of parts.

- Plywood and veneer presses: Allowed for faster production of curved forms and decorative surfaces.

2. Upholstery-Specific Developments

Several key developments transformed the upholstery industry:

- Mass-produced springs: Introduced in the mid-19th century, coil springs revolutionised seating comfort.

- Synthetic padding materials: Foam rubber, introduced in the 20th century, replaced horsehair and cotton batting.

- Staple guns and pneumatic tools: Improved speed and consistency in attaching fabric to frames.

- New textiles and synthetic fibres: Made materials more affordable, flame-retardant, and easy to clean.

3. Shift from Craft to Industry

The move from artisanal workshops to mechanised factories led to:

- Standardised designs: Furniture pieces were produced in bulk with less variation.

- Division of labour: Upholsterers began specialising in specific stages of the process.

- Reduced need for traditional skills: Some handcrafting techniques declined in favour of efficiency.

4. Positive and Negative Impacts

Positive:

- Wider access to upholstered furniture for the middle class.

- Greater consistency and speed in production.

- Availability of new design possibilities using new materials.

Negative:

- Loss of artisanal craftsmanship.

- Dependence on synthetic materials with environmental concerns.

- Alienation of workers due to repetitive tasks and reduced skill engagement.

Examples

- Coil springs were patented in 1857, revolutionising seating by adding bounce and comfort.

- The use of high-density polyurethane foam in the 1950s allowed for modular, sculpted sofa designs.

- Pneumatic staple guns, introduced in the 20th century, replaced hand-driven tacks, increasing speed.

Case Study

Case Study: From Hand Tools to High-Tech – Upholstery at MapleCraft (1910–2020)

MapleCraft Upholstery began as a family-run workshop in Durban in the early 20th century. At that time, upholsterers hand-stitched horsehair into webbed seats and used ornate fabrics imported from Europe. By the 1950s, the company had introduced electric sewing machines, foam cushions, and air-powered staple guns. Today, the business uses CAD (Computer-Aided Design) for pattern cutting and employs CNC machines to prepare frames.

Discussion points:

- How did MapleCraft adapt to technological changes without losing its brand identity?

- What challenges do modern upholsterers face when blending old and new techniques?

Critical Thinking Questions

- How did the Industrial Revolution change the materials used in upholstery?

- What are the advantages and disadvantages of mechanised upholstery tools?

- Do modern upholstery workshops still require traditional hand skills? Why or why not?

- In what ways can upholstery manufacturers today embrace new technologies while maintaining quality?

- Discuss the environmental implications of synthetic materials introduced during and after the Industrial Revolution.

### KT0104: The Impact of Factories on the Human Resources and Their Skills

Theoretical Learning Content

The emergence of factories during and after the Industrial Revolution transformed the landscape of work in the furniture and upholstery industries. These changes impacted employment patterns, skill requirements, working conditions, and the social roles of artisans and labourers. Understanding this historical shift helps learners recognise how the upholstery trade evolved from a highly specialised craft to an industrialised process.

1. Transformation of Work Settings

Before industrialisation:

- Upholsterers typically worked in small workshops or as independent artisans.

- They were responsible for the full range of tasks from frame preparation to final finishing.

- Skills were passed through apprenticeships and master–apprentice relationships.

With the rise of factories:

- Production moved into large-scale, mechanised settings.

- Work became segmented into discrete, repetitive tasks.

- Emphasis shifted from creative autonomy to efficiency and productivity.

2. Impact on Human Resources

- Job creation and urban migration: Factories drew large numbers of people into cities seeking employment.

- Standardised roles: Workers were trained for specific, often narrow functions, such as padding, cutting, stitching, or tacking.

- Deskilling: Traditional craftsmanship was sometimes replaced by mechanised processes, reducing the scope of skills required.

- Gendered labour roles: Women were often employed in upholstery and sewing sections, especially during wartime, though often underpaid.

3. Shift in Training and Education

- Apprenticeships declined, replaced by on-the-job training for narrow skills.

- The need for formal safety training increased due to factory machinery.

- Supervisory and quality control roles emerged as distinct career paths.

4. Health and Safety Considerations

- Crowded, poorly ventilated factory environments exposed workers to dust, chemical fumes, and repetitive strain injuries.

- Over time, legislation was introduced to protect workers, enforce minimum wages, regulate working hours, and improve conditions.

Examples

- A modern foam-cutting machine operator is trained to use a single piece of equipment rather than learning full upholstery techniques.

- Health and safety officers now form part of factory teams to ensure compliance with regulations and protect workers.

- Historically, during the World Wars, women entered upholstery factories in large numbers, filling roles vacated by men sent to war.

Case Study

Case Study: Skilled to Specialist – The Changing Role of an Upholstery Worker

In 1890, Samuel, a master upholsterer in Port Elizabeth, would handcraft furniture from frame to finish. By 1925, his grandson, Elias, worked in a factory where he was responsible only for cutting fabric patterns. While Elias earned a stable income, he lacked his grandfather’s comprehensive skills. In 2025, the great-granddaughter, Lindiwe, works in a modern upholstery plant using digital cutting machines and CAD layouts. Her role requires both technical skills and computer literacy, showing the evolution and re-specialisation of skills across generations.

Discussion points:

- What skills did Samuel possess that Elias did not need?

- How did the factory setting both limit and create opportunities for workers like Elias and Lindiwe?

Critical Thinking Questions

- How did the shift from workshops to factories affect the skills development of upholstery workers?

- In what ways did the rise of factories improve or worsen working conditions for upholsterers?

- How has the role of the upholsterer changed in terms of autonomy, creativity, and career progression?

- What modern training strategies are needed to prepare workers for today’s upholstery factories?

- Discuss the balance between efficiency and craftsmanship in the context of mass production and small business upholstery services.

## Integrated Assessment: KM-04-KT01

Historical and Technological Factors Influencing Furniture Manufacturing Processes

📝 Assessment Instruments Used

| Instrument Type | Purpose |
| --- | --- |
| Short Answer Questions | Test historical recall and conceptual understanding |
| Case Study with Application | Contextualise knowledge and interpret impact |
| Comparative Table Exercise | Analyse differences between systems |
| Extended Written Response | Integrate learning to evaluate socio-economic changes |

Assessment Activities

Activity 1: Short Answer Questions (IAC0101, IAC0102)

Total: 10 marks

Instructions: Answer each question in 2–4 sentences.

- Name two historical eras that significantly influenced furniture design.

- What was the impact of European royalty on furniture design during the 17th and 18th centuries?

- Briefly explain how the voyages of discovery influenced the types of materials used in upholstery.

- Identify two ways in which the Industrial Revolution affected furniture production methods.

- Name one new material and one tool that became widely used in furniture upholstery after the Industrial Revolution.

Activity 2: Case Study with Scenario-Based Questions (IAC0101, IAC0104)

Total: 15 marks

Case Study
“Elsa’s Upholstery Studio” is located in the Eastern Cape. Elsa restores antique furniture, often requiring historical accuracy in fabric selection, wood carving detail, and upholstery techniques. She recently visited a local museum that features colonial-era and Art Deco furniture, and she plans to incorporate historical design principles into her modern range. Elsa’s niece, Thandeka, works in a high-speed furniture factory assembling modular sofas using automated machinery and synthetic fabrics. They often discuss how different their workplaces and roles are.

Questions

- Identify two historical influences Elsa might study when restoring colonial-era furniture. (2)

- How do Elsa and Thandeka’s work settings differ in terms of required skills? (4)

- Explain one advantage and one disadvantage of working in a factory-based upholstery setting like Thandeka’s. (4)

- Which era introduced Art Deco furniture, and what are two visual characteristics of this style? (2)

- Discuss one way craftsmen like Elsa contribute to preserving furniture heritage. (3)

Activity 3: Comparative Table (IAC0103)

Total: 10 marks

Instructions: Complete the table by identifying differences between mass production and small business upholstery services.

| Criteria | Mass Production Upholstery | Small Business Upholstery |
| --- | --- | --- |
| Type of Tools Used |  |  |
| Variety in Design |  |  |
| Production Volume |  |  |
| Level of Customisation |  |  |
| Role of Skilled Artisans |  |  |

Activity 4: Extended Written Response (IAC0104)

Total: 15 marks

Instructions: In a paragraph of 150–200 words, describe how the establishment of factories during the Industrial Revolution influenced the lives of people working in them. Include reference to changes in skill requirements, working conditions, gender roles, and the organisation of labour.

✅ Model Answers

Activity 1: Short Answer Questions

- Baroque and Art Deco

- European royalty influenced elaborate, decorative styles using gilding, silk, and complex carving.

- New exotic materials like mahogany, spices, and silk influenced design and upholstery options.

- Machinery enabled mass production and introduced division of labour.

- Foam (material) and the pneumatic staple gun (tool)

Activity 2: Case Study

- British Colonial, Victorian

- Elsa uses hand tools and historical knowledge; Thandeka operates machines and follows standardised tasks.

- Advantage: steady income; Disadvantage: limited creativity

- Art Deco emerged in the 1920s–1930s; it features geometric shapes and rich materials.

- Craftsmen preserve traditional techniques and styles that would otherwise be lost.

Activity 3: Comparative Table

| Criteria | Mass Production Upholstery | Small Business Upholstery |
| --- | --- | --- |
| Type of Tools Used | Automated and pneumatic tools | Hand tools and selective machinery |
| Variety in Design | Limited, standardised | Wide, customised |
| Production Volume | High | Low to medium |
| Level of Customisation | Low | High |
| Role of Skilled Artisans | Minimal | Central |

Activity 4: Extended Written Response

The development of factories during the Industrial Revolution transformed the upholstery workforce. Artisans were replaced by machine operators trained to perform specific repetitive tasks. While factories created more jobs and led to urban migration, they often imposed long hours, poor ventilation, and limited labour rights. Many women were employed in sewing or stuffing sections, though they earned lower wages than men. Craftsmanship was replaced with efficiency, and knowledge transmission via apprenticeships declined. Though productivity increased, individual creativity and autonomy were reduced, affecting job satisfaction and community standing of tradespeople.

Marking Memo

| Activity | Total Marks | Key Marking Notes |
| --- | --- | --- |
| Short Answer Questions | 10 | 2 marks per question, factual and concise answers required |
| Case Study | 15 | Award full marks for contextualised and reflective answers |
| Comparative Table | 10 | 2 marks per row; focus on contrast and clarity |
| Extended Written Response | 15 | Mark for structure (3), content (10), grammar/clarity (2) |
| Total | 50 marks |  |

📊 Assessment Rubric (Summary)

| Rating | Descriptor |
| --- | --- |
| 5: Excellent | Demonstrates comprehensive and accurate understanding of all concepts |
| 4: Good | Sound understanding with minor factual or structural errors |
| 3: Satisfactory | Adequate understanding with moderate detail; some confusion in application |
| 2: Basic | Limited understanding; minimal contextual interpretation |
| 1: Poor | Inaccurate or incomplete; lacks relevance or fails to address the question |

### Facilitator Briefing Note

KM-04-KT01: Historical and Technological Factors Influencing Furniture Manufacturing Processes
SAQA ID: 103199 – Furniture Upholsterer | NQF Level 3 | Knowledge Module 04

🎯 Purpose of the Assessment

The purpose of this assessment is to evaluate the learner’s understanding of the historical and technological influences on furniture design and upholstery practices. It provides a foundation for appreciating how past events, innovations, and socio-economic changes have shaped contemporary upholstery work environments, materials, and tools.

This assessment integrates IAC0101–IAC0104 and carries a weight of 10% of the Knowledge Module.

🔍 Assessment Overview

| Component Type | Topics Assessed | Methodology |
| --- | --- | --- |
| Activity 1: Short Answers | IAC0101, IAC0102 | Recollection and concept |
| Activity 2: Case Study | IAC0101, IAC0104 | Contextual interpretation |
| Activity 3: Comparison | IAC0103 | Analytical table completion |
| Activity 4: Written Task | IAC0104 | Reflective synthesis |

🛠 Facilitation Guidelines

- Before the assessment:

  - Ensure that learners have covered KT0101–KT0104 in sufficient depth.

  - Provide a brief revision session on eras, the Industrial Revolution, and types of upholstery systems (mass vs. artisanal).

  - Discuss expectations for written responses, particularly in terms of structure, clarity, and critical thinking.

- During the assessment:

  - Maintain a quiet, controlled environment with minimal disruption.

  - Ensure learners work individually unless instructed otherwise.

  - Allow use of approved learning notes or prompts for the extended response only (optional, based on delivery strategy).

- After the assessment:

  - Mark according to the marking memo and allocate scores clearly per activity.

  - Use the assessment rubric to moderate borderline responses fairly.

  - Provide feedback highlighting both strengths and areas for improvement.

  - Record the results and prepare a summary for internal moderation.

📚 Evidence Requirements

Each learner’s submission must include:

- Completed activities 1–4 (in written or typed format)

- Learner’s name and date on each page

- Marked scripts with scores and feedback

- A signed assessment cover sheet (if required by your centre)

Scoring Summary

| Activity | Maximum Marks |
| --- | --- |
| Short Answers | 10 |
| Case Study | 15 |
| Comparison Table | 10 |
| Extended Response | 15 |
| Total | 50 |

Scores should be converted to the weighted 10% component for inclusion in the learner’s KM-04 record.

📌 Assessment Moderation Notes

- Sample at least 25% of completed assessments for internal moderation.

- Ensure consistency in awarding marks, especially for open-ended responses.

- Highlight any concerns or trends for future instructional improvement.

Top of Form

## KM-04-KT02: Furniture Types, Styles, and Construction (15%)

Purpose of the Knowledge Topic

The purpose of this knowledge topic is to provide learners with a comprehensive understanding of the types and styles of furniture—especially upholstered furniture—as well as the construction principles that ensure functionality, comfort, and durability. Learners will be introduced to industry-standard terminology, ergonomic concepts, and the structured flow of furniture manufacturing, from raw material to finished product. This knowledge is critical for quality control, client communication, production planning, and workplace efficiency.

In the context of upholstery, it is essential that learners understand how design decisions, construction methods, and ergonomic considerations influence the final product. This topic also covers the differences in production between factory-based mass manufacturing and small business operations, while highlighting the role of design and construction faults and the importance of practical problem-solving.

Key Knowledge Areas

This topic includes the following elements:

- KT0201: Types and styles of furniture, including upholstery, and their uses

- KT0202: Concepts of ergonomic design and standard dimensions of upholstered furniture

- KT0203: Main furniture construction principles (stability, squareness, ergonomics, measurements, conversions, etc.)

- KT0204: Furniture manufacturing process flow including materials, parts identification, machining, joints and assembly, frame preparing, upholstery, using the correct terminology

- KT0205: Furniture manufacturing process from raw wood to finished product

- KT0206: Concepts of furniture construction including frame construction, construction of padding and upholstery components

- KT0207: Impact of design and construction faults

- KT0208: Problem solving

Internal Assessment Criteria and Weight

Learners will be assessed on the following criteria:

- IAC0201: Define the terminology and abbreviations used in the furniture industry

- IAC0202: Identify the parts of various advanced furniture and their construction, using industry terminology

- IAC0203: Describe the mass production process of making furniture from raw wood to finished product with reference to the various departments and their contribution to the final product

- IAC0204: Describe the production process of making upholstered furniture as a small business owner

- IAC0205: The main furniture construction principles are identified and the impact on product quality is reasoned

Weight: 15%

Application in Furniture Upholstery

This knowledge equips learners to make informed design and construction decisions that affect both form and function. It enables them to communicate using accurate terminology, apply ergonomic principles, understand workflow processes, and troubleshoot construction faults. These skills are essential in both large-scale furniture manufacturing environments and bespoke upholstery workshops, ensuring quality outcomes that meet client and industry standards.

### KT0201: Types and Styles of Furniture, Including Upholstery, and Their Uses

Theoretical Learning Content

Furniture is designed to fulfil specific human needs such as sitting, resting, eating, and storage. It also plays an important aesthetic and functional role in both private and public spaces. The design and construction of furniture vary depending on its purpose, the user’s needs, and the context in which it will be used.

1. Types of Furniture

Furniture is generally categorised by its function. Common types include:

- Seating Furniture: Sofas, chairs, armchairs, stools, pouffes, benches

- Sleeping Furniture: Beds, bunk beds, sleeper couches, daybeds

- Storage Furniture: Wardrobes, cabinets, chests, shelving units

- Surface Furniture: Dining tables, coffee tables, desks, consoles

- Display Furniture: Sideboards, TV units, bookcases, glass cabinets

2. Styles of Furniture

Furniture styles are often defined by historical periods, cultural influences, and aesthetic trends. Key styles include:

- Traditional: Inspired by historical periods (e.g. Victorian, Edwardian), typically ornate with rich upholstery.

- Modern: Clean lines, minimalism, emphasis on function over ornamentation.

- Contemporary: Current trends; often incorporates a mix of materials such as metal, glass, and textiles.

- Rustic: Natural wood finishes, earthy textures, often hand-crafted or distressed.

- Scandinavian: Functional and minimalist with light colours, natural wood, and simple fabrics.

- Industrial: Exposed steel, raw wood, and utilitarian aesthetics, often found in urban lofts or cafes.

3. Upholstered Furniture

Upholstered furniture includes a combination of structural framework, padding (foam, wadding, or springs), fabric or leather coverings, and decorative elements such as piping or buttoning. It is typically found in:

- Residential applications: Living rooms, bedrooms, entertainment areas

- Hospitality and commercial environments: Hotels, restaurants, offices, reception areas

- Healthcare and institutional settings: Clinics, elderly care homes, libraries

4. Uses of Furniture

The functional use of furniture varies by setting:

- Domestic use: Comfort, personal expression, and lifestyle needs

- Commercial use: Durability, brand alignment, ease of maintenance

- Public and institutional use: Ergonomics, hygiene, safety, and accessibility

Understanding furniture types and styles enables upholsterers to recommend appropriate designs for specific client needs, spaces, and functional requirements.

Examples

- A Victorian high-back armchair with buttoned velvet upholstery is suited for formal sitting rooms or period-themed interiors.

- A modular Scandinavian sofa with washable slipcovers is ideal for urban flats or family homes with children.

- A bench-style seat upholstered in antimicrobial vinyl may be used in a hospital waiting area for durability and hygiene.

Case Study

Case Study: Mandla’s Multi-Style Workshop

Mandla operates a furniture workshop in Gqeberha, serving a diverse client base. He is currently working on three orders: a traditional Zulu-inspired lounge suite for a heritage hotel, a set of minimalist office reception chairs, and a rustic wooden bench with hemp padding for a farm guesthouse. Each project requires him to understand different furniture styles and their contextual uses. Mandla must advise on fabric choice, padding density, and finishing based on the client's functional and aesthetic preferences.

Discussion Points:

- How does Mandla ensure each furniture piece aligns with the intended environment?

- What risks might arise if style and context are not aligned?

Critical Thinking Questions

- How do furniture styles reflect the cultural or historical context in which they were developed?

- What considerations must an upholsterer keep in mind when recommending a style for commercial use?

- How can the same type of furniture (e.g. a sofa) differ significantly in design and function across different styles?

- Why is it important to understand furniture types and styles when restoring or reproducing historical furniture?

- How might a client’s lifestyle or setting influence your recommendation of a furniture type or upholstery material?

### KT0202: Concepts of Ergonomic Design and Standard Dimensions of Upholstered Furniture

Theoretical Learning Content

Ergonomics refers to the science of designing furniture that fits the human body and its movements, ensuring comfort, efficiency, and safety. In furniture upholstery, ergonomics plays a key role in shaping seating angles, cushioning support, backrest height, armrest placement, and overall usability.

Designing upholstered furniture that meets ergonomic standards improves user comfort, reduces physical strain, and increases the functional value of the product. Understanding standard dimensions helps ensure consistency in manufacturing and compliance with user expectations across different environments, whether residential, commercial, or institutional.

1. Ergonomic Principles in Upholstered Furniture

- Posture Support: Backrests should support the natural curve of the spine. Seat depth and height must allow the feet to rest flat on the floor.

- Pressure Distribution: Cushioning should distribute body weight evenly to reduce stress on pressure points, especially in seating.

- User Fit: Furniture should accommodate the anthropometric data (body measurements) of the intended user group (e.g. children, elderly, adults).

- Movement and Accessibility: Armrests, reclining angles, and seat height should assist users in sitting down and standing up with ease.

2. Standard Dimensions (Industry Guidelines)

While dimensions may vary by region, purpose, and user needs, the following are widely accepted as standard measurements for adult seating furniture:

| Feature | Standard Dimension (mm) |
| --- | --- |
| Seat Height (from floor) | 430–460 mm |
| Seat Depth | 450–550 mm |
| Seat Width | 450–600 mm (single seat) |
| Backrest Height | 450–600 mm |
| Armrest Height | 180–250 mm above seat |
| Sofa Total Height | 800–900 mm |
| Recline Angle (Backrest) | 95–110° |

Custom designs may deviate from these figures depending on user needs and aesthetic choices.

3. Importance of Ergonomics in Upholstery

- Improves comfort and reduces fatigue

- Promotes proper posture and reduces musculoskeletal issues

- Enhances user satisfaction and product value

- Ensures usability across different environments (e.g. office chairs vs. lounge sofas)

- Aligns with health and safety standards in institutional and commercial settings

Examples

- A reception area chair designed with firm cushions and higher seat height assists elderly users in standing up safely.

- A lounger sofa in a luxury hotel is designed with deep seats and angled backrests for prolonged relaxation.

- Office seating includes lumbar support and armrests positioned to prevent shoulder strain during long hours of sitting.

Case Study

Case Study: Comfortline’s Custom Ergonomic Sofa

Comfortline Upholstery was commissioned to design a waiting room sofa for a physiotherapy clinic in Johannesburg. The clients were mainly recovering patients, many with back and leg injuries. The design team used ergonomic data to select a firmer cushion foam, increased the seat height, added reinforced lumbar support, and ensured the seat edge did not compress underweight. After installation, client feedback indicated significant improvement in comfort and ease of use.

Discussion Points:

- Why did the team avoid soft, low-set cushions in this design?

- How does patient mobility influence the ergonomics of upholstered furniture?

Critical Thinking Questions

- What risks arise when furniture is designed without considering ergonomic principles?

- How do ergonomic requirements vary between domestic furniture and commercial or healthcare furniture?

- In what ways can upholstery materials (e.g. foam density, fabric type) affect ergonomic outcomes?

- Why is it important for upholsterers to understand standard dimensions when designing or altering furniture?

- How can ergonomics contribute to the success of a small upholstery business?

### KT0203: Main Furniture Construction Principles (Stability, Squareness, Ergonomics, Measurements, Conversions, etc.)

Theoretical Learning Content

The construction of quality furniture depends on a set of core principles that ensure the product is stable, correctly aligned, ergonomically suitable, and precisely measured. These principles are crucial for both functional integrity and aesthetic value. An understanding of construction principles enables upholsterers to identify faults early in the manufacturing process, leading to improved product lifespan and customer satisfaction.

1. Stability

Stability refers to the furniture's ability to remain firm, balanced, and secure under normal use. Key considerations include:

- Even weight distribution and proper load-bearing support

- Accurate fitting of joints and reinforcements

- Correct positioning of legs or base units

- Prevention of wobbling or tilting, especially in seating

2. Squareness

Squareness ensures that angles and joints meet correctly at 90 degrees (unless a specific design deviates intentionally). This is vital for:

- Visual alignment and professional finishing

- Structural balance

- Ensuring parts like drawers and cushions fit properly

Tools like set squares, callipers, and diagonal measurements help maintain squareness throughout frame construction.

3. Ergonomics

As covered in KT0202, ergonomic construction ensures that furniture supports the human body comfortably and safely. The application of ergonomic principles begins with frame measurements and affects:

- Seat depth and back height

- Armrest position

- Cushion firmness and profile shaping

4. Measurements and Accuracy

Accurate measurements are foundational to effective furniture construction. Key aspects include:

- Measuring dimensions in millimetres for precision

- Checking for consistency between design drawings and cut materials

- Accounting for allowances (e.g. foam compression, fabric stretch)

Measurements are often cross-referenced with technical drawings or templates.

5. Measurement Conversions

Upholsterers must sometimes convert between measurement systems:

- Imperial to Metric (inches to millimetres): 1 inch = 25.4 mm

- Volume and Area calculations for foam, fabric, or padding estimation

- Decimal to fraction conversions when reading certain international furniture specs

Examples

- A sofa frame that lacks internal corner blocks may appear square but fail under stress due to poor stability.

- A coffee table designed with incorrect angles may rock on uneven surfaces because it is not square.

- An armchair constructed with mismatched back and seat angles may cause discomfort and back strain due to poor ergonomic planning.

Case Study

Case Study: Quality Control at AfriHome Designs

AfriHome Designs manufactures contemporary furniture in Durban. During a recent production run, quality control flagged several ottomans that wobbled when placed on flat surfaces. Upon inspection, it was found that the internal bracing was uneven and the corner joints were not square. A redesign of the jig template and refresher training on frame squareness was implemented. The revised approach reduced defect rates by 40 percent in the following month.

Discussion Points:

- How did the lack of squareness affect product quality?

- Why is regular tool calibration and skills refreshment important in frame construction?

Critical Thinking Questions

- How does squareness contribute to the structural and visual integrity of a furniture piece?

- What are the potential consequences of poor measurement practices in furniture upholstery?

- In what ways can stability be compromised even when measurements appear correct?

- Why must an upholsterer understand both metric and imperial measurements?

- How can attention to construction principles give a competitive advantage to small-scale furniture manufacturers?

### KT0204: Furniture Manufacturing Processes Flow Including Materials, Parts Identification, Machining, Joints and Assembly, Frame Preparing, Upholstery, Using the Correct Terminology

Theoretical Learning Content

The furniture manufacturing process follows a structured flow from raw materials to the final upholstered product. Each stage in the process plays a specific role in ensuring the quality, functionality, and finish of the item. A clear understanding of the steps involved and the correct terminology is essential for effective communication, planning, and collaboration in the upholstery and furniture manufacturing sector.

1. Overview of Manufacturing Flow

The general sequence in furniture manufacturing typically includes the following stages:

- Design and Planning

  - CAD drawings, patterns, or templates

  - Selection of materials, finishes, and hardware

- Materials Procurement

  - Timber (solid wood, plywood, MDF)

  - Foam, fabric, webbing, springs, nails, glue

- Machining

  - Cutting, shaping, and sanding wood to specification

  - Drilling and routing for joinery

- Joints and Assembly

  - Assembling frame components using joints (e.g. mortise and tenon, dowel, biscuit)

  - Gluing, clamping, and screwing components

- Frame Preparation

  - Inspection and sanding

  - Application of corner blocks, braces, and reinforcements

- Upholstery Preparation

  - Application of webbing and springs

  - Padding with foam, wadding or traditional materials

  - Cutting and attaching upholstery fabrics or leathers

- Finishing and Quality Control

  - Trimming, buttoning, edging

  - Final inspection and fault correction

  - Labelling and packaging

2. Correct Terminology

Using accurate terminology ensures professional communication across departments:

- Frame: The wooden structure that forms the base of the furniture

- Webbing: Straps used for foundational seat support

- Tensioning: The process of stretching webbing or springs correctly

- Padding: Layers of foam, wadding or natural stuffing applied over the frame

- Tack strip: A rigid strip used to create clean fabric edges

- Skirting: A fabric or decorative trim around the base of upholstered furniture

- Mitre joint: A corner joint cut at an angle, commonly 45°, for aesthetic finishing

Understanding and applying the correct terminology enhances coordination between machinists, assemblers, upholsterers, and quality controllers.

Examples

- During machining, a cross-cut saw is used to cut wood to length, while a band saw may be used for curved sections.

- In assembly, a dowel joint may be used to ensure the frame remains square and stable.

- During upholstery, the use of a tack strip creates a clean, straight edge on the outside back of a chair.

Case Study

Case Study: Workflow Review at Sizwe Upholstery Works

Sizwe Upholstery Works in Polokwane began receiving complaints about inconsistencies in their armchair production. An internal review revealed that new employees were unfamiliar with standard terms used in work orders and failed to follow the correct order of operations. The manager instituted a workflow training programme, including illustrated process maps and a glossary of terms. Within two weeks, the error rate dropped significantly and team communication improved.

Discussion Points:

- What were the risks of miscommunication in Sizwe’s factory?

- How can documentation and terminology training reduce production faults?

Critical Thinking Questions

- Why is it important to follow a defined manufacturing process flow in furniture production?

- How can incorrect use of terminology affect quality and productivity?

- In which stages of the furniture manufacturing flow is an upholsterer most involved?

- How can knowledge of joints and machining benefit someone primarily working in upholstery?

- What strategies can be used to ensure all staff understand and use correct manufacturing terminology?

### KT0205: Furniture Manufacturing Process from Raw Wood to Finished Product

Theoretical Learning Content

The transformation of raw wood into a finished piece of upholstered furniture is a multi-step process that requires precise coordination, specialised skills, and adherence to quality standards. Understanding this end-to-end process enables upholsterers to appreciate the craftsmanship and technical considerations that influence the structure, durability, and appearance of the final product. It also promotes collaboration with other departments in a manufacturing or workshop environment.

1. Stages in the Furniture Manufacturing Process

- Selection and Preparation of Raw Wood

  - Hardwood (e.g. Meranti, oak) and softwood (e.g. pine) are selected based on strength, cost, and design requirements.

  - Timber is seasoned (air-dried or kiln-dried) to reduce moisture content and prevent warping.

- Cutting and Machining

  - Wood is cut to length and thickness using saws and planers.

  - Machining includes shaping, routing, drilling, grooving, and sanding.

- Joinery and Assembly

  - Components are joined using appropriate methods: dowels, mortise and tenon, screws, nails, or specialised fasteners.

  - Glue is applied, and clamps are used to set the joints.

- Frame Construction

  - The wooden frame is assembled to form the foundation of the furniture piece.

  - Reinforcements like corner blocks are added for stability and squareness.

- Sanding and Surface Preparation

  - Surfaces are sanded to ensure smoothness and remove imperfections.

  - Surfaces may be sealed if they will remain visible (e.g. show-wood chairs).

- Upholstery Process

  - The frame is webbed, and springs may be added depending on the seat design.

  - Padding materials (foam, batting, or horsehair) are applied.

  - Fabric or leather coverings are cut, sewn, and attached using staples, tacks, or tack strips.

- Finishing

  - Visible wood components are stained, lacquered, or painted.

  - Skirtings, decorative trimmings, and finishing touches are added.

- Inspection and Packaging

  - Quality control is conducted to check for defects or inconsistencies.

  - Finished furniture is cleaned, tagged, and packaged for dispatch or display.

2. Integration Across Departments

This process involves:

- Machining Department: Responsible for cutting and shaping wood

- Joinery Department: Assembly and preparation of the frame

- Upholstery Department: Application of springs, padding, and covers

- Finishing Department: Painting, staining, or sealing

- Quality Control Department: Final inspections and rectifications

Collaboration and shared knowledge between departments ensure efficiency and product consistency.

Examples

- A dining chair begins as lengths of kiln-dried pine. These are machined, joined, and reinforced into a frame. After upholstery, only the padded seat and show-wood legs remain visible.

- A sofa may incorporate both solid wood and manufactured board. The frame is assembled, padded with multi-density foam, covered in stitched fabric, and trimmed with piping or buttons.

Case Study

Case Study: From Forest to Lounge – The Journey of a Sofa

GreenRoots Furniture in Mpumalanga prides itself on sourcing sustainable timber from local plantations. The team documents each stage of the process, from the raw log to the final showroom piece. By tracing the lifecycle of their signature three-seater sofa, they educate customers on the craftsmanship and time required in production. Each sofa passes through the design office, cutting floor, joinery line, upholstery bay, and finishing station before reaching the client.

Discussion Points:

- How does transparency in the production process add value to the final product?

- What challenges might arise in coordinating departments to maintain this workflow?

Critical Thinking Questions

- Why is it important for upholsterers to understand the full furniture production process?

- At which stages of the process could faults most seriously affect the quality of the final upholstered product?

- How does wood preparation (e.g. drying) impact the longevity of the finished furniture?

- What are the benefits and challenges of integrating sustainability practices into the manufacturing process?

- How can good interdepartmental communication improve efficiency and quality in a furniture factory?

### KT0206: Concepts of Furniture Construction Including Frame Construction, Construction of Padding and Upholstery Components

Theoretical Learning Content

Understanding the concepts of furniture construction is essential for upholsterers, as the quality and longevity of the upholstered product rely heavily on the structural integrity of the frame and the appropriate use of padding and upholstery layers. Construction concepts influence comfort, stability, cost, aesthetic appeal, and compliance with safety standards.

1. Frame Construction

The frame is the skeletal structure of the furniture. It determines the overall shape, provides support, and holds the other components together. Key concepts include:

- Materials: Hardwoods (e.g. oak, Meranti) are often used for strength; softwoods (e.g. pine) may be used for economy.

- Joinery: Strong joints such as mortise and tenon, dowel, biscuit, or corner blocks are used to ensure durability.

- Reinforcements: Corner blocks, bracing, and gussets add strength and stability, especially in load-bearing areas.

- Squareness: Ensures accuracy in alignment to prevent distortion or weakness in the final product.

- Design Considerations: Curves, angles, and ergonomic shaping are built into the frame from the beginning.

2. Construction of Padding Components

Padding contributes to comfort, aesthetics, and functionality. It also shapes the upholstered form.

- Primary Layer – Springs and Webbing

  - Webbing: Jute, elastic, or synthetic straps provide foundation support.

  - Springs: Coil springs (traditional) or zig-zag springs (modern) are attached for comfort and bounce.

- Secondary Layer – Foam and Stuffing

  - Foam: High-density or multi-layer foam provides support and structure.

  - Batting: Polyester or wool batting is used over foam to create smooth contours.

  - Natural Fillings: Horsehair or coir fibre is used in traditional upholstery.

- Shaping Materials

  - Cardboard panels, tack strips, and edge rolls are used to create clean lines and firm edges.

3. Upholstery Components

- Upholstery Fabric: Selected for its aesthetic appeal, durability, and suitability to use context.

- Lining and Backing Materials: Calico, dust covers, or lining cloths protect internal components and improve finish.

- Trimmings: Piping, buttons, or decorative nails complete the visual style.

- Fastening Techniques: Staples, tacks, Velcro, or zips ensure secure and concealed fixing.

Examples

- A lounge chair frame built with dowelled joints and hardwood rails can support heavy use in hospitality settings.

- A modern office chair may use moulded plywood with high-resilience foam and polyurethane fabric for ergonomic contouring.

- A traditional armchair may include hand-tied coil springs, horsehair padding, and deep-buttoned leather upholstery.

Case Study

Case Study: Mbali’s Quality Control Dilemma

Mbali oversees production at a small upholstery workshop in Limpopo. A batch of recliner chairs began showing signs of sagging after only six months. Upon investigation, it was found that frames were constructed using softwood without proper corner bracing, and low-density foam was used in the seat padding. Mbali responded by updating the frame design to include hardwood supports and specifying higher-grade foam for seat bases.

Discussion Points:

- How did the poor selection of construction materials compromise the product quality?

- What role did both frame and padding design play in the product's failure?

Critical Thinking Questions

- Why is it essential for upholsterers to understand the construction of the furniture frame?

- How do the selection and layering of padding materials influence comfort and durability?

- In what ways do construction faults in either the frame or the padding affect the reputation of a furniture brand?

- How does knowledge of traditional versus modern upholstery techniques assist in custom or restoration work?

- What impact does the construction method have on the cost and labour time of an upholstered piece?

### KT0207: Impact of Design and Construction Faults

Theoretical Learning Content

Design and construction faults in furniture and upholstery can have far-reaching consequences, including product failure, customer dissatisfaction, increased returns, warranty claims, and reputational damage. Understanding the causes and impacts of these faults equips upholsterers to anticipate, identify, and prevent common errors during the design, frame-building, or upholstery stages.

1. Common Design Faults

Design faults originate in the conceptual or technical drawing stages. They include:

- Poor ergonomics: Designs that do not support correct posture or user comfort

- Inadequate structural planning: Shapes or forms that place excessive stress on weak points

- Non-standard dimensions: Oversized or undersized furniture that does not suit the intended space or user

- Over-complicated styling: Designs that are impractical to build or upholster, leading to inconsistency

2. Common Construction Faults

These faults occur during the physical manufacture and assembly of the furniture:

- Weak joints: Loose mortise and tenon, poorly glued dowels, or missing screws

- Poor squareness: Frames that are not correctly aligned, causing visible distortion

- Incorrect material usage: Using softwoods in load-bearing areas or low-density foam in high-use seating

- Inaccurate measurements: Resulting in misfitting components or uneven surfaces

- Improper tensioning of webbing and springs: Leading to seat sagging and discomfort

3. Consequences of Faults

- Loss of structural integrity: Items break or collapse prematurely

- Customer complaints and returns: Affecting business credibility and profitability

- Increased repair costs: Wastage of materials and time

- Safety hazards: Especially in commercial or institutional environments

- Brand damage: Inconsistent quality erodes trust in the product or company

4. Fault Detection and Prevention

- Visual inspections at each stage of the manufacturing process

- Testing load-bearing areas for strength and alignment

- Quality checklists to verify measurements and assembly order

- Skilled workmanship and continuous training of staff

Examples

- A three-seater couch designed with minimal internal bracing begins to sag in the centre after three months.

- A dining chair with poorly aligned legs wobbles, which may cause injury and damage flooring.

- An ottoman covered in oversized patterned fabric shows design fault due to pattern misalignment across seams.

Case Study

Case Study: Fault Recall at uKhuselo Furniture

uKhuselo Furniture, a mid-size manufacturer based in KwaZulu-Natal, had to recall a range of tub chairs after receiving numerous complaints about broken legs. The quality team discovered that the legs were attached using only glue and a single screw on softwood frames. In response, the company redesigned the frame using hardwood and reinforced it with double dowel joints and metal brackets. The recall, however, resulted in a major financial setback and temporary reputational harm.

Discussion Points:

- What should have been done during the design phase to prevent this issue?

- How could clearer quality protocols have averted the recall?

Critical Thinking Questions

- What types of faults are most likely to be overlooked during furniture construction, and why?

- How does ergonomics link to both design and construction quality?

- Why is it important for upholsterers to understand the structural elements beneath the padding and fabric?

- In what ways can early detection of design or construction faults save time and money?

- What processes can small businesses implement to reduce the occurrence of common upholstery faults?

### KT0208: Problem Solving

Theoretical Learning Content

Problem solving in the context of furniture upholstery and construction involves identifying faults, inconsistencies, or inefficiencies in design, materials, construction methods, or processes—and finding appropriate, cost-effective solutions. This competence is essential for ensuring quality control, meeting production deadlines, maintaining client satisfaction, and preventing material waste.

Effective problem solving requires a combination of technical knowledge, critical thinking, communication, and initiative. It is also essential that upholsterers can assess the root cause of a problem, not just its symptoms.

1. Common Problem Areas in Upholstery Manufacturing

- Material defects: Faulty foam, warped timber, flawed fabric

- Measurement errors: Incorrect dimensions leading to poor fit

- Construction faults: Weak joints, unsquare frames, misalignment

- Equipment failure: Broken staplers, blunt cutting tools

- Design impracticality: Aesthetic features that are hard to produce or upholster

- Time delays: Bottlenecks in the workflow due to miscommunication or poor planning

2. Steps in a Problem-Solving Process

- Identify the problem

  - Observe the symptom (e.g. wobble, sag, wrinkle)

  - Ask clarifying questions (e.g. Where? When? How often?)

- Analyse the cause

  - Investigate measurements, materials, equipment, or process flow

  - Consult work orders, specifications, or quality logs

- Generate possible solutions

  - Brainstorm feasible responses with relevant team members

  - Consider cost, time, and material implications

- Implement the solution

  - Apply changes, test modifications, or rework components

- Evaluate the result

  - Monitor for recurrence or secondary effects

  - Document the learning to avoid future repeats

3. Qualities of an Effective Problem Solver

- Detail orientation and strong observation skills

- Clear understanding of manufacturing and upholstery processes

- Willingness to take responsibility

- Communication and teamwork

- Ability to balance speed, quality, and cost

Examples

- A sofa frame arrives from the joinery department slightly out of square. The upholsterer identifies the issue before padding and returns the frame for correction.

- Fabric tensioning results in wrinkles on a backrest. The upholsterer adjusts the cutting pattern to allow for stretch and re-applies the fabric using a tack strip.

- A batch of foam collapses prematurely. After investigation, the team changes supplier and introduces incoming goods inspection protocols.

Case Study

Case Study: Resolving a Pattern Mismatch

At Lawndale Custom Upholstery in the Eastern Cape, a client returned a bespoke armchair because the printed fabric pattern was misaligned at the front seams. The upholsterer, Sipho, examined the cutting templates and realised they did not account for the fabric’s large repeat. Sipho corrected the templates, re-cut the panels, and successfully reupholstered the chair. He also documented the issue in a template update log for future orders with patterned fabric.

Discussion Points:

- What was the root cause of the problem?

- How did Sipho’s actions demonstrate problem-solving skills?

Critical Thinking Questions

- Why is it important to involve multiple team members in solving production-related problems?

- What steps can be taken to prevent recurring measurement or cutting errors?

- How does a culture of problem solving contribute to quality improvement in upholstery workshops?

- In your view, what distinguishes a good decision from a rushed or reactive fix?

- Why is it essential to document the solutions to technical problems?

## Integrated Assessment – KM-04-KT02

Furniture Types, Styles, and Construction
Total Marks: 60

Assessment Instruments Overview

| Instrument Type | Purpose |
| --- | --- |
| Matching Terminology | Assess understanding of industry terms (IAC0201) |
| Labelled Diagram Exercise | Identify furniture parts using correct terminology (IAC0202) |
| Short Written Response | Evaluate mass production workflow (IAC0203) |
| Case Study Application | Analyse small business furniture production (IAC0204) |
| Problem-Based Scenario | Demonstrate construction principles and quality reasoning (IAC0205) |

✍️ Assessment Activities

Activity 1: Matching Terminology (IAC0201 – 10 marks)

Instructions: Match each furniture industry term in Column A with the correct definition in Column B.

| Column A |  | Column B |
| --- | --- | --- |
| 1. Webbing | A | A strip used for clean upholstery edges |
| 2. Mortise and tenon | B | A method of covering internal framework with foam and fabric |
| 3. Upholstery | C | A traditional wood joint where one piece fits into a hole in another |
| 4. Tack strip | D | Fabric, leather, or vinyl used for final covering of padded furniture |
| 5. Show-wood | E | Exposed wood frame areas finished for decorative effect |
| 6. Batting | F | Soft fibre placed between foam and outer fabric for smoothness |
| 7. Frame | G | The internal wooden structure that forms the base of a furniture piece |
| 8. Ergonomics | H | Design principle focused on user comfort and body alignment |
| 9. Piping | I | Decorative trim usually placed along fabric seams |
| 10. Dowelling | J | Cylindrical rods used to reinforce wooden joints |

Activity 2: Labelled Diagram (IAC0202 – 10 marks)

Instructions: Identify and label the following parts on the diagram of a typical upholstered armchair. Use correct furniture construction terminology.
(Provide learners with a diagram showing: armrest, leg, seat base, backrest, frame, padding, webbing, tack strip, dust cover, and fabric cover.)

Activity 3: Short Written Response (IAC0203 – 10 marks)

Instructions: In approximately 120–150 words, describe the process of manufacturing furniture through mass production from raw wood to finished product. Your response should reference at least four departments involved in the workflow and describe their contribution.

Activity 4: Case Study and Scenario-Based Questions (IAC0204 – 15 marks)

Case Study
Sibongile’s Studio is a small upholstery business operating in the Free State. She designs and builds custom ottomans for boutique hotels. Her process includes sourcing wood, cutting and assembling frames, preparing upholstery materials, and hand-finishing each unit. She has recently taken on an apprentice who must learn how to follow the production flow efficiently.

Questions:

- List five steps Sibongile must follow from the start of production to completion. (5)

- What challenges might Sibongile face that a large factory does not? Provide two examples. (4)

- Why might a boutique client choose Sibongile’s small business rather than a larger manufacturer? (3)

- Suggest one way Sibongile could improve workflow while maintaining quality. (3)

Activity 5: Problem-Based Scenario (IAC0205 – 15 marks)

Scenario:
You are assessing a lounge chair built by a junior staff member. The seat sags when weight is applied, and one leg feels unstable. On inspection, you discover poor-quality webbing, no corner bracing, and visible misalignment.

Questions:

- Identify and explain three construction faults in this product. (6)

- What construction principles have been ignored in this case? (3)

- How would you rectify the faults to ensure product quality? (3)

- What is the potential impact of these issues on client satisfaction and business reputation? (3)

✅ Model Answers

Activity 1: Matching Terminology

1 - G, 2 - C, 3 - B, 4 - A, 5 - E, 6 - F, 7 - G, 8 - H, 9 - I, 10 - J

Activity 2: Labelled Diagram

Correctly labelled items (1 mark each): Armrest, Leg, Seat Base, Backrest, Frame, Padding, Webbing, Tack Strip, Dust Cover, Fabric Cover

Activity 3: Short Response

Sample points to include:

- Design stage (drawings and measurements)

- Machining department: cutting, shaping timber

- Joinery department: assembling frame

- Upholstery: webbing, springs, padding, fabric

- Finishing department: quality control, packaging

Activity 4: Case Study

- Source wood, cut components, assemble frame, add padding and upholstery, finish and inspect

- Limited tooling capacity; manual processes take longer

- Personalised service, handmade uniqueness

- Use modular templates or standardise fabric-cutting procedures

Activity 5: Problem-Based Scenario

- Weak webbing, lack of corner blocks, misalignment of legs

- Principles ignored: stability, squareness, measurement accuracy

- Replace webbing, reinforce frame, realign legs and brace joints

- Risk of returns, negative reviews, and reduced trust in the brand

Marking Memo

| Activity | Max Marks | Marking Notes |
| --- | --- | --- |
| Activity 1 | 10 | 1 mark per correct match |
| Activity 2 | 10 | 1 mark per correctly labelled part |
| Activity 3 | 10 | Clear explanation, logical structure, mentions at least 4 steps |
| Activity 4 | 15 | Allocate per question as listed; full marks for correct examples |
| Activity 5 | 15 | Reasoning and understanding of principles must be evident |
| Total | 60 |  |

📊 Assessment Rubric (Summary)

| Score Range | Descriptor |
| --- | --- |
| 5: Excellent | Clear, precise, technically accurate; full understanding of content |
| 4: Good | Mostly accurate with minor errors; good structure and reasoning |
| 3: Satisfactory | Adequate understanding; some conceptual or technical errors |
| 2: Basic | Partial understanding; lacks detail or accuracy in terminology or process |
| 1: Poor | Misunderstood task; vague or incorrect answers; lacks technical vocabulary |

### Facilitator Assessment Briefing

KM-04-KT02: Furniture Types, Styles, and Construction
SAQA ID: 103199 – Furniture Upholsterer | NQF Level 3 | Weight: 15%

🎯 Purpose of the Assessment

This integrated assessment is designed to measure learner competence against the Internal Assessment Criteria IAC0201 to IAC0205. The focus is on the learner’s ability to:

- Use correct terminology for furniture components and upholstery

- Identify structural parts of furniture and describe their purpose

- Explain the full furniture manufacturing process in both mass production and small business contexts

- Apply furniture construction principles to evaluate and solve quality-related faults

The assessment combines recall, application, analysis, and problem-solving and aligns with the required outcomes for Knowledge Module KM-04-KT02.

📝 Assessment Instruments Overview

| Instrument Type | Targeted IACs | Skills Assessed |
| --- | --- | --- |
| Matching Terminology | IAC0201 | Knowledge of industry terms |
| Diagram Labelling | IAC0202 | Visual recognition and correct terminology |
| Short Written Response | IAC0203 | Conceptual and process understanding |
| Case Study Analysis | IAC0204 | Practical application in SME context |
| Problem-Based Scenario | IAC0205 | Analytical reasoning and fault correction |

🛠 Facilitation Guidelines

Before the Assessment

- Ensure that all content for KT0201 to KT0208 has been covered in class.

- Review key terminology, construction principles, and workflow stages.

- Provide learners with examples of well-labelled diagrams and model short responses during revision.

During the Assessment

- Clearly explain each activity type and the number of marks allocated.

- Remind learners to label all sections, write clearly, and use correct technical vocabulary.

- For the diagram activity, provide either printed or digital diagrams with space for labelling.

- Monitor learners for independent work and integrity in responses.

After the Assessment

- Use the provided marking memo and model answers to guide marking.

- Moderate a sample of marked scripts for consistency.

- Record and retain learner performance results as per your centre’s assessment policy.

- Provide feedback to each learner on strengths and areas for improvement, especially regarding terminology use and logical reasoning in problem-solving.

📚 Evidence Requirements

Each learner’s submission must include:

- Completed answer sheet or assessment booklet for Activities 1 to 5

- Labelling of the upholstered armchair diagram

- Clearly numbered and structured written responses

- Learner name, date, and assessor signature on each script

These submissions will form part of the learner’s internal summative evidence for KM-04.

Scoring Summary

| Activity | Marks Allocated |
| --- | --- |
| Activity 1: Matching Terms | 10 |
| Activity 2: Diagram Labelling | 10 |
| Activity 3: Written Response | 10 |
| Activity 4: Case Study | 15 |
| Activity 5: Problem Scenario | 15 |
| Total | 60 |

Assessment marks must be converted to reflect the 15% weighting in the Knowledge Module.

📊 Moderation Notes

- Internal moderation should sample at least 25% of the submissions.

- Ensure that marking aligns with the rubric and that assessment judgments are fair, valid, and reliable.

- Document any adjustments made and submit a moderation summary with the results.

## KM-04-KT03: Equipment and Tools Used in Advanced Upholstery of Furniture (15%)

Purpose of the Knowledge Topic

The purpose of this knowledge topic is to equip learners with a comprehensive understanding of the various types of tools used in advanced upholstery, including hand tools, power tools, and pneumatic tools. Learners will gain the theoretical competence to identify, use, maintain, and store these tools correctly and safely, following both manufacturer instructions and workplace safety protocols.

This knowledge module underpins the practical application of advanced upholstery techniques by ensuring that learners can interpret pressure settings, apply routine maintenance procedures, select correct staple and nail sizes, and follow Material Safety Data Sheets (MSDS) and labelling protocols. It also reinforces the importance of using lockout devices and safety guards to prevent injury in the upholstery workspace.

Key Knowledge Areas

This topic will cover the following content areas:

- KT0301: Types and uses of hand tools

- KT0302: Types and uses of power tools

- KT0303: Types and uses of pneumatic tools

- KT0304: How to use the tools correctly and safely

- KT0305: Settings including pressure settings

- KT0306: Maintenance of different tools (sharpening, lubrication, tension, pressure, etc.)

- KT0307: Correct grease is used on the tools

- KT0308: Correct size staples and nails are used on the machine

- KT0309: Storage of tools

- KT0310: Reading labels and MSDS

- KT0311: Manufacturer specifications and instructions

- KT0312: Lockout devices and safety guards

Internal Assessment Criteria and Weight

Learners will be assessed against the following Internal Assessment Criteria:

- IAC0301: Tools are sharpened and stored correctly

- IAC0302: Tools are used according to manufacturer instructions

- IAC0303: Caution is exercised while moving and working with tools

- IAC0304: Correct pressure is set on the machine

- IAC0305: Safety pins on the tools are not broken

- IAC0306: Labels on the blade are observed to correspond with the machine specifications

Weight: 15%

Application in Upholstery Practice

Proficiency in identifying, operating, and maintaining advanced upholstery tools directly impacts the quality, safety, and efficiency of the work produced. Whether working in a small upholstery studio or a large manufacturing facility, learners must understand the relationship between tool functionality, material compatibility, and operational safety. This knowledge topic prepares learners to uphold industry standards, minimise risks, and contribute to a productive and well-maintained upholstery environment.

### KT0301: Types and Uses of Hand Tools

Theoretical Learning Content

Hand tools are essential to the practice of upholstery and are used in virtually every stage of the process, from frame preparation to the final finish. These tools do not require electricity or pneumatic power and are operated manually. Understanding the correct use and maintenance of hand tools ensures accuracy, efficiency, safety, and quality control in the production of upholstered furniture.

1. Common Upholstery Hand Tools and Their Uses

| Tool | Use |
| --- | --- |
| Staple remover | Removes staples and tacks from old upholstery without damaging the frame |
| Tack hammer | Drives small tacks or nails into the frame to secure webbing or fabric |
| Claw tool | Removes stubborn tacks or staples; often used in re-upholstery |
| Upholstery needles | For blind stitching, sewing buttons, or fixing corners |
| Shears or scissors | Cuts fabric, foam, and lining materials with accuracy |
| Rubber mallet | Taps frames or padding into place without damaging surfaces |
| Measuring tape/ruler | Measures dimensions of materials and furniture components |
| Chalk or fabric marker | Marks cutting lines and reference points on fabric |
| Webbing stretcher | Stretches webbing evenly across frames to provide proper tension |
| Sewing awl | Makes holes or assists in stitching in heavy fabrics and leather |

2. Characteristics of Quality Hand Tools

- Comfortable and ergonomic grip to reduce fatigue

- Made from durable, rust-resistant materials

- Sharp cutting edges for efficiency and clean results

- Proper weight distribution for controlled use

3. Best Practices for Use

- Always check that tools are in good condition before use

- Use the right tool for the job to prevent damage to materials or injury

- Follow correct techniques for tensioning, marking, and removing fasteners

- Return tools to their designated storage immediately after use

Examples

- A webbing stretcher ensures that the seat webbing is tight and evenly tensioned, providing proper support for padding and fabric.

- A claw tool used incorrectly can damage the wooden frame or injure the operator’s hand.

- Shears used only for fabric will retain their sharpness longer and produce cleaner cuts.

Case Study

Case Study: Thando’s Efficiency Boost

Thando is an apprentice upholsterer at a workshop in Mthatha. He noticed that the team often used general-purpose pliers to remove staples, which damaged frames and took longer. After receiving proper training, Thando introduced the consistent use of staple removers and claw tools. The workshop reduced rework time and material wastage. His attention to proper hand tool usage earned recognition from the foreman and improved overall productivity.

Discussion Points:

- How did Thando's choice of tools influence the quality and speed of work?

- What does this tell us about the importance of tool selection and technique?

Critical Thinking Questions

- Why is it important to differentiate between tools used for preparation and those used for finishing?

- How can incorrect use of a hand tool lead to poor workmanship or safety risks?

- What factors should be considered when selecting hand tools for a specific upholstery task?

- How does the maintenance of hand tools contribute to the long-term success of an upholstery business?

- In what ways can apprentices be encouraged to build good tool habits from early in their careers?

### KT0302: Types and Uses of Power Tools

Theoretical Learning Content

Power tools are electrically operated tools that assist upholsterers in tasks requiring speed, strength, or precision. They are especially useful in cutting, shaping, fastening, and finishing materials during upholstery processes. The appropriate use and maintenance of power tools significantly enhance productivity, reduce fatigue, and ensure consistency in professional furniture manufacturing environments.

1. Common Power Tools Used in Upholstery

| Power Tool | Primary Use |
| --- | --- |
| Electric staple gun | Drives staples into wood frames quickly and securely for fabric or webbing |
| Electric scissors | Cuts multiple layers of fabric or thick materials with precision and speed |
| Power drill | Creates holes for fixing components or assembling furniture frames |
| Sanding machine | Smooths wooden surfaces before finishing or upholstery begins |
| Jigsaw | Cuts curved or complex shapes in wood, MDF, or plywood |
| Electric screwdriver | Tightens or loosens screws during frame assembly or dismantling |
| Heat gun | Activates adhesive or softens materials for shaping (e.g. foam manipulation) |

2. Safety Features and Handling

- Use tools with protective covers and safety switches

- Always inspect cords and plugs before operation

- Operate only with dry hands and stable footing

- Wear personal protective equipment (PPE): gloves, goggles, dust masks

- Switch off and unplug before changing accessories or cleaning

3. Advantages of Power Tools in Upholstery

- Increased efficiency in repetitive or labour-intensive tasks

- Greater precision in cutting and fixing

- Consistent finish quality

- Reduced operator fatigue and time wastage

Examples

- An electric staple gun allows for faster application of fabric compared to manual tacking, reducing strain on the wrist.

- A power drill is used to install legs or brackets accurately into upholstered furniture frames.

- A heat gun assists in shaping foam or activating glue on vinyl coverings for a smooth, professional finish.

Case Study

Case Study: Accelerating Assembly at Moeti Designs

Moeti Designs, a growing upholstery business in Gaborone, introduced electric screwdrivers and jigsaws into its assembly and cutting sections. The team, previously reliant on hand tools, underwent a safety training session. Over the next two weeks, assembly times improved by 40%, and rework decreased due to more precise fitting. Moeti’s manager ensured regular tool maintenance and allocated task-specific power tools to reduce tool-sharing delays.

Discussion Points:

- How did introducing power tools affect the team's workflow?

- What safety measures ensured the successful transition to power-assisted operations?

Critical Thinking Questions

- What are the risks of using power tools without following manufacturer guidelines?

- How can the wrong choice of power tool lead to damage in upholstered products?

- What should be included in a pre-operation checklist for electrical tools in upholstery?

- How does the use of power tools impact the consistency of workmanship in high-volume environments?

- In what ways can power tools support both efficiency and quality when used correctly?

### KT0303: Types and Uses of Pneumatic Tools

Theoretical Learning Content

Pneumatic tools are powered by compressed air and are commonly used in advanced upholstery workshops due to their speed, power, and durability. These tools offer high output with low operator fatigue and are particularly effective in high-volume manufacturing settings. Understanding how to correctly operate, maintain, and regulate pneumatic tools is essential for safe and efficient upholstery work.

1. Common Pneumatic Tools in Upholstery

| Tool | Primary Use |
| --- | --- |
| Pneumatic staple gun | Attaches fabric, webbing, or lining to frames with precision and speed |
| Pneumatic nail gun | Drives nails or brads into wooden frames for joining or reinforcement |
| Air ratchet wrench | Fastens bolts or nuts when assembling heavy-duty furniture components |
| Pneumatic screwdriver | Used for high-speed screw driving in frame construction |
| Compressed air blower | Cleans debris from workstations and machinery |

2. Benefits of Pneumatic Tools

- Efficiency: Faster than electric or manual counterparts for repetitive tasks

- Power-to-weight ratio: Lighter tools with greater output reduce operator strain

- Durability: Fewer internal moving parts than electric tools; long-lasting with proper care

- Precision: Better control over fastening depth and staple/nail positioning

3. Operational Considerations

- Requires a reliable air compressor with regulated pressure

- Correct air pressure (typically 80–120 PSI) must be maintained depending on the tool

- Hoses should be properly connected and checked for leaks

- Safety guards and pressure regulators must be in working condition

- Lubrication (oil) must be applied to moving parts to prevent tool failure

4. Safety Practices

- Disconnect tools from the air supply when changing staples, nails, or clearing jams

- Never point pneumatic tools at oneself or others

- Always wear eye and ear protection

- Ensure tools have intact safety pins and guards

- Release air pressure from the system when not in use

Examples

- A pneumatic staple gun can fire hundreds of staples quickly, allowing large furniture pieces to be upholstered in a fraction of the time required by hand tools.

- An air ratchet wrench is used to assemble chair bases or attach brackets efficiently in a production line.

Case Study

Case Study: Air-Powered Precision at Lungile Upholstery

Lungile Upholstery in East London upgraded to pneumatic tools after experiencing delays with worn-out electric staple guns. After installation of a central compressor system, staff received training on operating pressure settings, safety protocols, and tool lubrication. Productivity improved and tool-related injuries decreased. The workshop also introduced a tool maintenance log to ensure regular oiling and pressure checks.

Discussion Points:

- How did the introduction of pneumatic tools improve workflow and safety?

- Why was tool maintenance key to the long-term success of the upgrade?

Critical Thinking Questions

- What are the key differences between pneumatic and electric upholstery tools in terms of function and safety?

- Why is correct air pressure critical when operating pneumatic tools?

- How can improper use of pneumatic tools affect upholstery quality?

- What preventive maintenance practices extend the lifespan of pneumatic equipment?

- What are the risks of failing to use safety features like guards or pressure regulators?

### KT0304: How to Use the Tools Correctly and Safely

Theoretical Learning Content

The correct and safe use of upholstery tools is fundamental to ensuring product quality, operator wellbeing, and workplace efficiency. Whether using hand tools, power tools, or pneumatic tools, upholstery professionals must understand how to operate each tool appropriately, maintain safe posture and handling, and apply preventative safety measures to avoid injuries or damage to materials and equipment.

1. General Guidelines for Tool Use

- Read and understand the manufacturer’s instructions for each tool before use

- Select the right tool for the task to avoid forcing or misapplying tools

- Use tools in a stable, clean, and well-lit area

- Secure the workpiece (e.g. frame or fabric) to avoid slips or uneven results

- Maintain proper posture and grip to reduce fatigue and prevent strain

- Wear appropriate Personal Protective Equipment (PPE) such as gloves, eye protection, and ear protection (especially when using power or pneumatic tools)

2. Tool-Specific Usage Considerations

- Hand Tools:

  - Use sharp blades and check handles for cracks or loose fittings

  - Cut away from the body and never use tools for unintended purposes

- Power Tools:

  - Inspect cords, switches, and guards before use

  - Unplug when changing blades or cleaning

- Pneumatic Tools:

  - Confirm pressure settings before connecting

  - Disconnect from air supply before clearing jams or adjusting fittings

3. Key Safety Practices in Upholstery Workshops

- Regular Tool Inspections: Daily checks for wear, damage, and correct functioning

- Safe Storage: Keep tools in designated storage units to prevent damage and accidents

- Emergency Protocols: Know the location of first-aid kits, fire extinguishers, and emergency shut-off valves

- Training: New users must be trained on tool operation and supervised during early use

- Lockout/Tagout Procedures: Used for tools under repair or when maintenance is being carried out

4. Common Safety Hazards and How to Prevent Them

| Hazard | Prevention |
| --- | --- |
| Electric shock | Check for frayed cords, use dry hands, unplug before maintenance |
| Eye injury from flying staples | Wear safety goggles |
| Repetitive strain injury | Use ergonomically designed tools, take regular breaks |
| Accidental firing of pneumatic gun | Keep finger off trigger until positioned, use safety lock pins |
| Slips or falls | Keep floor clean and free of trailing hoses or cords |

Examples

- A worker using a pneumatic stapler without eye protection risks injury from flying staples if the tool jams or misfires.

- A blunt upholstery knife requires more force to cut, increasing the risk of slipping and hand injuries.

- A sanding machine without a dust mask can lead to respiratory irritation in enclosed workshop spaces.

Case Study

Case Study: Safety Awareness Saves Time at Malebo Upholstery

Malebo noticed that tool-related accidents were increasing in her Pretoria-based workshop. A review revealed that staff were skipping safety checks and misusing staplers and drills. She implemented a “tool of the week” training programme and introduced mandatory PPE compliance checks. Within one month, tool-related incidents decreased by 70%, and productivity improved as fewer interruptions occurred.

Discussion Points:

- What were the initial risks in Malebo’s workshop?

- How did proactive safety training and protocols improve the work environment?

Critical Thinking Questions

- Why is it essential to follow manufacturer instructions even for frequently used tools?

- How can poor tool handling affect both safety and the quality of the finished product?

- What should you do if a tool malfunctions or feels unsafe during use?

- Why is correct storage part of overall tool safety?

- How does consistent training influence a safety-conscious workshop culture?

### KT0305: Settings Including Pressure Settings

Theoretical Learning Content

In advanced upholstery, accurate tool settings are essential to ensure the effectiveness, safety, and precision of operations. Pneumatic tools, in particular, rely on correctly calibrated pressure settings to function optimally. Improper settings can cause underperformance, material damage, safety hazards, and reduced tool lifespan. Understanding and applying correct settings contributes to workplace efficiency and consistent product quality.

1. Importance of Correct Tool Settings

- Ensures optimal tool performance

- Prevents over-driving or under-driving staples and nails

- Reduces material wastage (e.g. torn fabric, splintered frames)

- Enhances operator safety and prevents tool failure

- Extends the life of the upholstery tool and compressor unit

2. Standard Pressure Settings for Pneumatic Tools

| Tool Type | Recommended Pressure Range (PSI) |
| --- | --- |
| Pneumatic staple gun | 80 – 100 PSI |
| Pneumatic nail gun | 90 – 120 PSI |
| Air ratchet wrench | 60 – 90 PSI |
| Spray gun (adhesives/finishes) | 30 – 50 PSI |

PSI = Pounds per Square Inch (unit of air pressure)

Always refer to the manufacturer’s specification for exact pressure settings for a specific tool model.

3. Pressure Regulator and Monitoring

- Regulators control the airflow from the compressor to the tool

- Pressure gauges indicate the air pressure being delivered

- Adjustments should be made incrementally and only when the tool is disconnected

- A pressure relief valve is a safety feature that protects against excess pressure buildup

4. Indicators of Incorrect Pressure Settings

| Issue | Possible Cause |
| --- | --- |
| Staples do not penetrate fully | Pressure too low |
| Staples drive through material | Pressure too high |
| Tool jams frequently | Inconsistent pressure or incorrect fastener type |
| Loud air bursts or recoil | Excess pressure or tool misuse |

5. Best Practices for Pressure Settings

- Calibrate gauges regularly

- Train staff to read and adjust pressure safely

- Turn off compressor and bleed air lines before maintenance

- Do not exceed tool pressure limits under any circumstances

- Maintain compressors and check hoses for leaks that may affect pressure

Examples

- A pneumatic nail gun set at too high a pressure may drive nails too deep, cracking the frame or piercing visible layers.

- If a stapler is set too low, the staples may stick out or fall out, compromising the fabric hold and requiring rework.

- A pressure regulator fault can cause inconsistent depth across a row of staples, resulting in a poor visual finish.

Case Study

Case Study: Pressure Precision at Baile Upholstery Co.

Baile Upholstery Co. noticed quality inconsistencies across identical products. Investigation revealed that each workbench had slightly different pressure settings, and staff were unsure how to make adjustments. Management installed centralised air regulation panels and colour-coded pressure zones. After training, the team achieved uniform staple penetration and reduced fabric damage, increasing productivity and client satisfaction.

Discussion Points:

- How did inconsistent pressure settings affect Baile’s production quality?

- What measures ensured uniformity and improved outcomes?

Critical Thinking Questions

- Why is it necessary to adjust pressure settings for different tools or tasks?

- What are the risks of ignoring the manufacturer's recommended pressure settings?

- How can a pressure setting issue lead to time and material wastage?

- What role does training play in the correct use of pressure regulators?

- How can pressure monitoring be integrated into daily quality control procedures?

### KT0306: Maintenance of Different Tools (Sharpening, Lubrication, Tension, Pressure, etc.)

Theoretical Learning Content

Routine and preventative maintenance of upholstery tools is vital to ensure their safe operation, accuracy, and durability. Neglecting tool maintenance can lead to reduced productivity, poor workmanship, increased wear and tear, and even injury. A well-maintained tool not only performs better but also extends the life of materials and minimises downtime in the upholstery process.

1. Categories of Tool Maintenance

| Maintenance Type | Description |
| --- | --- |
| Sharpening | Applies to cutting tools such as shears and blades to maintain clean cutting |
| Lubrication | Reduces friction in moving parts of power and pneumatic tools |
| Tension Adjustment | Ensures correct tension in fastener tools and webbing stretchers |
| Pressure Calibration | Maintains correct PSI settings for pneumatic tools for safety and consistency |
| Cleaning | Removes dust, adhesive, or fabric fibres that can impair performance |

2. Tool-Specific Maintenance Practices

| Tool | Maintenance Task |
| --- | --- |
| Shears and knives | Regular sharpening with a whetstone or sharpening tool |
| Pneumatic staplers/nailers | Lubrication with pneumatic tool oil; check for air leaks |
| Power drills and sanders | Dust removal; checking brushes and motor housing |
| Air compressors | Drain water trap; inspect hoses; clean filters |
| Webbing stretchers | Check handle tension; inspect surface condition |
| Upholstery hammers | Inspect heads for wear and handle security |

3. Maintenance Schedule and Logs

- Daily: Visual checks, wipe-down, tool placement, leak inspection

- Weekly: Sharpening, oiling, air pressure checks, filter cleaning

- Monthly: Deep clean, inspect for wear or replacement parts

- As needed: After malfunction or excessive strain

Maintaining a Tool Maintenance Logbook helps track servicing, identify recurring issues, and ensures accountability.

4. Consequences of Poor Maintenance

- Blunt tools result in frayed fabrics or uneven cuts

- Unlubricated pneumatic tools can seize or misfire

- Loose fittings may lead to misalignment or injury

- Inconsistent pressure causes staple overdrive or underperformance

- Increased operational costs from early tool replacement

Examples

- A pair of dull fabric shears causes jagged edges, which are difficult to hem and spoil visual appearance.

- A compressor with a clogged filter causes pressure drops, leading to inconsistent stapling depth.

- A nail gun that is not oiled regularly may jam or shoot double nails, damaging the frame.

Case Study

Case Study: Maintenance Matters at FurnitureTech

At FurnitureTech, a mid-sized upholstery firm in the Western Cape, an audit revealed frequent tool breakdowns and increased downtime. The production manager instituted a weekly maintenance rota and issued a checklist for each tool. Apprentices were trained to sharpen cutting tools and oil pneumatic devices. Within two months, tool reliability improved, repairs decreased, and upholstery quality became more consistent.

Discussion Points:

- What practical measures did FurnitureTech take to improve tool performance?

- How can apprentices contribute to long-term tool care in a workshop?

Critical Thinking Questions

- Why is tool maintenance considered part of quality assurance in upholstery?

- How can poor maintenance lead to both safety hazards and poor customer satisfaction?

- What should a good tool maintenance checklist include for daily use?

- How does the correct sharpening technique differ between upholstery knives and fabric shears?

- What role does documentation (e.g. maintenance logbooks) play in managing workshop operations?

### KT0307: Correct Grease is Used on the Tools

Theoretical Learning Content

Greasing is an essential component of tool maintenance in upholstery workshops, particularly for pneumatic and mechanical tools that contain moving parts. Using the correct type of grease ensures smooth operation, reduces friction and wear, and protects tools from corrosion. Incorrect greasing—or using the wrong type of lubricant—can damage tools, compromise safety, and lead to costly breakdowns.

1. Functions of Grease in Upholstery Tools

- Reduces friction between moving parts

- Prevents overheating during continuous tool use

- Minimises wear and tear, extending tool lifespan

- Protects internal components from rust and corrosion

- Maintains consistent tool performance under pressure

2. Types of Grease and Where They Are Used

| Grease Type | Characteristics | Typical Use in Upholstery Tools |
| --- | --- | --- |
| Lithium-based grease | General-purpose, water-resistant, high temperature | Common in power tools, staple guns |
| Silicone grease | Lubricates rubber parts, prevents drying/cracking | Air hose connections, gaskets, seals |
| Pneumatic tool grease | Specially formulated for air tools | Internal lubrication of pneumatic nailers |
| Multi-purpose grease | Used for hinges, handles, and simple mechanisms | Manual tools with basic moving components |

Note: Oil and grease are different. Some tools require oil (e.g. pneumatic staplers) while others need grease for longer-lasting lubrication.

3. Application Guidelines

- Always refer to the tool manufacturer’s manual for recommended grease type and schedule

- Apply grease to clean, dry surfaces only to avoid contamination

- Avoid over-greasing—excess can attract dust and clog moving parts

- Use the correct applicator (e.g. grease gun, brush, cloth)

- Store grease in a sealed container, away from heat or contaminants

4. Signs That Grease Needs Reapplication

- Tool movement feels stiff or resistant

- Increased noise or vibration during use

- Evidence of rust, wear, or overheating

- Manufacturer-recommended intervals have passed

Examples

- A pneumatic stapler may require lithium grease inside its cylinder to ensure the piston moves smoothly during repeated firing.

- Using silicone grease on rubber hose seals prevents drying and air leaks, which could disrupt pressure stability.

Case Study

Case Study: The Right Grease Saves the Line at Kuhle Upholstery

At Kuhle Upholstery in KwaZulu-Natal, a pneumatic nailer stopped functioning mid-production. Upon inspection, the technician found that a high-viscosity industrial grease—meant for automotive bearings—had been used instead of pneumatic tool grease. The thick grease clogged the internal valve. After replacing the parts and applying the correct product, the tool was restored. The incident led to the introduction of a labelled grease station and training on lubrication procedures.

Discussion Points:

- What mistake led to the tool failure?

- How did the workshop correct the issue and prevent recurrence?

Critical Thinking Questions

- Why is it important to use tool-specific grease instead of general-purpose alternatives?

- What risks might arise from using too much or too little grease?

- How can staff be trained to identify the correct grease for each tool?

- What are the differences in grease application for manual, pneumatic, and power tools?

- How can poor greasing affect workplace productivity and client satisfaction?

### KT0308: Correct Size Staples and Nails Are Used on the Machine

Theoretical Learning Content

In upholstery, selecting the correct size of staples and nails for pneumatic and power fastening tools is essential to achieve strong, neat, and durable joints. Incorrect fastener sizes can lead to structural failure, material damage, safety risks, and rework. This topic addresses how to identify, select, and apply the correct size fasteners based on tool specifications, material types, and upholstery requirements.

1. Understanding Staple and Nail Specifications

Staples and nails used in upholstery machines are identified by several key attributes:

- Crown width (for staples): The top of the staple that lies flat against the surface (e.g. 5 mm to 12 mm)

- Leg length (for staples): The length of the legs that penetrate the material (e.g. 6 mm to 16 mm)

- Gauge or diameter (for nails): Thickness of the nail shaft, usually specified in gauge (e.g. 18-gauge, 16-gauge)

- Length (for nails): Determines how deep the nail penetrates the material (e.g. 15 mm to 40 mm)

These measurements must match both the machine’s capacity and the material requirements.

2. Factors That Influence Fastener Selection

| Consideration | Impact on Fastener Choice |
| --- | --- |
| Material thickness | Thicker materials require longer staples or nails |
| Type of wood or frame material | Softer woods may split if fasteners are too long or thick |
| Fabric and padding combination | Longer staples are needed for multi-layer upholstery |
| Tool compatibility | Machines have maximum and minimum accepted sizes |
| Aesthetic visibility | Thin-gauge, short fasteners are preferred in visible or delicate areas |

3. Common Errors and Consequences

| Error | Consequence |
| --- | --- |
| Using staples that are too short | Poor holding strength; fabric or webbing loosens over time |
| Using nails that are too long | Risk of penetrating through the frame or visible surfaces |
| Mismatched gauge for the tool | Jamming, misfiring, or tool damage |
| Mixing brands or types of fasteners | Inconsistent driving performance or tool malfunction |

4. Best Practices

- Always refer to the tool’s instruction manual for recommended fastener sizes

- Perform test firing on scrap material to ensure penetration depth and holding strength

- Store staples and nails in clearly labelled containers

- Never force fasteners into a machine they were not designed for

- Maintain a standardised inventory to reduce errors and confusion

Examples

- A 16 mm staple may be appropriate for fastening thick padding to a hardwood frame, while a 10 mm staple may be used for attaching lightweight fabric to plywood.

- An 18-gauge brad nail is suitable for attaching trim without splitting the wood, whereas a 15-gauge nail may be needed for load-bearing joints.

Case Study

Case Study: Staple Size Mix-Up at Tshepo Upholstery

Tshepo’s workshop recently had a quality issue where webbing came loose from several chairs. Investigation revealed that the apprentice had used 6 mm staples instead of the specified 14 mm length. These shorter staples did not penetrate the frame deeply enough to hold under tension. Tshepo revised the fastener chart, colour-coded the storage bins, and introduced a supervisor checklist before production began.

Discussion Points:

- What error caused the failure in Tshepo’s production?

- How did organisation and training contribute to preventing recurrence?

Critical Thinking Questions

- Why is it important to match fastener size to both material thickness and tool capability?

- What safety concerns can arise from incorrect staple or nail size?

- How can a workshop ensure consistent and accurate use of fasteners by all team members?

- What role does documentation (e.g. fastener charts) play in preventing production faults?

- In what ways does the correct use of staples and nails affect the long-term durability of the furniture?

### KT0309: Storage of Tools

Theoretical Learning Content

Proper storage of upholstery tools is critical for maintaining their condition, ensuring workshop safety, improving productivity, and reducing costs associated with damage or loss. Upholstery environments often involve sharp, pressurised, or electrically powered tools that require safe, dry, and well-organised storage systems to ensure they remain functional and accessible.

1. Importance of Proper Tool Storage

- Protects tools from damage: Prevents rust, dulling, cracking, or misalignment

- Reduces safety risks: Avoids accidents caused by tripping, falling, or grabbing tools incorrectly

- Improves workflow: Tools are easy to find, return, and use consistently

- Promotes accountability: Clear systems ensure all tools are returned and maintained properly

- Extends tool life: Tools stored in optimal conditions last longer and perform more reliably

2. Storage Methods by Tool Type

| Tool Type | Recommended Storage Method |
| --- | --- |
| Hand tools (e.g. hammers, scissors) | Pegboards, shadow boards, toolboxes with foam inserts |
| Power tools | Locked cabinets or drawers; cords coiled neatly |
| Pneumatic tools | Wall-mounted holders or padded drawers with pressure disconnected |
| Air hoses and cables | Reel holders, clips, or wall brackets to avoid tangling or tripping |
| Grease/oil and fasteners | Labelled, sealed containers in ventilated, cool areas |

3. Best Practices for Tool Storage in Upholstery Workshops

- Designate clearly labelled storage areas for each type of tool

- Use shadow boards to visually indicate missing tools

- Ensure ventilation and moisture control to avoid corrosion

- Store heavy tools at waist height to reduce lifting strain

- Separate sharp tools from general items to avoid injury

- Implement end-of-day tool checklists and inspections

- Assign responsibility to individuals or teams for tool care

4. Common Consequences of Improper Storage

| Improper Practice | Consequence |
| --- | --- |
| Leaving tools exposed to moisture | Rust, corrosion, electrical failure |
| Storing pneumatic tools with pressure on | Pressure wear, unintended discharge, seal damage |
| Tools stored in piles | Blunting of blades, snapped handles, safety hazard |
| Lack of labelling | Wasted time, misplacement, duplication of equipment |

Examples

- A rubber mallet stored on a damp concrete floor absorbs moisture and becomes brittle, leading to cracking during use.

- Fabric scissors stored in an open box with general hardware may become dull and lose precision.

- A pneumatic staple gun left connected to the air supply overnight may develop pressure leaks or malfunction unexpectedly.

Case Study

Case Study: Organised Workshop, Organised Work – Ntombi’s Solution

Ntombi manages an upholstery studio in Gauteng. Staff frequently misplaced tools, causing delays and small injuries due to scattered sharp tools. She implemented a shadow board system with photos and labels, issued personal tool kits for apprentices, and introduced a 10-minute daily closing routine for cleaning and tool checks. Within weeks, tool damage dropped, staff reported fewer injuries, and productivity improved noticeably.

Discussion Points:

- How did Ntombi’s storage solution support both safety and efficiency?

- Why is daily routine reinforcement important in tool management?

Critical Thinking Questions

- How does proper tool storage contribute to overall quality in an upholstery workshop?

- What risks are associated with leaving power or pneumatic tools plugged in or pressurised?

- In what ways can a workshop balance accessibility with security in storing expensive tools?

- What is the impact of poor tool organisation on a team’s time management?

- How can learners and apprentices be included in the responsibility of maintaining tool storage systems?

### KT0310: Reading Labels and MSDS

Theoretical Learning Content

In the upholstery workshop, being able to correctly read and interpret labels and Material Safety Data Sheets (MSDS) is vital for safety, compliance with legislation, and correct handling of tools, adhesives, finishes, lubricants, and other chemical-based materials. Labels and MSDS provide critical information about product identity, safe use, potential hazards, first-aid measures, and storage requirements.

1. What Is an MSDS?

A Material Safety Data Sheet (MSDS) is a document that provides detailed safety information about a chemical substance or product. It is legally required for all potentially hazardous substances used in the workplace.

Each MSDS typically includes the following sections:

- Product and company identification

- Hazard identification

- Composition/information on ingredients

- First-aid measures

- Firefighting measures

- Accidental release measures

- Handling and storage

- Exposure controls and personal protection

- Physical and chemical properties

- Stability and reactivity

- Toxicological information

2. Reading and Interpreting Product Labels

Labels on products such as spray adhesives, solvents, oils, and fasteners often include:

- Product name and batch number

- Usage instructions

- Hazard symbols (e.g. flammable, corrosive, irritant)

- Precautionary statements (e.g. “Use in well-ventilated area”)

- Storage and disposal information

- Expiry dates or pressure ratings

Understanding these symbols and statements ensures the product is used safely and correctly.

3. Workplace Responsibilities

- All staff must be trained to read and understand MSDS content relevant to their duties

- MSDS must be accessible on-site, either digitally or in printed form

- Labels must never be removed, defaced, or ignored

- Hazardous substances must be clearly labelled and stored according to MSDS requirements

- Employers must ensure compliance with the Occupational Health and Safety Act (OHSA) in South Africa

4. Benefits of Correct Interpretation

- Prevents accidents and health hazards (e.g. chemical burns, respiratory issues)

- Improves emergency response (first-aid, fire extinguishing, chemical spills)

- Ensures legal compliance with labour and environmental legislation

- Promotes responsible handling and disposal of chemical products

Examples

- A spray adhesive with a flammable symbol must be stored away from heat and open flames and used with proper ventilation.

- A lubricant label warns of skin irritation and instructs the user to wear gloves and wash hands after use.

- An MSDS for solvent-based contact glue indicates that the product should be used with a respirator and not stored in direct sunlight.

Case Study

Case Study: A Close Call at Nathi Upholstery

At Nathi Upholstery in Limpopo, a new staff member used a flammable aerosol adhesive next to a running heat gun. A flash fire occurred, but was quickly contained. The investigation revealed the employee had not read the product label or MSDS. Following the incident, Nathi organised a compulsory MSDS training session, displayed hazard posters in all work areas, and introduced an induction programme for new staff. No further incidents occurred.

Discussion Points:

- What critical safety steps were missed before the incident?

- How did the corrective actions reduce future risks?

Critical Thinking Questions

- Why is it not enough to rely only on visual inspection when handling chemicals and adhesives in the workshop?

- What challenges might arise when MSDS are not accessible or up to date?

- How do MSDS contribute to safe storage and correct disposal procedures?

- What role should supervisors play in reinforcing safe label reading practices?

- How can apprentices be supported in learning how to interpret product safety information?

### KT0311: Manufacturer Specifications and Instructions

Theoretical Learning Content

Manufacturer specifications and instructions refer to the detailed guidance provided by the tool or equipment manufacturer regarding the correct and safe use, maintenance, and limitations of their products. These documents are essential in advanced upholstery environments where tools and machinery must operate efficiently and safely under varying production conditions.

Failure to follow manufacturer instructions can result in injury, tool damage, voided warranties, and substandard workmanship. Learners must be trained to consult and apply these specifications as part of their day-to-day responsibilities.

1. What Are Manufacturer Specifications?

Manufacturer specifications typically include:

- Operating instructions

- Tool compatibility (e.g. fastener size, pressure rating)

- Recommended maintenance schedules

- Lubrication and greasing instructions

- Storage requirements

- Warranty conditions

- Parts replacement and service intervals

- Troubleshooting guidelines

These are usually found in printed manuals, product packaging, labels, or available online.

2. Why Following Specifications Matters

| Benefit | Impact |
| --- | --- |
| Ensures tool is used within limits | Prevents overloading or misuse |
| Maintains product warranty | Most warranties are void if instructions are not followed |
| Optimises tool performance | Efficiency, accuracy, and safety are maintained |
| Guides appropriate tool selection | Avoids using incorrect fasteners or attachments |
| Improves maintenance planning | Helps create accurate schedules and avoids premature tool failure |

3. Common Misinterpretations and Risks

| Misstep | Consequence |
| --- | --- |
| Ignoring maximum air pressure rating | Tool may burst, misfire, or cause injury |
| Using non-approved staples or nails | Tool may jam or fail to deliver correct fastening depth |
| Applying incorrect grease or oil | Leads to internal damage or seizure of moving parts |
| Skipping break-in periods or warm-up routines | Accelerates wear and reduces tool life |

4. How to Access and Apply Specifications

- Always keep manuals in a central, accessible place in the workshop

- Refer to tool labels and tags for quick reference on-site

- Access digital manuals or QR code guides where applicable

- Discuss tool use and specifications during team briefings or induction sessions

- Keep a specification sheet binder near the tool area with updated information

Examples

- A pneumatic stapler must be operated between 90–100 PSI as per the manufacturer’s manual. Operating it at 120 PSI causes misfires and voids the warranty.

- A spray gun specifies that only water-based finishes may be used. Using solvent-based products without consulting the manual causes seal damage and clogs the nozzle.

- A multi-head screwdriver requires specific torque settings for assembling steel-reinforced frames. Ignoring this leads to stripped threads.

Case Study

Case Study: Lost Warranty at Dineo’s Design Studio

Dineo’s team purchased new pneumatic tools and stored the manuals without review. When one of the staple guns failed due to incompatible staple use, the supplier declined to honour the warranty. Afterward, Dineo created a laminated summary of critical tool specifications and displayed it on the tool cabinet. Staff now consult the reference before using any new tool, and error reports have decreased.

Discussion Points:

- What key mistake caused the tool failure and voided the warranty?

- How did the corrective action improve compliance and performance?

Critical Thinking Questions

- Why is it important to understand pressure and fastener compatibility as specified by the manufacturer?

- What are the risks of assuming all similar tools operate the same way?

- How can the workshop ensure ongoing access to tool specification documents?

- In what ways can misunderstanding specifications lead to cost implications for the business?

- How would you include specification training in a new apprentice’s induction?

### KT0312: Lockout Devices and Safety Guards

Theoretical Learning Content

Lockout devices and safety guards are essential components of workplace safety in upholstery environments, particularly where power and pneumatic tools are used. These systems are designed to prevent accidental start-up, protect users from moving parts, and ensure that equipment is only operated under safe conditions.

All learners working in upholstery production environments must understand the purpose, proper use, and legal requirements for lockout and guarding systems in order to prevent injuries and comply with occupational health and safety regulations.

1. What Are Lockout Devices?

Lockout devices are physical mechanisms that isolate energy sources (electrical, pneumatic, or hydraulic) to prevent tools or machines from being powered on during:

- Maintenance or repair

- Cleaning

- Tool or part replacement

- Fault inspection

A lockout device may be accompanied by a tagout, which is a visible warning label placed on the locked equipment.

2. What Are Safety Guards?

Safety guards are barriers or protective covers installed on tools and machines to prevent:

- Accidental contact with moving parts

- Ejection of staples, nails, or debris

- Unintended activation of triggers or mechanisms

Guards may be fixed, interlocked, or self-adjusting, depending on the tool and its function.

3. Importance of Lockout and Guarding Systems

| Reason | Impact on Safety and Performance |
| --- | --- |
| Prevents accidental activation | Reduces risk of injury during servicing or adjustment |
| Controls energy sources | Ensures system is discharged before tool is handled |
| Protects the operator during use | Shields body parts from rotating, firing, or crushing components |
| Meets legal requirements | Ensures compliance with OHSA and manufacturer standards |
| Promotes responsible tool maintenance | Ensures tools are only used when fully functional and safe |

4. Best Practices for Use

- Always engage lockout systems before performing maintenance

- Verify that all energy sources are fully isolated before beginning work

- Never bypass or disable safety guards on tools or machinery

- Report missing or damaged guards immediately

- Train all staff and apprentices in proper lockout-tagout (LOTO) procedures

- Keep spare lockout kits and padlocks in accessible locations

Examples

- A pneumatic nail gun fitted with a trigger safety prevents accidental firing when not in contact with the work surface.

- A sanding machine with an interlocked guard cover prevents operation when the cover is open.

- During cleaning, a compressor switch is locked out to ensure no pressure builds up unexpectedly.

Case Study

Case Study: Locked and Safe at Vuyani Upholstery

During a regular maintenance check at Vuyani Upholstery, a junior staff member attempted to inspect a pneumatic tool without isolating the compressor. Fortunately, a senior technician intervened and demonstrated the lockout process using the designated air shut-off valve and lockout tag. The workshop supervisor thereafter mandated refresher training and added visual lockout instructions to each tool station. Incidents dropped, and insurance compliance was improved.

Discussion Points:

- What risks were prevented by the senior technician’s actions?

- How did visual aids and training improve the workshop’s safety culture?

Critical Thinking Questions

- Why is it important to isolate all forms of energy before maintaining a power or pneumatic tool?

- What legal and safety consequences could result from bypassing a safety guard?

- How can a workshop incorporate lockout-tagout (LOTO) systems effectively with minimal disruption?

- What responsibilities do both employers and employees have in maintaining tool safety features?

- How can learners be assessed on their competence to use safety guards and lockout systems correctly?

## Integrated Assessment – KM-04-KT03

Equipment and Tools Used in Advanced Upholstery of Furniture
Qualification: Furniture Upholsterer (SAQA ID 103199)
Knowledge Topic: KM-04-KT03
Internal Assessment Criteria: IAC0301–IAC0306
NQF Level: 3
Weight: 15%
Total Marks: 60

🛠️ Assessment Instruments Overview

| Instrument Type | Purpose | Targeted IACs |
| --- | --- | --- |
| Matching Exercise | Test comprehension of tool labels and specifications | IAC0306 |
| Observation Checklist Task | Demonstrate tool use and storage procedures | IAC0301, IAC0302, IAC0303 |
| Diagram Interpretation | Identify safety features and pressure settings on a tool diagram | IAC0304, IAC0305 |
| Scenario-Based Short Answer | Apply understanding of safety, pressure, and tool handling | IAC0301–IAC0306 |

✍️ Assessment Activities

Activity 1: Matching Exercise – Tool Labels and Specifications (10 marks)

Instructions: Match each item in Column A with the most accurate description from Column B.

| Column A |  | Column B |
| --- | --- | --- |
| 1. Maximum pressure rating | A | Stapler will only fire when in contact with a surface |
| 2. Tool model label | B | The maximum safe air pressure input for the tool (e.g. 100 PSI) |
| 3. Safety guard | C | Lists manufacturer, serial number, and specification details |
| 4. Compatible staple sizes | D | Prevents accidental activation or injury |
| 5. Safety trigger mechanism | E | Acceptable length and width of staples/nails for this model |

Activity 2: Observation Checklist – Practical Task (15 marks)

Instructions: The learner must demonstrate the following actions under supervision. Use the checklist to score.

| Task Performed | Yes/No | Marks |
| --- | --- | --- |
| A. Selects appropriate sharpening method for a hand tool |  | 3 |
| B. Returns tools to correct labelled storage area |  | 2 |
| C. Uses pneumatic stapler as per manufacturer guidance |  | 3 |
| D. Handles tool cautiously while moving between workstations |  | 2 |
| E. Checks air pressure before operation of tool |  | 3 |
| F. Confirms intact safety pin or trigger lock is in place |  | 2 |

Activity 3: Diagram Interpretation – Tool Pressure Settings and Safety Pins (10 marks)

Instructions: Study the diagram of a pneumatic stapler. Answer the following:

- Identify the area where pressure is regulated. (2)

- Label the location of the safety pin or trigger mechanism. (2)

- What is the maximum pressure allowed according to the label? (2)

- What might happen if the tool is operated without the safety guard in place? (2)

- State one reason why a safety pin must never be removed or bypassed. (2)

Activity 4: Scenario-Based Short Answer Questions (25 marks)

Scenario: You are assigned to inspect and operate a pneumatic stapler that has recently jammed. Upon inspection, you notice it has been over-lubricated, the pressure gauge is reading 130 PSI (above the recommended 100 PSI), and the safety pin is loose.

Answer the following questions:

- What should you do immediately before operating this tool? (3)

- What risk does the high pressure pose to the tool and operator? (4)

- How would you restore the tool to a safe operating condition? (5)

- Why is over-lubrication a concern, and how should you apply lubricant correctly? (5)

- If the tool label lists only certain staple sizes, why is it important not to use others? (4)

- What should be done if the safety pin is damaged or missing? (4)

✅ Model Answers

Activity 1 Answers

1 – B, 2 – C, 3 – D, 4 – E, 5 – A

Activity 2 Expected Observations

- A: Uses sharpening stone safely, tests sharpness – ✔

- B: Places tools in labelled shadow board – ✔

- C: Aligns tool properly, reads instructions – ✔

- D: Keeps tool facing downward, trigger safe – ✔

- E: Adjusts PSI on compressor before use – ✔

- F: Confirms lock pin is functional before starting – ✔

Activity 3 Model Answers

- Regulator dial located at air hose connection

- Safety pin or trigger at the base of handle

- Maximum pressure = 100 PSI

- Without the safety guard, staples could misfire, causing injury

- Safety pins prevent accidental firing; removal is a hazard

Activity 4 Model Answers

- Disconnect tool, inspect and reset pressure, read label

- High pressure can overdrive staples, damage material, or cause misfire

- Drain pressure, check PSI regulator, inspect safety pin, clean tool

- Excess oil may clog internals and attract dust; apply a few drops only

- Incorrect staples may jam, damage tool, or not hold material properly

- Tag out the tool, report the fault, and replace the safety component

Marking Memo

| Activity | Max Marks | Marking Guidelines |
| --- | --- | --- |
| Activity 1 | 10 | 2 marks per correct match |
| Activity 2 | 15 | 2–3 marks per task; assess both technique and safety |
| Activity 3 | 10 | 2 marks per correct and clearly stated answer |
| Activity 4 | 25 | Award full marks for safety-focused, logical, and accurate explanations |
| Total | 60 |  |

📊 Assessment Rubric

| Rating | Descriptor |
| --- | --- |
| 5: Excellent | Demonstrates comprehensive understanding and consistently applies safety |
| 4: Good | Good understanding with minor inconsistencies; follows safety with reminders |
| 3: Satisfactory | Moderate understanding; some application errors; minimal safety awareness |
| 2: Basic | Partial understanding; misses key principles or practices |
| 1: Poor | Lacks understanding; unsafe or inappropriate handling |

### Facilitator Assessment Briefing

KM-04-KT03: Equipment and Tools Used in Advanced Upholstery of Furniture
Qualification: Furniture Upholsterer (SAQA ID: 103199)
NQF Level: 3
Weight: 15%

🎯 Purpose of the Assessment

This integrated assessment is designed to evaluate learner competence against the Internal Assessment Criteria IAC0301 to IAC0306. The focus is on assessing learners’ ability to:

- Correctly sharpen and store tools

- Operate tools as per manufacturer instructions

- Apply safe handling practices

- Set pressure correctly on machines

- Check the condition of safety pins

- Verify tool labels against machine specifications

A variety of instruments have been used to allow for fair, valid, and comprehensive assessment of theoretical and practical understanding within the context of an upholstery workshop.

Assessment Instruments Overview

Assessment Instruments include:

- Matching Exercise – Terminology and specification identification

- Observation Checklist – Practical demonstration of tool use and storage

- Diagram Interpretation – Safety features and pressure settings

- Scenario-Based Short Answer Questions – Application of safety principles and corrective actions

Facilitation Guidelines

Before the Assessment

- Ensure that KT0301 to KT0312 content has been fully delivered.

- Revise tool use, maintenance, pressure setting, safety features, and interpretation of MSDS and labels.

- Allow learners to practise correct handling and use of pneumatic and power tools.

During the Assessment

- Conduct the activities in a clean, safe, and supervised workspace.

- Clearly explain instructions and allocate sufficient time for all components.

- Observe practical tasks directly and use the provided checklist.

- Verify learner understanding through both written and practical responses.

After the Assessment

- Use the marking memo and rubric to ensure objective and consistent scoring.

- Moderate at least 25% of the assessment tasks.

- Record learner performance and provide individualised feedback.

- Store completed assessments securely for moderation and verification.

📚 Evidence Requirements

Each learner submission must include:

- Completed answers for Activities 1 to 4

- A signed observation checklist

- Diagrams and written responses clearly labelled

- Learner’s name, date, and assessor signature on all documents

These form part of the summative internal evidence for KM-04.

🧮 Scoring Summary

| Activity | Marks |
| --- | --- |
| Matching Exercise | 10 |
| Observation Checklist | 15 |
| Diagram Interpretation | 10 |
| Scenario-Based Short Answers | 25 |
| Total | 60 |

Scores must be proportionally converted to reflect the 15% weight assigned to this Knowledge Module.

Moderation Notes

- Moderate a minimum of 25% of learner assessments.

- Confirm alignment with IAC0301 to IAC0306.

- Validate safety knowledge, practical ability, and use of correct tools and procedures.

- Document all moderation findings and file them with the assessment record.

## KM-04-KT04: Health and Safety in the Advanced Upholstery Department (15%)

Purpose of the Knowledge Topic

The purpose of this knowledge topic is to develop the learner’s understanding of the legal, procedural, and practical aspects of health and safety in an advanced upholstery production environment. This topic ensures that learners are equipped to identify risks, comply with legislation and internal protocols, respond appropriately to emergencies, and contribute to a safe and efficient workplace.

Learners will study the relevant legislative frameworks, understand the proper use of fire extinguishers and evacuation signage, and engage with the safe handling of tools, machines, hazardous substances, and incidents. The emphasis is on building safety awareness, accountability, and confidence to apply occupational health and safety procedures consistently and responsibly.

Key Knowledge Areas

This topic covers the following elements:

- KT0401: BCOE Act

- KT0402: Occupational Health and Safety (OHS) Act

- KT0403: General hazards in the workshop

- KT0404: Fire extinguishers and uses

- KT0405: Evacuation plans and signage in the workshop

- KT0406: Demarcation lines

- KT0407: Safe working procedures in the various machining operations

- KT0408: Machine safety (isolate the machine)

- KT0409: Personal safety (PPC and PPE)

- KT0410: Hazards and incidents

- KT0411: Lock-out devices and procedure

Internal Assessment Criteria and Weight

Learners will be assessed on their ability to:

- IAC0401: Carry out routines safely and in accordance with work requirements

- IAC0402: Report any faults identified with equipment to the relevant person

- IAC0403: Maintain a work area that is free from hazards

- IAC0404: Identify and confirm accessibility of fire extinguishers; ensure labels are intact

- IAC0405: Study and understand evacuation plans

- IAC0406: Adhere to demarcation lines and stack items in appropriate areas

- IAC0407: Confirm all machines are fitted with safety devices and free from foreign objects

- IAC0408: Test isolator and emergency buttons to ensure functionality

- IAC0409: Verify the availability of lock-out devices for all machines

- IAC0410: Use relevant personal protective equipment (PPE) in the workshop

- IAC0411: Report incidents and injuries to the relevant personnel

Weight: 15%

Application in the Upholstery Workshop

A strong safety culture is a hallmark of any successful upholstery enterprise. Learners must be aware of how to work safely with industrial tools, compressed air, sharp components, and combustible materials. By mastering the knowledge in this topic, learners contribute to legal compliance, reduction of workplace injuries, and creation of an efficient and responsible workshop environment. This knowledge is critical for sustaining productivity, protecting people and property, and promoting occupational professionalism.

### KT0401: BCOE Act

(Basic Conditions of Employment Act, 1997 – Act No. 75 of 1997)

Theoretical Learning Content

The Basic Conditions of Employment Act (BCOE Act) is a fundamental piece of South African labour legislation that governs minimum conditions of employment. While not specific to health and safety, it plays a crucial supporting role in establishing the foundation of fair and safe working environments, particularly in industrial settings such as upholstery workshops.

Upholstery learners and workers must understand how the BCOE Act interacts with occupational health and safety principles—ensuring their working hours, leave, rest breaks, and working conditions do not compromise physical or mental wellbeing.

1. Purpose of the BCOE Act

- To regulate basic employment conditions for all employees not covered by sector-specific bargaining councils

- To ensure fair labour practices are upheld across all industries

- To establish minimum rights regarding working hours, leave, payment, and termination

- To support a safe and healthy workplace through proper working time arrangements and fairness in the treatment of employees

2. Key Provisions Relevant to Health and Safety in Upholstery Work

| BCOE Provision | Relevance to Upholstery Environment |
| --- | --- |
| Working Hours (Section 9–10) | Excessive working hours increase fatigue and risk of injury in tool-heavy environments |
| Meal and Rest Breaks (Section 14) | Reduces repetitive strain and mental fatigue during long shifts |
| Overtime Limits (Section 10) | Protects workers from unsafe work conditions due to extended hours |
| Night Work (Section 17) | Special considerations must be made for lighting and safety at night |
| Annual Leave and Sick Leave | Ensures recovery and injury management are not compromised |
| Termination Notice Periods | Prevents sudden understaffing that may impact safe operation of machinery |

3. Rights and Responsibilities Under the BCOE Act

- Employers must:

  - Comply with maximum working hours and provide breaks

  - Maintain records of hours, leave, and contracts

  - Ensure that employees are not forced to work under unsafe fatigue-related conditions

- Employees must:

  - Adhere to working times and break periods

  - Report unsafe workloads or excessive overtime

  - Use rest periods to maintain alertness and readiness for manual tasks

Examples

- A machinist working a 12-hour shift without breaks is more likely to mishandle tools, increasing the risk of accidents—this is a BCOE violation.

- An employer denying sick leave after a back strain from lifting foam rolls may not only violate the Act but create further health risks.

Case Study

Case Study: Balance at Work – Compliance at Tsholo Upholstery

Tsholo Upholstery, based in Soweto, implemented a time-logging system that alerts staff after 5 continuous hours to take a mandatory lunch break. The system is aligned with the BCOE Act and reduces hand strain and fatigue-related errors. A Department of Labour inspector commended their commitment to integrating fair labour standards with workshop safety protocols.

Discussion Points:

- How did the employer align working conditions with legislative compliance?

- What benefits did this policy have on safety and productivity?

Critical Thinking Questions

- How does the BCOE Act support safer working conditions in physically demanding roles such as upholstery?

- Why is it important to enforce rest and break periods in a workshop environment?

- What can happen if learners or workers are unaware of their rights under the BCOE Act?

- How can excessive overtime impact the quality and safety of upholstery production?

- In what ways can small upholstery businesses practically implement BCOE compliance?

### KT0402: Occupational Health and Safety (OHS) Act

(Occupational Health and Safety Act, 1993 – Act No. 85 of 1993)

Theoretical Learning Content

The Occupational Health and Safety Act (OHS Act) is South Africa’s primary legislation for ensuring health and safety in the workplace. The Act aims to create a safe and healthy working environment for all employees and persons on site by placing duties on both employers and workers. In the context of the upholstery industry, where machinery, sharp tools, compressed air systems, and flammable materials are in constant use, understanding and applying the OHS Act is critical to protecting lives and maintaining compliance.

1. Purpose of the OHS Act

- To provide for the health and safety of persons at work

- To protect workers and visitors against hazards related to the use of tools, machinery, equipment, and materials

- To establish safety responsibilities for employers, employees, and health and safety representatives

- To promote preventative measures through inspections, training, and regulations

- To reduce workplace injuries and occupational illnesses

2. Key Provisions of the OHS Act Relevant to Upholstery

| OHS Act Provision | Application in the Upholstery Workshop |
| --- | --- |
| Section 8: Employer Duties | Maintain equipment, conduct risk assessments, ensure safe systems of work |
| Section 14: Employee Duties | Follow safety instructions, report hazards, use PPE, and avoid unsafe behaviour |
| General Safety Regulations (GSR) | Cover machine guarding, electrical safety, and fire prevention |
| Hazardous Chemical Substances Regulations | Apply when using adhesives, solvents, cleaning chemicals |
| First Aid and Emergency Procedures | Must be clearly displayed and understood by all personnel |

3. Responsibilities in Terms of the OHS Act

- Employers must:

  - Ensure that machinery is safe and regularly inspected

  - Provide Personal Protective Equipment (PPE) at no cost

  - Display safety signage and evacuation plans

  - Appoint and train Health and Safety Representatives

  - Record and report all incidents and injuries

- Employees must:

  - Use PPE correctly and consistently

  - Report any safety hazards or incidents immediately

  - Never interfere with or disable safety devices or guards

  - Follow all safety instructions and attend training

Examples

- A pneumatic stapler without a functional safety trigger must be taken out of use immediately as per OHS machinery safety requirements.

- A worker using solvent-based adhesive must wear gloves and a mask as required under the Hazardous Chemical Substances Regulations.

Case Study

Case Study: Averted Injury at Lesedi Upholstery

At Lesedi Upholstery, a floor supervisor noticed that the emergency stop button on the foam-cutting machine was loose. Because all staff were trained under the OHS Act requirements, the issue was reported and resolved before any incident occurred. The company logged the fault, initiated immediate repair, and updated their workshop hazard register.

Discussion Points:

- How did the employee’s training influence the safe outcome?

- What elements of the OHS Act were followed effectively?

Critical Thinking Questions

- What would be the consequences of failing to comply with the OHS Act in an upholstery environment?

- How do the employer and employee responsibilities work together to ensure workplace safety?

- Why is it important for all workers to understand what to do in an emergency or accident?

- What challenges might small upholstery workshops face in complying with the OHS Act, and how can they overcome them?

- How does the OHS Act promote a proactive safety culture in the furniture industry?

### KT0403: General Hazards in the Workshop

Theoretical Learning Content

In any upholstery environment, especially one using advanced equipment and industrial processes, numerous general hazards can pose risks to the health, safety, and wellbeing of workers. Recognising, mitigating, and managing these hazards is essential to prevent injuries, maintain productivity, and comply with the Occupational Health and Safety Act.

Learners must be able to identify both visible and hidden hazards in the upholstery workshop and follow procedures to report or correct them promptly.

1. Types of General Hazards in the Upholstery Workshop

| Hazard Category | Examples |
| --- | --- |
| Physical hazards | Sharp tools, hot glue guns, pneumatic pressure, moving machinery |
| Ergonomic hazards | Poor posture, repetitive tasks (e.g. stapling, sewing), heavy lifting |
| Chemical hazards | Adhesive fumes, solvent-based finishes, contact with skin-irritant substances |
| Electrical hazards | Damaged cords, overloaded plugs, ungrounded tools |
| Fire hazards | Flammable materials (e.g. foam, fabric, adhesives), blocked exits |
| Slips and trips | Spilled glue, trailing air hoses, loose fabric on floors |
| Noise hazards | Continuous operation of compressors and staple guns |
| Compressed air hazards | Airline detachment, unsafe pointing of nozzles, loud discharge |

2. Sources of Hazards

- Untrained personnel using power or pneumatic tools

- Poor housekeeping (e.g. clutter, blocked walkways, dirty work areas)

- Inadequate ventilation or lighting

- Defective or poorly maintained equipment

- Lack of clear signage or demarcation lines

- Non-use or incorrect use of Personal Protective Equipment (PPE)

3. Preventative Measures

- Conduct regular safety inspections

- Implement clear tool and material storage systems

- Train staff on hazard identification and response

- Ensure good housekeeping and waste management practices

- Maintain all machinery according to manufacturer guidelines

- Use proper signage and floor demarcation

- Ensure adequate PPE is available and used correctly

Examples

- A solvent-based adhesive stored near a heat gun poses a fire hazard.

- A damaged power cable on a sanding machine poses an electrical shock risk.

- A poorly ventilated gluing booth exposes staff to hazardous fumes over time.

Case Study

Case Study: A Trip Prevented at Naledi Upholstery

During a production rush, a roll of fabric was left unfastened in the walkway at Naledi Upholstery. A team member tripped but caught themselves before falling. The safety officer noted the near-miss and revised the workshop’s fabric storage protocol. Trailing hoses and unused rolls are now secured and stowed at all times.

Discussion Points:

- What type of hazard was present, and how was it prevented?

- What systems can be implemented to ensure such hazards are proactively managed?

Critical Thinking Questions

- Why is it important for every workshop employee—not just supervisors—to be trained in identifying hazards?

- What types of injuries could arise from poor workshop layout and clutter?

- How can hazard awareness improve both safety and efficiency in an upholstery environment?

- In your view, which types of hazards are most frequently overlooked, and why?

- How can near-miss reporting (like in the case study) contribute to a safer workplace culture?

### KT0404: Fire Extinguishers and Uses

Theoretical Learning Content

In an advanced upholstery workshop, the presence of flammable materials such as fabrics, foams, solvents, and adhesives creates a significant fire risk. Understanding the types of fire extinguishers, their correct application, and their placement and maintenance is critical for preventing fire-related injuries and damages.

All workers must be trained to identify, locate, and use the correct extinguisher depending on the class of fire. This knowledge also supports compliance with the OHS Act and workplace safety audits.

1. Fire Classes and Types of Fire Extinguishers

| Class of Fire | Description | Extinguisher Type |
| --- | --- | --- |
| Class A | Ordinary combustibles (wood, fabric, paper) | Water (Red), Dry Chemical (Red) |
| Class B | Flammable liquids (solvents, adhesives, petrol) | Foam (Cream), CO₂ (Black), Dry Chemical |
| Class C | Flammable gases (e.g. LPG) | Dry Chemical (Red) |
| Class D | Combustible metals (less common in upholstery) | Specialised Dry Powder |
| Class E (Electrical) | Electrical equipment and wiring | CO₂ (Black), Dry Chemical (Red) |
| Class F | Cooking oils and fats (not typical in upholstery) | Wet Chemical (Yellow) |

Upholstery workshops typically require CO₂, Dry Chemical, and Foam extinguishers.

2. Fire Extinguisher Colour Codes (South Africa)

| Colour Band | Extinguisher Type | Primary Use |
| --- | --- | --- |
| Red | Water or Dry Chemical | Class A, B, C fires |
| Black | Carbon Dioxide (CO₂) | Electrical fires (Class E) |
| Cream | Foam | Flammable liquid fires (Class B) |
| Yellow | Wet Chemical | Kitchen/cooking oil fires (Class F – not applicable here) |

3. Safe Use of Fire Extinguishers (PASS Method)

- Pull the pin

- Aim the nozzle at the base of the fire

- Squeeze the handle to discharge the extinguisher

- Sweep the nozzle side-to-side at the base of the fire

4. Placement and Maintenance

- Fire extinguishers must be:

  - Easily accessible and unobstructed

  - Mounted on walls at shoulder height, near exits or hazard zones

  - Clearly labelled, with intact seals and expiry dates visible

- Extinguishers should be serviced annually by an accredited professional

- Workshop personnel must be trained on location, type, and correct usage of extinguishers

Examples

- A CO₂ extinguisher is used on an electrical fire started by a sanding machine.

- A foam extinguisher is applied to a flammable liquid fire near a spilt adhesive container.

- A dry chemical extinguisher is used on a fire involving fabric and plastic foam.

Case Study

Case Study: Controlled Response at Zanele Upholstery

At Zanele Upholstery, a cloth soaked in adhesive was left near a heat gun and ignited. The team’s fire warden quickly identified the type of fire, used a foam extinguisher from the wall-mounted unit, and successfully extinguished the flames without injury. The extinguisher’s tag showed it had been serviced within the last six months.

Discussion Points:

- How did Zanele’s team respond correctly?

- What safety measures contributed to the successful handling of the incident?

Critical Thinking Questions

- Why is it important to match the correct extinguisher type to the class of fire?

- What are the risks of using a water-based extinguisher on an electrical or flammable liquid fire?

- How can regular fire extinguisher checks prevent disaster in an upholstery setting?

- What procedures should be followed if a fire extinguisher is found to be missing or tampered with?

- Why is fire extinguisher training necessary for every workshop employee, regardless of role?

Would you like me to proceed with KT0405: Evacuation Plans and Signage in the Workshop next?

### KT0405: Evacuation Plans and Signage in the Workshop

Theoretical Learning Content

Evacuation plans and safety signage are essential components of emergency preparedness in any workplace. In an advanced upholstery environment, where flammable materials, power tools, and compressed air systems are present, clear and accessible evacuation procedures and safety signs can mean the difference between order and chaos during a critical incident.

This topic helps learners understand the importance of emergency signage, how to interpret it correctly, and what to do in the event of an emergency requiring evacuation.

1. Purpose of Evacuation Plans and Signage

- To provide clear visual guidance during fire, gas leak, or electrical emergencies

- To ensure all personnel can exit safely and quickly from the workshop

- To comply with the Occupational Health and Safety Act and SANS (South African National Standards)

- To designate roles and responsibilities (e.g. fire warden, first aider)

- To support a culture of readiness and accountability

2. Key Components of an Effective Evacuation Plan

| Component | Description |
| --- | --- |
| Evacuation routes | Clearly marked, unobstructed pathways to exit points |
| Emergency exits | Doors leading to safe zones, well-lit, with panic-release mechanisms |
| Assembly points | Outdoor meeting locations for headcounts and communication |
| Roles and responsibilities | Who assists whom, checks rooms, and accounts for all personnel |
| Emergency contact details | Clearly listed for fire, ambulance, and workshop supervisors |
| Diagram-based layout | A floorplan posted in multiple visible areas of the workshop |

3. Types of Safety Signage in the Upholstery Workshop

| Sign Type | Purpose |
| --- | --- |
| Directional signage | Indicates evacuation routes and emergency exits (green signs) |
| Prohibition signs | Show what is not allowed (e.g. No Smoking – red circle) |
| Warning signs | Indicate hazards (e.g. Flammable materials – yellow triangle) |
| Mandatory signs | State required actions (e.g. Wear Safety Glasses – blue circle) |
| First aid and fire signs | Identify the location of equipment and personnel |

All signs must be SABS compliant, easily visible, clean, and securely fixed.

4. Roles During an Emergency

- All employees: Follow the evacuation route and go to the assembly point

- Appointed fire warden: Ensures all areas are cleared and assists others

- First aider: Attends to any injuries using the workshop first aid kit

- Supervisor or manager: Reports the incident and manages communication with emergency services

Examples

- A fire exit sign above the rear door leads to the open parking lot used as the assembly point.

- A floorplan posted near the time clock shows all staff how to reach emergency exits and locate extinguishers.

- A No Smoking sign near the flammable storage cupboard alerts staff to fire risk.

Case Study

Case Study: Fast and Safe at Sibusiso Upholstery

During a power surge that triggered a fire in the electrical panel, the staff at Sibusiso Upholstery evacuated using clearly marked emergency exits. Their weekly drills had prepared them well. At the assembly point, everyone was accounted for, and the fire department praised their preparedness. The floor manager updated the incident log and reported to management within 15 minutes.

Discussion Points:

- What role did preparedness and signage play in this successful evacuation?

- How could regular practice contribute to such an efficient outcome?

Critical Thinking Questions

- Why is it essential to conduct regular evacuation drills in an upholstery workshop?

- What risks are involved when signage is missing, unclear, or ignored?

- How can signage improve the safety of visitors or new staff unfamiliar with the workshop?

- What elements should you check when reviewing the condition and accuracy of emergency signage?

- How does a clear evacuation plan demonstrate responsibility and legal compliance in a business?

### KT0406: Demarcation Lines

Theoretical Learning Content

Demarcation lines are visual boundaries or markings used in workshops to separate zones of operation, define safe pathways, and ensure compliance with occupational health and safety standards. In an advanced upholstery environment, demarcation lines support the safe movement of people, storage of materials, and use of machinery by establishing clear rules for space utilisation.

Demarcation lines are a practical application of risk management—they prevent congestion, identify restricted areas, and reduce the likelihood of accidents caused by confusion or disorganisation.

1. Purpose of Demarcation Lines

- To separate high-risk zones (e.g. machinery areas) from walkways or storage

- To clearly define pedestrian routes through the workshop

- To indicate safe stacking zones for materials and equipment

- To identify no-go zones, maintenance areas, or emergency access points

- To assist in workflow planning, helping staff navigate the space safely and efficiently

2. Common Types of Demarcation Lines in Upholstery Workshops

| Colour | Purpose |
| --- | --- |
| Yellow lines | General walkways, caution zones, material movement paths |
| Red lines | Emergency access points, fire extinguishers, isolators |
| Green zones | Safety equipment locations (e.g. first aid kits, eyewash stations) |
| Blue markings | Mandatory instruction zones (e.g. PPE application area) |
| Black and yellow | Areas with moving machinery or where extra caution is needed |

Demarcation lines must comply with the OHS Act and SANS 1186 standards for safety signage and marking.

3. Responsibilities of Staff

- Keep within designated walkways during movement through the workshop

- Do not obstruct areas marked for emergency or safety use

- Stack materials only within marked storage zones

- Report faded, missing, or unclear markings to the supervisor

- Maintain cleanliness to ensure lines remain visible at all times

Examples

- A yellow line on the floor running from the cutting table to the storage area ensures staff moving heavy materials do not interfere with assembly activities.

- A red square demarcates the area around a wall-mounted fire extinguisher, which must remain unobstructed.

- Blue floor circles in the PPE station indicate where to stand while donning safety gear before entering the production zone.

Case Study

Case Study: Order and Safety at Bheki Upholstery

Bheki Upholstery introduced floor demarcation lines to address frequent congestion around the foam-cutting area. Yellow pathways were painted, and a red “No Storage” box was marked in front of the emergency exit. Staff were trained to respect the lines, and within a month, movement flowed more smoothly, stacking incidents were eliminated, and safety inspections noted marked improvement.

Discussion Points:

- What problems were solved by applying demarcation lines in this case?

- How did this contribute to improved safety and workflow?

Critical Thinking Questions

- How do demarcation lines support both health and safety and productivity in a busy upholstery workshop?

- What risks could arise if demarcation lines are ignored or allowed to fade?

- How should a workshop prioritise which areas to mark, and why?

- How can demarcation be used to improve safety for visitors or new employees?

- Who should be responsible for checking that lines are visible and correctly used?

### KT0407: Safe Working Procedures in the Various Machining Operations

Theoretical Learning Content

In the upholstery workshop, a variety of machining operations are carried out to prepare frames, cut materials, shape foam, and assemble furniture. These operations involve tools and machinery that pose significant risks if not handled according to established safety procedures. This topic focuses on promoting consistent, safe working practices during the use of cutting, drilling, sanding, pressing, and shaping equipment commonly found in an advanced upholstery department.

1. Purpose of Safe Working Procedures

- To prevent injuries such as cuts, burns, entanglement, and electric shock

- To reduce equipment damage due to misuse or operator error

- To maintain orderly and efficient workflow

- To support legal compliance with the OHS Act and General Safety Regulations

- To instil a safety-conscious mindset in all workshop personnel

2. Common Upholstery-Related Machining Operations

| Machining Operation | Associated Equipment | Safety Risk if Misused |
| --- | --- | --- |
| Frame cutting | Table saws, mitre saws | Amputation, kickback, blade contact |
| Hole boring and drilling | Pillar drills, handheld drills | Eye injury, clothing entanglement, hand injury |
| Foam shaping and trimming | Electric carving knives, band saws | Slips, hand injury, overheating of motor |
| Sanding and smoothing | Belt sanders, orbital sanders | Inhalation of dust, contact burns, noise exposure |
| Pressing and stapling | Pneumatic presses, staple guns | Recoil injuries, accidental firing, jamming hazards |

3. Safe Working Procedures – General Guidelines

- Conduct pre-use inspections (check for frayed cords, missing guards, blade tension)

- Use correct PPE (safety goggles, ear protection, dust masks, gloves)

- Keep hands clear of blades, moving parts, and pinch points

- Secure workpieces before cutting or drilling

- Use guards and safety features at all times

- Avoid distractions or unsafe behaviour while operating machinery

- Turn off and isolate equipment when not in use or before maintenance

- Follow manufacturer instructions for tool setup, use, and cleaning

4. Training and Supervision

- All operators must be trained and signed off as competent before operating machinery independently

- New learners or apprentices must be closely supervised during initial machine use

- Safety instructions and emergency shut-off procedures must be clearly displayed near each machine

Examples

- A foam shaper without a proper guide causes uneven cuts and increases the chance of finger injury.

- An unsecured frame on a mitre saw table may shift during cutting, leading to loss of control.

- A pneumatic stapler fired before making contact with the workpiece may rebound, causing hand injuries.

Case Study

Case Study: Supervised Safety at Mokgadi Upholstery

Mokgadi Upholstery introduced a coloured sticker system indicating which learners were certified to use which machines. This eliminated confusion about who could operate which equipment and allowed supervisors to monitor usage easily. As a result, incidents involving power tools dropped significantly, and learners reported feeling more confident under guided procedures.

Discussion Points:

- What risks were addressed by Mokgadi’s system?

- How did visual identifiers contribute to better safety management?

Critical Thinking Questions

- Why is it important for learners to be supervised when using high-risk equipment?

- How do standardised procedures improve safety during repetitive machining tasks?

- What consequences might arise from operating a machine without conducting a pre-use check?

- In what ways can poor lighting, noise, or clutter influence machine-related accidents?

- How can safe machine operation procedures be documented and reinforced daily in a busy upholstery environment?

### KT0408: Machine Safety (Isolate the Machine)

Theoretical Learning Content

Machine isolation is a critical component of workshop safety procedures. To isolate a machine means to disconnect it from all sources of power or energy before cleaning, maintenance, adjustment, or fault repair. In the upholstery workshop, where high-powered machinery and pneumatic systems are used daily, machine isolation is essential to prevent accidental activation, protect users, and comply with the Occupational Health and Safety Act.

1. What Is Machine Isolation?

Machine isolation refers to physically and electrically disengaging the machine from its energy source so that it cannot operate accidentally. Isolation procedures are typically followed before:

- Maintenance or repairs

- Blade changes or tension adjustments

- Clearing blockages or jams

- Replacing staples, nails, or lubricants

- Cleaning components or replacing filters

2. Types of Isolation Methods

| Type of Energy | Isolation Method |
| --- | --- |
| Electrical energy | Disconnect from mains power; switch off at breaker |
| Compressed air (pneumatic) | Close valve; bleed pressure; disconnect hose |
| Mechanical energy | Lock moving parts, engage emergency stop or mechanical lock |
| Stored energy | Release spring tension, drain compressed air, depressurise |

3. Safe Isolation Procedure (Basic Steps)

- Shut down the machine according to normal procedures

- Switch off the main power supply or disconnect air supply

- Lock out the isolator (where available) with a padlock or lockout tag

- Verify isolation by attempting to activate the machine (it should not start)

- Place a warning sign on the machine (“Do Not Use” or “Under Maintenance”)

- Release stored energy such as compressed air, rotating flywheels, or tension springs

- Only authorised personnel may restart machinery after isolation

4. When Must Machines Be Isolated?

- When performing routine maintenance or cleaning

- When foreign objects are lodged in moving parts

- When safety devices or guards need to be repaired or adjusted

- During power outages, if risk of unexpected restart exists

- Before training demonstrations involving exposed mechanisms

Examples

- A pneumatic staple gun should be disconnected from the air hose and bled of pressure before clearing a jam.

- A foam-cutting machine must be isolated from the electrical supply before replacing a blade.

- During deep cleaning, the compressor is isolated to prevent air line reactivation.

Case Study

Case Study: A Close Call at Sakhile Upholstery

At Sakhile Upholstery, a junior technician attempted to clear jammed foam from a sanding machine without isolating the machine. The machine restarted unexpectedly when another worker flipped the power switch, nearly causing serious injury. After the incident, the company introduced a mandatory lockout system, trained all staff on isolation procedures, and placed laminated signs on all machine stations.

Discussion Points:

- What isolation steps were missed in this situation?

- How did clear protocols and training help prevent future incidents?

Critical Thinking Questions

- Why is it not enough to simply switch off a machine without isolating it?

- How does stored energy in a machine pose a risk even when the power is off?

- What procedures should be followed to verify that isolation has been successful?

- Who should be authorised to isolate and restart machinery in the workshop?

- How can machine isolation procedures be reinforced for new staff and apprentices?

### KT0409: Personal Safety (PPC and PPE)

Theoretical Learning Content

In an upholstery workshop, workers are exposed to a variety of physical, mechanical, and chemical hazards. Ensuring personal safety requires the consistent and correct use of Personal Protective Clothing (PPC) and Personal Protective Equipment (PPE). These form the first line of defence against injury, exposure, and long-term health risks. All workers, including learners and apprentices, must be trained to use PPE appropriate to their role, tasks, and environment.

1. What Is PPC and PPE?

- Personal Protective Clothing (PPC) includes garments worn to protect the body from contact with harmful substances, surfaces, and environmental factors (e.g. aprons, overalls, flame-retardant suits).

- Personal Protective Equipment (PPE) includes items designed to protect specific parts of the body (e.g. gloves, safety glasses, respirators, ear protection).

2. Common PPC and PPE Used in Upholstery Workshops

| Item | Purpose |
| --- | --- |
| Safety boots | Protect feet from falling tools, tacks, and punctures |
| Cotton overalls | Prevent contact with dust, glue, foam particles, and oil |
| Safety goggles | Shield eyes from flying particles, dust, or stapler misfires |
| Ear protection (earmuffs) | Protect against long-term hearing damage from noisy tools |
| Respirators/masks | Filter fumes from spray adhesives and solvents |
| Gloves (varied types) | Protect against cuts, abrasion, adhesives, and mechanical injury |
| Hairnets or tied hair | Prevent hair from being caught in rotating machinery |

All PPC and PPE must be SABS-approved, well-fitted, and maintained in good condition.

3. Legal and Workplace Responsibilities

- Employers must:

  - Provide PPE free of charge

  - Train staff on how and when to use PPE

  - Replace worn-out or damaged PPE promptly

  - Ensure signage is displayed in PPE-required zones

- Employees must:

  - Wear the appropriate PPE at all times during tasks

  - Inspect equipment before use

  - Report defects or shortages immediately

  - Use PPE for its intended purpose and store it properly

4. Consequences of Not Wearing PPE

- Eye injuries from flying staples or particles

- Respiratory issues from inhaling adhesive fumes

- Hearing damage from long-term exposure to high-decibel machinery

- Cuts, abrasions, or burns due to unprotected handling of tools or chemicals

- Legal liability and non-compliance with OHS Act regulations

Examples

- A respirator is required when applying spray adhesive inside an enclosed booth.

- A cotton apron and gloves are worn while handling fabric soaked in solvent to prevent skin irritation.

- A learner using a band saw must wear goggles, a dust mask, and non-slip shoes to reduce injury risk.

Case Study

Case Study: Safety First at Ayanda Upholstery

During a safety audit, it was noted that several learners were not wearing gloves while using pneumatic staple guns. Ayanda Upholstery responded by hosting a refresher workshop, updating their PPE signage, and appointing a safety monitor. Incidents involving minor punctures and abrasions dropped, and learners became more aware of personal risk management.

Discussion Points:

- What led to the initial lapse in PPE usage?

- How did Ayanda Upholstery create a more proactive PPE culture?

Critical Thinking Questions

- How does the regular use of PPE promote both personal safety and workplace professionalism?

- What risks are involved when learners underestimate the need for specific PPE, such as gloves or goggles?

- How can small upholstery workshops manage PPE supply and replacement on a limited budget?

- Why is it important that PPE be properly fitted and stored correctly?

- What are the long-term health risks of failing to wear PPE when exposed to noise, dust, or chemicals?

### KT0410: Hazards and Incidents

Theoretical Learning Content

In an advanced upholstery workshop, hazards and incidents are inevitable if not actively managed. A hazard refers to anything with the potential to cause harm, while an incident is any unplanned event that results in—or could have resulted in—injury, illness, or damage. This topic prepares learners to proactively identify hazards, respond to incidents, and follow the correct reporting procedures in accordance with health and safety regulations.

1. Understanding Hazards

A hazard is any source of potential damage, harm, or adverse health effect. Hazards may be:

- Physical: Sharp tools, moving machinery, electrical components

- Chemical: Fumes from adhesives, solvents, and cleaning agents

- Ergonomic: Awkward postures, repetitive movements, manual lifting

- Biological: Dust or mould in fabric storage areas

- Environmental: Poor lighting, noise, or ventilation

2. Understanding Incidents

An incident is any unplanned event that could have, or did, result in:

- Injury or illness

- Equipment damage

- Fire, explosion, or environmental exposure

- Near-miss (a situation where no injury occurred but there was potential)

3. Incident Response Procedure

- Stop work immediately and ensure the area is safe

- Attend to any injured person(s) – call first aider if needed

- Report the incident to the supervisor or health and safety representative

- Secure the area to prevent further risk

- Record the incident in the incident register or reporting system

- Participate in an investigation if required

- Implement corrective actions to prevent recurrence

4. Reporting Requirements

- All incidents, no matter how minor, must be reported

- The incident register must include:

  - Date, time, and location of the incident

  - Description of what happened

  - Names of those involved and witnesses

  - Action taken

- Reports should be submitted promptly and handled confidentially

- Follow up must occur to ensure that corrective measures were effective

Examples

- A near-miss where a worker almost trips over an uncoiled air hose must be recorded and addressed through improved housekeeping.

- A chemical splash from misused cleaning fluid requires immediate first aid and investigation into PPE use and training gaps.

- An incident report that highlights repeated faults with a specific machine may trigger preventative maintenance or replacement.

Case Study

Case Study: Awareness Through Action at Lungi Upholstery

After a worker sustained a minor burn from a glue gun, Lungi Upholstery introduced a new reporting log and conducted weekly safety talks based on incident trends. Staff were encouraged to report all near-misses, which led to the discovery of a recurring tripping hazard near the foam storage. The area was reconfigured, signage was installed, and no further incidents were recorded.

Discussion Points:

- How did near-miss reporting improve workshop safety at Lungi Upholstery?

- Why is it important to investigate even seemingly minor incidents?

Critical Thinking Questions

- Why is it essential to report and record all hazards and incidents—even those that do not result in injury?

- How can incident reporting contribute to a safer workshop environment over time?

- What might discourage a learner from reporting a near-miss, and how can this be addressed?

- How can recurring incidents signal deeper problems in workshop systems or culture?

- In your opinion, what role does good communication play in reducing workshop incidents?

### KT0411: Lock-Out Devices and Procedure

Theoretical Learning Content

Lock-out devices and procedures are critical safety controls used to prevent the unintended start-up or release of energy from machinery during maintenance, cleaning, or repair. In an upholstery workshop—where power tools, compressed air, and rotating machinery are common—lock-out systems protect workers from serious injury or fatality. Learners must be trained to understand what lock-out means, when it must be used, and how to apply the correct procedures.

1. What Are Lock-Out Devices?

A lock-out device is a physical mechanism that prevents a machine from being started or powered while it is being serviced. These devices are installed on electrical switches, pneumatic valves, or machine control points and are usually accompanied by a tag-out system, which includes warning signs or labels.

2. Common Lock-Out Devices in Upholstery Workshops

| Device | Purpose |
| --- | --- |
| Padlock | Secures the machine isolator or air valve in the OFF position |
| Circuit breaker lock-out | Blocks electrical panel switches from being turned back on |
| Valve lock-out device | Prevents the opening of pneumatic air lines or compressors |
| Tag (tag-out) | Displays “Do Not Use” or “Under Maintenance” warnings |
| Hasp (multiple lock holder) | Allows several workers to place their padlocks on one isolation point |

3. Lock-Out Procedure

- Identify the machine or tool requiring servicing

- Inform all relevant staff and secure the area

- Shut down the machine as per standard procedure

- Isolate the energy source (electrical, pneumatic, etc.)

- Lock out the isolator using an approved device and personal padlock

- Attach a warning tag with your name, contact number, and reason for lock-out

- Verify isolation by attempting to start the machine

- Perform the maintenance or repair

- Remove the lock only when the work is complete and the area is clear

- Restart the machine following standard safe procedures

4. When Must Lock-Out Be Used?

- Before any maintenance, repair, or inspection

- When a tool is jammed or misfiring

- When safety guards or components need to be removed

- During machine cleaning or blade replacement

- Any time an unexpected startup would pose a risk

Examples

- A compressor is locked out using a valve device before replacing an air hose.

- A band saw is isolated and padlocked at the distribution board before a blade change.

- A glue press is locked and tagged when the electrical switch panel is being rewired.

Case Study

Case Study: Prevention Through Lock-Out at Jabu Upholstery

During a routine cleaning, a technician at Jabu Upholstery used a lock-out padlock on a pneumatic cutting machine’s air valve. Meanwhile, another worker attempted to activate the machine unaware of the maintenance. The lock-out system prevented start-up. This prompted the company to roll out lock-out kits at all workstations and provide formal training. No further incidents were recorded.

Discussion Points:

- What risk was eliminated by the technician’s use of a lock-out?

- How did system-wide implementation enhance workshop safety?

Critical Thinking Questions

- Why is it important to both lock out and tag out equipment?

- What could happen if a lock-out system is ignored or bypassed?

- Who should be authorised to remove a lock-out device?

- How can workshops ensure that lock-out procedures are followed consistently?

- What challenges might arise in training new staff to understand lock-out systems?

## Integrated Assessment – KM-04-KT04

Health and Safety in the Advanced Upholstery Department
Total Marks: 60
Assessment Weight: 15%

Assessment Instruments Overview

| Instrument Type | Purpose | Targeted IACs |
| --- | --- | --- |
| Observation Checklist | Practical demonstration of workshop safety routines | IAC0401, IAC0403, IAC0410 |
| Diagram Labelling | Understanding of fire extinguisher types and placement | IAC0404 |
| Short Written Response | Understanding evacuation plans, demarcation, reporting | IAC0402, IAC0405, IAC0406, IAC0411 |
| Scenario-Based Case Study | Application of machine safety and lock-out principles | IAC0407, IAC0408, IAC0409 |

✍️ Assessment Activities

Activity 1: Observation Checklist (15 marks)

Instructions: Observe the learner performing or simulating the following activities in a workshop environment.

| Observed Task | Yes/No | Marks |
| --- | --- | --- |
| 1. Carries out a safety routine before using a tool (IAC0401) |  | 3 |
| 2. Keeps workstation clean and materials stored safely (IAC0403) |  | 3 |
| 3. Wears appropriate PPE for the task (IAC0410) |  | 3 |
| 4. Identifies a visible hazard and explains how to report it (IAC0402) |  | 3 |
| 5. Demonstrates knowledge of correct stacking within demarcation lines (IAC0406) |  | 3 |

Activity 2: Diagram Labelling – Fire Extinguishers (10 marks)

Instructions: Label the fire extinguishers in the diagram below with the correct:

- Extinguisher type (Water, Foam, CO₂, Dry Chemical)

- Appropriate fire class (A, B, E)

- Colour band used in South Africa

(Provide diagram of four extinguisher silhouettes with space to label each)

Activity 3: Short Written Response (15 marks)

Instructions: Answer the following questions in writing:

- How do you report an equipment fault or hazard in your workshop? (IAC0402) (3)

- What should you do if you do not understand your workplace’s evacuation plan? (IAC0405) (3)

- Describe the purpose of demarcation lines and how they help reduce incidents. (IAC0406) (3)

- Explain how and why you would report an injury or near-miss to a supervisor. (IAC0411) (6)

Activity 4: Scenario-Based Case Study (20 marks)

Scenario:
You are working on the foam-cutting machine when you notice that the safety guard is loose, and there are offcuts blocking part of the blade. The emergency stop button appears worn, and a colleague says not to worry about it.

Questions:

- What should you do first before continuing with the task? (IAC0407) (3)

- What procedure should you follow to test the emergency button and why is this important? (IAC0408) (4)

- What is a lock-out device, and when must it be used? (IAC0409) (5)

- What risks are created by ignoring or bypassing safety devices and warning signs? (4)

- Who should be informed if the safety mechanism or isolator is faulty? (IAC0411) (4)

✅ Model Answers

Activity 1 – Observation Checklist

- Routine includes inspection, tool checks, PPE use – ✔

- Clean, orderly workstation – ✔

- Wears goggles, gloves, apron, earplugs – ✔

- Identifies a trailing cable as a hazard and knows to report it – ✔

- Recognises yellow lines and uses designated storage zone – ✔

Activity 2 – Diagram Labelling

- Water – Class A – Red

- Foam – Class A & B – Cream

- CO₂ – Class E – Black

- Dry Chemical – Class A, B, C – Red

Activity 3 – Short Written Response

- Inform supervisor verbally or in writing, enter in incident or fault register

- Ask supervisor or safety rep to explain plan; study signage or floorplans

- Demarcation lines guide safe movement, mark emergency areas, prevent obstruction

- Record injury in incident log, notify first aider and supervisor, follow workplace procedure

Activity 4 – Scenario-Based Case Study

- Stop work, isolate machine, report issue

- Press button while machine is running test mode; must stop instantly

- Device that prevents tool from being used; used during cleaning or repair

- Increased injury risk, legal liability, equipment damage

- Supervisor, health and safety rep, or workshop manager

Marking Memo

| Activity | Marks | Marking Guidance |
| --- | --- | --- |
| Activity 1: Observation | 15 | 3 marks per accurate demonstration |
| Activity 2: Diagram | 10 | 2.5 marks per correctly labelled extinguisher |
| Activity 3: Written | 15 | Logical, safety-aligned responses using correct terminology |
| Activity 4: Scenario | 20 | Full marks for hazard identification, safety actions, and protocol awareness |
| Total | 60 |  |

📊 Assessment Rubric

| Rating | Descriptor |
| --- | --- |
| 5 – Excellent | Demonstrates full understanding; applies safety principles confidently |
| 4 – Good | Mostly correct; minor errors; consistent awareness of workshop safety |
| 3 – Satisfactory | Basic understanding; may omit steps but shows general safety awareness |
| 2 – Basic | Partial understanding; unclear or incomplete responses |
| 1 – Poor | Unsafe practices; lacks understanding of workshop safety requirements |

### Facilitator Assessment Briefing

KM-04-KT04: Health and Safety in the Advanced Upholstery Department
Qualification: Furniture Upholsterer (SAQA ID: 103199)
NQF Level: 3
Weight: 15%

🎯 Purpose of the Assessment

This integrated assessment is designed to evaluate learner competence against Internal Assessment Criteria IAC0401 to IAC0411. The focus is on assessing the learner’s ability to:

- Apply safe work practices and routines in accordance with legal and workplace requirements

- Identify and report hazards, incidents, and equipment faults

- Demonstrate understanding of evacuation procedures, PPE, fire safety, and demarcation systems

- Handle machinery correctly, including the use of isolators and lock-out devices

The assessment includes theoretical and practical components using varied instruments to ensure comprehensive, valid, and authentic assessment.

Assessment Instruments Overview

The assessment includes:

- Observation Checklist

- Diagram Labelling of Fire Extinguishers

- Short Written Responses

- Scenario-Based Case Study

These tools allow the learner to demonstrate practical competence and theoretical understanding in a simulated or real workshop environment.

Facilitation Guidelines

Before the Assessment

- Ensure that KT0401 to KT0411 have been fully delivered

- Review all OHS principles, PPE usage, incident reporting, and fire safety

- Allow learners to practise evacuation procedures and workshop routines

During the Assessment

- Clearly explain all instructions for each activity

- Supervise the observation tasks in a safe, controlled environment

- Verify learner understanding through direct questioning and scenario-based tasks

After the Assessment

- Use the marking memo and rubric provided

- Ensure all evidence is complete and signed

- Moderate at least 25% of assessments for consistency and fairness

- Provide feedback to each learner and retain records for internal and external moderation

📚 Evidence Requirements

Each learner submission must include:

- Completed answer sheets for all assessment activities

- Completed and signed observation checklist

- Clear identification on all pages (name, date, assessor signature)

This evidence forms part of the internal summative assessment for KM-04.

Scoring Summary

| Activity | Marks |
| --- | --- |
| Observation Checklist | 15 |
| Diagram Labelling | 10 |
| Short Written Response | 15 |
| Scenario-Based Case Study | 20 |
| Total | 60 |

Scores must be proportionally converted to reflect the 15% weighting for this knowledge module.

🔍 Moderation Notes

- Review completeness and accuracy of learner submissions

- Verify scoring against the marking memo

- Document moderation findings in the moderation report

- Ensure alignment with the Internal Assessment Criteria IAC0401–IAC0411

## KM-04-KT05: Concepts and Principles of Measuring and Calculations Used in the Furniture Upholstery Processes (15%)

Purpose of the Knowledge Topic

This knowledge topic aims to equip learners with the foundational concepts and applied principles of measurement and calculation as they relate to furniture upholstery. Precision is critical in upholstery work, where even minor measurement errors can compromise quality, aesthetics, and function. Learners will explore the tools, systems, and techniques used to obtain, convert, and apply accurate measurements during upholstery processes.

The module prepares learners to use appropriate instruments, calculate material dimensions, determine allowances, and work with both straight and curved surfaces. Emphasis is placed on the importance of calibration, the selection of correct units, and the implications of measurement inaccuracies in an upholstery production environment.

Key Knowledge Areas

This topic covers the following elements:

- KT0501: Taking accurate measures

- KT0502: Units of measurement

- KT0503: Conversions

- KT0504: Formulae and calculations

- KT0505: Angles and curves

- KT0506: Standard sizes

- KT0507: Allowances and tolerances

- KT0508: Measuring instruments: Tape measure, Vernier, callipers, square

- KT0509: Calibrations

- KT0510: Accuracy and faults

Internal Assessment Criteria and Weight

Learners will be assessed on their ability to:

- IAC0501: Explain the application of various formulae in furniture upholstery

- IAC0502: Justify the importance of accuracy on the quality of the workpiece

- IAC0503: Explain the effect of inaccuracy and faults

- IAC0504: Identify and select measuring equipment for the job

- IAC0505: Use correct measuring units

- IAC0506: Use measuring equipment correctly to obtain accurate sizes

- IAC0507: Justify the importance of good-quality measuring tapes, Vernier callipers

Weight: 15%

Application in the Upholstery Workshop

In upholstery production, precision equals professionalism. Learners must be able to select the correct measuring tool for a task, apply formulas to determine fabric dimensions, angles, and curves, and ensure consistency in size and fit. Accurate measuring and calculation ensure waste reduction, quality assurance, and client satisfaction. This knowledge topic builds the learner’s ability to execute quality workmanship with confidence and mathematical competence, using both traditional and calibrated measuring devices.

### KT0501: Taking Accurate Measures

Theoretical Learning Content

Taking accurate measurements is a critical skill in upholstery, where every cut of fabric, foam, frame, or fitting depends on precision. Whether working on a custom-made sofa, restoring a dining chair, or assembling modular components, the upholsterer must be able to take reliable measurements using the correct tools and techniques. Inaccurate measuring leads to material waste, poor fit, safety risks, and client dissatisfaction.

Learners must develop a consistent measuring methodology, understand the importance of positioning and posture, and ensure that tools are correctly handled and maintained.

1. Principles of Accurate Measuring

To take accurate measurements:

- Use the right tool for the surface and material being measured

- Start from a fixed zero point on the tool (e.g. edge of a tape or calliper)

- Keep the tool straight and flat to prevent curvature or slack

- Measure multiple times for verification

- Note the reading clearly, using correct units

- Avoid assumptions—always measure both external and internal dimensions when required

2. Measuring Techniques

| Technique | Description |
| --- | --- |
| Direct measurement | Using a tool to measure length, width, or depth of a component |
| Comparison measurement | Matching two or more objects against a standard or previous measure |
| Inside and outside measuring | Measuring internal diameters or spaces vs external lengths or widths |
| Angle measurement | Using a square, bevel, or protractor to ensure consistent angles and joins |

3. Factors That Affect Accuracy

- Poor lighting

- Damaged or stretched measuring tapes

- Worn-out markings on tools

- User fatigue or distractions

- Measuring over uneven or soft surfaces (e.g. fabric, foam)

- Incorrect zero-point alignment

Examples

- Using a steel ruler or straight-edge to measure wood frames ensures rigidity and clear readings.

- When measuring fabric with a flexible tape, ensure it is flat against the surface and not twisted.

- For inside measurements, a Vernier calliper may be used to measure the depth or width of a groove or slot.

Case Study

Case Study: Ziyaad and Pieter’s Precision Payoff

At a collaborative upholstery workshop in Gqeberha, Ziyaad and Pieter were preparing materials for a curved-back occasional chair. Pieter measured the frame width and recorded 545 mm. Ziyaad, double-checking before cutting the foam, noted that the edge of the measuring tape was frayed and potentially unreliable. He replaced it with a calibrated tape measure and confirmed the measurement was actually 548 mm. The adjustment prevented a foam misfit and saved a valuable off-cut.

Discussion Points:

- What principle of measuring did Ziyaad demonstrate?

- How did teamwork help improve measurement accuracy in this case?

Critical Thinking Questions

- Why is re-measuring important, even if a previous reading seems accurate?

- What are the risks of relying on worn or uncalibrated measuring tools?

- How does workshop environment (e.g. temperature, lighting) influence measurement reliability?

- In what ways can apprentices be taught to build good measuring habits early on?

- How can measuring mistakes impact the cost and quality of upholstery products?

### KT0502: Units of Measurement

Theoretical Learning Content

Units of measurement provide a universal standard for quantifying length, width, volume, and mass in the upholstery workshop. Understanding and correctly applying these units is essential for ensuring consistency, accuracy, and communication—both within the team and when reading technical specifications, invoices, and material labels.

In upholstery, most measurements relate to length and area (e.g. fabric, foam, timber) but may also include volume (e.g. cushioning fill) and mass (e.g. adhesives, fabrics).

1. Systems of Measurement

| System | Common Units Used |
| --- | --- |
| Metric system | Millimetres (mm), centimetres (cm), metres (m), grams (g), kilograms (kg), litres (L) |
| Imperial system | Inches ("), feet (ft), yards (yd), pounds (lb) |

South Africa uses the metric system officially, but imperial units may appear in imported materials or legacy templates.

2. Standard Metric Units in Upholstery

| Measurement Type | Unit | Usage Example |
| --- | --- | --- |
| Length / Height / Width | mm / cm / m | Measuring fabric length or frame dimensions |
| Area | m² | Calculating fabric required for a sofa cover |
| Volume | L | Measuring foam volume or liquid adhesive |
| Weight / Mass | g / kg | Estimating transport load or fabric roll weight |

3. Common Conversions in the Workshop

| Conversion | Formula |
| --- | --- |
| cm to mm | Multiply by 10 |
| m to cm | Multiply by 100 |
| mm to m | Divide by 1 000 |
| inch to mm | Multiply by 25.4 |
| feet to metres | Multiply by 0.3048 |

Learners must be fluent in switching between units when working with international suppliers, reading machine specs, or comparing local and imported templates.

Examples

- A fabric roll labelled as 1.4 m wide and 25 m long has a total surface area of 35 m².

- A foam sheet measuring 500 mm × 600 mm must be converted to 0.5 m × 0.6 m when entered into a CAD system.

- A seat height of 450 mm is sometimes referred to as 17.7 inches in imported diagrams.

Case Study

Case Study: Priya’s Measurement Mix-Up

While preparing quotation sheets, Priya noticed that the imported catalogue listed fabric requirements in yards and inches. She mistakenly assumed 1 yard = 1 metre and over-ordered by 9%. After consulting with Monique, a more experienced upholsterer, she learnt to use a conversion chart and verified dimensions before purchasing. This improved both cost control and her confidence when working with international specifications.

Discussion Points:

- What was the root cause of the over-order?

- How did peer consultation lead to improved accuracy?

Critical Thinking Questions

- Why is it important to know both metric and imperial systems in an upholstery business?

- What steps can prevent unit confusion when working with imported equipment or materials?

- How can incorrect conversions impact client satisfaction and project budgets?

- What tools or resources can learners use to assist with accurate unit application?

- How can workshop teams standardise units to improve communication and reduce errors?

### KT0503: Conversions

Theoretical Learning Content

In the upholstery workshop, conversion of units is a vital skill that allows workers to adapt measurements between different systems and scales. This becomes especially important when interpreting imported specifications, supplier catalogues, technical drawings, and customer instructions that may use either the metric or imperial system.

Accurate conversions help maintain consistency, avoid material waste, and ensure that the final product meets design expectations.

1. Why Conversions Matter in Upholstery

- Upholstery projects often include components measured in different units (e.g. mm, cm, m, inches)

- Tools and equipment may be calibrated in imperial units, while material orders use metric

- Failing to convert units correctly can result in poor fit, mismatched parts, and cost overruns

2. Common Conversions and Formulas

| From | To | Formula / Factor |
| --- | --- | --- |
| Millimetres (mm) | Centimetres (cm) | ÷ 10 |
| Centimetres (cm) | Millimetres (mm) | × 10 |
| Metres (m) | Millimetres (mm) | × 1 000 |
| Inches (in) | Millimetres (mm) | × 25.4 |
| Feet (ft) | Metres (m) | × 0.3048 |
| Yards (yd) | Metres (m) | × 0.9144 |

Always round only at the final step of your calculation to avoid cumulative inaccuracies.

3. Practical Application in the Workshop

- Measuring foam height in millimetres but ordering fabric in metres

- Using supplier documentation that lists fastener spacing in inches

- Cutting frames to match an imported drawing printed in imperial units

- Calculating surface area of curved or custom shapes that require unit adjustments

Examples

- A client requests a cushion that is 18 inches wide. Converting this to metric:
18 × 25.4 = 457.2 mm

- A template indicates 1.5 yards of fabric. Converted to metres:
1.5 × 0.9144 = 1.372 m

- A frame drawing shows a leg height of 0.75 m. Converted to millimetres:
0.75 × 1 000 = 750 mm

Case Study

Case Study: Imraan Saves the Sofa

At a Cape Town upholstery shop, Imraan was asked to replicate a vintage English sofa. The original specifications were in feet and inches. Before beginning the build, he used a conversion worksheet to accurately convert all dimensions into millimetres. During assembly, a teammate suggested rounding early, but Imraan insisted on full accuracy until the end. The finished sofa matched perfectly, and the international client praised the precise dimensions.

Discussion Points:

- What was the risk of rounding too early in the process?

- How did Imraan’s approach uphold professional standards?

Critical Thinking Questions

- What could happen if unit conversions are done incorrectly at the cutting stage?

- How can workshops build systems that support accurate conversions during projects?

- Why is it important to know conversion formulas rather than relying solely on calculators?

- What tools or references should be readily available in a production environment for conversions?

- How can conversion errors impact costing, especially when dealing with imported materials?

### KT0504: Formulae and Calculations

Theoretical Learning Content

In upholstery, formulae and calculations are used to determine fabric requirements, foam volumes, costings, pattern repeats, allowances, and dimensions. Precision in calculations ensures materials are used efficiently, components fit correctly, and cost estimations are accurate. Learners must develop the ability to apply mathematical operations confidently and interpret formulas within the context of furniture design and production.

1. Common Formulae in Upholstery

| Purpose | Formula | Example |
| --- | --- | --- |
| Area of rectangle | Area = Length × Width | Fabric panel: 1.8 m × 0.6 m = 1.08 m² |
| Volume of foam cushion | Volume = Length × Width × Height | 0.5 m × 0.5 m × 0.1 m = 0.025 m³ |
| Circumference (round pouffe) | Circumference = π × Diameter | π × 0.6 m = 1.88 m |
| Allowance calculation | Final = Original Measurement + Seam/Tuck Allowance | 550 mm + 10 mm = 560 mm |
| Fabric cost | Cost = Area × Price per metre² | 1.08 m² × R120 = R129.60 |

π (pi) is approximately 3.1416 when calculating curves and circular items.

2. Mathematical Operations Used

- Addition and subtraction: for calculating allowances, deductions, and trims

- Multiplication and division: for scaling patterns, pricing, and volume

- Percentages: for calculating mark-ups, VAT, and waste

- Rounding rules: typically round up to ensure material sufficiency

- Unit consistency: ensure all measurements are in the same unit before applying a formula

Examples

- A chair seat measures 500 mm × 600 mm. With a 10 mm seam allowance on all sides, the final fabric size is:
(500 + 20) mm × (600 + 20) mm = 520 mm × 620 mm

- A 1.2 m wide roll of fabric is needed to cover a headboard measuring 2.4 m².
Required length = 2.4 ÷ 1.2 = 2 metres

- A curved ottoman with a 0.7 m diameter requires piping around the edge.
Circumference = π × 0.7 m = 2.2 metres of piping

Case Study

Case Study: Tamsyn’s Template Tally

Tamsyn was asked to prepare all foam pieces for ten identical ottomans. She calculated that each unit required 0.034 m³ of foam. Rather than estimating, she multiplied:
0.034 m³ × 10 = 0.34 m³ total foam required.
She added 10% to allow for cutting waste and offcuts:
0.34 m³ × 1.10 = 0.374 m³.
The team ordered exactly what was needed, avoiding excess and ensuring cost control.

Discussion Points:

- How did Tamsyn use formulas to improve material planning?

- What effect did including a waste margin have on budgeting?

Critical Thinking Questions

- Why is it essential to use correct formulas when calculating fabric or foam requirements?

- How does poor calculation affect both the budget and quality of an upholstery product?

- In what ways can technology (like spreadsheets or apps) support accuracy in workshop calculations?

- How should learners approach projects that involve multiple shapes and materials requiring different formulas?

- What is the importance of consistency in units when using mathematical formulas?

### KT0505: Angles and Curves

Theoretical Learning Content

In furniture upholstery, angles and curves play an essential role in both the design and construction of items such as armrests, headboards, backrests, and curved seating. Understanding how to measure and replicate these forms accurately is key to producing high-quality, ergonomic, and visually appealing products. Failure to account for angles and curves during measurement and layout can result in poorly fitting covers, discomfort, and excess material waste.

This topic teaches learners how to identify, measure, mark, and calculate curved and angled components in upholstery frames and fabric patterns.

1. Types of Angles and Where They Occur in Upholstery

| Type of Angle | Measurement | Examples in Upholstery |
| --- | --- | --- |
| Acute | Less than 90° | Inclined backrest, sloped armrest |
| Right | Exactly 90° | Square seat base, frame corners |
| Obtuse | More than 90° but less than 180° | Backrest-to-seat joint on lounge chairs |
| Reflex | More than 180° | Rare in upholstery, but possible in frame design |

2. Measuring Angles

- Carpenter’s square or try square: Confirms 90° angles in frame construction

- Adjustable bevel gauge: Used to replicate non-standard angles

- Protractor or angle finder: Measures or checks the angle directly

- Templates: Used for repeating curves and complex angled components

3. Curves in Upholstery Design

Curves require accurate measurement along both length and radius, especially when cutting:

- Foam for rounded cushions

- Fabric for arched headboards or ottomans

- Piping or edging materials that follow a curved line

Measuring a curve may involve:

- Using a flexible tape along the curve’s path

- Determining radius using radius templates

- Applying circumference formulas for circles or partial arcs (e.g. π × diameter)

4. Allowance for Bending and Stretching

- Material tension must be adjusted around curves to avoid puckering or folding

- Curves require additional seam allowances or easing techniques during upholstery

- Cutting foam for curved profiles may need compound angle techniques

Examples

- An arched backrest has a curve with a 700 mm diameter.
Circumference = π × 700 mm ≈ 2 199 mm, requiring at least 2.2 metres of fabric along the curve.

- A backrest inclined at 110° (obtuse angle) requires fabric panels joined with a complementary angle to prevent bunching.

- A rounded ottoman base with a 0.5 m radius needs a template for both top surface and piping.

Case Study

Case Study: Chadwin and Asha’s Corner Challenge

Chadwin and Asha worked together to upholster a custom corner couch. The seat cushion had a curved internal corner with a non-standard 135° angle. Chadwin created a bevel gauge template and transferred the angle to a plywood base. Asha used a flexible curve rule to measure the fabric path and added 10 mm allowance for ease. Their collaboration ensured a snug, wrinkle-free finish that matched the designer’s expectations.

Discussion Points:

- How did their use of tools assist in the accuracy of the finished piece?

- Why was understanding the curve’s path important for the fabric cut?

Critical Thinking Questions

- Why is it important to distinguish between straight and curved measurements in upholstery?

- How does accurate angle measurement impact both comfort and appearance in furniture?

- What could happen if fabric is cut for a curve using a straight measurement?

- How can apprentices be supported in developing confidence with curved surfaces and angled joints?

- What are some ways to reproduce complex angles consistently when making multiple units?

### KT0506: Standard Sizes

Theoretical Learning Content

In the upholstery industry, standard sizes refer to commonly accepted dimensions for furniture components and finished products. These standards help upholsterers to design, cut, and assemble pieces with consistency, efficiency, and compatibility across manufacturing processes. Standard sizes are particularly important when producing modular furniture, working with ready-made frames, or sourcing pre-cut foam and fabric.

Understanding and applying standard measurements reduces rework, improves fit, and ensures that upholstered items meet ergonomic and aesthetic expectations.

1. Common Upholstery Standard Sizes

| Component | Typical Measurement Range | Unit |
| --- | --- | --- |
| Dining chair seat height | 450 – 480 mm | mm |
| Lounge chair seat depth | 500 – 600 mm | mm |
| Ottoman height | 400 – 450 mm | mm |
| Headboard height | 1 000 – 1 400 mm above mattress | mm |
| Armrest height | 600 – 700 mm | mm |
| Cushion thickness | 100 – 150 mm (standard foam) | mm |
| Fabric roll width | 1.4 m – 1.5 m (standard upholstery fabric) | metres |

These measurements serve as a guideline and may be adapted for custom work or based on user specifications.

2. Applications of Standard Sizes

- Fabric planning: Based on expected seat and back sizes

- Foam cutting: Using thickness standards to pre-order foam sheets

- Ergonomics: Ensuring comfortable seating and arm support

- Frame design: Using industry-standard frame templates or CNC patterns

- Component matching: Sourcing standard-sized legs, zips, cushion inserts, and linings

3. Importance of Following Standard Sizes

- Improves workshop productivity and repeatability

- Simplifies procurement and stock control

- Reduces the risk of ergonomic failure or discomfort

- Ensures compatibility with covers, linings, and fixings

- Helps when scaling up production or outsourcing components

Examples

- A 1.4 m wide roll of fabric will typically cover two standard dining chair seats placed side by side.

- A seat cushion designed to be 500 mm × 500 mm × 100 mm allows for use of off-the-shelf foam blocks without cutting waste.

- A sofa base frame built to a height of 450 mm matches standard lounge foam height for easy assembly and consistent comfort.

Case Study

Case Study: Monique Meets the Mark

Monique was preparing components for a hospitality client who required 30 identical lounge chairs. By referring to SABS-aligned standards, she selected foam, fabric, and frames sized to match the industry norms. As a result, her cuts were efficient, all pieces fitted perfectly, and minimal material was wasted. Her team completed the order ahead of schedule and the client praised the professional consistency across the entire batch.

Discussion Points:

- How did Monique’s knowledge of standard sizes contribute to workshop efficiency?

- What would have been the risks of not consulting standard dimensions?

Critical Thinking Questions

- Why is it beneficial to use standard sizes in upholstery even for custom-designed furniture?

- What challenges can arise if measurements are taken without considering fabric and foam standard sizes?

- How can understanding standard sizes improve costing accuracy?

- How does knowledge of standard sizes assist when collaborating with frame builders or foam suppliers?

- In what ways might standard sizes need to be adapted for users with special physical requirements?

### KT0507: Allowances and Tolerances

Theoretical Learning Content

In upholstery, allowances and tolerances are crucial elements of accurate measurement and fitting. Allowances refer to additional space intentionally added to a measurement to account for seams, folds, movement, or material behaviour. Tolerances refer to the acceptable range of variation in a finished measurement that does not compromise the function or quality of the product.

Understanding when and how to apply allowances and tolerances ensures that upholstered components fit correctly, look professional, and function as intended—even when minor variations occur in materials or cutting.

1. What Is an Allowance?

An allowance is added during cutting or design to make room for:

- Seam allowances (typically 10–20 mm depending on the seam type)

- Foam compression

- Tuck-in and stapling margins

- Shrinkage in fabric (e.g. velvet or linen)

- Ease or fit for covers and cushions

Allowances vary depending on the material type and the component being upholstered.

2. What Is a Tolerance?

A tolerance is the range within which a measurement may deviate and still be acceptable. It allows for:

- Slight inaccuracies in cutting or measuring

- Tool limitations (e.g. blade width)

- Material expansion or compression (e.g. foam, batting)

- Human error within acceptable limits

| Example | Tolerance Range |
| --- | --- |
| Foam block cut to 500 mm | Acceptable tolerance: ± 2 mm |
| Wooden panel height of 700 mm | Acceptable tolerance: ± 1 mm |
| Fabric cover for a seat base | Allowance added: + 15 mm |

3. Key Guidelines

- Always confirm client expectations and fit type (tight, eased, loose) before applying allowances

- Use consistent methods for applying tolerances in cutting and assembly

- Communicate tolerance limits to fabricators and team members

- Record allowances and tolerances clearly in production documentation

Examples

- A chair seat cover includes a 10 mm seam allowance around all four sides, resulting in a cut size 20 mm wider and longer than the seat base.

- A foam insert for a back cushion is specified at 600 mm × 400 mm ± 5 mm, allowing for minor cutting variation that does not affect fit.

- When upholstering a curved ottoman, an upholsterer adds an extra 30 mm to accommodate tucking the fabric underneath without tension.

Case Study

Case Study: Refilwe’s Realisation

Refilwe was working on a set of scatter cushions. She cut the fabric panels to the exact foam size of 450 mm × 450 mm without any seam allowance. After stitching and inserting the foam, the covers were too small, resulting in overstretching and distorted corners. Her supervisor, Willem, explained how a 15 mm seam allowance would have prevented the issue. Refilwe revised her cutting guide and improved her work accuracy in future batches.

Discussion Points:

- How did omission of allowance affect Refilwe’s final product?

- Why is learning from errors valuable in developing craftsmanship?

Critical Thinking Questions

- What are the consequences of failing to add proper allowances when cutting fabric for upholstery?

- Why are tolerances necessary even when using calibrated cutting tools or templates?

- How can you determine the appropriate allowance for a specific material or project?

- In what ways can allowances and tolerances influence costing and material usage?

- How should a team communicate and document agreed tolerances in a shared production environment?

### KT0508: Measuring Instruments – Tape Measure, Vernier, Callipers, Square

Theoretical Learning Content

Upholstery processes demand precise and reliable measurement at all stages—from frame cutting to foam shaping and fabric fitting. The ability to identify, select, and use the correct measuring instrument is a core competency. Each tool is suited to specific measurement types, materials, and levels of precision.

This topic familiarises learners with the main measuring instruments used in upholstery: their functions, correct handling, and suitability for different tasks.

1. Overview of Common Measuring Instruments

| Instrument | Primary Use |
| --- | --- |
| Tape measure | Measuring long lengths of fabric, foam, or frames |
| Steel rule / ruler | Measuring short lengths, marking straight cuts |
| Vernier callipers | Measuring internal, external, and depth dimensions precisely |
| Callipers (manual) | Comparing diameters or widths of cylindrical or curved parts |
| Carpenter’s square | Checking right angles and ensuring squareness of corners |
| Set square / try square | Measuring and confirming fixed angles in assembly work |

2. Characteristics of Each Tool

| Tool | Precision | Flexibility | Common Material | Use Case |
| --- | --- | --- | --- | --- |
| Tape measure | Medium | Flexible | Steel / fibre | Measuring fabric, foam rolls, seat dimensions |
| Vernier callipers | High | Rigid | Stainless steel | Checking joint gaps, foam insert fit |
| Manual callipers | Moderate | Adjustable | Aluminium / steel | Repeating or transferring measurements |
| Square | High | Fixed | Steel or composite | Ensuring frame joints are perfectly square |

All measuring tools should be calibrated, clean, and undamaged to ensure accurate readings.

3. Correct Use and Handling

- Start from the zero mark, not the edge, especially for tape measures

- Read measurements at eye level to avoid parallax error

- Ensure surfaces are flat and stable when taking readings

- Avoid bending or forcing rigid tools (e.g. Vernier) into tight spaces

- Store tools in protective cases or toolboards to avoid knocks or moisture damage

Examples

- A tape measure is used to determine the fabric length for a sofa armrest: 1 800 mm.

- A Vernier calliper measures the inner diameter of a pre-drilled hole in a frame: 10.00 mm ± 0.01 mm accuracy.

- A square confirms that the corners of a cushion base frame are at exactly 90°, preventing warping during assembly.

Case Study

Case Study: Pieter’s Precision Upgrade

During a frame-fitting task, Pieter noticed inconsistencies between two supposedly identical components. Using a Vernier calliper instead of a ruler, he discovered that one slot was 1.2 mm narrower than the other. After adjusting the part, the frame fit perfectly. His colleague, Asha, commented on how this tiny difference saved the team hours of rework. Pieter began encouraging apprentices to use precision tools for all structural checks.

Discussion Points:

- How did the choice of measuring tool affect the outcome?

- What role did precision play in improving efficiency?

Critical Thinking Questions

- How do you decide which measuring tool to use for a specific upholstery task?

- What are the risks of using flexible tools like a tape measure when precision is required?

- Why is it important to store measuring tools properly in a workshop?

- How does calibration of instruments like Vernier callipers ensure accuracy?

- How can learners be trained to handle and maintain measurement tools confidently?

### KT0509: Calibrations

Theoretical Learning Content

Calibration is the process of checking and adjusting the accuracy of a measuring instrument by comparing it to a known standard. In the upholstery workshop, accurate measurements are essential for ensuring high-quality outcomes, particularly when using tools such as Vernier callipers, measuring tapes, and squares. Regular calibration helps to detect wear, environmental distortion, and handling errors, ensuring consistency and compliance with quality standards.

1. What Is Calibration?

Calibration refers to:

- Comparing the accuracy of a measuring instrument with a certified standard

- Making necessary adjustments to ensure the tool reflects true, precise values

- Recording the calibration status for traceability and verification

In some cases, if a tool cannot be adjusted, it must be replaced or recalibrated professionally.

2. Why Calibration Matters in Upholstery

| Aspect | Impact of Poor Calibration |
| --- | --- |
| Frame alignment | Misaligned joints, gaps, or uneven furniture structure |
| Foam sizing | Too-tight or loose fit into frames or covers |
| Fabric cutting | Pattern mismatch, wasted material |
| Quality control | Failure to meet client or specification requirements |

3. Tools That Require Calibration

| Tool | Calibration Need |
| --- | --- |
| Vernier callipers | Must measure to ±0.01 mm; check zero alignment regularly |
| Tape measure | Check for stretching, wear at the hook end, and print fading |
| Carpenter’s square | Must maintain a perfect 90° angle; use a reference edge |
| Laser or digital rulers | Verify against physical measurement standards |

4. Calibration Procedure (General Steps)

- Clean the tool to remove debris or dust

- Check the zero point (e.g. when jaws are closed, Vernier reads 0.00 mm)

- Compare to a known reference (e.g. a certified gauge block or steel rule)

- Adjust if possible or flag the tool as out of calibration

- Record the calibration in a tool register or logbook

- Tag or mark the tool with a calibration date and expiry (if applicable)

Examples

- A Vernier calliper checked against a 20 mm certified block reads 20.02 mm. The 0.02 mm difference may be within acceptable tolerance, but must be documented.

- A tape measure used regularly on foam cutting shows fraying at the tip and a 3 mm discrepancy—requiring replacement or tagging out of use.

Case Study

Case Study: Elmarie’s Inspection Day

Elmarie, the workshop manager at a mid-sized upholstery firm in Bloemfontein, instituted monthly tool calibration checks. She found that two of the measuring tapes used on the fabric table had stretched by 5 mm due to wear and repeated pulling. After replacing them and training staff on calibration awareness, fabric wastage decreased by 8% and customer rework dropped significantly. A simple system had a big impact.

Discussion Points:

- What caused the original calibration error?

- How did a preventative routine improve both quality and cost outcomes?

Critical Thinking Questions

- What risks are associated with using tools that have not been calibrated?

- How can workshops manage calibration when they have multiple sets of tools?

- What role should learners and junior staff play in identifying calibration issues?

- Why is it important to keep records of calibration activities?

- How does calibration support long-term product consistency and client satisfaction?

### KT0510: Accuracy and Faults

Theoretical Learning Content

In upholstery, accuracy is central to producing high-quality workpieces, maintaining workshop efficiency, and delivering client satisfaction. Small deviations in measurement, cutting, or fitting can result in visible faults, compromised comfort, and costly rework. This topic reinforces the value of accuracy at every stage—from measuring and marking to assembling and upholstering—while helping learners recognise and address the types of faults caused by inaccuracy.

1. What Is Accuracy in Upholstery?

Accuracy refers to how closely a measurement or task matches the intended specification. It involves:

- Taking correct dimensions

- Using consistent methods

- Following specified tolerances

- Applying proper alignment and fit techniques

- Verifying measurements before cutting or stitching

2. Common Types of Faults Caused by Inaccuracy

| Area of Inaccuracy | Resulting Fault |
| --- | --- |
| Measuring too short | Covers that are too tight; exposed foam or frame |
| Measuring too long | Baggy upholstery; excessive folding or tucking |
| Incorrect angle measurement | Frame misalignment; gaps at joints |
| Fabric miscutting | Pattern mismatch; visible distortions |
| Foam mis-sizing | Uneven surface, discomfort, or structural instability |

3. Consequences of Inaccuracy

- Material waste and increased cost

- Rework or loss of production time

- Poor product appearance and durability

- Client dissatisfaction or returns

- Safety risks in unstable frames or protruding fasteners

Accuracy is not only a technical skill, but a work ethic that supports quality and professionalism.

4. Promoting Accuracy in the Workshop

- Double-check all measurements before cutting

- Use the correct tools for each task

- Maintain a clean and organised workspace

- Record specifications clearly and check against design requirements

- Encourage peer-review or buddy checks for large projects

- Train apprentices to understand the why behind each measurement

Examples

- A 500 mm foam insert cut 5 mm too large must be trimmed or it will buckle the cover.

- A square frame assembled with angles slightly off from 90° appears skewed and affects stability.

- A fabric cut with no seam allowance leads to tension, seam rupture, or visible flaws.

Case Study

Case Study: Jody’s Redemption Through Precision

Jody, an apprentice upholsterer in Durban, produced a series of scatter cushions that looked distorted. Upon inspection, her supervisor, Naveen, discovered inconsistencies in her fabric cuts—none were within the 5 mm tolerance allowed. Jody was retrained on accurate tape measure usage, seam allowances, and quality checks. Her next batch was flawless, and her confidence grew with each accurate task completed.

Discussion Points:

- How did small measurement errors affect the product quality?

- What role did supervision and mentorship play in resolving the issue?

Critical Thinking Questions

- What systems can be implemented in a workshop to catch faults before they affect final assembly?

- How does attention to detail support both safety and aesthetics in furniture design?

- Why is it important for apprentices to experience and correct their own inaccuracies?

- What processes could be introduced to encourage accountability for accuracy in a team setting?

- In what ways can a culture of precision enhance a company’s reputation and client base?

## Integrated Assessment – KM-04-KT05

Knowledge Topic: Concepts and Principles of Measuring and Calculations Used in the Furniture Upholstery Processes
Qualification: Furniture Upholsterer (SAQA ID 103199)
NQF Level: 3
Internal Assessment Criteria: IAC0501–IAC0507
Assessment Weight: 15%
Total Marks: 60

🧪 Assessment Instruments Overview

| Instrument Type | Purpose | IACs Targeted |
| --- | --- | --- |
| Short Written Questions | Assess conceptual knowledge of formulae and accuracy | IAC0501, IAC0502, IAC0503 |
| Matching Activity | Identify and describe appropriate measuring tools | IAC0504, IAC0507 |
| Practical Task with Observation | Measure and record dimensions using correct tools | IAC0505, IAC0506 |
| Scenario-Based Application | Interpret and respond to real-world measuring challenge | All IACs |

✍️ Assessment Activities

Activity 1: Short Written Questions (15 marks)

Instructions: Answer all questions in your own words.

- Explain how formulae are used to calculate fabric area and foam volume in upholstery. (IAC0501) (5)

- Why is accuracy important when measuring foam, fabric, or frame components? (IAC0502) (5)

- What are two possible effects of measuring incorrectly in an upholstery project? (IAC0503) (5)

Activity 2: Matching – Measuring Tools (10 marks)

Instructions: Match each tool (Column A) to its description or application (Column B).

| Column A |  | Column B |
| --- | --- | --- |
| 1. Vernier calliper | A | Used to check 90° angles in wooden frames |
| 2. Tape measure | B | Measures external and internal dimensions with high accuracy |
| 3. Carpenter’s square | C | Ideal for long flexible measurements on fabric or foam |
| 4. Set square | D | Must be calibrated for reliable, repeated use |
| 5. Calibrated steel ruler | E | Flat rigid tool for precise short distances |

Activity 3: Practical Observation Task (15 marks)

Instructions: Learner must complete the following in a simulated or actual workshop environment.

| Task Performed | Yes/No | Marks |
| --- | --- | --- |
| 1. Selects appropriate measuring tools for a given object (IAC0504) |  | 3 |
| 2. Uses metric units correctly (mm/cm/m) (IAC0505) |  | 3 |
| 3. Measures an object to ±1 mm accuracy using correct tool (IAC0506) |  | 3 |
| 4. Explains why a worn measuring tape should not be used (IAC0507) |  | 3 |
| 5. Records measurement clearly and consistently (IAC0506) |  | 3 |

Activity 4: Scenario-Based Application (20 marks)

Scenario:
You are preparing fabric panels for ten dining chair seats. The standard seat base is 480 mm × 420 mm, and you are instructed to add a 20 mm seam allowance all around. Your colleague suggests using an old tape that is slightly frayed but “still works fine.”

Questions:

- Calculate the final cut size for one panel. (IAC0501) (4)

- What issues could arise from using a worn-out measuring tape? (IAC0503, IAC0507) (4)

- How would you justify the need to replace poor-quality measuring tools to your supervisor? (IAC0507) (4)

- What units should you use when marking the fabric, and why? (IAC0505) (4)

- Which tool would you choose for confirming that the panel corners are square? (IAC0504) (4)

✅ Model Answers

Activity 1 Answers:

- Formulae help calculate area (length × width) for fabric and volume (length × width × height) for foam, ensuring correct quantities are cut.

- Accuracy prevents waste, ensures fit, and upholds product quality.

- Inaccuracy may lead to oversized covers or misaligned frame joints.

Activity 2 Answers:
1 – B, 2 – C, 3 – A, 4 – D, 5 – E

Activity 3 Sample Observation:

- Tool: Vernier for slot width, tape for fabric

- Units: mm throughout

- Accuracy: Within 1 mm

- Justification: Frayed tape causes starting point errors

- Recording: Clearly written dimensions on worksheet

Activity 4 Answers:

- 480 mm + (20 mm × 2) = 520 mm; 420 mm + (20 mm × 2) = 460 mm

- Tape may stretch or misalign markings, causing cutting errors

- Poor-quality tools reduce efficiency and lead to client complaints; better to replace

- Metric (mm) to align with standards and reduce confusion

- Carpenter’s square for confirming 90° angles

Marking Memo

| Activity | Max Marks | Marking Guidance |
| --- | --- | --- |
| Activity 1 | 15 | 5 per answer; full marks for clear, accurate explanations |
| Activity 2 | 10 | 2 marks per correct match |
| Activity 3 | 15 | 3 marks per correctly completed task with accuracy and safety |
| Activity 4 | 20 | 4 marks per well-reasoned, technically sound response |
| Total | 60 |  |

📊 Assessment Rubric

| Rating | Descriptor |
| --- | --- |
| 5 – Excellent | Precise measurements, tool knowledge, and strong justification skills |
| 4 – Good | Minor errors, solid understanding, needs reinforcement of accuracy importance |
| 3 – Satisfactory | Meets minimum requirements; some gaps in justification or technical language |
| 2 – Basic | Limited awareness of measurement impact; relies on incorrect or inconsistent tool use |
| 1 – Poor | Unsafe or careless practice; little understanding of measurement principles |

### Facilitator Assessment Briefing

KM-04-KT05: Concepts and Principles of Measuring and Calculations Used in the Furniture Upholstery Processes
Qualification: Furniture Upholsterer (SAQA ID: 103199)
NQF Level: 3
Weight: 15%

🎯 Purpose of the Assessment

This integrated assessment evaluates learner competence against Internal Assessment Criteria IAC0501 to IAC0507. It focuses on the learner’s ability to:

- Apply mathematical formulae relevant to upholstery

- Justify the importance of measurement accuracy and the effects of inaccuracy

- Identify, select, and correctly use appropriate measuring equipment

- Demonstrate understanding of measuring units and equipment quality

The assessment uses both theoretical and practical instruments to ensure valid and holistic evaluation.

🧪 Assessment Instruments Overview

The integrated assessment includes:

- Short Written Questions

- Matching Activity

- Practical Observation Task

- Scenario-Based Application

These assessment instruments ensure coverage of knowledge, comprehension, application, and reasoning aligned to the IACs.

🧭 Facilitation Guidelines

Before the Assessment

- Ensure all topic content KT0501 to KT0510 has been covered

- Provide revision on measuring tools, units, formulae, and recording methods

- Allow learners time to practise calculations and tool handling

During the Assessment

- Read each question and task aloud for clarity

- Observe learners carefully during practical tasks

- Use the checklist provided in the observation task

- Encourage clarity in calculations and accurate recording

After the Assessment

- Use the model answers and memo to guide marking

- Moderate a minimum of 25% of assessments

- Provide individual feedback to each learner

- Record results and retain for internal and external verification

📚 Evidence Requirements

Each learner’s file must contain:

- Completed written and scenario-based responses

- Signed and dated observation checklist

- Clear tool identification and justification answers

- Assessor’s comments, score, and rubric rating

All evidence must be clearly labelled with the learner’s name and date of assessment.

🧮 Scoring Summary

| Activity | Marks |
| --- | --- |
| Short Written Questions | 15 |
| Matching Activity | 10 |
| Practical Observation Task | 15 |
| Scenario-Based Application | 20 |
| Total | 60 |

Scores must be scaled proportionally to reflect the 15% weighting of this knowledge topic.

🔍 Moderation Notes

- Confirm alignment with IAC0501 to IAC0507

- Ensure fairness, consistency, and clarity in marking

- Retain a copy of the moderation report with the assessment file

- Confirm learner competence based on observed and written evidence

​​

## KM-04-KT06: Engineering Drawings (15%)

Purpose of the Knowledge Topic

This knowledge topic develops the learner’s ability to read, interpret, and apply engineering drawings in the context of furniture upholstery. Engineering drawings are a universal language of design and production, and the ability to understand and act on this information ensures that upholstered components are accurately produced, assembled, and finished to professional standards.

Through this topic, learners will build competence in recognising drawing layouts, projections, symbols, labels, and scales, and in interpreting complex information such as hidden details, chamfers, and cutting lists. The topic also covers the ability to distinguish between drawing types and to relate graphic specifications to physical production tasks.

Key Knowledge Areas

This topic covers the following elements:

- KT0601: Read and interpret furniture specifications

- KT0602: Sketches and engineering drawings

- KT0603: Layout of drawings

- KT0604: Legends and symbols

- KT0605: Dimensions and labelling

- KT0606: Isometric views

- KT0607: Line types

- KT0608: Hidden details

- KT0609: Solid lines

- KT0610: Projections

- KT0611: To scale drawing

- KT0612: Cutting lists

Internal Assessment Criteria and Weight

Learners will be assessed on their ability to:

- IAC0601: Correctly interpret engineering drawings, taking into account line structures and dimensions

- IAC0602: Observe all drawing projections

- IAC0603: Confirm the scale of the drawing according to specification

- IAC0604: Understand and apply hidden detail information to the workpieces

- IAC0605: Interpret drawing labels and ensure these match the specification, including chamfers, diameters, radiuses, and depths

Assessment Weight: 15%

Application in the Upholstery Workshop

A skilled upholsterer must not only work by feel and technique but also understand formal visual instructions. Engineering drawings guide the construction of frames, the fitting of foam and coverings, and the integration of mechanisms or embellishments. By mastering this topic, learners gain the ability to participate in professional upholstery environments that involve collaboration with designers, frame-makers, CNC operators, and quality controllers, ensuring precision from design to delivery.

### KT0601: Read and Interpret Furniture Specifications

Theoretical Learning Content

Furniture specifications are detailed documents or diagrams that provide information about a furniture product’s design, construction, dimensions, materials, finish, and functionality. In upholstery production, reading and interpreting these specifications accurately is essential for ensuring the correct materials are used, the correct sizes are cut, and the finished piece matches the intended design.

Upholsterers must understand not only what is shown in the drawing but also the accompanying written specifications. These include tolerances, assembly sequences, surface finishes, fixing methods, and special instructions.

1. Components of a Furniture Specification

| Element | Purpose |
| --- | --- |
| Drawing title | Identifies the product and version |
| Dimensions | Specifies all lengths, widths, depths, and angles |
| Materials list | Describes timber, foam, fabric, fasteners, and finishes |
| Tolerance values | States acceptable variation in measurements |
| Assembly notes | Step-by-step fitting or sequencing guidelines |
| Symbols and legends | Explain marks for surface finish, direction of grain, etc. |
| Reference numbers | Link to cutting lists, part IDs, or supplier codes |

2. Why Specifications Matter in Upholstery

- Prevent errors during frame preparation and fabric cutting

- Ensure consistency in replication of products

- Allow collaboration with frame builders, designers, and quality controllers

- Provide a traceable link between design intent and workshop execution

- Guide sourcing of correct materials and quantities

3. Reading Tips for Upholsterers

- Read the entire specification before beginning any work

- Use a highlighter to identify material types, allowances, and special finishes

- Cross-reference dimensions on drawings with physical measurements

- Identify if the drawing is not to scale, and calculate accordingly

- Always clarify unfamiliar symbols or terms before proceeding

Examples

- A specification for a lounge chair includes:

  - Frame height: 850 mm

  - Seat depth: 600 mm

  - Fabric: 1.4 m wide, 3.2 m required

  - Foam: 100 mm medium density

  - Finish: Walnut stain, matte varnish

  - Notes: Chamfer all edges to 5 mm

- The upholsterer checks the drawing and identifies the grain direction, required foam thickness, and tucking method for the seat base before beginning.

Case Study

Case Study: Naledi Reads Ahead

Naledi, a junior upholsterer in a Johannesburg workshop, was tasked with preparing foam for a series of recliner backrests. The specification noted a 5 mm chamfer on all front edges and a 15 mm stitch allowance on the side flaps. Naledi carefully measured each section, cross-referenced the fabric grain, and clarified a vague label with her supervisor. Her precision led to the fastest and most accurate completion rate among the team—and no rework was needed.

Discussion Points:

- How did Naledi’s approach prevent errors?

- Why is clarification a sign of professionalism rather than inexperience?

Critical Thinking Questions

- Why is it important to read the full furniture specification before starting an upholstery task?

- How can misunderstanding a single line in a specification affect the entire project?

- In what ways do specifications support quality control?

- How can workshops ensure that all staff understand specification documents consistently?

- What challenges might arise when interpreting specifications created by external designers?

### KT0602: Sketches and Engineering Drawings

Theoretical Learning Content

In upholstery production, both sketches and engineering drawings are used to communicate design intent, construction details, and material specifications. While sketches are often freehand and conceptual, engineering drawings are formal, technical, and standardised, with accurate dimensions, projections, and scales. Learners must be able to distinguish between the two and understand how to interpret and apply information from each in practical upholstery tasks.

1. Difference Between Sketches and Engineering Drawings

| Feature | Sketch | Engineering Drawing |
| --- | --- | --- |
| Purpose | Quick visualisation of ideas | Precise manufacturing or assembly instructions |
| Accuracy | Approximate | Exact, measured, and to scale |
| Tools used | Pencil, paper, ruler (optional) | CAD software, drawing boards, compasses, templates |
| Standardisation | Informal, often unique | Follows technical drawing standards (e.g. ISO, SANS) |
| Level of detail | Low to medium | High – includes dimensions, symbols, views, tolerances |
| Application in workshop | Used in early stages of planning or small fixes | Used for production, quality control, and material preparation |

2. Key Functions in Upholstery

- Sketches may be used by upholsterers to visualise a detail such as a button layout, tufting pattern, or stitching method

- Engineering drawings are used to construct frames, cut components, and confirm proportions during manufacturing

- Both can be used together—a sketch helps communicate client changes, while drawings formalise those changes for production

3. Benefits of Understanding Both

- Improves communication with designers, supervisors, and clients

- Helps bridge creative and technical understanding

- Enables workshop staff to take initiative when resolving on-site challenges

- Supports innovation and precision in upholstery problem-solving

Examples

- An upholsterer sketches a modified curve for a headboard at a client’s request and submits it to the design office, which returns a dimensioned CAD drawing for approval and cutting.

- A side elevation sketch of a frame includes basic seat height, back angle, and leg position. The formal drawing includes these plus exact joinery, radius arcs, material codes, and hardware callouts.

Case Study

Case Study: Siraaj and the Sketch-to-Spec Upgrade

Siraaj, a mid-level upholsterer in Durban North, created a hand-drawn sketch of a modification requested by a client who wanted curved sides on an ottoman. He handed the sketch to the CAD technician, who produced a detailed engineering drawing with scale, projections, and cutting instructions. The final ottoman matched the revised design precisely. The client praised the team’s responsiveness and the professionalism of their workflow.

Discussion Points:

- How did Siraaj’s sketch contribute to the final product?

- Why was it important that the sketch be formalised as an engineering drawing?

Critical Thinking Questions

- Why should upholsterers be confident working with both sketches and formal drawings?

- What are the risks of using a sketch in production without converting it into a technical drawing?

- How does each format serve a different stage of the furniture-making process?

- What role does technology play in translating sketches into accurate engineering plans?

- In what ways can sketching still be valuable even in a digital, CAD-based environment?

### KT0603: Layout of Drawings

Theoretical Learning Content

The layout of engineering drawings refers to how information is structured, positioned, and presented on a technical drawing sheet. A well-organised layout ensures that critical information—such as views, dimensions, labels, and title blocks—is easily understood and accurately followed by the upholsterer or workshop team.

Understanding layout conventions enables learners to quickly interpret technical details, locate specific information, and identify how different components relate to each other.

1. Standard Layout Components of an Engineering Drawing

| Element | Purpose |
| --- | --- |
| Title block | Contains drawing number, scale, date, drafter’s name, and revision status |
| Border | Frames the drawing sheet and separates it from annotations or additions |
| View placement | Positions top, front, and side views in their standard projection order |
| Dimensions and notes | Indicate measurements, tolerances, and special instructions |
| Parts list / BOM | Lists all components shown, often with reference numbers |
| Revision block | Tracks changes to the drawing over time |
| Cutting list reference | Provides cross-reference to material preparation documentation |

2. Layout Standards and Conventions

- Drawings are usually laid out according to orthographic projection standards

- The front view is placed centrally with the top view above and side view to the right

- Isometric or 3D views may be included in the top corner or on a separate sheet

- Title blocks are typically placed at the bottom right-hand corner

- Information should flow logically and be readable from the bottom or right-hand side

3. Benefits of a Clear Layout

- Reduces the chance of errors or misinterpretation

- Speeds up understanding and execution by different team members

- Improves communication between design, production, and quality control teams

- Ensures compliance with industry standards for record-keeping and verification

Examples

- A furniture frame drawing places the front view in the centre, the side view on the right, and the top view above, with dimensions and a detailed cutting list shown to the left.

- The title block includes the client name, scale (1:5), drawing number (DF102-RevB), and the drafter’s initials with a completion date.

Case Study

Case Study: Teboho’s Quick Catch

Teboho was reviewing an armchair base drawing when he noticed that the dimensions for the leg height were missing from the front view. Because he understood how drawings are typically laid out, he quickly located the height value in the side view, where it had been included instead. His awareness prevented a production delay and ensured the team moved forward without confusion.

Discussion Points:

- What layout knowledge helped Teboho locate the missing information?

- How might a less experienced team member have responded differently?

Critical Thinking Questions

- Why is the layout of engineering drawings standardised across furniture manufacturing?

- How can a poorly organised drawing layout lead to errors in upholstery or frame construction?

- What advantages do title blocks provide in multi-phase or team-based projects?

- How does understanding layout assist when using printed or digital drawings in the workshop?

- What should an upholsterer do if they cannot find key information in the expected layout area?

### KT0604: Legends and Symbols

Theoretical Learning Content

Legends and symbols are used in engineering drawings to represent physical characteristics, materials, surfaces, and processes without overcrowding the drawing with words. They allow for a universal visual language that helps the upholsterer understand key details quickly and consistently. Correct interpretation of symbols is vital to avoid errors in construction, material selection, and finishing.

Legends are usually presented in a dedicated box or key, often placed in a consistent corner of the drawing.

1. Purpose of Legends and Symbols in Upholstery Drawings

- To simplify technical communication

- To show repeated features such as screws, foam, or grain direction

- To indicate surface treatments (e.g. sanding, varnishing, staining)

- To save space while maintaining clarity

- To ensure that drawings can be read internationally or across multilingual teams

2. Common Upholstery-Related Symbols and Their Meaning

| Symbol | Meaning |
| --- | --- |
| ⊥ or □ | Square section or 90° angle |
| Ø (e.g. Ø10) | Diameter of a hole or round section (e.g. 10 mm) |
| R (e.g. R25) | Radius of a curved edge (e.g. 25 mm) |
| // | Parallel surfaces |
| ≠ | Not equal / Tolerance required |
| Grain arrows | Direction of wood or fabric grain |
| Hatch patterns | Indicate material types (e.g. foam, wood, steel) |
| Triangles or dashed borders | Hidden details or internal features not visible externally |
| Surface finish lines | Sanding or polishing areas (represented by waves or dots) |

Upholstery-specific drawings may also include icons for tacking strips, piping, tufting points, and foam density.

3. Interpreting a Drawing Legend

- Always read the legend or key before reviewing the drawing itself

- Match each symbol to its meaning, especially for radius, diameter, and grain

- Where a custom symbol is used, refer to the explanatory note or callout

- In some digital drawings, clickable symbols may lead to expanded descriptions

Examples

- A drawing shows ⌀10 next to a hole in a frame – this tells the upholsterer that a 10 mm diameter dowel will fit in that location.

- A triangle with a dashed outline near a foam section indicates a hidden recess for a cushion insert.

- A line with a wavy symbol along one panel edge indicates sanding required before staining.

Case Study

Case Study: Andile's Discovery in the Details

Andile, a learner at a Port Elizabeth furniture cooperative, was reviewing a backrest drawing. He noticed an unfamiliar hatch pattern on the frame supports. Referring to the legend, he saw it indicated medium-density fibreboard (MDF), not solid pine. This small observation led the team to adjust the fasteners used and prevented structural failure.

Discussion Points:

- How did Andile use the legend to avoid a major fault?

- What would the consequences have been if the wrong material was assumed?

Critical Thinking Questions

- Why is it important for every upholsterer to become fluent in interpreting drawing symbols?

- How does the use of legends improve the speed and accuracy of work in the upholstery workshop?

- What risks arise when a symbol is misinterpreted or skipped?

- How can learners familiarise themselves with new or uncommon drawing symbols?

- Why is it valuable to refer to the legend even on drawings you have used before?

Bottom of Form

### KT0605: Dimensions and Labelling

Theoretical Learning Content

Dimensions and labelling are critical features of engineering drawings. They provide the exact measurements, positions, and characteristics of furniture components. Labelling ensures that each part is identified clearly, allowing the upholsterer to produce or assemble items precisely to the design specification. A drawing without correct dimensions and labels is incomplete and may result in costly errors.

1. Purpose of Dimensions in Drawings

- To communicate length, width, height, depth, and angles

- To specify internal and external measurements

- To indicate hole sizes, positions, radii, diameters, and chamfers

- To guide material preparation and cutting

- To ensure accurate scale and proportionality of all parts

2. Purpose of Labelling in Drawings

- To identify part names, reference numbers, and materials

- To show tolerances, surface finish, and machining instructions

- To communicate assembly order, direction, and orientation

- To indicate details like grain direction, foam density, or stitch points

3. Best Practice in Reading Dimensions and Labels

| Tip | Explanation |
| --- | --- |
| Always check the unit of measurement | Confirm whether mm, cm, or inches are used |
| Follow extension and leader lines | These show exactly what the dimension refers to |
| Interpret label abbreviations | E.g. “R” for radius, “CH” for chamfer, “THK” for thickness |
| Identify datum lines | Reference points from which dimensions are measured |
| Check scale and adjust physical measurements accordingly | Especially if drawing is not 1:1 |

4. Examples of Common Labelling

| Label | Meaning |
| --- | --- |
| Ø12 | Diameter of 12 mm |
| R25 | Radius of 25 mm (curve or arc) |
| CH 5 × 45° | Chamfer 5 mm wide at 45 degrees |
| 50 mm THK | 50 mm thickness |
| Part A – Beech | Indicates name and material of the component |

Examples

- A chair arm drawing labelled “Part A – Pine – 25 mm THK – CH 5 mm” tells the upholsterer to cut the piece from 25 mm thick pine and chamfer the edges 5 mm.

- Dimensions show the overall length (650 mm), width (80 mm), and location of dowel holes (150 mm from one end, Ø10).

Case Study

Case Study: Gail Gets It Right

Gail, an upholsterer in Cape Town, was assembling a set of ottoman legs. The drawing showed two components: “Part A – Ø40, CH 3 × 45°” and “Part B – Ø40, no chamfer.” Because she paid close attention to both the dimensions and the labels, she correctly identified which parts were for front-facing and rear support. Her careful reading prevented a mismatch and preserved the intended aesthetic of the client’s design.

Discussion Points:

- What difference did the label “CH 3 × 45°” make to Gail’s interpretation?

- How would the outcome have changed if she ignored the label?

Critical Thinking Questions

- Why is it necessary to read both the dimensions and the labels before beginning an upholstery task?

- How does poor interpretation of dimensions affect frame fitting or fabric cutting?

- What should a learner do if they do not understand a label or abbreviation in a drawing?

- Why is the scale of a drawing important when applying dimensions?

- How can consistent labelling improve teamwork and workshop efficiency?

### KT0606: Isometric Views

Theoretical Learning Content

An isometric view is a three-dimensional drawing representation used to visualise an object in a way that shows height, width, and depth on a single 2D drawing. In upholstery and furniture production, isometric views are particularly useful for helping the upholsterer understand the overall shape, structure, and assembly of a component or finished item.

Isometric drawings do not use perspective but instead display the object from an angle (typically 30° from the horizontal axis), allowing all three dimensions to be represented at equal scale.

1. Purpose of Isometric Views in Upholstery Drawings

- To give a complete, realistic picture of the furniture piece

- To help visualise how components fit together

- To guide construction or upholstery steps based on shape and profile

- To assist in explaining concepts to clients or apprentices

- To complement orthographic (2D) views by offering a spatial understanding

2. Characteristics of Isometric Views

| Feature | Description |
| --- | --- |
| 30° Angles | Sides recede at 30° to the horizontal axis |
| Equal Scale | All three axes (height, width, depth) are drawn to same scale |
| No Perspective Distortion | Objects do not get smaller in the distance |
| Corner View | Typically shows a front-right or front-left corner |
| Simplified Visual Detail | Hidden lines are often omitted for clarity |

3. Applications in Upholstery

- Showing how foam layers are stacked or shaped

- Indicating fabric wrapping around frames or curves

- Displaying the interaction between wooden structure and upholstery form

- Illustrating the full shape of items like round ottomans, L-shaped sofas, or sculpted headboards

Examples

- A fully upholstered couch may include an isometric view that shows the backrest angle, the seat width, and the armrest profile in one drawing.

- An isometric sketch of a corner chair might reveal how the legs, stretchers, and frame are joined—even if not detailed in the orthographic views.

Case Study

Case Study: Sihle Uses the Third Angle

Sihle, an apprentice in an East London upholstery workshop, was tasked with cutting foam for a custom sectional sofa. The orthographic drawings confused her as the pieces were all rectangular. However, when her supervisor showed her the isometric view, the overall L-shape made sense. She immediately understood how each foam piece fit, preventing unnecessary rework. She later requested isometric drawings for other projects to aid her understanding.

Discussion Points:

- How did the isometric view improve Sihle’s comprehension of the final form?

- Why might a new learner struggle with orthographic projections alone?

Critical Thinking Questions

- Why are isometric views helpful in understanding complex furniture structures?

- How can isometric drawings improve communication between the upholsterer and the client?

- What are the limitations of isometric views when it comes to dimensions and accuracy?

- Why is it important to study both isometric and orthographic views of the same object?

- How might using isometric sketches assist in explaining techniques to new team members?

### KT0607: Line Types

Theoretical Learning Content

In engineering and upholstery drawings, different line types are used to convey specific meanings. These lines distinguish between visible edges, hidden features, centre lines, cutting planes, and outlines of movement. Recognising and correctly interpreting these line types ensures that the upholsterer or production team understands what should be cut, assembled, or left concealed.

Each line has a defined appearance, such as thickness, pattern (solid, dashed, chain), and purpose. Failing to interpret line types correctly can result in mis-assembly, incorrect cutting, or damage to internal components.

1. Common Line Types and Their Meanings

| Line Type | Appearance | Purpose |
| --- | --- | --- |
| Continuous thick line | Solid, thick | Visible edges and outlines of an object |
| Continuous thin line | Solid, thin | Dimension lines, extension lines, leader lines |
| Dashed line | Short dashes | Hidden details not visible from current view |
| Chain thin line | Alternating long and short dashes | Centre lines of symmetrical objects |
| Phantom line / movement line | Dashed with dot segments | Indicates alternate positions or movement range |
| Cutting plane line | Thick dashed line with arrows | Shows where a section cut is taken |

2. Importance of Line Interpretation in Upholstery

- Cutting precision: Avoids damaging hidden components (e.g. spring units or foam cavities)

- Assembly accuracy: Ensures joinery and frame fitting match the internal layout

- Measurement application: Only visible lines are dimensioned; hidden lines are inferred

- Safety and efficiency: Minimises guesswork and reduces production delays

3. Standard Practices

- Lines are drawn in accordance with ISO/SANS conventions

- Only essential hidden features are shown to avoid clutter

- In multi-view drawings, line types guide the reader from one view to another (e.g. from top to section view)

- Always read the legend or title block if uncertain about line meanings

Examples

- A dashed line inside a cushion base drawing indicates the position of an internal support board.

- A chain line across a round ottoman base shows the centreline for symmetrical measurement.

- A phantom line in a recliner drawing shows the extended position of a leg rest when open.

Case Study

Case Study: Lindiwe Learns the Lines

During training at a workshop in Polokwane, Lindiwe misinterpreted a hidden dashed line as a visible cut line and mistakenly removed a central dowel section from a frame. Her supervisor, Mr Dlamini, reviewed the line types with her and helped her distinguish between continuous and dashed line meanings. Lindiwe began keeping a line type reference chart at her station and quickly became known for her improved interpretation skills.

Discussion Points:

- What caused the error in Lindiwe’s reading of the drawing?

- How did a simple visual tool (reference chart) support her learning?

Critical Thinking Questions

- Why do engineering drawings use different line types rather than written labels alone?

- What could happen if a hidden detail line is mistakenly interpreted as a visible edge?

- How can upholstery learners build fluency in recognising line types?

- What practical tools or visual aids can be used to support learning line type conventions?

- In what way does understanding line types improve collaboration with design and frame teams?

### KT0608: Hidden Details

Theoretical Learning Content

Hidden details in engineering drawings refer to components or features that are not visible from a particular view but are still essential to the design and function of the furniture item. These are typically represented by dashed lines and help the upholsterer understand the internal structure or concealed features such as dowels, slots, recesses, or internal foam layers.

Understanding hidden details is essential in upholstery, where frames, padding, fasteners, and substructures often influence how covers are applied and how the final product performs.

1. Common Hidden Details in Upholstery Drawings

| Hidden Feature | Example |
| --- | --- |
| Dowel joints | Hidden wooden pegs that connect two frame parts |
| Recesses or slots | For inserting panels, foam, or fittings |
| Internal bracing | Hidden support blocks inside armrests or backrests |
| Foam layer profiles | Non-visible shaping below the cover material |
| Tufting anchor points | Embedded hardware under the surface for buttons |
| Staple backing strips | Fixed inside the frame, not visible externally |

2. Representation of Hidden Details

- Shown using dashed lines in orthographic views

- May be referenced with callouts or detailed in section views

- Often accompanied by notes specifying location, size, or material

- In isometric or perspective drawings, may not be visible at all—requiring the reader to refer to sections or elevations

3. Importance of Interpreting Hidden Details

- Ensures correct fabric preparation, stapling, and tucking

- Prevents accidental damage to concealed joinery or supports

- Assists in planning upholstery sequences and tool access

- Supports quality control, especially where internal features affect comfort or strength

Examples

- A seat base drawing with dashed lines indicates the presence of a recessed area for a foam pad, which must be cut and fitted precisely.

- A chair leg section reveals internal slots for hidden cross-supports not shown in the main view.

- A backrest layout includes anchor points for tufted buttons, visible only as small dashed circles on the inner panel.

Case Study

Case Study: Dean and the Dowels

Dean, an upholsterer from Gqeberha, was fitting foam to a lounge chair frame. He overlooked a set of dashed lines on the base drawing and mistakenly stapled into a hidden dowel joint. The result was a loose leg and a returned product. After this, Dean reviewed hidden detail conventions and made it a habit to highlight all dashed lines before starting work. His quality improved, and he later helped train apprentices in drawing reading.

Discussion Points:

- What misunderstanding led to the structural failure in Dean’s product?

- How did highlighting hidden details change his approach?

Critical Thinking Questions

- Why is it important to identify hidden details before starting upholstery tasks?

- What kinds of damage or faults might result from ignoring hidden features?

- How can drawings be used in combination with physical inspection to verify hidden structures?

- What role do hidden details play in ensuring furniture durability and safety?

- How should teams communicate hidden details to ensure they are not missed during production?

### KT0609: Solid Lines

Theoretical Learning Content

In engineering and upholstery drawings, solid lines are the most common line type used to represent the visible edges, outlines, and surfaces of a furniture component. Also referred to as continuous thick lines, these lines define the shape and size of each part as it appears from a specific view. They are critical for understanding the true form of an object and guiding tasks such as cutting, shaping, assembling, or upholstering.

Understanding solid lines is foundational to reading technical drawings accurately.

1. Characteristics of Solid Lines

| Feature | Description |
| --- | --- |
| Continuous and unbroken | No dashes or spaces; drawn firmly and clearly |
| Thicker than dimension lines | To visually separate structure from annotations |
| Define visible edges | Represent actual physical outlines from the selected view |
| Used in orthographic and isometric views | Across all projection types |

2. Application in Upholstery Drawings

- Frame outlines for cutting templates

- Foam block profiles and visible cushion shapes

- Fabric panel sizes and edge stitching zones

- Placement of hardware like legs, hinges, and studs

- Outer dimensions used for material estimation

Solid lines are the first lines to look at when identifying what will be physically seen and touched in the final product.

3. Interpreting Solid Lines with Other Drawing Features

- Solid lines are often accompanied by:

  - Dashed lines for hidden features

  - Dimension lines to indicate sizes

  - Leader lines pointing to notes or specifications

- In section views, solid lines define the cut profile and exposed surfaces

Examples

- A front view drawing of a headboard uses solid lines to show the height, width, and arched profile that will be upholstered.

- A side view of a seat frame includes solid lines outlining the leg, backrest, and front rail — all of which will be visible after covering.

Case Study

Case Study: Marietjie Measures Twice

Marietjie, a senior upholsterer at a workshop in Bloemfontein, was preparing frame covers for a batch of custom tub chairs. She used the solid lines in the engineering drawing to cut fabric panels to size. By carefully tracing these outlines onto pattern paper, she ensured the fabric fit perfectly around all visible edges. The result was a clean, professional look with no overstretching or waste.

Discussion Points:

- What role did solid lines play in Marietjie’s fabric planning?

- How did the clarity of visible edges reduce errors in her work?

Critical Thinking Questions

- Why are solid lines the most important starting point when reading a drawing?

- How do solid lines assist upholsterers in preparing patterns or templates?

- What could go wrong if dashed or other line types are confused with solid lines?

- How can solid lines support accurate material calculations?

- In what ways do solid lines improve communication between design and production teams?

### KT0610: Projections

Theoretical Learning Content

In engineering drawing, projections are methods used to represent three-dimensional objects in two dimensions. These are essential for showing the true size, shape, and relationship of each component part from different angles. In upholstery and furniture production, projections allow upholsterers to understand the layout of frames, cushions, supports, and fasteners from multiple views.

By mastering projection systems, learners can interpret drawings more effectively, especially when working on complex or multi-component furniture.

1. Types of Projections Used in Upholstery Drawings

| Projection Type | Description |
| --- | --- |
| Orthographic Projection | Displays views from front, top, and side—each at 90° angles |
| Isometric Projection | Three-dimensional view with equal scaling on all axes (see KT0606) |
| Sectional Projection | Shows an internal view by cutting through a part to reveal hidden features |
| Auxiliary Projection | Used to represent inclined surfaces not visible in standard orthographic views |

2. Orthographic Projection Views

| View Name | What It Shows |
| --- | --- |
| Front view | Main outline and height of the object |
| Top view | Width and depth when looking from above |
| Side view | Height and depth from either left or right |

These views are placed according to projection standards and must be read together to get a complete understanding of the object.

3. Importance of Projections in Upholstery

- Guides the upholsterer on how the piece appears from all sides

- Helps locate cut-outs, tapers, angles, and concealed features

- Provides accurate measurement references from multiple angles

- Supports the preparation of foam, frames, and fabric panels

Examples

- A sofa armrest drawing includes front, top, and side views. The top view reveals the width tapering inward; the side view shows the curve of the armrest profile.

- A sectional projection reveals the internal foam layering of a backrest, helping the upholsterer understand where to sculpt or reinforce the shape.

Case Study

Case Study: Naveen’s View from Every Side

Naveen, a workshop lead at a manufacturer in Pietermaritzburg, was working on a custom upholstered bench with a hidden storage compartment. By studying the projections in the engineering drawing, he noted that the top view showed the lid profile, the front view showed leg placement, and the sectional view revealed the hinge recess inside the frame. His team avoided errors by following each projection methodically.

Discussion Points:

- How did Naveen’s understanding of projections improve the team's execution?

- Why are sectional projections useful for furniture with concealed parts?

Critical Thinking Questions

- Why is it necessary to review all projections in a drawing before beginning upholstery work?

- How do orthographic projections help in identifying measurement points not visible in one view?

- What problems might occur if an upholsterer relies only on a single view of a component?

- Why are sectional views especially valuable in upholstered furniture design?

- How can the correct reading of projections improve communication between design and production teams?

### KT0611: To Scale Drawing

Theoretical Learning Content

A to scale drawing is a technical drawing where all dimensions are represented in proportion to the actual size of the object. In upholstery production, working with scaled drawings is essential for understanding how components will fit together, how materials will be cut, and how the finished piece will relate to the design plan.

Scaling enables large furniture items to be represented on standard-size paper or screens without losing accuracy or detail. Interpreting scale correctly is vital for ensuring correct measurements, part matching, and material calculations.

1. What Is a Scale Drawing?

A scale drawing maintains the proportions of the original object, but the size is reduced or enlarged by a fixed ratio, such as:

| Scale | Meaning |
| --- | --- |
| 1:1 | Full size (actual size) |
| 1:2 | Half size – every 1 unit on drawing = 2 units in real life |
| 1:5 | 1 unit on drawing = 5 units in reality |
| 2:1 | Double size – used for small objects |

The scale is always indicated in the title block of the drawing.

2. Importance of Scale in Upholstery

- Ensures correct interpretation of dimensions

- Assists in accurate material estimation

- Helps visualise how components fit together in space-constrained settings

- Guides CNC cutting or manual template tracing

- Enables proportional analysis during client presentations or prototype development

3. Best Practices for Reading Scale Drawings

- Always check the scale value before taking any measurements

- Use a scale ruler to convert drawing dimensions to real-world sizes

- If a drawing is not to scale, it must be clearly labelled as such

- Be aware of drawings where different views may be scaled differently (e.g. detail views)

Examples

- A chair back panel drawn at 1:2 scale with a 300 mm height on paper means the actual height is 600 mm.

- A small component like a decorative tack plate may be drawn at 2:1 scale to highlight drilling positions for clarity.

Case Study

Case Study: Lerato’s Scaling Success

Lerato, a learner at a workshop in Vereeniging, was reviewing a 1:5 scale drawing of a deep-buttoned headboard. She needed to mark the fabric cutting panel accurately, but mistakenly assumed the drawing was 1:1. After a quick check of the title block, she corrected her calculations, converting all dimensions accordingly using a scale ruler. Her panels were cut precisely, and no fabric was wasted. Her facilitator later used her experience as a teaching example.

Discussion Points:

- What could have happened if Lerato had continued using the wrong scale?

- Why was the title block so important in this case?

Critical Thinking Questions

- Why must the scale be checked before using measurements from a drawing?

- What tools or methods can upholsterers use to convert scaled dimensions accurately?

- What are the risks of working from a drawing that is not labelled with its scale?

- How does the ability to read scale drawings enhance material planning and budgeting?

- Why might a designer choose to show a small component at a larger scale?

### KT0612: Cutting Lists

Theoretical Learning Content

A cutting list is a structured document that details the individual parts required to construct a piece of furniture. It includes information such as quantities, sizes, materials, and identifiers for each part. Cutting lists are derived from engineering drawings and form an essential bridge between design and production.

In upholstery, cutting lists help with the accurate preparation of frames, foam, fabric, and finishing materials, ensuring consistency, minimising waste, and improving workflow efficiency.

1. What Is Included in a Cutting List?

| Field | Description |
| --- | --- |
| Part number / ID | Unique identifier matching drawing or BOM |
| Description | Name of the part (e.g. Back rail, Seat foam, Leg) |
| Quantity | Number of identical pieces required |
| Length / Width / Thickness | Measurements of each part |
| Material | Type of material (e.g. pine, high-density foam, calico) |
| Special notes | Chamfers, grain direction, or allowances |

2. Purpose of a Cutting List in Upholstery

- Helps workshop staff prepare correct parts efficiently

- Ensures accurate ordering of materials and stock control

- Minimises off-cuts, waste, and misalignment during assembly

- Links design documents to physical production tasks

- Supports batching and bulk production consistency

3. Interpreting and Using a Cutting List

- Match the cutting list entries to the labelled drawings

- Cross-check dimensions and materials with the supplier or in-house inventory

- Check for allowances (e.g. foam compression, seam margins)

- Organise parts during pre-assembly for easy access and identification

- Maintain a log of completed cuts to avoid duplication or oversight

Examples

- A chair cutting list entry:

  - Part ID: FR002

  - Description: Seat base front rail

  - Quantity: 2

  - Size: 500 mm × 70 mm × 25 mm

  - Material: Meranti

  - Notes: Chamfer front edge 3 mm

- A foam cutting list may also specify density, contouring notes, and attachment zones.

Case Study

Case Study: Yaseen’s List Saves Time

Yaseen, an upholstery workshop assistant in Pretoria, was assigned to prepare all components for a custom ottoman order. Using the cutting list provided, he pre-cut all frame parts, foam inserts, and fabric panels. When assembly began, the supervisor was impressed by how smoothly the build progressed. Yaseen’s adherence to the list reduced delays and eliminated cutting errors, earning him recognition and a lead role in the next project.

Discussion Points:

- How did the cutting list improve efficiency in Yaseen’s case?

- Why is pre-planning with a cutting list helpful for teamwork?

Critical Thinking Questions

- How does a cutting list reduce material waste in an upholstery workshop?

- What might happen if measurements are taken from the drawing instead of the list?

- Why is it important for the cutting list to include material types as well as dimensions?

- In what ways can cutting lists support production planning and delivery scheduling?

- How should changes to a cutting list be handled and documented during production?

## Integrated Assessment – KM-04-KT06

Knowledge Topic: Engineering Drawings
Qualification: Furniture Upholsterer (SAQA ID: 103199)
NQF Level: 3
Internal Assessment Criteria:

- IAC0601: Engineering drawings correctly interpreted, taking into account line structures and dimensions

- IAC0602: All drawing projections are observed

- IAC0603: Scale of drawing is according to specification

- IAC0604: Hidden details are understood and information is applied to the work pieces

- IAC0605: Drawing labels are understood and according to specification, showing all details, chamfers, diameters, radiuses and depths
Assessment Weight: 15%
Total Marks: 60

🧪 Assessment Instruments Overview

| Instrument Type | Purpose | Targeted IACs |
| --- | --- | --- |
| Drawing Interpretation Task | Tests understanding of line types, views, and projections | IAC0601, IAC0602 |
| Scale Calculation Exercise | Evaluates ability to calculate real-world dimensions from scaled views | IAC0603 |
| Hidden Detail Application | Scenario-based question requiring identification of internal features | IAC0604 |
| Label Matching Task | Identifies labels and terminology in a technical drawing | IAC0605 |

✍️ Assessment Activities

Activity 1: Drawing Interpretation Task (15 marks)

Instructions: Study the engineering drawing provided. Then answer the questions.

- Identify three visible outlines shown using solid lines. (3)

- Name one line type that represents hidden internal features. (2)

- Describe what a centre line is used for. (2)

- From the top, side, and front views, summarise what information each projection provides. (8)

Activity 2: Scale Calculation Exercise (15 marks)

Instructions: The following drawing is shown at a scale of 1:5. Calculate the actual size for each of the following features.

- Seat width on drawing: 120 mm → Actual seat width = ? (5)

- Leg thickness on drawing: 10 mm → Actual leg thickness = ? (5)

- Backrest height on drawing: 150 mm → Actual height = ? (5)

Activity 3: Hidden Detail Application (15 marks)

Scenario: The drawing shows dashed lines inside the seat base. These indicate dowels connecting the side rails to the front and back rails. There is also a hidden recess on the inside of the armrest.

Questions:

- What does the dashed line signify in the seat base view? (3)

- Why is it important not to staple or drill into the recess area? (4)

- What action should be taken when a hidden feature is not immediately understood? (4)

- Provide one example of another hidden detail you may encounter in an upholstered product. (4)

Activity 4: Label Matching Task (15 marks)

Instructions: Match each label from the drawing to its correct meaning.

| Label | Meaning |
| --- | --- |
| Ø16 | A. Chamfer 5 mm at 45° |
| R25 | B. Radius of 25 mm |
| CH 5 × 45° | C. Diameter 16 mm hole or round feature |
| THK 30 | D. Part has a thickness of 30 mm |
| 60 mm depth | E. Indicates depth of a feature (e.g. mortise, recess) |

### Facilitator Assessment Briefing

KM-04-KT06: Engineering Drawings
Qualification: Furniture Upholsterer (SAQA ID: 103199)
NQF Level: 3
Assessment Weight: 15%

🎯 Purpose of the Assessment

This integrated assessment evaluates learner competence against Internal Assessment Criteria IAC0601 to IAC0605. It focuses on the learner’s ability to:

- Interpret engineering drawings with reference to line types, dimensions, and layout

- Understand and observe orthographic and sectional projections

- Apply correct interpretation of scale to determine actual dimensions

- Identify and apply hidden details within the context of workpiece preparation

- Read and apply drawing labels for chamfers, diameters, radii, and depths

🧪 Assessment Instruments Overview

The assessment includes:

- Drawing Interpretation Task

- Scale Calculation Exercise

- Hidden Detail Application (Scenario-Based)

- Label Matching Task

Each activity has been selected to align with one or more of the IACs, ensuring a comprehensive and valid assessment of both theoretical understanding and practical application.

🧭 Facilitation Guidelines

Before the Assessment

- Confirm that all content under KT0601 to KT0612 has been covered

- Review different line types, projection methods, and scaling conventions

- Provide visual examples and conduct small group reading practice using real or sample drawings

During the Assessment

- Supervise carefully and provide clarification on instructions as needed

- Allow the use of scale rulers or calculators during scale-related activities

- Observe how learners interpret the layout and drawing elements independently

After the Assessment

- Mark using the model answers and rubric provided

- Provide constructive feedback with examples if answers are incorrect

- Record scores and retain a copy of the assessment for internal moderation

- Ensure that a moderation sample is selected and verified (at least 25%)

📚 Evidence Requirements

Each learner’s assessment evidence should include:

- Completed worksheets or answer sheets for all four activities

- Any supporting rough calculations or sketches (especially for scaling)

- The completed assessment checklist and assessor commentary

- Mark breakdown and final rating from the rubric

All documents must be dated, signed, and clearly labelled with the learner’s name and assessment reference.

🧮 Scoring Summary

| Activity | Marks |
| --- | --- |
| Drawing Interpretation Task | 15 |
| Scale Calculation Exercise | 15 |
| Hidden Detail Application | 15 |
| Label Matching Task | 15 |
| Total | 60 |

Scores must be scaled proportionally to reflect the 15% weighting of this knowledge topic.

🔍 Moderation Notes

- Confirm that learner responses demonstrate a clear understanding of projections, line types, and dimensions

- Verify scale calculation accuracy using the 1:5 ratio as applied

- Ensure consistent marking across all learners using the shared memo and rubric

- Maintain moderation evidence and update internal tracking and learner reports

✅ Model Answers

Activity 1

- Chair seat outline, backrest outline, front leg profile

- Dashed line

- Used to show symmetry or centres of circles/holes

- Top = width and depth; Front = height and width; Side = height and depth

Activity 2

- 120 mm × 5 = 600 mm

- 10 mm × 5 = 50 mm

- 150 mm × 5 = 750 mm

Activity 3

- Internal dowel placement

- May damage structure or result in weak joinery

- Ask supervisor or refer to sectional view

- Staple backing strips, button anchors, frame reinforcements

Activity 4
1–C, 2–B, 3–A, 4–D, 5–E

🧾 Marking Memo

| Activity | Marks | Guidance |
| --- | --- | --- |
| Activity 1 | 15 | Correct line types and projection explanations |
| Activity 2 | 15 | Accurate calculations using 1:5 ratio |
| Activity 3 | 15 | Demonstrates understanding of hidden detail use |
| Activity 4 | 15 | 3 marks per correct match |
| Total | 60 |  |

📊 Assessment Rubric

| Rating | Descriptor |
| --- | --- |
| 5 – Excellent | Accurate interpretation and calculation; clearly links drawing elements to application |
| 4 – Good | Mostly correct; one or two minor misinterpretations |
| 3 – Satisfactory | Reasonable understanding; some concepts unclear |
| 2 – Basic | Limited knowledge; key elements missed or confused |
| 1 – Poor | Drawing not understood; unable to apply or explain basic elements |

## KM-04-KT07: Samples, Prototypes and Customer Specifications (5%)

Purpose of the Knowledge Topic

This knowledge topic introduces the learner to the essential concepts of product specifications, samples, prototypes, and customer requirements within the upholstery production environment. These elements form the foundation for ensuring that furniture products are manufactured in line with client expectations, functional requirements, and design intent.

By understanding and using specification documents and reference models, upholsterers ensure that production is accurate, consistent, and aligned with quality standards. This topic also prepares learners to interpret and apply technical information throughout the production process, from planning to quality control.

Key Knowledge Areas

This topic covers the following components:

- KT0701: Definition of product specifications, samples, prototypes, and customer specifications

- KT0702: Content of specification sheets

- KT0703: Interpretation of product specifications, sample or prototype

- KT0704: Uses of product specifications, sample or prototype

- KT0705: Types of product specifications, sample or prototype

Internal Assessment Criteria and Weight

Learners will be assessed on their ability to:

- IAC0701: Understand the product specifications, sample or prototype, lay plan, standard operating procedures, and production programme

- IAC0702: Describe the concept of product specifications, sample or prototype

- IAC0703: Explain the purpose of product specifications, sample or prototype

- IAC0704: Analyse the importance of product specifications, sample or prototype in the production process

- IAC0705: Identify and define the various aspects of product specifications, sample or prototype (e.g. description, measurements, materials, technical drawings)

- IAC0706: Reason the methods of interpretation of product specifications, sample or prototype

- IAC0707: List the types of product specifications, sample or prototype

Assessment Weight: 5%

Application in the Upholstery Workshop

In upholstery practice, samples and prototypes allow for client approval, testing, and refinement of a design before mass production. A well-documented specification sheet ensures that every team member—from frame maker to finisher—follows the same standards. By mastering this topic, learners develop the ability to interpret customer requirements, collaborate with design and production teams, and contribute to consistent, high-quality outputs across projects.

### KT0701: Definition of Product Specifications, Samples, Prototypes and Customer Specifications

Theoretical Learning Content

In upholstery production, clear and standardised communication tools are essential for ensuring that a product meets design requirements, client expectations, and functional standards. Among the most important tools are product specifications, samples, prototypes, and customer specifications. Each plays a distinct role in the development, approval, and execution of a furniture piece.

Understanding these terms provides learners with the vocabulary and framework needed to interpret design briefs, engage with quality control processes, and participate in product development.

1. Product Specifications

Definition: A product specification is a structured document or sheet that outlines the key technical and aesthetic characteristics of a furniture product.

It typically includes:

- Dimensions

- Materials to be used (timber, foam, fabric)

- Colours and finishes

- Assembly requirements

- Tolerances

- Quality standards

- Compliance instructions

A product specification serves as a central reference point for the entire production team.

2. Samples

Definition: A sample refers to a small or full-scale example of a particular component or material used to evaluate appearance, quality, or suitability.

Examples in upholstery:

- Fabric swatches showing texture and colour

- Foam samples for density comparison

- Stitching or buttoning samples

Samples assist in supplier selection, internal approvals, and customer demonstrations.

3. Prototypes

Definition: A prototype is a first full version of a complete product created before full production to test the design, structure, comfort, and visual appeal.

In upholstery, a prototype might include:

- A completed chair showing design lines, material use, and comfort level

- A single unit for client presentation and feedback before batch manufacturing

Prototypes allow for design refinement and identification of faults.

4. Customer Specifications

Definition: Customer specifications are the specific preferences, requirements, or constraints provided by the client regarding a furniture item.

This may include:

- Size limitations

- Preferred finishes or colours

- Branding or logos

- Functional requests (e.g. storage inside an ottoman)

- Price and timeline constraints

Customer specifications guide customisation and must be integrated into the product specification and production process.

Examples

- A fabric sample is approved by the client before being applied to the full prototype.

- A product specification sheet outlines all measurements, foam density, and fabric type for a custom sofa.

- A customer specification document requests that the chair height be reduced to accommodate a specific office desk height.

- A prototype lounge suite is manufactured to check scale and cushion comfort before full-scale production begins.

Case Study

Case Study: Sipho's Specification Success

Sipho, a junior upholsterer in a Port Elizabeth workshop, was involved in a hospitality project for 20 guestroom armchairs. He was given a prototype and a product specification sheet with all dimensions, foam types, and upholstery details. After thoroughly studying both, he noticed that the approved customer specification required a softer back cushion. By comparing the foam sample to the one used in the prototype, Sipho raised a query before full production, saving time and avoiding rework.

Discussion Points:

- How did understanding the difference between the prototype and the specification sheet help Sipho?

- Why is it important to cross-check all specification types before beginning work?

Critical Thinking Questions

- What is the difference between a sample and a prototype?

- Why must upholsterers understand customer specifications even if they are not in direct contact with the client?

- In what ways does a product specification reduce production errors?

- How can samples assist in sourcing materials that match the client’s expectations?

- Why is a prototype necessary even when all specifications are clearly written?

### KT0702: Content of Specification Sheets

Theoretical Learning Content

A specification sheet, also known as a spec sheet or tech sheet, is a detailed document that lists all the technical, material, dimensional, and aesthetic requirements of a furniture product. In upholstery production, it is a vital reference used to ensure that every component of the product is produced accurately, consistently, and in line with the client’s expectations.

A complete specification sheet enables seamless coordination between design, production, procurement, and quality control teams.

1. Key Elements of a Specification Sheet

| Component | Details Included |
| --- | --- |
| Product Name / Code | Official product reference and batch code |
| Measurements | All dimensions: height, width, depth, thickness (in mm or specified units) |
| Materials List | Frame timber, foam density, fabric name, finish type, adhesives |
| Construction Details | Frame joinery, fasteners, type of suspension (springs/webbing), stitching details |
| Tolerances | Allowable measurement variation for quality control |
| Assembly Instructions | Step-by-step build process, including sequences and special tools |
| Finishing Details | Upholstery method, fabric direction, piping, tufting, trims |
| Special Notes | Custom requests, client specifications, safety requirements, logo placement |
| Drawing Reference | Drawing number(s), scale, and version linked to visual layout |
| Quality Standards | Durability requirements, fire safety compliance, performance testing notes |

2. Why Complete Specification Sheets Are Important

- Ensure every team member has access to the same technical instructions

- Enable accurate cost estimation, cutting, and procurement

- Minimise waste and errors by providing clear tolerances and sequences

- Link visual drawings to written technical instructions

- Provide a traceable record for quality assurance and customer sign-off

Examples

- A dining chair specification sheet includes:

  - Height: 980 mm, Seat Height: 470 mm

  - Frame: Meranti

  - Seat Foam: 30/100 medium density

  - Fabric: Aruba 214 Grey (patterned)

  - Finishing: Double stitching with piping on outer edges

  - Drawing reference: DRW-CHAIR-2024-01 (scale 1:5)

- A headboard specification sheet outlines:

  - Shape: Arched, 1 200 mm high

  - Fabric direction: Vertical grain

  - Tufting: 6-button layout, 150 mm apart

  - Tolerance: ± 2 mm on frame width

Case Study

Case Study: Ashraf Checks the Sheet

Ashraf, a senior upholsterer at a Johannesburg contract furniture company, was asked to begin production on a set of sleeper couches. Before starting, he reviewed the specification sheet and noticed that the foam density listed differed from what had been pre-cut by the materials team. He halted production, cross-referenced the sheet with the project supervisor, and prevented the use of foam that would not meet the comfort standard required. His accuracy saved the company time, material, and client trust.

Discussion Points:

- What part of the specification sheet helped Ashraf identify the issue?

- How does a detailed spec sheet support accountability in the workshop?

Critical Thinking Questions

- Why is it important for a specification sheet to include both dimensions and materials?

- What problems could arise if tolerances are not clearly listed on the sheet?

- How can the absence of assembly instructions impact quality or consistency?

- Why should the spec sheet include a link to the relevant technical drawing?

- What role do specification sheets play in multi-person or team-based production environments?

### KT0703: Interpretation of Product Specifications, Sample or Prototype

Theoretical Learning Content

Interpreting product specifications, samples, or prototypes is a key skill in upholstery production. It involves analysing the available information—whether in written, visual, or physical form—and using it to guide material preparation, construction, assembly, and finishing. Interpretation means more than reading: it is about understanding the implications of dimensions, materials, tolerances, and finishes in terms of what must be done on the workshop floor.

Learners must develop the ability to connect specifications with tasks, ask clarifying questions when needed, and make informed decisions in line with what is specified or demonstrated.

1. Types of Interpretation Required

| Source | What to Interpret |
| --- | --- |
| Specification Sheet | Dimensions, foam densities, stitching details, fabric direction |
| Sample Material | Fabric quality, texture, colour, behaviour during application |
| Prototype Product | Comfort level, foam firmness, stitching layout, design proportions |
| Technical Drawing | Dimensions, chamfers, labels, tolerances |
| Customer Feedback | Special preferences not recorded in technical specs |

2. Skills Required to Interpret Correctly

- Reading and visual literacy (diagrams and drawings)

- Measurement conversion and tolerance checking

- Critical comparison of sample versus production material

- Communication with supervisors to confirm uncertainties

- Observation of details in finishings, even if not formally noted

3. Why Interpretation Is Critical in Upholstery

- Avoids incorrect material or method usage

- Enables consistent replication of approved samples or prototypes

- Allows for early identification of specification gaps

- Prevents costly rework by ensuring correct output the first time

- Supports quality assurance and client satisfaction

Examples

- A specification says “buttoning as per prototype”. The upholsterer must count the number and spacing of buttons on the prototype and apply that pattern to each production unit.

- A fabric sample marked “horizontal stripe” must be interpreted in cutting so that the stripe runs across the seat, not vertically.

- A tolerance of ± 3 mm on frame width requires the upholsterer to double-check measurements before applying covers.

Case Study

Case Study: Nandipha’s Keen Eye

Nandipha was assigned to upholster a set of dining chairs. The specification referred to “vertical grain” on the fabric. However, the actual sample attached to the document had the grain running horizontally. She discussed this with the production manager, who confirmed that the sample was correct and the sheet needed updating. Her careful interpretation ensured that the production matched the client's intent, not the text error.

Discussion Points:

- What allowed Nandipha to catch the discrepancy?

- How does interpretation sometimes require balancing written and visual information?

Critical Thinking Questions

- Why is interpretation more than just reading what is on a sheet?

- What strategies can upholsterers use to verify unclear or missing information?

- How can misinterpretation of a prototype lead to customer dissatisfaction?

- Why is it important to interpret both technical and customer specifications simultaneously?

- In what ways does correct interpretation support workplace professionalism and reputation?

### KT0704: Uses of Product Specifications, Sample or Prototype

Theoretical Learning Content

In upholstery production, product specifications, samples, and prototypes are not only reference tools—they are active instruments used throughout the design, production, and quality control phases. They provide direction, consistency, and verification at each stage, from the initial material sourcing to final product sign-off.

Understanding the uses of each helps upholsterers know when, how, and why to apply them during their workflow.

1. Uses of Product Specifications

| Use | Purpose in Upholstery Production |
| --- | --- |
| Material preparation | Guides cutting, ordering, and handling of foam, timber, fabric |
| Production consistency | Ensures all items in a batch follow the same measurements and techniques |
| Reference for tolerances | Supports dimensional accuracy and quality checks |
| Job costing and planning | Assists in estimating material quantities and labour hours |
| Compliance verification | Confirms safety, durability, or client standards are being met |

2. Uses of Samples

| Use | Purpose |
| --- | --- |
| Material validation | Tests fabric durability, stretch, visual compatibility |
| Supplier comparison | Assists in selecting the most appropriate supplier materials |
| Customer sign-off | Ensures the client approves fabric colour, feel, or finish before production |
| Training reference | Used to train new team members on the look and feel of expected outcomes |

3. Uses of Prototypes

| Use | Purpose |
| --- | --- |
| Design testing | Checks for flaws, fit, comfort, and proportion |
| Client demonstration | Offers the customer a physical preview before full production |
| Production alignment | Guides final assembly, foam shaping, and stitching layout |
| Quality benchmark | Used for inspection comparison during and after production |

Examples

- A prototyped lounge chair is used by the design team to test comfort, then passed to the workshop to match every production unit accordingly.

- A specification sheet showing 35/200 foam density ensures that the correct comfort level is replicated across all seat cushions.

- A sample swatch of velvet is used to confirm how the fabric performs when stretched over compound curves.

Case Study

Case Study: Naomi’s Quality Check

Naomi, an upholsterer at a Durban contract furniture workshop, noticed that a batch of barstools was not matching the prototype provided. The curve of the backrest was shallower than on the approved unit. She stopped production and compared the prototype to the current build, confirming the deviation. Her team reviewed the specification, noted a change in timber supplier that had affected bend radius, and corrected the problem before dispatch.

Discussion Points:

- What role did the prototype play in identifying the fault?

- How does using the prototype enhance production accuracy?

Critical Thinking Questions

- How do prototypes differ in use from specification sheets during production?

- Why are samples critical in managing client expectations?

- What risks exist if the wrong sample is referenced during material selection?

- How do product specifications support team collaboration and division of labour?

- Why is it important to retain prototypes and spec sheets even after production begins?

### KT0705: Types of Product Specifications, Sample or Prototype

Theoretical Learning Content

There are multiple types of product specifications, samples, and prototypes, each with a specific role in the furniture upholstery production process. By recognising the differences between these types, upholsterers can select the appropriate version for sourcing materials, planning production, communicating with clients, and ensuring quality consistency.

Understanding these types empowers learners to navigate both the creative and technical demands of client-driven and production-led upholstery work.

1. Types of Product Specifications

| Type | Purpose |
| --- | --- |
| Technical Specification | Provides exact measurements, materials, and tolerances for production |
| Design Specification | Focuses on aesthetic elements like fabric choice, colours, finishes |
| Functional Specification | Explains the use and performance requirements (e.g. load-bearing capacity) |
| Compliance Specification | Refers to standards such as fire resistance, safety codes, environmental rating |
| Client (Custom) Specification | Outlines unique requirements as per customer brief |

2. Types of Samples

| Sample Type | Purpose in Upholstery |
| --- | --- |
| Material Sample | Verifies texture, colour, durability, or stretch of upholstery materials |
| Construction Sample | Demonstrates how a component is built (e.g. jointing, springing) |
| Finish Sample | Shows paint, stain, or varnish results on timber or frame elements |
| Decorative Sample | Displays buttoning, piping, pleating, or tufting style |

3. Types of Prototypes

| Prototype Type | Use Case |
| --- | --- |
| Conceptual Prototype | Tests ideas in basic form—often made of low-cost or alternative materials |
| Functional Prototype | Checks ergonomics, joinery, durability, and frame construction |
| Presentation Prototype | Finished to full visual and functional standards for client approval |
| Pre-production Prototype | Final version used to test production sequence and tooling setup |

Examples

- A design specification might focus on the fabric colour and shape of the arms, while a technical specification outlines the dimensions of each frame member.

- A material sample shows three possible velvet finishes for client selection.

- A pre-production prototype is used to refine foam cutting patterns before rolling out a batch of chairs.

Case Study

Case Study: Kabelo’s Comparative Sample Board

Kabelo, an upholsterer working on a corporate reception suite, prepared a sample board with three types of fabric (leatherette, jacquard, and boucle), a stain finish on Beechwood, and a foam firmness comparison using 30/120 and 35/200 densities. The interior designer and client used this board to finalise their preferences. Later, Kabelo compared the samples to the presentation prototype before sign-off. The result was a well-coordinated, approved final product that exceeded the client’s expectations.

Discussion Points:

- How did Kabelo use different types of samples to support decision-making?

- Why was matching the prototype to the final sample critical before production?

Critical Thinking Questions

- What are the risks of using a design specification in place of a technical specification during production?

- Why is it helpful to differentiate between material samples and finish samples?

- How do presentation prototypes support both client approval and internal alignment?

- What should be done if the client’s custom specification conflicts with functional requirements?

- How can the different types of prototypes support efficiency in large-scale upholstery manufacturing?

## Integrated Assessment – KM-04-KT07

Knowledge Topic: Samples, Prototypes and Customer Specifications
Qualification: Furniture Upholsterer (SAQA ID: 103199)
NQF Level: 3
Internal Assessment Criteria:

- IAC0701–IAC0707
Assessment Weight: 5%
Total Marks: 40

🧪 Assessment Instruments Overview

| Instrument Type | Purpose | Targeted IACs |
| --- | --- | --- |
| Short Written Questions | Demonstrate understanding of concepts, purposes, and importance | IAC0702, IAC0703, IAC0704 |
| Matching Activity | Identify various aspects of specifications | IAC0705, IAC0707 |
| Scenario-Based Application | Reason and explain interpretation methods | IAC0701, IAC0706 |

✍️ Assessment Activities

Activity 1: Short Written Questions (15 marks)

Instructions: Answer the following in your own words.

- Define the term “product specification” in the context of upholstery. (3)

- What is the main purpose of creating a prototype before production begins? (3)

- Explain why product specifications are important in maintaining quality across all items in a batch. (3)

- List three key details usually found on a specification sheet. (3)

- Describe the role of a sample in client decision-making. (3)

Activity 2: Matching – Specification Types and Features (10 marks)

Instructions: Match each term in Column A with its correct explanation in Column B.

| Column A |  | Column B |
| --- | --- | --- |
| 1. Functional specification | A | Early version used for testing ideas |
| 2. Presentation prototype | B | Document focused on how the product is used and performs |
| 3. Technical specification | C | Finished sample for client sign-off |
| 4. Material sample | D | Provides details on foam, fabric, and frame materials |
| 5. Conceptual prototype | E | Used to verify texture, stretch, and visual appeal of upholstery fabrics |

Activity 3: Scenario-Based Application (15 marks)

Scenario:
You are working on a batch of armchairs for a hotel. You receive a specification sheet with incomplete stitching details. You have a prototype in the workshop and a sample board that was signed off by the client. The production programme includes delivery within 14 days.

Questions:

- What should your first step be in resolving the missing specification issue? (3)

- Which document or reference would be most useful to confirm the correct stitching method? (3)

- How does the lay plan or production programme impact your decision-making in this case? (3)

- What does this situation illustrate about the value of accurate interpretation? (3)

- List two team members you should consult with before making changes to the design. (3)

✅ Model Answers

Activity 1

- A written document describing dimensions, materials, finishes, and construction details of a product.

- To test the design, check proportions, ensure comfort, and make client-approved changes before mass production.

- Ensures consistency and reduces errors across repeated products.

- Dimensions, foam density, fabric type.

- It allows the client to see and approve the material before full production begins.

Activity 2
1–B, 2–C, 3–D, 4–E, 5–A

Activity 3

- Pause production and consult the prototype or supervisor.

- The prototype, as it reflects the final approved version.

- The delivery schedule may limit redesign time; interpretation must be quick but accurate.

- That not all information is always written and must be interpreted from other resources.

- Supervisor/production manager and quality controller/designer.

🧾 Marking Memo

| Activity | Marks | Guidance |
| --- | --- | --- |
| Activity 1 | 15 | 3 marks per complete and accurate response |
| Activity 2 | 10 | 2 marks per correct match |
| Activity 3 | 15 | 3 marks per well-reasoned, relevant response |
| Total | 40 |  |

📊 Assessment Rubric

| Rating | Descriptor |
| --- | --- |
| 5 – Excellent | Fully understands concepts; applies reasoning and interpretation effectively |
| 4 – Good | Good understanding; minor detail errors or slight interpretation gaps |
| 3 – Satisfactory | Basic understanding; demonstrates recall but limited application or reasoning |
| 2 – Basic | Some understanding; shows difficulty applying the concepts to real situations |
| 1 – Poor | Fails to demonstrate understanding or interpret the scenario correctly |

### Facilitator Assessment Briefing

KM-04-KT07: Samples, Prototypes and Customer Specifications
Qualification: Furniture Upholsterer (SAQA ID: 103199)
NQF Level: 3
Assessment Weight: 5%

🎯 Purpose of the Assessment

This integrated assessment evaluates learner competence against Internal Assessment Criteria IAC0701 to IAC0707, focusing on:

- Understanding the concept, types, and purposes of specifications, samples, and prototypes

- Applying interpretation skills to technical and customer-supplied documents

- Reasoning through real-world production scenarios involving missing or ambiguous instructions

- Linking specification content to standard operating procedures and production planning

🧪 Assessment Instruments Overview

The following tools are used:

- Short Written Questions – Tests theoretical understanding of definitions, purpose, and relevance

- Matching Task – Ensures correct identification of specification types and features

- Scenario-Based Application – Tests reasoning skills and decision-making in a production setting

Each instrument is mapped to specific IACs to ensure comprehensive coverage.

🧭 Facilitation Guidelines

Before the Assessment

- Ensure all content from KT0701 to KT0705 has been covered in class

- Review real examples of specification sheets, samples, and prototypes

- Reinforce key terminology and the relationships between documentation, visual references, and client expectations

During the Assessment

- Clarify unfamiliar words or terms without leading learners to the answer

- Allow learners to refer to printed examples of sample boards or specification sheets for contextual understanding

- Observe learners’ ability to reason through the scenario question independently

After the Assessment

- Use the model answers and rubric for fair and consistent marking

- Provide feedback on reasoning and clarity of explanations

- Retain assessment records for moderation and internal reporting

📚 Evidence Requirements

Each learner’s assessment file must include:

- Completed responses for all three assessment activities

- Supporting notes, sketches, or references used (where applicable)

- Marked rubric with assessor commentary

- All documents dated and signed by both assessor and learner

🧮 Scoring Summary

| Activity | Marks |
| --- | --- |
| Short Written Questions | 15 |
| Matching Task | 10 |
| Scenario-Based Application | 15 |
| Total | 40 |

This mark should be scaled in accordance with the 5% weighting allocated to KM-04-KT07 in the curriculum document.

🔍 Moderation Notes

- Check that learners demonstrate understanding of product specifications across formats (written, sample, and visual)

- Ensure responses to the scenario show realistic problem-solving grounded in production environments

- Use moderation reports to confirm alignment between facilitator, learner, and internal quality assurance standards

## KM-04-KT08: Efficiencies – Output, Scores and Targets (5%)

Purpose of the Knowledge Topic

This knowledge topic introduces learners to the principles and practices related to workplace productivity and efficiency in the context of upholstery production. It provides an essential foundation for understanding how targets, time management, waste reduction, and daily output monitoring contribute to the success of the production environment.

Learners will explore how to set, measure, and evaluate performance using productivity indicators, as well as how to address non-conformance and implement strategies to minimise waste. This topic empowers learners to become active participants in continuous improvement practices in their workplaces.

Key Knowledge Areas

This topic covers the following components:

- KT0801: Targets and productivity

- KT0802: Productivity principles and practices

- KT0803: Organising self and own targets and own time

- KT0804: Recording score and daily output

Internal Assessment Criteria and Weight

Learners will be assessed on their ability to:

- IAC0801: Analyse the purpose and application of targets

- IAC0802: Analyse reasons for non-conformance to targets

- IAC0803: Describe the calculation of individual efficiencies

- IAC0804: Explain and describe the processes involved in monitoring and recording daily targets

- IAC0805: Assess techniques to reduce wastage (e.g. end losses, remnants, splicing, shading)

Assessment Weight: 5%

Application in the Upholstery Workshop

In a production environment, each upholsterer contributes to team success through timely, consistent, and high-quality output. Learning how to monitor one’s daily performance, manage time, and minimise errors or material loss is critical. By mastering the principles taught in this topic, learners are better equipped to contribute to workplace efficiency, meet customer expectations, and support the sustainability of the business.

### KT0801: Targets and Productivity

Theoretical Learning Content

In an upholstery workshop, targets and productivity are central to maintaining workflow efficiency, meeting client deadlines, and ensuring profitability. Understanding how targets are set, monitored, and achieved helps upholsterers become accountable team members and contribute meaningfully to overall production performance.

This sub-topic introduces learners to the concept of workplace targets, what they measure, how they are defined, and how they support individual and organisational productivity.

1. What Are Production Targets?

Targets are measurable goals related to the amount of work or output expected within a given time frame. In upholstery, targets are usually based on:

- Number of units completed per shift or day

- Number of metres of fabric cut or applied

- Components assembled (e.g. number of chair backs upholstered)

- Time taken to complete specific processes (e.g. stitching or tacking)

Targets are set by supervisors based on production planning, material availability, and client order timelines.

2. What Is Productivity?

Productivity refers to the amount of output produced relative to the resources used (time, labour, and materials). High productivity means more quality work is completed in less time with minimal waste.

| Formula | Productivity = Output ÷ Time |

3. Types of Targets in Upholstery Production

| Type of Target | Example |
| --- | --- |
| Daily output target | Complete 6 ottoman covers per day |
| Quality target | 0% stitching defects; all piping aligned |
| Efficiency target | Complete 10 seat cushions in 6 hours |
| Waste reduction target | Less than 5% fabric remnant waste on large batch upholstery |

4. Benefits of Working with Targets

- Ensures consistent workflow and production rates

- Helps identify and correct performance issues

- Encourages time management and personal responsibility

- Allows production teams to plan deliveries and stock effectively

- Motivates staff and supports recognition for good performance

Examples

- A learner is expected to upholster five dining chair seats per day. If they complete four consistently, this may highlight a training need or an issue with time planning.

- A production team sets a target to reduce fabric waste by 10% by improving layout planning and sharing remnants.

Case Study

Case Study: Lwazi’s Performance Tracker

Lwazi, a junior upholsterer in a Gauteng-based workshop, was given a weekly target to complete 30 ottoman lids. On Monday and Tuesday, he fell behind. After reviewing his workstation and timing with his supervisor, he restructured his task flow and reached his target by Friday. His adjustment helped the team complete the full batch by the client’s deadline.

Discussion Points:

- How did targets help Lwazi identify and improve his performance?

- Why was communication with his supervisor key to meeting the target?

Critical Thinking Questions

- Why is it important for each team member to understand their individual production targets?

- What could happen if productivity is not measured or recorded in an upholstery workshop?

- How do productivity targets help supervisors make decisions about staffing and deadlines?

- What factors might prevent a team from reaching its output target, even with proper planning?

- How can personal time management influence the ability to meet daily production targets?

### KT0802: Productivity Principles and Practices

Theoretical Learning Content

Productivity principles and practices are foundational to efficient and effective upholstery production. They help upholsterers understand how to organise tasks, use time wisely, maintain quality, and contribute to a continuous improvement culture. When applied consistently, these practices support individual and team performance, reduce delays, and promote customer satisfaction.

This sub-topic focuses on strategies and behaviours that lead to sustained high productivity in the upholstery workplace.

1. Core Principles of Productivity

| Principle | Application in Upholstery |
| --- | --- |
| Time Management | Prioritise tasks and reduce idle time during cutting, stitching, or assembly |
| Task Sequencing | Follow correct order of operations for efficient workflow |
| Resource Planning | Ensure that materials, tools, and machines are ready before starting work |
| Minimising Waste | Use accurate measurements and efficient layout planning |
| Standardisation | Follow standard operating procedures (SOPs) to ensure consistency |
| Continuous Improvement (Kaizen) | Always look for better ways to organise the workspace or refine techniques |

2. Good Productivity Practices

- Prepare tools and materials before the shift starts

- Keep the workstation clean and organised to avoid delays

- Use batching techniques for repetitive tasks (e.g. tacking all seats first, then stapling)

- Report low stock or missing items before they cause production stoppages

- Record output daily to monitor individual and team performance

- Take breaks at allocated times to maintain focus during work hours

3. Factors That Influence Productivity

| Positive Factors | Negative Factors |
| --- | --- |
| Clear instructions and documentation | Incomplete specifications or unclear customer requests |
| Good lighting and ergonomic setup | Poor workstation design or cramped layouts |
| Motivated and trained workforce | Lack of training or low morale |
| Reliable machinery and sharp tools | Broken equipment or blunt tools causing delays |

Examples

- An upholsterer saves time by batching foam cutting for ten chairs in the morning and then batch-covering in the afternoon, instead of switching between tasks repeatedly.

- A team implements a colour-coded fabric tagging system to reduce confusion between different product lines during busy periods.

Case Study

Case Study: Thandi’s Toolkit Approach

Thandi, an upholsterer in Cape Town, realised she was wasting time each day walking back and forth to fetch tools. She created a mobile toolkit with all essentials (scissors, tacking tools, spare staples) and placed it next to her bench. Her productivity increased by 20%, and the supervisor asked other team members to follow her example.

Discussion Points:

- What productivity principle did Thandi apply?

- How did her idea contribute to team-wide improvement?

Critical Thinking Questions

- What small habits can an upholsterer adopt to improve daily productivity?

- How can poor task sequencing lead to delays or rework?

- Why is time management important in a shared production environment?

- How do standard operating procedures support productivity across different team members?

- What can a team do if external factors (e.g. power outages or late deliveries) disrupt productivity?

### KT0803: Organising Self and Own Targets and Own Time

Theoretical Learning Content

In a busy upholstery production environment, the ability to organise oneself, manage time effectively, and meet personal productivity targets is essential. This sub-topic helps learners develop habits that lead to self-discipline, accountability, and alignment with broader team and company goals.

Organising one's tasks and time means not only completing work efficiently, but also preparing for work, responding to interruptions, and adapting to shifting priorities while maintaining quality and consistency.

1. Key Elements of Self-Organisation in the Workplace

| Element | Description |
| --- | --- |
| Daily Planning | Preparing a to-do list aligned with production targets |
| Prioritisation | Identifying urgent vs important tasks (e.g. foam cutting before stitching) |
| Time Allocation | Estimating how long tasks will take and sticking to a schedule |
| Workspace Readiness | Ensuring materials and tools are ready before starting |
| Progress Monitoring | Checking how much has been completed and adjusting where needed |
| Break Discipline | Taking breaks at allocated times without exceeding limits |

2. Managing Own Targets

- Understand daily or weekly output requirements

- Record progress regularly (e.g. using a score sheet or time log)

- Identify when you are falling behind and seek help proactively

- Reflect on your performance at the end of each shift

- Discuss realistic goals with your supervisor if there are consistent challenges

3. Time-Wasters to Avoid

| Common Time-Wasters | Solutions |
| --- | --- |
| Frequent trips to fetch tools | Prepare workstation before shift starts |
| Waiting for supervisor clarification | Review spec sheets or communicate issues early |
| Idle chatting during work hours | Focus on targets and shift time management |
| Not following sequence of tasks | Use production flow plans or SOPs |

Examples

- An upholsterer starts the day by reviewing their output target, checks their toolset, pre-cuts fabric for the morning tasks, and sets a 2-hour block for foam shaping.

- Another employee, working without planning, spends the first 30 minutes of the day looking for fabric and confirming specs, which leads to missed targets by the afternoon.

Case Study

Case Study: Jabu’s Shift Turnaround

Jabu, a learner at a factory in East London, was struggling to meet his chair-back upholstery targets. After discussing the issue during his progress review, he began tracking his time and created a daily prep checklist. He soon realised he was losing 45 minutes per day on material prep and trips to the storeroom. By starting earlier and pre-packing materials, he exceeded his targets consistently.

Discussion Points:

- What changes helped Jabu improve his time use?

- How did self-monitoring lead to improved productivity?

Critical Thinking Questions

- How can self-organisation affect team performance in a shared workshop?

- Why is it important to recognise personal patterns of time-wasting?

- How does planning your tasks in advance improve your ability to meet daily targets?

- In what ways does managing your time contribute to customer satisfaction?

- What steps can a learner take to balance quality and speed without cutting corners?

### KT0804: Recording Score and Daily Output

Theoretical Learning Content

In upholstery production, recording score and daily output is a vital part of maintaining productivity, identifying training needs, and ensuring that production targets are being met. This process helps workers and supervisors track progress, allocate resources, and adjust workflows where necessary.

Recording output involves both quantitative tracking (units completed, time spent) and qualitative assessment (accuracy, finish, waste reduction). It forms part of a broader production monitoring and reporting system.

1. What Is Daily Output Recording?

Daily output recording refers to the documentation of work completed by an upholsterer or team during a shift or workday. This includes:

- Number of items completed (e.g. 8 seat cushions finished)

- Type of product worked on (e.g. dining chairs, headboards)

- Time taken to complete key tasks

- Reasons for shortfalls or delays (if applicable)

- Comments on material availability or technical issues

This information is usually recorded on a score sheet, production register, or digital logbook.

2. Purpose and Benefits

| Purpose | Benefits to the Upholstery Workshop |
| --- | --- |
| Monitor progress | Helps identify when a team is ahead or behind schedule |
| Identify performance trends | Supports training, recognition, or adjustment of targets |
| Traceability | Connects specific outputs to individual team members for accountability |
| Quality control | Links production outcomes to material usage, faults, or client feedback |
| Workflow planning | Allows supervisors to reassign tasks or redistribute workload efficiently |

3. Components of a Score Sheet

A typical daily score sheet may include:

| Field | Example Entry |
| --- | --- |
| Date | 12 April 2025 |
| Name | Sibusiso Mokoena |
| Task | Upholstering dining chair backs |
| Target | 10 units |
| Output | 9 units completed |
| Notes | 1 hour delay due to foam shortage |

Examples

- A learner uses a printed score sheet to record that they have cut and covered 5 ottoman tops, falling one unit short of the target due to a staple gun issue.

- A team supervisor uses the output records to determine that an employee consistently exceeds daily targets on chair backs and recommends them for further training on full-frame upholstery.

Case Study

Case Study: Jerome’s Accountability Boost

Jerome, a learner upholsterer in Mpumalanga, was asked to start recording his output daily after missing targets in his second month. He used a simple Excel log and began tracking not only what he completed, but also any issues affecting progress. This process made him more aware of how time was being spent and led to discussions with his supervisor on improving foam prep timing. Within two weeks, his scores improved, and his confidence grew.

Discussion Points:

- How did tracking his daily output help Jerome reflect on his performance?

- What role did documentation play in solving his production issue?

Critical Thinking Questions

- Why is it important for each upholsterer to record their own daily output?

- What kinds of patterns can be identified by looking at weekly score sheets?

- How can output recording support decision-making by workshop supervisors?

- In what ways does recording output promote responsibility and professionalism?

- What systems or tools could be used to make daily recording easier and more accurate?

## Integrated Assessment – KM-04-KT08

Knowledge Topic: Efficiencies – Output, Scores and Targets
Qualification: Furniture Upholsterer (SAQA ID: 103199)
NQF Level: 3
Internal Assessment Criteria (IACs):

- IAC0801: The purpose and application of targets are analysed

- IAC0802: Reasons for non-conformance to targets are analysed

- IAC0803: The calculation of individual efficiencies are described

- IAC0804: The processes involved in monitoring and recording daily targets is explained and described

- IAC0805: Techniques to reduce wastage (e.g. end losses, remnants, splicing, shading) are assessed
Assessment Weight: 5%
Total Marks: 40

🧪 Assessment Instruments Overview

| Instrument Type | Purpose | IACs Assessed |
| --- | --- | --- |
| Short Answer Questions | Assess understanding of concepts and processes | IAC0801, IAC0802, IAC0804 |
| Calculation Task | Assess ability to compute individual productivity and efficiency | IAC0803 |
| Case Study and Application | Assess application of waste-reduction techniques and reasoning | IAC0805, IAC0802, IAC0804 |

✍️ Assessment Activities

Activity 1: Short Answer Questions (15 marks)

Instructions: Answer the following in your own words.

- What is the purpose of setting daily production targets in an upholstery workshop? (3)

- Describe two possible reasons why an upholsterer might not meet their daily target. (4)

- Explain the importance of recording daily output on a score sheet. (4)

- List four items typically recorded on an individual production log. (4)

Activity 2: Efficiency Calculation (10 marks)

Instructions: Use the information provided to calculate the upholsterer’s individual efficiency.

Scenario:
Naledi is expected to upholster 12 dining chair backs per day. On Tuesday, she completed 10 units.

- Calculate her efficiency as a percentage. Show your working. (5)

- Interpret the result: What does this tell us about Naledi’s productivity for the day? (5)

Activity 3: Case Study Application (15 marks)

Scenario:
A team of upholsterers has been falling short of their weekly targets. On investigation, the supervisor discovers that materials are being wasted due to poor planning and disorganised fabric layout. There are also frequent delays due to unclear spec sheets and missing fasteners.

Questions:

- Identify two techniques that can be used to reduce material wastage in this scenario. (4)

- Explain how a better layout plan can improve productivity and reduce end losses. (3)

- Suggest two ways the team can monitor their output more effectively each day. (4)

- Describe one way that communication with team members can help prevent non-conformance to targets. (4)

✅ Model Answers

Activity 1

- Targets help measure productivity, ensure on-time delivery, and guide team planning.

- (i) Machine or tool breakdowns, (ii) Lack of time management or unclear instructions.

- It provides a record of work completed, supports performance reviews, and helps identify issues early.

- Date, task completed, quantity, reason for shortfall.

Activity 2

- Efficiency = (Actual ÷ Target) × 100 = (10 ÷ 12) × 100 = 83.33%

- Naledi achieved 83.33% of her target. This indicates she is slightly underperforming and may need to review her workflow.

Activity 3

- (i) Pre-plan fabric layout to minimise remnants, (ii) Share offcuts or reuse for smaller components.

- A layout plan reduces wasted material and ensures fabric is cut with minimal gaps between shapes.

- (i) Use a daily output register, (ii) Keep personal score sheets at each workstation.

- Open communication helps flag delays early and allows the team to adjust tasks or assist one another.

🧾 Marking Memo

| Activity | Marks | Notes |
| --- | --- | --- |
| Activity 1 | 15 | Accept clear, relevant responses with key concepts |
| Activity 2 | 10 | 5 for correct calculation, 5 for accurate interpretation |
| Activity 3 | 15 | Evaluate reasoning, practical relevance, and completeness |
| Total | 40 |  |

📊 Assessment Rubric

| Rating | Descriptor |
| --- | --- |
| 5 – Excellent | Thorough, accurate answers; correct calculation and practical application |
| 4 – Good | Mostly correct with some minor omissions or misinterpretations |
| 3 – Satisfactory | Basic understanding; may lack depth or real-world application |
| 2 – Basic | Shows awareness of concepts but lacks clarity or relevance |
| 1 – Poor | Incomplete or incorrect responses; lacks understanding |

### Facilitator Assessment Briefing

KM-04-KT08: Efficiencies – Output, Scores and Targets
Qualification: Furniture Upholsterer (SAQA ID: 103199)
NQF Level: 3
Assessment Weight: 5%

🎯 Purpose of the Assessment

This assessment evaluates learners’ understanding of efficiency concepts in upholstery production, including the application and monitoring of targets, productivity calculations, output recording, and techniques for reducing material waste. The aim is to ensure learners are able to manage their workday productively and apply practical solutions in a real-world workshop context.

🧪 Assessment Instruments Overview

- Short Answer Questions: Assesses conceptual understanding of targets, output recording, and productivity reasoning

- Efficiency Calculation Task: Tests learners’ ability to calculate and interpret individual efficiency percentages

- Scenario-Based Application: Applies reasoning to workplace challenges involving non-conformance, communication, and waste reduction

🧭 Facilitation Guidelines

Before the Assessment:

- Ensure KT0801 to KT0804 have been fully covered and reinforced through practical demonstration and daily score practice

- Provide learners with mock score sheets or workshop examples as preparation

- Revise the concepts of efficiency and waste reduction in group discussions

During the Assessment:

- Clarify task instructions without leading to specific answers

- Ensure learners use calculators where necessary and show all working steps

- Encourage time management during completion of the scenario-based activity

After the Assessment:

- Use the model answers and rubric for consistent and objective marking

- Provide clear, individual feedback and guide learners in identifying their own improvement areas

- Retain records and scoring breakdowns for moderation and internal reporting

📚 Evidence Requirements

Each learner’s file must include:

- Completed responses to all activities

- Any calculations or working notes used

- A signed and dated assessment checklist and mark sheet

- Rubric scores and assessor’s commentary

🧮 Scoring Summary

| Activity | Marks |
| --- | --- |
| Short Answer Questions | 15 |
| Efficiency Calculation Task | 10 |
| Case Study Application | 15 |
| Total | 40 |

This should be scaled according to the 5% weighting of KM-04-KT08 in the qualification structure.

🔍 Moderation Notes

- Confirm that learners demonstrate sound understanding of productivity language and output documentation

- Check calculations for accuracy and interpretation in context

- Ensure scenario responses reflect real-world workplace thinking and correct technique application

- Use moderation samples and feedback to calibrate future assessments

## KM-04-KT09: Inspection and Disposal of Finished Upholstery Products (5%)

Purpose of the Knowledge Topic

This knowledge topic introduces learners to the principles, processes, and importance of inspecting and disposing of finished upholstery products within a production environment. Upholsterers must be able to evaluate the quality of their work against production specifications, identify non-conformances, and apply appropriate disposal techniques in accordance with workplace policies.

This topic equips learners to contribute actively to maintaining high quality standards in the workshop by recognising defects, ensuring accurate reporting, and participating in corrective and preventative measures.

Key Knowledge Areas

This topic covers the following components:

- KT0901: Inspection principles

- KT0902: Techniques of disposing according to product type

- KT0903: Quality in manufacturing processes and products

Internal Assessment Criteria and Weight

Learners will be assessed on their ability to:

- IAC0901: Define inspection principles and practices and explain their function

- IAC0902: Describe the purpose of final product inspection

- IAC0903: Define product non-conformance and analyse reasons for its occurrence

- IAC0904: Justify the importance of reporting non-conformance

- IAC0905: Describe a range of in-company disposal techniques and explain their function

Assessment Weight: 5%

Application in the Upholstery Workshop

In upholstery manufacturing, final product inspection is critical for verifying quality, meeting customer expectations, and avoiding returns or reputational damage. Learners will apply their understanding of quality standards, non-conformance reporting, and disposal methods to support efficient and compliant production processes. This knowledge also contributes to building a culture of accountability and continuous improvement within the workplace.

Bottom of Form

### KT0901: Inspection Principles

Theoretical Learning Content

Inspection in upholstery manufacturing refers to the systematic process of examining finished products to ensure they meet the required quality standards, customer specifications, and workplace expectations. It is one of the final steps before products are dispatched to clients or moved to storage. Effective inspection supports quality assurance, customer satisfaction, and business reputation.

This sub-topic introduces learners to the principles of inspection, including what to inspect, when, how, and why, with emphasis on consistency, accuracy, and adherence to defined standards.

1. Purpose of Inspection

- To confirm that the finished product matches the specification sheet and prototype

- To identify any defects or inconsistencies before delivery

- To ensure that the functionality, appearance, and finish meet client expectations

- To provide documented feedback for continuous improvement

- To support decision-making for approval, repair, rework, or disposal

2. Core Principles of Inspection

| Principle | Application in Upholstery Production |
| --- | --- |
| Objectivity | Inspect based on facts, not opinion—use measurements and standards |
| Consistency | Apply the same inspection criteria to every product in the batch |
| Thoroughness | Check all relevant areas: stitching, foam firmness, upholstery lines, fasteners |
| Documentation | Record findings clearly using inspection checklists or reports |
| Timeliness | Conduct inspection before items are packaged or sent to dispatch |

3. What Is Inspected in Upholstery?

- Visual defects: stains, marks, wrinkles, stitching faults

- Structural quality: loose frames, rocking, uneven legs

- Measurement: final product dimensions, symmetry, compliance with specification

- Comfort: foam density and consistency, support

- Upholstery finish: alignment of patterns, pleating, tucking, or buttoning quality

Examples

- A quality controller uses a checklist to inspect a batch of tub chairs and finds that two items have misaligned fabric grain and are returned for re-upholstery.

- An upholsterer inspects their own completed dining chair to check that the piping is even and corners are well folded before submitting the piece for final QC.

Case Study

Case Study: Nomsa’s Catch

Nomsa, a junior upholsterer in a Pretoria factory, was asked to perform the final inspection on a batch of headboards. She discovered that three of them had uneven button spacing compared to the prototype. Although the stitching and fabric were perfect, she flagged the inconsistency. The team corrected the layout before delivery. Her inspection avoided potential client dissatisfaction and a costly return.

Discussion Points:

- What principle did Nomsa apply in her inspection?

- How did her observation contribute to the company’s quality reputation?

Critical Thinking Questions

- Why is it important to follow the same inspection process for every item in a production batch?

- How does accurate inspection benefit the customer and the company?

- What risks are associated with skipping or rushing through product inspection?

- How can a workplace ensure that all team members understand and apply consistent inspection standards?

- What role does inspection documentation play in supporting quality improvement?

### KT0902: Techniques of Disposing According to Product Type

Theoretical Learning Content

In upholstery production, not all completed items are suitable for dispatch. Some products may be damaged, faulty, or non-conforming and must be handled responsibly. Disposal does not always mean throwing the item away; instead, it refers to how an organisation manages non-conforming goods through rework, recycling, repurposing, or, in some cases, discarding.

Understanding techniques of disposing according to product type enables learners to contribute to workplace efficiency, sustainability, and compliance with in-company procedures.

1. Why Disposal Is Necessary

- To remove products that fail inspection or pose a risk to clients

- To clear storage and prevent clutter or confusion in dispatch areas

- To recover usable materials (e.g. foam, fabric, timber) from defective items

- To follow quality management systems and standard operating procedures

- To prevent reputational damage from shipping substandard goods

2. Disposal Techniques by Product Type

| Product Condition | Disposal Technique |
| --- | --- |
| Repairable faults | Sent to rework station for correction (e.g. refitting fabric, fixing a loose leg) |
| Material faults | Recover usable components (e.g. foam, timber), dispose of failed parts responsibly |
| Minor defects | Sold as seconds or factory outlet items at a discounted price |
| Severe structural faults | Disassembled and disposed of following company waste protocols |
| Hazardous or spoiled items | Disposed of through registered waste service providers (especially if involving adhesives or chemicals) |

3. Disposal Must Be:

- Authorised: Done only under instruction or according to SOPs

- Documented: Each disposal must be recorded and justified

- Safe: Follow health and safety practices when disassembling or discarding

- Environmentally Responsible: Recycle and repurpose where possible

Examples

- A dining chair with a cracked leg is sent to the repairs team. After fixing the defect, it is re-inspected before being approved.

- A sofa with a stained fabric cover beyond repair is disassembled. Foam is repurposed for padding smaller items, and fabric is disposed of following waste policies.

Case Study

Case Study: Reuben’s Decision

Reuben, a production supervisor in an upholstery workshop in Limpopo, finds five upholstered benches that do not meet specification due to fabric shading inconsistency. Rather than discarding them, he checks with management and has them reclassified as factory seconds to be sold through the showroom at a reduced rate. This decision saves the company material cost and reduces waste.

Discussion Points:

- How did Reuben apply responsible disposal techniques?

- What did this example show about matching disposal technique to product condition?

Critical Thinking Questions

- Why is it important to follow in-company disposal procedures rather than making informal decisions?

- What factors should be considered before choosing to repair, repurpose, or discard a non-conforming product?

- How does disposal contribute to quality assurance and customer satisfaction?

- What are the risks of failing to dispose of rejected products correctly?

- In what ways can disposal techniques support environmental sustainability in the upholstery sector?

### KT0903: Quality in Manufacturing Processes and Products

Theoretical Learning Content

In upholstery manufacturing, quality refers to how well a product meets the required specifications, standards, and customer expectations. Quality must be embedded throughout the entire manufacturing process—from design and material selection to frame building, upholstery, finishing, and inspection. Every team member plays a role in ensuring quality is consistent, measurable, and continuously improved.

This topic equips learners with a clear understanding of what quality means in practice and how to maintain it in daily production.

1. What Is Quality in Upholstery Production?

Quality means that:

- The final product is safe, durable, and comfortable

- Materials and workmanship match the approved specification or prototype

- Aesthetics such as stitching, piping, and pattern alignment are consistent and accurate

- The product is functional, stable, and defect-free

2. Quality in the Manufacturing Process

| Production Stage | Quality Considerations |
| --- | --- |
| Frame Assembly | Accurate joinery, squareness, strength, absence of loose parts |
| Foam and Padding | Correct density, no lumps or voids, properly glued and shaped |
| Fabric Cutting and Layout | Efficient use of fabric, correct grain direction, minimised shading variation |
| Upholstery Application | Even tension, neat corners, correct use of fasteners and trims |
| Finishing | Clean surface, aligned patterns, no staples or glue visible |
| Inspection and Packaging | Final checks, proper labelling, protective packaging |

3. Indicators of Good Quality

- Accurate dimensions according to specification

- Symmetry and uniformity between items in a batch

- Smooth, tight upholstery without wrinkles or uneven pull

- Proper alignment of buttons, pleats, and piping

- Functioning mechanisms (if applicable), e.g. reclining or drawer systems

4. Role of the Upholsterer in Maintaining Quality

- Read and apply specifications carefully

- Report material defects or unclear instructions immediately

- Use correct tools and standard operating procedures (SOPs)

- Maintain a clean, organised workspace

- Participate in inspections and take corrective action when necessary

Examples

- A batch of ottomans is inspected. One unit has a misaligned fabric pattern due to incorrect grain orientation during cutting. It is flagged and corrected before dispatch.

- An upholsterer double-checks foam density against the specification sheet before gluing, ensuring the correct level of comfort and resilience.

Case Study

Case Study: Rosina’s Quality Routine

Rosina works in a quality-conscious upholstery studio in Gqeberha. Before beginning each job, she reviews the specification sheet, checks that materials are correct, and sets up her tools in advance. She inspects each completed item herself, even before the final QC check. Her team’s rejection rate dropped by 60% after she shared her workflow during a team training session.

Discussion Points:

- How did Rosina’s approach impact her team’s overall quality?

- What habits can other upholsterers adopt from her routine?

Critical Thinking Questions

- What are the consequences of overlooking small quality issues during production?

- How does each stage of the manufacturing process influence the final product’s quality?

- In what ways can poor material handling affect product appearance and performance?

- Why is it important to apply quality control even when the customer may not immediately notice minor flaws?

- How can teams create a culture of quality in a busy upholstery production environment?

## Integrated Assessment – KM-04-KT09

Knowledge Topic: Inspection and Disposal of Finished Upholstery Products
Qualification: Furniture Upholsterer (SAQA ID: 103199)
NQF Level: 3
Internal Assessment Criteria (IACs):

- IAC0901: Inspection principles and practices are defined and the function thereof is explained

- IAC0902: The purpose of final product inspection is described

- IAC0903: Product non-conformance is defined and reasons for the occurrence of non-conformance are analysed

- IAC0904: The importance of reporting non-conformance is justified

- IAC0905: A range of in-company disposal techniques are described and the function thereof is explained
Assessment Weight: 5%
Total Marks: 40

🧪 Assessment Instruments Overview

| Instrument Type | Purpose | IACs Assessed |
| --- | --- | --- |
| Short Answer Questions | Define and explain inspection and non-conformance concepts | IAC0901, IAC0902, IAC0903 |
| Scenario-Based Questions | Apply inspection and reporting procedures to practical examples | IAC0903, IAC0904 |
| Matching Exercise | Identify disposal techniques and their correct application | IAC0905 |

✍️ Assessment Activities

Activity 1: Short Answer Questions (15 marks)

Instructions: Answer the following clearly and in full sentences.

- Define “inspection” and explain its function in upholstery production. (4)

- What is the purpose of conducting a final product inspection before dispatch? (3)

- Define “non-conformance” and list two possible reasons it may occur during production. (4)

- Why is it important to report non-conforming products in the upholstery workshop? (4)

Activity 2: Scenario-Based Questions (15 marks)

Scenario:
You are completing a final check on a batch of sofas. You notice that two of the sofas have uneven armrests and one has visible staple marks on the rear panel. These issues are not in line with the approved specification.

Questions:

- What should your immediate response be to this situation? (3)

- Which team member or department should be notified about the defects? (2)

- Describe how these defects could have occurred during the manufacturing process. (4)

- What documentation or tools should be used to report the non-conformance? (3)

- Why is it necessary to investigate and report every non-conforming item, even if it seems minor? (3)

Activity 3: Matching Exercise – Disposal Techniques (10 marks)

Instructions: Match each product condition in Column A with the most appropriate disposal technique in Column B.

| Column A: Product Condition |  | Column B: Disposal Technique |
| --- | --- | --- |
| 1. Frame cracked beyond repair | A | Resell at a reduced price with disclaimer |
| 2. Fabric shows slight shading | B | Reupholster or rework with matching fabric |
| 3. Loose button on backrest | C | Return to repair station for rework |
| 4. Severe structural misalignment | D | Disassemble and follow in-company waste protocol |
| 5. Slight scratch on leg | E | Mark as factory second and sell in staff or clearance outlet |

✅ Model Answers

Activity 1

- Inspection is the process of checking a finished product to ensure it meets quality standards. It ensures defects are caught early and products meet customer expectations.

- To verify the item is safe, functional, and matches specifications before leaving the factory.

- Non-conformance is when a product does not meet set standards. Causes include incorrect measurements or using the wrong materials.

- Reporting ensures the issue is addressed, prevents recurrence, and maintains customer satisfaction.

Activity 2

- Isolate the affected items and mark them for review.

- Report to the quality controller or production supervisor.

- Uneven armrests may result from incorrect frame assembly; staple marks could be due to poor finishing or tool misuse.

- Use the non-conformance report form or inspection checklist.

- Small defects affect quality perception and client trust, and repeated errors could indicate deeper training or process issues.

Activity 3
1–D, 2–B, 3–C, 4–D, 5–E

🧾 Marking Memo

| Activity | Marks | Comments |
| --- | --- | --- |
| Activity 1 | 15 | Full marks for clear, complete definitions and correct reasoning |
| Activity 2 | 15 | Marks based on practical application and relevant documentation |
| Activity 3 | 10 | 2 marks per correct match |
| Total | 40 |  |

📊 Assessment Rubric

| Rating | Descriptor |
| --- | --- |
| 5 – Excellent | Clear understanding, accurate application, and insightful analysis |
| 4 – Good | Sound knowledge, minor errors in interpretation or application |
| 3 – Satisfactory | Adequate understanding, some gaps in detail or clarity |
| 2 – Basic | Partial knowledge with noticeable misunderstanding or vagueness |
| 1 – Poor | Very limited or incorrect understanding; does not meet outcome expectations |

### Facilitator Assessment Briefing

KM-04-KT09: Inspection and Disposal of Finished Upholstery Products
Qualification: Furniture Upholsterer (SAQA ID: 103199)
NQF Level: 3
Assessment Weight: 5%

🎯 Purpose of the Assessment

This integrated assessment evaluates the learner’s ability to define and apply quality inspection principles, understand non-conformance, and implement proper disposal procedures for faulty products. The goal is to ensure that learners are prepared to uphold workshop standards through accurate inspection, responsible reporting, and appropriate decision-making regarding rework or disposal.

🧪 Assessment Instruments Overview

- Short Answer Questions – Assess learners’ understanding of inspection, non-conformance, and reporting

- Scenario-Based Questions – Evaluate application of inspection procedures and decision-making

- Matching Task – Test understanding of disposal methods appropriate to product condition

🧭 Facilitation Guidelines

Before the Assessment

- Review inspection checklists and SOPs during practical sessions

- Reinforce understanding of acceptable tolerances and quality markers

- Provide examples of disposal tags, fault logs, or non-conformance reports

During the Assessment

- Clarify instructions where needed without leading responses

- Ensure all calculations and reasoning steps are clearly shown

- Encourage learners to relate responses to real upholstery scenarios

After the Assessment

- Use the provided model answers and rubric for consistent marking

- Discuss common non-conformance types observed during training

- Provide feedback on inspection skills during practical observations

📚 Evidence Requirements

Each learner’s evidence file must include:

- Completed assessment with clearly written responses

- All relevant calculations or rationale, especially in scenario answers

- Completed rubric and mark sheet, signed by assessor and learner

- Copies of any workplace documentation used in the assessment (if applicable)

🧮 Scoring Summary

| Activity | Marks |
| --- | --- |
| Short Answer Questions | 15 |
| Scenario-Based Application | 15 |
| Matching Task – Disposal | 10 |
| Total | 40 |

Scores should be scaled to reflect the 5% weighting of this topic in the overall qualification structure.

🔍 Moderation Notes

- Check that learners link theory to practical processes in inspection and reporting

- Ensure that answers demonstrate a clear grasp of in-company disposal expectations

- Include moderation feedback notes where learner interpretations vary significantly

- Retain assessment copies and rubric evidence for QCTO or SETA verification

Bottom of Form