FURNITURE UPHOLSTERER

KM13 MANUFACTURING PRINCIPLES AND PROCESSES FOR UPHOLSTERED FURNITURE PROTOTYE MANUFACTURING AND REENGINEERING

FACILITATOR GUIDE

Table of Contents

KM-13: Manufacturing Principles and Processes for Upholstered Furniture Prototype Manufacturing and Reengineering	5

KM-13-KT01: Principles of Manufacturing Upholstered Furniture Prototypes (10%)	7

KT0101: Terminology and Definitions	9

KT0102: Principles and Concepts of Prototype Manufacturing	12

KT0103: Methods of Prototype Modelling	15

KT0104: Specifications and Design Concepts, including 2D and 3D technical drawings, renderings, sketches, samples, cutting and sewing plans, and written work instructions	17

KT0105: Prototyping Process, Procedures and Role Players and Techniques	20

KT0106: Definition and Purpose of Dry-Run / Mock-Up Assembly	23

KT0107: Standards Set Out by Prototyping with Reference to Foams, Fabrics and Frames	25

KT0108: Sequence of Prototyping Manufacturing	28

KT0109: Accuracy, Detail and Faults	31

📝 Integrated Assessment for KM-13-KT01	34

‍🏫 Facilitator Assessment Briefing	40

KM-13-KT02: Prototype Component Construction – Frame Construction (20%)	42

KT0201: Terminology, Definitions and Abbreviations of Upholstered Furniture Frame Components and Parts	44

KT0202: Raw Materials Used to Make Frames for Simple, Complex, Fully Upholstered and Show Wood Furniture	47

KT0203: Types and Functions of Machines Used to Make Frames	50

KT0204: Principles and Concepts of Joints and Joint Construction in Upholstered Furniture Frames	53

KT0205: Principles and Concepts of Upholstered Furniture Frame Construction with Reference to Cutting Lists and Tolerances, Routing Sheets, Material Selection and Fault Finding, Quality Requirements and Materials Handling	56

KT0206: Personal Protective Equipment and Hazards and Unsafe Acts in the Frame Making Area	59

KT0207: Principles and Concepts of Making Furniture Frames for Mechanical or Powered Movement Attachments (Reclining, Lifting, Heating or Vibrating Mechanisms)	62

Integrated Assessment – KM-13-KT02	65

🏫 Facilitator Assessment Briefing	71

KM-13-KT03: Prototype Component Construction – Modern Seat Construction (20%)	73

KT0301: Principles of Constructing Shaped Seats Using Modern Springs and Twine / Cord Tying and Knotting Techniques	75

KT0302: Differentiate Between Densities of Foam Used to Construct Components for Upholstered Furniture Seats, Arms, Backs	78

KT0303: Principles of Selecting and Layering Different Densities of Foam to Construct Components for Complex Upholstered Furniture Seats, Arms, Backs Including Wing Back Chairs	81

KT0304: Safety and Correct Handling of Hand and Powered Cutting Tools to Prepare and Cut Foam	84

KT0305: Personal Protective Equipment and Safe Preparation and Spray Gun Setting Processes for Applying Adhesive to Foam	87

KT0306: Principles of Applying Adhesives to Foam Using Manual and Spraying Techniques	90

KT0307: Principles of Manual Foam Pulling Techniques to Construct and Shape Layers of Foam to Form Upholstered Furniture Components	95

Integrated Assessment – KM-13-KT03	99

‍🏫 Facilitator Assessment Briefing	103

KM-13-KT04: Prototype Component Construction – Traditional Seat Construction (20%)	105

KT0401: Terminology, Definitions and Abbreviations	107

KT0402: Principles and Concepts of Traditional Frame Preparation, Raw Materials and Consumables	110

KT0403: Hand Tools and Power Tools Used for Traditional Upholstered Furniture Foundations	113

KT0404: Principles and Concepts of Arranging and Attaching Webbing and Coil Springs, and Tying Springs to Produce Seats of Different Shapes and Firmness	116

KT0405: Principles of Inserting and Arranging Padding Around and Over the Springs to Produce the Desired Seat Shape	119

KT0406: Principles of Sprung Edges and Edge Rolls	122

Integrated Assessment – KM-13-KT04	125

🏫 Facilitator Assessment Briefing	129

KM-13-KT05: Testing, Evaluation and Finalisation of Upholstery Prototypes (15%)	131

KT0501: Significance of Prototyping for Troubleshooting Potential Problems Prior to Production	133

KT0502: Principles and Methods of Testing, Evaluating and Modifying Prototypes	136

KT0503: Implications of Faults Not Identified Through the Prototype Process in Production, Sale and Use of the Product	139

KT0504: Importance of Engaging with Prospective Users and Producers of the Production Model	142

KT0505: Principles and Concepts of Calculating and Evaluating Resource and Capacity Costs and Capabilities	146

KT0506: Principles of Adjusting, Adapting, or Re-engineering Prototypes	149

Integrated Assessment – KM-13-KT05	152

🏫 Facilitator Assessment Briefing	155

KM-13-KT06: Product Re-engineering (15%)	157

KT0601: Definitions and Terminology Used When Re-engineering Upholstered Furniture	159

KT0602: Principles and Concepts of Re-engineering, Re-prototyping and Re-designing Upholstered Furniture Prototypes	162

KT0603: Factors Influencing the Decision to Re-engineer Prototypes	165

KT0604: Fault Finding and Root Cause Analysis of the Process, Design or Materials Faults	168

KT0605: Record Keeping	172

Integrated Assessment – KM-13-KT06: Product Re-engineering	175

‍🏫 Facilitator Assessment Briefing	179

# KM-13: Manufacturing Principles and Processes for Upholstered Furniture Prototype Manufacturing and Reengineering

NQF Level 4 | Credits 17

13.1 Purpose of the Knowledge Module

The purpose of this Knowledge Module is to provide learners with a comprehensive understanding of the principles, methods and processes used in the manufacturing of upholstered furniture prototypes, including bespoke and bulk production furniture. The module equips learners with the foundational and applied knowledge necessary to construct, evaluate, and reengineer upholstered furniture products in line with industry standards.

Learners will be enabled to interpret and apply manufacturing principles, utilise construction techniques specific to both modern and traditional seat designs, and critically analyse the outcomes of prototype testing and reengineering interventions. This learning will support competence in designing and improving production processes and in producing high-quality furniture items tailored to customer requirements.

The module covers all essential aspects of upholstery prototype production, from component construction through to evaluation and refinement. By the end of this module, learners will have the knowledge to confidently participate in and contribute to the complete upholstery manufacturing cycle, with an emphasis on prototype manufacturing and continuous improvement.

13.2 Key Knowledge Areas

The Knowledge Module consists of the following six key topics:

- KM-13-KT01: Principles of manufacturing upholstered furniture prototypes (10%)

- KM-13-KT02: Prototype component construction – Frame construction (20%)

- KM-13-KT03: Prototype component construction – Modern seat construction (20%)

- KM-13-KT04: Prototype component construction – Traditional seat construction (20%)

- KM-13-KT05: Testing, evaluation and finalisation of upholstery prototypes (15%)

- KM-13-KT06: Product reengineering (15%)

Each topic addresses specific manufacturing concepts, materials, processes, and quality principles related to upholstery prototype development and improvement.

13.3 Internal Assessment Criteria and Weighting

The internal assessment for this Knowledge Module will evaluate the learner’s ability to:

- Demonstrate knowledge of manufacturing principles and construction processes used in upholstery prototype production.

- Identify and apply suitable techniques for the construction of both traditional and modern seat components.

- Explain and apply testing and evaluation procedures for upholstery prototypes.

- Demonstrate the ability to assess and implement reengineering principles to address product faults and improve design and production.

Each topic is weighted in alignment with the overall credit allocation and topic complexity, with specific assessment criteria detailed under each Knowledge Topic.

13.4 Application in the Furniture Upholstery Sector

This module is directly aligned with occupational responsibilities and contexts found in furniture upholstery manufacturing environments, particularly in departments focused on prototyping, custom manufacturing, and product development. Skills acquired in this module will support the production of high-quality prototypes that are aligned with client specifications, cost-effective production methods, and modern design innovations.

By mastering the competencies in KM-13, learners will contribute meaningfully to innovation and quality assurance in upholstery manufacturing operations. These outcomes reflect core expectations for Furniture Upholsterers operating at NQF Level 4 within South Africa’s broader manufacturing and design sector.

## KM-13-KT01: Principles of Manufacturing Upholstered Furniture Prototypes (10%)

Introduction for the Facilitator Guide

Purpose of the Knowledge Topic

The purpose of this Knowledge Topic is to introduce learners to the foundational principles and practices involved in the manufacturing of upholstered furniture prototypes. Prototyping plays a critical role in the design-to-production cycle, allowing manufacturers to evaluate the feasibility, accuracy, and functionality of design concepts before progressing to bulk production or bespoke manufacture.

This topic provides a comprehensive understanding of prototype development as a structured, iterative process involving the interpretation of specifications, technical drawings, and work instructions. Learners will explore methods of prototype modelling, the sequencing of processes, mock-up assemblies, and quality benchmarks set for frame, foam, and fabric components during prototyping.

Emphasis is placed on using correct terminology, understanding standard procedures, and applying quality standards to ensure prototypes meet design intentions and functional requirements. The knowledge gained from this topic equips learners to identify and prevent faults at the early stages of development, ensuring successful transition to production with minimal rework or quality issues.

Key Knowledge Areas

This topic will cover the following key elements:

- KT0101 Terminology and definitions

- KT0102 Principles and concepts of prototype manufacturing

- KT0103 Methods of prototype modelling

- KT0104 Specifications and design concepts, including 2D and 3D technical drawings, renderings, sketches, samples, cutting and sewing plans, and written work instructions

- KT0105 Prototyping process, procedures and role players and techniques

- KT0106 Definition and purpose of dry-run / mock-up assembly

- KT0107 Standards set out by prototyping with reference to foams, fabrics and frames

- KT0108 Sequence of prototyping manufacturing

- KT0109 Accuracy, detail and faults

Internal Assessment Criteria and Weight (10%)

This Knowledge Topic will be assessed through the following criteria:

- IAC0101 Principles of manufacturing upholstered prototypes are discussed referring to correct definitions and using correct terminology.

- IAC0102 Requirements for and function of specifications, designs, drawings, cutting and sewing plans, and written work instructions are identified and elaborated.

- IAC0103 Standards applicable to the manufacturing of upholstery prototypes are discussed.

- IAC0104 Implications of product faults not identified through the prototype and pattern making process are analysed.

Application in the Furniture Upholstery Sector

Mastery of this topic prepares learners to effectively participate in and contribute to upholstery prototype development within the furniture manufacturing environment. By understanding and applying these principles, learners ensure that design intentions are realised with accuracy and that potential production issues are identified and resolved during the prototyping phase. This contributes to improved product quality, cost-effectiveness, and customer satisfaction in both bespoke and bulk manufacturing contexts.

### KT0101: Terminology and Definitions

🌟 Learning Outcome

By the end of this topic, learners should be able to define and correctly use key terms related to the prototype manufacturing of upholstered furniture, demonstrating an understanding of their application in real manufacturing settings.

Key Concepts and Definitions

Introduce and discuss the following core terms with contextual application:

| Term | Definition |
| --- | --- |
| Prototype | A first full-scale and functional model of a furniture item, developed to test design concepts, materials, methods and usability before full production. |
| Dry-run / Mock-up | A preliminary version of the prototype assembled without final finishing to test fit, scale, structure, and feasibility. |
| Specification Sheet | A document detailing materials, dimensions, finishes, hardware, and construction notes used for manufacturing a prototype. |
| Technical Drawing | A 2D or 3D visual representation of a furniture product, showing dimensions, views, sections, and design features. |
| Modelling Techniques | Methods used to create scaled or full-size versions of a design, including CAD drawings, foam block modelling, or physical maquettes. |
| Tolerance | The allowable variation in dimension or design features that will not affect the function or appearance of the furniture piece. |
| Renderings | High-quality visual representations or computer-generated images used to show what the final product will look like. |
| Deviation | A departure from the agreed specifications, which may result in non-conformance or design failure. |
| Fit-for-purpose | A term used to describe a prototype that meets its intended design, functional, and quality objectives. |

Encourage learners to create a glossary of these and other emerging terms as they progress through the module.

💡 Example: Understanding Terms in Context

Scenario:
A design studio is developing a new upholstered bench for a corporate office client. The design team provides a technical drawing showing dimensions and views from multiple angles. A prototype is manufactured based on the cutting plan, using materials as per the specification sheet. During assembly, it becomes apparent that a slight deviation in foam thickness causes the bench to appear uneven. A mock-up is then created with adjusted foam to test the new solution before final production.

Discussion Points:

- Why was a mock-up essential in this case?

- What role did the specification sheet play in identifying the problem?

- How does understanding the correct terminology help in resolving such issues?

📦 Case Study: Precision in Terminology

Case:
Lebo is a junior upholsterer in a furniture manufacturing company. She is tasked with producing a prototype of a wingback chair. The design includes both 2D technical drawings and a rendering. However, she misunderstands the term "dry-run" and proceeds to upholster the prototype fully instead of assembling a structure-only version for client approval.

Outcome:
The company incurs unnecessary material costs and time delays. This mistake highlights the critical importance of terminology in prototype planning and cost control.

Facilitator Reflection Questions:

- How could the misinterpretation of "dry-run" have been avoided?

- What steps can a team take to ensure all members understand the technical language of prototyping?

- How does this impact the communication between designers and upholsterers?

🗣️ Critical Thinking Questions

Pose these questions to promote reflection and group discussion:

- Why is it important for all team members in a prototyping department to use the same definitions and terminology?

- How might incorrect terminology affect the quality, cost, and timing of a prototype project?

- Can you think of a real-life example where misunderstanding a term led to a mistake? How was it resolved?

- If you were training a new upholsterer, how would you ensure they understand key prototype terminology?

🛠️ Facilitation Tips

- Use physical samples and visual aids to demonstrate terms such as mock-up, rendering, and technical drawing.

- Pair learners and assign them terms to research and present with examples.

- Integrate a short quiz at the end of the session to test terminology comprehension.

- Encourage learners to maintain a “Prototype Terminology Workbook” that they update as they move through KM-13.

### KT0102: Principles and Concepts of Prototype Manufacturing

🌟 Learning Outcome

Learners will understand the core principles, goals, and practical concepts that underpin prototype manufacturing in the upholstered furniture industry. They will be able to explain the purpose and value of prototyping and apply these principles within a structured manufacturing environment.

🧠 Key Concepts

Introduce the following concepts with explanations and practical relevance:

| Concept | Explanation |
| --- | --- |
| Prototyping as a Problem-Solving Tool | Used to test, validate, and refine design intentions before full-scale production. |
| Iterative Development | The process of revising and refining designs through multiple versions of prototypes. |
| Design Translation | Converting a concept or rendering into a tangible product that reflects form, function, and usability. |
| Risk Mitigation | Identifying design, material, or construction problems early to avoid waste, customer dissatisfaction, or manufacturing issues. |
| Collaboration | Prototyping requires effective collaboration between designers, upholsterers, cutters, frame builders, and quality assurance specialists. |
| Cost-Benefit Efficiency | Prototypes are resource-intensive but cost-saving in the long term by preventing costly errors. |
| Functionality and Aesthetics Testing | Validating not only the look and feel of a piece but also its comfort, strength, and ergonomic value. |

💡 Example: Applying Prototype Principles

Scenario:
A boutique hotel orders custom-made upholstered headboards for all 15 rooms. The design team proposes an intricate tufted design with a unique angled shape. Before mass-producing the units, a prototype is created to test the tufting, scale, and foam density. The team realises that the angled shape creates pressure points and that the specified foam is too soft. Adjustments are made, and the final version is approved.

Discussion Points:

- How did prototyping prevent a costly mistake?

- Which departments needed to collaborate during this process?

- How was customer satisfaction ensured?

📦 Case Study: Prototype Failure Prevents Production Loss

Case:
Simphiwe, a frame-maker at a furniture workshop, receives instructions to build the internal structure for a new lounge chair. The prototype, when assembled, is unstable and fails under pressure during testing. Upon review, it is found that the stress distribution was poorly calculated and reinforcements were insufficient. The prototype process allows engineers and upholsterers to redesign the joint placement and foam layer build-up before final production.

Facilitator Reflection Questions:

- What principle of prototyping is demonstrated in this case?

- How does a failed prototype contribute to the success of the final product?

- What role did communication play in identifying the problem?

🗣️ Critical Thinking Questions

- Why is prototyping considered both a design and manufacturing activity?

- How do prototype processes support innovation and quality assurance?

- In what ways can poor prototyping affect the reputation of a furniture business?

- Should all custom furniture items go through a prototyping phase? Justify your view.

🛠️ Facilitation Tips

- Display examples of physical prototypes in various stages (e.g., raw frame, dry-run, fully upholstered).

- Invite a local upholsterer or product developer to discuss how prototypes are used in their workshop.

- Use a role-play activity to simulate a team prototyping meeting involving design, manufacturing, and quality assurance inputs.

- Assign small group work to define key concepts and present a fictional prototype plan.

### KT0103: Methods of Prototype Modelling

🌟 Learning Outcome

Learners will be able to identify, describe, and compare various methods of modelling used during the prototyping phase of upholstered furniture manufacturing. They will gain practical insight into how these methods contribute to evaluating design concepts, functionality, materials, and construction techniques.

🧠 Key Methods of Prototype Modelling

| Method | Description and Use |
| --- | --- |
| Physical Full-Scale Prototypes | Actual-size, functional prototypes constructed from production or mock-up materials to assess structure, comfort, and finish. |
| Scaled Models | Smaller versions (e.g., 1:10 scale) used to explore proportions and spatial impact before committing to full-scale. |
| Cardboard or Foam Core Mock-Ups | Low-cost and quick method for visualising shape and ergonomics during the early design phase. |
| Digital Modelling (CAD/3D Software) | Tools such as SketchUp or SolidWorks used to create precise 3D models with dimensional accuracy. |
| Hybrid Models | Combination of physical and digital methods—e.g., a digital frame model and physical upholstered prototype. |
| Material Study Models | Miniature components (e.g., layered foam samples) made to test material interactions. |
| Virtual Reality (VR) or AR Visualisation | Emerging methods allowing designers and clients to 'experience' the piece before any materials are cut. |

💡 Example: Choosing the Right Modelling Method

Scenario:
Thando’s team is designing a modern reception bench for a high-traffic corporate setting. Because of the importance of material performance and durability, they use CAD software to map out the internal structure, then create a foam core mock-up to evaluate shape and size. They follow this with a full-scale prototype upholstered in substitute materials to test assembly and comfort.

Discussion Points:

- Why were multiple modelling methods used?

- What did each step allow the team to evaluate?

- How did these steps manage risk and reduce costs?

📦 Case Study: Digital vs. Physical Prototyping

Case:
Nomvula, an upholstery student, is tasked with producing a new ottoman design for a client. She relies only on digital rendering to convey her design, but during physical production, she finds the corners difficult to stitch cleanly, and the foam shape does not hold as intended. This illustrates the limitations of using only digital methods without verifying physical feasibility through modelling.

Facilitator Reflection Questions:

- What could Nomvula have done differently to avoid the issue?

- How do digital models complement—but not replace—physical models?

- What manufacturing challenges are hard to predict digitally?

🗣️ Critical Thinking Questions

- How do you decide which prototype modelling method is most appropriate for a specific furniture item?

- What are the risks of skipping the physical modelling phase in upholstery production?

- How does prototype modelling improve communication between designers, upholsterers, and clients?

- What challenges do modern software tools address in the design process—and where do they fall short?

🛠️ Facilitation Tips

- Demonstrate examples of 3D modelling using free software (e.g., SketchUp) and compare this with a simple physical mock-up made from foam or board.

- Invite learners to create a miniature prototype using scrap materials or cardboard to model a concept item.

- Host a debate on digital versus physical prototyping.

- Assign a research task to explore the use of hybrid modelling in real-world furniture design studios.

### KT0104: Specifications and Design Concepts, including 2D and 3D technical drawings, renderings, sketches, samples, cutting and sewing plans, and written work instructions

🌟 Learning Outcome

Learners will understand how to interpret and utilise various design documentation and specifications that guide the prototype manufacturing process. They will be able to explain the function and interdependence of visual, technical, and written design elements used in upholstery prototyping.

🧠 Key Concepts and Documentation Types

| Item | Purpose and Use |
| --- | --- |
| 2D Technical Drawings | Provide accurate orthographic views (top, front, side) with dimensions and construction details. Used for measuring, planning, and frame building. |
| 3D Renderings | Visual representations created using CAD software to show realistic images of the final product from various angles. Useful for client approval and marketing. |
| Freehand Sketches | Used during early conceptualisation to visualise form, ideas, and modifications before digital refinement. |
| Physical Samples | Swatches of fabric, foam layers, trims, and components presented to check compatibility and quality. |
| Cutting Plans | Guide the layout and dimensions of material pieces for efficient material use and accurate cutting. |
| Sewing Plans | Provide instructions for assembly order, seam allowances, stitch types, and sewing techniques. |
| Written Work Instructions | Step-by-step procedures that include safety, equipment setup, quality checks, and assembly techniques. These formalise and standardise the prototype process. |

💡 Example: Coordinating Specifications in Practice

Scenario:
A designer provides a set of 2D drawings, a 3D rendering, and a cutting plan for a high-back chair with deep buttoning. During production, the upholsterer finds that the seam allowance is not specified in the sewing plan. Using the written work instructions, they confirm the standard seam allowance used in the workshop. Adjustments are made, and the prototype is completed accurately.

Discussion Points:

- Which documentation was missing a detail?

- How did the work instructions help prevent an error?

- How does each document contribute to accuracy?

📦 Case Study: Misalignment in Design Communication

Case:
In a small upholstery business, multiple artisans are working on a set of banquettes. The designer submitted detailed 3D renderings but failed to include cutting plans and sewing instructions. As a result, different upholsterers used slightly different foam thicknesses and seam allowances, leading to inconsistent outcomes. The project had to be redone.

Facilitator Reflection Questions:

- What caused the inconsistency?

- Which specification tools could have prevented the issue?

- How does this highlight the importance of interlinked documentation?

🗣️ Critical Thinking Questions

- Why is it important to have both visual and written specifications in upholstery prototype manufacturing?

- Which specification document do you think plays the most critical role during production, and why?

- What risks emerge when specifications are unclear, outdated, or incomplete?

- How can a prototype team ensure effective communication between design and production?

🛠️ Facilitation Tips

- Show examples of each specification type (real or sample): drawings, renderings, cutting plans, etc.

- Assign learners a product (e.g., an ottoman) and have them generate a basic cutting plan and work instructions.

- Create a matching activity where learners pair each specification with its correct purpose.

- Organise a peer review where learners critique each other's sample documentation for clarity and usability.

### KT0105: Prototyping Process, Procedures and Role Players and Techniques

🌟 Learning Outcome

Learners will understand the structured sequence of steps involved in the upholstery prototyping process. They will be able to identify the responsibilities of each role player, describe standard procedures followed in prototype development, and apply appropriate techniques for successful prototype creation.

🧠 Key Concepts

| Element | Description |
| --- | --- |
| Prototyping Process | A sequence of actions beginning with design intent and ending with prototype evaluation. It includes design review, material selection, frame construction, upholstery, testing, and feedback. |
| Standard Procedures | Defined steps such as briefing, sign-offs, quality checks, and documentation reviews. These ensure consistency, accuracy, and alignment with design expectations. |
| Role Players | Key stakeholders include designers, upholsterers, cutters, frame builders, sewing technicians, quality controllers, and clients. Each has a unique responsibility and contribution. |
| Techniques | Practical approaches include mock-up assembly, marking for fit and symmetry, test-stitching, applying tacking strips, foam profiling, and dry assembly. Techniques vary by material, design complexity, and intended use. |

🪜 Typical Prototyping Process Flow

- Design Briefing and Concept Review

- Material and Component Selection

- 2D/3D Drawing and Spec Review

- Frame Construction

- Dry Assembly / Mock-Up

- Marking, Cutting, and Sewing

- Trial Upholstery

- Evaluation and Adjustments

- Client Review and Sign-Off

💡 Example: Process in Action

Scenario:
A client commissions a bespoke recliner with built-in storage. The designer prepares technical drawings. The frame builder constructs the internal frame based on those drawings. A dry assembly is performed and handed to the upholstery team, who test different foam layering techniques. A quality controller flags that the fabric pulls at the seams under pressure. The designer and upholsterer jointly resolve the issue before client presentation.

Discussion Points:

- Which roles were involved?

- How did each step support prototype quality?

- What technique resolved the fabric tension issue?

📦 Case Study: Skipping the Process

Case:
A manufacturer receives an urgent order for custom settees. To save time, they skip the mock-up phase and go directly into upholstering. During client inspection, it is noted that the seat depth is too short and the arms are uncomfortably high. The entire batch must be scrapped, leading to financial loss.

Facilitator Reflection Questions:

- Which part of the process was neglected?

- What technique could have identified the issue earlier?

- What lessons can be drawn about procedure compliance?

🗣️ Critical Thinking Questions

- Why is it important to have a defined process when producing upholstery prototypes?

- What can go wrong if role players do not collaborate effectively?

- Which technique would you consider most critical in early-stage prototyping, and why?

- How would you handle a situation where a client requests changes after the prototype is nearly complete?

🛠️ Facilitation Tips

- Provide a flowchart of the prototyping process and ask learners to match steps with responsibilities.

- Conduct a group role-play where learners are assigned different roles (designer, upholsterer, client, etc.) to simulate a prototype briefing and review session.

- Display examples of dry-assembled frames and discuss the techniques used in their assessment.

- Host a peer discussion on which techniques learners find most challenging or interesting and why.

### KT0106: Definition and Purpose of Dry-Run / Mock-Up Assembly

🌟 Learning Outcome

Learners will understand what a dry-run or mock-up assembly entails in the context of upholstery prototype manufacturing. They will be able to describe its purpose, identify when it should be used, and explain how it contributes to the refinement of design and production techniques.

🧠 Key Concepts

| Term | Definition |
| --- | --- |
| Dry-Run Assembly | A trial assembly of a furniture piece using temporary methods (e.g., clamps, staples, untrimmed foam) to assess alignment, proportions, stability, ergonomics, and overall design feasibility before final upholstery. |
| Mock-Up | A partial or full-scale model used for study, testing, and client review. May be simplified or use alternative materials. Often interchangeable with dry-run but may include aesthetic features for review. |
| Purpose | To identify design flaws, test joinery, confirm dimensions, evaluate comfort, and ensure fit of components before committing to final materials and production. |

🔍 When and Why to Use a Dry-Run / Mock-Up

- To test whether the internal structure supports the intended design.

- To assess the comfort, proportions, and ergonomics of a piece.

- To verify whether materials and construction techniques are suitable.

- To involve clients in early-stage evaluation before investing in final materials.

- To avoid costly rework and ensure manufacturing feasibility.

💡 Example: Using a Dry-Run to Solve Structural Issues

Scenario:
A design for an L-shaped modular sofa is produced with a metal frame insert. Before upholstery, a dry-run is conducted to test weight distribution and corner stability. It is discovered that the joint between sections flexes under pressure. Engineers reinforce the frame before covering. This avoids a major product failure.

Discussion Points:

- What would have happened if the dry-run was skipped?

- How did the process support quality control?

- How did the team avoid material wastage?

📦 Case Study: A Costly Oversight

Case:
Sipho is an apprentice upholsterer working on a bespoke ottoman with a hidden storage compartment. He skips the mock-up step and proceeds to final upholstery. Upon completion, the lid rubs against the sides and the storage space is not accessible. The entire upholstery must be removed, corrected, and reapplied.

Facilitator Reflection Questions:

- What problem could a dry-run have exposed early?

- How can mock-ups save time and money?

- How would you advise Sipho to approach similar projects in future?

🗣️ Critical Thinking Questions

- Why is it important to do a dry-run even when the design appears simple?

- How can client feedback during mock-up stages influence the final prototype?

- When might it be acceptable to skip a mock-up or dry-run phase—and what are the risks?

- How does mock-up assembly enhance communication between the design and production teams?

🛠️ Facilitation Tips

- Show learners a partially assembled mock-up with labelled parts to illustrate key learning points.

- Provide templates or checklists used during dry-run assessments (e.g., structural checklist, proportion evaluation form).

- Conduct a class activity where learners critique a sample prototype layout based on a dry-run inspection.

- Invite a senior upholsterer to discuss real-world cases where dry-runs prevented costly errors.

### KT0107: Standards Set Out by Prototyping with Reference to Foams, Fabrics and Frames

🌟 Learning Outcome

Learners will be able to identify and apply quality and manufacturing standards relevant to the selection and use of foams, fabrics, and frame materials in upholstery prototype development. They will understand the importance of these standards in maintaining product performance, aesthetics, and safety.

🧠 Key Standards and Quality Benchmarks

| Component | Standard Considerations |
| --- | --- |
| Foams | Density (kg/m³), compression resistance, recovery rate, fire retardancy, suitability for application (e.g., seat vs. backrest). Must comply with health, comfort, and safety standards. |
| Fabrics | Martindale rub test (abrasion resistance), pilling resistance, lightfastness, seam slippage, flame resistance, colourfastness, and stretch tolerance. Chosen based on use case, durability, and design fit. |
| Frames | Timber moisture content (8–12%), joint strength, alignment accuracy, species suitability (e.g., Saligna vs. Pine), use of correct adhesives, reinforcement, and fasteners. Must comply with structural integrity and safety standards. |

📜 Relevant South African and Industry Standards

- SABS 0177 – Fire testing of upholstered composites

- ISO 3385 – Assessment of foam fatigue

- SABS 1383 – Upholstered furniture standards for general domestic use

- Manufacturer specifications – From foam and textile suppliers regarding application-specific use

- Internal workplace quality benchmarks – May vary depending on client profile and production values

💡 Example: Applying Standards in Prototyping

Scenario:
A custom ottoman is prototyped using foam supplied at a discount. The foam fails within two weeks during initial user testing. Upon review, it is found that the foam was not intended for high-compression seating applications and had a density below 22 kg/m³, which is below recommended seating standards.

Discussion Points:

- What standards were not followed?

- How can suppliers support prototype quality assurance?

- What could the team have done differently?

📦 Case Study: Client Complaint Traced to Fabric Choice

Case:
A retail client returns a prototype lounge chair because the fabric has faded within two months. Testing reveals the fabric's lightfastness rating was too low for placement in a sunlit room. The team failed to consider SABS specifications related to UV exposure for indoor upholstery.

Facilitator Reflection Questions:

- What quality standard was neglected?

- How could the issue have been predicted during prototyping?

- How can prototyping phases incorporate such testing?

🗣️ Critical Thinking Questions

- Why is it essential to test and confirm component standards during the prototype phase?

- How do the quality standards for prototyping differ (if at all) from those for full production?

- What role does documentation from suppliers (e.g., spec sheets) play in prototype evaluation?

- Can a prototype be considered successful if it does not comply with standard tolerances?

🛠️ Facilitation Tips

- Display labelled samples of foam with different densities and ask learners to guess appropriate uses.

- Use fabric swatches and a Martindale tester or video demonstration to explain abrasion resistance.

- Organise a mock "quality audit" where learners evaluate a sample prototype against a checklist of standards.

- Invite a materials supplier to present specifications and standards used in their product range.

### KT0108: Sequence of Prototyping Manufacturing

🌟 Learning Outcome

Learners will understand the correct sequence of steps involved in the prototyping manufacturing process for upholstered furniture. They will be able to apply this structured workflow to ensure accuracy, minimise error, and improve collaboration across all departments involved in prototype production.

🧠 Typical Sequence of Prototyping Manufacturing

| Step | Activity Description |
| --- | --- |
| 1. Design Brief and Concept Approval | Client needs are translated into design objectives; sketches, 2D and 3D drawings are created and signed off. |
| 2. Specification Finalisation | Final specs include dimensions, materials, frame type, foam density, fabric selection, and finishing details. |
| 3. Frame Construction | The internal structure is built according to drawings; joints and reinforcements are verified. |
| 4. Dry-Run / Mock-Up Assembly | A test assembly is conducted to verify fit, proportions, structural integrity, and aesthetics. |
| 5. Cutting and Sewing | Patterns are marked and cut; sewing of covers and fitting of linings and trims begins. |
| 6. Upholstery Application | Foam is applied to the frame, followed by fitting and stretching of the fabric using appropriate techniques. |
| 7. Finishing Touches | Legs, trims, skirts, buttons or nail-heads are added; the prototype is cleaned and inspected. |
| 8. Evaluation and Quality Checks | Functional and visual tests are performed, and client feedback is sought if applicable. |
| 9. Revision and Finalisation | Adjustments are made as necessary, and updated documentation is compiled. |

💡 Example: Following the Sequence for Efficiency

Scenario:
A team receives an urgent order for a tufted bench prototype. They rush into cutting foam and fabric before completing a mock-up assembly. Once assembled, the seam lines do not align with the button placement. The team must re-cut all fabric, wasting time and material. After revisiting the sequence, they redo the project correctly and submit it on time.

Discussion Points:

- Which step in the sequence was skipped?

- What impact did this have on resources?

- How did returning to the correct sequence improve outcomes?

📦 Case Study: Adherence to the Sequence Ensures Client Satisfaction

Case:
A high-end hotel commissions a custom headboard. The team follows the full prototyping sequence, including client preview after mock-up assembly. The client suggests subtle changes in height and button spacing. The feedback is incorporated, and the final product exceeds expectations.

Facilitator Reflection Questions:

- What advantages were gained by following the sequence?

- How did client involvement improve the final result?

- What lessons can be drawn about feedback and iteration?

🗣️ Critical Thinking Questions

- Why is it important to follow a defined manufacturing sequence during prototyping?

- In which situations might the sequence need to be adapted, and how can that be managed without compromising quality?

- What role does documentation play in maintaining consistency throughout the sequence?

- How can each stage in the sequence be used to prevent future errors?

🛠️ Facilitation Tips

- Use a visual flowchart on the wall to map each stage and add sticky notes for learner questions or comments.

- Assign small groups to present each step of the process with visual aids or practical demonstrations.

- Conduct a "spot the error" activity using a shuffled sequence of manufacturing steps and have learners re-order it.

- Invite a senior prototype technician to talk through a real-life production workflow and its importance.

### KT0109: Accuracy, Detail and Faults

🌟 Learning Outcome

Learners will understand the importance of precision and attention to detail during prototype manufacturing. They will be able to identify common types of faults, explain their causes, and describe the impact of inaccuracies on product quality, functionality, and client satisfaction.

🧠 Key Concepts

| Concept | Explanation |
| --- | --- |
| Accuracy | The degree to which the manufactured prototype conforms exactly to the design specifications, including dimensions, angles, seam positions, and component placements. |
| Detail | The inclusion of all specified design elements (e.g., button placement, seam finishes, piping, stitch types, leg positioning) exactly as documented. |
| Faults | Any deviation from the design brief or technical specifications that affects aesthetics, structure, function, or user comfort. These may result from human error, material defects, or process misalignment. |

🔍 Common Prototype Faults and Causes

| Fault Type | Examples | Possible Causes |
| --- | --- | --- |
| Dimensional Inaccuracy | Incorrect height, seat depth, or width | Misreading of technical drawings, cutting errors |
| Upholstery Symmetry Issues | Uneven pleats, off-centre buttons | Incorrect marking, poor alignment during upholstery |
| Structural Weakness | Wobbly frame, collapsing backrest | Faulty joints, incorrect timber, weak adhesives |
| Surface Imperfections | Wrinkles, puckering, sagging fabric | Incorrect tensioning, wrong foam density |
| Assembly Faults | Poor joinery, misaligned fittings | Rushed assembly, lack of dry-run testing |

💡 Example: Impact of Inaccuracy in Final Review

Scenario:
During the prototype review of a designer armchair, it is noticed that the buttoning is misaligned by 30 mm, making the entire front panel appear skew. The cutting was accurate, but the fabric was not properly marked before application. As a result, the entire upholstery must be redone.

Discussion Points:

- Where did the error occur in the process?

- What could have prevented the fault?

- How does such a small measurement have a large visual impact?

📦 Case Study: Failure to Detect Faults During Prototyping

Case:
A showroom prototype is produced without double-checking the seam alignment between the seat and the backrest. The misalignment is minor but noticeable. The design proceeds to production without revision. Customers begin returning the product due to the visible fault. The reputation of the company suffers, and production halts to correct the pattern.

Facilitator Reflection Questions:

- What quality control measures were missing?

- What was the long-term impact of overlooking a small fault?

- How can a prototype inspection checklist assist in preventing this?

🗣️ Critical Thinking Questions

- Why is it necessary to pay attention to both structural accuracy and small decorative details in a prototype?

- What is the relationship between attention to detail and brand perception in the upholstered furniture industry?

- What systems can be introduced to help detect and correct faults early in the prototyping process?

- How can a team learn from a fault without assigning blame, and improve future processes?

🛠️ Facilitation Tips

- Conduct a “fault identification” activity using images of finished prototypes with subtle defects; ask learners to spot and analyse them.

- Provide physical examples of accurate vs. inaccurate prototype details (e.g., seam finishes, leg positioning, foam shape).

- Develop a class inspection checklist to be used in simulated prototype reviews.

- Invite learners to reflect on a time when they made or observed an error in a workshop setting—what was the cause and lesson learned?

## 📝 Integrated Assessment for KM-13-KT01

Knowledge Topic: Principles of Manufacturing Upholstered Furniture Prototypes
NQF Level: 4 | Credits: Weight 10%
Internal Assessment Criteria:

- IAC0101: Principles of manufacturing upholstered prototypes are discussed referring to correct definitions and using correct terminology.

- IAC0102: Requirements for and function of specifications, designs, drawings, cutting and sewing plans, and written work instructions are identified and elaborated.

- IAC0103: Standards applicable to the manufacturing of upholstery prototypes are discussed.

- IAC0104: Implications of product faults not identified through the prototype and pattern making process are analysed.

📚 Section A: Terminology Matching (IAC0101)

Instructions: Match the terms in Column A with the correct definitions in Column B. Write the correct letter next to the corresponding number.

| Column A (Terms) | Column B (Definitions) |
| --- | --- |
| 1. Dry-run assembly | A. Step-by-step production instructions |
| 2. Cutting plan | B. Trial assembly used to verify dimensions |
| 3. Specification sheet | C. Document that describes dimensions, finishes, materials |
| 4. Work instructions | D. Diagram showing layout of material parts |
| 5. Rendering | E. Realistic visualisation of a furniture item |

[5 marks]

🧠 Section B: Short Answer Questions (IAC0102 and IAC0103)

Instructions: Answer the following questions in your own words.

- Explain the purpose of a sewing plan in prototype development.

- Why is it important to include both 2D drawings and written work instructions when manufacturing a prototype?

- List three standards that should be considered when selecting foam for an upholstered prototype.

- What is the function of the Martindale rub test in upholstery prototyping?

- Describe two consequences of using a fabric that does not meet industry standards in an upholstery prototype.

[10 marks]

📦 Section C: Case Study – Fault Identification (IAC0104)

Instructions: Read the case study below and answer the questions that follow.

Case Study:
A design team completes a prototype for a luxury lounge chair. The chair passes internal inspections and is sent to the client. Two weeks later, the client reports that the seat cushion compresses excessively, and the back fabric has started sagging. On investigation, it is revealed that a lower-grade foam was used for the seat, and no tension check was performed on the backrest fabric. The errors originated in the prototype phase but were overlooked.

Questions:

- Identify the two main faults in the prototype.

- What steps during the prototype process could have prevented these faults?

- Explain the impact of these faults on client satisfaction and production cost.

- How could proper use of specifications and standards have avoided the problems described?

[15 marks]

🧑‍🎨 Section D: Practical Activity – Document Review and Analysis (IAC0102, IAC0103, IAC0104)

Instructions:
You are given a set of prototype documentation including:

- A specification sheet

- A 2D technical drawing

- A cutting plan

- A foam sample and fabric swatch

- A set of work instructions

(Use simulated samples provided by the facilitator.)

Tasks:

- Review the documents and identify two elements that are unclear or incomplete.

- Suggest improvements for those two elements.

- Evaluate whether the foam and fabric samples meet the standards required for a high-use public lounge chair.

- Prepare a brief checklist for final inspection of a prototype to prevent faults prior to client review.

[20 marks]

✅ Total: 50 Marks

✅ Model Answers and Marking Memo

Section A: Terminology Matching (5 marks)

| Item | Correct Answer |
| --- | --- |
| 1 | B |
| 2 | D |
| 3 | C |
| 4 | A |
| 5 | E |

Allocation: 1 mark per correct match.

Section B: Short Answer Questions (10 marks)

- Purpose of a sewing plan
To guide the upholsterer in understanding seam allowances, stitch types, sewing sequences, and joinery required for cover assembly.
(2 marks)

- Importance of 2D drawings and instructions
2D drawings give technical dimensions and geometry; written instructions provide step-by-step procedures ensuring clarity and consistency.
(2 marks)

- Three foam standards

  - Minimum density (e.g., 30 kg/m³ for seating)

  - Compression resistance

  - Recovery rate and fatigue test (ISO 3385)
(1 mark each – 3 marks total)

- Martindale rub test
Measures the fabric’s resistance to abrasion; helps determine fabric durability for different settings.
(1 mark)

- Consequences of substandard fabric

  - Premature wear or fading

  - Compromised aesthetics and client dissatisfaction
(1 mark)

Section C: Case Study – Fault Identification (15 marks)

- Faults identified

  - Low-grade foam leading to seat compression

  - No tension check on fabric leading to sagging
(2 marks)

- Preventive steps

  - Ensure correct foam is selected as per spec sheet

  - Conduct a dry-run and inspect upholstery tension
(3 marks)

- Impact

  - Loss of client confidence

  - Rework costs, production delays
(4 marks – 2 per point)

- Avoidance through specs/standards

  - Spec sheets should stipulate foam density and supplier rating

  - Standards ensure material suitability is confirmed before use
(6 marks – 3 per point)

Section D: Practical Activity (20 marks)

- Unclear/incomplete elements
e.g., Missing seam allowances in cutting plan, no grain direction in fabric layout
(4 marks – 2 per issue)

- Suggestions for improvement
e.g., Add detailed notes on tolerances, include seam symbols
(4 marks – 2 per improvement)

- Foam and fabric evaluation
Foam fails compressive resistance for public use; fabric passes Martindale but lacks flame resistance certification
(6 marks – 3 per material)

- Prototype inspection checklist (sample)

  - Confirm dimensions against drawings

  - Check symmetry, joints, and foam adhesion

  - Evaluate fabric alignment and tension
(6 marks – 2 per relevant and realistic point)

🧮 Mark Allocation Summary

| Section | Marks |
| --- | --- |
| Section A | 5 |
| Section B | 10 |
| Section C | 15 |
| Section D | 20 |
| Total | 50 |

📊 Assessment Rubric

| Competency | Outstanding (5) | Competent (4) | Partially Competent (3) | Limited (2) | Not Yet Achieved (1) |
| --- | --- | --- | --- | --- | --- |
| Demonstrates understanding of prototype principles (IAC0101) | Clear, accurate definitions with relevant terminology throughout | Mostly accurate, minor omissions | Basic understanding, some terms unclear | Vague understanding | No correct definitions used |
| Interprets specification documents (IAC0102) | Fully interprets and applies all document types | Minor gaps, most documents understood | Understands documents but misses connections | Basic recognition only | Unable to interpret any |
| Applies relevant standards (IAC0103) | Cites correct standards with contextual application | General reference to standards | Minimal connection to practice | Basic recall only | No awareness of standards |
| Analyses faults and implications (IAC0104) | Comprehensive fault analysis with real insight | Competent analysis and suggestions | Identifies faults but lacks deeper analysis | Misses key implications | No identification of faults or impact |

### ‍🏫 Facilitator Assessment Briefing

Knowledge Module: KM-13
Knowledge Topic: KT0101 – KT0109
Assessment Title: Integrated Assessment for KM-13-KT01
Weighting: 10%
NQF Level: 4
Duration: 1.5–2 hours (suggested)
Total Marks: 50
Assessment Type: Integrated (includes case study, practical activity, short answers, terminology)
Assessment Method: Open-book, classroom-based written and practical task

🎯 Purpose of the Assessment

This integrated assessment evaluates the learner’s applied theoretical knowledge and understanding of the full range of concepts and principles related to upholstery prototype manufacturing. It draws from all topic elements (KT0101–KT0109) and aligns strictly to the internal assessment criteria (IAC0101 to IAC0104). The tasks promote critical thinking, accuracy, fault detection, and alignment to workplace practices.

📘 Assessment Objectives

The learner is expected to:

- Correctly define and apply upholstery prototype terminology.

- Identify and elaborate on the purpose and function of specification documents.

- Discuss relevant materials, design, and prototyping standards.

- Analyse the causes and consequences of faults in prototype production.

🧰 Resources Required

Facilitator must provide:

- Sample specification sheets

- Sample 2D and 3D drawings

- Fabric and foam swatches

- Work instruction templates or extracts

- Marking memo and rubric

- Printed learner response sheets or answer booklets

👩‍🏫 Facilitation Tips

- Introduce the assessment with a brief revision session.

- Clarify terms like "mock-up," "Martindale test," and “tolerances” beforehand.

- Allow learners to engage in peer discussion during the case study prep, if appropriate.

- Supervise practical review to ensure learners engage meaningfully with samples.

✔️ Assessment Conditions

- This is a classroom-based, supervised assessment.

- Learners may use dictionaries, course notes, and curriculum documents.

- Internet-enabled devices should not be allowed unless specified.

🔎 Assessment Principles Observed

- Fairness: Includes a variety of question types to suit learning styles.

- Transparency: Assessment criteria are explicitly shared with learners.

- Validity: Questions reflect actual prototype development procedures.

- Reliability: Marking guide ensures consistency.

- Authenticity: Tasks simulate real upholstery manufacturing contexts.

## KM-13-KT02: Prototype Component Construction – Frame Construction (20%)

Facilitator Guide Introduction

📘 Purpose of the Knowledge Topic

The purpose of this Knowledge Topic is to equip learners with the theoretical understanding and applied knowledge required for the accurate construction of furniture frames used in upholstered prototypes. Learners will explore the different types of frame components, materials, machines, joints, and techniques involved in producing frames for both fully upholstered and show wood furniture items, including those with powered or mechanical functions.

Learners will develop a detailed understanding of frame construction principles such as cutting lists, tolerances, material properties, joint durability, and quality standards. They will also consider relevant safety measures, personal protective equipment (PPE), and typical faults encountered during the frame-making process. This knowledge lays a critical foundation for producing structurally sound, high-quality furniture frames that align with design specifications, safety standards, and durability expectations.

🧠 Key Knowledge Areas

This topic includes the following elements:

- KT0201 Terminology, definitions and abbreviations of upholstered furniture frame components and parts

- KT0202 Raw materials used to make frames for simple, complex, fully upholstered and show wood furniture

- KT0203 Types and functions of machines used to make frames

- KT0204 Principles and concepts of joints and joint construction in upholstered furniture frames

- KT0205 Principles and concepts of upholstered furniture frame construction with reference to cutting lists and tolerances, routing sheets, material selection and fault finding, quality requirements and materials handling

- KT0206 Personal protective equipment and hazards and unsafe acts in the frame making area

- KT0207 Principles and concepts of making furniture frames for mechanical or powered movement attachments (reclining, lifting, heating or vibrating mechanisms)

🎯 Internal Assessment Criteria and Weighting (20%)

This topic will be assessed through the following criteria:

- IAC0201 Parts and components of furniture frames are defined using industry accepted terminology and abbreviations

- IAC0202 Raw materials used for manufacturing of upholstery frames are identified and described with reference to respective characteristics and properties

- IAC0203 Personal protective equipment and hazards and unsafe acts in the frame making area are analysed

- IAC0204 Materials and processes used to prepare, stain and coat solid wood components to achieve the required finish are listed and uses explained

- IAC0205 Joints commonly used in upholstered furniture frames are evaluated in terms of function and durability

- IAC0206 Construction of frames for mechanical or powered movement attachments are evaluated in terms of function and durability

- IAC0207 Faults that can affect the performance of frames are identified, analysed and corrective actions are suggested

🪑 Application in the Upholstered Furniture Sector

This Knowledge Topic supports the development of job-specific skills for learners working in frame construction workshops, prototype development units, and specialised production areas for upholstered furniture. Mastery of the content enables learners to engage in informed decision-making regarding material selection, design durability, safety precautions, and mechanical integrations—all of which are vital to high-performance prototype production in both bespoke and commercial settings.

### KT0201: Terminology, Definitions and Abbreviations of Upholstered Furniture Frame Components and Parts

🌟 Learning Outcome

Learners will be able to accurately define, use, and interpret industry-standard terms and abbreviations relating to the components and structural parts of upholstered furniture frames. They will also develop familiarity with technical language used in frame manufacturing documentation and workshop communication.

🧠 Key Concepts and Terms

| Term / Abbreviation | Definition |
| --- | --- |
| Rail | A horizontal frame member used in the base, seat, or back sections of the frame. |
| Stretcher | A support bar connecting frame legs to prevent racking and add strength. |
| Joint | The connecting area between two frame members, e.g., mortise and tenon. |
| Seat Box | The boxed area that forms the seating structure of the chair or sofa. |
| Back Post | Vertical frame component that supports the backrest. |
| Webbing Rail | A rail positioned to anchor webbing for seat suspension. |
| Top Rail / Bottom Rail | Uppermost or lowermost horizontal components of a frame section. |
| Arm Support | The sub-frame piece that provides structural support for upholstered arms. |
| Face Frame | The visible part of a frame, especially in show wood furniture. |
| MDF | Medium-Density Fibreboard – a type of engineered board. |
| PB | Particleboard – a composite panel made from wood chips and resin. |
| Ply / Plywood | A strong layered wood board used in frame panels or backs. |
| Hardwood | Dense wood from deciduous trees (e.g., oak, beech, saligna). |
| Softwood | Lightweight wood from coniferous trees (e.g., pine, spruce). |
| KD Frame | Knock-down frame, designed for flat-pack delivery and on-site assembly. |

Encourage learners to build a personal terminology reference sheet for use in later modules and workplace settings.

💡 Example: Workshop Communication Scenario

Scenario:
A frame builder receives a specification that reads: “Cut the front bottom rail (pine), back top rail (ply), and prepare arm supports for assembly. Allow for 3° back rake on the rear uprights.”

Discussion Points:

- What does each term refer to?

- Why is material type (pine vs. ply) specified?

- What does back rake indicate?

📦 Case Study: Misinterpretation of Frame Terminology

Case:
Nico, a new team member, misreads the cutting list and assumes that the “seat rail” refers to the same dimension as the “arm rest rail.” This results in structural instability once the seat is assembled and upholstered. The piece fails quality inspection due to sagging under load.

Facilitator Reflection Questions:

- Which misunderstanding occurred?

- How can common terminology prevent production errors?

- How would better labelling or training have helped?

🗣️ Critical Thinking Questions

- How does consistent use of terminology improve teamwork and safety in frame construction?

- What challenges might arise when industry terminology differs between workshops or manufacturers?

- How would you explain the difference between softwood, hardwood, and composite board to a new apprentice?

- Why is it important to understand abbreviations in cutting lists and routing sheets?

🛠️ Facilitation Tips

- Bring labelled frame parts (real or mock) into the classroom and allow learners to physically identify and name each.

- Assign pairs to create flashcards of terms and quiz each other.

- Review real cutting lists or assembly drawings and identify terms and abbreviations.

- Provide a visual reference chart of all frame components for classroom display.

### KT0202: Raw Materials Used to Make Frames for Simple, Complex, Fully Upholstered and Show Wood Furniture

🌟 Learning Outcome

Learners will be able to identify, describe, and compare various raw materials used in frame construction for different categories of upholstered furniture. They will understand the structural and aesthetic implications of each material choice and be able to justify selection based on function, design type, and durability.

🧠 Key Raw Material Categories

| Material Type | Description and Application |
| --- | --- |
| Hardwoods | Dense, durable woods from deciduous trees. Common types: oak, Saligna, beech, blackwood. Ideal for high-load areas, show wood finishes, and precision joinery. |
| Softwoods | Lightweight, fast-growing coniferous timber such as pine. Used for internal frame structures where high load is not applied; cost-effective. |
| Composite (Particle) Board | Made from wood chips and resin, economical and consistent in thickness. Limited load-bearing capacity. Common in flat-pack and lightweight internal frame parts. |
| Medium Density Fibreboard (MDF) | Uniform engineered board, smooth surface, suitable for routing. Prone to water damage; best used in covered internal panels. |
| Plywood | Layered sheets of veneer for structural strength. Suitable for panels, curved forms, and high-tension areas. Available in varying grades. |
| Veneered Boards | Thin hardwood layer applied to board substrates. Offers aesthetic appeal of hardwood at lower cost. Used in show wood designs. |
| Melamine-Coated Boards | Chipboard or MDF with a hardwearing plastic surface. Used for non-visible support structures and flat surfaces. |
| Hardboard | Thin, dense board used for backing, linings, or support panels in furniture base structures. |

🛋️ Application by Furniture Type

| Furniture Type | Material Usage |
| --- | --- |
| Simple Upholstered Furniture | Often constructed with pine and particleboard; cost-effective and functional. |
| Complex Upholstered Furniture | Combination of pine and plywood with hardwood in high-stress areas. |
| Fully Upholstered Furniture | Uses concealed timber (pine, MDF, composite) – structural integrity takes priority over aesthetics. |
| Show Wood Furniture | High-grade hardwoods with visible finishes; often veneered or stained for decorative quality. |

💡 Example: Material Specification by Design Intent

Scenario:
A client requests a high-end lounge chair with visible arms and legs in a dark stain. The internal frame can be made of pine, but the visible parts must use blackwood to match the client’s aesthetic. Veneered plywood is used in the back panel to reduce cost while maintaining finish quality.

Discussion Points:

- Why is blackwood preferred for visible elements?

- What risks exist if pine is used in high-load joints?

- Why choose plywood for the back panel?

📦 Case Study: Incorrect Material Selection

Case:
A junior frame maker constructs an ottoman base using MDF for the top support panel and particleboard for the rails. Over time, the MDF begins to crack and sag under pressure from repeated sitting. Investigation reveals that plywood or hardwood was required due to the load-bearing function.

Facilitator Reflection Questions:

- What fault occurred and why?

- What properties of MDF made it unsuitable?

- How can such issues be prevented during prototyping?

🗣️ Critical Thinking Questions

- How does the choice of frame material affect the durability and safety of a furniture item?

- What are the advantages and disadvantages of using engineered boards (e.g., MDF, particleboard)?

- Why is it important to consider both strength and aesthetic properties when selecting materials for show wood furniture?

- In which scenarios would a softwood frame be more appropriate than hardwood?

🛠️ Facilitation Tips

- Provide physical material samples (or images) for learners to handle and examine. Discuss their texture, weight, grain, and strength.

- Have learners group materials by function: structural, aesthetic, economical, moisture-sensitive.

- Use furniture examples (photos or real items) and ask learners to identify what materials were likely used where—and why.

- Organise a materials selection challenge: give learners a design brief and ask them to justify material selection for each component.

### KT0203: Types and Functions of Machines Used to Make Frames

🌟 Learning Outcome

Learners will be able to identify the main types of machinery used in frame construction for upholstered furniture and explain the specific function of each machine. They will understand how to select appropriate equipment based on material, joinery, and production needs, as well as the role of these machines in achieving accuracy and efficiency in prototyping.

🧠 Key Machine Types and Functions

| Machine | Primary Function |
| --- | --- |
| Panel Saw | Used to cut large sheet materials (e.g., MDF, particleboard) into components of specific dimensions with clean, straight edges. |
| Crosscut Saw / Radial Arm Saw | Accurate crosscutting of timber to length, ideal for rail and leg preparation. |
| Rip Saw | Cuts timber along the grain; used to reduce timber width. |
| Band Saw | Makes curved or irregular cuts in solid wood or panel boards. |
| Surface Planer | Flattens and smooths surfaces of timber; ensures consistent thickness on one side. |
| Thickness Planer | Reduces timber thickness uniformly; used after surface planing. |
| Spindle Moulder | Shapes and profiles timber edges; often used for decorative edges or groove joints. |
| Drill Press / Pedestal Drill | Drills accurate holes at consistent angles and depths for dowels or fixings. |
| Multi-Borer (Optional) | Drills multiple, parallel dowel holes at once, increasing speed and alignment accuracy. |
| Portable Powered Drill and Screwdriver | Used for assembling frames or inserting mechanical fasteners. |
| Profile Sander | Smooths contoured or irregular surfaces, particularly in show wood frames. |

🪚 Machine Application Across Frame Production Stages

| Stage | Typical Machines Used |
| --- | --- |
| Initial Cutting | Panel saw, crosscut saw, rip saw |
| Shaping and Sizing | Surface planer, thickness planer, band saw |
| Joinery and Drilling | Drill press, spindle moulder, multi-borer |
| Assembly | Portable drills, clamps |
| Finishing | Profile sander, hand tools |

💡 Example: Machine Selection by Component

Scenario:
A show wood armchair requires curved arm supports, precision dowel joinery, and routed grooves for decorative features. The frame maker selects a band saw for the curve, a multi-borer for the dowel holes, and a spindle moulder for the grooving. A surface planer is used to prep the hardwood before shaping.

Discussion Points:

- What would be the result of using a panel saw instead of a band saw?

- Why is machine choice linked to both function and material type?

- How do these machines contribute to prototype quality?

📦 Case Study: Inappropriate Machine Use

Case:
Ayanda, a new frame technician, attempts to cut small internal frame rails using a panel saw instead of a crosscut saw. The cuts are rough and uneven, and the rails do not align properly during assembly. This results in a structurally unstable prototype and delays in the production schedule.

Facilitator Reflection Questions:

- What was the error in Ayanda’s machine selection?

- How did this affect quality and time?

- What training or signage might prevent similar mistakes?

🗣️ Critical Thinking Questions

- Why is it important to understand the specific function of each woodworking machine used in frame construction?

- How can incorrect machine selection affect safety and final product quality?

- What role does accuracy play in selecting a drill press versus a portable drill?

- How do different machines contribute to the efficiency of the prototype process?

🛠️ Facilitation Tips

- Show learners video demonstrations or live demonstrations (if possible) of each machine in use.

- Create a classroom visual board: display images and safety facts for each listed machine.

- Allow learners to classify machines into categories (cutting, shaping, drilling, sanding) as a group exercise.

- Conduct a mock "machine fault scenario" and ask learners to identify what went wrong and how it could be corrected.

### KT0204: Principles and Concepts of Joints and Joint Construction in Upholstered Furniture Frames

🌟 Learning Outcome

Learners will understand the principles behind joint selection and construction methods in upholstered furniture frame manufacturing. They will be able to identify common joint types, explain their function, evaluate their durability, and justify their use based on furniture type and design intent.

🧠 Key Joint Types and Their Applications

| Joint Type | Function and Use |
| --- | --- |
| Mortise and Tenon Joint | Traditional, strong joint suitable for show wood arms and legs. Common in high-end chairs and sofas. |
| Dowel Joint | Cylindrical wooden pins are inserted into drilled holes; used in mass production for frame rails and internal structures. |
| Corner Block Reinforcement | Triangular or square blocks glued and screwed into frame corners to increase strength. Used in seat boxes and base corners. |
| Screwed Joint | Quick, mechanical fastener used with adhesives; suitable for non-visible parts and temporary fixtures. |
| Butt Joint | Simplest joint—end of one piece butted against another. Low strength unless reinforced. |
| Lap Joint / Half-lap | Provides a flat joint between overlapping members—often used in rectangular frame structures. |
| Mitre Joint | Angled joint used where aesthetic finishing is important, e.g., show wood edges. |
| Stapled Joint | Used in combination with glue for fast production of internal frames; not suitable for high-stress parts unless reinforced. |

🛋️ Joint Selection by Furniture Type

| Furniture Item | Preferred Joint Types |
| --- | --- |
| Show wood dining chair | Mortise and tenon, dowel, mitre |
| Fully upholstered armchair | Dowel, screwed, stapled with corner blocks |
| Ottoman or bench seating | Corner blocks, dowel, lap joints |
| Recliners and motion furniture | Reinforced dowels, mortise and tenon, screwed metal bracket supports |

💡 Example: Joint Selection Based on Load

Scenario:
A prototype for a three-seater sofa requires strong joints in the seat box due to anticipated weight load. The frame builder uses corner blocks with glue and screw reinforcement at all rail intersections, combined with dowel joints for the cross members.

Discussion Points:

- Why were multiple joint types used?

- What risks would a stapled-only construction present?

- How does joint durability affect long-term customer satisfaction?

📦 Case Study: Joint Failure in Show Wood Chair

Case:
A handcrafted show wood chair is returned after two weeks of use due to a loose front leg. On inspection, the joint is revealed to be a butt joint reinforced only with a screw, rather than a mortise and tenon as required. The customer is dissatisfied with the durability and finish.

Facilitator Reflection Questions:

- What joint should have been used and why?

- How can poor joint selection affect aesthetics and structural integrity?

- What might quality assurance checklists include to catch such mistakes?

🗣️ Critical Thinking Questions

- How do you decide which joint type to use in different parts of a furniture frame?

- Why might a stapled joint be acceptable in some cases but not others?

- What are the advantages of using traditional joints like mortise and tenon over modern alternatives?

- How can poor joint construction affect user safety?

🛠️ Facilitation Tips

- Provide physical or model samples of joints for learners to handle and inspect.

- Use illustrations to compare joint types, and have learners annotate their functions and stress points.

- Invite learners to construct a mini frame using different joints and test for strength (mock versions).

- Assign a task where learners must select and justify joints for each component of a frame in a given design brief.

### KT0205: Principles and Concepts of Upholstered Furniture Frame Construction with Reference to Cutting Lists and Tolerances, Routing Sheets, Material Selection and Fault Finding, Quality Requirements and Materials Handling

🌟 Learning Outcome

Learners will gain a comprehensive understanding of the planning and execution processes involved in upholstered furniture frame construction. They will be able to interpret cutting lists and routing sheets, apply material selection principles, monitor quality requirements, and identify common faults in frame production. They will also understand how effective materials handling supports workflow efficiency and safety.

🧠 Key Concepts and Tools

| Component | Function and Application |
| --- | --- |
| Cutting List | A detailed document listing every timber/component size required for a specific frame, including dimensions, quantities, timber types, and joinery instructions. Ensures accuracy and limits waste. |
| Tolerance | Acceptable variation in size or dimension. Important for consistency, accurate joints, and assembly fit. Common woodworking tolerances range between ±0.5 mm to ±2 mm depending on design. |
| Routing Sheet | A document outlining each processing step and the sequence of operations. Used to guide production from raw timber to final frame assembly. |
| Material Selection | Based on properties such as strength, stability, moisture content, and grain pattern. Different parts of the frame may require different types of wood or engineered boards. |
| Fault Finding | Identifying defects in raw material (e.g., knots, warping), machine errors (e.g., misaligned cuts), or workmanship faults (e.g., loose joints, poor adhesive application). |
| Quality Requirements | Standards for strength, alignment, appearance, surface finish, and dimensional accuracy. Frames should pass load, movement, and visual inspection tests before approval. |
| Materials Handling | Safe and efficient movement, storage, and organisation of timber and components. Prevents damage, improves workflow, and ensures safety in the frame shop. |

🪑 Applied Concepts: Workflow from Planning to Assembly

- Begin with design drawings and convert them into a cutting list.

- Prepare routing sheet to define production steps.

- Select appropriate materials based on structural role.

- Cut and machine components within tolerance limits.

- Assemble and inspect frame for faults or deviations.

- Handle and store completed units with proper stacking and protection procedures.

💡 Example: Applying Planning Documents in Frame Production

Scenario:
A high-volume furniture factory introduces a new dining chair prototype. The design team provides a full cutting list and routing sheet. During the dry assembly, a side rail does not fit correctly due to a misread tolerance. A quality inspection highlights the 3 mm deviation, and the piece is remade using the correct dimension.

Discussion Points:

- How did the cutting list help identify the mistake?

- Why is it important to standardise tolerances?

- What corrective measures ensured quality?

📦 Case Study: Poor Material Selection and Handling

Case:
Sipho selects pine rails from a stack stored in a damp corner of the workshop. After assembly, the frame warps during drying. Investigation reveals high moisture content and storage damage. The mistake costs the company time and materials.

Facilitator Reflection Questions:

- What should Sipho have checked before using the timber?

- How could better handling and storage have prevented the issue?

- What checks can be introduced into the routing sheet to verify material condition?

🗣️ Critical Thinking Questions

- Why are cutting lists and routing sheets essential tools in both prototype and bulk frame production?

- What challenges can arise when tolerances are not respected during cutting and assembly?

- How does quality control during material selection reduce the risk of frame failure?

- In what ways does good materials handling support occupational health and safety?

🛠️ Facilitation Tips

- Provide learners with a cutting list and routing sheet for a sample chair. Ask them to identify missing or unclear elements.

- Demonstrate tolerance with physical timber samples—show the difference between within and out-of-tolerance cuts.

- Create a fault-finding activity with real or simulated faulty frame parts and ask learners to suggest corrective actions.

- Conduct a “materials handling audit” in the workshop: learners identify risks and propose improvements.

### KT0206: Personal Protective Equipment and Hazards and Unsafe Acts in the Frame Making Area

🌟 Learning Outcome

Learners will be able to identify the appropriate personal protective equipment (PPE) required for frame manufacturing activities, recognise common workplace hazards, and describe unsafe acts that can lead to injury or product damage. They will understand the importance of adhering to health and safety procedures and regulations within a woodworking and frame-making environment.

🧠 Key Concepts: PPE and Safety in Frame Making

| Personal Protective Equipment (PPE) | Purpose / Application |
| --- | --- |
| Safety Glasses / Goggles | Protect eyes from dust, woodchips, and splinters during cutting, drilling, and sanding. |
| Hearing Protection (Earplugs/Earmuffs) | Prevent hearing loss due to prolonged exposure to machinery noise. |
| Dust Mask / Respirator | Filters airborne particles during sanding, machining MDF or treated timber. |
| Safety Footwear (Steel-toe Boots) | Prevent injury from dropped timber or tools. Must have slip-resistant soles. |
| Gloves (optional, depending on task) | Used when handling rough timber or loading stock. Not used near rotating machinery. |
| Workshop Apron or Overalls | Protect clothing and skin; reduce snagging risks. |
| High-Visibility Vest (if required) | Enhances visibility in shared spaces or during loading operations. |

⚠️ Common Hazards in Frame Making

| Hazard | Examples / Risks |
| --- | --- |
| Rotating Machinery | Risk of entanglement or injury if clothing, hair, or jewellery is not secured. |
| Wood Dust | Respiratory hazards, especially from MDF and hardwoods; potential fire risk. |
| Manual Handling | Risk of back injury or strain when lifting heavy timber. |
| Sharp Tools and Blades | Laceration risks from improperly handled saws, chisels, or drills. |
| Noise | Prolonged exposure leads to hearing damage. |
| Poor Housekeeping | Slips, trips, and falls from offcuts, cords, or spilled glue. |
| Inadequate Ventilation | Leads to build-up of dust or fumes in enclosed areas. |

🚫 Unsafe Acts to Avoid

- Operating machinery without appropriate PPE

- Disabling machine safety guards

- Wearing loose clothing or jewellery near rotating tools

- Using damaged or blunt tools

- Lifting incorrectly or lifting loads that are too heavy without help

- Ignoring spills, dust accumulation, or clutter in work areas

- Failing to report hazards or unsafe conditions

💡 Example: Risk Reduction through Correct PPE Use

Scenario:
Lebo is cutting frame components using a radial arm saw. She wears her dust mask and earmuffs, but forgets to wear safety glasses. A small chip flies off the wood and strikes her in the face, narrowly missing her eye. She now consistently wears her full PPE, and her team uses this incident in toolbox talks.

Discussion Points:

- What PPE was missing and what was the consequence?

- How could her team reinforce safe behaviour?

- What are appropriate responses to near-miss incidents?

📦 Case Study: Unsafe Acts Lead to Injury

Case:
During a busy morning, Siphiwe removes the guard on the spindle moulder to speed up profiling work. His sleeve catches on the rotating head, causing a serious hand injury. The supervisor identifies that Siphiwe was not following safe work procedures and had not attended the most recent safety refresher training.

Facilitator Reflection Questions:

- What unsafe act occurred?

- How could it have been prevented?

- What responsibilities do both the worker and employer share?

🗣️ Critical Thinking Questions

- Why is PPE only one part of a broader safety system in a woodworking environment?

- What attitudes and behaviours can lead to unsafe acts, even when safety rules are known?

- How would you respond if you noticed a co-worker consistently ignoring PPE requirements?

- In what ways can a clean, well-organised workspace reduce accident risk?

🛠️ Facilitation Tips

- Display a visual PPE checklist at the training venue or workshop entrance.

- Conduct a mock “hazard hunt” in the workshop or classroom using photos or simulated stations.

- Provide real or sample PPE for learners to inspect, identify, and match to relevant tasks.

- Facilitate a group discussion on recent near-misses (real or fictional) and how they were handled.

### KT0207: Principles and Concepts of Making Furniture Frames for Mechanical or Powered Movement Attachments (Reclining, Lifting, Heating or Vibrating Mechanisms)

🌟 Learning Outcome

Learners will understand the specialised design, structural, and functional requirements involved in constructing furniture frames that accommodate mechanical or powered movement mechanisms. They will explore various types of mechanisms, their integration points, reinforcement needs, and safety considerations when prototyping such products.

🧠 Key Concepts and Components

| Type of Movement Mechanism | Function and Application |
| --- | --- |
| Reclining Mechanism | Allows backrest and footrest movement; typically used in lounge chairs and motion sofas. |
| Lifting Mechanism | Assists the user in standing up or sitting down; often found in geriatric or disability-support chairs. |
| Heating System | Embedded in cushions for user comfort; requires fire-resistant construction and careful material routing. |
| Vibrating Mechanism | Produces controlled vibrations for massage or therapeutic purposes; embedded in seat or backrest panels. |

🪑 Structural Requirements for Mechanised Frames

| Construction Principle | Purpose |
| --- | --- |
| Reinforced Joints and Rails | Must support repetitive stress and dynamic loads from motors and user movement. |
| Precision Alignment | Essential to ensure that actuators and brackets engage and move smoothly. |
| Use of Hardwood or Engineered Supports | Mechanical frames often use saligna, beech, or laminated plywood to resist flexing. |
| Clearances and Access Panels | Frames must accommodate cable routing, motor units, and allow access for repair. |
| Load Testing and Tolerance Control | Final prototypes must be tested for structural integrity under powered movement. |

⚙️ Examples of Mechanism Integration Points

- Base Frame – must support a reclining chassis or lift system.

- Backrest – may be attached to a pivot mechanism that adjusts angle or supports vibration.

- Footrest Extension – requires secure mounting of metal arms or tracks that extend smoothly.

- Power Supply Routing – must be insulated and inaccessible to avoid fire or mechanical risk.

💡 Example: Prototyping a Reclining Chair Frame

Scenario:
A team is tasked with developing a recliner for a cinema lounge. The client requests powered footrests and adjustable lumbar support. The prototype uses steel brackets mounted into Saligna side rails, and the seat box is reinforced with additional dowelled rails. Channels are routed into the side panels to guide cabling from the control switch to the motor. The frame is tested for smooth recline and noise-free operation.

Discussion Points:

- What materials were selected and why?

- How was motor access accommodated?

- What quality checks should follow final assembly?

📦 Case Study: Design Fault in Powered Chair Frame

Case:
A power lift chair prototype fails after repeated testing—the seat frame begins to crack around the motor bracket. Upon investigation, the timber used was pine, which lacked sufficient strength to absorb the movement load. The design is revised to include a laminated hardwood reinforcement beam and proper steel mounting plates.

Facilitator Reflection Questions:

- What fault occurred and what caused it?

- How did the revision address structural performance?

- What learning can be applied in future designs?

🗣️ Critical Thinking Questions

- How do mechanical and powered features change the way a frame must be designed and assembled?

- What risks must be considered when integrating electronics or moving parts into upholstered furniture?

- How does the choice of frame material influence the long-term function of a powered mechanism?

- Why is prototype testing especially important for frames with powered movement?

🛠️ Facilitation Tips

- Use manufacturer catalogues or videos to show reclining and lifting mechanisms in operation.

- Provide learners with exploded views or diagrams of mechanised frames.

- Invite learners to draft reinforcement and cable routing plans for a powered chair prototype.

- Discuss SABS or CE compliance requirements for electrical safety in upholstered products.

## Integrated Assessment – KM-13-KT02

Knowledge Topic: Prototype Component Construction – Frame Construction
NQF Level: 4 | Weighting: 20% | Total Marks: 60
Assessment Type: Integrated (Written + Practical Analysis + Case-Based)

Section A: Terminology and Raw Materials (IAC0201 & IAC0202)

Total: 14 marks

Instructions: Answer the following questions in your own words.

- Define the following frame components using correct terminology and abbreviations:
a) Seat rail
b) Arm support
c) MDF
d) KD frame
(1 mark each – 4 marks)

- Complete the table below by identifying the properties and typical use of each raw material:

| Material | Key Property | Common Application |
| --- | --- | --- |
| Oak | _________________________________ | ___________________________________ |
| Pine | ___________________________________________ | _____________________________________________ |
| Particleboard | ___________________________________________ | _____________________________________________ |
| Melamine-coated board | ___________________________________________ | _____________________________________________ |
| Plywood | ___________________________________________ | _____________________________________________ |

(2 marks per row – 10 marks)

Section B: Health and Safety (IAC0203)

Total: 8 marks

Instructions: Identify and explain.

- List three types of PPE required in the frame-making area and describe the risk they are meant to mitigate. (1 mark per item + 1 per explanation – 6 marks)

- Describe two unsafe acts that may occur in a frame shop and their potential consequences. (1 mark per act + 1 per consequence – 2 marks)

Section C: Frame Construction Techniques and Finishing (IAC0204 & IAC0205)

Total: 14 marks

Instructions: Answer all questions.

- List four materials or coatings used in the preparation and finishing of show wood frame components. (1 mark each – 4 marks)

- Explain the process and purpose of staining wood in the context of frame manufacturing. (3 marks)

- Compare mortise and tenon joints with dowel joints in terms of:
a) Strength
b) Durability
c) Suitability for show wood furniture
(1 mark each – 3 marks)

- Explain why corner block reinforcements are commonly used in ottomans and fully upholstered seat boxes. (4 marks)

Section D: Powered Mechanisms and Functional Frames (IAC0206)

Total: 10 marks

Scenario-Based Question:
You are prototyping a recliner with a powered footrest and lift mechanism. Answer the following:

- Identify two structural modifications needed in the frame to support powered movement. (2 marks)

- Suggest suitable materials for load-bearing areas and explain why they are appropriate. (3 marks)

- Describe how cable routing should be considered in the frame design. (2 marks)

- List three quality checks that should be performed after installing powered components. (1 mark each – 3 marks)

Section E: Fault Analysis and Corrective Action (IAC0207)

Total: 14 marks

Instructions: Read the case study and answer the questions that follow.

Case Study:
A batch of ottomans is returned due to frame collapse during use. Inspection reveals that softwood rails were too narrow to support suspension tension. Further faults include visible knots in rails and assembly that is not square.

- List three faults found in this frame and explain their impact. (2 marks each – 6 marks)

- Suggest three corrective actions to improve the frame’s strength and durability. (2 marks each – 6 marks)

- What check during prototyping could have prevented this product failure? (2 marks)

✅ Model Answers and Marking Memo

Section A: Model Answers (14 marks)

a) Seat rail – Horizontal frame member supporting the seat base
b) Arm support – Structural timber supporting armrest position
c) MDF – Medium-Density Fibreboard
d) KD Frame – Knock-down Frame

| Material | Property | Application |
| --- | --- | --- |
| Oak | Strong, attractive grain | Show wood frames |
| Pine | Lightweight, affordable | Internal frame rails |
| Particleboard | Low cost, not water resistant | Back panels or non-load-bearing |
| Melamine-coated board | Durable, wipe-clean surface | Flat-pack items, drawer bases |
| Plywood | Layered, strong, flexible | Curved frames, structural panels |

Section B: Model Answers (8 marks)

- Safety glasses – protect from flying debris

- Ear protection – reduce risk of hearing loss

- Dust mask – prevents inhalation of wood dust

- Disabling safety guard – may lead to finger injury

- Lifting heavy timber alone – causes back strain

Section C: Model Answers (14 marks)

- Water-based stain, spirit-based stain, varnish, lacquer

Staining enhances grain visibility, gives uniform colour, and prepares wood for final sealing
3.
a) Mortise and tenon – stronger
b) More durable
c) Better suited for visible show wood
4.
Corner blocks increase frame strength at joints, especially where tension is high; stabilise the seat box.

Section D: Model Answers (10 marks)

- Reinforced rails, metal bracket supports

- Saligna or laminated plywood – high strength and low flex

- Cables should run through insulated channels, away from moving joints

- Test range of movement

- Check noise levels

- Inspect for exposed wires or unsecured parts

Section E: Model Answers (14 marks)

- Narrow softwood rails – inadequate for load

- Loose knots – weak points

- Assembly not square – misaligned support

- Increase rail thickness

- Use hardwood instead of softwood

- Inspect timber for defects before cutting

Prototype strength test and visual quality inspection before production.

📊 Assessment Rubric

| Competency | Outstanding (5) | Competent (4) | Partially Competent (3) | Limited (2) | Not Yet Achieved (1) |
| --- | --- | --- | --- | --- | --- |
| IAC0201 – Terminology | Accurate, complete use of terms and abbreviations | Mostly correct with minor errors | Basic understanding, a few gaps | Vague or incomplete | No understanding |
| IAC0202 – Raw Materials | Materials identified, described with precision and context | Good awareness of properties | General description only | Lacks detail | Incorrect or omitted |
| IAC0203 – PPE and Safety | Clearly identifies PPE and hazards, strong reasoning | Reasonable answers, slight generalisation | Some relevance, lacks clarity | Incomplete, vague | Not addressed |
| IAC0204 – Finishing Materials and Processes | Comprehensive list and clear explanation | Adequate list and purpose provided | Some elements missing or weak explanation | Minimal detail | Lacks understanding |
| IAC0205 – Joint Evaluation | Insightful comparison and application | Competent explanation with example | Basic contrast, lacks critical evaluation | Generalised | Not addressed |
| IAC0206 – Mechanical Frame Construction | Demonstrates strong applied understanding | Answers functional, lacks nuance | Some confusion, lacks depth | Lacks application | Not yet competent |
| IAC0207 – Fault Identification and Correction | Clearly identifies faults and proposes effective actions | Most faults identified, some practical solutions | Basic diagnosis, partial remedies | Vague suggestions | Incorrect or no answers |

### 🏫 Facilitator Assessment Briefing

Knowledge Module: KM-13
Knowledge Topic: KT0201 – KT0207
Assessment Title: Integrated Assessment for KM-13-KT02: Prototype Component Construction – Frame Construction
Weighting: 20%
NQF Level: 4
Total Marks: 60
Duration: 2 to 2.5 hours (suggested)
Assessment Type: Integrated Summative Assessment
Assessment Method: Classroom-based written assessment and case study

🎯 Purpose of the Assessment

This integrated assessment is designed to measure the learner’s ability to recall, apply, analyse, and evaluate core concepts related to frame construction within the upholstered furniture prototyping process. It focuses on terminology, materials, tools, health and safety, joint application, finishing processes, powered movement structures, and fault analysis. Learners must demonstrate applied competence and foundational understanding in alignment with the requirements of occupational performance.

📘 Assessment Objectives

The learner should be able to:

- Use correct terminology and abbreviations for frame components (IAC0201)

- Identify and describe raw materials used in frame construction, including hardwoods, softwoods, and engineered boards (IAC0202)

- Analyse PPE requirements, hazards, and unsafe acts in a frame-making environment (IAC0203)

- List and explain finishing processes for solid wood components (IAC0204)

- Evaluate joints for various furniture types in terms of function and durability (IAC0205)

- Evaluate the construction needs of frames designed for powered movement (IAC0206)

- Identify common frame faults, analyse causes, and recommend corrective actions (IAC0207)

Resources Required

Facilitator must prepare or provide:

- Learner answer booklets or sheets

- Examples of frame joint types and raw materials (real or illustrated)

- Case study handout

- Sample cutting lists, routing sheets, or technical documentation (if applicable)

- Marking memo and rubric

- Visual aids on PPE and powered mechanisms (optional)

✔️ Assessment Conditions

- This is a supervised, classroom-based assessment.

- Learners may use course notes, curriculum documents, and PPE reference charts.

- Electronic devices are not permitted unless explicitly approved for specific sections.

- Facilitators may clarify questions but not lead learners to correct responses.

🧪 Assessment Principles Observed

- Fairness: Clear alignment to taught outcomes and task instructions

- Transparency: Criteria and expectations explained in advance

- Validity: Tasks measure knowledge and application related to KT0201–KT0207

- Reliability: Consistent scoring supported by a detailed memo and rubric

- Authenticity: Real-world case study ensures relevant application of learning

- Compliance: Designed in accordance with QCTO internal assessment guidelines

## KM-13-KT03: Prototype Component Construction – Modern Seat Construction (20%)

Facilitator Guide Introduction

📘 Purpose of the Knowledge Topic

The purpose of this Knowledge Topic is to equip learners with the knowledge and skills required to construct modern upholstered seat components for prototypes using current materials and techniques. Learners will explore the construction of shaped seats using foam and spring systems, as well as the methods and tools used to prepare, cut, and assemble these materials safely and effectively.

This topic develops learners’ understanding of foam types and densities, appropriate layering techniques, and safe application of adhesives using both manual and spraying methods. The emphasis is on modern approaches to seating construction, such as manual foam pulling, sculpting, and contouring to achieve ergonomic and aesthetic standards in contemporary furniture designs.

🧠 Key Knowledge Areas

This topic includes the following content elements:

- KT0301 Principles of constructing shaped seats using modern springs and twine / cord tying and knotting techniques

- KT0302 Differentiate between densities of foam used to construct components for upholstered furniture seats, arms, backs

- KT0303 Principles of selecting and layering different densities of foam to construct components for complex upholstered furniture seats, arms, backs including wing back chairs

- KT0304 Safety and correct handling of hand and powered cutting tools to prepare and cut foam

- KT0305 Personal protective equipment and safe preparation and spray gun setting processes for applying adhesive to foam

- KT0306 Principles of applying adhesives to foam using manual and spraying techniques

- KT0307 Principles of manual foam pulling techniques to construct and shape layers of foam to form upholstered furniture components

🎯 Internal Assessment Criteria and Weighting (20%)

This Knowledge Topic will be assessed through the following internal assessment criteria:

- IAC0301 Parts and components of modern seats are defined using industry accepted terminology and abbreviations

- IAC0302 Raw materials used for manufacturing of modern seats are identified and described with reference to respective characteristics and properties

- IAC0303 Principles of modern seat construction for prototypes are described with reference to handling, cutting and manual foam pulling techniques

🪑 Application in the Upholstered Furniture Sector

Modern upholstered furniture often relies on precise construction techniques and advanced materials to achieve consistent comfort, durability, and design appeal. Mastering modern seat construction methods prepares learners to contribute effectively to prototype development, where adaptability, material knowledge, and safe tool use are critical. The competencies developed in this topic are essential for producing high-quality, ergonomically designed seating solutions that meet both functional and aesthetic requirements in contemporary upholstery settings.

### KT0301: Principles of Constructing Shaped Seats Using Modern Springs and Twine / Cord Tying and Knotting Techniques

🌟 Learning Outcome

Learners will be able to describe and demonstrate the construction of shaped upholstered seats using modern spring systems. They will understand the correct tying and knotting techniques used with twine or cord to secure springs, ensuring seat durability, tension, and consistent support in prototype furniture.

🧠 Key Concepts and Techniques

| Component / Concept | Function and Application |
| --- | --- |
| Shaped Seats | Curved or contoured seats that support the body ergonomically. Require consistent internal structure and tension. |
| Modern Springs | Typically sinuous (zig-zag), webbed, or pocket coil systems. Provide flexible support across the seat frame. |
| Twine / Cord | Used to tie and stabilise spring systems in fixed positions. Polyester, nylon, or jute are common materials. |
| Tying Techniques | Include clove hitch, diagonal, straight front, and lateral tie-offs. Maintain spring alignment under tension. |
| Knotting Principles | Tight, secure knots ensure the spring does not move out of alignment with repeated use. Knot placement is critical. |

🪑 Common Modern Spring Types in Prototyping

| Spring Type | Description and Use |
| --- | --- |
| Sinuous Springs | Zig-zag steel wires; easy to install in modern frames; commonly stapled into position and later secured with cord. |
| Webbing (Rubber or Synthetic) | Strips woven across the seat frame; flexible, lightweight, and staple-fixed. Used in minimalist or soft seating designs. |
| Coil / Pocket Springs | Individually wrapped metal coils; offer zoned support. Used in premium prototypes or hybrid traditional-modern seats. |

💡 Example: Constructing a Shaped Modern Seat

Scenario:
A prototype for a lounge chair requires a shaped seat that dips slightly in the centre for ergonomic effect. The upholsterer installs sinuous springs across the base, ties the springs to side rails using jute twine in a diagonal pattern to maintain consistent spacing, and secures each tie-off with a tight clove hitch. A layer of foam and batting is added for comfort and shape retention.

Discussion Points:

- Why were sinuous springs chosen?

- What tying method was used and why?

- How do spring placement and tension affect the final comfort of the chair?

📦 Case Study: Spring Misalignment in Prototype Seat

Case:
A prototype bench collapses unevenly underweight. Upon inspection, it is discovered that the springs were installed with inconsistent spacing and without proper tying. Some springs shifted under load, causing sagging in the centre. The issue was traced back to insufficient use of twine and poorly executed knots.

Facilitator Reflection Questions:

- What was the direct cause of the failure?

- How could this have been prevented?

- What quality assurance steps should have been taken?

🗣️ Critical Thinking Questions

- How do different spring types influence the feel and function of a modern upholstered seat?

- What are the consequences of incorrect or loose tying in a spring system?

- Why is consistency important when applying tying patterns and tension?

- How might shaped seating requirements change the selection or placement of springs?

🛠️ Facilitation Tips

- Demonstrate a tying technique (e.g., clove hitch or diagonal tie) using cord, a frame sample, and dowels to simulate springs.

- Show examples of different spring systems and allow learners to practice identifying and positioning them.

- Provide cut-away diagrams or photos showing spring layouts and twine placements in shaped seats.

- Have learners compare the feel of well-tensioned versus poorly tied seat frames (mock-ups).

### KT0302: Differentiate Between Densities of Foam Used to Construct Components for Upholstered Furniture Seats, Arms, Backs

🌟 Learning Outcome

Learners will be able to identify and differentiate between foam densities used in various parts of upholstered furniture. They will understand how foam density affects firmness, resilience, durability, and comfort, and be able to select the appropriate foam for seats, arms, and backs during prototype construction.

🧠 Key Concepts: Foam Density and Application

| Term | Definition / Application |
| --- | --- |
| Foam Density (kg/m³) | The mass of foam per cubic metre. Higher density usually indicates higher durability and quality. |
| High-Density Foam (30–45 kg/m³) | Used for seat bases and load-bearing areas; firm and long-lasting. |
| Medium-Density Foam (24–30 kg/m³) | Common in backs and armrests where comfort and flexibility are needed. |
| Low-Density Foam (18–23 kg/m³) | Used in layering, decorative features, or low-load areas; soft and easy to shape. |
| Compression Resistance (ILD/CLD) | Measures how much force is needed to compress foam; helps assess firmness. |
| Open Cell vs. Closed Cell Foam | Open-cell foams are softer and breathable. Closed-cell foams are firmer and more water-resistant. |

🛋️ Application by Furniture Component

| Component | Preferred Foam Density | Reason |
| --- | --- | --- |
| Seat Cushion | 30–40 kg/m³ (high density) | Supports user weight, resists sagging |
| Armrest | 24–30 kg/m³ (medium density) | Comfort with moderate structural function |
| Backrest | 20–28 kg/m³ (medium to low density) | Requires cushioning and softness |

💡 Example: Foam Selection in Wingback Chair Prototype

Scenario:
A prototype wingback chair is being developed for an upscale hotel lounge. The upholsterer selects high-density foam for the seat to support frequent use, medium-density foam for the arm pads, and low-density contoured foam for the upper wing areas to ensure comfort and form.

Discussion Points:

- Why was foam differentiated by area?

- What would happen if low-density foam were used in the seat?

- How does user experience influence foam choice?

📦 Case Study: Foam Failure in Commercial Seating

Case:
A batch of dining chairs developed for a restaurant prototype used medium-density foam (24 kg/m³) for the seat cushions to save costs. Within two weeks of testing, the seats began to sag and flatten. A review showed that the foam was not rated for daily, high-load use and needed to be replaced with 35 kg/m³ density foam.

Facilitator Reflection Questions:

- What property of foam was misunderstood?

- How should the foam have been tested or evaluated?

- How can foam selection be linked to client environment and expectations?

🗣️ Critical Thinking Questions

- What is the relationship between foam density and comfort over time?

- How might foam density influence the design and aesthetics of a furniture piece?

- How would you determine the appropriate density for a chair that is both functional and decorative?

- Why is foam choice particularly important during prototyping and not just final production?

🛠️ Facilitation Tips

- Provide physical foam samples of different densities for learners to squeeze, weigh, and compare.

- Have learners label areas of a prototype (seat, arm, back) with recommended foam types and justify their choice.

- Demonstrate a compression test to show how different foams respond underweight.

- Use real client briefs or scenarios for learners to plan foam selections.

### KT0303: Principles of Selecting and Layering Different Densities of Foam to Construct Components for Complex Upholstered Furniture Seats, Arms, Backs Including Wing Back Chairs

🌟 Learning Outcome

Learners will understand the principles and decision-making involved in selecting and layering different foam densities to construct ergonomic, aesthetically pleasing, and structurally sound components for complex upholstered furniture. They will gain insight into how foam layering enhances comfort, contouring, and performance in prototype development.

🧠 Key Concepts

| Concept | Explanation / Application |
| --- | --- |
| Foam Layering | Combining multiple foam densities to achieve a desired shape, comfort level, or functional requirement. |
| Base Layer (High Density) | Provides structural support, especially in seats and lower arms. |
| Transition Layer (Medium Density) | Improves cushioning and contours between firm base and soft top. |
| Comfort Layer (Low Density) | Used as top layer to create softness, shape, and tactile comfort. |
| Contouring | The sculpting and positioning of foam to follow ergonomic curves (e.g., lumbar area or headrest wings). |
| Adhesive Compatibility | Each foam layer must be securely bonded using compatible adhesives, without affecting flexibility or causing hard spots. |
| Prototype Evaluation | Layering decisions must be tested during prototyping to assess seat depth, back contour, and return resilience. |

🪑 Application by Furniture Type and Function

| Furniture Feature | Layering Strategy |
| --- | --- |
| Wingback Chair (Seat) | 35–40 kg/m³ base + 28 kg/m³ medium + 20 kg/m³ soft topper |
| Wingback Chair (Backrest) | 28 kg/m³ base with contoured 20 kg/m³ top for head and lumbar zones |
| Arm Padding (Decorative Arms) | Medium-density inner with low-density crown for visual rounding |

💡 Example: Foam Strategy for a Contemporary Wingback

Scenario:
A custom wingback is designed with sweeping curves and a high headrest. The prototype uses a high-density base layer to support weight in the seat, a medium-density contour layer for the backrest, and a soft topper foam layer to finish the upper wings and shoulders. The foam is hand-shaped and layered using spray adhesive.

Discussion Points:

- How does layering affect seat depth and comfort?

- What would happen if medium-density foam were used alone?

- How do layering decisions affect stitching and upholstery tension?

📦 Case Study: Poor Layering Leads to Premature Deformation

Case:
In a furniture prototype for a boutique waiting area, the arm pads and seat cushions were built using a single medium-density foam block. After two weeks of showroom use, flat spots and collapsing edges appeared. No high-density base was used, and no transition foam was applied. The company revised the prototype to include layered foam with a reinforced base and topper.

Facilitator Reflection Questions:

- What design flaws were revealed?

- What could have been evaluated during dry fitting or mock-up?

- How do foam layers work together to resist long-term deformation?

🗣️ Critical Thinking Questions

- Why is foam layering more suitable than using a single block in complex shapes?

- How do ergonomics influence the choice of foam layering in chairs like wingbacks or recliners?

- What factors must be considered when selecting adhesives between foam layers?

- How can you test whether the foam layering in a prototype will hold up to real-world use?

🛠️ Facilitation Tips

- Provide a foam layering exercise: learners combine 3 foam densities on a test board to simulate a seat or arm.

- Use a cross-section sample of a chair showing layered foam for demonstration.

- Present design briefs (e.g., for lounge chair, ottoman, recliner) and ask learners to sketch foam layering strategies.

- Encourage learners to evaluate recovered furniture or old prototypes to identify layering successes and failures.

### KT0304: Safety and Correct Handling of Hand and Powered Cutting Tools to Prepare and Cut Foam

🌟 Learning Outcome

Learners will be able to demonstrate safe and effective handling of both hand-operated and powered cutting tools used in foam preparation. They will understand the hazards associated with foam cutting, the safety precautions required, and how to maintain accuracy and control during the cutting process.

🧠 Key Concepts: Safety and Tool Handling

| Tool Type | Application | Safety Considerations |
| --- | --- | --- |
| Long Blade Utility Knife | Cutting soft foam or trimming edges | Always cut away from body; use sharp blades only |
| Foam Saw (Vertical Blade) | Precision vertical cuts in thick foam | Guarded blade; use push guides; secure foam on table |
| Hot Wire Cutter | For detailed shaping or curved cuts | Avoid contact with heated wire; adequate ventilation for fumes |
| Electric Carving Knife | Quick cuts in medium foam (prototype use) | Hold both handles; check cords and blade locking |
| Band Knife Machine | For high-volume precise cuts | Must be operated only by trained individuals; blade tracking and tension must be correct |
| Scissors / Shears | Small trimming work | Keep sharp and clean; avoid snags in delicate foam |

⚠️ Hazards in Foam Cutting

| Hazard | Examples |
| --- | --- |
| Lacerations | Using dull blades with force or not paying attention to cut path |
| Burns or Heat Injury | Touching hot wires or cutting foam without temperature regulation |
| Inhalation of Fumes | Cutting treated foams or overheating hot wire cutters |
| Electric Shock | Damaged power cords, using electric tools near liquids |
| Poor Visibility / Lighting | Misjudged cuts, tool slippage, incorrect angles |

🦺 Safety Best Practices

- Always wear PPE: safety gloves, dust mask, goggles if cutting rigid or coated foams

- Ensure cutting surfaces are clean and level

- Check all electrical equipment before and after use

- Use controlled, smooth motions – avoid forcing blades through foam

- Clamp or stabilise foam when using vertical blades

- Never leave powered cutters unattended while plugged in

- Keep fingers at a safe distance from blade path

💡 Example: Cutting Foam for a Sculpted Armrest

Scenario:
Learners are preparing a curved foam layer for a high-back armchair armrest. The instructor demonstrates the use of a vertical foam saw. The foam is stabilised on a non-slip surface, the operator wears cut-resistant gloves, and the blade is tested for proper alignment before beginning the cut.

Discussion Points:

- Why is blade condition important?

- What PPE is essential here?

- What factors determine whether a powered or manual tool should be used?

📦 Case Study: Minor Injury During Foam Cutting

Case:
During a prototype session, Thabo uses a long utility knife to trim a seat cushion. The blade is dull, and he applies excess pressure. The knife slips and causes a shallow hand injury. Investigation reveals no gloves were worn, and the table was cluttered, limiting space.

Facilitator Reflection Questions:

- What unsafe actions contributed to this incident?

- How could the environment have been prepared better?

- What lesson can be drawn for team safety briefings?

🗣️ Critical Thinking Questions

- When should you choose a manual foam knife instead of a powered cutter?

- What are the signs that a foam cutting blade needs replacement?

- How does workspace layout contribute to foam cutting safety?

- What are the long-term risks of inhaling foam dust or fumes, and how can they be prevented?

🛠️ Facilitation Tips

- Arrange a supervised tool-handling demonstration with safe foam materials.

- Let learners handle unplugged powered tools to identify features and parts.

- Create a safety checklist learners can use before operating any foam cutting tool.

- Role-play incident response: what to do if a tool malfunctions or a minor injury occurs.

### KT0305: Personal Protective Equipment and Safe Preparation and Spray Gun Setting Processes for Applying Adhesive to Foam

🌟 Learning Outcome

Learners will be able to identify and correctly use personal protective equipment (PPE) when applying adhesives to foam. They will also understand the steps involved in safely preparing the workspace, handling adhesives, and setting up spray equipment to ensure safe and effective application.

🧠 Key Concepts: PPE and Spray Application Safety

| Aspect | Details |
| --- | --- |
| Personal Protective Equipment | Includes gloves (nitrile or chemical resistant), safety goggles, respirator mask (organic vapour rated), and protective overalls. |
| Adhesive Types | Most common is contact adhesive (solvent- or water-based). May be flammable and require ventilation. |
| Spray Gun Preparation | Involves selecting correct nozzle size, adjusting air pressure, checking hose connections, and testing spray pattern. |
| Work Area Preparation | Area must be well-ventilated, free of ignition sources, and lined with protective sheets or boards to avoid overspray. |
| Foam and Surface Readiness | Foam should be clean, dry, and dust-free; adhesive applied evenly to both surfaces for proper bond. |
| Safe Handling and Storage | Adhesive containers must be sealed when not in use, labelled clearly, and stored upright away from heat. |
| Spray Direction and Distance | Maintain correct angle and 150–200 mm distance for uniform coverage. Overspray and over-saturation must be avoided. |

🛡️ Key PPE for Adhesive Spraying

| PPE Item | Purpose |
| --- | --- |
| Respirator Mask | Protects lungs from inhaling fumes or fine particles |
| Safety Goggles | Prevents eye irritation or chemical splash injury |
| Gloves (Nitrile) | Protects hands from adhesive contact and skin irritation |
| Apron / Overalls | Shields body and clothing from overspray |
| Ear Protection | May be required in high-noise spray booths or factory settings |

💡 Example: Setting Up to Spray Adhesive in a Prototyping Workshop

Scenario:
A learner is preparing to apply contact adhesive to the backrest foam of a prototype. The instructor reviews the setup: a clean and well-ventilated spray booth, full PPE worn, adhesive stirred and transferred to a labelled container, spray gun tested on scrap foam, and correct overlap maintained during spraying.

Discussion Points:

- Why is spray pattern testing important before actual application?

- What PPE might be omitted if safety procedures are not reinforced?

- What happens if adhesive is applied too heavily or unevenly?

📦 Case Study: Spray Booth Safety Violation

Case:
In a training facility, a learner uses a spray gun without wearing a respirator. After ten minutes, he feels dizzy and nauseous. The facilitator investigates and finds that although the booth has an extractor fan, the adhesive was solvent-based, and fumes built up quickly due to poor airflow and improper PPE use.

Facilitator Reflection Questions:

- What risk was ignored and how could it have been avoided?

- What are the responsibilities of the facilitator in this case?

- How can safety be reinforced consistently in spray areas?

🗣️ Critical Thinking Questions

- Why is adhesive application one of the most hazardous tasks in upholstered furniture prototyping?

- What signs indicate a spray gun is not set correctly?

- How can PPE compliance be monitored effectively during adhesive application?

- What environmental risks are associated with adhesive overspray, and how can they be minimised?

🛠️ Facilitation Tips

- Demonstrate the correct donning and doffing of PPE before any adhesive handling activity.

- Allow learners to perform spray gun setting (without adhesive) using clean compressed air to understand pressure and pattern.

- Provide a laminated PPE and equipment checklist for use in workshop sessions.

- Conduct a short quiz or group task identifying hazards in illustrated spray booth setups.

### KT0306: Principles of Applying Adhesives to Foam Using Manual and Spraying Techniques

🌟 Learning Outcome

Learners will understand how to correctly apply adhesives to foam using both manual (brush or roller) and spraying techniques. They will learn the importance of even application, surface preparation, safety, and environmental considerations in achieving strong and durable bonds between foam and other materials during prototype construction.

🧠 Key Concepts: Adhesive Application Methods

| Method | Description | Best Used For |
| --- | --- | --- |
| Manual Application | Adhesive is applied using a brush or roller | Small surface areas, touch-ups, or where precise control is needed |
| Spray Application | Adhesive is atomised and sprayed using a spray gun | Large surfaces, high-volume tasks, complex shapes requiring even coverage |

🧩 Adhesive Application Principles

| Principle | Explanation |
| --- | --- |
| Even Coverage | Adhesive must be applied in a consistent layer—neither too thick nor too light—to ensure full bonding and prevent soak-through or poor adhesion. |
| Double-Sided Application | Adhesive is usually applied to both bonding surfaces (e.g., foam and substrate) and allowed to become tacky before pressing. |
| Drying / Tack Time | The adhesive must be left to dry slightly (until tacky) before joining components to avoid slippage or glue failure. |
| Bonding Pressure | Light uniform pressure should be applied across the surface to activate the adhesive bond. Clamping is generally not required for foam. |
| Compatibility | Foam type and adhesive must be compatible to avoid chemical degradation or weak bonding. |
| Temperature and Ventilation | Warm environments help adhesives cure evenly; poor ventilation can affect drying time and introduce safety risks. |

🛋️ Adhesive Use in Furniture Prototype Scenarios

| Task | Recommended Method |
| --- | --- |
| Small curved foam piece on armrest | Manual (brush/roller) |
| Full foam layer on seat platform | Spray application |
| Repositioning misaligned foam | Manual (limited) with contact touch-up |
| Layering foam in wingback chair | Combination of manual for shaping and spray for flat surfaces |

💡 Example: Foam Adhesion on Contoured Backrest

Scenario:
A learner is instructed to build up three layers of foam for a curved backrest. They use spray adhesive to cover each foam layer evenly, allow 30 seconds of drying time for tack, and align carefully before pressing. On inspection, no lifting or slippage is noted.

Discussion Points:

- Why was spray adhesive chosen over a brush?

- What signs indicate that the adhesive was applied correctly?

- What happens if adhesive is not allowed to tack before bonding?

📦 Case Study: Adhesive Failure Due to Incorrect Application

Case:
During prototype review, sections of the foam on a headrest begin to lift. It is discovered that the adhesive was applied too heavily with a brush, soaking into the foam and reducing tack. Additionally, only one surface was coated, and pressure was uneven during bonding.

Facilitator Reflection Questions:

- Which application principles were not followed?

- What could have been done differently during the process?

- How should bonding failures be addressed during prototyping?

🗣️ Critical Thinking Questions

- What are the advantages and disadvantages of manual versus spray adhesive application in prototyping?

- How does poor adhesive application affect product quality and longevity?

- Why is timing critical when using contact adhesives on foam?

- What indicators show that an adhesive has reached the correct tack point for bonding?

🛠️ Facilitation Tips

- Allow learners to practise both spray and manual adhesive techniques on scrap foam and board surfaces.

- Demonstrate what happens when adhesive is applied too thickly or joined too soon.

- Use a side-by-side comparison chart of brush vs spray applications.

- Encourage learners to inspect and test their own bonded samples under gentle pressure or pull tests.

Bottom of Form

### KT0307: Principles of Manual Foam Pulling Techniques to Construct and Shape Layers of Foam to Form Upholstered Furniture Components

🌟 Learning Outcome

Learners will be able to apply manual foam pulling techniques to construct and shape layered foam assemblies into aesthetically accurate and structurally functional upholstered furniture components. They will understand how tension, alignment, and tool handling influence the final profile and consistency of prototype seat, back, and arm shapes.

🧠 Key Concepts: Manual Foam Pulling

| Concept | Explanation / Application |
| --- | --- |
| Foam Pulling | The act of manually stretching or repositioning foam over frames or between layers to ensure contouring and full contact. |
| Shaping Through Tension | Applying tension during bonding or before upholstering to achieve curves, soften transitions, or eliminate air gaps. |
| Layered Shaping | Pulling outer foam layers over a shaped inner core to refine edges, depth, and dimensional consistency. |
| Surface Sculpting | Adjusting foam placement to visually and physically enhance a component’s form before cover application. |
| Tack Points and Adhesive Hold | Foam must be pulled and anchored at tension points to prevent shifting once adhesive sets. |
| Evenness and Symmetry | Pulling must result in symmetrical foam coverage—particularly important in seats, backs, and padded arms. |

✋ Manual Foam Pulling Tools and Aids

| Tool / Aid | Use |
| --- | --- |
| Hands and palm pressure | Main shaping method during positioning |
| Straight edge or profile guides | Used to check curves and consistency |
| Adhesive tack and release | Foam is gently pulled into final position while adhesive is still repositionable |
| Stapling or temporary pinning | Used during dry-fit to check alignment (prior to final bonding) |
| Trimming knife / scissors | Used to clean up after pulling and shaping |

🪑 Examples of Foam Pulling in Upholstery Prototypes

| Component | Purpose of Pulling Technique |
| --- | --- |
| Chair Arm Pads | Achieve rounded, uniform shape before fitting top fabric |
| Seat Cushions | Ensure no sagging or bulging in layered foam structure |
| Wingback Profiles | Control edge shape and depth of upper “wings” and backrest |

💡 Example: Foam Pulling on a Decorative Arm Panel

Scenario:
A learner is constructing a rolled arm for a prototype occasional chair. The layered foam is bonded to the frame, then manually pulled outward and downward to soften the shoulder and eliminate sharp transitions. The instructor guides the learner to check the front profile for symmetry before trimming excess.

Discussion Points:

- What would have happened if the foam was not pulled evenly?

- Why is manual foam pulling important before fabric is applied?

- How does this technique support the final upholstered form?

📦 Case Study: Poor Contour Due to Incomplete Pulling

Case:
A curved backrest panel is upholstered with visible high spots and uneven bulging. On review, it is found that foam layers were bonded directly without pulling the final top layer into full contact with the base curves. The fabric amplified the foam inconsistency, requiring full rework.

Facilitator Reflection Questions:

- What mistake occurred during foam preparation?

- How could the issue have been spotted earlier?

- What lessons apply to future prototype checks?

🗣️ Critical Thinking Questions

- What risks emerge if foam is not pulled and shaped manually during construction?

- Why is symmetry critical in foam pulling—especially on visible or mirrored components?

- How do you know when a foam layer has been correctly tensioned?

- What are the signs of successful foam shaping before fabric is applied?

🛠️ Facilitation Tips

- Provide each learner with a shaped plywood frame to practise pulling and shaping layered foam.

- Use mirrors or profile boards to check for symmetry and surface shape after pulling.

- Ask learners to take before-and-after photos of their foam work for peer critique.

- Facilitate a group demonstration using low-VOC spray adhesive followed by foam tensioning and profile correction.

## Integrated Assessment – KM-13-KT03

Knowledge Topic: Prototype Component Construction – Modern Seat Construction
NQF Level: 4
Weighting: 20%
Total Marks: 50
Assessment Type: Integrated Formative Assessment (Written and Scenario-Based)

Section A: Terminology and Components (IAC0301)

Total: 10 marks
Instructions: Match the terms below with the correct definitions. Write the correct letter next to each number.

| Term | Definition |
| --- | --- |
| 1. Seat base | A. A stretchable wire form used for modern support systems |
| 2. Foam pulling | B. The action of stretching and shaping foam by hand |
| 3. Sinuous spring | C. The lower structural part of an upholstered chair or sofa |
| 4. Pocket spring unit | D. Encased coils used in ergonomic seating applications |
| 5. Tack time | E. The wait period between adhesive application and bonding |

(2 marks per item – 10 marks)

Section B: Raw Materials and Foam Selection (IAC0302)

Total: 15 marks
Instructions: Answer the following short questions.

- List three different foam densities and describe where each would typically be used in upholstered furniture. (6 marks)

- Identify two factors that affect the performance of foam in seating prototypes. (4 marks)

- Explain the consequences of using low-density foam in a high-use seating application. (5 marks)

Section C: Construction Methods and Techniques (IAC0303)

Total: 25 marks

Case Study

A design team is developing a prototype for a modern ergonomic armchair. The prototype includes a shaped seat using high-density foam over sinuous springs. The backrest and arms are layered using multiple densities and shaped by hand. The seat foam must be cut to profile and bonded using spray adhesive. Manual foam pulling is required to achieve the correct contour before upholstery.

Questions:

- Identify three critical safety practices when cutting foam with powered tools. (3 marks)

- Describe the process of preparing foam and surfaces before applying adhesive. (4 marks)

- Explain why spray application may be preferred over manual adhesive application in prototype settings. (4 marks)

- What are the key principles of foam pulling when shaping a curved backrest? (4 marks)

- List three signs that foam has been shaped correctly prior to applying the final cover. (3 marks)

- Propose a step-by-step process for layering foam in the seat and backrest of the chair. (7 marks)

✅ Model Answers and Marking Memo

Section A Answers (10 marks)

1 – C: The lower structural part of an upholstered chair or sofa
2 – B: The action of stretching and shaping foam by hand
3 – A: A stretchable wire form used for modern support systems
4 – D: Encased coils used in ergonomic seating applications
5 – E: The wait period between adhesive application and bonding

(2 marks each)

Section B Answers (15 marks)

- High-density foam (35–40 kg/m³) – seat base

- Medium-density foam (24–30 kg/m³) – arm padding

- Low-density foam (18–23 kg/m³) – top layers and decorative contouring
(2 marks per item: 1 for density, 1 for correct usage – 6 marks)

- Foam density and compression rating

- Exposure to load and usage frequency
(2 marks each – 4 marks)

- Seats flatten quickly, poor support, reduces product life, affects user comfort and aesthetics
(5 marks – must address at least 3 relevant consequences)

Section C Answers (25 marks)

- Use sharp blades

- Wear cut-resistant gloves and mask

- Keep hands clear of cutting line
(1 mark each – 3 marks)

- Ensure foam and bonding surface are clean and dry

- Apply adhesive evenly to both surfaces

- Allow proper tack time before pressing
(4 marks)

- Fast, even coverage

- Reduces adhesive waste and prevents over-saturation
(2 marks per point – 4 marks)

- Apply tension evenly

- Shape follows ergonomic curve

- Symmetry between left and right sides

- Anchor edges securely
(1 mark each – 4 marks)

- Smooth surface

- No visible high/low spots

- Firm contact at all points with base
(1 mark each – 3 marks)

- Cut and prepare base layer

- Bond to frame or surface

- Layer transition foam for shape/form

- Apply top contour layer

- Pull and shape manually

- Inspect for evenness and contour

- Trim and final check before upholstery
(1 mark per step – 7 marks)

📊 Assessment Rubric

| Competency | Outstanding (5) | Competent (4) | Partially Competent (3) | Limited (2) | Not Yet Achieved (1) |
| --- | --- | --- | --- | --- | --- |
| IAC0301 – Terminology | Fully accurate and clear | Mostly accurate | Some terms unclear | Limited use | Incorrect or missing |
| IAC0302 – Raw Materials and Properties | Strong understanding, correctly applied | Generally accurate | Basic knowledge shown | Vague or incomplete | Not demonstrated |
| IAC0303 – Construction Principles | Clearly explained with accurate examples | Sound understanding | Some relevant knowledge | Minimal understanding | Lacks clarity and accuracy |

### ‍🏫 Facilitator Assessment Briefing

Knowledge Module: KM-13
Knowledge Topic: KT0301 – KT0307
Assessment Title: Integrated Assessment for KM-13-KT03: Prototype Component Construction – Modern Seat Construction
Weighting: 20%
NQF Level: 4
Total Marks: 50
Duration: 2 hours (recommended)
Assessment Type: Integrated Formative Assessment
Assessment Method: Structured written questions and a case-based scenario

🎯 Purpose of the Assessment

This assessment evaluates the learner’s ability to apply theoretical knowledge and practical reasoning to the construction of modern upholstered seat components. The assessment focuses on understanding seat components and materials, selecting and differentiating foam densities, and applying safe and effective construction methods, including cutting, adhesive application, and foam pulling. Learners must show integrated competence aligned to real-world upholstery prototype environments.

📘 Assessment Objectives

By completing this assessment, the learner should be able to:

- Define and use industry terminology and abbreviations related to modern seat construction (IAC0301)

- Identify and describe raw materials, particularly foams, used in constructing seat, back, and arm components (IAC0302)

- Describe modern seat construction principles with specific reference to foam handling, cutting, adhesive application, and manual shaping (IAC0303)

🧰 Resources Required

Facilitators must prepare or provide:

- Learner answer booklets or printed assessments

- Diagrams or images of modern seat components

- Foam samples of different densities (optional)

- Sample adhesive containers, spray gun visuals, or labels for identification tasks

- Marking memorandum and assessment rubric

✔️ Assessment Conditions

- This is a closed-book, supervised classroom assessment.

- No mobile phones or electronic devices permitted.

- Learners must complete the task individually.

- All safety-related content must be addressed as part of formative workplace readiness.

- Time allocations must be managed to allow for feedback after completion.

🧪 Assessment Principles Observed

- Fairness: Questions align clearly with learning activities and topic coverage

- Validity: Assessment targets appropriate knowledge and application outcomes per the curriculum

- Authenticity: Tasks simulate real upholstery decision-making contexts

- Reliability: Marking supported by clear memo and rubric

- Transparency: Learners should be informed of the assessment criteria beforehand

## KM-13-KT04: Prototype Component Construction – Traditional Seat Construction (20%)

Facilitator Guide Introduction

📘 Purpose of the Knowledge Topic

The purpose of this Knowledge Topic is to develop learners’ understanding of the traditional methods and materials used in constructing seat components for upholstered furniture prototypes. This includes the use of hand-tied coil springs, webbing, natural padding, and traditional edge-forming techniques. Learners will become familiar with the historical craftsmanship practices that underpin high-quality upholstery, with emphasis on accurate terminology, material properties, and tool application.

The learning prepares learners to execute traditional seat construction methods with attention to form, firmness, and durability—ensuring alignment with both aesthetic expectations and structural requirements during prototype development.

🧠 Key Knowledge Areas

This topic covers the following elements:

- KT0401 Terminology, definitions and abbreviations

- KT0402 Principles and concepts of traditional frame preparation, raw materials and consumables

- KT0403 Hand tools and power tools used for traditional upholstered furniture foundations

- KT0404 Principles and concepts of arranging and attaching webbing and coil springs, and tying springs to produce seats of different shapes and firmness

- KT0405 Principles of inserting and arranging padding around and over the springs to produce the desired seat shape

- KT0406 Principles of sprung edges and edge rolls

🎯 Internal Assessment Criteria and Weighting (20%)

This topic is assessed according to the following criteria:

- IAC0401 Parts and components for traditional seat construction are defined using industry accepted terminology and abbreviations

- IAC0402 Raw materials used for manufacturing of traditional seats are identified and described with reference to respective characteristics and properties

- IAC0403 Methods of positioning and interweaving webbing and the arranging, attaching and stuffing of coil springs to produce seats of different shapes and firmness are described and analysed for effectiveness

- IAC0404 Various types and functions of knots and knotting and tying off techniques used when working with coil sprung foundations are described and analysed for effectiveness

- IAC0405 Function of edge rolls and sprung edges are described and analysed for effectiveness

🪑 Application in the Upholstered Furniture Sector

Traditional seat construction remains highly relevant in bespoke, antique, restoration, and heritage upholstery contexts. It also provides a foundational skillset that enriches modern upholstery practices. By mastering these traditional techniques, learners can contribute to the production of high-end and historically accurate prototypes, offering clients and manufacturers depth of craftsmanship and technical versatility.

### KT0401: Terminology, Definitions and Abbreviations

🌟 Learning Outcome

Learners will be able to define and correctly use traditional upholstery seat construction terms and abbreviations. They will develop confidence in communicating about traditional seat-building processes, materials, and components using industry-standard language, contributing to clarity and professionalism in the workshop environment.

🧠 Key Terminology and Abbreviations

| Term / Abbreviation | Definition |
| --- | --- |
| Webbing | Strong woven jute or synthetic straps stretched across the seat frame to form a support base for coil springs. |
| Coil Spring | A metal spiral spring tied to webbing to provide structured support and resilience in traditional seats. |
| Edge Roll | A stitched or formed roll of coir fibre, hair, or foam placed at seat edges to define contour and comfort. |
| Stuffing / Fill | Natural or synthetic materials (e.g., hair, hessian, cotton felt) used to pad and shape seats. |
| Bridle Tie | A spring-tying method where each spring is connected to its neighbour, maintaining consistent tension and alignment. |
| Hog Ring | A metal fastener used to attach springs and webbing to frames or to link materials. |
| Hessian / Burlap | A coarse woven cloth used under stuffing to support or separate layers. |
| Sprung Edge | A construction where the front edge of the seat includes a spring unit for added flexibility and comfort. |
| Twine | Heavy-duty thread (jute or polyester) used for tying springs in coil-sprung seat construction. |
| Tack Rail | The wooden frame section to which webbing or hessian is tacked during foundation building. |

Encourage learners to keep a “Traditional Upholstery Glossary” as they move through this module.

💡 Example: Using Terminology in a Build Brief

Scenario:
An upholsterer is asked to repair the sprung edge of a Victorian chair prototype. They explain that the edge roll has lost definition and needs to be restuffed and retied with bridle knots. New jute webbing and a hessian barrier will be installed.

Discussion Points:

- What terminology indicates the traditional methods being used?

- Why is precision in naming components critical during team communication?

- What misunderstandings might arise without shared technical vocabulary?

📦 Case Study: Confusion Caused by Terminology Misuse

Case:
In a team project, one learner confuses edge roll with stuffing roll, leading to the incorrect installation of coir across the entire spring area. The result is a seat with poor definition and improper weight distribution, requiring removal and rework.

Facilitator Reflection Questions:

- What could have been done to clarify terminology beforehand?

- How does a shared language promote workshop efficiency and product consistency?

- What role can illustrated glossaries or toolboards play in learning?

🗣️ Critical Thinking Questions

- How does using correct terminology benefit the prototype review and inspection process?

- In what ways can traditional terms differ between manufacturers or regions, and how should this be handled?

- Why is it important to understand the materials described by each term (e.g., edge roll vs. fill)?

- How might incorrect terminology impact client understanding during custom upholstery consultation?

🛠️ Facilitation Tips

- Use real samples (edge roll, hessian, spring, etc.) and label each for classroom reference.

- Conduct a matching exercise with terms and visual representations.

- Provide an illustrated handout for learners to keep and annotate.

- Assign learners to explain a process (e.g., “building a sprung edge”) using as many correct terms as possible.

### KT0402: Principles and Concepts of Traditional Frame Preparation, Raw Materials and Consumables

🌟 Learning Outcome

Learners will understand how traditional frames are prepared to receive foundations and fillings in upholstered seat construction. They will be able to identify and evaluate the raw materials and consumables typically used in traditional upholstery prototypes, with a focus on structural integrity, durability, and authenticity of method.

🧠 Key Concepts: Traditional Frame Preparation

| Principle | Application in Traditional Upholstery |
| --- | --- |
| Inspection of Frame Joinery | Check integrity of joints—especially mortise and tenon—for stability before applying tensioning materials. |
| Surface Preparation | Sand or plane frame edges; remove debris, nails or tacks from re-used frames. |
| Tack Rail Identification | Locate frame areas where webbing, hessian, and edge rolls will be affixed. |
| Reinforcement if Required | Add blocks or supports where frame lacks sufficient rigidity or load-bearing structure. |
| Marking Out Fixing Points | Pencil marks or templates used to align webbing or spring centre points. |
| Protective Sealing | In some cases, a light sealant or liner is applied to wood if required by the client or environmental conditions. |

🧵 Raw Materials and Consumables in Traditional Upholstery

| Material / Consumable | Purpose |
| --- | --- |
| Jute Webbing | Creates a base platform for springs; staple or tacked to frame rails. |
| Coil Springs | Provides deep, responsive support; available in various heights and gauges. |
| Twine (Linen or Jute) | Used to tie springs securely in 8-way or diagonal patterns. |
| Hog Rings / Upholstery Nails | Fasten materials or springs to frames; used with pliers or tack hammer. |
| Edge Roll (Coir / Hair / Cabled) | Builds up the seat edge for comfort and form. |
| Scrim / Hessian / Burlap | Provides a barrier between springs and stuffing. |
| Stuffing (Horsehair / Cotton Felt / Coir) | Traditional filling used to create shape and comfort. |
| Tacks / Staples | Used to affix all materials and linings to the frame. |
| Glue (Animal / Contact Adhesive) | Used sparingly to support positioning of layers (in certain styles). |

💡 Example: Preparing a Traditional Frame for a Coil Spring Seat

Scenario:
A learner is preparing a reproduction Edwardian chair frame. The facilitator inspects the joints, confirms the use of solid beech rails, and assists in marking the webbing tack positions. Webbing is tensioned and fixed using traditional tacks. Hessian is laid over the webbing, and coil springs are hand-stitched to the foundation.

Discussion Points:

- Why is joinery inspection important before building the foundation?

- What determines where the webbing should be placed?

- How does choice of webbing or tacks impact durability?

📦 Case Study: Inadequate Frame Preparation

Case:
A prototype Victorian dining chair begins to creak and shift during prototype review. Upon disassembly, it is found that the joints were not reinforced after restoration, and the webbing was stapled into weakened wood. This caused the spring tension to compromise the frame integrity.

Facilitator Reflection Questions:

- What should have been done to prepare this frame for upholstery?

- How did modern staples contribute to the failure?

- What lesson can be applied to future restoration-based prototypes?

🗣️ Critical Thinking Questions

- How does traditional frame preparation differ from modern flat-pack or CAD-based frame construction?

- What risks are involved in not checking frame condition prior to applying webbing and springs?

- How does material selection for edge rolls or stuffing impact the overall quality of the seat?

- What criteria would you use to select between coir, horsehair, or cotton felt in a prototype?

🛠️ Facilitation Tips

- Display various raw materials and pass around frame offcuts to inspect tack rails and joint details.

- Provide an old or practice frame and guide learners through inspection, marking, and tensioning layout.

- Compare traditional vs. modern consumables (e.g., tacks vs. staples, twine vs. zip ties) and discuss implications.

- Encourage learners to document their material usage in a prototype log with reasons for each selection.

### KT0403: Hand Tools and Power Tools Used for Traditional Upholstered Furniture Foundations

🌟 Learning Outcome

Learners will be able to identify, describe, and safely use the range of hand tools and selected power tools required in traditional upholstered furniture foundation construction. This includes tools used for attaching webbing, tying springs, forming edge rolls, and handling natural materials such as hessian, twine, and coir.

🧠 Key Tools and Their Applications

| Tool Type | Name | Function / Use |
| --- | --- | --- |
| Hand Tools | Webbing Stretcher | Tensions webbing before tacking or stapling |
|  | Tack Hammer / Magnetic Hammer | Drives upholstery tacks or removes them with precision |
|  | Spring Needle | Long needle used to stitch springs to webbing or hessian |
|  | Regulator | Smooths and shapes stuffing under hessian; used to adjust contours |
|  | Twine / Cord Roller | Used to dispense twine evenly during tying off |
|  | Upholstery Scissors | Cuts hessian, webbing, twine, or stuffing materials |
|  | Curved Needles | Used for blind stitching or securing hessian edges |
|  | Mallet | Used to secure tacks without damaging tools or frame |
| Power Tools | Staple Gun (manual or pneumatic) | May be used in hybrid traditional construction for speed |
|  | Cordless Drill | Occasionally used to pre-drill tack holes in hardwood frames |
|  | Electric Shears / Foam Cutter | Where modern methods blend into traditional builds |

🔧 Tool Handling and Maintenance Principles

- Sharpness and cleanliness of scissors and blades ensure clean cuts and prevent fraying of natural fibres.

- Correct posture and grip reduce strain during tack application or webbing tensioning.

- Safe handling of spring needles is crucial, especially when stitching under tension.

- Storage and organisation of hand tools in a roll or tool tray helps avoid injury and increases efficiency.

- Compressed air staple guns should only be used by trained individuals in approved settings.

💡 Example: Using Tools to Build a Traditional Spring Foundation

Scenario:
A learner is building a coil spring seat on a traditional armchair. They use a webbing stretcher to secure three jute straps, attach each with a tack hammer, and use a spring needle to stitch the springs through hessian. The regulator is then used to adjust the stuffing once laid over the base.

Discussion Points:

- Why is the webbing stretcher critical to seat integrity?

- What could happen if a spring needle is not used correctly?

- How does tool selection support precision and traditional craftsmanship?

📦 Case Study: Incorrect Use of Modern Tool in Traditional Foundation

Case:
During a time-constrained build, a learner uses a heavy-duty staple gun to attach webbing directly to an antique frame. The staples pierce too deep, damaging the tack rail. Later, the webbing loosens, and the springs shift, compromising seat support.

Facilitator Reflection Questions:

- What tool misuse occurred, and what were the consequences?

- When, if ever, is a power tool appropriate in traditional seat foundation work?

- What training could have prevented this error?

🗣️ Critical Thinking Questions

- How does the choice of tools reflect the values of traditional upholstery?

- In what ways can modern tools assist rather than replace traditional craftsmanship?

- Why is it important to maintain traditional tool use even when time and cost pressures exist?

- How do safety and accuracy go hand in hand when using spring needles and regulators?

🛠️ Facilitation Tips

- Provide a hands-on demonstration of each tool in a guided session.

- Let learners practise tensioning webbing with a stretcher, driving tacks, and regulating a filled panel.

- Create a tool identification exercise using physical tools or a printed worksheet.

- Compare a traditional tack-set foundation with one done using modern shortcuts and discuss the visual and functional differences.

### KT0404: Principles and Concepts of Arranging and Attaching Webbing and Coil Springs, and Tying Springs to Produce Seats of Different Shapes and Firmness

🌟 Learning Outcome

Learners will be able to arrange and secure webbing and coil springs in a traditional seat structure, using appropriate tying patterns and knotting techniques to achieve various seat shapes and levels of firmness. They will understand how spring layout and tying tension influence the structural performance and comfort of upholstered furniture prototypes.

🧠 Key Concepts: Webbing and Coil Spring Foundation Building

| Component / Process | Purpose / Application |
| --- | --- |
| Webbing Arrangement | Forms the base structure for spring attachment. Typically applied in a criss-cross or parallel pattern for even load distribution. |
| Webbing Tensioning | Webbing must be stretched using a webbing stretcher for maximum firmness and support. |
| Spring Types and Sizes | Coil springs come in varying heights and gauges. Larger springs provide more bounce; smaller springs provide firmness. |
| Spring Placement | Springs are placed at even intervals based on seat size and desired firmness. A tighter pattern gives firmer support. |
| Sewing / Tying to Webbing | Springs are stitched to webbing using a spring needle and twine to prevent shifting. |
| Tying Patterns | Includes straight, diagonal, and 8-way hand-tying. More tying points improve firmness and consistency. |
| Tying Tension | Uniform tying tension ensures spring alignment and seat shape retention. |
| Knotting Techniques | Common knots: clove hitch, square knot, and slip knot. Each knot must hold tension under load. |

Seat Types and Firmness Configuration Examples

| Seat Type | Configuration Strategy |
| --- | --- |
| Firm Dining Chair Seat | Tightly woven webbing; short, firm coil springs; 8-way tying with high tension |
| Soft Lounge Chair Seat | Wider spring spacing; medium-height coils; diagonal tie; less tension |
| Deep Armchair / Club Chair | Tall coil springs; tight webbing; dense tying pattern for central support and perimeter shaping |

💡 Example: Building a Traditional Firm Seat

Scenario:
A learner is building a firm seat for a reproduction dining chair. They apply five strips of jute webbing in parallel, secure them with a webbing stretcher, and tack them to the frame. Five 4-inch coil springs are evenly spaced and sewn to the webbing. Springs are tied in an 8-way pattern using jute twine with clove hitches at each intersection.

Discussion Points:

- Why was the 8-way tie used in this case?

- How do tying patterns impact the movement and support of the seat?

- What would happen if tying tension was inconsistent?

📦 Case Study: Inconsistent Spring Tying Results in Uneven Seat

Case:
A prototype slipper chair is presented for showroom review. The front left corner of the seat is noticeably softer. On inspection, it is discovered that the spring in that area was loosely tied with only two-directional ties, and the webbing below had minimal tension. The seat was deemed unfit for presentation.

Facilitator Reflection Questions:

- What construction errors were made, and how did they affect the final product?

- How could tying consistency have been checked during the build?

- What impact do such details have on client perception of craftsmanship?

🗣️ Critical Thinking Questions

- Why is an 8-way tie considered the gold standard in traditional upholstery?

- How can webbing arrangement affect not only firmness but also spring alignment?

- How would you test the firmness and shape of a newly tied spring system before proceeding to the next upholstery stage?

- In what situations might fewer spring ties be acceptable, and why?

🛠️ Facilitation Tips

- Provide each learner with a frame section to practise webbing application and tensioning using traditional methods.

- Demonstrate tying patterns step-by-step using coloured twine to show directionality and symmetry.

- Conduct a group exercise comparing spring layouts and seat firmness through hands-on testing.

- Use diagrams showing various spring placements and tying methods for classroom display.

### KT0405: Principles of Inserting and Arranging Padding Around and Over the Springs to Produce the Desired Seat Shape

🌟 Learning Outcome

Learners will understand how to insert and arrange padding materials in traditional upholstery to shape and stabilise the seat profile. They will learn how padding interacts with spring foundations and fabric tensioning, and how correct layering contributes to comfort, symmetry, and durability in prototype construction.

🧠 Key Concepts in Padding Application

| Concept / Component | Explanation / Use |
| --- | --- |
| Stuffing Base (Under Layer) | Coarse hair or coir used to fill around and over springs, creating bulk and structure |
| Primary Stuffing Materials | Natural fibres such as horsehair, hog hair, cotton felt, or coir; shaped using regulators and stitched into place |
| Layering | Multiple layers are built up to achieve correct seat height and contour (e.g., base layer, shaping layer, finishing layer) |
| Stuffing Tie / Blind Stitch | Used to hold the shape and anchor stuffing over spring units without visible seams |
| Edge Roll Support | Padding is shaped to blend with edge roll, reinforcing comfort and silhouette |
| Surface Finish | Top layer is often fine cotton felt or wool to smooth contours before applying the cover fabric |

🪑 Padding Principles by Seat Type

| Seat Type | Padding Strategy |
| --- | --- |
| Traditional Dining Chair | Medium stuffing around springs, low crown, structured edge |
| Lounge Armchair | Deep stuffing for comfort, contoured back slope, high crown |
| Antique Reproduction | Hair stuffing with pronounced crown, strong edge definition using stuffing ties |

💡 Example: Shaping a High-Crowned Armchair Seat

Scenario:
A learner has completed the webbing and spring tie phase. They begin building up coir and horsehair padding in concentric layers, pulling and shaping each using a regulator. The centre of the seat is crowned slightly higher than the edges, and blind stitching is applied to compress and anchor the filling.

Discussion Points:

- What determines the shape and height of the crown?

- How do stuffing ties help hold form and prevent shifting?

- What are the risks of uneven or over-packed stuffing?

📦 Case Study: Poor Padding Causes Seat Collapse

Case:
In a prototype club chair, the seat padding was applied too thinly, with minimal shaping over the springs. Within days of user testing, the seat dips in the centre, and the edge lacks definition. Post-review reveals inconsistent material application and no stuffing ties.

Facilitator Reflection Questions:

- What padding principles were not followed?

- How could this fault have been prevented through testing and shaping?

- How important is symmetry in visual and functional outcomes?

🗣️ Critical Thinking Questions

- Why is it important to blend padding materials smoothly between the centre and edges of a seat?

- What are the advantages of using natural padding materials over synthetic in traditional upholstery?

- How does the shape of the padding influence fabric tension and fit during final upholstery?

- What inspection checks should be made before applying the top fabric?

🛠️ Facilitation Tips

- Provide sample stuffing materials for learners to handle and compare (coir, horsehair, felt, cotton wool).

- Demonstrate crown building using a pre-sprung base and allow learners to practise blind stitching and regulating.

- Create a visual step-by-step of padding build-up from spring to surface finish.

- Conduct a fault-identification activity using improperly padded seat samples.

### KT0406: Principles of Sprung Edges and Edge Rolls

🌟 Learning Outcome

Learners will understand the structural and comfort-related function of edge rolls and sprung edges in traditional upholstery. They will learn how to construct and integrate these components to form resilient, aesthetically pleasing seat front edges that support correct ergonomics and uphold traditional furniture-making standards.

🧠 Key Concepts: Sprung Edges and Edge Roll Construction

| Element | Explanation / Purpose |
| --- | --- |
| Sprung Edge | A front seat edge that includes a spring unit, allowing flexibility and increased comfort. Common in high-end and antique furniture. |
| Fixed Front Edge | A non-sprung front rail with padding only; firm, more stable but less forgiving over time. |
| Edge Roll | A formed roll of coir, hair, or stitched material placed at the seat edge to shape and support the front profile. Can be loose, stitched, or integrated. |
| Stitched Edge | A hand-sewn edge built up from filling material and shaped using blind stitching and stuffing ties. |
| Edge Roll Materials | Coir rolls, stitched hair rolls, stitched foam (in hybrids), and stitched canvas tubes (in traditional builds). |
| Ergonomics | Edge rolls help guide thigh support and distribute pressure; sprung edges increase depth and flexibility. |
| Durability Factor | Properly constructed edge rolls maintain their shape over time, while a sprung edge adds resilience to daily wear. |

🪑 Edge Roll and Sprung Edge Application by Furniture Style

| Furniture Style | Edge Construction Feature |
| --- | --- |
| Victorian Club Chair | Deep edge roll with stitched sprung edge |
| Edwardian Armchair | Subtle stitched edge with medium crown and sprung edge |
| Dining Chair (Traditional) | Rolled front edge without springs for firmer seating |
| Heritage Lounge Prototype | Sprung edge with blended coir roll for comfort and form |

💡 Example: Constructing a Sprung Edge in a Lounge Chair

Scenario:
A learner builds a lounge chair prototype that requires a responsive seat edge. The coil springs extend to the very front rail. A canvas tape is stretched over the spring line. A coir edge roll is shaped and blind-stitched along the front curve, forming a smooth transition into the seat crown.

Discussion Points:

- Why was a sprung edge selected over a fixed edge?

- How does the edge roll support the fabric shape and tension?

- What stitching techniques are used to secure the edge without visual disruption?

📦 Case Study: Flat Front Edge Fails Aesthetic and Comfort Test

Case:
In a bespoke prototype armchair, the upholsterer skips the edge roll step and pads the front edge with felt only. The fabric sits flat and fails to hold tension at the curve. Over time, the padding compresses, and the fabric appears slack and rippled.

Facilitator Reflection Questions:

- What fault occurred in the edge construction?

- How did the omission of an edge roll affect both comfort and appearance?

- What traditional techniques could have been applied?

🗣️ Critical Thinking Questions

- What are the functional advantages of using a sprung edge in traditional seat construction?

- How does an edge roll contribute to the life span of both upholstery fabric and the seat foundation?

- In which situations would a fixed edge be more appropriate than a sprung edge?

- What steps ensure symmetry and firmness when stitching and shaping edge rolls?

🛠️ Facilitation Tips

- Use a pre-built frame section with a coil spring extension to demonstrate the creation of a sprung edge.

- Let learners practise positioning and stitching coir rolls using blind stitching techniques.

- Provide cross-section illustrations or models showing different edge configurations.

- Assess learners’ stitched edge for symmetry, firmness, and contour before applying top fabric.

## Integrated Assessment – KM-13-KT04

Knowledge Topic: Prototype Component Construction – Traditional Seat Construction
NQF Level: 4
Weighting: 20%
Total Marks: 50
Assessment Type: Integrated Written Assessment with Case Study and Structured Questions

Section A: Terminology and Materials (IAC0401 and IAC0402)

Total: 14 marks

Instructions: Answer the questions below.

- Match the traditional upholstery terms in Column A with their correct definitions in Column B. (1 mark each – 5 marks)

| Column A | Column B |
| --- | --- |
| A. Edge roll | 1. Fastening device for attaching webbing or springs |
| B. Twine | 2. Woven support strap secured across seat rails |
| C. Webbing | 3. Rolled padding placed along seat front edge |
| D. Hog ring | 4. Heavy-duty string used for spring tying |
| E. Burlap / Hessian | 5. Coarse cloth used as a barrier under stuffing |

- List three natural raw materials used in traditional upholstery and briefly describe one property of each. (6 marks)

- What is the difference between a sprung edge and a fixed edge in traditional seat construction? (3 marks)

Section B: Webbing, Springs, and Tying Techniques (IAC0403 and IAC0404)

Total: 20 marks

Case Study:
A traditional armchair prototype is being constructed using hand-tied coil springs. The webbing has been stretched and secured using tacks. The upholsterer plans to install five coil springs and tie them in a firm configuration. The client has requested a firm seat with defined support and consistent movement.

Instructions: Answer the following questions based on the case study.

- Describe the correct method of positioning and tensioning webbing for this seat type. (4 marks)

- Explain how the springs should be arranged and tied to meet the client's request for firmness. (5 marks)

- Identify two tying patterns commonly used in coil spring upholstery. (2 marks)

- List and describe the function of two knot types used when tying springs. (4 marks)

- How would inconsistent tying tension affect the seat’s shape and function? (5 marks)

Section C: Edge Rolls and Padding Application (IAC0405)

Total: 16 marks

Instructions: Answer the following questions in your own words.

- Describe the purpose of an edge roll in a traditional seat. (3 marks)

- What are the characteristics of a well-formed stitched edge? (4 marks)

- Explain how padding is built up around the springs to achieve the desired crown shape. (5 marks)

- Why is padding symmetry important before applying the top cover? (4 marks)

✅ Model Answers and Marking Memo

Section A – 14 marks

- Matching:
A – 3, B – 4, C – 2, D – 1, E – 5 (5 marks)

- Examples:

- Horsehair: durable and springy

- Coir: coarse and provides firm structure

- Cotton felt: soft, used as a top smoothing layer (6 marks)

- Sprung edge includes springs at the front rail for flexibility; fixed edge uses padding only and is more rigid. (3 marks)

Section B – 20 marks

- Webbing is applied in parallel or criss-cross pattern, stretched tightly using a webbing stretcher, and secured with tacks. (4 marks)

- Springs placed evenly, stitched to webbing, tied in 8-way or diagonal pattern, tensioned consistently for firm response. (5 marks)

- Straight front tie; 8-way hand tie (2 marks)

- Clove hitch: secures twine under tension

- Slip knot: holds temporary tension during tying process (4 marks)

- Inconsistent tension leads to uneven seat height, poor support, misaligned spring movement. (5 marks)

Section C – 16 marks

- Edge roll provides contour, defines seat shape, and distributes thigh pressure evenly. (3 marks)

- Firmly shaped, symmetrical, smooth, and securely stitched to seat base. (4 marks)

- Padding is built in layers—coarse base, shaped crown with hair or felt, blind stitched to hold contour. (5 marks)

- Symmetry ensures comfort, aesthetic quality, and uniform tension when fabric is applied. (4 marks)

📊 Assessment Rubric

| Competency | Outstanding (5) | Competent (4) | Partially Competent (3) | Limited (2) | Not Yet Achieved (1) |
| --- | --- | --- | --- | --- | --- |
| IAC0401 – Terminology and Components | Accurate and confident use of full technical vocabulary | Mostly accurate with minor errors | Basic understanding of terms | Limited grasp | Incorrect or no use of terms |
| IAC0402 – Raw Materials and Characteristics | Strong identification and justification | Clear understanding of materials | Surface-level answers | Minimal relevance | No understanding |
| IAC0403 – Webbing and Spring Construction | Clearly sequenced process, technically precise | Adequate sequencing and description | Partial explanation, some gaps | Limited knowledge | Incorrect or missing |
| IAC0404 – Knotting and Tying Techniques | Strong grasp of technique and function | Describes core functions clearly | General understanding | Limited detail or incorrect use | Not demonstrated |
| IAC0405 – Edge Rolls and Padding | Articulates shape, function and outcome clearly | Explains key elements correctly | Some knowledge evident | Lacks structure or logic | Inaccurate or missing response |

### 🏫 Facilitator Assessment Briefing

Knowledge Module: KM-13
Knowledge Topic: KT0401 – KT0406
Assessment Title: Integrated Assessment for KM-13-KT04: Prototype Component Construction – Traditional Seat Construction
Weighting: 20%
NQF Level: 4
Total Marks: 50
Duration: 2 hours (recommended)
Assessment Type: Integrated Written Assessment
Assessment Method: Structured questions with a case study scenario

🎯 Purpose of the Assessment

This assessment evaluates the learner’s understanding of traditional seat construction within the context of furniture upholstery prototypes. Learners will demonstrate knowledge of traditional terminology, raw materials, and the correct use of tools and methods for foundation building, spring tying, padding, and shaping seat edges. The assessment also tests the learner’s ability to analyse the effectiveness of tying techniques, knots, and padding profiles as applied in traditional upholstery settings.

📘 Assessment Objectives

The learner should be able to:

- Define and use correct industry terminology and identify components used in traditional seat construction (IAC0401)

- Identify and describe raw materials and consumables, including natural fibres and traditional foundation materials (IAC0402)

- Describe the process of arranging and tying webbing and coil springs to produce seats of varying firmness and shape (IAC0403)

- Identify and analyse knotting and tying techniques for coil spring application (IAC0404)

- Explain the function and construction of edge rolls and sprung edges in a traditional seating context (IAC0405)

🧰 Resources Required

Facilitator must prepare or provide:

- Printed learner assessment sheets or booklets

- Visual references: diagrams of tying patterns, knots, edge rolls

- Foam and spring samples or practice frames (optional)

- Marking memo and rubric

- Learners' personal glossaries for reference (if permitted)

✔️ Assessment Conditions

- This is a supervised written assessment, conducted individually.

- No textbooks or electronic devices may be used unless otherwise approved.

- All written responses must be in the learner’s own words.

- Practical demonstration tasks may follow or be assessed in a separate practical assessment.

🧪 Assessment Principles Observed

- Fairness: Questions reflect the learning outcomes and content covered during instruction

- Transparency: Learners are informed of the criteria and expectations beforehand

- Validity: Tasks measure real-world upholstery knowledge and process reasoning

- Reliability: Consistent marking supported by a detailed rubric and memo

- Authenticity: Scenarios reflect genuine prototype construction and upholstery methods

## KM-13-KT05: Testing, Evaluation and Finalisation of Upholstery Prototypes (15%)

Facilitator Guide Introduction

📘 Purpose of the Knowledge Topic

The purpose of this Knowledge Topic is to provide learners with the critical thinking and practical evaluation skills needed to test, assess, and finalise upholstery prototypes prior to production. Prototyping plays a central role in identifying design flaws, assessing performance, and validating production feasibility. Learners will explore how faults that go undetected in the prototype phase can impact quality, cost, client satisfaction, and product safety.

This topic develops learners’ ability to evaluate structural and aesthetic aspects of prototypes, engage constructively with users and production teams, and understand how to re-engineer or modify designs based on testing feedback. Learners will also be introduced to resource and capacity evaluation to inform decision-making during final adjustments.

🧠 Key Knowledge Areas

This topic includes the following learning elements:

- KT0501 Significance of prototyping for troubleshooting potential problems prior to production

- KT0502 Principles and methods of testing, evaluating and modifying prototypes

- KT0503 Implications of faults not identified through the prototype process in production, sale and use of the product

- KT0504 Importance of engaging with prospective users and producers of the production model

- KT0505 Principles and concepts of calculating and evaluating resource and capacity costs and capabilities

- KT0506 Principles of adjusting, adapting, or re-engineering prototypes

🎯 Internal Assessment Criteria and Weighting (15%)

This topic will be assessed against the following criteria:

- IAC0501 The significance of prototyping for troubleshooting potential problems prior to production is analysed and evaluated

- IAC0502 Implications of faults not identified through the prototype process in production, sale and use of the product is reasoned

🪑 Application in the Upholstered Furniture Sector

Testing and evaluation are essential to bridging the gap between design and production. This knowledge ensures that upholstery prototypes are robust, feasible, and ready for market expectations. Learners will be able to critically assess prototypes, identify risks, and implement design adjustments to meet quality, cost, and usability standards. This also supports collaborative development with suppliers, manufacturers, and end-users—key to successful product finalisation in commercial upholstery production.

### KT0501: Significance of Prototyping for Troubleshooting Potential Problems Prior to Production

🌟 Learning Outcome

Learners will understand the critical role of prototyping in identifying design, material, ergonomic, and construction-related issues before entering mass production. They will be able to analyse how effective troubleshooting during the prototype stage improves quality, reduces costs, enhances user satisfaction, and prevents design-related product failures.

🧠 Key Concepts: The Role of Prototyping in Troubleshooting

| Aspect | Explanation / Purpose |
| --- | --- |
| Functional Testing | Verifying that the seat performs correctly under load, movement, or repeated use. |
| Material Suitability Checks | Evaluating whether foam density, fabric behaviour, or frame integrity meet the design brief. |
| Assembly Sequencing Validation | Ensuring that the build process is efficient, logical, and repeatable at production scale. |
| Visual and Aesthetic Checks | Identifying inconsistencies in shape, symmetry, or finish that may be unacceptable to the client. |
| Ergonomic and Comfort Feedback | Testing seat height, back angle, armrest height, and user fit. |
| Cost-Efficiency Evaluation | Identifying where resources are being wasted or where construction could be optimised. |
| Client Sign-Off Opportunity | Prototyping allows design approval or revision before manufacturing commitments are made. |

🪑 Why Troubleshooting is Critical Before Production

| Benefit | Resulting Outcome |
| --- | --- |
| Detects hidden structural weaknesses | Prevents product failure and recalls |
| Improves communication with suppliers | Ensures clear production specifications |
| Avoids cost overruns | Reduces material waste and labour time |
| Enhances market readiness | Delivers a refined, tested product |
| Confirms feasibility of design | Reduces risk during scaling-up |

💡 Example: Prototype Testing Prevents a Production Fault

Scenario:
A lounge chair prototype is assembled using a new eco-foam material. During user testing, the foam compresses too rapidly and fails to recover its shape. The team replaces it with higher-resilience foam after verifying the problem. This adjustment prevents the defect from being reproduced in the 300-unit production run.

Discussion Points:

- What issue was identified, and how?

- What would the consequences have been if the fault went undetected?

- How did prototyping prevent a reputational and financial loss?

📦 Case Study: Skipped Prototyping Results in Rework

Case:
A production team skips the prototyping phase for a dining chair range. Once distributed, clients complain about backrest height being too short for comfort. The company must recall and retrofit 100 chairs, incurring high labour and transport costs.

Facilitator Reflection Questions:

- What design issue could have been addressed in prototyping?

- What value would user engagement during prototyping have added?

- How can a prototype act as a decision-making tool?

🗣️ Critical Thinking Questions

- Why is it risky to proceed directly to production without prototype evaluation?

- How can feedback from users or production staff during prototyping improve the final product?

- What types of problems can be difficult or costly to fix once a product enters mass production?

- How does a well-executed prototype support cross-functional collaboration?

🛠️ Facilitation Tips

- Have learners examine an example prototype and list what should be tested before sign-off.

- Present a design brief and ask learners to predict possible failure points without prototyping.

- Use a checklist format for learners to evaluate a prototype sample or mock-up in a classroom activity.

- Encourage learners to conduct a simulated user review and document key feedback.

### KT0502: Principles and Methods of Testing, Evaluating and Modifying Prototypes

🌟 Learning Outcome

Learners will be able to apply structured methods for testing and evaluating upholstery prototypes. They will understand how to collect, interpret, and apply feedback to make appropriate adjustments or modifications to improve quality, functionality, manufacturability, and user satisfaction.

🧠 Key Concepts: Prototype Testing and Evaluation

| Phase | Purpose / Method |
| --- | --- |
| Functional Testing | Simulate user interaction: sit, shift, apply pressure to assess foam, frame, joints, spring system, and tension. |
| Dimensional Accuracy | Measure against technical drawings or specification sheets (e.g., seat height, back angle, width). |
| Visual Evaluation | Inspect aesthetic finish, symmetry, stitching, fit, material matching, and surface tension. |
| Ergonomic Assessment | Use user testing and posture analysis to determine comfort, reach, and support. |
| Feedback Collection | Record observations, interview users, and production staff using standardised forms. |
| Cost Analysis | Review time taken, material usage, and rework needed to estimate production feasibility. |
| Modification Decision-Making | Apply testing outcomes to adjust design, process, or material selection before finalisation. |

🔁 Common Prototype Adjustments

| Element | Adjustment Example |
| --- | --- |
| Seat depth or height | Foam thickness altered, frame base lowered or lifted |
| Armrest contour | Reshaped for comfort or reduced to save material |
| Backrest angle | Modified to improve lumbar support |
| Edge finish | Seam style changed to improve alignment or reduce labour |
| Material substitution | Higher-grade or more cost-effective material selected based on user wear testing |

💡 Example: Testing Leads to Material Change

Scenario:
A prototype ottoman was upholstered in a faux suede fabric. During testing, the material pilled and stretched under light friction. The evaluation team recommended switching to a performance-grade textile. The switch improved durability and reduced potential returns from clients.

Discussion Points:

- How did the testing method reveal a long-term risk?

- What factors influenced the final decision?

- How did evaluation benefit both the user and the manufacturer?

📦 Case Study: Lack of Evaluation Causes Product Launch Delays

Case:
A design company produces a batch of boutique chairs without testing the angle of the curved backrest. Customers report discomfort after extended sitting. The team must redesign the back angle and produce new prototype backs. The delay postpones the product launch by three weeks.

Facilitator Reflection Questions:

- How could early testing have avoided this issue?

- Why is comfort testing essential even for non-standard designs?

- How can modifications be integrated into future production planning?

🗣️ Critical Thinking Questions

- What methods would you use to ensure a prototype meets comfort, aesthetic, and structural expectations?

- How can you balance design intent with feedback received during prototype evaluation?

- What are the risks of over-modifying a prototype based on one set of user feedback?

- How can changes made during modification be tracked to avoid production confusion?

🛠️ Facilitation Tips

- Provide learners with a mock evaluation checklist to assess an existing prototype sample.

- Role-play a client feedback session and ask learners to determine which issues require design changes.

- Compare two prototype versions (e.g., Version A and B) and ask learners to justify which performs better.

- Assign a modification report task, asking learners to log observed issues and propose three improvements with reasoning.

### KT0503: Implications of Faults Not Identified Through the Prototype Process in Production, Sale and Use of the Product

🌟 Learning Outcome

Learners will understand the consequences of failing to identify and resolve faults during the prototype stage. They will be able to reason how design, structural, or comfort-related faults—if carried into production—can impact manufacturing efficiency, customer satisfaction, product reputation, safety, and cost.

🧠 Key Concepts: Consequences of Unresolved Prototype Faults

| Area Affected | Examples of Implications |
| --- | --- |
| Production | Rework, scrap, or bottlenecks caused by flaws in assembly sequence or material performance. |
| Time and Scheduling | Delays due to last-minute design changes or reconfiguration. |
| Costs and Resources | Increased material wastage, labour inefficiency, tool downtime, and client penalties. |
| End-User Experience | Discomfort, dissatisfaction, returns, or negative reviews due to overlooked ergonomic or quality issues. |
| Product Safety | Risks from structural failure (e.g., collapsing frame, spring failure, foam disintegration). |
| Brand Reputation | Poor market reception, client distrust, reduced repeat business. |

🔍 Commonly Overlooked Faults in Prototypes

| Fault Type | Example |
| --- | --- |
| Frame instability | Seat wobbles or creaks under load, due to poor bracing. |
| Foam misselection | Collapsed seat cushion due to using incorrect density. |
| Stitching issues | Seams split due to excessive tension or misalignment. |
| Incorrect angles | Backrest too upright, causing discomfort. |
| Material incompatibility | Fabric pilling or shrinking after use. |

💡 Example: Missed Spring Fault Affects Production

Scenario:
A coil spring placement issue is missed during prototyping. In full production, 50 chairs are assembled with uneven seat bounce. After delivery, clients complain, and the chairs are returned. The cost of disassembly, correction, and re-delivery significantly impacts profit margin and trust.

Discussion Points:

- How did the absence of fault detection during prototyping affect multiple business areas?

- What process step could have caught the problem?

- What lessons can be learned about prototype inspection?

📦 Case Study: User Feedback Ignored During Prototyping

Case:
During initial user testing of a hospitality banquette seat, users reported discomfort in backrest angle. The design team, concerned with visual appeal, proceeded with the original design. Once installed in a hotel, negative reviews appear online, and the client requests design alterations, requiring costly rework.

Facilitator Reflection Questions:

- What feedback should have influenced a design change?

- What were the long-term implications of not acting on feedback?

- How could early testing with real users have changed the outcome?

🗣️ Critical Thinking Questions

- Why is identifying and correcting faults during prototyping more cost-effective than after production?

- How can a minor fault in a prototype result in large-scale issues once mass production begins?

- What strategies can be used to ensure no critical feedback is overlooked?

- How can prototype sign-off procedures help reduce risk?

🛠️ Facilitation Tips

- Provide learners with case examples of production failures traced back to incomplete prototyping.

- Assign a fault-mapping task: learners identify a potential issue, its production impact, and a resolution method.

- Discuss how different departments (design, production, QA, sales) are affected by unresolved faults.

- Ask learners to develop a pre-production fault checklist for a chosen furniture piece.

### KT0504: Importance of Engaging with Prospective Users and Producers of the Production Model

🌟 Learning Outcome

Learners will understand the value of early and ongoing engagement with both prospective users and producers during the prototype development and finalisation process. They will be able to identify how input from these stakeholders helps refine usability, ensure manufacturability, and improve design outcomes before full-scale production.

🧠 Key Concepts: Stakeholder Engagement in Prototyping

| Stakeholder Group | Role in Prototype Development |
| --- | --- |
| Prospective Users | Provide feedback on comfort, functionality, and aesthetics. Test usability and ergonomics. Help assess real-world application. |
| Production Team / Fabricators | Assess the feasibility of translating the prototype into scalable, efficient manufacturing processes. Provide input on assembly, material handling, and tooling. |
| Sales and Marketing Team | Offer insight into consumer expectations, market trends, and client preferences. Ensure alignment with brand identity. |
| Suppliers | Give input on material availability, costs, and alternatives that may influence production choices. |

🤝 Benefits of Stakeholder Engagement

| Benefit | Example |
| --- | --- |
| Improves user-centred design | A hotel guest chair is modified based on comfort feedback from guests during testing. |
| Identifies manufacturing constraints early | A complex curved component is flagged by fabricators as time-consuming; the design is adjusted to ease production. |
| Prevents post-production conflicts | Sales team concerns about seam visibility are addressed during prototyping, preventing complaints from clients later. |
| Builds collaborative ownership | Teams feel invested in the design outcome, improving internal alignment and communication. |

💡 Example: User Engagement Improves Lounge Chair Design

Scenario:
During prototype testing of a contemporary lounge chair, elderly users report difficulty standing due to the low seat height. The design team raises the seat by 40 mm and adjusts the back angle. The revised prototype is approved and positively reviewed by the client, who places a larger order.

Discussion Points:

- What role did user feedback play in the success of the final product?

- How might the product have failed in market acceptance without this change?

- What would be the result if user needs were ignored?

📦 Case Study: Lack of Production Engagement Causes Delays

Case:
A design team finalises an upholstered bench prototype without consulting the factory. During production rollout, workers flag that a concealed bracket requires non-standard tools and extra labour. The production schedule is delayed, and costs rise.

Facilitator Reflection Questions:

- At what stage should the production team have been involved?

- How could proactive engagement have prevented the issue?

- What processes could be implemented to formalise production input?

🗣️ Critical Thinking Questions

- Why is it important to test a prototype with the intended user group, even if the design team is confident in its design?

- How can communication between designers and producers reduce the risk of costly rework?

- What tools or methods can be used to collect and apply stakeholder feedback efficiently?

- How would you structure a review session involving users and producers to ensure useful input is obtained?

🛠️ Facilitation Tips

- Role-play a stakeholder review session: assign learners roles as users, fabricators, and designers.

- Create a feedback matrix: learners map comments from users and producers and propose design responses.

- Invite guest speakers (e.g., upholsterers, production managers) to discuss how early engagement improves workflow.

- Guide learners in preparing a stakeholder engagement plan for a sample prototype project.

### KT0505: Principles and Concepts of Calculating and Evaluating Resource and Capacity Costs and Capabilities

🌟 Learning Outcome

Learners will be able to apply foundational principles to calculate and evaluate the resource and production capacity required for upholstery manufacturing following the prototype phase. This includes assessing labour, materials, time, and equipment to determine whether a design can be produced within realistic and sustainable cost and capacity limits.

🧠 Key Concepts: Resource and Capacity Evaluation

| Concept | Explanation / Application |
| --- | --- |
| Resource Costing | Calculating the cost of materials (foam, fabric, timber, fasteners), labour (hours per unit), and overheads based on prototype data. |
| Capacity Evaluation | Assessing how many units can be produced within available time, staff, and equipment constraints. |
| Labour Hours per Unit | Estimating how much time is needed for each production step, informed by prototype construction. |
| Material Yields | Evaluating how efficiently materials are used (e.g., foam sheet cut plans, fabric layouts) to minimise waste. |
| Tooling and Equipment Capability | Identifying whether current equipment supports scaling production of the prototype without bottlenecks or excessive downtime. |
| Batch Size and Scheduling | Determining whether the workshop can handle the required output within delivery timelines. |

📊 Evaluation Metrics

| Metric | Example |
| --- | --- |
| Cost per Unit | Total material + labour + overhead ÷ units produced |
| Yield Percentage | Material used ÷ material purchased × 100 |
| Production Time per Unit | Total time for build ÷ number of units |
| Labour Cost per Unit | Hourly wage × time spent per product |
| Available vs. Required Capacity | Hours of factory time available vs. required production time |

💡 Example: Estimating Resource Use Based on Prototype Build

Scenario:
The prototype for a club chair takes 5.5 hours to construct, using 2.2 linear metres of fabric, 1.3 sheets of foam, and specific wood frame parts. The production team calculates that each unit will cost R1 450 in materials and R750 in labour, with an overhead of R300 per chair. This information helps confirm the product’s feasibility within the target price range.

Discussion Points:

- What prototype data helped determine material and labour costs?

- How does the prototype help with scheduling future production runs?

- What factors might affect these costings once scaled?

📦 Case Study: Resource Overload in Scaling

Case:
A design company accepts an order for 150 bespoke ottomans based on a prototype. Once production begins, they realise the piping detail doubles the labour time. Because this was not measured during the prototype phase, the schedule falls behind and the company incurs overtime costs.

Facilitator Reflection Questions:

- What resource or time estimate was overlooked in the prototype phase?

- How could this have been captured and tested earlier?

- How does prototype-based costing protect the business from such errors?

🗣️ Critical Thinking Questions

- How can prototype data help identify where cost savings or time efficiencies can be introduced in production?

- What is the risk of underestimating production capacity during prototype evaluation?

- How would you explain the importance of material yield and labour time to a client questioning the final price?

- What steps can be taken to make prototype-based costing more accurate?

🛠️ Facilitation Tips

- Provide learners with a prototype scenario and ask them to prepare a basic cost and capacity estimate.

- Introduce a capacity worksheet where learners estimate batch sizes based on available work hours.

- Conduct a simulated costing exercise using sample material prices, time logs, and yield data.

- Facilitate a class discussion on balancing quality, cost, and production feasibility.

### KT0506: Principles of Adjusting, Adapting, or Re-engineering Prototypes

🌟 Learning Outcome

Learners will understand how to critically evaluate a prototype after testing and apply appropriate modifications. They will be able to distinguish between minor adjustments, adaptations for specific production constraints or market needs, and full re-engineering decisions when foundational elements of the design must be reworked for viability, performance, or cost reasons.

🧠 Key Concepts: Prototype Adjustment, Adaptation, and Re-engineering

| Concept | Definition / Application |
| --- | --- |
| Adjustment | Small changes to fine-tune the product (e.g., seat height increased by 20 mm; changing stitching detail). |
| Adaptation | Modifications made to suit a different user group, context, or production method (e.g., replacing a curved seam with a flat one for easier manufacturing). |
| Re-engineering | Redesigning a part or the entire structure due to fundamental design faults, material incompatibility, or cost-related feasibility issues. |
| Iterative Design | The process of testing, modifying, and re-testing to refine a prototype for final production. |
| Design Rationalisation | Streamlining features or materials to reduce costs, simplify processes, or improve manufacturability. |
| Performance-Based Change | Altering components based on durability or ergonomic testing data (e.g., changing frame joinery for strength). |

🔁 When to Apply Each Level of Change

| Type of Change | Triggered By |
| --- | --- |
| Adjustment | Minor comfort complaints, visual refinement needs, assembly sequence improvements |
| Adaptation | Different user profile (e.g., elderly users), shift to different production tooling, material change |
| Re-engineering | Structural failure, significant cost overrun, consistent negative feedback, or safety risks |

💡 Example: Re-engineering a Frame for Strength

Scenario:
During prototype testing, a stylish open-leg frame for a commercial bench shows signs of deflection under repeated use. The design team decides to re-engineer the base structure using laminated cross-supports and inserts to increase rigidity. While the visual style is retained, the construction method changes entirely.

Discussion Points:

- What factors justified re-engineering instead of small adjustments?

- How did the change balance aesthetic and functional priorities?

- What lessons were applied from the prototype evaluation?

📦 Case Study: Adaptation for Market Expansion

Case:
A lounge chair prototype originally designed for private homes is being considered for hospitality use. The team adapts the design by replacing the delicate textile with a stain-resistant commercial fabric, and reinforcing the suspension system for high-traffic usage.

Facilitator Reflection Questions:

- What distinguishes this as an adaptation rather than re-engineering?

- How can market considerations influence prototype modification?

- What documentation is necessary to track these changes?

🗣️ Critical Thinking Questions

- How can you determine whether a prototype issue requires an adjustment, adaptation, or full re-engineering?

- What are the risks of re-engineering late in the development process?

- How can stakeholder input guide the modification process?

- How do you ensure that modifications still honour the original design intent and brief?

🛠️ Facilitation Tips

- Provide three different prototype “fault” scenarios and ask learners to classify the required level of change.

- Guide learners to complete a “Modification Log” outlining the type of change, reason, and anticipated impact.

- Simulate a change request from a client and ask learners to revise a component of the design accordingly.

- Invite learners to reflect on an earlier project and suggest one change they would now make, and why.

## Integrated Assessment – KM-13-KT05

Knowledge Topic: Testing, Evaluation and Finalisation of Upholstery Prototypes
NQF Level: 4
Weighting: 15%
Total Marks: 40
Assessment Type: Structured Written Assessment and Case-Based Analysis

Section A: Significance of Prototyping (IAC0501)

Total: 20 marks
Instructions: Answer the following questions in your own words.

- Explain the purpose of prototyping in upholstery design and production. (4 marks)

- List three key areas of product performance that are tested during the prototyping process. (6 marks)

- Describe two benefits of identifying production-related issues during the prototyping phase. (4 marks)

- Provide one example of a product design fault that could be identified and corrected during the prototype phase. (2 marks)

- Analyse the impact of skipping the prototyping process before launching a new furniture item. (4 marks)

Section B: Implications of Unresolved Faults (IAC0502)

Total: 20 marks
Scenario-Based Question:

A mid-sized furniture company produced a range of upholstered barstools based on a design that was not properly prototyped. Shortly after delivery to a hospitality venue, the backrests began to loosen. It was discovered that the internal brackets used to secure the back were insufficient for repeated commercial use. The company had to recall and repair the batch, leading to additional costs, reputational harm, and customer dissatisfaction.

Instructions: Answer the questions below using the case scenario and your own knowledge.

- Identify the fault that should have been discovered during prototyping. (2 marks)

- Discuss how this fault impacted:
a) Production costs (2 marks)
b) Customer satisfaction (2 marks)
c) Product safety (2 marks)

- Describe two long-term consequences for the business caused by unresolved faults entering the market. (4 marks)

- Suggest two practical ways to prevent similar problems in future projects. (4 marks)

- Reason why it is important to evaluate structural components and user feedback before approving final production. (4 marks)

✅ Model Answers and Marking Memo

Section A – 20 marks

- Prototyping identifies problems with design, materials, ergonomics, or construction before full-scale production. (4 marks)

- Areas tested:

  - Structural integrity

  - Aesthetic alignment and quality

  - Ergonomics and user comfort (2 marks each – 6 marks)

- Benefits include:

  - Reduced risk of rework

  - Improved communication with production team (2 marks each – 4 marks)

- Example: A chair back that is too upright for comfort, adjusted during prototyping. (2 marks)

- Skipping prototyping can lead to costly errors, user complaints, reputational damage, and inefficient production. (4 marks)

Section B – 20 marks

- Fault: Inadequate bracket strength in high-use environments. (2 marks)

- a) Extra labour and transport costs for recall and repair (2 marks)
b) Negative user experience and reviews (2 marks)
c) Increased risk of user injury or legal liability (2 marks)

  - Loss of future contracts from the client

  - Damage to market reputation and loss of trust (2 marks each – 4 marks)

  - Conduct load testing during prototype review

  - Include feedback loop from intended users (2 marks each – 4 marks)

- Structural evaluation ensures product reliability; user feedback ensures the product meets expectations and functions correctly. (4 marks)

📊 Assessment Rubric

| Competency | Outstanding (5) | Competent (4) | Partially Competent (3) | Limited (2) | Not Yet Achieved (1) |
| --- | --- | --- | --- | --- | --- |
| IAC0501 – Significance of Prototyping | Insightful, clearly explained and applied | Accurately explained with examples | General understanding | Vague or incomplete | No understanding shown |
| IAC0502 – Reasoning on Fault Implications | Logically reasoned, practical and holistic | Sound reasoning with relevant examples | Partial or limited understanding | Incomplete, with basic examples | No valid response provided |

### 🏫 Facilitator Assessment Briefing

Knowledge Module: KM-13
Knowledge Topic: KT0501 – KT0506
Assessment Title: Integrated Assessment for KM-13-KT05: Testing, Evaluation and Finalisation of Upholstery Prototypes
Weighting: 15%
NQF Level: 4
Total Marks: 40
Duration: 90 minutes (recommended)
Assessment Type: Integrated Written Assessment
Assessment Method: Structured questions and scenario-based analysis

🎯 Purpose of the Assessment

This assessment evaluates learners’ ability to reason, analyse, and apply knowledge related to the testing and evaluation of upholstery prototypes. It explores the value of prototyping as a tool for problem-solving, highlights the implications of unresolved faults, and encourages learners to think critically about how adjustments and stakeholder feedback influence production readiness.

📘 Assessment Objectives

Learners completing this assessment should be able to:

- Analyse and evaluate the significance of prototyping in identifying and resolving production problems prior to scaling up (IAC0501)

- Reason the implications of untested or unresolved faults on the quality, safety, and marketability of upholstery products (IAC0502)

🧰 Resources Required

Facilitators must provide:

- Printed learner assessment scripts

- Supporting materials (optional): fault case studies, sample product images, or prototype evaluation checklists

- Marking memo and assessment rubric

- Answer sheets or notebooks

✔️ Assessment Conditions

- The assessment must be completed individually under supervision.

- No use of electronic devices or textbooks is allowed.

- Facilitators may clarify instructions but may not guide answers.

- Learners should use clear reasoning and full sentences to support their answers.

🧪 Assessment Principles Observed

- Fairness: The questions align with the taught content and practical application

- Transparency: Learners are informed of the structure, duration, and expectations in advance

- Validity: Assessment items measure knowledge and reasoning within the prototype development cycle

- Reliability: Consistent marking ensured through the use of model answers and rubric

- Authenticity: The tasks simulate real-world upholstery prototyping challenges

## Bottom of Form

## KM-13-KT06: Product Re-engineering (15%)

Facilitator Guide Introduction

📘 Purpose of the Knowledge Topic

The purpose of this Knowledge Topic is to equip learners with the ability to critically assess, plan, and apply re-engineering processes to upholstery prototypes that do not meet design, material, or production performance expectations. It provides a comprehensive understanding of re-engineering, re-prototyping, and re-designing within the upholstered furniture sector and guides learners in identifying process and product faults through structured analysis and effective communication.

Learners will also understand the cost implications of re-engineering versus the consequences of continuing with a faulty product, and they will develop the skills to contribute to team-based review and decision-making through effective record keeping and root cause analysis.

🧠 Key Knowledge Areas

This topic includes the following content areas:

- KT0601 Definitions and terminology used when re-engineering upholstered furniture

- KT0602 Principles and concepts of re-engineering, re-prototyping and re-designing upholstered furniture prototypes

- KT0603 Factors influencing the decision to re-engineer prototypes

- KT0604 Fault finding and root cause analysis of the process, design or materials faults

- KT0605 Record keeping

🎯 Internal Assessment Criteria and Weighting (15%)

This topic will be assessed against the following criteria:

- IAC0601 Distinguish between and explain the concepts of re-engineering, re-prototyping and re-designing upholstered furniture prototypes

- IAC0602 Describe and compare the cost of re-engineering prototypes with the cost of not doing so, and explain the impact on productivity levels

- IAC0603 Describe manufacturing process faults, including resource availability and capacity, organisational constraints such as health and safety considerations, and technical and time constraints that will result in process re-engineering

- IAC0604 Describe the materials performance faults that could result in re-prototyping

- IAC0605 List the product design faults including function and ergonomic design considerations, that require re-designing

- IAC0606 Define the concept of root cause analysis and explain how to apply it to identify prototype process, design and material faults

- IAC0607 Explain the function and importance of record keeping and regular team meetings to review work plans to report on progress, delays, quality issues and assembly methods

🪑 Application in the Upholstered Furniture Sector

Product re-engineering is an essential process in maintaining quality assurance and production viability in the upholstered furniture industry. This knowledge area empowers learners to identify underlying issues in materials, processes, or design, and take corrective action before full-scale production. It reinforces the principle that successful product development is iterative, collaborative, and responsive to user, technical, and organisational needs.

### KT0601: Definitions and Terminology Used When Re-engineering Upholstered Furniture

🌟 Learning Outcome

Learners will be able to define and accurately use key terminology related to the re-engineering, re-prototyping, and re-design of upholstered furniture prototypes. This understanding will allow them to participate meaningfully in discussions about process changes, quality assurance, and prototype refinement.

🧠 Key Terminology and Definitions

| Term | Definition / Application |
| --- | --- |
| Re-engineering | A structured process of redesigning production methods, materials, or construction to correct faults, improve performance, or align with capacity and cost constraints. |
| Re-prototyping | The act of building a revised version of the prototype after identifying flaws that impact performance, comfort, or manufacturability. |
| Re-designing | Revising the original product concept or function, typically due to major flaws in ergonomics, aesthetics, or client dissatisfaction. |
| Root Cause Analysis | A problem-solving method used to identify the underlying cause(s) of faults in design, process, or materials. |
| Process Fault | An error or inefficiency in the production workflow (e.g., incorrect assembly sequence or tool mismatch). |
| Design Fault | A flaw in the product’s dimensions, ergonomics, or aesthetic that limits its usability or visual appeal. |
| Material Performance Fault | When a material fails to perform as expected under intended conditions (e.g., foam collapsing, fabric tearing). |
| Corrective Action | A change made to eliminate the cause of a detected fault or non-conformity. |
| Iteration | Repeating stages of prototyping or design to improve a product incrementally. |

💡 Example: Understanding the Difference Between Re-engineering and Re-designing

Scenario:

A lounge chair prototype is rejected by the client because it is too low for elderly users. The team raises the seat height by 30 mm without changing the overall design. This is an example of re-engineering. However, if the team had to create a new product with a completely different base structure and arm profile, it would be considered re-designing.

Discussion Points:

- What defines the level of change needed?

- How does terminology affect the decision-making process?

- Why is it important to classify the type of change correctly?

📦 Case Study: Miscommunication Due to Incorrect Terminology

Case:
A production manager receives a report stating that the prototype needs “re-design”, but the issue was simply a misalignment in the leg brackets. As a result, the design team begins major revisions unnecessarily, wasting time and resources.

Facilitator Reflection Questions:

- What misunderstanding occurred here?

- How could correct terminology have prevented this issue?

- Why is precision in communication critical in re-engineering processes?

🗣️ Critical Thinking Questions

- How does using clear terminology improve teamwork during the prototype evaluation and modification process?

- What is the difference between re-prototyping and re-designing, and why is it important to distinguish between the two?

- When should the term “root cause” be used, and what does it imply about the problem-solving approach?

- Why is “iteration” an important term in the context of upholstery prototyping?

🛠️ Facilitation Tips

- Present learners with a glossary of re-engineering terms and ask them to use each in a sentence related to upholstery prototyping.

- Create a classroom exercise where learners classify examples as re-engineering, re-prototyping, or re-designing.

- Use visual aids showing product versions across iterations to highlight how terminology changes with the level of adjustment.

- Incorporate a role-play activity where learners simulate a design review meeting using correct terminology to describe faults and solutions.

### KT0602: Principles and Concepts of Re-engineering, Re-prototyping and Re-designing Upholstered Furniture Prototypes

🌟 Learning Outcome

Learners will be able to distinguish between re-engineering, re-prototyping, and re-designing, and apply each concept appropriately in response to identified faults in upholstered furniture prototypes. They will also understand the principles that guide when and why each of these corrective approaches is used during the product development lifecycle.

🧠 Key Principles and Concepts

| Concept | Definition | When Applied |
| --- | --- | --- |
| Re-engineering | Adjusting or modifying the existing construction process, material use, or manufacturing method to improve feasibility, cost, or quality. | When production challenges, time constraints, or minor performance issues arise. |
| Re-prototyping | Building a new prototype version using revised specifications based on performance or usability faults. | When material failure, comfort issues, or structural faults are identified during testing. |
| Re-designing | Revisiting and changing the original design intent, aesthetics, or product function. | When fundamental flaws exist in function, usability, or client satisfaction, or when intended use changes. |

🔁 Guiding Principles for Each Process

| Principle | Application |
| --- | --- |
| Root cause drives the process | Always identify whether the fault stems from material, process, or design. |
| Minimal effective intervention | Begin with re-engineering unless re-design is clearly necessary. |
| Cost and time impact must be measured | Compare corrective options based on production timelines and resources. |
| Stakeholder input is essential | Include client, fabricator, and end-user feedback to inform changes. |
| Document every change | Maintain clear records of what was changed, why, and when. |

💡 Example: Choosing Between Re-engineering and Re-prototyping

Scenario:
During the prototype phase of a showroom armchair, it is found that the seat foam compresses too quickly, leading to user discomfort. The design is kept the same, but the foam density is increased, and a different bonding adhesive is used. This is a re-engineering action. However, if the backrest were redesigned for better lumbar support and a new prototype was built, that would be re-prototyping.

Discussion Points:

- Why was full re-prototyping not necessary in the first instance?

- How does foam selection relate to performance and testing feedback?

- What documentation is required when changing materials?

📦 Case Study: Misapplication of Re-design Instead of Re-engineering

Case:
A furniture company re-designs an entire lounge chair after a supplier reports a shortage of the original armrest fabric. Instead of sourcing a similar alternative and modifying the stitch pattern slightly (re-engineering), the team redraws the arm profile entirely. This causes delay and increases tooling costs.

Facilitator Reflection Questions:

- What would have been a more efficient corrective approach?

- How can distinguishing between corrective paths save time and cost?

- When is re-design justified despite higher costs?

🗣️ Critical Thinking Questions

- What factors should be considered when deciding between re-engineering, re-prototyping, or re-designing a prototype?

- How can a structured problem-solving approach prevent unnecessary redesign?

- What are the risks of making large-scale design changes too early or too late in the prototype process?

- How does each approach affect overall project timelines and cost projections?

🛠️ Facilitation Tips

- Guide learners through a “decision tree” activity where they diagnose a fault and recommend a corrective path.

- Provide side-by-side examples of a re-engineered, re-prototyped, and re-designed component for discussion.

- Use real-world scenarios and ask learners to justify their corrective decision and its impact on cost and schedule.

- Encourage learners to keep a reflection log of decisions made during their own prototyping tasks, labelled by type of intervention.

### KT0603: Factors Influencing the Decision to Re-engineer Prototypes

🌟 Learning Outcome

Learners will be able to identify and evaluate the various factors that influence the decision to re-engineer a furniture prototype. They will understand how to weigh cost, time, quality, safety, and stakeholder requirements to make informed, practical decisions during product development and refinement.

🧠 Key Factors Influencing the Re-engineering Decision

| Factor | Explanation / Application |
| --- | --- |
| Cost Implications | Re-engineering may reduce overall costs (e.g., through more efficient materials or faster construction), but initial redesign must be justified. |
| Production Feasibility | Whether the current design can be scaled up with available tools, machinery, and labour without significant difficulty. |
| Performance Issues | If the prototype fails in comfort, support, safety, or material durability, re-engineering may be required. |
| Material Availability or Substitution | Certain materials may be delayed or discontinued, requiring modifications to the structure or finish. |
| Health and Safety Constraints | New compliance or risk factors may emerge during testing, prompting structural or procedural changes. |
| Manufacturing Constraints | Limitations of available workshop space, skills, or equipment that prevent successful production of the current prototype. |
| Client Requirements | Changes requested by the client after prototype review (e.g., softer arms, modified dimensions). |
| Time Constraints | When deadlines require faster methods or adjustments to reduce labour or complexity. |
| Quality Assurance Feedback | Internal checks or early user testing may flag flaws that need to be addressed to maintain product standards. |

🧩 Strategic Decision-Making: When to Re-engineer

| Indicator | Recommended Response |
| --- | --- |
| Foam collapses during stress testing | Replace foam and reassess support structure (re-engineering) |
| Assembly takes too long in trial build | Streamline assembly steps or modify design for quicker construction |
| Product cannot be built with existing tools | Adjust joinery or component sizing |
| User requests functional change | Alter only relevant parts unless a full redesign is warranted |

💡 Example: Balancing Cost and Function

Scenario:
The prototype of a show wood armchair includes a sculpted armrest requiring CNC shaping. The production team indicates that CNC costs will exceed budget. The design is re-engineered by modifying the profile to allow manual shaping using jigs and routers.

Discussion Points:

- What factors influenced the re-engineering choice?

- How did cost and tooling availability shape the solution?

- What quality standards were preserved?

📦 Case Study: Ignoring Constraints Leads to Production Failure

Case:
A design team finalises a sectional sofa prototype without considering the limited width of the upholstery workshop’s doorway. Once manufacturing begins, units cannot be moved into the finishing area without partial disassembly. The team must re-engineer the frame into modular sections, causing delays.

Facilitator Reflection Questions:

- Which constraints were overlooked?

- What assessment step could have predicted the issue?

- How can pre-production checks prevent re-engineering under pressure?

🗣️ Critical Thinking Questions

- How should time and budget constraints be balanced against the need for a functional, high-quality product?

- What tools or techniques can help identify whether re-engineering is necessary?

- How do safety and ergonomic compliance affect re-engineering decisions?

- In what ways can team collaboration improve re-engineering decisions?

🛠️ Facilitation Tips

- Create a decision matrix exercise where learners rate several factors (cost, time, function, safety) and choose whether re-engineering is warranted.

- Present real or simulated prototype briefs and have learners identify potential re-engineering triggers.

- Ask learners to conduct a “constraint audit” of a design and list how each constraint might impact manufacturing.

- Encourage peer review and discussion of when a modification crosses into re-engineering territory.

### KT0604: Fault Finding and Root Cause Analysis of the Process, Design or Materials Faults

🌟 Learning Outcome

Learners will be able to conduct fault finding and apply the principles of root cause analysis (RCA) to systematically identify and address the origin of faults in the design, materials, or production process of upholstered furniture prototypes. This equips them to make informed decisions about corrective actions and re-engineering steps.

🧠 Key Concepts: Fault Finding and Root Cause Analysis

| Concept | Explanation / Application |
| --- | --- |
| Fault Finding | The process of identifying specific errors or failures in a product's function, construction, appearance, or usability. |
| Root Cause | The underlying reason a fault occurred—not just the symptom. For example, loose joints may be due to poor-quality adhesive, not just poor technique. |
| Root Cause Analysis (RCA) | A structured method to trace problems back to their true origin, often using tools such as the “5 Whys”, Fishbone Diagram, or Pareto Analysis. |
| Process Fault | A breakdown or inefficiency in how the prototype is built (e.g., incorrect sequence, excessive handling, or tooling mismatch). |
| Design Fault | A miscalculation in the size, proportions, ergonomics, or joinery details that affects functionality. |
| Material Fault | Performance issues due to incorrect or poor-quality foam, fabric, frame materials, or adhesives. |

🛠️ Tools and Techniques for RCA

| Tool / Method | Purpose |
| --- | --- |
| 5 Whys | Repeatedly asking “Why?” to move from symptoms to causes. |
| Fishbone (Ishikawa) Diagram | Visual tool used to categorise potential causes into types (e.g., materials, methods, environment, equipment, people). |
| Checklists / Fault Logs | Used to record common or recurring problems during prototype builds. |
| User Feedback Reports | A valuable input for identifying discomfort, usability issues, or aesthetic flaws. |

💡 Example: Using RCA to Solve a Sagging Seat Issue

Scenario:
A prototype armchair shows sagging in the front third of the seat after two days of testing. The team applies the “5 Whys”:

- Why is the seat sagging? – The foam has compressed.

- Why did the foam compress? – It was too soft for the seat structure.

- Why was that foam selected? – It was assumed sufficient based on appearance.

- Why was no weight test done? – Testing was skipped due to time pressure.

- Why was there time pressure? – Schedule was not revised after material delivery delays.

Discussion Points:

- What is the actual root cause?

- What corrective actions would resolve it?

- How could the process be improved?

📦 Case Study: Fault Finding in a Traditional Sofa Frame

Case:
During reupholstery of a traditional three-seater, the team discovers the left seat has dropped. Upon inspection, they find broken webbing and a loose corner block. A root cause analysis identifies that staples were used instead of tacks during the last repair, and the webbing was incorrectly tensioned.

Facilitator Reflection Questions:

- What were the immediate and root causes of the fault?

- What process or training improvement is needed?

- How would this issue affect product lifespan and client satisfaction?

🗣️ Critical Thinking Questions

- Why is it important to distinguish between a fault’s symptom and its root cause?

- What are the risks of fixing a fault without conducting a root cause analysis?

- How can collaborative RCA improve workshop efficiency and product quality?

- When is it appropriate to escalate a fault for full design review?

🛠️ Facilitation Tips

- Run a fault analysis workshop using furniture mock-ups or case-based descriptions.

- Provide learners with sample RCA tools (5 Whys template, fishbone diagram) to apply in teams.

- Assign each group a fault scenario and ask them to identify both symptoms and root cause.

- Debrief each example with a focus on how proper diagnosis would save time and cost.

### KT0605: Record Keeping

🌟 Learning Outcome

Learners will understand the purpose and importance of effective record keeping during prototype development and re-engineering processes. They will learn how documentation supports decision-making, progress tracking, quality control, team communication, and accountability throughout the prototype lifecycle.

🧠 Key Concepts: Record Keeping in Upholstery Prototyping

| Record Type | Purpose / Function |
| --- | --- |
| Prototype Build Log | Tracks time taken, materials used, processes followed, and any deviations from the plan. |
| Fault and Modification Reports | Records problems encountered, analysis results, and corrective actions taken. |
| Meeting Minutes and Action Logs | Summarises feedback, decisions, and responsibilities from review meetings. |
| Quality Control Checklists | Verifies that each stage of the prototype meets performance, safety, and aesthetic standards. |
| Client Sign-Off Forms | Captures approvals, requested changes, and scope confirmations for final revisions. |
| Material Cost and Yield Records | Assists in future costing, budgeting, and material planning. |

📋 Why Record Keeping Is Critical

| Benefit | Application |
| --- | --- |
| Supports Consistency and Repeatability | Allows successful builds to be replicated for future production. |
| Improves Communication Across Teams | Provides shared visibility of decisions and delays. |
| Enables Traceability | Identifies when and why changes were made, and by whom. |
| Aids in Time and Cost Analysis | Helps determine actual resource use against estimates. |
| Forms Part of Quality Assurance Systems | Documentation supports compliance with standards and justifies rework or adjustments. |

💡 Example: Keeping Records During a Prototype Revision

Scenario:
A learner is tasked with adjusting the arm profile of a prototype chair based on client feedback. They record the requested change in a prototype modification log, update the sketch in the project file, and submit the change for team discussion. After testing the adjustment, the new arm design is signed off by the client and added to the production specification sheet.

Discussion Points:

- What documentation was generated?

- How does this record trail support final production?

- What would be the risk of making this change without records?

📦 Case Study: Lack of Records Causes Assembly Errors

Case:
A furniture company finalises a prototype but does not record the specific foam layering method used for the seat. During production, variations emerge between units. The team cannot confirm which approach was approved, and rework is required. This causes delays and costs.

Facilitator Reflection Questions:

- What record was missing, and what was the consequence?

- How could simple documentation have prevented the issue?

- What ongoing documentation tools should be implemented?

🗣️ Critical Thinking Questions

- How can proper record keeping reduce confusion and delays during re-engineering?

- What are the key records you would keep during a prototype modification?

- How can documented feedback from meetings improve quality and teamwork?

- What are the risks of relying on verbal instructions or memory during prototype changes?

🛠️ Facilitation Tips

- Provide learners with a set of record templates (e.g., fault log, meeting minutes, change control form) to complete during a prototype simulation.

- Assign a role-playing activity where one group plays a client, and another maintains design documentation based on feedback.

- Review examples of clear and unclear records and ask learners to evaluate their usefulness.

- Encourage learners to maintain their own prototype journals or design logs as part of their training projects.

## Integrated Assessment – KM-13-KT06: Product Re-engineering

Knowledge Topic: KT0601 – KT0607
NQF Level: 4
Weighting: 15%
Total Marks: 50
Assessment Type: Structured Written Assessment and Case-Based Evaluation

Section A: Understanding Core Concepts (IAC0601, IAC0602)

Total: 14 marks

- Define and distinguish between the following three concepts:
a) Re-engineering
b) Re-prototyping
c) Re-designing
(2 marks each – 6 marks)

- Describe one scenario where re-prototyping would be more appropriate than re-designing. (2 marks)

- Compare the cost of re-engineering a prototype versus the cost of continuing to production without correcting a known fault.
Explain how this choice affects productivity. (6 marks)

Section B: Identifying Manufacturing and Material Faults (IAC0603, IAC0604, IAC0605)

Total: 18 marks

Instructions: Read the following scenario and answer the questions.

Scenario:
During the final prototype review of a recliner chair, the production manager identifies that:

- The lever mechanism does not align properly when mounted using standard jigs

- A batch of foam for the backrest has shrunk after application

- Some users reported the arm height was too low for support

- Identify and explain two manufacturing process faults present in this scenario. (2 marks each – 4 marks)

- Explain how resource capacity and technical constraints could influence the decision to re-engineer the frame assembly. (4 marks)

- Describe the material performance fault and explain why it would result in re-prototyping. (4 marks)

- List two design faults that would justify re-designing the armrest component, based on ergonomic considerations. (3 marks)

- What organisational constraint must be considered when adjusting the mechanical lever assembly? (3 marks)

Section C: Root Cause Analysis and Record Keeping (IAC0606, IAC0607)

Total: 18 marks

- Define root cause analysis and explain how it could be applied to investigate the faulty lever mechanism in the scenario. (6 marks)

- List three records or documents that should be maintained during the re-engineering and prototype revision process. (3 marks)

- Explain the function and importance of:
a) Regular team meetings during prototype revision (3 marks)
b) Work plan progress reports and fault tracking (3 marks)
c) Assembly method documentation (3 marks)

✅ Model Answers and Marking Memo

Section A – 14 marks

a) Re-engineering – Modification to materials or processes to improve manufacturability
b) Re-prototyping – Rebuilding the prototype to test changes in materials, function, or construction
c) Re-designing – Changing the product’s concept, function, or core design elements (6 marks)

- Foam density fails during testing – re-prototype with new foam rather than change the design (2 marks)

Re-engineering incurs short-term cost (e.g., R500–R1000 in materials/labour), but saves large-scale rework cost later (e.g., R10 000+).
Productivity is preserved by avoiding disruption to the production line (6 marks)

Section B – 18 marks

- Jig incompatibility with component (process fault)

- Incorrect sequence or misalignment affecting assembly (4 marks)

- Limited tooling capability affects part alignment

- Staff not trained to manually adjust brackets affects productivity (4 marks)

- Foam shrinkage is a performance fault; it compromises comfort and requires a revised prototype to test alternatives (4 marks)

- Low arm height affects posture and support

- Arm width may prevent comfortable forearm positioning (3 marks)

- Health and safety risks from misaligned mechanisms must be considered to avoid injury (3 marks)

Section C – 18 marks

Root Cause Analysis is the method of identifying the true cause of a fault.
Use 5 Whys:

- Why is the lever misaligned? – Because it does not fit the jig.

- Why does it not fit? – Jig made for a different bracket version.

- Why was it not adjusted? – Version change was not documented, etc. (6 marks)

- Modification log

- Quality assurance checklist

- Meeting notes (3 marks)

a) Ensures communication, team alignment, and problem-solving (3 marks)
b) Tracks changes and avoids duplicated efforts (3 marks)
c) Prevents errors in repeat builds and ensures accuracy in future production (3 marks)

📊 Assessment Rubric

| Competency | Outstanding (5) | Competent (4) | Partially Competent (3) | Limited (2) | Not Yet Achieved (1) |
| --- | --- | --- | --- | --- | --- |
| IAC0601 – Differentiating Key Concepts | Fully explained with precise distinctions | Mostly accurate definitions | Definitions somewhat blurred | Limited understanding | Incorrect or missing |
| IAC0602 – Cost and Productivity Impact | Clearly justified with comparative insight | Reasoned comparison with examples | Generalised with some relevance | Lacks depth or relevance | Not addressed |
| IAC0603 – Process Faults and Constraints | Realistic and well-analysed examples | Mostly accurate with context | Limited understanding | Simplistic or vague | Incorrect or off-topic |
| IAC0604 – Material Faults | Correctly identified and linked to RCA | Reasonably explained | Partial understanding | Limited insight | Not attempted |
| IAC0605 – Design Faults | Functional, ergonomic, and user-focused | Valid and relevant examples | General response only | Missing ergonomics link | Not demonstrated |
| IAC0606 – RCA Application | Structured, logical and applied clearly | Mostly logical and complete | Some application with gaps | Basic or superficial | Incorrect or not explained |
| IAC0607 – Record Keeping and Meetings | Integrated, relevant and insightful | Accurate and clearly explained | Partially addressed | Incomplete or unclear | Not addressed |

### Bottom of Form‍🏫 Facilitator Assessment Briefing

Knowledge Module: KM-13
Knowledge Topic: KT0601 – KT0607
Assessment Title: Integrated Assessment for KM-13-KT06: Product Re-engineering
Weighting: 15%
NQF Level: 4
Total Marks: 50
Duration: 90 minutes (recommended)
Assessment Type: Integrated Written Assessment
Assessment Method: Structured questions and scenario-based analysis

🎯 Purpose of the Assessment

This assessment evaluates the learner’s ability to distinguish between re-engineering, re-prototyping, and re-designing; identify faults related to process, design and materials; apply root cause analysis; and explain the importance of accurate record keeping during prototype revisions. It develops the learner’s capacity to engage in reflective, responsive, and structured problem-solving in upholstery product development.

📘 Assessment Objectives

Upon completion of this assessment, learners should be able to:

- Define and differentiate re-engineering, re-prototyping, and re-designing (IAC0601)

- Analyse cost and productivity implications of re-engineering decisions (IAC0602)

- Identify process, design, and material faults in prototype construction (IAC0603–IAC0605)

- Apply root cause analysis to detect and solve underlying issues (IAC0606)

- Explain how record keeping and review meetings contribute to quality assurance and production planning (IAC0607)

🧰 Resources Required

Facilitators should provide:

- Printed assessment instruments

- Visuals or case studies (optional) for practical examples

- Model answers and assessment rubric

- Templates for change logs, fault reports or team meeting notes (if applicable)

✔️ Assessment Conditions

- Individual, supervised written assessment

- No electronic devices or resource materials permitted

- Facilitators may clarify instructions but may not guide content

- All responses must be the learner’s own work

🧪 Assessment Principles Observed

- Fairness: Aligned to taught content and workplace relevance

- Validity: Tests both theoretical understanding and workplace application

- Reliability: Clear marking memorandum and rubric provided

- Transparency: Learners informed of criteria and structure in advance

- Authenticity: Tasks simulate real-world scenarios of product troubleshooting and refinement