FURNITURE UPHOLSTERER

KM14 CUTTING AND SEWING OF FABRIC AND MATERIAL FOR UPHOLSTERED FURNITURE PROTOTYPES

FACILITATOR GUIDE

Table of Contents

KM-14: Cutting and Sewing of Fabric and Material for Upholstered Furniture Prototypes	3

KM-14-KT01: Lay and Cut Upholstery Covers	5

KT0101 – Cutting Patterns and Templates	7

KT0102: Cutting Plans	10

KT0103: Cutting Lays – To Scale, Full Size	13

KT0104: Fabric Types and Characteristics	16

KT0105: Cutting Lay Options	19

KT0106: Cutting Lay Information	22

KT0107: Cutting – Methods, Tools, and Equipment	25

Facilitator Assessment Briefing: KM-14-KT01 – Lay and Cut Upholstery Covers (50%)	28

Integrated Assessment: KM-14-KT01 – Lay and Cut Upholstery Covers (50%)	32

KM-14-KT02: Cover Making for Upholstered Prototypes – Sewing of Components	36

KT0201: Upholstery Cut Components and Parts	38

KT0202: Sewing Machine Types, Functions and Capabilities	41

KT0203: Different Types and Styles of Stitching	44

KT0204: Sewing Machine Parts and Attachments – Identification and Their Uses	47

KT0205: Flaws and Defects of Consumables and Sewing (Raw Material Faults, Process Faults, Product Faults)	50

KT0206: Quilting Methods	53

KT0207: Sewing Work Station Housekeeping	56

Integrated Assessment: KM-14-KT02 – Cover Making for Upholstered Prototypes: Sewing of Components (50%)	59

Facilitator Assessment Briefing: KM-14-KT02 – Cover Making for Upholstered Prototypes	64

# KM-14: Cutting and Sewing of Fabric and Material for Upholstered Furniture Prototypes

NQF Level 2 | Credits: 10

14.1 Purpose of the Knowledge Module

The primary purpose of this Knowledge Module is to develop foundational competencies in learners related to the laying out, cutting, and sewing of fabric and material used in the manufacture of upholstered furniture prototypes. The learning focuses on cover making and covering methods applicable to bespoke upholstery furniture, upholstery repairs, and bulk production scenarios.

This module contributes to the development of occupational competence by equipping learners with a solid understanding of the principles and procedures needed to ensure precision in cutting and accurate assembly during the upholstery fabrication process. The module aligns with the scope of work performed by semi-skilled upholsterers, upholstery prototype manufacturers, and sewing machine operators, among other occupational roles within the upholstery industry.

14.2 Key Knowledge Areas

The following Knowledge Topics are addressed in this module:

- KM-14-KT01: Lay and Cut Upholstery Covers (50%)

- KM-14-KT02: Cover Making for Upholstered Prototypes: Sewing of Components (50%)

Each topic is weighted equally and is designed to support learners in understanding not only the techniques and sequences involved but also the importance of precision, material characteristics, and workplace specifications.

14.3 Internal Assessment Criteria and Weight

| Internal Assessment Criteria | Weight |
| --- | --- |
| IAC0101: Understanding of cutting and laying procedures, including fabric alignment and allowances. |  |
| IAC0102: Understanding and application of sewing principles for upholstery components. |  |
| IAC0103: Selection and correct usage of tools, equipment, and machines required for cutting and sewing. |  |
| IAC0104: Recognition and rectification of faults in the cutting and sewing process. |  |
| IAC0105: Application of safety procedures and correct handling of materials. |  |
| IAC0106: Quality control checks for cut and sewn upholstery covers. |  |
| Total Weight for KM-14 | 100% |

14.4 Application in Upholstery Manufacturing

This Knowledge Module has direct application in both the prototype and production phases of furniture upholstery. Mastery of these competencies ensures learners can contribute effectively to the manufacture of high-quality upholstered products. Accurate cutting and sewing are crucial for achieving proper fit, function, and visual appeal in the final furniture item. The module also emphasises safe working practices and adherence to quality standards, which are essential in real-world upholstery production environments.

## KM-14-KT01: Lay and Cut Upholstery Covers

Weight: 50%

Purpose of the Knowledge Topic

The purpose of this Knowledge Topic is to equip learners with the theoretical and foundational knowledge required to accurately lay and cut upholstery fabrics and materials in preparation for sewing and assembly. The focus is on understanding and applying the principles and practices of cutting in a professional upholstery manufacturing environment, whether for bespoke furniture prototypes or bulk production.

The topic builds the learner's competence in interpreting fabric characteristics, using cutting plans and templates, and applying geometric and mathematical principles to optimise material utilisation. Learners will develop an understanding of the appropriate use of tools and equipment for manual and power-assisted cutting operations, and how to position and mark patterns correctly for single and multi-layer applications. Emphasis is also placed on the correct handling of fabric types and the preparation of cutting lays in a safe, efficient, and quality-focused manner.

Key Topic Elements Covered

- KT0101: Cutting patterns and templates for one-offs, multiple use, short-term use, and long-term use

- KT0102: Cutting plans

- KT0103: Cutting lays – to scale and full-size

- KT0104: Fabric types and characteristics – including patterned, stripe, repeat pattern, stretch, and pile

- KT0105: Cutting lay options – single lay and multi-lay, fabric widths and lengths, use of the cutting table, warp and weft requirements

- KT0106: Cutting lay information – fabric dimensions, quantities, template requirements, and specific cutting details

- KT0107: Cutting methods, tools, and equipment

Internal Assessment Criteria and Weight

The assessment of this Knowledge Topic is weighted at 50% of the overall module. Learners will be expected to demonstrate an understanding of:

- IAC0101: Application of mathematical and geometrical principles to sketch cutting plans while considering the characteristics and properties of different upholstery fabrics

- IAC0102: Defining the principles of preparing, measuring, laying, and securing fabric accurately on the cutting table

- IAC0103: Discussing the principles of positioning and marking patterns for both single and multi-lay cutting techniques

- IAC0104: Analysing safe and correct procedures for handling hand-operated and powered cutting tools and equipment

- IAC0105: Describing the terminology and abbreviations used for naming and marking cut upholstery cover components

Application in Upholstery Manufacturing

Proficiency in this knowledge area is fundamental to ensuring accuracy and efficiency in the preparation of fabric components used in upholstered furniture. It directly impacts material utilisation, product quality, and production timelines. Mastery of this topic allows learners to support precision cutting tasks in a variety of upholstery contexts, from handcrafted bespoke pieces to scaled industrial production environments, contributing significantly to workplace productivity and quality assurance.

### KT0101 – Cutting Patterns and Templates

Topic Element: Cutting patterns and templates – for one-offs; for multiple use; short-term use; long-term use

1. Purpose of the Topic Element

This topic element introduces learners to the different types of cutting patterns and templates used in the upholstery industry. It explores their characteristics, uses, durability, and relevance in various production contexts such as bespoke upholstery, upholstery prototyping, and large-scale manufacturing.

2. Key Concepts to Cover

| Type | Description | Typical Material | Usage |
| --- | --- | --- | --- |
| One-off | Single-use templates made for custom or prototype pieces. | Cardboard, craft paper | Bespoke designs |
| Multiple use | Durable templates for repeated use in batch or bulk production. | Plywood, plastic sheeting | Factory setting |
| Short-term use | Temporary patterns that last only a few uses. | Medium-grade card | Design development stage |
| Long-term use | Highly durable patterns retained for extended periods. | Hardboard, acrylic, PVC | High-volume standardised products |

3. Examples for Classroom Discussion

- Example 1 – One-Off Use:
A client orders a custom-shaped ottoman for a boutique hotel. The upholsterer creates a paper pattern to match the unique contours of the frame. Since it is a once-off piece, a reusable template is not necessary.

- Example 2 – Multiple Use:
A small upholstery workshop produces 15 identical dining chairs for a restaurant. A rigid plastic template is made for the seat cushion to ensure consistency and efficiency across the batch.

- Example 3 – Short-Term Use:
During the design phase of a new prototype, medium-weight card is used to experiment with shapes before committing to the final template.

- Example 4 – Long-Term Use:
A factory that manufactures recliners uses CNC-cut acrylic templates for all cushion components. These are stored in a labelled pattern library for future use.

4. Case Study: Thandi’s Upholstery Workshop

Thandi runs a small upholstery business in Durban. She recently received a bulk order for 40 identical sofa cushions for a corporate office. Initially, she used a paper template made from the first prototype. However, as the project progressed, the paper edges began fraying, resulting in inconsistencies in the cuts. A colleague suggested creating a long-lasting MDF template. Once she switched, the cutting time decreased, and quality improved.

Facilitator Reflection Questions:

- What mistake did Thandi make when selecting the original template material?

- How did changing to a longer-lasting material affect her workflow?

5. Critical Thinking Questions

- Why is it important to match the template material to the type of upholstery work (e.g., bespoke vs mass production)?

- What would be the consequence of using a worn-out pattern for a multi-lay cutting job?

- How might the selection of the wrong pattern material impact safety and efficiency in the cutting department?

- How can a pattern library support quality control in a medium-sized upholstery business?

- What sustainability concerns might arise from using short-lived materials like single-use card or paper?

6. Practical Classroom Activity

Activity: Pattern Durability Experiment

Provide learners with different materials (cardboard, paper, plastic sheet, MDF offcut). In small groups, ask them to:

- Cut out a standard shape (e.g., seat pad)

- Use it five times to trace onto fabric

- Observe wear and tear, and record:

  - Ease of use

  - Accuracy retention

  - Comfort of handling

Discussion: Which material held up best and why? Which would you use for a new range of dining chairs?

### KT0102: Cutting Plans

1. Purpose of the Topic Element

The purpose of this topic element is to enable learners to understand the principles and components of cutting plans used in upholstery production. Cutting plans are detailed guides that show how to position patterns or templates on fabric to maximise material use, ensure consistent production, and maintain the quality and alignment of components. They are essential for reducing waste and ensuring repeatability in upholstery manufacturing.

2. Key Concepts to Cover

- Definition of a Cutting Plan:
A cutting plan is a visual layout or map that guides how pattern pieces are arranged on fabric rolls before cutting begins. It includes positioning, spacing, orientation, grain direction, and fabric repeat alignment.

- Functions of Cutting Plans:

  - Optimise material usage

  - Minimise fabric waste

  - Ensure accurate pattern matching

  - Ensure correct grain direction (warp/weft alignment)

  - Maintain consistency in repeated production

- Information Typically Included in a Cutting Plan:

  - Fabric width and length

  - Pattern piece positions

  - Directional markers (e.g., pile direction, stripes)

  - Cutting sequence

  - Lay configuration (single or multi-lay)

  - Notching and alignment marks

  - Identification codes and job card reference

3. Examples for Classroom Discussion

- Example 1 – Matching Fabric Repeats:
For a floral patterned fabric, the cutting plan must allow for the repeat pattern so the flowers align across all cushions. This may require additional fabric and strategic layout to avoid visible mismatches.

- Example 2 – Avoiding Fabric Waste:
A poorly planned layout for a straight-backed chair results in many offcuts. By revising the cutting plan, rotating some symmetrical pieces, and grouping small pieces in unused gaps, the updated plan reduces waste by 15%.

- Example 3 – Accounting for Grain Direction:
A cutting plan for velvet must ensure all pieces are laid in the same pile direction. A mismatch will cause shading and a visible difference in appearance between components.

4. Case Study: Sipho’s Factory Floor Dilemma

Sipho, a junior upholsterer at a small factory in East London, is asked to prepare 10 backrest panels for a series of armchairs using a leftover roll of striped fabric. He traces each pattern piece by eye and begins cutting. During assembly, he realises that the stripes are misaligned and uneven. His supervisor then shows him how to read and follow a cutting plan that had been designed for this exact fabric.

Facilitator Reflection Questions:

- What mistake did Sipho make in his process?

- How could a cutting plan have helped him avoid material wastage and rework?

5. Critical Thinking Questions

- Why is it important to consider the width and length of the fabric roll before developing a cutting plan?

- How do cutting plans contribute to the sustainability of upholstery manufacturing?

- What risks are introduced if cutting plans are not followed or are ignored on the factory floor?

- What adjustments might need to be made to a cutting plan if a fabric defect is found mid-lay?

- In what scenarios might a digital cutting plan offer more advantages than a hand-drawn one?

6. Practical Classroom Activity

Activity: Create a Mock Cutting Plan

Provide learners with:

- Paper templates of basic upholstery pieces (e.g., armrest, seat pad, backrest)

- Fabric offcuts or brown paper representing 1.4 m width material

- Rulers, markers, and scissors

Task:

- Draw a cutting plan that fits all pieces onto the fabric as efficiently as possible.

- Consider pattern direction, symmetry, and spacing.

Presentation: Each group presents their plan and explains the logic used to minimise waste and optimise layout.

### KT0103: Cutting Lays – To Scale, Full Size

1. Purpose of the Topic Element

This topic introduces learners to the concept of cutting lays, with a focus on two methods: scaled lays and full-size lays. Cutting lays are structured arrangements of pattern pieces placed onto fabric to guide efficient and accurate cutting. Understanding how to read, prepare, and apply cutting lays helps upholsterers align material usage with design and production needs.

2. Key Concepts to Cover

| Type of Cutting Lay | Description | Application |
| --- | --- | --- |
| To Scale Lay | A scaled-down version of the layout used for planning, instruction, or illustration (e.g., 1:10 or 1:5 ratio). | Teaching, design planning, pre-cutting visualisation. |
| Full Size Lay | A life-size layout of all pattern pieces arranged on fabric or a cutting table to guide the actual cutting process. | Final layout for cutting fabric components. |

Additional Concepts:

- Accuracy: Importance of exact positioning to avoid waste.

- Grain Direction Indicators: Used to align fabric for stretch, pattern or pile.

- Marker Use: Chalk, pencils or digital projection for full-size lays.

- Manual vs. Digital: Some larger manufacturers use automated laying machines or CAD systems.

3. Examples for Classroom Discussion

- Example 1 – To Scale Lay:
In a training session, learners are shown a scaled layout of six pattern pieces arranged on a diagram of fabric. This helps them visualise how space is used efficiently before trying it full size.

- Example 2 – Full Size Lay:
On a production floor, a full-size lay is prepared for a set of ten seat covers. The cutting table is used to lay out all templates, with grain arrows aligned to the weft direction, and chalk is used to mark each component.

4. Case Study: Feroza’s Apprenticeship

Feroza is an apprentice in a custom upholstery shop in Kimberley. She is tasked with preparing a scaled cutting lay for a bespoke sofa using graph paper and small cardboard templates. Later, her mentor asks her to convert this into a full-size lay on the cutting table. Feroza forgets to account for the real fabric width, resulting in two arm pieces being cut slightly off the weft. During sewing, the pieces twist and pucker.

Facilitator Reflection Questions:

- What was the impact of not transitioning properly from a scale lay to full size?

- What lesson did Feroza learn about fabric orientation and scale accuracy?

5. Critical Thinking Questions

- What are the benefits of starting with a scale lay before doing a full-size lay?

- Why is fabric grain direction important when preparing a cutting lay?

- How might digital cutting lay software improve accuracy and reduce errors?

- What challenges might arise when preparing full-size lays in small or crowded workspaces?

- How can communication between patternmakers and cutters ensure a cutting lay is effective and efficient?

6. Practical Classroom Activity

Activity: Scale to Full Size Conversion

Materials:

- Graph paper

- Small pattern templates (paper or cardboard)

- Brown paper or fabric offcut

- Measuring tape, ruler, chalk or markers

Task:

- Step 1: In pairs, draw a 1:10 scale cutting lay of three pattern pieces on graph paper.

- Step 2: Translate the layout into a full-size lay on brown paper or fabric using correct dimensions and spacing.

- Step 3: Mark the grain line and indicate notches and labels on all pieces.

Discussion:
Compare final full-size lays for accuracy, space utilisation, and pattern alignment.

### KT0104: Fabric Types and Characteristics

(Patterned, Stripe, Repeat Pattern, Stretch, Pile)

1. Purpose of the Topic Element

The purpose of this topic element is to equip learners with an understanding of various upholstery fabric types and their defining characteristics. By exploring fabric structure and surface features such as patterns, stripes, pile, stretch, and repeats, learners will gain essential knowledge required for accurate cutting, laying, and pattern alignment. This knowledge supports high-quality upholstery outcomes and the avoidance of costly material errors.

2. Key Concepts to Cover

| Fabric Characteristic | Description | Implications for Cutting and Layout |
| --- | --- | --- |
| Patterned | Fabrics with printed or woven motifs. May be abstract, geometric, or floral. | Must align design motifs during laying. |
| Stripe | Lines running parallel to warp or weft. Can be uniform or irregular. | Directional consistency required. Misalignment is visually obvious. |
| Repeat Pattern | Design that repeats at measured intervals. | Cutting must begin at specific points to align across panels. Extra fabric may be required. |
| Stretch | Woven with elasticity (e.g., lycra blend, jersey). | Must account for stretch direction. Avoid distortion during marking and cutting. |
| Pile | Raised surface fibres (e.g., velvet, chenille). | Directional pile must be consistent to avoid colour variation. Requires careful handling. |

3. Examples for Classroom Discussion

- Example 1 – Stripe Fabric on Dining Chairs:
A set of dining chairs is upholstered in a striped fabric. If the stripe is not aligned identically on each seat, the result appears untidy and unprofessional.

- Example 2 – Repeat Pattern in Sofas:
A geometric fabric with a repeat of 30 cm requires careful measuring and additional material to ensure that the centre of the pattern is aligned across all backrest panels.

- Example 3 – Stretch Fabric on Ottomans:
A stretch fabric is selected for a round ottoman. If laid incorrectly, the fabric distorts when the piece is used, creating sagging and wrinkling.

- Example 4 – Pile Direction on Armchairs:
Velvet fabric must be laid in one direction (usually downward). If two panels are cut with opposite pile direction, they reflect light differently, appearing to be different colours.

4. Case Study: Mpho’s Velvet Order

Mpho, a junior upholsterer in Polokwane, is preparing velvet cushions for a boutique hotel. He cuts several pieces but notices that once sewn, some appear darker than others. His supervisor explains that pile direction has not been matched. Mpho learns to stroke the velvet in one direction to determine the nap, and ensures all pieces face the same way when recut.

Facilitator Reflection Questions:

- What caused the colour variation in Mpho’s cushions?

- How can this issue be prevented when working with pile fabrics?

5. Critical Thinking Questions

- What should be considered when planning the layout of a repeat-pattern fabric for a headboard?

- How might failing to observe the stretch direction affect the durability and appearance of an upholstered item?

- Why does pile direction matter, even when two fabric pieces are cut to the same shape?

- What would be the impact on production costs if the repeat pattern is not planned for in advance?

- How could you test the stretch and pile direction of a fabric before cutting it?

6. Practical Classroom Activity

Activity: Fabric Identification and Layout Planning

Materials:

- Fabric swatches: patterned, striped, stretch, pile, and plain

- Rulers, markers, and tape

- Brown paper for mock layout

Task:

- Step 1: In small groups, examine each swatch and label it with its main characteristic(s).

- Step 2: Using mock templates, draw a layout for a seat cushion and backrest using the patterned and pile fabric swatches.

- Step 3: Indicate grain, repeat alignment, and pile direction.

Presentation:
Each group explains how they would lay out and cut the pieces to maintain fabric integrity and aesthetic consistency.

### KT0105: Cutting Lay Options

1. Purpose of the Topic Element

This topic develops learners' knowledge of the various options for laying out fabric in preparation for cutting upholstery components. By understanding the distinctions between single and multi-lay setups, and how factors such as fabric width, length, and directionality (warp and weft) influence cutting accuracy and efficiency, learners will become more adept in handling diverse upholstery jobs, from custom pieces to scaled production.

2. Key Concepts to Cover

| Term | Description | Application |
| --- | --- | --- |
| Single Lay | One layer of fabric laid at a time. Allows precision in pattern matching and handling delicate or directional fabric. | Used in bespoke upholstery or when dealing with pile and patterned fabrics. |
| Multi-Lay | Several layers stacked and cut together using powered cutting tools. | Common in bulk production to save time and improve productivity. |
| Fabric Width and Length | Standard widths range from 137 cm to 280 cm. Length depends on roll size. | Influences layout choices and number of repeats that can fit. |
| Cutting Table | Flat surface used for measuring, laying, marking, and cutting fabric. Should match fabric roll width. | Ensures ergonomic working height and straight lays. |
| Warp and Weft | Warp: yarns running lengthwise. Weft: yarns running crosswise. | Grain direction affects fabric stretch, drape, and strength. Must be aligned with pattern orientation. |

3. Examples for Classroom Discussion

- Example 1 – Single Lay for Velvet:
A velvet sofa seat cover must be cut using a single lay to maintain the pile direction. Using a multi-lay method would flatten the pile and cause pile direction inconsistencies.

- Example 2 – Multi-Lay in Factory:
A furniture factory needs 100 seat pads for a commercial order. Layers of canvas are stacked and cut together using an electric blade saw. Time is saved and pieces are identical.

- Example 3 – Fabric Width Planning:
A patterned jacquard fabric is only 140 cm wide. The cutter arranges all pieces strategically on the lay to ensure the full design repeat fits, minimising waste.

4. Case Study: Samkele’s Efficiency Error

Samkele, a newly employed cutter in a Gqeberha furniture workshop, decides to stack four layers of stretch fabric to speed up cutting for an order of pouffes. Although this approach worked well for non-stretch canvas the week before, the stretch fabric slips during cutting. The resulting pieces are misaligned and misshapen, causing delays and wastage.

Facilitator Reflection Questions:

- What was the root of Samkele’s error in judgement?

- How should the properties of stretch fabrics have influenced his choice of lay method?

5. Critical Thinking Questions

- Why is multi-lay cutting unsuitable for certain fabrics such as pile or stretch textiles?

- How does the warp and weft direction influence the strength and shape of the finished product?

- In what ways can cutting tables be adapted to handle wider fabrics or large-volume projects?

- What safety considerations must be observed when cutting through multiple layers of fabric with powered equipment?

- How would you explain the importance of consistent grain direction to a new learner in the upholstery department?

6. Practical Classroom Activity

Activity: Lay Method Planning Exercise

Materials:

- Fabric offcuts of different types (e.g., stretch, pile, patterned)

- Brown paper, rulers, chalk or marker

- Paper mock templates for cushions or backrests

Task:

- In pairs, assess each fabric and decide whether to apply a single or multi-lay method.

- Draw a layout plan on brown paper, showing grain direction, orientation, and lay type.

- Explain why the selected lay method is appropriate for each fabric.

Presentation:
Groups present their layouts and justify their choices based on the fabric characteristics and expected product outcome.

### KT0106: Cutting Lay Information

1. Purpose of the Topic Element

This topic provides learners with the ability to read, interpret, and apply detailed cutting lay information. Understanding these specifications ensures accurate and efficient cutting of upholstery materials, aligned with quality, quantity, and production targets. Proper use of this information supports precision in aligning fabric characteristics, selecting suitable templates, and ensuring consistency whether working in single-lay or multi-lay setups.

2. Key Concepts to Cover

| Cutting Lay Information Component | Description | Application |
| --- | --- | --- |
| Fabric Width and Length | Refers to the usable dimensions of the fabric roll. | Determines how many pieces fit across and along the lay. |
| Patterns and Templates Required | Specific shapes or components to be cut. May be provided physically or as drawings. | Ensures correct components are included in the layout. |
| Quantities | Total number of each pattern piece needed. | Impacts decision to use single-lay or multi-lay methods. |
| Single Lay | Fabric laid in one layer for cutting. | Preferred for matching patterns or delicate fabrics. |
| Multi-Lay | Several fabric layers stacked for bulk cutting. | Suitable for repetitive shapes in large volumes. |
| Specific Cutting Details | Notes such as “align to pattern repeat”, “cut with pile down”, “mirror pair”, “leave seam allowance”. | Prevents errors during cutting and assembly. |

3. Examples for Classroom Discussion

- Example 1 – Reading Cutting Lay Instructions:
A job card instructs the cutter to “cut 6 seat covers using Pattern A, single lay, align to stripe, with 1.5 cm seam allowance.” Learners must understand and apply each instruction step-by-step.

- Example 2 – Using Fabric Width Efficiently:
A roll of fabric is 150 cm wide. Each seat cover is 48 cm wide. A layout plan allows for three pieces per width, with a small offcut margin. Learners calculate total length needed for 12 covers.

- Example 3 – Identifying Required Templates:
A cutting lay includes reference to "Template B1 – Backrest outer" and "Template B2 – Backrest inner." Mixing these up results in poor fit during upholstery.

4. Case Study: Anathi’s Cutting Confusion

Anathi is new to the upholstery team at a workshop in George. She is given a cutting lay with fabric details, template codes, and quantity requirements. However, she overlooks the note stating “mirror left and right armrest panels.” During assembly, the left and right covers are mismatched, wasting five metres of expensive fabric.

Facilitator Reflection Questions:

- What critical detail did Anathi miss in the cutting lay?

- What checks should she have done before beginning to cut?

5. Critical Thinking Questions

- Why is it important to calculate total fabric needed using both width and length specifications before cutting begins?

- What are the risks of not following specific cutting instructions, such as seam allowances or directionality?

- How does recording quantities on the cutting lay improve production control and accountability?

- Why might single-lay instructions be used even for high-volume jobs?

- What system would you recommend to organise and track multiple templates used in one cutting lay?

6. Practical Classroom Activity

Activity: Cutting Lay Interpretation Exercise

Materials:

- Mock cutting lay sheets with job card-style instructions

- Brown paper and fabric offcuts

- Paper templates (labelled A, B, C)

- Measuring tape, marker pens

Task:

- In groups, read and interpret a provided cutting lay

- Identify which templates are needed, how many of each, and the correct orientation

- Draw a section of the layout to scale, including directional arrows and seam allowances

Presentation:
Each group presents their interpretation and layout. Peers and facilitator verify accuracy and adherence to specifications.

### KT0107: Cutting – Methods, Tools, and Equipment

1. Purpose of the Topic Element

This topic equips learners with knowledge of the various cutting methods used in upholstery production, as well as the correct use and care of the associated hand and powered cutting tools. Understanding the function, safe handling, and maintenance of cutting equipment is essential for accurate, efficient, and hazard-free cutting of upholstery fabrics and materials in both bespoke and industrial contexts.

2. Key Concepts to Cover

| Category | Description | Common Examples |
| --- | --- | --- |
| Cutting Methods | Techniques used to separate fabric pieces according to template outlines. | Freehand cutting, cutting with templates, stack cutting. |
| Hand Tools | Manually operated tools used for precision cutting. | Tailor’s shears, utility knives, rotary cutters, chalk scissors. |
| Powered Tools | Electrically driven tools for speed and efficiency, especially with multi-lay fabric. | Electric rotary cutters, straight blade machines, band knives. |
| Safety Equipment | Items that protect users during cutting tasks. | Cutting gloves, safety glasses, blade guards. |
| Maintenance | Cleaning and sharpening tools to ensure accurate and smooth cutting. | Regular oiling, sharpening shears, replacing blades. |

3. Examples for Classroom Discussion

- Example 1 – Hand Cutting with Shears:
Used when precision is needed, especially for delicate or expensive fabrics. Requires skill to maintain straight lines and smooth edges.

- Example 2 – Electric Rotary Cutter in Multi-Lay Cutting:
Cuts through up to ten layers of non-stretch fabric quickly and uniformly. Requires proper alignment and pressure control.

- Example 3 – Using a Template with a Utility Knife:
Templates are placed over the fabric, and the cutter uses a sharp knife to follow the shape. Ideal for straight lines and tight corners.

4. Case Study: Nomvula’s First Week in the Cutting Room

Nomvula, a learner in an upholstery workshop in Mpumalanga, is asked to assist in cutting pile fabric using an electric rotary cutter. She starts cutting immediately without testing the blade or checking the pressure. The blade snags on the third layer, leaving a jagged edge. Her supervisor explains that the blade was dull and the speed too fast for pile fabric. Nomvula is taught how to test blade sharpness and adjust the tool for different fabric types.

Facilitator Reflection Questions:

- What were the safety and quality risks in Nomvula’s cutting?

- What procedures should she have followed before beginning the task?

5. Critical Thinking Questions

- Why is it important to match the cutting tool to the type and thickness of the fabric?

- How do blunt blades affect the quality and safety of cutting operations?

- What are the advantages and disadvantages of hand tools compared to powered cutting tools?

- What health and safety protocols must be observed when using an electric cutter?

- How can maintenance logs support tool safety and operational efficiency in a workshop?

6. Practical Classroom Activity

Activity: Tool Identification and Demonstration

Materials:

- Range of cutting tools (shears, rotary cutters, utility knives)

- Samples of fabric (canvas, velvet, stretch fabric)

- PPE (gloves, glasses)

Task:

- Learners rotate between stations, identifying each tool and practicing a supervised cut.

- At each station, they record:

  - Tool name and function

  - Fabric type used

  - Any challenges faced

- Demonstrator shows how to clean, sharpen, and store each tool safely.

Discussion:
Learners reflect on which tool gave the most control, cleanest cut, or presented the greatest safety concern and why.

## Facilitator Assessment Briefing: KM-14-KT01 – Lay and Cut Upholstery Covers (50%)

Assessment Purpose:

To assess the learner’s knowledge and application of cutting principles for upholstery fabric. The focus is on accurate planning, laying, cutting, and marking of cover components, applying both theoretical understanding and practical competence.

Assessment Context:

This assessment should be conducted in a simulated or actual upholstery cutting environment, using real or mock materials. Learners should have access to tools, templates, and a range of fabric samples (including patterned, pile, and stretch).

Assessment Instruments Used:

- Written Knowledge Test (Short Answer and Labelling) – to assess mathematical principles, terminology, and marking standards

- Practical Demonstration – to assess preparation, laying, and safe tool use

- Scenario-Based Question – to assess reasoning and application of lay selection and tool choice

- Observation Checklist and Oral Questions – during practical tasks

Pre-Assessment Preparation:

- Ensure cutting tables, templates, tools, and fabrics are available

- Safety brief must be conducted before practical activities

- Learners may work individually or in pairs where appropriate

- Allow sufficient time for both planning and execution of tasks

Internal Assessment: KM-14-KT01 (50%)

IAC0101: Application of maths and geometry in sketching cutting plan

Instrument: Short-answer + Sketch Task

Question:
Sketch a cutting plan on A4 paper for 3 seat covers using a fabric 140 cm wide. Each cover is 45 cm wide and 50 cm long. Indicate grain direction and seam allowances of 2 cm.

Model Answer:

- Correct layout with three panels aligned across fabric width

- Grain direction arrows all facing the same way

- Labels for 2 cm seam allowances on all sides

- Mathematical spacing totals: 3 × 45 cm + allowances = within 140 cm

Mark Allocation:

- Sketch layout: 3 marks

- Mathematical accuracy: 3 marks

- Grain direction indication: 2 marks

- Seam allowance marking: 2 marks
Total: 10 marks

IAC0102: Define principles of preparing, measuring, laying and securing fabric

Instrument: Practical Demonstration + Oral Questions

Task:
Demonstrate how to prepare, measure, lay and secure striped fabric on a cutting table. Answer oral questions about why each step is necessary.

Checklist Criteria (10 marks):

- Table surface clean and cleared – 1

- Fabric aligned straight and smoothed – 2

- Use of weights or clamps to secure fabric – 2

- Measurement confirmed using tape/ruler – 2

- Oral explanation of importance of accurate prep – 3

Model Oral Points:

- Prevent shifting and distortion

- Ensures straight grain alignment

- Maximises fabric use and quality

IAC0103: Positioning and marking of patterns for single and multi-lay cutting

Instrument: Scenario-Based Question

Scenario:
You need to cut 10 identical cushion covers from plain canvas. You have 5 metres of 150 cm wide fabric. Would you use a single or multi-lay? Explain your choice. Then describe how you would position and mark the templates.

Model Answer:

- Choose multi-lay due to quantity and fabric type (5 marks)

- Position templates aligned to grain and butt-jointed to reduce waste (3 marks)

- Mark outlines with chalk or tailor's pencil, using template labels and notches (2 marks)

Total: 10 marks

IAC0104: Safe and correct handling of hand and powered cutting tools

Instrument: Practical Demonstration + Oral Questions

Task:
Demonstrate use of tailor’s shears and rotary cutter to cut two seat pad shapes. Explain the precautions taken before and during cutting.

Checklist (10 marks):

- Shears: clean, sharp, used away from body – 3

- Rotary cutter: tested, guard used, cut with steady motion – 3

- Fabric held securely while cutting – 2

- Oral safety explanation – 2

Model Oral Points:

- Check blades before use

- Always cut on a protected surface

- Keep hands clear of blade path

- Store tools safely after use

IAC0105: Terminology and abbreviations for marking cut components

Instrument: Short Answer + Labelling Activity

Task:
Define the following and label them on a cut fabric mock-up:

- Grain line

- Notch

- Seam allowance

- Mirror image

- Pattern code

Model Answers:

- Grain line: line indicating direction of fabric grain

- Notch: small cut for alignment

- Seam allowance: extra fabric around edge for joining

- Mirror image: reversed piece for left/right side

- Pattern code: identifier on template or cut piece

Labelling (5 marks) + Definitions (5 marks) = 10 marks

Marking Memo (Total = 50 Marks)

| IAC Code | Task Description | Total Marks |
| --- | --- | --- |
| IAC0101 | Sketch and apply geometry to cutting plan | 10 |
| IAC0102 | Prepare, lay, and secure fabric | 10 |
| IAC0103 | Scenario reasoning and marking technique | 10 |
| IAC0104 | Safe use of cutting tools | 10 |
| IAC0105 | Terminology and component labelling | 10 |
| Total |  | 50 Marks |

Assessment Rubric: KM-14-KT01 – Lay and Cut Upholstery Covers

| Rating | Descriptor |
| --- | --- |
| 5: Excellent | Demonstrates full understanding and application with no errors. All actions precise and well-reasoned. Safe and efficient technique. |
| 4: Competent | Meets all criteria with minor errors or omissions. Demonstrates good understanding and acceptable accuracy. |
| 3: Partially Competent | Understands basic principles but shows gaps in execution or explanation. Some inaccuracies present. |
| 2: Limited Competence | Demonstrates minimal understanding. Requires supervision and support to complete tasks. Safety concerns noted. |
| 1: Not Yet Competent | Unable to perform task. Multiple errors, lack of understanding, and/or unsafe practices observed. |

## Integrated Assessment: KM-14-KT01 – Lay and Cut Upholstery Covers (50%)

NQF Level 2 | Credits: Part of 10-credit Knowledge Module | Weight: 50%

Instructions to the Learner

- Read all instructions carefully before beginning each section.

- You will complete a combination of written, practical, and scenario-based tasks.

- You may ask your facilitator for clarification if needed.

- All practical tasks must be demonstrated under supervision.

- Marks are allocated per task and the pass mark is 70% (35 out of 50).

IAC0101: Apply Maths and Geometry to Sketch a Cutting Plan

Total: 10 Marks | Written Task

Task:
Sketch a cutting plan on the grid below for three (3) seat covers, each measuring 45 cm wide x 50 cm long, using a fabric 140 cm wide. Show grain direction, spacing, and add a 2 cm seam allowance on each side.

📐 Use the space below to draw your sketch:

📄 Grain Direction Label: _____________________________
✏️ Seam Allowance Indicated? ☐ Yes ☐ No

IAC0102: Define Principles of Preparing, Measuring, Laying and Securing Fabric

Total: 10 Marks | Practical Task + Oral Questions

Task:
Demonstrate how to prepare, measure, lay, and secure striped fabric on a cutting table. You will be assessed on your technique and oral explanation of each step.

✅ Tick off as you demonstrate:

| Step | Completed |
| --- | --- |
| Cleaned and cleared the cutting surface | ☐ |
| Aligned fabric with grain on table | ☐ |
| Smoothed and secured fabric (weights/clamps) | ☐ |
| Measured using ruler or tape | ☐ |

Oral Questions:

- Why is it important to lay fabric flat and straight?

- What happens if fabric is not properly secured?

- What tools can be used to help hold fabric in place?

✍️ Facilitator to write brief learner responses:

IAC0103: Discuss Principles of Positioning and Marking for Single and Multi-Lay Cutting

Total: 10 Marks | Scenario-Based Task

Scenario:
You need to cut 10 identical cushion covers using plain canvas fabric that is 150 cm wide and 5 metres long.

Question 1:
Would you use a single-lay or multi-lay cutting method? Explain your answer.

✍️ Your response:

Question 2:
Describe how you would position and mark the templates for cutting. Include what tools you would use.

✍️ Your response:

IAC0104: Analyse Safe and Correct Use of Hand and Powered Cutting Tools

Total: 10 Marks | Practical Demonstration + Oral Questions

Task:
Demonstrate how to use:

- Tailor’s shears (manual)

- Rotary cutter (powered)

You will be assessed on:

- Safe handling of each tool

- Cutting technique

- Care before, during, and after use

✅ Tick off actions demonstrated:

| Safety Action | Done? |
| --- | --- |
| Checked blade sharpness before cutting | ☐ |
| Used guard (rotary cutter) | ☐ |
| Held fabric securely | ☐ |
| Kept hands away from blade | ☐ |

Oral Questions:

- What must you check before using a rotary cutter?

- How should cutting tools be stored after use?

✍️ Facilitator to record learner response:

IAC0105: Describe Terminology and Abbreviations for Marking Cut Cover Components

Total: 10 Marks | Written + Labelling Task

Instructions:
Define the following terms, then label them on the mock cut piece or diagram provided by the facilitator.

| Term | Definition |
| --- | --- |
| Grain Line | ______________________________________ |
| Notch | ______________________________________ |
| Seam Allowance | ______________________________________ |
| Mirror Image | ______________________________________ |
| Pattern Code | ______________________________________ |

✂️ Label the following on the mock-up or diagram: ☐ Grain line ☐ Seam allowance
☐ Notch ☐ Pattern code ☐ Mirror indication

✅ Total Marks: ______ / 50

✔️ Minimum Pass Mark: 35/50

## KM-14-KT02: Cover Making for Upholstered Prototypes – Sewing of Components

NQF Level 2 | Weight: 50%

Purpose of the Knowledge Topic

The purpose of this Knowledge Topic is to develop foundational theoretical knowledge and understanding related to the sewing processes used in constructing upholstery covers for furniture prototypes. This topic focuses on the different machine types, stitching styles, and key operational considerations involved in assembling cut fabric components into complete upholstery covers.

Learners are introduced to the core elements of cover-making, including the identification and application of appropriate machines, stitch types, and attachments. Additionally, the topic addresses fault detection in consumables and sewing processes, ensuring learners can identify quality issues and understand their implications. An important emphasis is also placed on workstation housekeeping, highlighting the impact of cleanliness, ergonomics, and safety on the quality and consistency of the final product.

Key Topic Elements Covered

- KT0201: Upholstery cut components and parts

- KT0202: Sewing machines – types, functions, and capabilities

- KT0203: Different types and styles of stitching

- KT0204: Sewing machine parts and attachments – identification and use

- KT0205: Flaws and defects in consumables and sewing – including raw material, process, and product faults

- KT0206: Quilting methods

- KT0207: Sewing work station housekeeping

These elements form the theoretical base upon which practical application of cover construction is built, forming a critical bridge between cutting processes and final upholstery assembly.

Internal Assessment Criteria and Weight

This Knowledge Topic contributes 50% to the overall Knowledge Module weighting. The assessment will focus on the following criteria:

- IAC0201: Identification of machine types including lockstitch, overlock, and multi-needle industrial sewing machines, with explanations of their functions

- IAC0202: Identification and explanation of different types and styles of stitching and their functions

- IAC0203: Identification of sewing machine parts and attachments, with explanation of their uses and functions

- IAC0204: Identification and analysis of flaws and defects in consumables and sewing (raw material, process, and product faults), with explanation of the impact on production, sales, and product usage

- IAC0205: Description of sewing work station housekeeping practices in terms of safety, ergonomics, and contamination control

Application in Upholstery Manufacturing

Competence in this topic ensures learners are capable of constructing high-quality upholstery covers using appropriate methods and tools. Whether working with bespoke prototype furniture or preparing for bulk upholstery production, learners will be able to operate industrial sewing equipment, produce accurate and durable seams, detect and respond to defects, and maintain a clean, safe, and organised work area. This contributes significantly to overall product quality, production efficiency, and customer satisfaction in the furniture upholstery environment.

### KT0201: Upholstery Cut Components and Parts

1. Purpose of the Topic Element

The purpose of this topic is to introduce learners to the various fabric components that are cut, assembled, and sewn during the cover-making process for upholstered furniture prototypes. Understanding these components, their function, and their correct identification is critical for accurate assembly, proper fit, and a high-quality finish. This topic lays the foundation for stitching operations by ensuring that learners can visually and conceptually differentiate between the cut parts of an upholstery cover.

2. Key Concepts to Cover

| Component | Description | Typical Application |
| --- | --- | --- |
| Seat Panel | Main top surface of a seat | Flat seat cushions |
| Backrest Panel | Vertical surface against which the back rests | Sofas, dining chairs, armchairs |
| Side Panel / Boxing | Vertical side band that connects top and bottom pieces | Ottomans, cushions |
| Arm Cover | Component that covers the chair or sofa arm | Fixed or removable arms |
| Base Skirt | Decorative or functional panel at the bottom of a frame | Covers bases or legs |
| Piping Strip | Narrow strip often sewn into seams for definition | Decorative edging |
| Zipper Panel / Closure Panel | Section containing zips or hook-and-loop fasteners | Access panels for covers |
| Lining / Undercover | Internal or underside fabric | Dust covers, base lining |

3. Examples for Classroom Discussion

- Example 1 – Dining Chair Components:
A dining chair cover consists of a backrest panel, a seat pad cover, and a zipped skirt panel. Each must be correctly oriented during sewing.

- Example 2 – Cushion Construction:
A scatter cushion includes a front panel, a back panel (often with a zip), and a boxing strip around the edges. If the boxing is cut too narrow, the cushion will not have the correct thickness.

- Example 3 – Piping Error:
In a prototype sofa, piping strips were omitted from the arm seams, resulting in a saggy and undefined look. Adding properly cut piping components corrected the shape and added detail.

4. Case Study: Zanele's First Prototype

Zanele, an upholstery trainee in a Cape Town studio, is tasked with assembling a removable seat cover for a custom armchair. She receives a bundle of cut fabric labelled "Seat Top," "Boxing A," and "Zip Panel B," but confuses the boxing and zip pieces during pinning. The result is a twisted, uneven cover. After feedback, Zanele learns to label components clearly and compare shapes before sewing.

Facilitator Reflection Questions:

- What could Zanele have done to avoid the error?

- How does accurate identification of cut components affect the final product?

5. Critical Thinking Questions

- Why is it important to correctly label and identify each cut upholstery component before sewing?

- How do boxing strips contribute to the shape and fit of a cushion or seat?

- What happens if the front and back panels of a cushion are mismatched in size or grain direction?

- In what ways can poor understanding of component parts delay production?

- How does the inclusion or exclusion of decorative components like piping impact design and durability?

6. Practical Classroom Activity

Activity: Component Identification and Matching

Materials:

- A set of pre-cut fabric mock-up components

- Labels (A, B, C, etc.)

- Templates or diagrams showing each component’s intended position

Task:

- Learners work in groups to match fabric pieces with diagrams of a prototype chair or cushion

- Each learner labels the cut component and states its function (e.g., "Arm panel – covers left arm of chair")

- Learners discuss what would happen if that component were cut upside-down or mislabelled

Follow-Up Discussion:
How did the team decide which piece belonged where? What role did fabric direction, shape, and markings play in the identification?

### KT0202: Sewing Machine Types, Functions and Capabilities

1. Purpose of the Topic Element

This topic introduces learners to the main types of sewing machines used in the upholstery industry. Understanding the function and capability of each machine type is vital for selecting the appropriate machine for specific sewing tasks. Learners will also gain insight into the differences between domestic and industrial machines, and how the choice of machine affects speed, stitch quality, and product durability in upholstered furniture manufacturing.

2. Key Concepts to Cover

| Machine Type | Function | Typical Use in Upholstery |
| --- | --- | --- |
| Lockstitch Machine (Single Needle) | Creates a straight stitch using upper and lower threads | Joining panels, seams, zips |
| Overlocker / Serger | Trims and encloses fabric edges while stitching | Edge finishing to prevent fraying |
| Walking Foot Machine | Moves top and bottom fabric layers evenly | Sewing multiple layers, textured or slippery fabrics |
| Double-Needle Machine | Produces parallel rows of stitching | Decorative seams, strength stitching |
| Multi-Needle Machine | Allows several rows of stitching simultaneously | Quilting or repetitive decorative effects |
| Zigzag Machine | Makes zigzag stitches | Used for stretch seams or reinforcing points |
| Cylinder Bed Machine | Allows stitching around curved or tubular shapes | Upholstered arms, piping on round cushions |

3. Examples for Classroom Discussion

- Example 1 – Walking Foot Machine:
When sewing thick leatherette seat panels, a walking foot machine ensures both layers are fed evenly, preventing slippage or uneven stitching.

- Example 2 – Overlocker Use:
After assembling a backrest cover, an overlocker is used to neaten raw edges and reduce the risk of seam splitting during use.

- Example 3 – Zigzag Stitch for Reinforcement:
A zigzag machine is used at corners of a cushion zip to strengthen the area and prevent fraying due to stress.

4. Case Study: Luthando’s Machine Confusion

Luthando is working at a factory in East London producing upholstered dining chairs. He uses a basic lockstitch machine to attach foam-backed vinyl to the chair's base. The vinyl sticks to the machine bed and feeds unevenly. His supervisor introduces him to the walking foot machine, which is better suited to layered and sticky materials.

Facilitator Reflection Questions:

- Why was the lockstitch machine unsuitable for Luthando’s task?

- How did the walking foot machine improve his results?

5. Critical Thinking Questions

- What would happen if you tried to sew foam-backed fabric on a standard domestic machine?

- Why is it important to match the machine type to the fabric and stitch type?

- In what situations would you choose a double-needle machine over a single-needle machine?

- How does machine capability affect production time and finish quality?

- What risks arise from using the wrong machine for curved components such as chair arms or bolsters?

6. Practical Classroom Activity

Activity: Machine Type Demonstration and Matching Exercise

Materials:

- Access to several types of industrial machines or video demonstrations

- Fabric swatches (leatherette, canvas, quilted material, stretch fabric)

- Machine cards with images and names

Task:

- Learners observe demonstrations or handle machine samples

- Each group matches a fabric type and sewing task to the correct machine

- Groups present their reasoning: “We chose the walking foot machine for canvas piping because...”

Optional Worksheet Extension: Fill in a table describing:

- Machine name

- Main function

- One advantage

- One limitation

### KT0203: Different Types and Styles of Stitching

1. Purpose of the Topic Element

This topic introduces learners to the different types and styles of stitches used in the manufacture of upholstery covers. Each stitch type serves a functional and sometimes decorative purpose. Learners will understand where and why each stitch is used, how stitch selection affects seam strength and aesthetics, and how incorrect stitching can compromise the quality of upholstered furniture.

2. Key Concepts to Cover

| Stitch Type / Style | Description | Function / Use in Upholstery |
| --- | --- | --- |
| Straight Stitch | A basic lockstitch using single needle | Joins panels together – most common stitch |
| Top Stitch | Visible stitch sewn on top of seams | Reinforcement and decorative detailing |
| Double Stitch / Twin Needle | Two parallel rows stitched simultaneously | Strengthens seams and adds visual detail |
| Zigzag Stitch | Side-to-side stitch pattern | Allows stretch, prevents fraying, reinforces corners |
| Overlock Stitch | Looping stitch that binds edges | Neat finish and fray protection |
| Box Stitch | Reinforced square or rectangular stitch | High-stress points, e.g., corners or straps |
| French Seam | Double-folded seam with no visible raw edge | Decorative and durable – rarely used in industrial upholstery |
| Piping Stitch | Stitching applied over corded piping | Used on cushions, arms, and edges for form and finish |

3. Examples for Classroom Discussion

- Example 1 – Top Stitching for Detail:
A modern lounge chair uses visible top stitching in a contrasting colour to create a high-end finish while reinforcing seams.

- Example 2 – Overlocking to Prevent Fray:
Raw edges of a cotton canvas cushion are overlocked before assembly to reduce fraying during use and washing.

- Example 3 – Piping Stitch for Structure:
A bolster cushion is edged with piping to define its cylindrical shape and reinforce the seam under tension.

4. Case Study: Tsepo’s Stitch Selection Error

Tsepo, an apprentice upholsterer in Bloemfontein, is assembling backrest covers for restaurant booths. He uses a zigzag stitch to join two large panels. After installation, the seams appear wavy and unstable. His supervisor explains that a straight stitch with reinforced top stitching would have been more suitable for the high-tension vertical panels.

Facilitator Reflection Questions:

- What went wrong in Tsepo’s choice of stitch?

- How does stitch selection affect seam durability in high-use environments?

5. Critical Thinking Questions

- Why might a top stitch be used in addition to a straight stitch in upholstery?

- What factors influence whether a zigzag or straight stitch is more appropriate?

- What types of stitching would be appropriate for a removable scatter cushion cover? Why?

- How could decorative stitching improve or harm a product’s market appeal?

- What stitch types are best suited for synthetic leather versus heavy woven fabrics?

6. Practical Classroom Activity

Activity: Stitch Sample Board

Materials:

- Fabric swatches (canvas, vinyl, stretch fabric)

- Sewing machines set up with different stitch settings

- Thread in contrasting colours

Task:

- Each learner sews a sample of the following: straight stitch, top stitch, overlock, zigzag, and piping stitch.

- Learners mount their stitched samples on a labelled board or worksheet.

- Below each stitch, they write:

  - Stitch name

  - Use case

  - Fabric suitability

  - Decorative or functional or both

Discussion:
Which stitches were easiest to execute? Which gave the cleanest finish? How might fabric type influence your stitch choice?

### KT0204: Sewing Machine Parts and Attachments – Identification and Their Uses

1. Purpose of the Topic Element

The purpose of this topic is to enable learners to correctly identify the essential parts and attachments of industrial sewing machines and to understand their individual functions. Proper use of these parts and accessories ensures the safe operation, maintenance, and adaptability of machines for various upholstery sewing tasks. This foundational knowledge supports learners’ ability to operate machines confidently and troubleshoot basic sewing issues.

2. Key Concepts to Cover

| Part / Attachment | Function / Description | Typical Use in Upholstery |
| --- | --- | --- |
| Presser Foot | Holds fabric flat while sewing | Variants include zipper foot, piping foot |
| Needle | Penetrates fabric to form stitch with bobbin | Comes in various sizes and types (e.g., leather, stretch) |
| Bobbin and Bobbin Case | Holds lower thread | Must be correctly wound and inserted |
| Feed Dogs | Metal teeth that pull fabric through machine | Controls fabric movement and stitch length |
| Tension Discs / Regulator | Controls thread tightness | Crucial for balanced stitching |
| Thread Guides | Direct upper thread through machine path | Prevent tangling and looping |
| Throat Plate / Needle Plate | Sits under presser foot with slot for needle | Often marked with seam guides |
| Hand Wheel | Manually raises and lowers needle | Used for precise control |
| Stitch Selector / Lever | Allows adjustment of stitch length or type | Controls density and type of stitch |
| Special Attachments | Used for specific sewing functions | Piping foot, walking foot, quilting guides, zipper foot |

3. Examples for Classroom Discussion

- Example 1 – Zipper Foot in Use:
When inserting a zip into a cushion cover, a standard presser foot is too wide. A narrow zipper foot allows the stitching line to run close to the teeth.

- Example 2 – Incorrect Tension:
If the top thread tension is too tight, the seam puckers. If too loose, loops appear on the underside. Adjusting the tension discs restores proper balance.

- Example 3 – Piping Foot Attachment:
When stitching corded piping into a cushion seam, the piping foot helps hold the cord in place and guides a clean, close stitch.

4. Case Study: Karabo’s Puckered Panels

Karabo is preparing multiple cushion covers for a client. After assembling them, she notices the top fabric is puckering while the lining remains flat. Her facilitator identifies that she used the incorrect presser foot and the top tension was too tight for the heavy fabric. After switching to a walking foot and adjusting tension, her stitching becomes balanced and smooth.

Facilitator Reflection Questions:

- What part(s) caused the stitch distortion in Karabo’s project?

- How did the attachment choice influence the fabric feeding?

5. Critical Thinking Questions

- Why is it important to match the needle type and size to the fabric being sewn?

- How does incorrect threading or tension affect the final stitch appearance and strength?

- What problems could arise if the bobbin is inserted incorrectly or wound unevenly?

- Which special attachment would you use to sew piping around a curved armrest? Why?

- How does proper knowledge of machine parts contribute to workplace safety and productivity?

6. Practical Classroom Activity

Activity: Sewing Machine Parts and Function Matching

Materials:

- Industrial sewing machine(s)

- Laminated cards with part names and functions

- Common attachments: zipper foot, piping foot, walking foot

- Worksheet with a blank diagram of a sewing machine

Task:

- Learners work in pairs to match parts and attachments to the correct location on a real or illustrated machine

- Then, using the worksheet, they label the parts and describe the function of five key components

Extension Task:

- Select two attachments and describe a real-world upholstery task where each would be used

Discussion:
Which part or attachment was new to you? How might using the wrong attachment affect your sewing outcome?

### KT0205: Flaws and Defects of Consumables and Sewing (Raw Material Faults, Process Faults, Product Faults)

1. Purpose of the Topic Element

This topic introduces learners to common faults and defects that can occur at various stages of the upholstery sewing process. These may stem from the raw materials used (fabric, thread, zips), the process of sewing itself (machine setup, stitching technique), or may be visible in the final product (misalignment, seam failure). Understanding how to identify and prevent these issues is essential for ensuring that finished upholstery products meet quality standards and are fit for purpose.

2. Key Concepts to Cover

| Category | Type of Fault / Defect | Description and Example |
| --- | --- | --- |
| Raw Material Faults | Fabric flaws | Runs, holes, shading, misprints, inconsistent stretch (e.g. stripes not aligned across bolt) |
|  | Thread quality | Fraying, uneven thickness, snapping thread |
|  | Zips and fasteners | Misaligned teeth, weak pullers, broken stops |
| Process Faults | Uneven stitch length | Due to incorrect tension, feed dog problems, or improper machine calibration |
|  | Puckering | Often from poor tension or incorrect thread/fabric combination |
|  | Skipped stitches | Result of worn needles or incorrect needle type |
|  | Thread loops | Incorrect threading or bobbin issues |
| Product Faults | Misaligned panels | Causes asymmetry, affects aesthetics |
|  | Weak seams | Causes tearing during use or cleaning |
|  | Unfinished edges | May result in fraying and product failure over time |
|  | Inconsistent piping or topstitching | Affects visual quality and product value |

3. Examples for Classroom Discussion

- Example 1 – Raw Fabric Fault:
A batch of polyester canvas has visible weft lines that differ in colour. When panels are cut, the seat covers appear mismatched. Proper inspection before cutting would have identified the inconsistency.

- Example 2 – Process Fault:
A student sews cushion covers using poor tension settings, causing puckering. When the cushion is stuffed, the seams pull unevenly and distort the shape.

- Example 3 – Product Fault:
A backrest panel is topstitched unevenly. Although functional, the aesthetic defect results in customer dissatisfaction and the product is returned.

4. Case Study: Norma’s Return Complaint

Norma works in a small upholstery business in Port Elizabeth. A client returns a pair of armchair covers, complaining that one is tighter and puckered while the other fits smoothly. On inspection, Norma notices that the tighter cover was made from a piece of stretch fabric cut on the cross-grain and stitched with uneven tension.

Facilitator Reflection Questions:

- What raw material and process faults contributed to this product defect?

- How could these issues have been identified earlier?

5. Critical Thinking Questions

- Why is it important to inspect fabric and fasteners before sewing begins?

- How do skipped stitches or loops impact the strength and appearance of seams?

- What preventative measures can be taken to avoid product faults like misalignment and weak seams?

- What might be the cost implications of failing to identify and correct defects before dispatching a product?

- How does attention to detail during the sewing process contribute to customer satisfaction and brand reputation?

6. Practical Classroom Activity

Activity: Fault Identification and Troubleshooting Exercise

Materials:

- Sample panels with pre-prepared faults (e.g. puckering, misaligned stripes, loose threads, broken zips)

- A checklist or table for categorising defects

Task:

- Learners work in pairs to inspect each sample

- For each, identify:

  - Type of fault (raw material / process / product)

  - Cause (if known)

  - Suggested correction or prevention

Presentation:

- Each pair presents one fault and explains how they would fix it and prevent recurrence in a production environment

### KT0206: Quilting Methods

1. Purpose of the Topic Element

This topic introduces learners to the various quilting methods used in the production of padded or decorative upholstery covers. Quilting refers to the process of stitching together two or more layers of fabric with padding or batting between them, using either functional or decorative stitch patterns. Understanding quilting methods is essential for producing products that require softness, insulation, textural appeal, or enhanced structure in upholstery design.

2. Key Concepts to Cover

| Quilting Method | Description | Common Upholstery Applications |
| --- | --- | --- |
| Straight-Line Quilting | Simple lines stitched at regular intervals through all layers | Mattress covers, modern wall panels |
| Diamond Quilting | Criss-crossed lines form diamond-shaped stitched areas | Bedheads, high-end booth seating |
| Channel Quilting | Parallel vertical or horizontal stitched channels | Armrests, automotive interiors |
| Free-Motion Quilting | Curved or organic patterns stitched manually or digitally | Custom luxury cushions, decorative panels |
| Tack-and-Jump Quilting | Points are stitched down while skipping spaces in between | Lightweight quilting for low-use items |
| Embossed/Pattern Quilting | Use of programmed machines to stitch logos, motifs, or decorative patterns | Branded furniture, showpieces |
| Trapunto Quilting | Layered technique where specific sections are raised with extra padding | Mostly decorative, used in bespoke items |

3. Examples for Classroom Discussion

- Example 1 – Channel Quilting in Booth Seating:
A restaurant booth uses vertical channel quilting on the backrest panels to provide both structure and a retro look. Stitch lines are evenly spaced and filled with thin foam.

- Example 2 – Diamond Quilting on a Headboard:
A luxury bedhead is upholstered using a large-scale diamond quilt pattern with decorative topstitching in contrasting thread.

- Example 3 – Free-Motion Quilting for Decorative Cushions:
A custom scatter cushion is quilted in swirling, organic shapes, offering a soft feel and unique appearance.

4. Case Study: Lerato’s Quilted Panels

Lerato is asked to prepare quilted seat panels for a boutique waiting area in Johannesburg. She selects straight-line quilting and adds foam between the layers. After stitching several panels, her supervisor notices the foam is bunching unevenly. Upon review, they realise she did not baste or pin the layers before stitching. After correcting this and adjusting the presser foot pressure, her panels are neat and consistent.

Facilitator Reflection Questions:

- What quilting method did Lerato use and what went wrong?

- What should be done before stitching quilt layers together?

5. Critical Thinking Questions

- What is the functional benefit of quilting in upholstered furniture?

- How does the choice of stitch pattern affect the aesthetic appeal of a product?

- Why is it important to secure all layers properly before quilting begins?

- What factors should be considered when selecting a quilting method for a seat versus a cushion?

- What role does thread thickness and spacing play in the success of a quilting method?

6. Practical Classroom Activity

Activity: Create a Quilted Sample Panel

Materials:

- Fabric top layers, batting/foam, backing fabric

- Sewing machines (preferably walking foot or quilting attachment)

- Rulers, chalk, safety pins or basting spray

Task:

- Each learner creates a 30 x 30 cm quilted sample using one of the following: straight-line, diamond, or channel quilting

- They mark their stitching lines, secure layers, and sew through all layers

- Learners mount and label their samples on a board with:

  - Quilting method used

  - Stitch length

  - Materials layered

Group Reflection:
Compare the softness, consistency, and alignment of each method. What worked well? What challenges were faced?

### KT0207: Sewing Work Station Housekeeping

1. Purpose of the Topic Element

This topic equips learners with the knowledge and habits required to maintain a clean, safe, and efficient sewing work station. Effective housekeeping reduces the risk of injury, prevents contamination of materials, improves workflow, and contributes to the overall quality of the final upholstered product. It also reinforces the professional standards expected in upholstery manufacturing environments.

2. Key Concepts to Cover

| Housekeeping Aspect | Description | Importance in Upholstery |
| --- | --- | --- |
| Cleanliness | Regular removal of thread, dust, fabric scraps | Prevents machine blockages and contamination of products |
| Tool Storage | Neatly storing scissors, needles, pins, bobbins, etc. | Prevents injuries, improves efficiency |
| Waste Management | Using designated bins for fabric and thread waste | Keeps work space clear, reduces trip hazards |
| Work Surface Maintenance | Keeping the sewing table and surrounding area clean and organised | Allows accurate fabric handling and layout |
| Machine Area Safety | Ensuring cords, pedals, and power sources are secured | Prevents electric hazards and tripping |
| Ergonomics | Proper seating height, posture, and lighting | Reduces fatigue and strain, improves accuracy |
| Labelling and Documentation | Clear labelling of cut components, job cards, and threads | Reduces confusion and errors in sewing tasks |

3. Examples for Classroom Discussion

- Example 1 – Loose Threads in Machine Base:
A learner neglects to clear thread scraps under the needle plate. The thread builds up and causes jamming during stitching, wasting time and damaging fabric.

- Example 2 – Poor Lighting and Posture:
A student strains to see needle alignment due to poor overhead lighting. After adjusting the task light and raising the chair, stitching becomes more accurate and consistent.

- Example 3 – Unsecured Foot Pedal Cable:
A foot pedal cord crosses a walkway. Another learner trips, resulting in a minor injury and halted sewing session. Re-routing the cable prevents recurrence.

4. Case Study: Farieda’s Untidy Work Station

Farieda is responsible for assembling cushion covers. Over a week, her work table becomes cluttered with bobbins, offcuts, loose threads, and unlabelled panels. She begins mixing up orders and spends more time looking for tools than sewing. After receiving feedback, she develops a routine: cleaning every 2 hours, using trays for tools, and setting up labelled bins for different offcut types.

Facilitator Reflection Questions:

- What risks did Farieda’s untidy station pose to production and safety?

- How did small changes improve her efficiency and product quality?

5. Critical Thinking Questions

- What are the risks of having loose threads, needles, and scissors scattered around the work station?

- How can poor ergonomics affect the health and productivity of a sewing operator?

- Why is it important to clean around the sewing machine and under the feed dogs regularly?

- How does labelling and storage help prevent mistakes in multi-part upholstery jobs?

- What systems can you put in place to maintain sewing station cleanliness in a high-output environment?

6. Practical Classroom Activity

Activity: Set Up and Maintain a Sewing Work Station

Materials:

- Basic workstation setup: sewing machine, fabric, tools, threads, bins

- Checklists: safety, cleanliness, tool placement

- Labels and containers for organisation

Task:

- Each learner sets up their work station, ensuring:

  - Machine area is clean and free of clutter

  - Tools are neatly stored and within reach

  - Waste bins are used and labelled

  - Ergonomic adjustments are made (chair, light, foot pedal)

- After a 1-hour sewing session, learners perform a clean-up routine

Assessment:
Learners complete a self-evaluation checklist and peer assess a partner’s station using provided criteria.

## Integrated Assessment: KM-14-KT02 – Cover Making for Upholstered Prototypes: Sewing of Components (50%)

Assessment Purpose:

To assess the learner’s theoretical understanding and practical application of sewing machines, stitching types, parts and attachments, defect identification, and workstation housekeeping, as related to the manufacture of upholstery covers.

Assessment Structure and Instruments

| Assessment Criteria | Assessment Method | Weight |
| --- | --- | --- |
| IAC0201 | Written + Practical Identification | 10 marks |
| IAC0202 | Matching Exercise + Practical Sample Stitching | 10 marks |
| IAC0203 | Labelling Activity + Oral Questions | 10 marks |
| IAC0204 | Case Study + Analysis | 10 marks |
| IAC0205 | Observation + Checklist | 10 marks |
| Total |  | 50 marks |

Assessment Tasks

Task 1 – IAC0201: Identify Machine Types and Functions (10 Marks)

Instructions: Match each machine type to its description and correct use. Then identify the machines available in the workshop.

A. Matching (5 marks):

| Machine Type | Description | Function |
| --- | --- | --- |
| A. Lockstitch | ___ | ___ |
| B. Overlocker | ___ | ___ |
| C. Walking Foot | ___ | ___ |
| D. Zigzag | ___ | ___ |
| E. Multi-needle | ___ | ___ |

Model Answers:

- A: Basic single needle, for joining panels

- B: Trims and binds raw edges

- C: Feeds multiple fabric layers evenly

- D: Creates zigzag motion, allows stretch

- E: Produces multiple parallel rows for strength/decorative effect

B. Practical Identification (5 marks):
Point out each machine in the training centre and describe one job it would be used for in upholstery.

Task 2 – IAC0202: Identify Stitch Types and Functions (10 Marks)

Instructions: Complete the matching table and produce 3 short stitch samples.

A. Matching (5 marks):

| Stitch Type | Function |
| --- | --- |
| 1. Straight Stitch | ___ |
| 2. Overlock Stitch | ___ |
| 3. Top Stitch | ___ |
| 4. Zigzag Stitch | ___ |
| 5. Piping Stitch | ___ |

Model Answers:

- 1: Basic joining of panels

- 2: Neat finish and prevents fraying

- 3: Decorative and reinforcement

- 4: For stretch and reinforcing seams

- 5: Stitching around cording on cushions or edges

B. Practical Stitch Samples (5 marks):
Learners sew short samples of three stitches using scrap fabric: straight, zigzag, and overlock.

Task 3 – IAC0203: Identify Sewing Machine Parts and Attachments (10 Marks)

Instructions: Label parts on a machine diagram and answer oral questions about selected parts.

A. Diagram Labelling (5 marks):

Parts to label:

- Presser Foot

- Needle

- Bobbin Case

- Feed Dogs

- Tension Disc

B. Oral Questions (5 marks):
For two randomly selected parts, explain:

- What is its function?

- What happens if it is not working properly?

Model Answer Examples:

- Presser Foot: Holds fabric down. Without it, fabric shifts.

- Tension Disc: Controls thread tension. Loose tension causes looping.

Task 4 – IAC0204: Identify and Analyse Sewing Defects (10 Marks)

Instructions: Read the case study and answer the questions.

Case Study: Lindiwe is sewing backrest covers for an armchair using leatherette. She notices skipped stitches and puckering. Her fabric roll has a scarred section and her stitch length is inconsistent. After finishing, the seam pulls apart during inspection.

Questions (5 marks):

- Identify one raw material fault.

- Identify one process fault.

- What is one product fault?

- How could these have been prevented?

- What is the impact on the final product?

Model Answers:

- Scarred leatherette

- Incorrect stitch length or tension

- Seam failure

- Inspect fabric, adjust machine before sewing

- Reduces product quality and causes returns

Task Extension (5 marks): Learners view actual samples with faults and classify each as raw material, process, or product fault.

Task 5 – IAC0205: Workstation Housekeeping (10 Marks)

Instructions: Set up and clean your work station, then complete a self-assessment checklist.

Checklist (10 marks):

| Action | Completed? |
| --- | --- |
| Tools neatly stored | ☐ |
| Surface clean and clear | ☐ |
| Thread/fabric waste removed | ☐ |
| Lighting adjusted for visibility | ☐ |
| Foot pedal and cables secured | ☐ |
| Chair adjusted for posture | ☐ |
| Fabric labelled and stacked | ☐ |
| No food or drinks present | ☐ |
| Bins available and used | ☐ |
| Overall workspace safe and ergonomic | ☐ |

Facilitator observes and confirms with tick sheet.

✅ Marking Memo

| IAC | Total Marks | Key Points for Assessment |
| --- | --- | --- |
| IAC0201 | 10 | Matching definitions + identifying correct machines + verbal use case |
| IAC0202 | 10 | Stitch-function understanding + correct execution of stitch samples |
| IAC0203 | 10 | Accurate labelling + functional understanding of two parts |
| IAC0204 | 10 | Fault classification + impact and prevention reasoning |
| IAC0205 | 10 | Clean, safe, ergonomic workstation with full checklist compliance |
| TOTAL | 50 marks | Minimum pass: 35 (70%) |

🧵 Assessment Rubric

| Rating | Descriptor |
| --- | --- |
| 5 – Excellent | Demonstrates full understanding and application. All tasks completed to a high standard. Uses correct terms and accurate identification throughout. |
| 4 – Competent | Meets all outcomes with minor inaccuracies. Good application of concepts. Few or no safety issues. |
| 3 – Partially Competent | Understands basic concepts but misses some connections or has some errors. Practical execution inconsistent. |
| 2 – Limited Competence | Major gaps in understanding or execution. Requires support or repeated instruction. |
| 1 – Not Yet Competent | Fails to meet outcomes. Unsafe practices or inability to perform tasks without full assistance. |

Would you like me to proceed with the Facilitator Assessment Briefing next, then generate a combined Word document including both components?

### Facilitator Assessment Briefing: KM-14-KT02 – Cover Making for Upholstered Prototypes

Purpose:

To assess theoretical knowledge and practical skills related to industrial sewing in the upholstery context. The focus is on selecting appropriate machines, applying correct stitch types, identifying key components and attachments, and recognising sewing faults while maintaining a clean and safe workstation.

Assessment Context:

This assessment must be conducted in a simulated or real upholstery sewing environment. Machines, tools, sample fabrics (including foam-backed and patterned), attachments, and faulted materials must be available.

Assessment Instruments Used:

- Written identification and matching tasks

- Practical demonstrations

- Case study analysis

- Observation checklist

Preparation Checklist:

- Ensure all required machines are working and clearly labelled

- Lay out sample fabrics, threads, and attachments

- Prepare examples of faulty materials for analysis in Task 4

- Print workstation housekeeping checklists for Task 5

- Ensure learners have adequate space and lighting to complete their tasks

Marking Guidance:

Use the provided marking memo and rubric to assess each task. Observe learners closely during practical activities and engage them with follow-up oral questions where necessary to verify understanding.
Minimum required to achieve competence: 70% (35 out of 50 marks)