UPHOLSTERY COVER FITTER

PM02 COVER PREPARED UPHOLSTERED FRAMES WITH FABRIC AND OTHER MATERIALS

LEARNER GUIDE

Table of Contents

INTRODUCTION	3

Introduction to PM-02: Cover Prepared Upholstered Frames with Fabric and Other Materials	4

PM-02, Cover Prepared Upholstered Frames with Fabric and Other Materials, NQF Level 2, Credits 20	6

PM-02-PS01: Identify different styles, types and designs of upholstered furniture	8

Practical Application Learning Content for PA0101	9

Practical Application Learning Content for PA0102	12

Practical Application Learning Content for PA0103	15

Practical Application Learning Content for PA0104	17

Practical Application Learning Content for PA0105	19

Practical Application Learning Content for PA0106	22

Applied Knowledge Learning Content for AK0101	24

GUIDELINES	27

Case Study: Practical Assessment on Upholstered Furniture Identification	29

PM-02-PS02: Identify various types of furniture frame construction and frame components using the correct terminology	33

Practical Application Learning Content for PA0201	34

Practical Application Learning Content for PA0202	36

Practical Application Learning Content for PA0203: Identify and Distinguish Between Frames According to Purpose	38

Practical Application Learning Content for PA0204: Identify and Distinguish Between the Four Back Styles of Furniture Frames	40

Practical Application Learning Content for PA0205: Identify Seating Frames According to the Three Main Types	42

Practical Application Learning Content for PA0206: Identify the Wood and Board Products Used in the Construction of the Frame(s) According to Their Application	44

Practical Application Learning Content for PA0207: Identify and Distinguish Between Joints and Bracings Used in the Frame Construction	46

Practical Learning Content for PA0208: Identify and Distinguish Between the Long, Short, and End Grains of Wood and Their Ability to Hold Staples and Nails	48

GUIDELINES	50

Case Study: New Line Development at Metropolitan Furniture Inc.	52

PM-02-PS03: Read and interpret specifications for upholstery covers	55

Practical Learning Content for PA0301: Identify the Main Components of Furniture Frames Using Industry Standard Terminology	57

Practical Learning Content for PA0302: Identify the Main Components of Covers for Furniture Frames Using Industry Standard Terminology	59

Practical Learning Content for PA0303: Identify the Type of Cover Material	61

Practical Learning Content for PA0304: Identify the Dimensions of the Cover Components	63

Practical Learning Content for PA0305: Identify the Final Size and Allowances for the Cover Components	65

Practical Learning Content for PA0306: Identify the Characteristics of Fabrics that Influence Cover Fitting	67

Practical Learning Content for PA0307: Identify the Characteristics of Cover Design that Influence Cover Construction	69

Practical Learning Content for PA0308: Identify the Construction Process of Each of the Main Groups of Furniture	71

Practical Learning Content for PA0309: Identify the Components of Each Construction	73

Applied Knowledge Learning Content for AK0301: Specification Reading Techniques Using Industry Terminology and Abbreviations	75

Applied Knowledge Learning Content for AK0302: Measuring Techniques	77

Applied Knowledge Learning Content for AK0303: Cover Material Types	79

Applied Knowledge Learning Content for AK0304: Fabric Technology and Design Aesthetics	81

GUIDELINES	83

Case Study: Refurbishment of a Vintage Armchair	85

PM-02-PS04: Identify raw materials required for cover fitting operations using the correct terminology	88

Practical Learning Content for PA0401: Interpret Specifications and Identify the Specified Prepared Frames	89

Practical Learning Content for PA0402: Interpret Specifications and Identify the Specified Prepared Cover Components for the Frames	91

Practical Learning Content for PA0403: Identify the Specified Bottom Covers for the Frames	93

Applied Knowledge Learning Content for AK0401: Numeracy and Measuring Skills	95

Applied Knowledge Learning Content for AK0402: Material Selection and Fault and Defect Identification	97

Applied Knowledge Learning Content for AK0403: Raw Material Specifications and Components for Cover Fitting	99

GUIDELINES	101

Case Study: Upholstery Project for a Boutique Hotel Chain	103

PM-02-PS05: Determine the ancillary materials required for cover fitting	106

Practical Learning Content for PA0501: Interpret Specifications and Identify the Specified Ancillary Materials	107

Practical Application Learning Content for PA0502: Inspect and Select the Specified Ancillary Materials	109

Applied Knowledge Learning Content for AK0501: Material Selection and Fault and Defect Identification	111

GUIDELINES	113

Case Study: Ancillary Materials Procurement for Elite Furniture Designs	115

PM-02-PS06: Determine the consumables required for cover fitting	117

Practical Application Learning Content for PA0601: Interpret Specifications and Identify the Quantity, Type, and Dimensions of Consumables	118

Applied Knowledge Learning Content for AK0601: Numeracy and Measuring Skills	121

Applied Knowledge Learning Content for AK0602: Consumable Selection and Fault and Defect Identification	123

GUIDELINES	125

Case Study: Launch of a New Upholstery Line at Modern Designs Inc.	127

PM-02-PS07: Identify, prepare and safely apply hand tools and equipment for fitting covers to frames	130

Practical Application Learning Content for PA0701: Interpret Specifications and Identify Hand Tools Required to Cover Prepared Frames	131

Practical Application Learning Content for PA0703: Check Hand Tools and Equipment for Defects, Sharpness, and Safety	133

Applied Knowledge Learning Content for AK0701: Tacking and Covering Techniques	135

Applied Knowledge Learning Content for AK0702: Sewing Techniques	137

Applied Knowledge Learning Content for AK0703: Faults and Maintenance	139

GUIDELINES	141

Case Study: Upholstery Workshop Readiness at Custom Comfort Furniture	143

PM-02-PS08: Identify, prepare and safely operate pneumatic tools for fitting covers to frames	146

Practical Application Learning Content for PA0801: Interpret Specifications and Identify the Sise and Type of Pneumatic Tools Required	147

Practical Application Learning Content for PA0802: Identify and Match Nails and Staples to the Correct Pneumatic Tools	149

Practical Application Learning Content for PA0803: Identify the Parts of the Tools, Disassemble and Assemble an Air Stapler and Air Gun, Load the Nails and Staples into the Correct Power and Pneumatic Tools	151

Practical Application Learning Content for PA0804: Identify and Check Safety Devices	153

Practical Application Learning Content for PA0805: Operate the Tools According to Standard Operating Procedures	155

Practical Application Learning Content for PA0806: Conduct Minor Maintenance on Pneumatic Tools	157

Practical Application Learning Content for PA0807: Store the Tools	159

Applied Knowledge Learning Content for AK0801: Compressed Air and Optimum Air Pressure Required for Various Pneumatic Tools	161

Applied Knowledge Learning Content for AK0802: Consumables	163

Applied Knowledge Learning Content for AK0803: Pneumatic Tool Maintenance	165

GUIDELINES	167

Case Study: Equipment Readiness Assessment at Elite Upholstery Services	169

PM-02-PS09: Start and stop a compressor and connect pneumatic tools while following safe standard operating procedures	172

Practical Application Learning Content for PA0901: Identify the Two Main Types of Compressor	173

Practical Application Learning Content for PA0902: Check that Planned Maintenance has Taken Place as Scheduled, and that Legal Certification is Up to Date	176

Practical Application Learning Content for PA0903: Identify and Check Safety Devices and Conduct Pre-start Checks and Procedures	178

Practical Application Learning Content for PA0904: Monitor the Oil and Air Pressure Gauges and Follow Procedures to Report Problems	180

Practical Application Learning Content for PA0905: Connect Air Lines to the Compressor or Air Outlet and Connect and Operate Pneumatic Tools at the Required Air Pressure	182

Practical Application Learning Content for PA0906: Execute Shutdown Procedures in the Correct Sequence and Clean the Area Around the Compressor	185

Applied Knowledge Learning Content for AK0901: Types of Compressors and Their Uses	187

Applied Knowledge Learning Content for AK0902: Compliance with the Occupational Health and Safety Act Regulations for Vessels Under Pressure	190

Applied Knowledge Learning Content for AK0903: Standard Safe Operating Procedures (SOPs)	193

GUIDELINES	196

Case Study: Comprehensive Equipment Review at Apex Upholstery	198

PM-02-PS10: Apply measuring techniques and calculations applicable to frame covering	201

Practical Application Learning Content for PA1001: Identify and Distinguish Between Measuring Tools	202

Practical Application Learning Content for PA1002: Use a Tape Measure and Other Measuring Equipment	204

Practical Application Learning Content for PA1003: Accurately Apply and Calculate Measuring Units Related to Covering of Furniture Frames	206

Practical Application Learning Content for PA1004: Accurately Apply Scale	208

Practical Application Learning Content for PA1005: Accurately Measure Frames and Consumables	210

Practical Application Learning Content for PA1006: Measure and Mark Covers According to the Frame (Back, Seat, and Arms)	212

Practical Application Learning Content for PA1007	214

Practical Application Learning Content for PA1008: Calculate Allowances	216

GUIDELINES	218

Case Study: Technical Drawing Development for Elite Design Upholstery	220

PM-02-PS11: Plan and prepare for covering of prepared frames	223

Practical Application Learning Content for PA1101	226

Practical Application Learning Content for PA1102: Interpret the Information in the Specifications into a Detailed Work Plan and Cutting List	229

Practical Application Learning Content for PA1103	232

Practical Application Learning Content for PA1104: Ensure Availability of Sufficient Raw Materials to Ensure Efficiency	235

Practical Application Learning Content for PA1105: Prepare the Work Area for Frame Preparation Operations	237

Practical Application Learning Content for PA1106: Read and Understand Production Cards	239

Practical Application Learning Content for PA1107: Set the Machine	242

Practical Application Learning Content for PA1108: Conduct Pre-Start-Up Checks (Operator Will Sign)	244

GUIDELINES	247

Case Study: Efficient Upholstery Project at Classic Interiors	249

PM-02-PS12: Cover a range of drop-on and drop-in seats of various shapes and sizes	252

GUIDELINES	253

Case Study: Restoration of a Set of Antique Dining Chairs	255

PM-02-PS13: Cover wedge shaped box arm seat frames, with and without corner pleats	258

GUIDELINES	260

Case Study: Reupholstery of a Wedge-Shaped Box Arm Sofa	262

PM-02-PS14: Fit covers to prepared frames using visible and concealed finishing techniques	265

GUIDELINES	267

Case Study: Reupholstery of a Mid-Century Modern Armchair	269

PM-02-PS15: Fit covers to prepared frames using relief cuts to relieve tension on the fabric and to achieve the specified finish	272

GUIDELINES	274

Case Study: Upholstering a Vintage Scroll Arm Chair	276

Case Study: Upholstering a Vintage Scroll Arm Chair	276

PM-02-PS16: Identify and inspect covered frames against specifications and repair and report defects ensuring quality	279

GUIDELINES	282

Case Study: Quality Inspection and Repair of Upholstered Furniture Frames	284

PM-02-PS17: Apply health and safety, housekeeping, ergonomics and productive processes when fitting covers to upholstery frames	287

GUIDELINES	290

Case Study: Health and Safety, Housekeeping, Ergonomics, and Productivity in an Upholstery Workshop	292

PM-02-PS18: Perform routine cleaning, quality change cleaning and housekeeping of the work area	295

GUIDELINES	298

Case Study: Maintaining a Clean and Safe Upholstery Workshop	300

PM-02-PS19: Record and report frame covering production information	303

GUIDELINES	306

Case Study: Recording and Reporting Frame Covering Production Information	309

# INTRODUCTION

The modules outlined encompass a wide range of skills and knowledge areas, essential for those pursuing a qualification in furniture upholstery. These materials will be designed to cater to the National Qualifications Framework (NQF) Level 2 to Level 4, with varying credit allocations that signify the depth and complexity of each module.

The objective of these learning materials is to provide learners with a clear, structured, and in-depth understanding of each aspect of furniture upholstery. This ranges from foundational skills such as preparing upholstered frames, to more advanced techniques like deep buttoning and reengineering furniture products. Each module has been carefully developed to ensure it aligns with the NQF standards and meets the required learning outcomes.

These modules are designed not only to impart technical knowledge and practical skills but also to foster an understanding of the broader context in which these skills are applied. This includes an appreciation of quality standards, efficiency in manufacturing processes, and the ability to troubleshoot and rectify defects.

The learning materials will comprise a mix of theoretical content, practical demonstrations, step-by-step guides, and assessments. This approach ensures that learners gain a holistic understanding of the upholstery process, from initial design and pattern making to the final stages of inspection and quality control. Emphasis will be placed on real-world application, ensuring that learners are well-equipped to apply their skills in a professional setting.

In summary, these materials aim to provide a comprehensive, detailed, and practical learning experience that will prepare learners for a successful career in furniture upholstery. The progression from NQF Level 2 to Level 4 allows for a gradual increase in complexity and depth, ensuring that learners are challenged and engaged throughout their educational journey.

# Introduction to PM-02: Cover Prepared Upholstered Frames with Fabric and Other Materials

In the skilled craft of furniture upholstery, the precise covering of prepared frames stands as a fundamental competency that transforms a simple structure into a functional and aesthetically pleasing piece of furniture. Module PM-02, a crucial part of the Furniture Upholsterer curriculum, focuses on empowering learners with the practical skills necessary to adeptly fit and attach prepared covers and loose material panels onto these frames. Utilising staples, tacks, and glue, this module not only enhances the learner’s technical prowess but also deepens their appreciation for the intricacies of upholstered furniture design.

Learners embarking on this module will journey through a series of structured practical skill sets, designed to build a comprehensive understanding from identifying different styles and construction of upholstered furniture to mastering the application of finishing techniques that meet industry standards. This module is structured to ensure that each participant can:

- Understand various styles, types, and designs of upholstered furniture.

- Become proficient in identifying furniture frame construction and its components.

- Read and interpret specifications for upholstery covers with accuracy.

- Select appropriate raw and ancillary materials necessary for cover fitting operations.

- Operate both hand tools and advanced pneumatic tools safely and efficiently.

Furthermore, this module will cover essential skills such as measuring, planning, and the practical application of health and safety standards. The learner will also gain the ability to perform routine and quality-specific cleaning tasks, maintain accurate production records, and identify and rectify defects in covered frames, ensuring a product that is not only functional but meets quality expectations.

By the completion of PM-02, learners will be equipped not only with the skills required for covering frames but also with a deepened understanding of the materials and processes that contribute to the creation of high-quality upholstered furniture. This module is pivotal for those aspiring to excel in the furniture upholstery industry, providing them with the tools and knowledge to succeed in a competitive market.

This introduction aims to set the stage for learners and stakeholders by outlining the importance of the skills they will acquire and their application in the professional field of upholstery.

# PM-02, Cover Prepared Upholstered Frames with Fabric and Other Materials, NQF Level 2, Credits 20

Purpose of the Practical Skill Modules

The focus of the learning in this module is on providing the learner an opportunity to Fit and attach prepared covers and loose material panels onto prepared upholstery frames using staples, tacks and/or glue

The learner will be required to:

- PM-02-PS01: Identify different styles, types and designs of upholstered furniture

- PM-02-PS02: Identify various types of furniture frame construction and frame components using the correct terminology

- PM-02-PS03: Read and interpret specifications for upholstery covers

- PM-02-PS04: Identify raw materials required for cover fitting operations using the correct terminology

- PM-02-PS05: Determine the ancillary materials required for cover fitting

- PM-02-PS06: Determine the consumables required for cover fitting

- PM-02-PS07: Identify, prepare and safely apply hand tools and equipment for fitting covers to frames

- PM-02-PS08: Identify, prepare and safely operate pneumatic tools for fitting covers to frames

- PM-02-PS09: Start and stop a compressor and connect pneumatic tools while following safe standard operating procedures

- PM-02-PS10: Apply measuring techniques and calculations applicable to frame covering

- PM-02-PS11: Plan and prepare for covering of prepared frames

- PM-02-PS12: Cover a range of drop-on and drop-in seats of various shapes and sizes

- PM-02-PS13: Cover wedge shaped box arm seat frames, with and without corner pleats

- PM-02-PS14: Fit covers to prepared frames using visible and concealed finishing techniques

- PM-02-PS15: Fit covers to prepared frames using relief cuts to relieve tension on the fabric and to achieve the specified finish

- PM-02-PS16: Identify and inspect covered frames against specifications and repair and report defects ensuring quality

- PM-02-PS17: Apply health and safety, housekeeping, ergonomics and productive processes when fitting covers to upholstery frames

- PM-02-PS18: Perform routine cleaning, quality change cleaning and housekeeping of the work area

- PM-02-PS19: Record and report frame covering production information

## PM-02-PS01: Identify different styles, types and designs of upholstered furniture

Scope of Practical Skill

Given a range of samples, prototypes and photographs of different styles, types and designs of upholstered furniture, including full seats, loose cushioned seats, full back seats, loose cushioned back seats, club arm seats and scroll arm seats the learner must be able to:

- PA0101 Identify and categorise the upholstered furniture product according to types, uses and styles

- PA0102 Identify and categorise the upholstered furniture products according to the raw materials used in the manufacturing process

- PA0103 Identify and categorise the accessories according to the uses and furniture design

- PA0104 Identify the main groups of upholstered seats

- PA0105 Identify the main groups of upholstered backs

- PA0106 Identify the main groups of upholstered arms

Applied Knowledge

- AK0101 Upholstered furniture types, designs and styles

Internal Assessment Criteria

- IAC0101 Photographs, samples and prototypes of upholstered furniture are correctly categorized according to the two main groups of upholstered seats

- IAC0102 Photographs, samples and prototypes of upholstered furniture are correctly categorized according to the two main groups of upholstered backs

- IAC0103 Photographs, samples and prototypes of upholstered furniture are correctly categorized according to the two main groups of upholstered arms

### Practical Application Learning Content for PA0101

Identify and Categorise the Upholstered Furniture Product According to Types, Uses, and Styles

Understanding the various types, uses, and styles of upholstered furniture is essential for professionals in the furniture upholstery industry. This knowledge allows for the accurate classification and assessment of furniture pieces, ensuring they meet both aesthetic desires and functional needs of the marketplace. Below is a structured guide to help learners develop proficiency in this area, complete with practical applications and examples.

1. Introduction to Furniture Types

Definition and Examples:

- Stationary Upholstery: These are pieces like sofas and loveseats that are not designed to move or recline. For example, a traditional Chesterfield sofa is a stationary piece characterised by its deep button tufting and high arms.

- Motion Upholstery: This category includes furniture that reclines, such as recliners, sofa beds, and lift chairs. An example is the reclining sectional that offers various angles of adjustment for enhanced comfort.

2. Understanding Furniture Uses

Explanation and Context:

- Residential Use: Furniture designed for home environments, prioritising comfort, durability, and style. A plush, overstuffed sectional sofa serves as a comfortable, inviting centrepiece in a family room.

- Commercial Use: These pieces are intended for public spaces like offices, hotels, and restaurants, where durability and easy maintenance are crucial. For example, bench seating upholstered in commercial-grade vinyl offers durability and easy cleaning for high-traffic areas.

3. Exploring Furniture Styles

Detailed Styles with Characteristics:

- Traditional Style: Features classic details, luxurious fabrics, and a refined look. For instance, a wingback chair with cabriole legs and brocade fabric upholstery is a hallmark of traditional styling.

- Modern Style: Known for its minimalist aesthetics, clean lines, and monochromatic colours. A modern style sofa might feature streamlined shapes and neutral upholstery materials.

- Contemporary Style: This style represents current trends and often combines elements from various styles. It could be exemplified by a modular sofa with bold colour choices and geometric patterns.

Practical Exercises

Exercise 1: Furniture Identification and Categorisation

- Task: Given a set of photographs featuring different furniture pieces, learners will identify and categorise each item based on type, use, and style.

- Objective: To apply theoretical knowledge in a practical scenario, reinforcing the learner’s ability to discern differences in furniture design.

Exercise 2: Style Matching

- Task: Match upholstery samples with the correct furniture style category, using swatches of fabric and images of completed furniture pieces.

- Objective: This exercise will help learners connect the dots between materials and the overall style of a furniture piece, fostering a deeper understanding of how fabric choices influence design aesthetics.

Real-World Application

Field Trip: Visit a furniture manufacturing company or a large furniture showroom. Observe the different types of furniture on display, discuss their uses with the staff, and identify their styles. This real-world exposure will cement the learner’s ability to classify and evaluate furniture effectively.

Group Discussion: Post-visit, a group discussion could be facilitated where learners share their observations and insights about the types, uses, and styles of furniture they encountered. This discussion helps consolidate knowledge and offers peer learning opportunities.

Through these practical applications, learners will not only grasp the theoretical aspects of furniture types, uses, and styles but also learn to apply this knowledge in real-world contexts, thereby enhancing their skills and confidence in furniture upholstery.

### Practical Application Learning Content for PA0102

Identify and Categorise the Upholstered Furniture Products According to the Raw Materials Used in the Manufacturing Process

Understanding the raw materials used in the manufacturing of upholstered furniture is pivotal for professionals in the industry. This knowledge not only aids in identifying the quality and suitability of furniture for specific uses but also informs decisions regarding maintenance and durability. Below, we offer a detailed exploration of how different raw materials can influence the function and aesthetic of upholstered furniture, coupled with practical exercises to enhance learning.

1. Introduction to Raw Materials in Upholstery

Types of Raw Materials:

- Fabrics: Include natural fibres (cotton, linen, wool), synthetic fibres (polyester, nylon), and leather. Each material varies in texture, durability, and care requirements.

- Support Structures: Typically made from wood, metal, or plastic. The choice of material affects the weight, durability, and cost of the furniture.

- Cushioning: Foam (polyurethane, memory foam), down, or batting are commonly used. The type of cushioning affects comfort, shape retention, and how the furniture wears over time.

- Adhesives and Other Fixatives: These are used to secure the fabric to the frame and may include water-based adhesives, hot glue, or staples.

2. Recognising Materials by Use and Quality

Detailed Analysis with Context:

- Leather vs. Fabric Upholstery: Leather is durable, easy to clean, and ages well, making it suitable for high-use areas like waiting rooms. Fabric offers a wider variety of patterns and colours and is generally more affordable, suitable for residential use.

- Solid Wood vs. Engineered Wood Frames: Solid wood is robust and long-lasting, ideal for premium furniture. Engineered wood offers a cost-effective solution with adequate durability for modern disposable furniture.

- Foam Density in Cushioning: High-density foam provides better durability and comfort but at a higher cost, ideal for furniture intended for daily use.

Practical Exercises

Exercise 1: Material Identification Workshop

- Task: Present learners with various samples of upholstery materials including different types of fabrics, woods, metals, and cushioning.

- Objective: Identify each material and discuss its properties, advantages, and typical uses. This will enhance the learner’s ability to choose appropriate materials for specific furniture designs.

Exercise 2: Raw Material Matching

- Task: Given a scenario or a specific furniture design requirement (e.g., a sofa for a family room), learners must select appropriate materials based on their properties such as durability, comfort, and aesthetic appeal.

- Objective: This exercise helps learners apply their knowledge of materials in a simulated real-world design process, reinforcing the decision-making skills necessary for material selection.

Real-World Application

Factory Visit: Arrange a tour of a furniture manufacturing facility. Here, learners can see first-hand how different materials are used in production, observe the processes involved, and discuss with crafts makers about the choices of materials for different types of furniture.

Group Project: Following the factory visit, learners can work in groups to design a piece of furniture. Each group chooses materials based on specific criteria (cost, use, aesthetic) and presents their choices and design to the class.

These practical learning activities are designed to deeply engage learners with the physical and functional aspects of upholstery materials, ensuring they can confidently identify and select appropriate materials for various types of upholstered furniture. This foundational knowledge is crucial for both the design and production stages of furniture manufacturing.

### Practical Application Learning Content for PA0103

Identify and Categorise the Accessories According to the Uses and Furniture Design

In the craft of furniture upholstery, accessories play a pivotal role in enhancing both the functionality and the aesthetic appeal of furniture. This module focuses on equipping learners with the ability to identify and appropriately categorise various accessories based on their usage and integration into different furniture designs. Below are structured activities and examples designed to build a comprehensive understanding of upholstery accessories.

1. Introduction to Upholstery Accessories

Types of Accessories:

- Cushions and Pillows: These add comfort and can introduce color and texture to furniture.

- Legs and Feet: Made from materials like wood, metal, or plastic, these elements support the furniture and can define the style of the piece.

- Trims and Edgings: Include piping, nail head trim, and tassels that enhance aesthetic appeal and can highlight or complement the furniture’s design.

- Handles and Buttons: Functional and decorative, these are used in pieces like storage ottomans or tufted sofas.

2. Functions and Stylistic Roles of Accessories

Detailed Analysis with Context:

- Decorative vs. Functional: For instance, nail head trims can serve a decorative purpose outlining the shape of the piece, while also potentially hiding seams or upholstery edges.

- Style Definition: Wooden legs might be intricately carved to complement traditional furniture styles, whereas sleek, metallic legs could enhance modern furniture designs.

- Enhancements for Comfort: Pillows and cushions are primarily for comfort but can also be used to introduce seasonal colour schemes or trends into the decor.

Practical Exercises

Exercise 1: Accessory Identification Relay

- Task: Provide a set of various accessories (legs, cushions, handles, trims) and photos of different furniture styles.

- Objective: Learners must match accessories to the corresponding furniture styles, discussing why each accessory is suitable for the particular style based on design principles and functionality.

Exercise 2: Custom Accessory Design

- Task: Given a basic sofa design, learners must choose accessories that would transform the sofa to fit a specified style (e.g., Victorian, contemporary, rustic).

- Objective: This exercise encourages creativity and deeper understanding of how accessories can alter the perception and use of a furniture piece.

Real-World Application

Workshop Collaboration: Collaborate with a local furniture maker or designer to visit their workshop. Observe how different accessories are chosen and applied to furniture pieces during the manufacturing process.

Design Challenge: Learners participate in a competition to redesign a standard piece of furniture using accessories. Each group presents their redesigned piece, explaining their choice of accessories and the impact on the furniture’s style and functionality.

Through these activities, learners will gain a practical understanding of how accessories can be effectively used to enhance both the function and style of upholstered furniture. This knowledge is crucial for anyone involved in the design and production of upholstered pieces, ensuring that the final products not only meet functional requirements but also appeal to consumer tastes and preferences.

### Practical Application Learning Content for PA0104

Identify the Main Groups of Upholstered Seats

In the field of furniture upholstery, understanding the different groups of upholstered seats is crucial for both design and functionality purposes. This module aims to equip learners with the skills to identify and categorise various types of upholstered seats, enhancing their ability to work with a wide range of furniture designs. Below is a comprehensive guide to understanding the main groups of upholstered seats, supported by practical exercises and examples.

1. Introduction to Types of Upholstered Seats

Main Groups of Upholstered Seats:

- Fixed Seat: These seats are directly integrated into the frame of the furniture and are not removable. Common in both traditional and contemporary sofas, fixed seats often offer a streamlined look and can be designed for firm or plush comfort.

- Cushioned Seat: Also known as loose seat cushions, these are independent cushions placed over the seat frame. They are easily removable for cleaning or replacement. Cushioned seats allow for more customisability in terms of firmness and can adapt more readily to the user’s body.

- Tight Seat: This type of seat is upholstered directly over the seat frame without any visible cushioning. Tight seats are typically firmer and maintain their shape well, offering a neat and orderly appearance ideal for formal settings.

2. Recognising Seat Types in Context

Detailed Analysis with Context:

- Comfort and Style Considerations: Fixed seats might be chosen for their durability and ease of maintenance in commercial settings, whereas cushioned seats might be preferred in residential settings for their comfort and ease of customisation.

- Design Implications: Tight seats are often found in modern minimalist designs where less visual clutter is desired, while cushioned seats might be used in more traditional or rustic designs for a homier and more welcoming feel.

Practical Exercises

Exercise 1: Seat Type Identification Challenge

- Task: Provide learners with several examples of furniture, either through images or actual pieces in a workshop setting, and ask them to identify and categorise the seat type.

- Objective: Reinforce the learner’s ability to distinguish between fixed, cushioned, and tight seats, and discuss the practical implications of each choice in furniture design.

Exercise 2: Design Application Project

- Task: Learners are given a scenario where they must select the best type of seat for a given furniture piece intended for a specific environment (e.g., a luxury hotel lobby, a family home living room, a busy café).

- Objective: This exercise helps learners apply their knowledge to real-world scenarios, enhancing their understanding of how seat choice affects functionality and style.

Real-World Application

Field Trip: Organise a visit to a furniture manufacturing facility or a large furniture showroom. Observe different types of seats being made or displayed, and discuss their design and functional aspects with professionals.

Group Discussion: After the field trip, hold a discussion where learners share their observations and insights about different seat types they encountered, discussing why particular designs were used in specific furniture settings.

These activities are designed to deepen learners' understanding of the different types of upholstered seats and their applications in various furniture designs. By engaging with both theoretical and practical aspects of upholstery, learners will be better equipped to make informed decisions in their future projects within the industry.

### Practical Application Learning Content for PA0105

Identify the Main Groups of Upholstered Backs

Identifying and categorising the main groups of upholstered backs is essential for professionals in the furniture design and manufacturing industry. This knowledge helps in selecting the appropriate back type that complements the functionality and aesthetic of a piece of furniture. Below, we outline structured learning content that includes detailed descriptions, practical exercises, and real-world applications to deepen understanding of upholstered backs.

1. Introduction to Types of Upholstered Backs

Main Groups of Upholstered Backs:

- Tight Back: These are upholstered backs where the upholstery is attached directly to the frame of the furniture. Tight backs are known for maintaining their shape and providing a clean, streamlined appearance. They are commonly found in more formal settings and offer substantial support.

- Cushioned Back: This type includes loose cushions that can be removed for cleaning or replaced. Cushioned backs provide a softer, more comfortable feel and allow for more flexibility in terms of design and customisation.

- Channel Back: Featuring vertical grooves or channels that are sewn into the upholstery, channel backs add a decorative element while also providing enhanced support and comfort. This style is often used in art deco and contemporary designs.

- Tufted Back: Characterised by patterns of buttons or tufts that create indentations in the upholstery. Tufting adds both aesthetic appeal and additional support, and it is popular in both traditional and modern furniture styles.

2. Recognising Back Types in Context

Detailed Analysis with Context:

- Aesthetic and Functional Implications: Tight backs offer a neat appearance and are easy to maintain, making them suitable for high-traffic commercial environments. Cushioned backs, being more casual, are ideal for residential settings where comfort is a priority.

- Design Flexibility: Channel backs provide a unique look that can serve as a focal point in a room, whereas tufted backs are versatile enough to enhance the luxury of a living room or the formality of an office setting.

Practical Exercises

Exercise 1: Back Type Identification Workshop

- Task: Learners are presented with various images or actual examples of furniture and are tasked with identifying the type of upholstered back and discussing its characteristics.

- Objective: This exercise reinforces the ability to distinguish between different types of upholstered backs and understand their applications.

Exercise 2: Design Specification Role-play

- Task: In a role-play setting, learners are given a client profile and a scenario (e.g., designing a boutique hotel lobby or a cosy home library) and must choose the appropriate type of upholstered back that meets both aesthetic and functional requirements.

- Objective: Encourages practical application of knowledge in selecting upholstered backs based on specific design criteria and client needs.

Real-World Application

Site Visit: Arrange a visit to a local furniture workshop or showroom where different styles of upholstered backs are on display. Discuss with designers or craftspeople about the processes involved in creating various back styles.

Group Project: Learners collaborate to design a piece of furniture for a mock client, selecting materials, and the type of upholstered back. Each group presents their design, justifying their choices based on the client's needs and the intended use of the furniture.

By engaging with these practical learning activities, learners will develop a deeper understanding of the different types of upholstered backs and their specific uses in furniture design. This knowledge is crucial for making informed decisions that affect both the functionality and aesthetic quality of upholstered furniture.

### Practical Application Learning Content for PA0106

Identify the Main Groups of Unpolished Arms

Understanding the different types of upholstered arms is crucial for furniture design and manufacturing. This module will equip learners with the skills to identify and categorise upholstered arms based on their structural and aesthetic characteristics. The learning content provided below includes detailed information, practical exercises, and examples to aid in the comprehension and application of knowledge regarding upholstered arms.

1. Introduction to Types of Upholstered Arms

Main Groups of Upholstered Arms:

- Rolled Arms: These arms curve outward and are typically padded, contributing to a classic and comfortable look. Rolled arms are common in traditional styles like English roll arm sofas.

- Track Arms: Also known as straight arms, they have clean lines and a squared profile, lending a modern and sleek appearance to the furniture. They are less padded than rolled arms, emphasising a more contemporary look.

- Sloped Arms: These arms slope down from the back of the sofa towards the front, offering a casual and relaxed feel. They are often seen in casual or transitional furniture styles.

- Flared Arms: Arms that flare outward and upward, adding a dynamic visual element to the furniture. Flared arms can be seen in modern and eclectic styles and can vary in the degree of flare and padding.

2. Recognising Arm Types in Context

Detailed Analysis with Context:

- Aesthetic and Functional Implications: Rolled arms provide a soft, inviting look and extra comfort, suitable for living rooms or dens. Track arms, with their minimalistic appeal, are ideal for spaces where space saving is crucial, such as small apartments.

- Design Flexibility: Sloped arms offer a middle ground between formal and casual, fitting well in family rooms or sunrooms. Flared arms can serve as a statement piece in a room, suitable for more adventurous or thematic interior designs.

Practical Exercises

Exercise 1: Arm Type Recognition Drill

- Task: Learners are shown various images or actual pieces of furniture, each featuring different types of arms. They must identify the arm style and discuss why it might be chosen for a particular setting.

- Objective: This drill helps to solidify the learner's ability to recognise and understand the practical applications of different arm types in furniture design.

Exercise 2: Concept Design Challenge

- Task: Learners are tasked with designing a piece of furniture for a specified space (e.g., a luxury hotel suite or a corporate office lobby). They must choose an appropriate arm style that complements both the function and aesthetic of the room.

- Objective: This challenge encourages learners to apply their knowledge creatively, considering both the visual impact and the functional requirements of the furniture.

Real-World Application

Workshop Session: Arrange a workshop with a local furniture craftsman or visit a manufacturing plant. Observing the construction and upholstery process of different arm styles can provide invaluable practical insights.

Collaborative Project: Learners collaborate in teams to refurbish an old piece of furniture by changing its arm style. This project allows them to apply their theoretical knowledge in a hands-on setting, transforming an existing piece into a new style.

By engaging with these structured learning activities, learners will develop a nuanced understanding of the different types of upholstered arms and their implications for furniture design. This foundation is essential for anyone involved in the crafting or design of upholstered furniture, enabling them to make informed choices that enhance both the style and functionality of their creations.

### Applied Knowledge Learning Content for AK0101

Upholstered Furniture Types, Designs, and Styles

Understanding the different types, designs, and styles of upholstered furniture is essential for professionals in the furniture manufacturing and design industry. This knowledge enables them to make informed decisions about design adaptations, material selections, and marketing strategies. The following content provides a comprehensive overview of upholstered furniture types, designs, and styles, complete with examples to enhance the practical application of this knowledge.

1. Overview of Upholstered Furniture Types

Upholstered furniture is categorised based on its functionality, design elements, and the level of comfort it provides. Here are the main types:

- Sofas: Typically seats three or more people, with options ranging from fixed back to sectional pieces.

- Loveseats: Smaller than sofas, designed to seat two people comfortably.

- Armchairs: Single-seating pieces that often match the material and design of a sofa or loveseat.

- Ottomans: Upholstered seats without backs or arms, used as footrests or extra seating.

- Chaises: Long chairs allowing for leg extension, often used in lounging areas.

2. Designs in Upholstered Furniture

The design of upholstered furniture often refers to the specific features and construction details that define its functionality and aesthetic appeal:

- Frame Design: The structure of the furniture which can be made of wood, metal, or plastic. The frame design dictates the overall shape and durability of the piece.

- Support System: Includes springs, webbing, or slats that provide support to the cushions, affecting the comfort and lifespan of the furniture.

- Cushioning: Various types of foam, down, or hybrids are used, impacting the firmness, comfort, and recovery rate of the seating surface.

- Fabric and Finishing: Choices range from natural fibers like cotton and wool to synthetics like polyester. The fabric affects the look, feel, and durability of the furniture.

3. Styles of Upholstered Furniture

Understanding different styles helps professionals design pieces that resonate with specific market segments and interior design trends:

- Traditional Style: Features intricate designs, rich colours, and luxurious fabrics. Examples include Victorian sofas with ornate wooden carvings and plush, velvety fabric.

- Modern Style: Characterised by minimalistic designs, clean lines, and neutral colours. Modern pieces often use materials like microfiber or leather for a sleek look, such as a minimalist sofa with straight, clean lines and firm cushions.

- Contemporary Style: Blends elements of both modern and traditional styles but leans towards current trends. It often includes bold colours or patterns and innovative materials.

- Rustic Style: Emphasises natural beauty, with an earthy colour palette and rugged textures, often seen in lodge or cabin settings. Rustic pieces might use distressed leather or suede.

- Eclectic Style: A mix of various styles, often boldly juxtaposed to create a unique and personalised aesthetic. An eclectic approach might combine a traditional chesterfield sofa with modern metallic accents.

Practical Application and Examples

Case Study Analyses: Provide learners with case studies of different furniture pieces designed for specific markets or needs. For instance, analyse a luxury recliner designed for home theatre setups, discussing the ergonomic design, material choices, and style considerations.

Design Simulation: Learners are tasked with designing a piece of upholstered furniture for a hypothetical client. They must consider the client's needs, the intended use of the furniture, and the overall design theme of the space where the furniture will be placed.

Field Trips: Organise visits to furniture showrooms or trade shows where learners can see and experience a variety of furniture styles and constructions firsthand. Discuss with designers or brand representatives about the thought process behind different furniture designs.

By engaging with these detailed explanations and practical exercises, learners will gain a thorough understanding of the different types, designs, and styles of upholstered furniture. This will enable them to create well-informed and appealing furniture designs tailored to varying tastes and functional requirements.

### GUIDELINES

You are required to perform the task below and your Facilitator/Assessor will assess your work based on the observation checklist attached.

Practical Assessment Task for PM-02-PS01: Identify different styles, types and designs of upholstered furniture

Assessment Title:

Purpose

Guidelines to the Learner:

1. You will be presented with a range of samples or photographs of different wooden furniture products and accessories. These will include various styles, types, and uses.  Refer to Knowledge Module KM13 and the additional brochure with examples of upholstered furniture.

2. You are required to examine each sample or photograph and perform the following tasks:

Assessment Criteria:

Assessment Duration:

- The assessment should be completed within a specified time frame, typically 2-3 hours, to simulate a realistic working environment.

Feedback and Evaluation:

- Detailed feedback will be provided on the accuracy and relevance of your categorisations.

- The assessor will highlight areas of strength and suggest improvements where necessary.

Additional Notes:

- This practical assessment is designed to test not only your ability to recognise and categorise different furniture types and accessories but also your understanding of the broader context in which these items are used.

- It is essential to approach this task with attention to detail and a thorough understanding of the varied aspects of upholstered furniture design and manufacturing.

### Case Study: Practical Assessment on Upholstered Furniture Identification

Scenario:

You are an intern at a renowned furniture design studio, and part of your internship involves assisting in the categorisation and documentation of new furniture pieces received from various manufacturers. These pieces will be featured in an upcoming furniture exhibition. Your task is to categorise each piece based on photographs, samples, and prototypes provided, focusing on their upholstered seats, backs, and arms.

Materials Provided:

1. Photographs: High-resolution images of various upholstered furniture pieces.

2. Samples: Fabric and upholstery material samples used in the furniture.

3. Prototypes: Miniature models of the upholstered furniture.

Tasks:

1. Categorise Upholstered Seats: Identify and categorise the upholstered seats into fixed, cushioned, or tight based on the materials provided.

2. Categorise Upholstered Backs: Classify the upholstered backs as tight back, cushioned back, channel back, or tufted back.

3. Categorise Upholstered Arms: Determine if the upholstered arms are rolled, track, sloped, or flared.

Scenario-Based Questions:

Question 1:

You are presented with a photograph of a sofa that features a sleek design with a low back and minimal cushioning. The arms of the sofa are straight and appear to be level with the back.

- How would you categorise the upholstered seats, backs, and arms of this sofa?

Question 2:

A fabric sample comes attached to a prototype of an armchair featuring large, plush cushions that are visibly separable from the frame, and the back is adorned with vertical stitching lines.

- Identify and categorise the types of upholstered seats and backs from this description.

Question 3:

You receive a miniature model of a vintage-style chair with heavily padded arms that curl outward at the ends and a back that features deep button tufts.

- What are the categories for the upholstered arms and backs of this chair?

Marking Rubric:

| Criteria | Exemplary (3 points) | Satisfactory (2 points) | Needs Improvement (1 point) |
| --- | --- | --- | --- |
| Seat Identification | Correctly identifies and categorises the type of seat with clear justification based on visual cues. | Correctly identifies the type but with minimal or unclear justification. | Incorrectly identifies or categorises the seat type without justification. |
| Back Identification | Correctly identifies and categorises the type of back with detailed justification referencing specific design features. | Correctly identifies the back type but with minimal or unclear justification. | Incorrectly identifies or categorises the back type, lacks justification. |
| Arm Identification | Accurately identifies and categorises the arm type with comprehensive details and clear references to the provided materials. | Correctly identifies the arm type but lacks detail or clear justification. | Incorrectly identifies or categorises the arm type, with no or incorrect justification. |

Notes:

Notes:

- Points are awarded based on accuracy, detail, and justification provided.

- The total score will be out of 9 points, where each section carries a maximum of 3 points.

This assessment aims to test the practical and theoretical understanding of furniture categorisation, crucial for effective communication and documentation in furniture design and sales sectors.

EVALUATION CHECKLIST

| NAME: …………………………………….…. SURNAME: ……………………………….…. ID…………………………………………….... | LEARNER SIGNATURE DATE: |
| --- | --- |

| EVALUATION CRITERIONS | MET REQUIREMENTS | DID NOT MEET REQUIREMENTS | COMMENTS OR ACTION REQUIRED |
| --- | --- | --- | --- |
| Ability of the learner to: |  |  |  |

| GENERAL COMMENTS OF OBSERVER: |  |  |
| --- | --- | --- |
| Assessor Name ……………………………… | Signature ……………………………………. | Date …………………………………. |

## PM-02-PS02: Identify various types of furniture frame construction and frame components using the correct terminology

Scope of Practical Skills

Given examples or pictures of various shapes, styles and types of frames and product specifications the learner must be able to:

PA0201 Identify and distinguish between different frame constructions according to types and styles

- PA0202 Identify and distinguish the components of upholstered seats

- PA0203 Identify and distinguish between frames according to purpose

- PA0204 Identify and distinguish between the four back styles of furniture frames

- PA0205 Identify seating frames according to the three main types

- PA0206 Identify the wood and board products used in the construction of the frame(s) according to their application

- PA0207 Identify and distinguish between joints and bracings used in the frame construction

- PA0208 Identify and distinguish between the long, short and end grains of wood and their ability to hold staples and nails

Applied Knowledge

Internal Assessment Criteria

- IAC0201 Frame components are correctly identified for fully upholstered and show wood seats

- IAC0202 Seating frames are identified according to their purpose (such as resting and lounging, dining and writing, transport, occasional use)

- IAC0203 Furniture frames are identified according to the four back styles (camel, shell, pillow, curved)

- IAC0204 Seating frames are identified according to the three main types (mono-frame, knock down frame, modular frame)

- IAC0205 Board components and wood used to make furniture frames are identified according to their application (MDF, chip board, soft wood, hard wood)

- IAC0206 Joints and bracings used in frame construction are identified (mortise and tenon joints, dowelled joints, housing and stapled joints, lap joints to lengthen rails, finger joint bracings like cross rails, bracing rails, corner blocks, etc.)

-  IAC0207 The long, short and end grains of solid wood frame components are identified

### Practical Application Learning Content for PA0201

Identify and Distinguish Between Different Frame Constructions According to Types and Styles

The ability to identify and distinguish between various furniture frame constructions is crucial for professionals in the furniture design and manufacturing industry. This knowledge enables them to ensure the structural integrity and aesthetic quality of furniture. The following content provides detailed insights into different frame constructions, along with practical exercises to aid in the understanding and application of these concepts.

1. Overview of Frame Construction Types

Understanding the different types of frame constructions involves recognising the materials used and the methods applied in constructing the furniture frame. Here are some common types:

- Solid Wood Frames: Often used for their durability and strength. Solid wood frames are typically found in high-quality furniture and can be made from hardwoods like oak, maple, or teak, which are known for their longevity.

- Metal Frames: Common in contemporary and industrial furniture styles, metal frames provide a modern look and are usually constructed from materials such as aluminium, steel, or iron. They are preferred for their strength and ability to hold modern, minimalist shapes.

- Plywood Frames: A cost-effective alternative to solid wood, plywood frames are lighter and offer flexibility in terms of shaping. They are often used in commercial furniture or in designs where cutting costs without significantly compromising on durability is crucial.

- Particle Board Frames: These are made from compressed wood chips and resin, often used in budget-friendly furniture. While not as durable as solid wood or metal, particle board frames are suitable for light-use items and offer significant cost savings.

2. Styles of Frame Construction

Frame styles can vary widely, often influenced by the era of design, the intended use of the furniture, and aesthetic considerations:

- Traditional Frames: Feature ornate carvings and mouldings, often constructed from solid wood. These frames are heavy and designed to convey a sense of antiquity and luxury.

- Modern Frames: Emphasise clean lines and minimalistic forms, often utilising metal or polished wood. These frames are designed to support a contemporary aesthetic with less visual clutter.

- Transitional Frames: Blend elements of traditional and modern designs. Often constructed from a combination of metal and wood, these frames cater to a broad range of tastes by balancing classic and contemporary elements.

Practical Exercises

Exercise 1: Frame Identification Workshop

- Task: Learners are provided with a series of images and physical examples of furniture frames. They must identify the type of material used and classify the frame according to the style.

- Objective: To develop the ability to quickly and accurately determine frame types and styles from visual and tactile examination.

Exercise 2: Material Benefits and Drawbacks Discussion

- Task: In groups, learners discuss the advantages and disadvantages of different frame materials and styles, referencing specific examples from the workshop.

- Objective: To enhance understanding of why certain frame materials and styles are chosen for specific furniture designs based on factors like cost, durability, and aesthetic preferences.

Real-World Application

Site Visit: Arrange a tour of a furniture manufacturing facility where learners can see the construction of different types of frames in real-time. Discussions with craftspeople and designers can provide deeper insights into the choice of materials and construction techniques.

Project Assignment: Learners are tasked with designing a piece of furniture, selecting an appropriate frame type and style for a specified scenario (e.g., a high-traffic café vs. a luxury residential living room). They must justify their choices based on the materials' properties, expected usage, and customer expectations.

By engaging with these detailed explanations and practical activities, learners will develop a robust understanding of furniture frame constructions. This knowledge is essential for ensuring that furniture not only meets the desired aesthetic standards but also holds up to the intended use and environmental conditions.

### Practical Application Learning Content for PA0202

Identify and Distinguish the Components of Upholstered Seats

Understanding the various components of upholstered seats is crucial for furniture design and upholstery professionals. This knowledge allows for the proper assembly, maintenance, and customisation of furniture to meet specific comfort and durability standards. The following content outlines the main components of upholstered seats, providing practical exercises and examples to facilitate learning and application.

1. Overview of Upholstered Seat Components

Upholstered seats are composed of several key components, each contributing to the overall comfort, style, and functionality of the furniture:

- Frame: The skeleton of the seat, usually made of wood, metal, or a combination of both. The frame defines the shape and structural integrity of the seat.

- Support System: Includes springs, webbing, or rigid panels that provide a foundation for the cushions. Types of support systems include:

- Sinuous Springs: S-shaped springs that provide durable and supportive seating.

- Eight-Way Hand-Tied Springs: Traditional and very comfortable, these springs are tied by hand for maximum support and longevity.

- Webbing: Made of fabric or synthetic fibers, used in lighter applications or modern furniture with a sleeker design.

- Padding: Various materials such as foam, batting, or down are used to cover the frame and springs, adding comfort and shaping the seat.

- Foam: Available in several densities, it is the most common padding due to its availability and cost-effectiveness.

- Batting: Often wrapped around foam, batting can be made from cotton, wool, or polyester fibers to add an extra layer of comfort and help smooth out shapes.

- Down: Feather fillings are used in luxury furniture for exceptional softness and a plush look.

- Upholstery Fabric or Leather: The outer material that covers the entire assembly. Choices in material affect the aesthetic, tactile feel, and durability of the seat. Options range from natural fabrics like cotton and linen to synthetics like polyester and microfiber, as well as leather and faux leather.

2. Practical Exercises

Exercise 1: Component Identification Quiz

- Task: Learners are shown a series of cross-sectional diagrams and real seat samples with visible components. They must identify and label each component of the upholstered seats.

- Objective: To develop the ability to recognise and understand the function of different seat components from visual clues.

Exercise 2: Material Matching Game

- Task: Given different scenarios (e.g., high-traffic public seating, luxury residential lounge chairs), learners must choose appropriate materials for the frame, support system, padding, and upholstery.

- Objective: To enhance decision-making skills regarding material selection based on usage requirements and budget constraints.

3. Real-World Application

Workshop Activity: Arrange a hands-on session in a local upholstery workshop where learners can disassemble and reassemble different types of seats. This will help them understand the construction techniques and the interaction between different materials.

Design Challenge: Learners design a custom seat for a specific application (e.g., a children's reading chair). They must select suitable materials for each component and justify their choices based on the intended use, comfort requirements, and durability needs.

4. Evaluation and Feedback

Provide feedback on the practical exercises with emphasis on the rationale behind material choices and the correctness of component identification. This feedback will help learners refine their understanding and approach to designing and upholstering seats.

By engaging with these educational activities, learners will gain a comprehensive understanding of the components that make up upholstered seats. This knowledge is crucial for ensuring that furniture is not only aesthetically pleasing but also functionally sound and durable.

### Practical Application Learning Content for PA0203: Identify and Distinguish Between Frames According to Purpose

In the field of furniture design and manufacturing, identifying the purpose of different furniture frames is essential for creating products that meet specific functional requirements and customer expectations. This module provides learners with practical exercises and examples to understand how to classify frames based on their intended purposes.

1. Overview of Frame Purposes

Frames serve as the skeleton of furniture, providing structure and defining its functionality. Different types of frames are designed to cater to specific purposes:

- Residential Frames: Designed for use in home environments, focusing on comfort and aesthetics. These frames are often made with softer, more flexible materials that prioritise comfort over extreme durability.

- Commercial Frames: Built to withstand frequent use in public spaces such as offices, hotels, and restaurants. These frames are generally sturdier and made from more durable materials like hardened woods or metal to handle heavier usage.

- Outdoor Frames: Constructed to resist weather conditions and external elements. Materials like treated wood, metal, or heavy-duty plastics are used to ensure longevity and resistance to moisture and sunlight.

- Specialty Frames: These include frames designed for specific functionalities such as foldable beds, reclining chairs, or modular furniture that can be reconfigured for various uses.

2. Practical Exercises

Exercise 1: Frame Purpose Matching

- Task: Learners are presented with images and descriptions of various furniture items. They must match each item with the correct type of frame purpose (Residential, Commercial, Outdoor, and Specialty).

- Objective: This exercise helps learners apply their knowledge to real-world scenarios, improving their ability to select appropriate frame materials and designs based on furniture usage.

Exercise 2: Material Selection Simulation

- Task: Given a scenario where learners need to design furniture for a specified location (e.g., a beachfront cafe, a home office), they must choose suitable frame materials that fit the environment’s demands.

- Objective: Learners will demonstrate their understanding of material properties and their applications in different settings, ensuring the longevity and functionality of the furniture.

3. Real-World Application

Field Trip: Organise a visit to a furniture manufacturing plant where different types of frames are produced. Observing the production process provides practical insights into how different materials are used for various frame purposes.

Group Project: Assign learners to design a piece of furniture for a hypothetical client with specific needs (e.g., a fast-food restaurant chain needs durable seating, a library needs comfortable reading chairs that are easy to move). They must select frame types and materials based on these requirements and present their designs to the class.

4. Evaluation and Feedback

Rubric for Exercises:

- Accuracy of Matching/Selection: Correct identification and matching of frame purposes with scenarios.

- Justification of Choices: Quality of explanation provided for material and type selections.

- Creativity in Design: Innovative approaches in meeting the client's needs and environmental demands.

By engaging with these activities, learners will deepen their understanding of the relationship between frame purpose and material selection, enhancing their ability to create furniture that is both functional and appropriate for its intended use. This knowledge is crucial for successful furniture design and production, ensuring products are not only aesthetically pleasing but also suitable for their environments and usage.

### Practical Application Learning Content for PA0204: Identify and Distinguish Between the Four Back Styles of Furniture Frames

Understanding the back styles of furniture frames is crucial for furniture designers and manufacturers. It impacts not only the aesthetic appeal of a piece but also its comfort and ergonomic support. This module aims to equip learners with the necessary skills to identify and differentiate between various back styles, enhancing their ability to design and produce furniture tailored to specific needs and preferences.

1. Overview of Back Styles in Furniture Frames

Back styles in furniture play a pivotal role in defining the look and function of a piece. Here are the four main styles typically encountered:

- Tight Back: A tight back style means the back cushions are sewn directly onto the frame. This style is sleek and offers a clean appearance with minimal maintenance since the cushions do not move or require fluffing.

- Cushion Back: Cushion back furniture features loose cushions that can be removed for cleaning or replaced. This style allows for more comfort and a plusher feel, as well as the flexibility to adjust the cushions as needed.

- Channel Back: Channel back styles are characterised by vertical channels stitched into the upholstery to create a distinct pattern. This not only adds visual depth but also provides enhanced support along the length of the back.

- Camelback: Named for its arched shape reminiscent of a camel’s hump, the camelback style combines both aesthetic appeal and comfort. The middle of the back is typically higher than the sides, providing natural lumbar support.

2. Practical Exercises

Exercise 1: Back Style Identification Drill

- Task: Learners are presented with a series of furniture images where only the backs are visible. They must identify and label the back style of each piece.

- Objective: This exercise sharpens the learners’ ability to recognise and articulate differences between the back styles based solely on visual cues.

Exercise 2: Design Application Role-Play

- Task: In a role-playing scenario, learners act as furniture designers who must choose the appropriate back style for a client’s specific needs (e.g., a waiting room in a pediatrician's office, a luxury hotel lobby).

- Objective: Learners apply their knowledge to select back styles that meet specific functional and aesthetic requirements, demonstrating their understanding of the implications of each style.

3. Real-World Application

Field Trip: Visit a furniture showroom or manufacturing facility to see various back styles in person. Discuss with professionals about the design choices and customer feedback on different styles.

Group Project: Learners are tasked to design a small living room set that includes a sofa and two chairs. Each piece must incorporate a different back style, and learners must present their designs, explaining how the chosen back styles complement the overall design theme and intended use of the space.

4. Evaluation and Feedback

Rubric for Exercises:

- Correct Identification: Accuracy in identifying back styles from images.

- Rationale: Quality of explanations provided for back style choices in design scenarios.

- Risk-taking and innovation in design for the group project.

By engaging with these structured learning activities, learners will not only deepen their theoretical knowledge of back styles but also enhance their practical skills in applying this knowledge in real-world settings. This dual focus ensures that learners are well-prepared to meet the demands of the furniture design and manufacturing industry, capable of making informed decisions that affect both the form and function of their products.

### Practical Application Learning Content for PA0205: Identify Seating Frames According to the Three Main Types

Identifying the different types of seating frames is essential for professionals in the furniture design and manufacturing industry, as it affects the structural integrity, comfort, and aesthetic appeal of the furniture. This module provides practical exercises and examples to help learners understand and categorise the three main types of seating frames.

1. Overview of Seating Frame Types

Seating frames are the foundation upon which the entire construction of a seat is built. Here are the three main types:

- Solid Frame: This type of frame is typically constructed from a single material such as wood or metal. Solid frames are known for their durability and strength, making them suitable for furniture that requires long-lasting support.

- Assembled Frame: Assembled frames are made from multiple components that are joined together. This can include combinations of wood, metal, and other materials. Joints in assembled frames are crucial and can include techniques like dovetailing, mortise-and-tenon, and others.

- Modular Frame: Modular frames are designed to be flexible and customisable. They are often used in modern furniture designs where parts can be added, removed, or rearranged according to user needs. This type includes sectional sofas, modular office furniture, and other reconfigurable pieces.

2. Practical Exercises

Exercise 1: Frame Type Matching Quiz

- Task: Learners are presented with various images of furniture, and they must determine which of the three main types of frames each piece uses.

- Objective: This quiz helps reinforce the learner’s ability to quickly identify different frame types from visual cues, enhancing their understanding of frame construction and design considerations.

Exercise 2: Frame Construction Analysis

- Task: Learners are given different frame examples in a workshop setting. They must analyse and report on the construction techniques used, identifying whether the frame is solid, assembled, or modular.

- Objective: This hands-on exercise aims to deepen learners' practical knowledge of how different frame types are constructed and the implications for durability and use.

3. Real-World Application

Site Visit: Organise a visit to a local furniture manufacturing company where learners can observe the production of different types of frames. Interaction with craftsmen and designers will provide insights into the decision-making process behind choosing a frame type for specific furniture designs.

Group Design Project: Learners are tasked with designing a piece of furniture for a specific use (e.g., residential, commercial, outdoor). They must choose an appropriate frame type based on the furniture’s required functionality and user needs, then present their design with a justification for their choice of frame type.

4. Evaluation and Feedback

Rubric for Exercises:

- Accuracy: Correct identification of the frame type from images and real examples.

- Analytical Skills: Depth of analysis in construction techniques and understanding of frame properties.

- Creativity and Application: Innovation and practical application in the group design project, including justification of frame type choice based on intended use.

By engaging in these activities, learners will gain a comprehensive understanding of the different types of seating frames and their applications. This knowledge is crucial for designing furniture that not only meets aesthetic standards but also performs well in its intended environment.

### Practical Application Learning Content for PA0206: Identify the Wood and Board Products Used in the Construction of the Frame(s) According to Their Application

In the furniture manufacturing industry, selecting the appropriate wood and board products for frame construction is crucial for ensuring durability, functionality, and aesthetic appeal. This module provides detailed practical exercises and examples to help learners identify various wood and board products and understand their specific applications in furniture construction.

1. Overview of Wood and Board Products

Different types of wood and engineered wood products offer unique properties that make them suitable for various furniture applications:

- Solid Wood: Commonly used in furniture for its strength, durability, and aesthetic appeal. Types like oak, maple, and walnut are popular for high-end furniture due to their hardness and grain patterns.

- Plywood: Consists of multiple layers of wood veneer glued together to form a strong and stable sheet. Plywood is widely used in modern furniture due to its versatility and resistance to warping.

- Particle Board: Made from compressed wood fibres and resin, particle board is more affordable and lighter than solid wood or plywood, making it suitable for inexpensive furniture.

- MDF (Medium Density Fibreboard): Known for its smooth surface and uniform density, MDF is ideal for painted furniture pieces and intricate designs where the wood grain pattern is not desired.

- OSB (Oriented Strand Board): Constructed from compressed layers of wood strands in specific orientations, OSB is strong and used in both furniture and construction for areas requiring structural integrity.

2. Practical Exercises

Exercise 1: Material Identification and Application

- Task: Learners are provided with samples of solid wood, plywood, particle board, MDF, and OSB. They must identify each material and discuss its benefits and drawbacks in furniture frame construction.

- Objective: To develop an ability to correctly identify different wood and board products and understand their applications, enhancing material selection skills.

Exercise 2: Match the Material to Furniture Type

- Task: Using a set of furniture design specifications, learners must choose the most appropriate wood or board product for various furniture items like bookshelves, dining tables, and sofas.

- Objective: Learners apply their knowledge to practical scenarios, selecting materials based on the furniture’s intended use, durability requirements, and cost constraints.

3. Real-World Application

Workshop Activity: Organise a hands-on workshop in a local furniture factory or a woodworking workshop where learners can see the transformation of these materials into furniture frames. Discussions with experienced crafts makers can provide deeper insights into material choices.

Group Project: Learners design a small piece of furniture (e.g., a coffee table or a nightstand) specifying the type of wood or board product for each part of the frame. They must justify their choices based on the product’s properties and the intended use of the furniture.

4. Evaluation and Feedback

Rubric for Exercises:

- Identification Accuracy: Correct identification and understanding of the different wood and board products.

- Rational Material Selection: Quality of material choices for specified furniture types, including justification based on cost, durability, and aesthetic requirements.

- Innovation and Design: Creativity in the group project design, effectiveness in material usage, and overall presentation skills.

By engaging with these educational activities, learners will gain a comprehensive understanding of the various wood and board products available for furniture frame construction. This knowledge ensures that future furniture designs are not only visually appealing but also structurally sound and suited to their intended applications.

### Practical Application Learning Content for PA0207: Identify and Distinguish Between Joints and Bracings Used in the Frame Construction

Understanding the different types of joints and bracings is crucial for furniture design and construction. This knowledge ensures the structural integrity and longevity of furniture pieces. This module will provide learners with practical exercises and examples to help them identify and differentiate various joints and bracings used in furniture frame construction.

1. Overview of Joints and Bracings

Joints and bracings are fundamental in furniture construction, providing the necessary support and stability. Here are the main types commonly used in furniture frames:

- Dovetail Joint: Known for its use in drawer construction, the dovetail joint involves interlocking pins and tails that create a durable and strong connection without the need for nails or screws.

- Mortise and Tenon Joint: A classic woodworking joint that involves a tenon (a projection) on the end of one piece and a mortise (a cavity) into which it fits on the other piece. This type of joint is valued for its strength and aesthetic simplicity.

- Dowel Joint: Uses small wooden pegs (dowels) fitted into corresponding holes to connect pieces of wood. It is less visible than other joints and provides a clean appearance.

- Butt Joint: The simplest and most common type of joint where the ends of two pieces of wood are butted together and fixed with adhesives, nails, or screws. It is often reinforced with plates or brackets for extra stability.

- Bracket Joint: Involves using a bracket or a brace to reinforce a joint. Brackets can be made of metal or wood and are used to support angles and corners in furniture frames.

2. Practical Exercises

Exercise 1: Joint Identification Workshop

- Task: Learners are provided with various examples of furniture frames featuring different joints and bracings. They must identify each type and discuss its use and advantages in furniture construction.

- Objective: This workshop will enhance learners' ability to recognise and understand different types of joints and bracings, improving their skills in assessing furniture quality and construction.

Exercise 2: Joint Assembly Challenge

- Task: In a practical session, learners are given the tools and materials to construct small frame samples using at least three different types of joints mentioned above.

- Objective: By actively constructing joints, learners will gain hands-on experience and a deeper understanding of the structural implications and practical applications of each joint type.

3. Real-World Application

Site Visit: Arrange a visit to a furniture manufacturing plant where learners can observe professional carpenters and joiners at work. Watching the application of various joints and bracings in real-time will provide valuable insights into their practical usage.

Group Project: Learners design and build a piece of furniture using at least two different types of joints or bracings. They must document their work process, highlighting why specific joints were chosen and how they contribute to the overall stability and aesthetics of the furniture.

4. Evaluation and Feedback

Rubric for Exercises:

- Accuracy of Identification: Correctly identifying the type of joints and bracings from visual inspection or practical demonstration.

- Application Skills: Effectiveness in using different joints and bracings in practical assembly tasks.

- Creativity and Problem Solving: Innovation in the design and construction of the group project, especially in the application of joints for structural and aesthetic purposes.

These practical activities are designed to deepen learners' understanding of the critical role that joints and bracings play in furniture construction, equipping them with the skills needed to choose and apply the most appropriate methods for different types of furniture.

### Practical Learning Content for PA0208: Identify and Distinguish Between the Long, Short, and End Grains of Wood and Their Ability to Hold Staples and Nails

Understanding wood grain orientation is crucial for furniture making, affecting both the aesthetic appeal and structural integrity of the piece. This module offers practical exercises to help learners identify different wood grains and evaluate their properties, particularly how well they hold fasteners like staples and nails.

1. Overview of Wood Grain Types

Wood grain refers to the direction, texture, and appearance of the fibres in wood. Recognising different grain types is vital for selecting appropriate joining techniques and fasteners:

- Long Grain (Edge Grain): Runs along the length of the wood, typically visible on the edges of boards. It is the strongest and most stable direction in wood and holds fasteners well, making it ideal for structural components of furniture.

- Short Grain (Face Grain): Runs perpendicular to the long grain and is usually visible on the surface of the board. This grain is less strong and can be more prone to splitting when fasteners are applied.

- End Grain: Found at the top or bottom of a piece of wood, where the growth rings are visible. End grain absorbs and releases moisture more quickly than other grain types, making it susceptible to splitting. It does not hold fasteners as securely as long or short grains and often requires pre-drilling or different types of joinery.

2. Practical Exercises

Exercise 1: Grain Identification and Fastener Test

- Task: Learners are provided with samples of wood showing long, short, and end grains. They must identify each grain type and then attempt to insert staples and nails into each, noting the ease of insertion and the hold strength.

- Objective: To develop an understanding of how different grains affect the ability of wood to hold fasteners, and to recognise potential issues like splitting or poor hold.

Exercise 2: Joinery Technique Selection

- Task: Given different furniture assembly scenarios, learners must choose the best type of joinery technique based on the grain orientation of the wood involved. They will use their knowledge of grain types to select whether to use nails, screws, glue, or alternative joining methods like dowels or biscuits.

- Objective: To apply knowledge of wood grains in practical joinery decisions, ensuring both aesthetic appeal and structural integrity.

3. Real-World Application

Workshop Activity: Arrange a workshop in a carpentry shop where learners can observe and participate in the preparation and assembly of wood components. Focus on how professionals select and apply different fasteners based on wood grain.

Group Project: Learners design and create a small wooden item such as a picture frame or a simple stool. They must document their wood choice, grain orientation considerations, and fastening methods used, explaining their decisions and outcomes.

4. Evaluation and Feedback

Rubric for Exercises:

- Identification Accuracy: Correct identification of wood grain types from samples.

- Practical Application: Effectiveness and appropriateness of fastener choice and application in wood samples.

- Design and Innovation: Creativity in the group project, the effectiveness of joinery techniques, and the ability to justify choices based on grain orientation.

By engaging with these educational activities, learners will gain a comprehensive understanding of how wood grain affects furniture construction. This knowledge will empower them to make informed choices about materials and methods, enhancing both the functionality and durability of their woodworking projects.

### GUIDELINES

You are required to perform the task below and your Facilitator/Assessor will assess your work based on the observation checklist attached.

Practical Assessment Task for PM-02-PS02: Identify various types of furniture frame construction and frame components using the correct terminology

Assessment Title:

Purpose

Guidelines to the Learner:

1. You will be presented with a range of samples or photographs of different wooden furniture products and accessories. These will include various styles, types, and uses.  Refer to Knowledge Module KM13 and the additional brochure with examples of upholstered furniture.

2. You are required to examine each sample or photograph and perform the following tasks:

Assessment Criteria:

Assessment Duration:

- The assessment should be completed within a specified time frame, typically 2-3 hours, to simulate a realistic working environment.

Feedback and Evaluation:

- Detailed feedback will be provided on the accuracy and relevance of your categorisations.

- The assessor will highlight areas of strength and suggest improvements where necessary.

Additional Notes:

- This practical assessment is designed to test not only your ability to recognise and categorise different furniture types and accessories but also your understanding of the broader context in which these items are used.

- It is essential to approach this task with attention to detail and a thorough understanding of the varied aspects of upholstered furniture design and manufacturing.

### Case Study: New Line Development at Metropolitan Furniture Inc.

Scenario:

Metropolitan Furniture Inc. is in the process of developing a new line of furniture aimed at urban markets. The new line includes a range of furniture items from dining chairs to luxury sofas, each designed with specific uses and environments in mind. You, as a furniture design consultant, have been brought in to assist in ensuring that each piece meets specific design criteria based on material, structure, and functionality. Your insights will help guide the production team in creating furniture that is both aesthetically pleasing and structurally sound.

Tasks:

Your role involves the assessment and recommendation of materials, frame designs, and construction methods. You will need to apply your knowledge to identify and evaluate various aspects of furniture design as specified in the internal assessment criteria.

Questions:

Question 1: (IAC0201)

During a review of prototypes, you observe two different chair models: one is fully upholstered, and the other features visible wooden elements alongside upholstery. How would you identify and distinguish the frame components used in each chair?

Question 2: (IAC0202)

You are presented with four different seating designs intended for various uses: a recliner, a dining chair, a bus seat, and an entryway bench. How would you identify the seating frames according to their intended purposes?

Question 3: (IAC0203)

Examine a range of sofas and categorise them according to the four back styles described: camel, shell, pillow, and curved. Provide a rationale for each categorisation.

Question 4: (IAC0204)

Identify the frame type for each of the following furniture items: an office workstation, a portable massage table, and a sectional living room sofa.

Question 5: (IAC0205)

You need to recommend materials for a new line of bedroom furniture. Choose appropriate materials for the bed frames, dressers, and nightstands.

Question 6: (IAC0206)

A luxury dining set requires specific joints and bracings. What types would you recommend for the chairs and the dining table?

Question 7: (IAC0207)

When inspecting wood samples for a custom cabinet, how would you identify the long, short, and end grains of the wood to ensure optimal placement of fasteners?

Summary:

This case study not only tests your understanding of the materials and construction techniques but also your ability to apply these in practical, real-world design scenarios. Each answer should demonstrate thorough knowledge and a clear rationale that aligns with industry standards and practical application in furniture design.

EVALUATION CHECKLIST

| NAME: …………………………………….…. SURNAME: ……………………………….…. ID…………………………………………….... | LEARNER SIGNATURE DATE: |
| --- | --- |

| EVALUATION CRITERIONS | MET REQUIREMENTS | DID NOT MEET REQUIREMENTS | COMMENTS OR ACTION REQUIRED |
| --- | --- | --- | --- |
| Ability of the learner to: |  |  |  |

| GENERAL COMMENTS OF OBSERVER: |  |  |
| --- | --- | --- |
| Assessor Name ……………………………… | Signature ……………………………………. | Date …………………………………. |

## PM-02-PS03: Read and interpret specifications for upholstery covers

Scope of Practical Skill

Given a range of written specifications including technical drawings, cutting plans and job cards, and a variety of samples and prototypes of upholstered furniture, including dining room seats, arm chairs, sofas, corner units, ottomans and foot stools, and a variety of types of prepared covers of the seven main upholstery material types (woven plain, woven patterned, printed cretonne, pile, knitted jersey, coated fabrics and animal skins), and a cloth tape measure, the learner must be able to:

- PA0301 Identify the main components of furniture frames using industry standard terminology

- PA0302 Identify the main components of covers for furniture frames using industry standard terminology

- PA0303 Identify the type of cover material

- PA0304 Identify the dimensions of the cover components

- PA0305 Identify the final size and allowances for the cover components

- PA0306 Identify the characteristics of fabrics that influence cover fitting

- PA0307 Identify the characteristics of cover design that influence cover construction

- PA0308 Identify the construction process of each of the main groups of furniture

- PA0309 Identify the components of each construction

Applied Knowledge

- AK0301 Specification reading techniques using industry terminology and abbreviations

- AK0302 Measuring techniques

- AK0303 Cover material types

- AK0304 Fabric technology and design aesthetics

Internal Assessment Criteria

- IAC0301 Main components of furniture frames are identified and labelled: back, front, legs, rails, arms, corner blocks, joints, seat, wing, show wood

- IAC0302 Design information for cover components for furniture is identified from the specifications: inside wing (IW), outside wing (OW), inside arms (IA), outside arms (OA), seat back (SB), seat front, (SF), inside back (IB), outside back (OB), front border (FB), facia (F), cushion back (CB), cushion front (CF), piping (P)

- IAC0303 Dimensions, including allowances, of cover components are correctly interpreted (length first followed by the width) and recorded in millimetres

- IAC0304 Cover materials are classified according to the main types of upholstery cover materials (woven plain, woven patterned, printed cretonne, pile, knitted jersey, coated fabrics and animal skins)

- IAC0305 Pile direction is correctly identified

- IAC0306 Pattern repeats are correctly identified

IAC0307 Repeat patterns are matched taking into consideration highly visible areas first, and in the right order (inside backs, seat cushions, fronts of seats and then inside arms)

### Practical Learning Content for PA0301: Identify the Main Components of Furniture Frames Using Industry Standard Terminology

The ability to correctly identify and name the main components of furniture frames is essential for professionals in the furniture design and manufacturing industries. This skill ensures clear communication and precision in the construction and upholstery processes. Below, we provide detailed practical learning content and examples to help learners master the identification of furniture frame components using industry-standard terminology.

1. Overview of Furniture Frame Components

Understanding the structure of furniture frames involves recognising various key components:

- Rails: Horizontal support elements that connect the legs of the furniture. Rails can be categorised into top rails, bottom rails, and mid rails depending on their position.

- Legs: The vertical components that support the furniture from the ground. Legs can vary widely in style and may be integrated into the frame or attached separately.

- Stiles: Vertical pieces that form the main structure of the frame, particularly in chairs and panel furniture.

- Aprons: Horizontal pieces that connect legs below the surface of the furniture, adding structural stability and often contributing to the aesthetic of the piece.

- Stretchers: Support bars that run between the legs, usually at or near the bottom of a frame, to provide additional stability.

- Panels: Flat sections that fill the frame, often seen in the sides or back of furniture.

2. Practical Exercises

Exercise 1: Frame Component Identification

- Task: Learners are presented with various furniture frames, both assembled and unassembled. They must identify and label each component using the correct industry terminology.

- Objective: To develop the ability to recognise and correctly name the structural elements of furniture frames.

Exercise 2: Role-Playing as Quality Inspectors

- Task: In a simulated quality inspection setting, learners must examine a series of furniture pieces and report on the construction by referencing specific frame components.

- Objective: This exercise helps learners apply their knowledge in a practical context, ensuring they understand the importance of each component's role in overall furniture integrity.

3. Real-World Application

Field Trip: Arrange a visit to a local furniture manufacturing facility. Observing the assembly line and discussing with craftsmen and designers will provide learners with practical insights into how frame components are integrated into finished products.

Group Project: Learners design a simple piece of furniture, such as a chair or table. They must create a blueprint that includes all frame components, using correct terminology, and then build a prototype based on their design.

4. Evaluation and Feedback

Rubric for Exercises:

- Identification Accuracy: Correctly identifying and naming frame components.

- Application in Inspection: Effectiveness in applying terminology to real furniture pieces during quality checks.

- Creativity and Accuracy in Design: Innovativeness and precision in designing and building the prototype.

By engaging with these educational activities, learners will gain a thorough understanding of the different components that make up furniture frames. This knowledge is crucial for ensuring that furniture is not only aesthetically pleasing but also structurally sound and built to last.

### Practical Learning Content for PA0302: Identify the Main Components of Covers for Furniture Frames Using Industry Standard Terminology

Being able to accurately identify and describe the components of furniture covers is essential for professionals in the upholstery industry. This skill ensures that they can communicate effectively about their work and meet design and fabrication specifications precisely. This module provides detailed practical learning content and examples to help learners master the identification of cover components using industry-standard terminology.

1. Overview of Cover Components

A comprehensive understanding of the various parts that make up furniture covers is critical for successful upholstery work:

- Body Fabric: The main fabric that covers the largest visible areas of the furniture.

- Welting (or Piping): Fabric-covered cord that is sewn along the seams or edges of the upholstery to define shapes and edges, adding strength and decoration.

- Deck: The fabric that is placed underneath the seat cushions, extending from the front edge of the seat to where it meets the back.

- Skirt: A fabric panel that hangs from the edge of the seat or the base of the furniture to the floor, often used to conceal legs or casters.

- Gusset: The piece of fabric that connects the top and bottom panels of a cushion, often seen in boxed-edge cushions.

- Panels: Separate pieces of fabric that are sewn together to form the cover, tailored to fit specific areas like the arms, back, or sides of furniture.

2. Practical Exercises

Exercise 1: Cover Component Identification

- Task: Learners are provided with several dismantled furniture covers. They must identify and label each component, using the correct industry terminology.

- Objective: To enhance learners' ability to recognise and name the different parts of furniture covers correctly.

Exercise 2: Matching Covers to Furniture

- Task: Given a variety of furniture types and their corresponding covers, learners must match each cover to the correct piece of furniture, identifying and explaining the purpose of each component.

- Objective: This exercise helps learners understand how different components fit and function in various furniture designs.

3. Real-World Application

Workshop Activity: Arrange a hands-on workshop in an upholstery studio where learners can observe and participate in the cover fitting process. This allows them to see how different components are assembled and fitted onto furniture frames.

Group Project: Learners work in teams to design and create a furniture cover for a given piece of furniture, such as a chair or a small sofa. They must draft a design that includes all necessary components and then construct the cover, applying the terminology they've learned.

4. Evaluation and Feedback

Rubric for Exercises:

- Accuracy of Identification: Precision in identifying and labelling the components of furniture covers.

- Application and Matching: Effectiveness in matching covers to furniture pieces and correctly explaining component functions.

- Creativity and Technical Skill in Project: Innovation in design and skill in executing the cover construction for the group project.

By engaging with these educational activities, learners will gain a deep understanding of the components involved in furniture covers. This knowledge is crucial for ensuring that upholstery work is not only functional and attractive but also meets specific design and structural requirements.

### Practical Learning Content for PA0303: Identify the Type of Cover Material

Accurately identifying different types of upholstery cover materials is essential for professionals in the furniture manufacturing and upholstery industry. This skill ensures that they can choose appropriate materials based on the functionality and aesthetic requirements of the furniture. This module provides practical learning content with examples to help learners recognise and differentiate various upholstery materials.

1. Overview of Upholstery Cover Materials

Understanding the properties and applications of different types of upholstery materials is crucial for selecting the right fabric for a specific furniture piece:

- Woven Plain Fabrics: Characterised by simple weave patterns such as plain weave, twill, or satin. These fabrics are versatile and durable, suitable for general upholstery.

- Woven Patterned Fabrics: Feature intricate designs woven directly into the fabric using multiple yarns of different colours. Ideal for statement pieces or decorative accents.

- Printed Cretonne: A plain-weave fabric made from cotton featuring vibrant prints. It's less durable than other woven fabrics but popular for its aesthetic appeal in low-traffic areas.

- Pile Fabrics: Include materials like velvet or corduroy that have a raised surface or 'pile', known for their texture and luxurious feel. They are used in both high-end and comfortable furniture designs.

- Knitted Jersey: A stretchable material made from knitting fibers together. While not commonly used for main furniture covers, it may be used for specialised furnishings that require stretch.

- Coated Fabrics: Fabrics such as vinyl or leatherette that have a protective coating to provide durability and ease of maintenance. Suitable for high-traffic or commercial environments.

- Animal Skins: Including leather and suede, these are premium materials known for their durability, comfort, and high-end appeal. They require specific maintenance but age well and are highly valued in furniture upholstery.

2. Practical Exercises

Exercise 1: Material Identification Lab

- Task: Learners are given samples of each type of material listed above. They must identify each material and discuss its typical use cases in upholstery.

- Objective: To enhance learners' ability to recognise different materials and understand their applications in upholstery.

Exercise 2: Material Suitability Challenge

- Task: Given a series of client scenarios (e.g., a family with young children, a high-end hotel lobby, a home theatre room), learners must choose the most suitable upholstery material for each case.

- Objective: This exercise helps learners apply their knowledge to practical scenarios, choosing materials that best fit the client's needs and the furniture's intended use.

3. Real-World Application

Field Trip: Organise a visit to a local fabric warehouse or upholstery shop. This allows learners to see and touch various upholstery materials and understand how they are used in actual furniture pieces.

Group Project: Learners design a small piece of furniture (e.g., an ottoman or side chair) for a specific setting. They must select an appropriate cover material, justify their choice based on the material’s properties, and demonstrate how it fits into the overall design concept.

4. Evaluation and Feedback

Rubric for Exercises:

- Identification Accuracy: Correctly identifying each type of material and its characteristics.

- Application Effectiveness: Appropriateness of material selection for given scenarios.

- Innovation and Design Rationale: Creativity in the design project and the rationale behind material choices.

Through these educational activities, learners will gain a comprehensive understanding of the various types of upholstery materials and their suitable applications. This knowledge is essential for making informed decisions that affect the aesthetics, functionality, and durability of upholstered furniture.

### Practical Learning Content for PA0304: Identify the Dimensions of the Cover Components

Accurately identifying and measuring the dimensions of upholstery cover components is a fundamental skill for professionals in the furniture manufacturing and upholstery industry. This precision is crucial for ensuring that covers fit perfectly and look professional. This module provides practical learning content with examples to help learners effectively measure and understand the dimensions of various upholstery components.

1. Overview of Measuring Upholstery Components

Proper measurement techniques are essential to ensure that fabric covers fit the furniture frame correctly, taking into account not only the size but also allowances for seams, stretching, and other adjustments:

- Measuring Tape: A flexible ruler used to measure the dimensions of furniture. It can conform to the shapes and contours of upholstery, providing accurate measurements.

- Dimensions to Measure:

- Width: Measure across the seat, back, or arm horizontally.

- Depth: Measure from the front to the back of the seat or component.

- Height: Measure from the base to the highest point of the component (for backs or arms).

- Seam Allowances: Add extra fabric to the measured dimensions to account for seams where the fabric will be sewn together. Standard seam allowances might vary, but a common allowance is about 1/2 inch to 1 inch.

- Fitting Allowances: Additional fabric may be needed for tucking and fitting, especially in upholstered seats and backs where the fabric needs to wrap around the cushions or padding.

2. Practical Exercises

Exercise 1: Measurement Practice

- Task: Learners are provided with a variety of furniture pieces. They must use a cloth measuring tape to measure the dimensions of different components, such as seats, backs, and arms, and record these measurements accurately.

- Objective: To practice measuring techniques and understand how to apply seam and fitting allowances.

Exercise 2: Mock-up Creation

- Task: Using the measurements taken, learners create a paper or fabric mock-up of one of the furniture covers, adjusting for seam and fitting allowances.

- Objective: This exercise helps learners understand the importance of accurate measurements and allowances in creating a well-fitting upholstery cover.

3. Real-World Application

Workshop Activity: Arrange a session in an upholstery workshop where learners can participate in the process of measuring, cutting, and fitting upholstery fabrics onto actual furniture pieces.

Group Project: Learners work in teams to upholster a piece of furniture, starting from taking measurements to cutting the fabric and fitting the cover. They must document each step, focusing on how they applied their measurements and adjusted for allowances.

4. Evaluation and Feedback

Rubric for Exercises:

- Accuracy of Measurements: Precision in measuring various components.

- Application of Allowances: Effectiveness in applying seam and fitting allowances in mock-up creation.

- Execution and Problem Solving: Skill and creativity in the group upholstery project, particularly in handling challenges related to measurements and fit.

Through these activities, learners will gain a deep understanding of the complexities involved in measuring and preparing upholstery covers. This knowledge is essential for ensuring that the final product not only fits well but also meets the aesthetic and functional standards expected in professional upholstery.

### Practical Learning Content for PA0305: Identify the Final Size and Allowances for the Cover Components

Understanding how to determine the final size and allowances for upholstery cover components is essential for ensuring a perfect fit and professional finish in furniture manufacturing and upholstery. This module provides practical learning content with examples, focusing on how learners can accurately calculate and apply these measurements.

1. Overview of Determining Final Size and Allowances

Proper measurement and allowance calculation ensure that upholstery covers fit precisely, accounting for fabric behaviour and the specific requirements of the furniture piece:

- Final Size Calculation: Involves adding necessary allowances to the basic dimensions (width, depth, height) of the furniture component to accommodate seams, fitting, and any stylistic additions.

- Seam Allowance: Typically about 1/2 inch to 1 inch added around each edge where the fabric will be sewn to another piece of fabric. This space is crucial for creating durable seams without distorting the fit.

- Fitting Allowance: Extra fabric added to allow for ease in fitting the cover onto the furniture. This is particularly important for materials with less stretch or for covers that require a snug fit.

- Pattern Matching Allowance: Additional fabric required when working with patterned materials to ensure that patterns align correctly across seams. This may vary depending on the repeat size of the pattern.

2. Practical Exercises

Exercise 1: Calculating Allowances

- Task: Learners are given the base dimensions of various furniture components. They must calculate the final size for each component, incorporating seam, fitting, and pattern matching allowances.

- Objective: To practice and understand the calculation of different types of allowances needed for upholstery covers.

Exercise 2: Fabric Cutting Simulation

- Task: Using the final dimensions calculated in Exercise 1, learners cut mock-up fabric pieces (using paper or inexpensive fabric) for different furniture components.

- Objective: This exercise helps learners apply their calculations in a practical scenario, understanding the impact of allowances on the material requirements and the fitting process.

3. Real-World Application

Workshop Activity: Arrange a workshop in an upholstery studio where learners can observe and participate in the measuring, cutting, and fitting process, applying their knowledge of allowances to real furniture pieces.

Group Project: Learners team up to design and create upholstery covers for a set piece of furniture. Each group is responsible for measuring, calculating allowances, cutting fabric, and fitting the cover, documenting their process and decisions.

4. Evaluation and Feedback

Rubric for Exercises:

- Accuracy of Calculations: Precision in the calculation of final sizes and allowances.

- Skill in Material Handling: Effectiveness in cutting and handling materials based on calculated dimensions.

- Collaboration and Problem Solving: Teamwork in the group project and ability to solve fitting issues that arise from allowance considerations.

By engaging with these educational activities, learners will develop a thorough understanding of how to calculate and apply the necessary allowances for creating upholstery covers. This knowledge ensures that the upholstery not only fits well but also maintains the aesthetic integrity and functional requirements of the furniture design.

### Practical Learning Content for PA0306: Identify the Characteristics of Fabrics that Influence Cover Fitting

For upholstery professionals, understanding how the inherent characteristics of fabrics influence cover fitting is essential for achieving the best results in furniture upholstery. This module provides detailed practical learning content with examples to help learners identify and evaluate these fabric characteristics effectively.

1. Overview of Fabric Characteristics Influencing Cover Fitting

Different fabrics bring unique challenges and advantages to upholstery projects. Key characteristics to consider include:

- Stretchability: Fabrics with high elasticity, like some knits and woven fabrics with elastane, can conform more easily to complex shapes but may also be more challenging to work with to maintain tension and avoid warping.

- Weight and Thickness: Heavier and thicker fabrics offer durability and better coverage but may be harder to manipulate and fit tightly in corners and curves. Lighter fabrics might be easier to handle but can wear out more quickly or show imperfections in the upholstery base.

- Texture and Pile Direction: Fabrics with a distinct texture or pile (like velvet) need careful handling to ensure the pile direction is consistent across all pieces of the cover, affecting both the aesthetics and the feel of the finished product.

- Weave Tightness: Tightly woven fabrics are less likely to stretch or distort and can provide a crisp, tailored look. Loosely woven fabrics might offer a softer look but can be prone to stretching or sagging.

- Moisture Absorption and Shrinkage: Natural fibres like cotton and linen can absorb moisture and shrink. It's important to pre-wash these materials or account for potential shrinkage in the sizing of covers.

2. Practical Exercises

Exercise 1: Fabric Performance Testing

- Task: Learners are provided with fabric samples displaying different characteristics. They must perform tests to evaluate stretchability, weight, and texture effects on cover fitting.

- Objective: To understand how different fabric qualities impact the ease of fitting and the final appearance of upholstery.

Exercise 2: Material Selection Scenario

- Task: Given several client scenarios (e.g., high-traffic family living room, luxury hotel lobby, sunny reading nook), learners must choose appropriate fabrics based on the characteristics discussed and justify their choices.

- Objective: This exercise helps learners apply their knowledge of fabric characteristics to make informed choices tailored to specific usage scenarios and client needs.

3. Real-World Application

Workshop Activity: Arrange a hands-on fitting session in an upholstery workshop where learners can practice applying fabrics with various characteristics to furniture pieces. Observing the effects of different material choices on the fitting process and the final product will reinforce theoretical knowledge.

Group Project: Learners work in teams to reupholster a piece of furniture, selecting fabric based on its characteristics and the intended use of the furniture. They must document their process, noting how fabric choice affected their approach and the outcomes.

4. Evaluation and Feedback

Rubric for Exercises:

- Understanding and Application: Accuracy in identifying fabric characteristics and predicting their effects on upholstery.

- Decision Making: Effectiveness in selecting appropriate fabrics for given scenarios and justifying those choices.

- Technical Skill and Creativity: Quality of upholstery work in the group project, especially how well the team managed fabric characteristics during the fitting process.

Through these educational activities, learners will gain a comprehensive understanding of the critical role fabric characteristics play in upholstery. This knowledge ensures that professionals can choose the best materials for their projects, leading to higher quality, durability, and customer satisfaction in their upholstery work.

### Practical Learning Content for PA0307: Identify the Characteristics of Cover Design that Influence Cover Construction

Understanding the characteristics of cover design is crucial for upholstery professionals to ensure that the covers not only fit well but also enhance the aesthetic and functional quality of the furniture. This module provides detailed practical learning content with examples to help learners identify key design features that influence the construction process of upholstery covers.

1. Overview of Cover Design Characteristics

Several design characteristics are critical when planning and constructing upholstery covers. These include:

- Seam Placement: Determines how pieces of fabric are joined. Strategic seam placement can enhance durability and affect the visual appeal of the furniture. For instance, seams should align with structural elements of the furniture to reinforce strength and improve shape retention.

- Pattern Direction and Symmetry: Important in patterned fabrics where visual continuity is key. Patterns should be aligned and symmetrical from one section of the furniture to another to ensure a cohesive look.

- Darts and Pleats: Used to fit fabric smoothly over curved or angular sections of furniture. Darts remove excess fabric, providing a flat fit, while pleats gather fabric, allowing for expansion and shape accommodation.

- Sippers and Closures: Placement of sippers and other closures affects both the functionality and the ease of removing and fitting the cover. They should be strategically placed for easy access while maintaining the aesthetic integrity of the design.

- Reinforcements: Areas subject to high wear, such as seat corners, may require additional reinforcement to prevent tearing and wear. This can involve double stitching or the addition of a more durable material patch.

2. Practical Exercises

Exercise 1: Design Analysis

- Task: Learners are given different cover designs, including technical drawings and finished samples. They must analyse and identify key design characteristics and their implications for the construction process.

- Objective: To enhance learners' ability to interpret design documents and predict construction challenges.

Exercise 2: Mock-up Creation

- Task: Using fabric and basic sewing materials, learners create a simple mock-up of an upholstery cover that incorporates various design features such as seams, darts, and sippers.

- Objective: This hands-on activity helps learners understand the practical implications of design decisions in upholstery cover construction.

3. Real-World Application

Workshop Activity: Arrange a workshop in an upholstery studio where learners can participate in the design and construction of a furniture cover from start to finish. Focus on how different design characteristics are handled in a real upholstery setting.

Group Project: Learners work in teams to design and construct a cover for a piece of furniture. They must consider all aspects of the cover's design, from material choice to the placement of seams and sippers, documenting their decisions and the reasons behind them.

4. Evaluation and Feedback

Rubric for Exercises:

- Design Comprehension: Accuracy in identifying and explaining the impact of design features on construction.

- Practical Skill Application: Quality of the mock-up and how well the learner managed to incorporate design features.

- Creativity and Problem-Solving: Innovation in the group project design and effectiveness in addressing construction challenges.

Through these educational activities, learners will develop a comprehensive understanding of how different elements of cover design affect the upholstery process. This knowledge is essential for creating high-quality, functional, and attractive upholstery covers, ensuring they meet both aesthetic standards and durability requirements.

### Practical Learning Content for PA0308: Identify the Construction Process of Each of the Main Groups of Furniture

Understanding the construction processes specific to different groups of furniture is crucial for professionals in the furniture design and manufacturing industry. This module provides detailed practical learning content with examples to help learners grasp the varied processes involved in constructing different types of furniture.

1. Overview of Furniture Construction Processes

The construction process varies significantly across different types of furniture, influenced by the intended use, structural requirements, and design preferences:

- Sofas and Armchairs: These typically involve a frame construction, padding addition, and finally the upholstery. The frame is usually made of wood or metal, padding consists of foam or other cushioning materials, and the upholstery can be any fabric chosen based on the furniture’s use and aesthetic.

- Tables and Desks: Construction starts with the frame, often involving legs and a support structure, followed by attaching the top surface, which may be wood, glass, or metal. Joints are crucial for stability and can include techniques like mortise and tenon or dowels.

- Cabinetry (e.g., Wardrobes, Bookcases): Focuses on precise joinery to ensure stability and optimise storage. Construction typically involves assembling sides, shelves, and a back before adding doors and hardware. Techniques such as dado and rabbet joints are common for securing shelves.

- Beds: Include building a headboard (if applicable), frame, and base (which may include slats or a solid platform). The complexity can vary with design, from simple platforms to elaborate frames with additional features like storage or folding mechanisms.

2. Practical Exercises

Exercise 1: Construction Simulation

- Task: Learners are given diagrams and materials to simulate the construction process of a simple piece of furniture, such as a small table or a chair, focusing on understanding each step and the sequence of assembly.

- Objective: To provide hands-on experience with basic construction techniques and the assembly order.

Exercise 2: Group Construction Project

- Task: In teams, learners design and build a more complex piece of furniture, such as a sofa or a bed frame. Each team is responsible for planning their construction process, selecting materials, and executing the build.

- Objective: To deepen understanding of the construction processes for more complex furniture, emphasising problem-solving and teamwork.

3. Real-World Application

Site Visit: Arrange a tour of a furniture manufacturing facility where different types of furniture are constructed. This allows learners to observe firsthand the various construction processes and machinery involved.

Industry Speaker: Invite a seasoned furniture maker to discuss the nuances of furniture construction, focusing on advanced techniques and material considerations for high-end or specialised furniture.

4. Evaluation and Feedback

Rubric for Exercises:

- Technical Skills: Accuracy and quality of construction in both the simulation and group project.

- Understanding of Processes: Depth of understanding shown in planning and executing furniture construction.

- Innovation and Problem Solving: Creativity in design and effectiveness in solving construction challenges.

By engaging with these educational activities, learners will gain a comprehensive understanding of the construction processes for various types of furniture. This knowledge is crucial for ensuring that the finished products are not only aesthetically pleasing but also structurally sound and functionally effective.

### Practical Learning Content for PA0309: Identify the Components of Each Construction

For professionals in the furniture manufacturing and upholstery industry, being able to accurately identify and understand the components that make up different furniture constructions is essential. This module provides detailed practical learning content with examples to help learners master the identification of components for various furniture constructions.

1. Overview of Components in Furniture Construction

Each type of furniture has a set of specific components that are crucial to its structure and function:

- Sofas and Chairs: Components typically include the frame (legs, arms, back, and rails), seating support (webbing, springs), padding, and upholstery. The frame is the skeleton on which everything else is built, supports provide comfort and durability, padding shapes the furniture, and upholstery defines its final appearance and texture.

- Tables and Desks: Main components include the tabletop, legs, aprons (the frame pieces connecting the legs under the table), and possibly drawers or extensions. The stability of a table largely depends on the strength and design of these components.

- Cabinetry (Wardrobes, Bookcases, etc.): Components include the carcass (sides, top, bottom), back panel, shelves, doors, and hardware (hinges, handles). The carcass provides the basic structure, while doors and hardware facilitate access and security.

- Beds: Key components are the headboard, footboard (if applicable), side rails, and the base (which can include slats, panels, or a box spring area). Each part contributes to both the aesthetic and functional integrity of the bed.

2. Practical Exercises

Exercise 1: Component Identification Drill

- Task: Learners are provided with various pieces of furniture either as physical items or detailed diagrams. They must identify and list all the components of each piece.

- Objective: To enhance the learners’ ability to recognise and name the different parts of furniture accurately.

Exercise 2: Component Function Analysis

- Task: After identifying the components, learners analyse the function of each component in a given piece of furniture and discuss how these functions impact the overall design and use of the furniture.

- Objective: This exercise helps learners understand not just what the components are, but why they are important to the furniture’s construction and function.

3. Real-World Application

Workshop Activity: Organise a session in a local workshop or school shop where learners can disassemble and then reassemble pieces of furniture. This hands-on activity helps them see firsthand how the components fit together and function as a whole.

Design Project: Learners are tasked with designing a piece of furniture. They must specify all components, explain their choices, and describe how each component contributes to the design and function of the final piece.

4. Evaluation and Feedback

Rubric for Exercises:

- Accuracy in Identification: Precision and correctness in identifying furniture components.

- Understanding of Functionality: Depth of understanding displayed in the function analysis of each component.

- Creativity and Practical Application: Innovation in the design project and effectiveness in applying theoretical knowledge to practical construction.

Through these educational activities, learners will develop a thorough understanding of the components involved in furniture construction. This knowledge is crucial for anyone involved in the design, manufacturing, or repair of furniture, as it ensures that they can create and maintain high-quality, functional pieces.

### Applied Knowledge Learning Content for AK0301: Specification Reading Techniques Using Industry Terminology and Abbreviations

For professionals in the furniture design and manufacturing industry, the ability to accurately read and interpret specifications is crucial. This skill ensures that furniture is made to exact standards and meets both functional and aesthetic requirements. This module provides practical learning content with examples to help learners master specification reading techniques using industry terminology and abbreviations.

1. Overview of Specification Reading

Specifications for furniture typically include detailed instructions on dimensions, materials, assembly methods, and finishing touches. Understanding these documents requires familiarity with common industry terminology and abbreviations.

- Dimensions and Tolerances: Specifications often detail the exact dimensions of each component, along with allowable tolerances (e.g., ±2mm).

- Material Specifications: These include types of wood, fabric, metal, etc., often abbreviated (e.g., MDF for Medium-Density Fiberboard).

- Assembly Instructions: Detailed steps on how to assemble the furniture, including which fasteners or adhesives to use.

- Finishing Specifications: Instructions on sanding, painting, staining, or sealing the finished piece.

- Abbreviations and Symbols: Common abbreviations include DIM (dimension), TOL (tolerance), FT (feet), IN (inches), MM (millimeters), and PCS (pieces).

2. Practical Exercises

Exercise 1: Specification Decoding Drill

- Task: Learners are provided with a series of specification sheets for different pieces of furniture. They must decode the specifications, translating abbreviations and technical language into plain English.

- Objective: To practice and improve the ability to quickly and accurately interpret specification sheets.

Exercise 2: Mock Specification Creation

- Task: Learners create a complete specification for a simple piece of furniture, such as a coffee table or bookshelf. They must use correct industry terminology and abbreviations.

- Objective: This exercise helps learners apply their knowledge in a practical setting, ensuring they can communicate effectively and professionally in written specifications.

3. Real-World Application

Interactive Webinar: Host a webinar with a professional technical writer or a seasoned furniture designer who specialises in creating specifications. This provides learners with insights into the best practices in specification writing and interpretation.

Group Project: Learners are divided into teams to interpret a set of complex furniture specifications and then build a model or prototype based on their interpretation. They must document their process, highlighting how they applied the specification details.

4. Evaluation and Feedback

Rubric for Exercises:

- Accuracy of Interpretation: How correctly learners interpret and apply specifications.

- Clarity and Precision in Documentation: The quality of the specifications created by the learners, including their use of industry terminology and abbreviations.

- Innovation and Application: Creativity in the design and construction process during the group project, and effectiveness in applying specification details.

Through these activities, learners will develop a deep understanding of how to read and apply specifications in the furniture manufacturing process. This knowledge is essential for ensuring that products are built correctly and meet predefined quality standards, leading to better outcomes in both custom and mass-production environments.

### Applied Knowledge Learning Content for AK0302: Measuring Techniques

Accurate measurement is fundamental in furniture design and manufacturing, ensuring that pieces are not only aesthetically pleasing but also functional and fit for purpose. This module provides detailed learning content on measuring techniques, complete with examples to help learners apply these skills effectively in real-world scenarios.

1. Overview of Essential Measuring Techniques

Precise measurements are critical in every stage of furniture making, from initial design to final assembly. Mastery of various measuring tools and techniques is essential:

- Using a Tape Measure: The flexible tape measure is commonly used for taking linear measurements of furniture, such as height, width, and depth. It is suitable for curved and irregular surfaces as well.

- Calipers and Micrometers: These tools provide highly accurate measurements and are especially useful for measuring thickness and small dimensions that require precision to the millimeter or smaller.

- Square and Ruler: Used for ensuring angles are correct and for measuring and marking straight cuts. A combination square also allows for depth measurements.

- Digital Measuring Tools: Laser measures and digital calipers can provide quick and highly accurate readings and are particularly useful in large-scale production environments.

2. Practical Exercises

Exercise 1: Measurement Accuracy Drill

- Task: Learners practice measuring various components of furniture using different tools (tape measure, ruler, calipers). They measure dimensions such as length, width, thickness, and diagonal measurements to check for squareness.

- Objective: To develop proficiency in using various measuring tools and understanding which tool is appropriate for different measuring tasks.

Exercise 2: Real-World Measurement Application

- Task: Learners are given a small project, such as designing and creating a simple piece of furniture. They must take all necessary measurements, mark the materials accurately, and cut them according to their marks.

- Objective: This exercise helps learners apply their measuring skills in a practical setting, emphasising the importance of accuracy in ensuring the final product's quality and fit.

3. Real-World Application

Field Trip: Arrange a visit to a local furniture manufacturing facility or a professional woodshop. Observe professional craftsmen at work, focusing on how they measure and mark materials.

Interactive Workshop: Conduct a workshop where learners can engage in a furniture building task, starting from reading a blueprint to measuring, marking, and cutting materials. Each step should be supervised and feedback provided on their measuring techniques.

4. Evaluation and Feedback

Rubric for Exercises:

- Precision of Measurements: Accuracy in obtaining and recording measurements.

- Correct Tool Usage: Appropriateness of measuring tool selection for different tasks.

- Execution and Craftsmanship: Quality of the cuts and assembly based on accurate measurements, particularly in the real-world application exercise.

Through these educational activities, learners will gain comprehensive skills in measuring techniques essential for furniture design and manufacturing. These skills ensure that furniture pieces are not only built to specifications but also meet high standards of quality and precision.

### Applied Knowledge Learning Content for AK0303: Cover Material Types

Understanding different types of cover materials is crucial for professionals in the upholstery industry. Each material type brings unique properties that can significantly affect the appearance, comfort, and durability of upholstered furniture. This module provides practical learning content on various cover material types, along with examples to help learners apply this knowledge effectively in their upholstery projects.

1. Overview of Cover Material Types

Cover materials vary widely and are chosen based on factors such as the intended use of the furniture, client preferences, budget, and the overall design aesthetic. Here are some common types of cover materials:

- Woven Fabrics: These include materials like cotton, linen, and wool, which offer a traditional look and feel. They are versatile and can come in various weaves that affect their texture and durability.

- Synthetic Fabrics: Such as polyester, nylon, and acrylic, which are generally more durable and easier to maintain than natural fibers. They are often used in commercial settings or in homes with pets and children due to their stain resistance and durability.

- Leather: Known for its durability, ease of maintenance, and premium look. Leather ages well and is preferable in high-end furniture designs. Varieties include full-grain, top-grain, and bonded leather.

- Vinyl: A less expensive alternative to leather that is easy to clean and maintain. It is commonly used in both residential and commercial settings where durability and budget are key considerations.

- Microfiber: A densely woven synthetic fabric known for its softness and durability. It is highly stain-resistant and often used for furniture that requires a lot of maintenance, such as children's furniture.

- Velvet: Known for its luxurious texture and depth of color, velvet is a popular choice for statement pieces. However, it requires more care to maintain its look over time.

2. Practical Exercises

Exercise 1: Material Identification and Application

- Task: Learners are given samples of various upholstery materials and are asked to identify each type and discuss its properties, advantages, and ideal usage scenarios.

- Objective: To develop an understanding of different upholstery materials and their applications, enhancing the ability to make informed choices based on project requirements.

Exercise 2: Material Durability and Maintenance Testing

- Task: Learners perform tests on different materials to assess their durability and ease of maintenance. This might include stain resistance tests, wear and tear simulations, and cleanability assessments.

- Objective: To understand the practical implications of each material type in real-world settings, focusing on long-term use and maintenance.

3. Real-World Application

Field Trip: Arrange a visit to a fabric mill or upholstery workshop where different types of cover materials are used. This allows learners to see the materials in a larger context and understand how they are processed and applied.

Project-Based Learning: Learners select a piece of furniture to reupholster, choosing an appropriate material based on their assessment of the client's needs and the furniture's use. They must justify their choice and document the reupholstery process.

4. Evaluation and Feedback

Rubric for Exercises:

- Knowledge of Materials: Accuracy in identifying materials and understanding their properties.

- Practical Application: Effectiveness in applying knowledge of materials to choose suitable options for different uses.

- Project Execution and Justification: Quality of the reupholstery project and the rationale behind material choices.

By engaging with these educational activities, learners will develop a comprehensive understanding of the types of cover materials available and their appropriate applications. This knowledge is crucial for ensuring that upholstery projects meet both functional requirements and aesthetic expectations.

### Applied Knowledge Learning Content for AK0304: Fabric Technology and Design Aesthetics

For professionals in the upholstery and interior design industries, understanding the intersection of fabric technology and design aesthetics is crucial. This module provides practical learning content on how advances in fabric technology enhance design aesthetics and functionality, complete with examples to help learners apply this knowledge to their projects.

1. Overview of Fabric Technology and Design Aesthetics

Fabric technology encompasses the latest advancements in material engineering that make fabrics more durable, easier to maintain, and environmentally friendly, while design aesthetics focus on the visual and tactile qualities that appeal to sensory perceptions and meet design trends.

- Performance Fabrics: These are engineered to resist stains, moisture, and UV light while retaining a pleasant aesthetic quality. Examples include microfiber and crypton fabrics, which are popular in both residential and commercial upholstery due to their durability and ease of maintenance.

- Eco-friendly Fabrics: Made from sustainable materials such as organic cotton, bamboo, or recycled polyester. These fabrics are gaining popularity due to increased environmental awareness and consumer demand for sustainable products.

- Smart Textiles: Incorporate technology to add functionality to the fabric. Examples include temperature-regulating fabrics that adjust to body heat and light-emitting fabrics that can change color or pattern based on environmental stimuli.

- Textile Prints and Patterns: Advances in digital printing technology have allowed for more complex and vibrant designs on fabric, enhancing the aesthetic flexibility for designers.

2. Practical Exercises

Exercise 1: Performance Fabric Testing

- Task: Learners are provided with samples of various performance fabrics. They conduct tests to evaluate stain resistance, durability, and ease of cleaning.

- Objective: To understand the practical benefits of different performance fabrics and how they can be incorporated into design projects.

Exercise 2: Eco-friendly Material Analysis

- Task: Learners research and present on different eco-friendly fabrics, discussing their production processes, benefits, and potential applications in upholstery.

- Objective: To promote awareness of sustainable materials and their impact on design and functionality.

3. Real-World Application

Design Project: Learners are tasked with designing a piece of furniture for a specific setting (e.g., a high-traffic café, a luxury hotel lobby, or a sustainable home). They must choose appropriate fabrics based on their functionality, aesthetic qualities, and the project's environmental impact.

Workshop: Host a workshop with a textile manufacturer or a fabric technologist to discuss the latest trends and technologies in fabric design. This provides learners with insights into how fabric technologies are developed and applied in the industry.

4. Evaluation and Feedback

Rubric for Exercises:

- Understanding of Fabric Technologies: Accuracy in explaining the properties and benefits of various fabric technologies.

- Application in Design: Effectiveness in selecting and justifying fabric choices for the design project.

- Innovation and Sustainability: Creativity in design and the use of sustainable practices.

Through these educational activities, learners will gain a thorough understanding of how fabric technology can enhance the functionality and aesthetic appeal of upholstery materials. This knowledge is essential for creating high-quality, durable, and visually appealing furniture that meets modern demands and trends.

### GUIDELINES

You are required to perform the task below and your Facilitator/Assessor will assess your work based on the observation checklist attached.

Practical Assessment Task for PM-02-PS03: Read and interpret specifications for upholstery covers

Assessment Title:

Purpose

Guidelines to the Learner:

1. You will be presented with a range of samples or photographs of different wooden furniture products and accessories. These will include various styles, types, and uses.  Refer to Knowledge Module KM13 and the additional brochure with examples of upholstered furniture.

2. You are required to examine each sample or photograph and perform the following tasks:

Assessment Criteria:

Assessment Duration:

- The assessment should be completed within a specified time frame, typically 2-3 hours, to simulate a realistic working environment.

Feedback and Evaluation:

- Detailed feedback will be provided on the accuracy and relevance of your categorisations.

- The assessor will highlight areas of strength and suggest improvements where necessary.

Additional Notes:

- This practical assessment is designed to test not only your ability to recognise and categorise different furniture types and accessories but also your understanding of the broader context in which these items are used.

- It is essential to approach this task with attention to detail and a thorough understanding of the varied aspects of upholstered furniture design and manufacturing.

### Case Study: Refurbishment of a Vintage Armchair

Scenario:

Metropolitan Design House has undertaken a project to refurbish a vintage armchair that will be featured in an upcoming home design exhibition. The armchair requires complete reupholstery using high-quality materials and a sophisticated design that respects its original style while incorporating modern durability and comfort features. Your task is to guide the refurbishment process from analysing the existing furniture frame to selecting and applying the appropriate fabrics.

Tasks:

Your role involves identifying and labeling various components of the furniture frame, interpreting and applying specifications for new cover components, and choosing the correct materials and patterns to achieve a high-quality finish.

Scenario-Based Questions:

Question 1: (IAC0301)

You are provided with a diagram of the armchair’s frame. Identify and label the following components:

- Back, Front, Legs, Rails, Arms, Corner Blocks, Joints, Seat, Wing, Show Wood

Question 2: (IAC0302)

Based on the specifications provided for the new fabric covers, identify where each component should be placed: IW, OW, IA, OA, SB, SF, IB, OB, FB, F, CB, CF, P.

Question 3: (IAC0303)

Given the dimensions of a particular cover component (e.g., Inside Back: 500mm x 400mm), explain how to record these measurements correctly, including allowances.

Marking Rubric:

| Criteria | Exemplary (3 points) | Satisfactory (2 points) | Needs Improvement (1 point) |
| --- | --- | --- | --- |
| Accuracy of Identification | Correctly identifies and labels all furniture components. | Correctly identifies most components with minor errors. | Incorrectly identifies several components. |
| Application of Fabric Specifications | Accurately applies specifications to each component. | Mostly accurate but with some confusion over components. | Incorrect application of specifications to components. |
| Measurement Precision | Measurements recorded accurately, including correct allowances. | Generally accurate but occasionally imprecise allowances. | Frequent errors in measurement recording and allowances. |

Through this case study, learners will deepen their understanding of furniture construction and upholstery, enhancing their ability to work effectively on real-world refurbishment projects.

EVALUATION CHECKLIST

| NAME: …………………………………….…. SURNAME: ……………………………….…. ID…………………………………………….... | LEARNER SIGNATURE DATE: |
| --- | --- |

| EVALUATION CRITERIONS | MET REQUIREMENTS | DID NOT MEET REQUIREMENTS | COMMENTS OR ACTION REQUIRED |
| --- | --- | --- | --- |
| Ability of the learner to: |  |  |  |

| GENERAL COMMENTS OF OBSERVER: |  |  |
| --- | --- | --- |
| Assessor Name ……………………………… | Signature ……………………………………. | Date …………………………………. |

## PM-02-PS04: Identify raw materials required for cover fitting operations using the correct terminology

Scope of Practical Skill

Given a range of specifications including technical drawings, written instructions, a range of samples and prototypes of upholstered furniture, prepared furniture frames including dining room seats, arm chairs, sofas, corner units, ottomans and foot stools, and prepared covers for the frames, the learner must be able to:

- PA0401 Interpret specifications and identify the specified prepared frames

- PA0402 Interpret specifications and identify the specified prepared cover components for the frames

- PA0403 Identify the specified bottom covers for the frames

Applied Knowledge

- AK0401 Numeracy and measuring skills

- AK0402 Material selection and fault and defect identification

- AK0403 Raw material specifications and components for cover fitting

Internal Assessment Criteria

- IAC0401 All the required raw materials are available and comply with specification

- IAC0402 Prepared frames inspected and faults and defects identified (wrong frame type supplied, prepared foundation design, shape and firmness faults)

- IAC0403 Prepared covers inspected and faults and defects identified, including type of covers supplied, quantity of covers, fabric type, sizes and seam allowances, fabric flaws, sewing faults, ties not attached or incorrectly attached, correct pattern matching, rail roading, and pile direction

- IAC0404 Covers turned inside out and checked that the components are marked using the correct abbreviations and symbols and positioning on the frame

- IAC0405 Specified frames are all available and meet the specifications

- IAC0406 All specified prepared cover components are available and meet the specifications

- IAC0407 All specified bottom covers are available and meet the specifications

### Practical Learning Content for PA0401: Interpret Specifications and Identify the Specified Prepared Frames

The ability to accurately interpret specifications and identify prepared frames based on those specifications is essential for professionals in the furniture design and upholstery industry. This module provides detailed practical learning content to help learners master these skills, with examples to illustrate how to apply them effectively.

1. Overview of Interpreting Specifications for Furniture Frames

Understanding technical specifications is crucial for correctly identifying and preparing furniture frames. Specifications typically include detailed drawings, dimensions, material types, and other special requirements:

- Technical Drawings: These provide a visual representation of the furniture frame, including dimensions and the relationship between different components.

- Dimensions: Specifications will list the sise requirements for each part of the frame, often in both metric and imperial units.

- Material Specifications: Details about the types of materials to be used, such as the specific grade of wood or type of metal.

- Finishing Details: Information on the type of finish required, whether it involves staining, painting, or sealing.

2. Practical Exercises

Exercise 1: Specification Analysis

- Task: Learners are provided with a set of specifications for a dining chair frame. They must analyse the specifications to identify all required materials and dimensions.

- Objective: To develop the ability to read and understand detailed technical specifications and translate them into a list of materials and measurements needed for the frame.

Exercise 2: Frame Identification Relay

- Task: In a workshop setting, various types of furniture frames are displayed. Learners must use a set of specifications to identify which frame matches the provided details.

- Objective: This exercise helps learners apply their analytical skills in a practical scenario, enhancing their ability to quickly and accurately identify furniture frames based on specific criteria.

3. Real-World Application

Field Trip: Arrange a visit to a furniture manufacturing plant where learners can observe how professionals interpret specifications and prepare frames for different types of furniture. This visit should include demonstrations of measuring, cutting, and assembling frames according to specifications.

Group Project: Learners are divided into teams and given a project to design a furniture frame from scratch. They must create detailed specifications, including technical drawings and material lists, and then construct a prototype of the frame based on their designs.

4. Evaluation and Feedback

Rubric for Exercises:

- Accuracy of Specification Analysis: How accurately learners can interpret specifications and identify correct materials and dimensions.

- Effectiveness in Frame Identification: The ability to correctly match frames with their specifications in a practical setting.

- Creativity and Precision in Design: Assessment of the group project based on the innovativeness of the design and the accuracy of the prototype construction.

Through these educational activities, learners will gain a comprehensive understanding of how to interpret detailed furniture frame specifications. They will develop the skills necessary to ensure that frames are accurately prepared, which is crucial for the overall quality and functionality of finished furniture pieces.

### Practical Learning Content for PA0402: Interpret Specifications and Identify the Specified Prepared Cover Components for the Frames

Understanding how to interpret specifications for cover components and accurately identifying those components are crucial skills in upholstery. This module provides practical learning content to help learners master these skills, incorporating real-world examples to guide them through the process effectively.

1. Overview of Interpreting Specifications for Cover Components

Specifications for upholstery covers include details about material types, dimensions, and specific features required for different parts of the furniture. These specifications ensure that the covers fit perfectly and meet both functional and aesthetic standards:

- Material Specifications: Information about the type of fabric or leather, including any required treatments such as stain resistance or UV protection.

- Dimensional Specifications: Detailed measurements for each part of the cover, including allowances for seams and fitting.

- Component Details: Specifications often break down the cover into its components, such as cushions, arms, backs, and skirts, specifying details for each.

- Pattern and Texture Requirements: Details on how patterns should align, particularly for patterned fabrics, and instructions on the direction of fabric textures or piles.

2. Practical Exercises

Exercise 1: Specification Reading and Component Identification

- Task: Learners are provided with a set of specifications for sofa covers. They must read through these specifications and list all the components required, such as seat cushions, back cushions, arm covers, etc.

- Objective: To develop proficiency in extracting detailed information from specifications and correctly identifying all the cover components needed for assembly.

Exercise 2: Matching Components to Frames

- Task: Using a variety of frames and prepared cover components, learners must match the components to the correct frames based on the specifications provided.

- Objective: This exercise helps learners apply their understanding of specifications in a practical setting, ensuring they can correctly match cover components to their respective frames.

3. Real-World Application

Workshop Activity: Arrange a workshop in an upholstery studio where learners can observe professionals interpreting specifications and preparing covers. Include demonstrations on cutting fabrics to specification and sewing components.

Group Project: Learners are given a furniture frame and a set of specifications to create a complete set of cover components. They must measure, cut, and sew the materials according to the specifications and then fit the completed covers to the frame.

4. Evaluation and Feedback

Rubric for Exercises:

- Accuracy in Component Identification: Precision in identifying and listing all required cover components from the specifications.

- Effectiveness in Matching: Ability to correctly match cover components to their respective frames.

- Technical Skill and Creativity in Project Execution: Quality of the sewing and fitting in the group project, along with creative problem-solving in interpreting the specifications.

Through these educational activities, learners will gain a comprehensive understanding of how to read and apply specifications for upholstery covers. This knowledge is vital for ensuring that the covers not only fit well but also meet specific design requirements, leading to high-quality, durable, and aesthetically pleasing furniture.

### Practical Learning Content for PA0403: Identify the Specified Bottom Covers for the Frames

The ability to correctly identify and prepare bottom covers for furniture frames is crucial in upholstery, as these components play a significant role in the overall durability and finish of the piece. This module offers practical learning content to help learners master the identification and application of bottom covers, with examples to guide their understanding and skill development.

1. Overview of Bottom Covers in Furniture Frames

Bottom covers are typically used on the underside of furniture to conceal internal components, provide additional dust protection, and enhance the furniture's overall finish. These covers must be durable, appropriately fitted, and easy to install or remove for repairs:

- Material Selection: Common materials for bottom covers include lightweight, durable fabrics like cambric or synthetic dust cover fabrics. These materials are chosen for their ability to resist wear and tear while protecting the interior components of the furniture.

- Specifications for Bottom Covers: Specifications might include material type, dimensions, and attachment details such as whether to use staples, tacks, or hook-and-loop fasteners.

- Functionality and Aesthetics: While primarily functional, bottom covers should also be neat and tidy as they contribute to the furniture's perceived quality, especially in pieces where the underside may occasionally be visible.

2. Practical Exercises

Exercise 1: Material Identification and Specification Matching

- Task: Learners are provided with samples of different bottom cover materials along with a range of specifications. They must match each material to its appropriate specification based on the furniture type and intended use.

- Objective: To enhance the ability to choose suitable materials for bottom covers based on specific durability requirements and aesthetic considerations.

Exercise 2: Bottom Cover Fitting Workshop

- Task: In a practical workshop setting, learners are given furniture frames and materials to create and fit bottom covers. They must cut the material to sise, attach it using the specified method, and ensure it fits snugly and securely without sagging or gaping.

- Objective: This hands-on activity helps learners understand the process of fitting bottom covers, from measuring and cutting to the final attachment, emphasising precision and neatness.

3. Real-World Application

Field Trip: Arrange a visit to a professional upholstery shop or furniture manufacturing facility where learners can observe the installation of bottom covers on a variety of furniture pieces. This visit can highlight different techniques and the importance of bottom covers in commercial furniture production.

Capstone Project: Learners undertake a project to refurbish a piece of furniture, including the design and installation of a new bottom cover. They must document their process, explaining their material choice and installation method, and discuss how the bottom cover contributes to the furniture's overall functionality and aesthetics.

4. Evaluation and Feedback

Rubric for Exercises:

- Material Selection Accuracy: Correctness in matching materials with their specifications.

- Quality of Installation: Precision and neatness in cutting, fitting, and securing the bottom cover to the frame.

- Application of Knowledge: Effectiveness in applying learned skills to the capstone project, including rationale for material choices and techniques used.

By engaging with these educational activities, learners will develop a thorough understanding of the roles and requirements of bottom covers in furniture upholstery. They will acquire the skills needed to ensure that these components contribute effectively to the furniture's durability, functionality, and overall aesthetic quality.

### Applied Knowledge Learning Content for AK0401: Numeracy and Measuring Skills

For professionals in fields that involve fabrication, design, or construction, numeracy and measuring skills are essential. These skills are foundational in ensuring accuracy in planning, executing, and assessing projects. This module provides detailed learning content to help learners effectively develop and apply numeracy and measuring skills in practical settings, using examples relevant to the furniture design and manufacturing industry.

1. Overview of Numeracy and Measuring Skills

Numeracy skills in the context of furniture manufacturing involve the ability to handle and interpret numbers and mathematical operations, which are crucial for measurements, cost estimation, and material optimisation:

- Basic Arithmetic: Addition, subtraction, multiplication, and division are used for calculating dimensions, scaling designs, and adjusting specifications.

- Geometry: Understanding shapes, properties of shapes, and spatial thinking is crucial for designing furniture and determining fabric cuts.

- Unit Conversion: Converting between different measurement units, such as inches to millimeters, is essential in workshops that use varied measurement standards.

- Precision and Accuracy: Ability to measure and mark with precision, crucial for ensuring that all components fit together correctly.

2. Practical Exercises

Exercise 1: Measuring and Calculation Drill

- Task: Learners are given a set of dimensions for a hypothetical piece of furniture and asked to calculate material needs, including adjustments for waste and coverage.

- Objective: To practice arithmetic operations in a real-world context, emphasising accuracy in calculations that impact material costs and efficiency.

Exercise 2: Geometric Layout Creation

- Task: Using basic geometric principles, learners design a simple furniture layout plan, requiring them to calculate angles, circumferences, and area distributions.

- Objective: To apply geometric skills to solve practical design problems, enhancing spatial reasoning and precision in layout planning.

3. Real-World Application

Workshop on Measuring Techniques: Conduct a workshop focusing on advanced measuring techniques using various tools such as digital calipers, angle finders, and laser measures. This session should also cover the importance of precision in measurements and common pitfalls in measuring practices.

Project-Based Learning: Learners participate in a group project to create a piece of furniture from scratch. They must use their numeracy skills to plan the project, calculate material requirements, and execute the measurements and cuts needed to assemble the piece.

4. Evaluation and Feedback

Rubric for Exercises:

- Calculation Accuracy: How accurately learners can perform arithmetic operations and apply them to practical tasks.

- Application of Geometric Skills: Effectiveness in using geometric principles to design and plan layouts.

- Technical Proficiency: Precision in measuring and marking materials during the project phase.

By engaging with these educational activities, learners will develop robust numeracy and measuring skills, crucial for professional success in any field that involves physical creation or design. These skills ensure that projects are not only designed innovatively but also executed with high precision, leading to better quality and efficiency.

### Applied Knowledge Learning Content for AK0402: Material Selection and Fault and Defect Identification

Selecting the right materials and identifying potential faults and defects are critical skills in furniture design and manufacturing. This module provides detailed learning content on how to choose appropriate materials based on specific project requirements and how to inspect materials for quality, with examples to illustrate these practices in action.

1. Overview of Material Selection

Material selection is a fundamental step in furniture design that directly affects the product's durability, functionality, and aesthetic appeal:

- Strength and Durability: Materials must be chosen based on the expected use and load-bearing requirements of the furniture. For example, hardwoods like oak and maple are preferred for their strength in load-bearing parts such as legs and frames.

- Aesthetic Considerations: The visual impact of materials, including grain, color, and texture, should align with the design vision. For instance, walnut is often chosen for visible parts due to its fine grain and warm color.

- Environmental Factors: Materials should be suitable for the climate and conditions where the furniture will be used. For outdoor furniture, materials like teak or high-grade metals that resist corrosion and UV damage are essential.

- Cost-Efficiency: Budget constraints often dictate material choices. It's important to balance cost with quality, opting for the best materials that project budgets will allow.

2. Practical Exercises

Exercise 1: Material Selection Challenge

- Task: Learners are given several client scenarios (e.g., a budget-conscious cafe, a high-end home office) and must choose materials that fit the client's needs and budget.

- Objective: To practice making informed material choices based on a variety of factors including cost, aesthetics, and functionality.

Exercise 2: Material Inspection Workshop

- Task: Learners examine batches of wood, metal, and fabric, identifying any faults or defects such as warping, corrosion, or inconsistent dye lots.

- Objective: To develop skills in quality assurance, ensuring that all materials meet high standards before they are used in manufacturing.

3. Real-World Application

Field Trip: Organise a visit to a material supplier or a manufacturing plant where learners can see how materials are processed and prepared for commercial use. They should focus on how quality control measures are implemented.

Group Project: Learners design and build a small piece of furniture, selecting all materials themselves. They must document their selection process, noting why each material was chosen and how they inspected materials for quality.

4. Evaluation and Feedback

Rubric for Exercises:

- Material Selection: How well learners choose materials appropriate for the design requirements and client scenarios.

- Quality Inspection: Effectiveness in identifying and documenting material faults and defects.

- Project Execution: Quality of the final build in the group project, and how well the chosen materials were justified in terms of design and functionality.

Through these educational activities, learners will gain comprehensive skills in selecting materials appropriate for various types of furniture and identifying potential material faults that could compromise the quality of the final product. These skills are crucial for ensuring that furniture not only meets the desired aesthetic standards but is also built to last.

### Applied Knowledge Learning Content for AK0403: Raw Material Specifications and Components for Cover Fitting

For professionals in the upholstery and furniture manufacturing sectors, understanding the specifications of raw materials and the components required for cover fitting is essential. This module provides comprehensive learning content on how to interpret and apply these specifications to ensure high-quality, durable, and aesthetically pleasing furniture covers.

1. Overview of Raw Material Specifications and Components

Effective cover fitting relies on precise knowledge of raw material properties and the components involved in the upholstery process:

- Fabric Types and Their Specifications: Fabrics vary widely in weight, weave, elasticity, and durability. Specifications might include:

- Weight: Measured in grams per square meter (GSM), indicating the heaviness of the fabric.

- Weave: Describes how the fabric is woven; common weaves include plain, twill, and satin.

- Elasticity: Important for fitting covers snugly over furniture frames without tearing or distorting.

- Durability: Measured by tests like the Martindale rub test, which indicates how a fabric withstands wear and tear.

- Leather and Synthetics: Specifications cover aspects such as thickness, grain (for leather), and backing material. These materials are often chosen for their durability and ease of maintenance.

- Component Breakdown: Understanding how to cut and assemble various parts of the cover:

- Cushions: Often require softer, more flexible materials that can conform to shapes.

- Arms and Backs: Need more rigid materials that can maintain shape and provide support.

- Skirting and Trim: Might use lighter or more decorative materials.

2. Practical Exercises

Exercise 1: Spec Sheet Analysis

- Task: Learners are provided with spec sheets for various upholstery materials. They must analyse these to determine suitable applications for each material in different parts of furniture.

- Objective: To develop the ability to interpret raw material specifications and apply this information to select appropriate materials for specific upholstery components.

Exercise 2: Material Matching Game

- Task: Learners receive descriptions of several furniture pieces requiring new upholstery. Based on the material specifications, they must choose the most appropriate fabric or leather for each piece.

- Objective: This exercise helps learners practice matching material properties with the functional and aesthetic needs of furniture, enhancing their decision-making skills.

3. Real-World Application

Supplier Interaction: Arrange a session with a fabric supplier or a visit to a material showroom where learners can see and feel different materials and discuss their specifications with experts.

Design Project: Learners undertake a project to design and upholster a piece of furniture, starting from selecting materials based on provided specifications to completing the upholstery. They must document their choices and the reasons behind them.

4. Evaluation and Feedback

Rubric for Exercises:

- Accuracy in Spec Sheet Analysis: Precision and correctness in interpreting material specifications.

- Appropriateness of Material Selection: Suitability of chosen materials for the described furniture pieces.

- Project Execution and Innovation: Quality of the final upholstered piece and creativity in material usage and fitting techniques.

Through these educational activities, learners will gain a thorough understanding of how to select and use various upholstery materials based on detailed specifications. This knowledge ensures that the upholstery not only meets the desired functional requirements and aesthetic preferences but also adheres to high standards of quality and durability.

### GUIDELINES

You are required to perform the task below and your Facilitator/Assessor will assess your work based on the observation checklist attached.

Practical Assessment Task for PM-02-PS04: Identify raw materials required for cover fitting operations using the correct terminology

Assessment Title:

Purpose

Guidelines to the Learner:

1. You will be presented with a range of samples or photographs of different wooden furniture products and accessories. These will include various styles, types, and uses.  Refer to Knowledge Module KM13 and the additional brochure with examples of upholstered furniture.

2. You are required to examine each sample or photograph and perform the following tasks:

Assessment Criteria:

Assessment Duration:

- The assessment should be completed within a specified time frame, typically 2-3 hours, to simulate a realistic working environment.

Feedback and Evaluation:

- Detailed feedback will be provided on the accuracy and relevance of your categorisations.

- The assessor will highlight areas of strength and suggest improvements where necessary.

Additional Notes:

- This practical assessment is designed to test not only your ability to recognise and categorise different furniture types and accessories but also your understanding of the broader context in which these items are used.

- It is essential to approach this task with attention to detail and a thorough understanding of the varied aspects of upholstered furniture design and manufacturing.

### Case Study: Upholstery Project for a Boutique Hotel Chain

Scenario:

You are an upholstery manager at a large furniture manufacturing company tasked with overseeing a critical project for a boutique hotel chain. The project involves upholstering a series of furniture pieces, including sofas, armchairs, and dining chairs. You are provided with specifications for all materials, frames, and covers, and your role is to ensure that everything complies with these specifications before beginning the upholstery process.

Scenario-Based Questions:

Question 1: (IAC0401 & IAC0405)

Upon receiving a shipment of materials and frames, how do you verify that all the required raw materials are available and comply with the specifications provided? Additionally, confirm that the specified frames are available and meet the specifications.

Question 2: (IAC0402)

During inspection of the frames, you notice a few inconsistencies with the specifications. How do you identify and document these faults and defects?

Question 3: (IAC0403 & IAC0406)

You are reviewing the prepared covers and need to inspect them for any faults. What faults should you look for, and how do you ensure all cover components meet specifications?

Question 4: (IAC0404 & IAC0407)

How do you check that covers are correctly marked and positioned on the frame and verify that all specified bottom covers are available and meet the specifications?

Marking Memo:

- Compliance with Specifications: 3 points for fully meeting specifications, 2 points for minor deviations, 1 point for significant issues.

- Documentation Quality: 3 points for detailed and clear documentation, 2 points for adequate documentation with some missing details, 1 point for poor or incomplete documentation.

- Corrective Actions: 3 points for proactive and effective solutions, 2 points for satisfactory solutions with some delays, 1 point for ineffective or delayed responses.

Through these assessments, learners will demonstrate their ability to effectively manage an upholstery project, ensuring that all materials and components meet the required standards before proceeding with the final upholstery, which is crucial for the success of the project and the satisfaction of the client.

EVALUATION CHECKLIST

| NAME: …………………………………….…. SURNAME: ……………………………….…. ID…………………………………………….... | LEARNER SIGNATURE DATE: |
| --- | --- |

| EVALUATION CRITERIONS | MET REQUIREMENTS | DID NOT MEET REQUIREMENTS | COMMENTS OR ACTION REQUIRED |
| --- | --- | --- | --- |
| Ability of the learner to: |  |  |  |

| GENERAL COMMENTS OF OBSERVER: |  |  |
| --- | --- | --- |
| Assessor Name ……………………………… | Signature ……………………………………. | Date …………………………………. |

## PM-02-PS05: Determine the ancillary materials required for cover fitting

Scope of Practical Skill

Given a range of specifications including technical drawings, written instructions, a range of samples and prototypes of upholstered furniture, prepared furniture frames including dining room seats, arm chairs, sofas, corner units, ottomans and foot stools, and prepared covers for the frames, the learner must be able to:

- PA0501 Interpret specifications and identify the specified ancillary materials

- PA0502 Inspect and select the specified ancillary materials

Applied Knowledge

- AK0501 Material selection and fault and defect identification

Internal Assessment Criteria

- IAC0501 All specified ancillary materials available

### Practical Learning Content for PA0501: Interpret Specifications and Identify the Specified Ancillary Materials

For professionals in upholstery and furniture design, accurately interpreting specifications and identifying the correct ancillary materials based on these specifications is crucial. This skill ensures that the final product meets both aesthetic and functional standards. This module provides detailed practical learning content with examples to help learners master these skills.

1. Overview of Ancillary Materials in Upholstery

Ancillary materials in upholstery include components that complement and enhance the structural integrity, comfort, and visual appeal of furniture. These may include:

- Gimps: Decorative trim used to cover upholstery tacks or staples and provide a clean finish.

- Piping (also known as cording): Fabric-covered cords used to define the edges of cushions and furniture outlines.

- Skirting: Fabric panels that hang from the edges of furniture like sofas and chairs to the floor, often used to conceal legs or casters.

- Braids: Decorative trims that can be sewn into seams or used to embellish upholstery.

- Valances: Fabric drapery hung around the base of a bed or along the lower edge of a sofa to add a decorative touch.

- Studs and Fake Nails: Often used to enhance the aesthetic of a piece by adding metallic accents or simulating traditional nailhead trim.

2. Practical Exercises

Exercise 1: Specification Reading and Material Identification

- Task: Learners are given a set of specifications detailing a fictional upholstery project, including diagrams and descriptions of the required ancillary materials.

- Objective: Learners must interpret these specifications to list all ancillary materials needed, identifying their type, quantity, and intended use.

- Example: Specifications might indicate the need for brass studs for a vintage armchair. Learners would note the number of studs required, the spacing specified, and any particular pattern or design mentioned.

Exercise 2: Ancillary Material Matching Game

- Task: Using a variety of ancillary materials laid out, learners must match these materials to project specifications provided in multiple sets of plans.

- Objective: This exercise helps reinforce learners' ability to connect specification details with physical materials, enhancing their understanding of how ancillary items contribute to the overall design.

- Example: Learners could be given several types of piping and asked to match them to furniture sketches that specify dimensions and fabric types suitable for each type of piping.

3. Real-World Application

Material Selection Workshop: Organise a workshop with a local upholstery supplier where learners can see and handle various ancillary materials. The supplier can discuss the applications, durability, and aesthetic considerations of each type.

Group Project: Learners work in teams to design a small piece of upholstered furniture, selecting all ancillary materials based on a set of given specifications. They must justify their choices based on the functional and aesthetic requirements of the project.

4. Evaluation and Feedback

Rubric for Exercises:

- Accuracy in Material Identification: Correctly identifying and listing materials as per specifications.

- Understanding of Material Application: Demonstrating knowledge of why certain ancillary materials are used in specific applications.

- Creativity in Material Use: Innovative use of ancillary materials in the group project, contributing to the functionality and aesthetics of the final piece.

By engaging with these educational activities, learners will develop a thorough understanding of how to interpret specifications for ancillary materials and apply this knowledge to actual upholstery projects. This expertise ensures that finished furniture items are not only functionally sound but also align with design intentions, ultimately leading to higher quality and customer satisfaction.

### Practical Application Learning Content for PA0502: Inspect and Select the Specified Ancillary Materials

The ability to effectively inspect and select ancillary materials as specified in upholstery projects is crucial for ensuring the quality and integrity of the final product. This module provides detailed practical learning content to guide learners through the process of inspecting and selecting the right ancillary materials for upholstery, complete with practical examples.

1. Overview of Inspecting Ancillary Materials

Proper inspection of ancillary materials involves checking for quality, consistency, and suitability for specific upholstery tasks:

- Quality Control: Assess the material's quality, looking for any defects such as fraying on braids, uneven coloring in gimps, or cracks in fake nails.

- Consistency: Ensure that all materials from the same batch match in color, texture, and sise, which is especially important for visible components like piping or studs.

- Suitability: Verify that the materials meet the specific requirements of the project, such as the correct type of skirting for a sofa or the appropriate style of studs for a traditional armchair.

2. Practical Exercises

Exercise 1: Material Inspection Drill

- Task: Learners are provided with a variety of ancillary materials (e.g., skirting, piping, gimps). They must inspect these materials for any defects or inconsistencies and assess their suitability based on a provided set of project specifications.

- Objective: To develop skills in identifying quality issues and matching materials to specific project requirements.

- Example: Learners inspect a batch of velvet gimps for color uniformity and check if the elasticity of the piping is suitable for a curved ottoman project.

Exercise 2: Selection Simulation

- Task: Given a scenario where learners must choose ancillary materials for several different projects, they must select the appropriate materials based on durability, aesthetic needs, and client specifications.

- Objective: This exercise helps learners apply their inspection findings to make informed decisions about material selection.

- Example: Choosing between metal and upholstered studs for a luxury headboard, considering both the design impact and the installation process.

3. Real-World Application

Supplier Interaction Workshop: Arrange a workshop with a supplier of upholstery materials where learners can interact directly with a wide range of ancillary materials. This interaction should include discussions on the latest materials, their applications, and maintenance requirements.

Project-Based Learning: Learners undertake a project to refurbish a piece of furniture, where they must choose all ancillary materials themselves. They should document their selection process, explaining why each material was chosen based on the inspection outcomes and project needs.

4. Evaluation and Feedback

Rubric for Exercises:

- Quality Inspection Skills: Accuracy and thoroughness in identifying defects and inconsistencies.

- Appropriateness of Selection: Suitability of selected materials for the projects, based on the criteria set out in the specifications.

- Application of Knowledge: Effectiveness in applying the inspection results to make decisions that enhance the project's quality and meet client expectations.

Through these educational activities, learners will develop a comprehensive understanding of how to inspect and select ancillary materials for upholstery projects. They will gain the skills necessary to ensure that all materials used not only meet the high standards required for premium upholstery work but also enhance the functionality and aesthetic appeal of the finished pieces.

### Applied Knowledge Learning Content for AK0501: Material Selection and Fault and Defect Identification

For professionals in furniture manufacturing and upholstery, the ability to select the right materials and identify faults and defects is essential to ensuring quality and durability in their projects. This module provides practical learning content to enhance these critical skills, complete with real-world examples to facilitate understanding and application.

1. Overview of Material Selection and Fault Identification

Selecting the correct materials involves understanding their properties, suitability for the task, and ensuring they are free from defects that could compromise the finished product:

- Material Properties: Important properties include durability, colorfastness, ease of maintenance, and appropriateness for the specific piece of furniture (e.g., high-traffic areas might require more durable materials).

- Suitability: Materials must be appropriate for the specific design and functional requirements of the furniture. For instance, outdoor furniture materials should have UV resistance and moisture repellence.

- Fault and Defect Identification: Essential for maintaining quality, this includes checking for inconsistencies in texture, color, strength, and any manufacturing defects that could affect performance or aesthetics.

2. Practical Exercises

Exercise 1: Material Evaluation and Selection

- Task: Learners are given a range of material samples (e.g., textiles, leathers, synthetics) and a set of project specifications. They must evaluate each material against the specifications to determine the most suitable choices for different furniture components.

- Objective: To develop the ability to assess material properties and make informed selections based on specific project needs.

- Example: Choosing between polyester and natural cotton fabric for a sofa intended for use in a sunny room, considering factors like UV resistance and fade resistance.

Exercise 2: Fault and Defect Detection Workshop

- Task: Learners inspect batches of materials for potential faults and defects. This includes looking for color variation, texture inconsistency, and any physical defects like tears or weak spots.

- Objective: To enhance skills in identifying material faults and understanding their potential impacts on the final product.

- Example: Inspecting a roll of upholstery vinyl for rips, weak seams, or discoloration that could affect the appearance or durability of a restaurant booth.

3. Real-World Application

Vendor Interaction Session: Arrange for learners to meet with material suppliers to discuss how they ensure material quality and what defect rates are considered acceptable. This could include a tour of a manufacturing facility where materials are tested.

Group Project: Learners are tasked with creating a small piece of furniture from scratch, selecting all materials themselves. They must justify their material choices based on specific criteria and ensure all materials are inspected and free from defects before use.

4. Evaluation and Feedback

Rubric for Exercises:

- Material Selection Accuracy: Evaluates how well learners select materials that fit the project specifications and design requirements.

- Defect Identification Skills: Assesses the learner’s ability to detect and correctly identify material defects.

- Application and Justification: Measures the learner's ability to apply their knowledge in a practical setting, including their rationale for material choices and their approach to handling identified defects.

Through these educational activities, learners will gain a comprehensive understanding of material selection and defect identification, crucial for producing high-quality furniture that meets both functional and aesthetic standards. These skills ensure that professionals can make informed decisions that lead to successful project outcomes and high customer satisfaction.

### GUIDELINES

You are required to perform the task below and your Facilitator/Assessor will assess your work based on the observation checklist attached.

Practical Assessment Task for PM-02-PS05: Determine the ancillary materials required for cover fitting

Assessment Title:

Purpose

Guidelines to the Learner:

1. You will be presented with a range of samples or photographs of different wooden furniture products and accessories. These will include various styles, types, and uses.  Refer to Knowledge Module KM13 and the additional brochure with examples of upholstered furniture.

2. You are required to examine each sample or photograph and perform the following tasks:

Assessment Criteria:

Assessment Duration:

- The assessment should be completed within a specified time frame, typically 2-3 hours, to simulate a realistic working environment.

Feedback and Evaluation:

- Detailed feedback will be provided on the accuracy and relevance of your categorisations.

- The assessor will highlight areas of strength and suggest improvements where necessary.

Additional Notes:

- This practical assessment is designed to test not only your ability to recognise and categorise different furniture types and accessories but also your understanding of the broader context in which these items are used.

- It is essential to approach this task with attention to detail and a thorough understanding of the varied aspects of upholstered furniture design and manufacturing.

### Case Study: Ancillary Materials Procurement for Elite Furniture Designs

Scenario:

You are an operations manager at Elite Furniture Designs, tasked with overseeing the procurement and readiness of ancillary materials for an upcoming production run. The project involves the creation of a luxury line of sofas and armchairs that require a variety of ancillary materials, including high-end piping, durable gimps, and decorative studs. Your role is to ensure that all specified ancillary materials are available and ready for use as per the project specifications.

Scenario-Based Question:

Question: (IAC0501)

You receive a shipment of ancillary materials for the new furniture line. Describe the steps you would take to ensure that all specified ancillary materials are available as per the project requirements. Include how you would verify the quantity and quality of these materials.

Marking Memo:

| Criteria | Exemplary (3 points) | Satisfactory (2 points) | Needs Improvement (1 point) |
| --- | --- | --- | --- |
| Inventory Accuracy | Correctly matches the inventory with specifications and identifies all discrepancies. | Mostly accurate but overlooks minor discrepancies. | Fails to accurately match inventory with specifications. |
| Quality Inspection | Thoroughly inspects all materials, correctly identifies defects, and provides detailed documentation. | Conducts an inspection but may miss minor defects or provide incomplete documentation. | Insufficient inspection, overlooks significant defects, or fails to document issues. |
| Communication and Resolution | Proactively communicates with suppliers, providing detailed information and ensuring timely resolution. | Communicates with suppliers but may lack detail or urgency in resolving issues. | Poor communication with suppliers, leading to unresolved issues or delays. |

By engaging with these activities, you will ensure that the ancillary materials meet the high standards required for the luxury furniture line, thereby maintaining the quality and timeline of the project. This process not only minimises potential production delays but also upholds the brand's reputation for quality and attention to detail.

EVALUATION CHECKLIST

| NAME: …………………………………….…. SURNAME: ……………………………….…. ID…………………………………………….... | LEARNER SIGNATURE DATE: |
| --- | --- |

| EVALUATION CRITERIONS | MET REQUIREMENTS | DID NOT MEET REQUIREMENTS | COMMENTS OR ACTION REQUIRED |
| --- | --- | --- | --- |
| Ability of the learner to: |  |  |  |

| GENERAL COMMENTS OF OBSERVER: |  |  |
| --- | --- | --- |
| Assessor Name ……………………………… | Signature ……………………………………. | Date …………………………………. |

## PM-02-PS06: Determine the consumables required for cover fitting

Scope of Practical Skill

Given a range of specifications including technical drawings, written instructions, a range of samples and prototypes of upholstered furniture, prepared furniture frames including dining room seats, arm chairs, sofas, corner units, ottomans and foot stools, and prepared covers for the frames, the learner must be able to:

- PA0601 Interpret specifications and identify the quantity, type and dimensions of consumables including staples, blind stitching needles, twine, thread, tacks and cardboard and metal tack strips

Applied Knowledge

- AK0601 Numeracy and measuring skills

- AK0602 Consumable selection and fault and defect identification

Internal Assessment Criteria

- IAC0601 All required consumables are fit for purpose

- IAC0602 All required consumables available

### Practical Application Learning Content for PA0601: Interpret Specifications and Identify the Quantity, Type, and Dimensions of Consumables

For upholstery professionals, the ability to accurately interpret specifications and identify the necessary consumables for a project is crucial. This module provides detailed practical learning content with examples to help learners effectively manage and apply their knowledge of consumables, which are essential for ensuring the structural integrity and aesthetic quality of upholstered furniture.

1. Overview of Common Upholstery Consumables

Understanding each type of consumable and its application is the first step in effective project management:

- Staples: Used for attaching fabric to furniture frames. Specifications may detail staple sise and type, which vary depending on the thickness of the material and the hardness of the frame.

- Blind Stitching Needles: Essential for creating hidden seams in upholstery. The specifications might list different sises or types of needles required based on the thickness and type of fabric.

- Twine: Often used for tying springs or securing structural components internally. The strength and thickness required will be specified based on the load-bearing needs of the furniture.

- Thread: Threads can vary widely in material and thickness. Specifications will often include the type of thread (e.g., nylon, polyester) and the weight or gauge suitable for the fabric and the stress points of the furniture.

- Tacks: Used for traditional upholstery methods where staples are not suitable. Specifications should indicate sise and material, which affect the tack's durability and appearance.

- Cardboard and Metal Tack Strips: Used to create clean lines and secure fabric edges. The specifications should detail the dimensions and types of strips needed for different parts of the furniture.

2. Practical Exercises

Exercise 1: Specification Decoding

- Task: Learners are given a set of specifications for an upholstery project. They must identify and list all required consumables, noting the specified types, quantities, and dimensions.

- Objective: To enhance skills in analysing and extracting detailed information from technical documents.

- Example: Specifications for reupholstering a vintage sofa might list specific sises of staples for hardwood frames, the gauge of twine for tying springs, and types of metal tack strips for armrests.

Exercise 2: Consumable Selection Simulation

- Task: Using a simulated inventory, learners select consumables needed for a specified project. They must ensure their choices match the specifications in type, sise, and quantity.

- Objective: To practice applying specification data in selecting appropriate consumables for various tasks.

- Example: Given a project to upholster a set of dining chairs, learners choose the correct type and sise of thread for sewing cushion covers and select the appropriate length and gauge of staples for securing the fabric.

3. Real-World Application

Material Management Workshop: Conduct a workshop with a local upholstery shop focusing on how to manage and select consumables inventory. Include a demonstration of various techniques for using these materials in upholstery.

Project-Based Learning: Learners undertake a project to upholster a piece of furniture, from reading the specifications to selecting and applying the correct consumables. They must document their process, highlighting how each choice was made in accordance with the specifications.

4. Evaluation and Feedback

Rubric for Exercises:

- Accuracy in Decoding Specifications: Precision in interpreting specifications and correctly identifying needed consumables.

- Effectiveness in Material Selection: Suitability of selected consumables for the project, based on compliance with the specifications.

- Application in Practical Project: Quality of execution in the project-based learning activity, including how effectively the consumables were used.

Through these educational activities, learners will develop a comprehensive understanding of how to interpret specifications for upholstery consumables and apply this knowledge to ensure high-quality outcomes in their projects. This expertise is crucial for maintaining standards of durability and aesthetics in professional upholstery work.

### Applied Knowledge Learning Content for AK0601: Numeracy and Measuring Skills

In the field of upholstery and furniture manufacturing, numeracy and measuring skills are fundamental to ensuring that projects are executed with precision and efficiency. This module provides comprehensive learning content to help professionals develop these skills, complete with practical examples to enhance their understanding and application in real-world scenarios.

1. Overview of Numeracy and Measuring Skills

Effective numeracy and measuring skills involve more than just the ability to use measuring tapes and rulers; they encompass a broader range of competencies:

- Basic Arithmetic: Essential for calculating dimensions, quantities, and costs. This includes addition, subtraction, multiplication, and division, which are routinely used to adjust measurements and scale designs.

- Unit Conversion: Critical when working with diverse measurement systems. For example, converting inches to millimeters or yards to meters ensures that specifications meet international standards or client preferences.

- Geometric Measurements: Necessary for calculating area and volume, especially when determining the amount of fabric or filling material required for specific furniture pieces.

- Precision and Accuracy: Vital for ensuring that all parts of a furniture piece fit together seamlessly and function as intended. This includes precise cutting, spacing, and placement of components like foam, fabric, and structural supports.

2. Practical Exercises

Exercise 1: Dimension Calculation and Conversion

- Task: Learners are presented with a furniture blueprint that uses imperial units. They must calculate the required material dimensions and convert these measurements into metric units for a supplier who uses the metric system.

- Objective: To practice unit conversions and reinforce accuracy in measurement calculations.

- Example: Converting the length of a sofa from 72 inches to centimeters, ensuring that the conversion is accurate to avoid material shortages or excess.

Exercise 2: Material Estimation

- Task: Given the dimensions of a furniture item, learners must calculate the amount of fabric needed, considering the width of the fabric roll and pattern repeats.

- Objective: To develop skills in geometric measurements and material estimation, accounting for fabric orientation and pattern matching.

- Example: Determining how many yards of a 54-inch wide fabric are needed to upholster a chair that requires three panels, each measuring 40 inches by 20 inches, with a fabric pattern repeat of 12 inches.

3. Real-World Application

Workshop on Advanced Measuring Tools: Conduct a workshop where learners can practice using digital measuring devices such as laser measures and digital angle finders. These tools offer greater precision and can streamline the measuring process.

Project-Based Learning: Assign learners the task of designing and creating a custom cushion. They must calculate all necessary dimensions, convert these dimensions as needed, estimate material quantities, and execute the project to specifications.

4. Evaluation and Feedback

Rubric for Exercises:

- Accuracy of Measurements: How precisely learners measure and calculate dimensions and conversions.

- Efficiency in Material Estimation: Effectiveness in estimating material needs without significant waste or shortfall.

- Application and Execution: Quality of the final product in the project-based learning activity, reflecting the accuracy and practical application of numeracy and measuring skills.

Through these educational activities, learners will gain a solid foundation in numeracy and measuring skills, enabling them to handle complex upholstery projects with confidence and precision. These skills are crucial for reducing errors and waste, thereby increasing productivity and satisfaction in their craftsmanship.

### Applied Knowledge Learning Content for AK0602: Consumable Selection and Fault and Defect Identification

For professionals in upholstery and furniture manufacturing, the ability to select the appropriate consumables and identify any faults or defects in these materials is crucial. This module provides detailed learning content to enhance these essential skills, including practical examples to facilitate understanding and application.

1. Overview of Consumable Selection

Selecting the right consumables involves understanding their properties and how these properties interact with different types of upholstery tasks:

- Staples: Needed for attaching fabric to frames. Variations include different leg lengths and gauges, suitable for different thicknesses of material and types of wood or metal frames.

- Tacks and Nails: Used for decorative purposes or traditional upholstery. Choosing the correct sise and type (e.g., brass, stainless steel) affects both the appearance and durability of the furniture.

- Thread: Must be chosen based on the material it will be used with. Heavy-duty threads are required for thicker materials, while finer threads are suitable for delicate fabrics.

- Adhesives: Selection depends on the materials being bonded. Some adhesives offer more flexibility or a stronger bond, important for areas of furniture that experience frequent use.

- Tack Strips: Cardboard tack strips are used for straight edges, while metal tack strips are used for more demanding applications requiring stronger hold.

2. Practical Exercises

Exercise 1: Consumable Matching Game

- Task: Learners receive a variety of upholstery scenarios and must select the appropriate consumables for each, considering the material, durability needs, and aesthetic requirements.

- Objective: To practice making informed choices about consumables based on specific upholstery needs.

- Example: Choosing the right type of thread and needle for upholstering a leather sofa to ensure the seams are strong and the finish is clean.

Exercise 2: Fault and Defect Detection Workshop

- Task: Learners inspect batches of consumables (e.g., rolls of thread, boxes of staples) for any defects such as rust on tacks, fraying threads, or weak adhesive.

- Objective: To develop skills in identifying defects in consumables that could compromise the quality of the upholstery work.

- Example: Identifying frayed threads which could break during sewing, leading to weak seams that might tear under normal use.

3. Real-World Application

Supplier Interaction: Arrange for learners to visit a consumables supplier to learn about the production processes and quality control measures for different types of upholstery consumables.

Project-Based Learning: Assign a project where learners must reupholster a piece of furniture, selecting all consumables themselves. They must document their choices, explaining why each consumable was chosen and inspecting these for any faults before use.

4. Evaluation and Feedback

Rubric for Exercises:

- Correctness of Consumable Selection: Evaluates how well learners select consumables appropriate for the material and task.

- Defect Identification Accuracy: Assesses the ability to detect and identify faults in consumables.

- Application and Justification: Measures how effectively learners apply their consumable choices in practical upholstery tasks.

Through these activities, learners will not only enhance their ability to select the right consumables for different tasks but also ensure that these materials are of high quality and free from defects. This knowledge is crucial for maintaining the standards of durability and aesthetics expected in professional upholstery work.

### GUIDELINES

You are required to perform the task below and your Facilitator/Assessor will assess your work based on the observation checklist attached.

Practical Assessment Task for PM-02-PS06: Determine the consumables required for cover fitting

Assessment Title:

Purpose

Guidelines to the Learner:

1. You will be presented with a range of samples or photographs of different wooden furniture products and accessories. These will include various styles, types, and uses.  Refer to Knowledge Module KM13 and the additional brochure with examples of upholstered furniture.

2. You are required to examine each sample or photograph and perform the following tasks:

Assessment Criteria:

Assessment Duration:

- The assessment should be completed within a specified time frame, typically 2-3 hours, to simulate a realistic working environment.

Feedback and Evaluation:

- Detailed feedback will be provided on the accuracy and relevance of your categorisations.

- The assessor will highlight areas of strength and suggest improvements where necessary.

Additional Notes:

- This practical assessment is designed to test not only your ability to recognise and categorise different furniture types and accessories but also your understanding of the broader context in which these items are used.

- It is essential to approach this task with attention to detail and a thorough understanding of the varied aspects of upholstered furniture design and manufacturing.

### Case Study: Launch of a New Upholstery Line at Modern Designs Inc.

Scenario:

You are the production manager at Modern Designs Inc., a company that is about to launch a new line of custom upholstered furniture. Your role involves ensuring that all consumables needed for the upholstery process are not only available but also fit for purpose to meet the high standards expected for this new line. The furniture range includes high-end sofas, armchairs, and dining chairs, requiring a variety of consumables such as staples, threads, tacks, and adhesives.

Scenario-Based Questions:

Question 1: (IAC0601)

How do you verify that all consumables like staples, threads, tacks, and adhesives are fit for purpose for upholstering a range of high-end furniture?

Question 2: (IAC0602)

Describe the process you would use to ensure that all required consumables are available before the start of the upholstery project.

Marking Memo:

| Criteria | Exemplary (3 points) | Satisfactory (2 points) | Needs Improvement (1 point) |
| --- | --- | --- | --- |
| Fit for Purpose Verification | Thoroughly tests all consumables and ensures all meet high standards; well-documented process. | Tests most consumables adequately; minor issues in documentation or standards met. | Fails to adequately test consumables or to document testing processes. |
| Availability of Consumables | Excellent management of inventory with all consumables available as required; proactive in ordering and monitoring. | Generally good inventory management but with minor oversights; reactive rather than proactive ordering. | Poor management of inventory, leading to shortages; lacks effective monitoring and ordering processes. |

By addressing these scenarios, you will demonstrate a comprehensive understanding of the importance of meticulous preparation in ensuring the success of high-end upholstery projects. This case study reinforces the necessity of rigorous standards in consumable management to achieve excellence in product quality and customer satisfaction.

EVALUATION CHECKLIST

| NAME: …………………………………….…. SURNAME: ……………………………….…. ID…………………………………………….... | LEARNER SIGNATURE DATE: |
| --- | --- |

| EVALUATION CRITERIONS | MET REQUIREMENTS | DID NOT MEET REQUIREMENTS | COMMENTS OR ACTION REQUIRED |
| --- | --- | --- | --- |
| Ability of the learner to: |  |  |  |

| GENERAL COMMENTS OF OBSERVER: |  |  |
| --- | --- | --- |
| Assessor Name ……………………………… | Signature ……………………………………. | Date …………………………………. |

## PM-02-PS07: Identify, prepare and safely apply hand tools and equipment for fitting covers to frames

Scope of Practical Skill

Given a range of specifications including technical drawings, written instructions, a range of samples and prototypes of upholstered furniture, prepared furniture frames including dining room seats, arm chairs, sofas, corner units, ottomans and foot stools, and prepared covers for the frames, the learner must be able to:

- PA0701 Interpret specifications and identify the hand tools required to cover the prepared frames

- PA0702 Identify the needles used for blind stitching

- PA0703 Check hand tools and equipment for defects, sharpness, and safety, and take appropriate steps to rectify problems

Applied Knowledge

- AK0701 Tacking and covering techniques

- AK0702 Sewing techniques

- AK0703 Faults and maintenance

Internal Assessment Criteria

- IAC0701 Hand tools used for attaching specified covers to frames are available

- IAC0702 Needles used for blind stitching are available

- IAC0703 Hand tools and equipment used for covering are checked and tested for defects, sharpness and safety and appropriate steps taken to rectify problems

### Practical Application Learning Content for PA0701: Interpret Specifications and Identify Hand Tools Required to Cover Prepared Frames

Understanding how to interpret technical specifications and accurately identify the hand tools required for upholstery tasks is crucial for professionals in the furniture making and upholstery industry. This module offers detailed practical learning content with examples to help learners effectively manage and apply these skills in real-world scenarios.

1. Overview of Essential Hand Tools in Upholstery

A thorough knowledge of upholstery hand tools and their specific applications is necessary for selecting the right tools based on project specifications:

- Tape Measures: Used for obtaining accurate measurements of furniture frames and fabric.

- Marking Chalk: Essential for marking measurements on fabrics without leaving permanent marks.

- Magnetic Tack Hammer: Dual-purpose tool used for driving tacks and also featuring a magnetic tip for picking them up.

- Shears and Scissors: Shears are used for cutting heavy fabric or multiple layers, while scissors are suited for finer, more precise cuts.

- Tack and Staple Lifters: Tools designed for removing old tacks and staples from furniture frames.

- Trimming Knives: Used to trim excess fabric and foam to ensure a clean finish.

2. Practical Exercises

Exercise 1: Tool Identification from Specifications

- Task: Learners are provided with a set of specifications for an upholstery project that includes several types of furniture pieces. They must identify and list all necessary hand tools required for each type of furniture based on the tasks described.

- Objective: To develop the ability to analyse specifications and determine the appropriate tools for different upholstery tasks.

- Example: For a project specifying the reupholstery of a set of dining chairs, learners would identify tape measures for sising, shears for cutting the new fabric, marking chalk for layout, and a magnetic tack hammer for fitting the fabric to the frame.

Exercise 2: Tool Selection Role-Play

- Task: In a simulated upholstery workshop, learners assume the role of an upholsterer receiving a new project. They must select all tools they would use based on a provided specification sheet.

- Objective: This exercise helps reinforce the practical application of selecting tools, encouraging learners to consider factors such as the material type and the precision required.

- Example: Given specifications for upholstering a vintage sofa, learners choose tools including shears for heavy fabric, tack lifters for removing old upholstery, and trimming knives for adjusting new padding.

3. Real-World Application

Workshop Participation: Arrange a workshop where learners can observe experienced upholsterers at work, focusing on how they select and use different tools for various tasks. This observation should include demonstrations of tool maintenance and safety practices.

Project-Based Learning: Assign learners the task of completing a small upholstery project from start to finish. They must use the project specifications to choose their tools, measure and cut fabric, and fit it to a frame, documenting each step and the tools used.

4. Evaluation and Feedback

Rubric for Exercises:

- Accuracy in Tool Identification: How well learners identify and list the correct tools needed for the specified tasks.

- Effectiveness in Tool Selection: Learners' ability to select the appropriate tools for a given project, demonstrating understanding of tool functions and applications.

- Quality of Practical Application: Assessment of the final project, focusing on the appropriateness of tool usage and the quality of the upholstery work.

Through these educational activities, learners will gain a comprehensive understanding of how to interpret upholstery specifications and correctly identify the necessary hand tools for various projects. These skills are essential for ensuring high-quality results in upholstery, enhancing both the functionality and aesthetics of finished furniture pieces.

### Practical Application Learning Content for PA0703: Check Hand Tools and Equipment for Defects, Sharpness, and Safety

Maintaining hand tools and equipment in optimal condition is crucial for ensuring both the quality of work and safety in the upholstery process. This module provides practical learning content designed to equip learners with the skills necessary to inspect their tools for defects, maintain sharpness, and uphold safety standards, supplemented by real-world examples.

1. Overview of Tool and Equipment Maintenance

Proper maintenance and regular inspection of upholstery tools are essential for efficient and safe operations:

- Inspecting for Defects: Regular checks to identify wear, damage, or malfunction in tools such as fraying in the handles of shears, loose heads on hammers, or bent staples in staple guns.

- Maintaining Sharpness: Ensuring cutting tools like shears, scissors, and trimming knives are sharp to provide clean cuts and reduce the effort needed, which also minimises the risk of accidents.

- Ensuring Safety: Verifying that all safety features of tools and equipment are functioning correctly, such as safety locks on staple guns and the integrity of protective guards.

2. Practical Exercises

Exercise 1: Tool Inspection Drill

- Task: Learners are provided with a variety of common upholstery tools and must conduct a thorough inspection to identify any defects or signs of wear.

- Objective: To develop proficiency in recognising issues that could affect tool performance or user safety.

- Example: Learners inspect a set of scissors for dullness and handle integrity, a tack hammer for a secure head, and a staple gun for proper functioning and safety features.

Exercise 2: Sharpening and Maintenance Workshop

- Task: Conduct a hands-on workshop where learners practice sharpening cutting tools and performing basic maintenance tasks like tightening loose components or lubricating moving parts.

- Objective: To ensure learners can maintain their tools in a condition that maximises functionality and safety.

- Example: Learners use a sharpening stone to sharpen shears and scissors, apply oil to hinges, and replace worn-out parts on a staple gun.

3. Real-World Application

Field Visit to a Tool Repair Shop: Arrange a visit to a professional tool repair shop where learners can observe experts at work repairing and maintaining various types of tools. Discussions can include tips on recognising tool wear and the best practices for tool maintenance.

Project-Based Assessment: Assign learners the task of refurbishing a set of upholstery tools. They must assess the tools for any issues, perform necessary maintenance, and demonstrate their use in an upholstery project, documenting each step.

4. Evaluation and Feedback

Rubic for Exercises:

- Inspection Accuracy: How accurately learners identify defects and potential safety issues in tools.

- Maintenance Skills: Effectiveness in performing maintenance tasks such as sharpening, tightening, and lubricating.

- Application in Project: Quality of the maintenance work as evidenced in the performance of the tools during the upholstery project.

Through engaging in these activities, learners will not only enhance their ability to maintain upholstery tools in optimal condition but also ensure they can perform their tasks safely and efficiently. Proper tool maintenance is a critical skill that contributes significantly to the quality of the upholstery work and the longevity of the tools themselves.

### Applied Knowledge Learning Content for AK0701: Tacking and Covering Techniques

Mastering tacking and covering techniques is essential for upholstery professionals who strive to deliver high-quality furniture that meets both aesthetic and functional standards. This module provides detailed learning content on various tacking and covering methods, including practical examples to help learners apply these techniques effectively.

1. Overview of Tacking and Covering Techniques

Understanding different tacking and covering techniques allows upholstery professionals to adapt to various furniture styles and materials:

- Tacking Techniques:

- Traditional Tacking: Using upholstery tacks and a magnetic tack hammer to secure fabric to the furniture frame. Ideal for antique furniture where the use of modern staples might not be appropriate.

- Stapling: Employing an upholstery stapler to fasten fabric more quickly and securely, suitable for modern furniture and heavier fabrics.

- Covering Techniques:

- Pull and Stretch: The basic technique where fabric is pulled taut over the frame and secured. Essential for achieving a smooth finish without wrinkles or puckering.

- Pleating and Folding: Techniques used to handle excess fabric at corners or curves, ensuring a tailored fit and decorative detail.

- Blind Stitching: A method for stitching seams that are not visible on the final product, enhancing the furniture’s aesthetic appeal.

2. Practical Exercises

Exercise 1: Stapling and Tacking Practice

- Task: Learners practice attaching fabric to a furniture frame using both tacks and staples. They must consider the type of frame and fabric to decide the best attachment method.

- Objective: To develop proficiency in choosing and using the appropriate tacking method for different furniture types.

- Example: Securing a delicate fabric on an antique wooden chair frame using traditional tacks to preserve the wood and maintain authenticity.

Exercise 2: Fabric Stretching and Pleating Workshop

- Task: Learners apply fabric to various frame shapes, practicing stretching, pleating, and folding techniques to achieve a professional finish.

- Objective: To master the manipulation of fabric on complex frame shapes, ensuring a smooth and aesthetically pleasing result.

- Example: Upholstering a wingback chair, focusing on neatly pleating the fabric at the wings and arms.

3. Real-World Application

On-Site Training at an Upholstery Shop: Arrange for learners to spend a day in a professional upholstery shop, where they can observe and participate in real-world tacking and covering tasks under the guidance of experienced upholsterers.

Capstone Project: Each learner undertakes a project to fully upholster a piece of furniture, employing both tacking and covering techniques. They must document their process, highlighting the techniques used and the rationale behind their choices.

4. Evaluation and Feedback

Rubric for Exercises:

- Technique Proficiency: Accuracy and neatness in tacking and covering, including appropriate choice of tacks or staples.

- Quality of Finish: Absence of visible defects such as wrinkles or loose fabric, and neatness of pleats and folds.

- Creative Application and Problem Solving: Ability to adapt techniques to different frame styles and materials, demonstrating innovative problem-solving.

Through these educational activities, learners will not only refine their tacking and covering skills but also enhance their ability to make strategic decisions based on material and design requirements. This knowledge is essential for producing durable, beautiful furniture that stands the test of time.

### Applied Knowledge Learning Content for AK0702: Sewing Techniques

Sewing techniques are fundamental to the craft of upholstery, enabling professionals to assemble various fabric pieces into cohesive and aesthetically pleasing furniture covers. This module provides detailed learning content on essential sewing techniques, complete with practical examples to help learners refine their skills and apply them effectively in upholstery projects.

1. Overview of Sewing Techniques in Upholstery

Effective sewing techniques can greatly influence the final appearance and durability of upholstered furniture. Understanding various methods and when to use them is crucial:

- Straight Stitching: The most basic and commonly used stitch in upholstery for joining two pieces of fabric. It provides a strong, straight seam and is typically performed with a heavy-duty sewing machine.

- Sigsag Stitching: Used to finish fabric edges or to join stretchable materials. It prevents the edges from fraying and offers elasticity to the seam, making it ideal for materials like knitted fabrics.

- Top Stitching: A decorative stitching technique applied on top of a seam primarily for aesthetic purposes but also to add strength to the seam. It is visible and should be done with precision to enhance the furniture’s look.

- Blind Stitching: A method for creating seams that are virtually invisible on the face of the fabric. It is often used in areas where visible stitches may detract from the look of the furniture, such as on decorative pillows or fine upholstery.

- Piping and Welting: Techniques that involve sewing a cord wrapped in fabric into the seams of cushions, pillows, or furniture edges to define the shape and add a decorative touch.

2. Practical Exercises

Exercise 1: Seam Sewing Practice

- Task: Learners use various types of fabric to practice straight, sigsag, and top stitching. They must focus on adjusting the tension and stitch length to suit different fabric types.

- Objective: To gain proficiency in basic and decorative stitching techniques.

- Example: Sewing a straight seam on heavyweight upholstery fabric and then applying a top stitch for added durability and style.

Exercise 2: Piping and Blind Stitching Workshop

- Task: Learners create piping and then sew it into a seam, followed by practicing blind stitching on a sample piece.

- Objective: To master advanced sewing techniques that enhance the functionality and aesthetics of upholstered furniture.

- Example: Making and sewing piping along the edges of a cushion cover, then completing a blind stitch to close the final seam invisibly.

3. Real-World Application

Field Trip to a Textile Factory: Arrange a visit to a textile factory where industrial sewing machines and techniques are used. This can provide insight into the professional handling of various fabrics and sewing methods.

Project-Based Learning: Assign learners to design and sew a decorative throw pillow incorporating multiple sewing techniques learned in the module. They must use straight stitching, piping, and blind stitching, documenting each step and explaining their technique choices.

4. Evaluation and Feedback

Rubric for Exercises:

- Technique Proficiency: Accuracy and neatness in executing different stitches.

- Quality of Finish: Overall look and durability of the seams, particularly how well the blind stitches and piping are integrated into the project.

- Creative Application and Problem Solving: Effectiveness in choosing sewing techniques that enhance the design and functionality of the project.

Through these educational activities, learners will develop a comprehensive understanding of sewing techniques crucial for high-quality upholstery work. They will enhance their ability to choose the appropriate stitching method based on material properties and design requirements, ensuring their upholstery projects are not only durable but also visually appealing.

### Applied Knowledge Learning Content for AK0703: Faults and Maintenance

In the field of upholstery, maintaining tools and equipment in optimal condition and quickly identifying and addressing faults are essential skills that enhance both productivity and product quality. This module provides in-depth learning content on how to manage faults and perform routine maintenance, complete with examples to help learners understand the practical implications.

1. Overview of Fault Identification and Maintenance

Regular maintenance of upholstery tools and equipment ensures their longevity and reliability. Understanding common faults and knowing how to fix them prevents delays and potential accidents:

- Common Tool and Equipment Faults:

- Staple Gun Jams: Often caused by using incorrect staples or debris accumulation.

- Sewing Machine Issues: These can include thread bunching, inconsistent stitching, or needle breakage, often resulting from improper threading or incorrect tension settings.

- Dull Cutting Tools: Scissors or shears that become dull over time, making cutting difficult and imprecise.

- Routine Maintenance Practices:

- Regular Cleaning: Removing dust, fabric fibers, and other debris from sewing machines, staple guns, and cutting tools.

- Lubrication: Applying appropriate lubricants to moving parts to ensure smooth operation.

- Sharpening: Using sharpening stones or tools to maintain the edge of cutting instruments.

2. Practical Exercises

Exercise 1: Tool Fault Identification and Repair

- Task: Learners are presented with various upholstery tools that have been intentionally set up with common faults. They must identify the fault, diagnose the cause, and perform the necessary repair or adjustment.

- Objective: To develop the ability to quickly and accurately fix common tool issues.

- Example: A sewing machine that is skipping stitches. Learners would check threading, needle condition, and tension settings, making adjustments as necessary.

Exercise 2: Routine Maintenance Workshop

- Task: Conduct a hands-on maintenance session where learners perform cleaning, lubrication, and sharpening on a set of upholstery tools.

- Objective: To instill regular maintenance habits that prolong tool life and enhance performance.

- Example: Demonstrating how to clean and oil a sewing machine and how to sharpen shears using a sharpening stone.

3. Real-World Application

Maintenance Schedule Creation: Learners create a maintenance schedule for the upholstery workshop that includes daily, weekly, and monthly tasks to ensure all tools and equipment are kept in optimal condition.

Field Experience: Arrange for learners to spend a day with a maintenance technician specialising in upholstery equipment. They can observe advanced diagnostic and repair techniques that are crucial for maintaining industrial-grade machinery.

4. Evaluation and Feedback

Rubric for Exercises:

- Fault Identification Accuracy: Effectiveness in identifying and correctly diagnosing tool faults.

- Maintenance Skill Proficiency: Quality of maintenance performed, including thoroughness of cleaning, correctness of lubrication, and precision in sharpening.

- Application of Knowledge: Ability to apply maintenance skills in a real-world setting, ensuring tools and equipment function reliably and efficiently.

Through engaging with these educational activities, learners will gain essential skills in managing the upkeep of their tools and rectifying common faults, ensuring they can maintain a high standard of workmanship in their upholstery projects. This knowledge is crucial not only for personal productivity but also for safety and longevity in the upholstery profession.

### GUIDELINES

You are required to perform the task below and your Facilitator/Assessor will assess your work based on the observation checklist attached.

Practical Assessment Task for PM-02-PS07: Identify, prepare and safely apply hand tools and equipment for fitting covers to frames

Assessment Title:

Purpose

Guidelines to the Learner:

1. You will be presented with a range of samples or photographs of different wooden furniture products and accessories. These will include various styles, types, and uses.  Refer to Knowledge Module KM13 and the additional brochure with examples of upholstered furniture.

2. You are required to examine each sample or photograph and perform the following tasks:

Assessment Criteria:

Assessment Duration:

- The assessment should be completed within a specified time frame, typically 2-3 hours, to simulate a realistic working environment.

Feedback and Evaluation:

- Detailed feedback will be provided on the accuracy and relevance of your categorisations.

- The assessor will highlight areas of strength and suggest improvements where necessary.

Additional Notes:

- This practical assessment is designed to test not only your ability to recognise and categorise different furniture types and accessories but also your understanding of the broader context in which these items are used.

- It is essential to approach this task with attention to detail and a thorough understanding of the varied aspects of upholstered furniture design and manufacturing.

### Case Study: Upholstery Workshop Readiness at Custom Comfort Furniture

Scenario:

You are the workshop manager at Custom Comfort Furniture, responsible for ensuring that the upholstery section is fully equipped and operational for an upcoming large order of custom sofas and chairs. This involves checking the availability and condition of all necessary hand tools, ensuring that specific needles for blind stitching are stocked, and verifying that all tools are safe and effective for use.

Scenario-Based Questions:

Question 1: (IAC0701)

You are preparing for a busy week at the upholstery workshop. How do you ensure that all hand tools used for attaching specified covers to frames are available and ready for use?

Question 2: (IAC0702)

Blind stitching is a critical technique for your upcoming project. How do you verify that the right needles for blind stitching are available?

Question 3: (IAC0703)

Describe the process you would use to check and test hand tools and equipment for defects, sharpness, and safety, and outline the steps to rectify any problems found.

Marking Memo:

| Criteria | Exemplary (3 points) | Satisfactory (2 points) | Needs Improvement (1 point) |
| --- | --- | --- | --- |
| Availability of Tools and Needles | All required tools and needles are available, with extras on hand for emergency needs. | Most tools and needles are available; minor shortages that do not impact project start. | Significant shortages of tools and needles, potentially delaying project commencement. |
| Condition and Safety of Tools | Tools are in excellent condition, well-maintained, and all safety features are functional. | Tools are generally in good condition; minor maintenance issues noted but not critical. | Tools are poorly maintained, with safety features non-functional or missing, posing risks. |
| Effectiveness of Maintenance Actions | Maintenance actions are timely, effectively restoring tools to optimal condition, ensuring project readiness. | Maintenance is performed but with some delays; most tools restored to functional condition. | Ineffective maintenance actions, with continued tool malfunctions or safety hasards unresolved. |

This case study encourages learners to understand and apply comprehensive tool and equipment management practices, ensuring that they are prepared to deliver high-quality upholstery work efficiently and safely.

EVALUATION CHECKLIST

| NAME: …………………………………….…. SURNAME: ……………………………….…. ID…………………………………………….... | LEARNER SIGNATURE DATE: |
| --- | --- |

| EVALUATION CRITERIONS | MET REQUIREMENTS | DID NOT MEET REQUIREMENTS | COMMENTS OR ACTION REQUIRED |
| --- | --- | --- | --- |
| Ability of the learner to: |  |  |  |

| GENERAL COMMENTS OF OBSERVER: |  |  |
| --- | --- | --- |
| Assessor Name ……………………………… | Signature ……………………………………. | Date …………………………………. |

## PM-02-PS08: Identify, prepare and safely operate pneumatic tools for fitting covers to frames

Scope of Practical Skill

Given a range of specifications including technical drawings, written instructions, a range of samples and prototypes of upholstered furniture, prepared furniture frames including dining room seats, arm chairs, sofas, corner units, ottomans and foot stools, and prepared covers for the frames, the learner must be able to:

- PA0801 Interpret specifications and identify the size and type of pneumatic tools required

- PA0802 Identify and match the nails and staples to the correct pneumatic tools

- PA0803 Identify the parts of the tools, disassemble and assemble an air stapler and air gun, load the nails and staples into the correct power and pneumatic tools

- PA0804 Identify and check safety devices

- PA0805 Operate the tools according to standard operating procedures

- PA0806 Conduct minor maintenance by cleaning the magazine with a brush or compressed air, lubricate the catch mechanism with a drop of oil at specified intervals

- PA0807 Store the tools

Applied Knowledge

- AK0801 Compressed air and optimum air pressure required for various pneumatic tools

- AK0802 Consumables

- AK0803 Pneumatic tool maintenance

Internal Assessment Criteria

- IAC0801 The power and pneumatic staple and nailing guns are identified, checked, and tested to be fit for purpose

- IAC0802 Consumables for the power and pneumatic tools are identified, matched with and loaded into the correct power and pneumatic tools

- IAC0803 Tools are cleaned and minor maintenance conducted by cleaning the magazines and other moveable parts with a brush or compressed air, lubricating parts and catch mechanisms and checking their power supply lines and mechanisms for worn and broken airlines and connections.

- IAC0804 Tools are stored as per requirements

### Practical Application Learning Content for PA0801: Interpret Specifications and Identify the Sise and Type of Pneumatic Tools Required

Accurately interpreting specifications and identifying the correct pneumatic tools for upholstery projects are crucial skills that can significantly impact the efficiency and quality of work. This module provides detailed learning content on how to read and understand project specifications, and select the appropriate pneumatic tools based on those specifications, complete with practical examples.

1. Understanding Project Specifications

The ability to interpret detailed project specifications is essential for selecting the right pneumatic tools. Specifications typically include:

- Material Type: The type of fabric or leather and the frame material (wood, metal) often dictates the type of pneumatic tool required.

- Thickness and Hardness: These factors influence the sise and power of the tool needed to effectively penetrate the materials without causing damage.

- Project Scale: Larger projects might require more robust tools that can handle extended use without overheating or wear.

2. Pneumatic Tool Types

Different types of pneumatic tools are suited for various tasks in upholstery:

- Pneumatic Staple Guns: Used for fastening fabric to frames, available in various sises and gauges. The choice depends on the thickness of the material and the density of the frame.

- Pneumatic Nail Guns (Tacking Guns): Used for attaching heavier materials or where more hold strength is required. Nail sise and gun power vary based on application needs.

- Specialty Pneumatic Tools: Some tasks may require tools with unique capabilities, such as angle nailers for tight spaces.

3. Practical Exercises

Exercise 1: Specification Analysis

- Task: Learners are given specifications for three different upholstery projects: a delicate antique chair, a large commercial sofa, and a series of dining room chairs. They must determine which pneumatic tools are appropriate for each project.

- Objective: To practice interpreting specifications and making informed decisions on tool selection.

- Example: For the antique chair, a light-duty pneumatic staple gun with fine staples is chosen to avoid damaging the fragile frame. For the commercial sofa, a heavy-duty staple gun capable of handling thicker fabrics and denser foam is selected.

Exercise 2: Tool Matching Simulation

- Task: Learners use a provided tool catalog to match tools to a list of project specifications. They must consider factors such as tool sise, staple/nail sise, and material hardness.

- Objective: To enhance familiarity with the types of pneumatic tools available and their specific uses.

- Example: Matching a medium-duty pneumatic tacking gun with 1-inch nails for dining chairs that require secure fastening of a heavier fabric over a hardwood frame.

4. Real-World Application

Tool Selection Workshop: Conduct a workshop where learners can physically handle different pneumatic tools, observe their operation, and discuss their applications with an experienced upholsterer.

Project-Based Learning: Assign a project where learners must choose and use pneumatic tools to complete an upholstery task. They should justify their tool choices based on the project specifications and document their process.

5. Evaluation and Feedback

Rubric for Exercises:

- Accuracy in Tool Selection: How well learners select appropriate tools based on specifications.

- Understanding of Specifications: Learners' ability to interpret and apply information from specifications.

- Practical Skill Application: Effectiveness and appropriateness of tool use in the project-based learning exercise.

Through engaging with these activities, learners will develop a comprehensive understanding of how to interpret specifications and select suitable pneumatic tools, enhancing their efficiency and effectiveness in upholstery projects. This skill set is essential for professional upholstery work, ensuring high-quality results that meet or exceed project requirements.

### Practical Application Learning Content for PA0802: Identify and Match Nails and Staples to the Correct Pneumatic Tools

Properly matching nails and staples to their respective pneumatic tools is crucial for achieving optimal results in upholstery projects. This module provides learners with the knowledge and skills necessary to make informed choices about fasteners, ensuring they are compatible with the tools and suitable for the materials and tasks at hand.

1. Understanding Nails and Staples

Each type of fastener has specific characteristics designed for different upholstery applications:

- Staples: Come in various widths, lengths, and gauges. The choice of staple depends on the thickness and hardness of the material being fastened, as well as the durability required.

- Nails: Available in various lengths and diameters, often chosen based on the density of the material and the need for a more permanent fastening. Specialty nails may also be used for decorative purposes.

2. Matching Fasteners to Pneumatic Tools

The correct operation of pneumatic tools depends heavily on using the appropriate nails or staples:

- Tool Compatibility: Each pneumatic tool is designed to accommodate specific sises and types of fasteners. Using incompatible nails or staples can lead to tool damage, poor results, or safety hasards.

- Material Considerations: The choice of nails or staples also depends on the furniture piece's material. For instance, softer woods require different fasteners compared to harder woods or metal frames.

3. Practical Exercises

Exercise 1: Fastener Selection Quis

- Task: Learners are given a range of scenarios involving different furniture types and materials. They must select the appropriate staples or nails for each scenario, considering the material properties and the specifications of available pneumatic tools.

- Objective: To practice identifying the correct fasteners for various upholstery tasks.

- Example: Choosing the right gauge and length of staple for securing a heavy fabric to a solid hardwood frame without splitting the wood.

Exercise 2: Tool and Fastener Matching Activity

- Task: Provide learners with several pneumatic tools and a variety of nails and staples. Learners must match each fastener type to the correct tool and explain their choices.

- Objective: To enhance understanding of tool specifications and fastener compatibility.

- Example: Matching a narrow-crown staple gun with fine wire staples for delicate fabric work, ensuring that the staples fit the magasine and driver channel of the gun.

4. Real-World Application

Interactive Workshop: Organise a workshop where learners can experiment with different combinations of pneumatic tools and fasteners on sample materials, observing the results and discussing the outcomes with an instructor.

Project-Based Learning: Assign learners the task of reupholstering a piece of furniture, requiring them to choose and use specific pneumatic tools and fasteners. They must document their selections, the reasons for their choices, and any challenges encountered.

5. Evaluation and Feedback

Rubric for Exercises:

- Correctness of Fastener Selection: Accuracy in choosing fasteners that are appropriate for the material and task.

- Understanding of Tool Compatibility: Learners' ability to match fasteners with the correct pneumatic tools based on tool specifications.

- Application in Practical Projects: Effectiveness of fastener and tool choices in achieving a high-quality finish in the upholstery project.

Through these educational activities, learners will develop a comprehensive skill set for selecting and using the appropriate nails and staples for various upholstery projects. This knowledge ensures that upholstery work is not only aesthetically pleasing but also structurally sound and durable.

### Practical Application Learning Content for PA0803: Identify the Parts of the Tools, Disassemble and Assemble an Air Stapler and Air Gun, Load the Nails and Staples into the Correct Power and Pneumatic Tools

Understanding the mechanics of pneumatic tools such as air staplers and air guns is essential for effective and safe operation. This module provides comprehensive practical learning content to help learners master the skills necessary to identify tool components, disassemble and assemble tools, and properly load them with the correct fasteners.

1. Anatomy of Pneumatic Tools

First, learners need to be familiar with the basic parts and functions of pneumatic tools:

- Body: The main frame of the tool that houses internal mechanisms.

- Magasine: The part of the tool that holds the staples or nails. It typically includes a spring mechanism to feed the fasteners into the firing position.

- Trigger: The mechanism used to activate the tool’s firing operation.

- Safety Mechanism: A feature designed to prevent accidental firing. It usually requires that a specific part of the tool be pressed against the work surface to enable the trigger.

- Compressor Attachment: The point at which the air hose is connected, supplying compressed air to the tool.

- Exhaust: Where the air is expelled after each shot, which sometimes can be adjusted to direct the air flow away from the user.

2. Practical Exercises

Exercise 1: Tool Component Identification

- Task: Learners are provided with several disassembled pneumatic tools. They must identify all parts and discuss their functions.

- Objective: To familiarise learners with the components of air staplers and air guns.

- Example: Identifying and explaining the role of the magasine, compressor attachment, and safety mechanism in an air stapler.

Exercise 2: Disassembly and Assembly

- Task: Under supervision, learners disassemble and then reassemble air staplers and air guns, ensuring all parts are correctly and safely positioned.

- Objective: To practice the skills needed to maintain and repair pneumatic tools.

- Example: Carefully disassembling an air gun, inspecting each part for wear or damage, and then reassembling it correctly.

Exercise 3: Loading Fasteners

- Task: Learners use different models of air staplers and air guns to practice loading nails and staples, paying attention to the correct orientation and sise for each tool.

- Objective: To ensure learners can efficiently and safely load pneumatic tools with the appropriate fasteners.

- Example: Loading 1/2-inch staples into a narrow-crown stapler and 2-inch nails into a framing air gun, checking for correct fit and feed.

3. Real-World Application

Tool Maintenance Workshop: Organise a workshop focusing on the routine maintenance of pneumatic tools, including cleaning, lubricating, and troubleshooting common issues.

Project-Based Learning: Assign learners the task of using pneumatic tools to complete an upholstery project. They must demonstrate their ability to disassemble, clean, reassemble, and load the tools as part of their project workflow.

4. Evaluation and Feedback

Rubric for Exercises:

- Accuracy of Component Identification: How well learners identify and explain the function of each tool part.

- Proficiency in Disassembly and Assembly: Effectiveness and safety demonstrated during the disassembly and reassembly process.

- Correctness in Loading Fasteners: Accuracy in selecting and loading the correct fasteners into the tools.

Through these activities, learners will develop a thorough understanding of how pneumatic upholstery tools work. This knowledge ensures they can perform essential maintenance and are prepared to use these tools effectively and safely in their upholstery projects.

### Practical Application Learning Content for PA0804: Identify and Check Safety Devices

Safety is paramount when using pneumatic tools in upholstery. Understanding and ensuring the functionality of safety devices on these tools is critical to prevent accidents and ensure a safe working environment. This module provides detailed learning content on how to identify, check, and maintain safety devices on pneumatic tools, complete with practical examples.

1. Understanding Safety Devices on Pneumatic Tools

Safety devices are integrated into pneumatic tools to prevent accidental firing and other operational hasards. Familiarity with these mechanisms is essential:

- Trigger Lock: Prevents the tool from firing unless the lock is actively disengaged.

- Contact Safety Tip: Often located at the nose of the tool, it requires the tool to be pressed against the work surface before it will fire, preventing accidental discharge into the air.

- Anti-Jam Mechanism: While not a safety feature per se, it contributes to safe operation by preventing tool malfunctions that could lead to unsafe handling situations.

- Exhaust Systems: Directs the air away from the user and the work surface, preventing debris from being blown into the face or eyes.

2. Practical Exercises

Exercise 1: Safety Device Identification

- Task: Learners are provided with various models of pneumatic tools. They must identify and label each safety feature on these tools.

- Objective: To ensure learners can recognise and understand the purpose of different safety devices.

- Example: Identifying the trigger lock and contact safety tip on a pneumatic staple gun, explaining how each contributes to safe operation.

Exercise 2: Safety Check Simulation

- Task: Conduct a hands-on exercise where learners perform safety checks on pneumatic tools to ensure all safety devices are functioning correctly.

- Objective: To practice regular safety inspections as part of tool maintenance routines.

- Example: Checking the functionality of the contact safety tip by ensuring it requires appropriate pressure against a surface to activate the trigger.

3. Real-World Application

Safety Workshop: Organise a workshop with a focus on tool safety, inviting an expert to demonstrate the correct handling of tools with various safety features. Include troubleshooting common issues related to safety mechanisms.

Safety Audit Project: Assign learners the task of conducting a safety audit on the workshop’s pneumatic tool collection. They should check each tool’s safety devices, document any issues, and recommend actions to rectify identified problems.

4. Evaluation and Feedback

Rubric for Exercises:

- Understanding of Safety Devices: Accuracy in identifying and explaining the function of safety features.

- Effectiveness in Conducting Safety Checks: Thoroughness and accuracy in performing safety checks on pneumatic tools.

- Application of Safety Practices: Implementation of safety checks and maintenance in a real-world setting, demonstrating a proactive approach to safety.

By engaging with these educational activities, learners will gain a comprehensive understanding of the safety features of pneumatic upholstery tools. This knowledge is crucial not only for their own safety but also for maintaining a safe working environment, ensuring that upholstery projects are completed without incident.

### Practical Application Learning Content for PA0805: Operate the Tools According to Standard Operating Procedures

Operating pneumatic tools according to standard operating procedures (SOPs) is crucial for ensuring both safety and efficiency in upholstery projects. This module provides detailed learning content on how to use pneumatic tools properly, with a focus on SOPs that enhance the user's ability to achieve professional results while minimising risks.

1. Overview of Standard Operating Procedures for Pneumatic Tools

Standard Operating Procedures for pneumatic tools typically cover several key aspects:

- Preparation: Checking the tool's condition before use, ensuring it is clean, correctly assembled, and properly lubricated.

- Loading: Properly loading the fasteners (nails or staples) into the tool, ensuring they are the correct type and sise, and that the magasine is securely locked.

- Operation: Using the tool in the manner for which it was designed, including handling techniques that prevent recoil injuries and ensure precise application.

- Safety Measures: Wearing appropriate personal protective equipment (PPE), maintaining a clean work area, and following safety protocols to prevent accidental discharge or other injuries.

2. Practical Exercises

Exercise 1: Tool Preparation and Loading

- Task: Learners receive an unloaded pneumatic tool and must go through the steps of inspecting, cleaning, and loading the tool according to the SOPs.

- Objective: To ensure learners can prepare pneumatic tools safely and correctly for use.

- Example: Learners inspect a pneumatic staple gun for wear, clean any debris from the magasine, lubricate moving parts, and load it with the correct staples ensuring they fit properly and feed smoothly.

Exercise 2: Operating Pneumatic Tools

- Task: Using a variety of pneumatic tools, learners must perform specific upholstery tasks that replicate common job site conditions. They must follow all operational procedures, including proper stance and tool placement.

- Objective: To practice using pneumatic tools in a controlled environment to build confidence and skill in their operation.

- Example: Upholstering a chair seat by correctly positioning the pneumatic staple gun at the appropriate angle and depth to secure the fabric without damaging it or the underlying frame.

3. Real-World Application

Supervised Tool Operation Session: Arrange for learners to work on actual upholstery projects under supervision, where they can apply their skills in using pneumatic tools following SOPs. Feedback should be provided on their technique and adherence to safety practices.

SOP Development Activity: Have learners collaborate to create or revise a set of SOPs for a new pneumatic tool in the workshop. This should include steps for preparation, loading, operation, and safety.

4. Evaluation and Feedback

Rubric for Exercises:

- Preparation and Loading Accuracy: Accuracy in executing the preparation and loading phases according to SOPs.

- Operational Proficiency: Skill and precision in using the tools, including adherence to operational procedures.

- Safety Compliance: Consistency in following safety guidelines and using PPE appropriately during operations.

Through these educational activities, learners will develop a deep understanding of how to operate pneumatic tools correctly according to standard operating procedures. They will gain practical skills that enhance their efficiency and safety in upholstery work, ensuring high-quality results while minimising risks associated with tool use.

### Practical Application Learning Content for PA0806: Conduct Minor Maintenance on Pneumatic Tools

Routine maintenance of pneumatic tools is crucial to ensure their longevity and optimal performance. This module focuses on teaching learners how to conduct minor maintenance tasks, such as cleaning the magasine and lubricating the catch mechanism. By mastering these tasks, learners can prevent common issues like tool malfunctions and excessive wear.

1. Understanding Minor Maintenance Tasks

Minor maintenance tasks for pneumatic tools generally include:

- Cleaning the Magasine: This involves removing accumulated dust, debris, and oil residues that can affect the tool’s performance. This task is vital for preventing jams and ensuring smooth operation.

- Lubricating the Catch Mechanism: Proper lubrication reduces friction in moving parts, prevents rust, and ensures the tool operates efficiently.

2. Practical Exercises

Exercise 1: Cleaning the Magasine

- Task: Learners are provided with pneumatic tools that have been used in various upholstery tasks. They must clean the magasine using a brush or compressed air.

- Objective: To practice the correct cleaning techniques that prevent tool jams and maintain performance.

- Example: Using a soft brush to remove debris from the magasine of a pneumatic staple gun, followed by a blast of compressed air to clear out any remaining particles.

Exercise 2: Lubricating the Catch Mechanism

- Task: After cleaning, learners must apply lubricant to the catch mechanism of the tool.

- Objective: To ensure learners can correctly identify the lubrication points and apply the right amount of lubricant.

- Example: Dropping a small amount of machine oil on the moving parts of the catch mechanism, ensuring not to over-lubricate which can attract dirt and cause slippage.

3. Real-World Application

Maintenance Workshop: Conduct a workshop focusing on the routine maintenance of various types of pneumatic tools used in upholstery. Include a demonstration of more complex maintenance tasks and allow learners to practice under supervision.

Maintenance Log Activity: Have learners maintain a log for each tool they use during their training, recording each maintenance activity. This log can help learners understand the importance of regular maintenance and how it impacts tool performance.

4. Evaluation and Feedback

Rubric for Exercises:

- Thoroughness of Cleaning: Assess how effectively learners remove all visible debris and dust without damaging the tool.

- Accuracy of Lubrication: Evaluate whether learners apply the correct amount of lubricant to the right parts without over-lubricating.

- Adherence to Maintenance Schedules: Monitor how well learners follow prescribed maintenance intervals during their training projects.

These educational activities are designed to equip learners with the necessary skills to maintain their pneumatic tools effectively. By understanding and implementing routine maintenance tasks, learners ensure that their tools remain in excellent condition, thereby extending their lifespan and maintaining high performance in upholstery projects. This practice not only saves costs by reducing the need for repairs but also ensures safety and reliability in their operations.

### Practical Application Learning Content for PA0807: Store the Tools

Proper storage of pneumatic tools is crucial for maintaining their longevity and ensuring they are safe and ready to use for the next project. This module provides guidance on effective storage techniques, helping learners understand the best practices for keeping pneavy tools in optimal condition.

1. Importance of Proper Tool Storage

Proper storage of pneumatic tools prevents damage, protects against environmental elements, and helps maintain organisation in the workspace. It also reduces the risk of accidental injury and ensures tools are in good working order when needed.

2. Practical Exercises

Exercise 1: Storage Setup

- Task: Learners design and set up a storage system for pneumatic tools in a mock workshop environment. They must consider factors like accessibility, safety, and protection from environmental factors.

- Objective: To practice creating an efficient and safe storage system.

- Example: Designing a pegboard system where each tool has a labeled spot. Tools are stored with safety caps on, and smaller accessories like staples and nails are kept in labeled bins underneath.

Exercise 2: Cleaning and Preparing Tools for Storage

- Task: After using the pneumatic tools in a project, learners must clean and prepare them for storage following a checklist of steps to ensure they are ready for future use.

- Objective: To reinforce the importance of tool maintenance before storage.

- Example: Cleaning the tool with compressed air, applying a light coat of lubricant to prevent rust, and checking that all safety locks are engaged before placing the tool on the pegboard.

3. Real-World Application

Storage Audit Project: Assign learners to evaluate and improve the storage systems in an existing workshop. They should identify shortcomings in the current system and implement improvements based on best practices learned in the module.

Routine Storage Practice: During their training period, learners are responsible for storing all tools used at the end of each day. This routine helps reinforce good storage habits and teaches responsibility and care for tools.

4. Evaluation and Feedback

Rubic for Exercises:

- Effectiveness of Storage Design: How well the storage system meets the needs for safety, accessibility, and protection from environmental damage.

- Thoroughness in Preparing Tools for Storage: Accuracy and completeness in following the prescribed steps for cleaning and preparing tools for storage.

- Consistency in Routine Practices: Regularity and reliability in storing tools properly at the end of each workshop session.

Through these educational activities, learners will develop a thorough understanding of the importance of proper tool storage and the best practices to implement it effectively. By ensuring pneumatic tools are well-maintained and correctly stored, learners can extend the life of the tools and maintain a safer, more organised workshop environment.

### Applied Knowledge Learning Content for AK0801: Compressed Air and Optimum Air Pressure Required for Various Pneumatic Tools

Understanding the role of compressed air in operating pneumatic tools and knowing the optimum air pressure required for different types of tools are crucial for efficient and safe operations. This module offers comprehensive insights into the mechanics of compressed air systems, including how to adjust and maintain the correct air pressure for various pneumatic tools used in upholstery.

1. Basics of Compressed Air for Pneumatic Tools

Pneumatic tools operate using compressed air delivered through a compressor, which stores air in a pressurised tank. The effectiveness of these tools depends significantly on using the correct air pressure, which can vary depending on the tool and the task:

- CFM (Cubic Feet per Minute): This measures the volume of air that a compressor can deliver at a specific pressure. Different tools require different CFM ratings to function correctly.

- PSI (Pounds per Square Inch): This is the pressure at which the air is delivered. Pneumatic tools have recommended PSI settings based on the tool’s design and intended use.

2. Practical Exercises

Exercise 1: Understanding Air Pressure Requirements

- Task: Learners are given data sheets for various pneumatic tools including staple guns, nail guns, and sanders. They must identify the CFM and PSI requirements for each tool and compare these with the capabilities of available compressors.

- Objective: To familiarise learners with the air pressure and volume requirements of different tools.

- Example: Determining that a pneumatic staple gun requires 2 CFM at 90 PSI to operate efficiently and checking if the workshop’s compressor can meet this demand.

Exercise 2: Adjusting Air Pressure

- Task: Using a compressor, learners adjust the air pressure to match the requirements for operating a pneumatic nail gun used for framing.

- Objective: To practice setting the correct air pressure for a tool, ensuring optimal performance and safety.

- Example: Adjusting the compressor to deliver 4 CFM at 120 PSI for a heavy-duty nail gun and testing the tool to ensure it operates smoothly without any malfunction.

3. Real-World Application

Compressor Maintenance Workshop: Organise a workshop where learners can learn about maintaining air compressors, including checking for leaks, changing filters, and servicing the motor and tank to ensure optimal performance.

Tool and Compressor Setup Task: Each learner sets up a pneumatic tool from start to finish, including connecting the tool to the compressor, adjusting the air pressure, and testing the tool on various materials.

4. Evaluation and Feedback

Rubric for Exercises:

- Accuracy in Identifying Requirements: How well learners can identify and articulate the air volume and pressure requirements for different tools.

- Skill in Adjusting Air Pressure: Effectiveness and precision in adjusting compressors to the correct settings for various tools.

- Application of Knowledge: Ability to apply theoretical knowledge in real-world settings, effectively setting up and using pneumatic tools.

Through engaging with these activities, learners will develop a thorough understanding of how compressed air systems work and how to correctly use and maintain them in upholstery projects. This knowledge ensures that pneumatic tools are used efficiently, safely, and effectively, maintaining high standards of craftsmanship in upholstery work.

### Applied Knowledge Learning Content for AK0802: Consumables

Understanding the role and management of consumables in the operation of pneumatic tools is essential for upholstery professionals. This module provides a detailed exploration of various consumables such as nails, staples, and oils that are vital for the proper functioning of pneumatic tools, offering practical examples to deepen learners' understanding and enhance their efficiency.

1. Overview of Consumables for Pneumatic Tools

Consumables in the context of pneumatic tools include all the components that need to be replenished regularly:

- Nails and Staples: These vary in sise and type, selected based on the specific requirements of the upholstery project. For instance, finer staples may be used for delicate fabrics, while thicker, more robust nails may be necessary for securing hardwood materials.

- Lubricants: Oils and greases are crucial for maintaining the smooth operation of pneumatic tools. Proper lubrication reduces wear and tear and prevents mechanical failures.

- Air Tool Oil: This specific type of lubricant is designed to cope with the high-speed operation of pneumatic tools, ensuring that internal components are protected against friction and corrosion.

2. Practical Exercises

Exercise 1: Selecting Nails and Staples

- Task: Learners are given different types of upholstery projects, from delicate antique restoration to robust commercial furniture production. They must choose the appropriate nails or staples for each project.

- Objective: To enhance the ability to select the right consumable based on material, tool type, and project needs.

- Example: Choosing 1/4-inch staples for securing light fabric on a softwood frame and 1-inch nails for attaching heavy-duty fabric to a metal frame.

Exercise 2: Lubrication Techniques

- Task: Learners perform lubrication on various pneumatic tools using the correct type of air tool oil.

- Objective: To develop routine maintenance skills that prolong the life and efficiency of pneumatic tools.

- Example: Demonstrating the proper application of air tool oil to a pneumatic staple gun, focusing on the frequency of lubrication based on usage patterns.

3. Real-World Application

Consumable Management Workshop: Conduct a workshop focused on the efficient management and use of consumables, including inventory tracking, proper storage, and handling practices to maximise usability and minimise waste.

Project-Based Learning: Learners undertake a project where they are responsible for all aspects of consumable management, from selection through application, documenting their choices and the reasons behind them.

4. Evaluation and Feedback

Rubric for Exercises:

- Accuracy in Consumable Selection: Measures how effectively learners choose consumables suitable for different types of upholstery tasks.

- Proficiency in Maintenance Practices: Assesses the learner's ability to apply proper lubrication techniques and the understanding of maintenance frequency.

- Application in Project Management: Evaluates the learner's ability to manage consumables efficiently in a project setting, focusing on minimising waste and ensuring quality.

Through engaging with these educational activities, learners will gain a comprehensive understanding of consumables related to pneumatic tools, enhancing their ability to maintain and use these tools effectively. This knowledge is crucial for ensuring that upholstery projects are completed efficiently, with high standards of quality and durability.

### Applied Knowledge Learning Content for AK0803: Pneumatic Tool Maintenance

Effective maintenance of pneumatic tools is crucial for ensuring their longevity and reliability in upholstery projects. This module provides comprehensive insights into maintaining these tools, including the steps for routine checks and repairs, as well as practical examples to help learners implement these maintenance practices proficiently.

1. Overview of Pneumatic Tool Maintenance

Regular maintenance of pneumatic tools involves several key practices that help prevent mechanical failures and ensure the tools operate efficiently:

- Daily Inspection: Checking the tool for any signs of wear or damage, ensuring all parts are secure and functional.

- Cleaning: Removing debris, dust, and lubricant buildup from the tool, especially from the air vents and moving parts.

- Lubrication: Applying the correct type of oil to moving parts to reduce friction and wear, which is critical for the high-speed operation of pneumatic tools.

- Air Filter Check: Inspecting and cleaning or replacing air filters regularly to ensure optimal air flow and prevent contaminants from entering the tool.

2. Practical Exercises

Exercise 1: Daily Inspection and Cleaning

- Task: Learners perform a daily inspection and cleaning on a pneumatic staple gun and an air nailer.

- Objective: To master the routine of inspecting and cleaning pneumatic tools, identifying potential issues before they lead to tool failure.

- Example: Checking an air nailer for loose screws and worn seals, cleaning around the trigger and the head where debris tends to accumulate.

Exercise 2: Lubrication and Air Filter Maintenance

- Task: Learners lubricate the moving parts of a pneumatic tool and check or replace the air filter.

- Objective: To practice proper lubrication techniques and understand the importance of clean air supply in pneumatic tool performance.

- Example: Applying a few drops of air tool oil to the air inlet of a pneumatic hammer and replacing a clogged air filter with a new one.

3. Real-World Application

Maintenance Schedule Development: Guide learners to develop a maintenance schedule for pneumatic tools used in an upholstery shop, including daily, weekly, and monthly tasks.

Maintenance Workshop: Organise a workshop where learners can disassemble, diagnose, and repair common issues in pneumatic tools under the guidance of an experienced technician.

4. Evaluation and Feedback

Rubric for Exercises:

- Effectiveness in Inspection and Cleaning: Assesses how thoroughly the learner inspects and cleans the tool, identifying any minor issues that could escalate into major problems.

- Proficiency in Lubrication and Filter Maintenance: Evaluates the accuracy and appropriateness of lubrication and the timely replacement of air filters.

- Consistency in Maintenance Practices: Measures the learner’s adherence to the developed maintenance schedule during project-based learning.

Through engaging with these educational activities, learners will acquire essential skills in maintaining pneumatic tools, which will enhance their ability to perform upholstery work efficiently and safely. These maintenance skills not only prevent downtime due to tool failures but also extend the lifespan of the tools, making them more cost-effective for the business.

### GUIDELINES

You are required to perform the task below and your Facilitator/Assessor will assess your work based on the observation checklist attached.

Practical Assessment Task for PM-02-PS08: Identify, prepare and safely operate pneumatic tools for fitting covers to frames

Assessment Title:

Purpose

Guidelines to the Learner:

1. You will be presented with a range of samples or photographs of different wooden furniture products and accessories. These will include various styles, types, and uses.  Refer to Knowledge Module KM13 and the additional brochure with examples of upholstered furniture.

2. You are required to examine each sample or photograph and perform the following tasks:

Assessment Criteria:

Assessment Duration:

- The assessment should be completed within a specified time frame, typically 2-3 hours, to simulate a realistic working environment.

Feedback and Evaluation:

- Detailed feedback will be provided on the accuracy and relevance of your categorisations.

- The assessor will highlight areas of strength and suggest improvements where necessary.

Additional Notes:

- This practical assessment is designed to test not only your ability to recognise and categorise different furniture types and accessories but also your understanding of the broader context in which these items are used.

- It is essential to approach this task with attention to detail and a thorough understanding of the varied aspects of upholstered furniture design and manufacturing.

### Case Study: Equipment Readiness Assessment at Elite Upholstery Services

Scenario:

You are the workshop supervisor at Elite Upholstery Services, tasked with ensuring that all upholstery tools and equipment are ready for a major upcoming project involving the production of high-end office furniture. This project requires precision and reliability from both your team and the equipment they use.

Scenario-Based Questions:

Question 1: (IAC0801)

Describe the process you would use to ensure that both the power and pneumatic staple and nailing guns are identified, checked, and tested to be fit for purpose for the upcoming project.

Question 2: (IAC0802)

How would you ensure that the consumables for the power and pneumatic tools are correctly identified, matched with, and loaded into the appropriate tools?

Question 3: (IAC0803)

Explain how you would clean and conduct minor maintenance on the tools, detailing each step you take.

Question 4: (IAC0804)

What steps would you take to ensure that all tools are stored according to requirements after maintenance?

Marking Memo:

| Criteria | Exemplary (3 points) | Satisfactory (2 points) | Needs Improvement (1 point) |
| --- | --- | --- | --- |
| Tool Testing and Inspection | Thoroughly tests all tools and effectively identifies any issues needing repair. | Adequately tests tools but may overlook minor issues. | Insufficient testing, missing significant functional problems. |
| Consumable Management | Correctly identifies and perfectly matches consumables to tools, confirmed by successful test firing. | Mostly correct, but with minor errors in matching, not impacting performance. | Incorrectly matches consumables, leading to tool malfunctions or damage. |
| Maintenance and Cleaning | Performs comprehensive cleaning and maintenance, enhancing tool performance and safety. | Conducts basic maintenance, but some areas are not thoroughly addressed. | Neglects necessary maintenance, leading to potential tool failure. |
| Storage Practices | Stores tools in an organised, secure manner that protects and prolongs tool life. | Generally stores tools correctly but lacks optimal organisation. | Poor storage practices that risk damage or loss of tools. |

Through this case study, learners demonstrate their ability to manage and maintain upholstery tools effectively, ensuring they are fully prepared for intensive use in upcoming projects. This scenario underscores the importance of meticulous equipment care in maintaining high standards of operational efficiency and safety in an upholstery workshop.

EVALUATION CHECKLIST

| NAME: …………………………………….…. SURNAME: ……………………………….…. ID…………………………………………….... | LEARNER SIGNATURE DATE: |
| --- | --- |

| EVALUATION CRITERIONS | MET REQUIREMENTS | DID NOT MEET REQUIREMENTS | COMMENTS OR ACTION REQUIRED |
| --- | --- | --- | --- |
| Ability of the learner to: |  |  |  |

| GENERAL COMMENTS OF OBSERVER: |  |  |
| --- | --- | --- |
| Assessor Name ……………………………… | Signature ……………………………………. | Date …………………………………. |

## PM-02-PS09: Start and stop a compressor and connect pneumatic tools while following safe standard operating procedures

Scope of Practical Skill

Given access to, and pictures of, various types of portable and fixed compressors, air lines and pneumatic tools the learner must be able to:

- PA0901 Identify the two main types of compressor

- PA0902 Check that planned maintenance has taken place as scheduled, and that legal certification is up to date

- PA0903 Identify and check safety devices and do pre-start checks and procedures in the correct sequence

- PA0904 Monitor the oil and air pressure gauges and follow procedures to report problems

- PA0905 Connect air lines to the compressor or air outlet and connect and operate pneumatic tools at the required air pressure

- PA0906 Do shut down procedures in the correct sequence and clean the area around the compressor

Applied Knowledge

- AK0901 Types of compressors and their uses

- AK0902 Compliance with the Occupational Health and Safety Act Regulations for vessels under pressure

- AK0903 Standard safe operating procedures

Internal Assessment Criteria

- IAC0901 Types of compressor identified

- IAC0902 Compressor maintenance records and legal certification checked and required steps taken if there is a problem

- IAC0903 Start up, monitor the oil and air pressure gauges, and shut down the compressor and dryer following the correct sequence

### Practical Application Learning Content for PA0901: Identify the Two Main Types of Compressor

Understanding the different types of compressors is fundamental for anyone working with pneumatic tools, as the choice of compressor can significantly impact the efficiency and suitability of the work being performed. This module aims to equip learners with the knowledge to identify and differentiate between the two main types of compressors used in industrial and workshop settings: portable compressors and fixed (stationary) compressors.

1. Overview of Compressor Types

- Portable Compressors: These are smaller, lightweight, and designed for easy transportation around a site or between locations. They are ideal for light to medium-duty tasks and are commonly used where the work area changes frequently or in remote locations without direct electrical power.

- Fixed Compressors: Larger and designed for permanent installation in a single location, fixed compressors have a higher capacity and can deliver more power. They are suited for heavy-duty use in industrial applications where a constant supply of compressed air is required.

2. Practical Exercises

Exercise 1: Identification and Features Overview

- Task: Learners are presented with several compressors, both in images and in a workshop setting. They must identify which are portable and which are fixed, noting key features that help differentiate them.

- Objective: To enable learners to visually identify and categorise different types of compressors based on their physical characteristics and capabilities.

- Example: Learners identify features such as sise, mobility (presence of wheels), storage capacity, and power source (electricity or gasoline).

Exercise 2: Suitability Assessment

- Task: Given a series of job scenarios, learners must decide which type of compressor is most suitable for each scenario, explaining their choice based on the compressor's features.

- Objective: To apply knowledge of compressor types to real-world applications, ensuring the best fit for specific tasks.

- Example: Choosing a portable compressor for a furniture restoration project at a client's home vs. selecting a fixed compressor for continuous use in a large-scale furniture manufacturing workshop.

3. Real-World Application

Field Trip: Organise a visit to a local dealer or a large workshop where different types of compressors are used. This will allow learners to see these compressors in operation and understand their applications in a professional setting.

Comparative Study Project: Assign learners to research and compile a report on various brands and models of portable and fixed compressors, focusing on performance metrics like CFM output, power requirements, maintenance needs, and typical use cases.

4. Evaluation and Feedback

Rubric for Exercises:

- Accuracy in Identification: Effectiveness in correctly identifying compressors as portable or fixed based on visual and functional characteristics.

- Understanding of Features and Applications: Depth of understanding shown in the explanations for choosing specific compressor types for given scenarios.

- Quality of Comparative Study: Thoroughness and technical accuracy in comparing different compressor models and their suitability for various tasks.

Through engaging with these activities, learners will develop a thorough understanding of the primary types of compressors, enhancing their ability to choose the right equipment for specific upholstery projects. This foundational knowledge is essential for efficient and effective use of pneumatic tools in various upholstery tasks.

### Practical Application Learning Content for PA0902: Check that Planned Maintenance has Taken Place as Scheduled, and that Legal Certification is Up to Date

Ensuring that compressors are regularly maintained and legally certified is critical for their safe and efficient operation. This module provides learners with the necessary knowledge and skills to verify maintenance records and compliance with legal certifications for compressors.

1. Understanding Maintenance and Certification Requirements

- Scheduled Maintenance: Regular maintenance of compressors includes checking and changing the oil, inspecting filters and hoses, and ensuring that all components are functioning correctly. Scheduled maintenance helps prevent mechanical failures and extends the life of the equipment.

- Legal Certification: Depending on the jurisdiction, compressors may require periodic inspections and certifications to ensure they meet specific safety standards. These certifications must be kept current and be readily available for review.

2. Practical Exercises

Exercise 1: Maintenance Record Review

- Task: Learners are given access to maintenance records for various types of compressors. They must review these records to confirm that all scheduled maintenance has been performed as required.

- Objective: To ensure learners can accurately interpret maintenance logs and understand the importance of regular upkeep.

- Example: Verifying that an oil change and filter replacement were completed on a fixed compressor according to the manufacturer’s recommended intervals.

Exercise 2: Certification Verification

- Task: Learners are provided with a set of documents pertaining to the legal certification of compressors. They must check the dates and details to ensure that each compressor is compliant with local safety regulations.

- Objective: To practice confirming the validity of legal certifications and understand the regulatory requirements for compressor operation.

- Example: Ensuring that a portable compressor has a valid pressure vessel certificate that meets the national safety standards and has not expired.

3. Real-World Application

Audit Simulation: Conduct a simulated audit where learners must perform a comprehensive check of maintenance and certification for all compressors in a workshop setting. This simulation includes assessing physical conditions of the compressors, reviewing maintenance records, and verifying legal documents.

Workshop Organisation: Task learners with organising and updating a maintenance and certification tracking system for a workshop’s compressors, using software or a manual tracking method to keep records up-to-date.

4. Evaluation and Feedback

Rubric for Exercises:

- Accuracy in Record Review: Effectiveness in identifying discrepancies in maintenance logs and understanding the implications of missed maintenance.

- Compliance Verification: Accuracy and thoroughness in checking legal certifications and understanding regulatory requirements.

- System Organisation Skills: Competence in setting up or maintaining an organisational system for tracking maintenance and certifications.

By engaging with these activities, learners will gain essential skills in managing the upkeep and compliance of compressors, ensuring that they are always ready for use and operate within the legal requirements. This knowledge is crucial for maintaining high safety and efficiency standards in any workshop using pneumatic tools.

### Practical Application Learning Content for PA0903: Identify and Check Safety Devices and Conduct Pre-start Checks and Procedures

Ensuring that pneumatic tools and compressors are safe before use is crucial to prevent accidents and maintain operational efficiency. This module provides detailed practical learning content to help learners systematically identify safety devices, conduct pre-start checks, and follow correct procedures to ensure equipment is ready and safe for operation.

1. Understanding Safety Devices and Pre-start Procedures

Safety devices on compressors and pneumatic tools can include pressure relief valves, emergency shutoffs, and thermal overload protectors. These devices are designed to prevent equipment failure and safeguard users from potential hasards. Pre-start checks involve inspecting these devices, ensuring connections are secure, and verifying that the equipment is in good working condition.

2. Practical Exercises

Exercise 1: Safety Device Identification and Inspection

- Task: Learners are presented with various types of compressors and pneumatic tools and must identify and inspect each safety device present on the equipment.

- Objective: To familiarise learners with different safety devices and their functions, ensuring they know how to check each device properly.

- Example: Identifying and testing the pressure relief valve on an air compressor to ensure it opens at the correct pressure setting.

Exercise 2: Conducting Pre-start Checks

- Task: Using a checklist, learners perform a series of pre-start checks on a compressor setup, including inspecting hoses, connections, and overall equipment condition.

- Objective: To practice the sequence of operational checks that confirm the equipment is safe and ready for use.

- Example: Checking for any air leaks in the hose connections, ensuring all fittings are tight, and verifying that the air filter is clean and properly installed.

3. Real-World Application

Safety Audit Project: Learners conduct a safety audit on the workshop’s pneumatic equipment, using a standardised checklist to assess and record the condition of safety devices and readiness of the equipment.

Maintenance and Safety Workshop: Organise a workshop where learners can demonstrate their ability to perform pre-start checks and safety inspections under the guidance of an experienced technician, followed by a discussion on troubleshooting common issues found during these checks.

4. Evaluation and Feedback

Rubric for Exercises:

- Accuracy in Identifying Safety Devices: How effectively learners can identify and explain the purpose of each safety device.

- Thoroughness of Pre-start Checks: The completeness and accuracy of the pre-start checks performed, with attention to detail in following the prescribed sequence.

- Application of Safety Practices: Learners' ability to apply these checks in a real-world setting, demonstrating a proactive approach to equipment safety.

By engaging in these activities, learners will develop a comprehensive understanding of the critical role that safety devices and pre-start checks play in the operation of pneumatic tools and compressors. This knowledge ensures that all equipment is operated safely and efficiently, maintaining high standards of workplace safety and operational readiness.

### Practical Application Learning Content for PA0904: Monitor the Oil and Air Pressure Gauges and Follow Procedures to Report Problems

Effective monitoring of oil and air pressure gauges is crucial for maintaining the operational integrity and safety of pneumatic tools and compressors. This module provides detailed instructions and examples on how to accurately monitor these gauges and follow proper procedures for reporting any identified issues.

1. Understanding Gauge Monitoring

- Oil Pressure Gauges: These gauges monitor the oil pressure in the compressor, which is crucial for lubricating moving parts. Low oil pressure can indicate a leak or that oil needs to be topped up, which if ignored, can lead to compressor failure.

- Air Pressure Gauges: These measure the pressure of the air delivered by the compressor. It’s essential to ensure that the pressure matches the tool requirements. Over-pressurisation can lead to tool damage or pose a safety risk, while under-pressurisation can result in inadequate tool performance.

2. Practical Exercises

Exercise 1: Reading and Interpreting Gauges

- Task: Learners are provided with various scenarios where they read oil and air pressure gauges on a compressor. They must interpret the readings and decide whether they fall within the normal operating range.

- Objective: To practice accurately reading gauges and understanding their implications for compressor and tool performance.

- Example: Learners observe an oil pressure gauge reading that is lower than the recommended level and must decide the appropriate action based on the equipment’s manual.

Exercise 2: Reporting Procedures

- Task: After identifying an issue from gauge readings, learners follow a standardised procedure to report the problem. This might involve documenting the issue in a maintenance log, notifying a supervisor, or initiating a service request.

- Objective: To ensure learners are familiar with and can effectively use reporting protocols to address maintenance issues.

- Example: Learners find that the air pressure is consistently too high, risking tool and safety integrity. They must complete a maintenance request form detailing their findings and the potential risks.

3. Real-World Application

Simulated Monitoring Drill: Set up a continuous monitoring drill where learners monitor oil and air pressure gauges over a day. They record the readings at regular intervals to practice identifying and responding to any fluctuations.

Maintenance Report Writing Workshop: Conduct a workshop on how to write effective maintenance reports, focusing on clarity, accuracy, and the importance of timely communication in maintenance practices.

4. Evaluation and Feedback

Rubric for Exercises:

- Accuracy in Reading Gauges: Evaluates how precisely learners can read and interpret gauge data.

- Effectiveness in Identifying Issues: Assesses the learner’s ability to correctly identify when gauge readings indicate a potential problem.

- Proficiency in Reporting: Measures how well learners follow reporting procedures, including the clarity and detail of the reports they produce.

Through these activities, learners will gain the skills necessary to monitor critical operational gauges and effectively communicate issues, ensuring that pneumatic equipment is maintained at optimal conditions for safe and efficient operation. These practices are vital for preventing equipment failure and ensuring workplace safety.

### Practical Application Learning Content for PA0905: Connect Air Lines to the Compressor or Air Outlet and Connect and Operate Pneumatic Tools at the Required Air Pressure

Connecting and operating pneumatic tools correctly is crucial for ensuring their efficiency and longevity. This module offers comprehensive guidance on how to properly connect air lines to compressors or air outlets and how to operate pneumatic tools at the correct air pressure.

1. Understanding Connection and Operation Procedures

- Air Line Connections: Securely attaching air lines to the compressor and ensuring there are no leaks is critical for maintaining consistent air pressure and tool performance.

- Tool Connection: Connecting pneumatic tools to the air lines requires understanding of the type of connectors used and ensuring a tight fit to avoid air leaks and potential pressure drops.

- Operating Pressure: Each tool has a specific air pressure requirement, typically listed in the tool’s manual. Operating a tool at its correct pressure ensures optimal performance and reduces wear on the tool.

2. Practical Exercises

Exercise 1: Connecting Air Lines and Tools

- Task: Learners are given a variety of compressors and pneumatic tools and must correctly attach the air lines and tools. They use soap solution to check for leaks at the connections.

- Objective: To practice making secure connections and to emphasise the importance of leak testing.

- Example: Learners connect a pneumatic staple gun to a compressor using a quick-connect fitting. After ensuring the connection is secure, they apply a soap solution to check for air leaks.

Exercise 2: Setting and Monitoring Air Pressure

- Task: Learners set the air pressure on the compressor according to the specifications of the pneumatic tool they are using. They then operate the tool and monitor its performance, making adjustments to the air pressure if needed.

- Objective: To gain proficiency in adjusting and maintaining the correct air pressure for different tools.

- Example: Adjusting the compressor to deliver 90 PSI for a finishing nailer and testing the tool on a piece of scrap wood to ensure it drives nails correctly without damaging the wood or the tool.

3. Real-World Application

On-Site Training Session: Organise a session at a local workshop where learners can connect and use various pneumatic tools on actual projects, under the supervision of experienced professionals.

Tool and Compressor Setup Contest: Conduct a timed contest where learners demonstrate their ability to quickly and correctly set up tools and compressors, emphasising speed, accuracy, and safety.

4. Evaluation and Feedback

Rubric for Exercises:

- Connection Accuracy: How effectively learners connect air lines and tools without causing leaks.

- Pressure Setting and Adjustment: Accuracy in setting the air pressure to the tool’s specifications and making fine adjustments based on the tool’s performance.

- Operational Efficiency: How smoothly and effectively the tool operates once connected, with additional points for identifying and correcting issues without guidance.

Through engaging in these exercises, learners will develop a solid foundation in connecting and operating pneumatic tools efficiently. These skills are essential for ensuring that the tools perform effectively and safely, reducing downtime and maintenance costs while increasing productivity in upholstery projects.

### Practical Application Learning Content for PA0906: Execute Shutdown Procedures in the Correct Sequence and Clean the Area Around the Compressor

Proper shutdown of compressors and cleaning the surrounding area are crucial steps to ensure safety, extend the life of the equipment, and maintain a professional and safe working environment. This module provides a systematic approach to performing these tasks, with detailed steps and practical examples.

1. Overview of Shutdown Procedures

Shutdown procedures are designed to safely decompress equipment, turn off power sources, and secure the area. These steps prevent accidental starts and reduce wear on the compressor.

2. Practical Exercises

Exercise 1: Standard Shutdown Procedure

- Task: Learners perform a step-by-step shutdown of a pneumatic compressor system, including turning off the power, releasing air pressure safely, and disconnecting the air lines.

- Objective: To ensure learners can safely and efficiently shut down pneumatic equipment following the correct sequence.

- Example:

1. Turn off the compressor switch to stop air compression.

2. Open the release valve to safely vent remaining air pressure.

3. Disconnect the air lines from the compressor and tools.

4. Turn off the main power supply at the circuit breaker if required.

Exercise 2: Cleaning and Organising the Work Area

- Task: After shutting down the compressor, learners clean and organise the area, including proper storage of tools and disposal of any debris or waste material.

- Objective: To cultivate habits of maintaining a clean and organised workspace, which is essential for safety and efficiency.

- Example:

1. Coil air hoses neatly and hang them on designated hooks.

2. Place tools back in their assigned storage locations.

3. Sweep the area to remove debris and wipe down surfaces to remove oil or moisture.

3. Real-World Application

Safety and Maintenance Workshop: Organise a workshop focusing on equipment maintenance and safe shutdown practices, where learners can practice these skills on different types of compressors and settings.

Routine Shutdown Audits: Implement a routine where learners are responsible for shutting down and cleaning up at the end of each day in a training workshop, with feedback provided on their adherence to procedures.

4. Evaluation and Feedback

Rubric for Exercises:

- Adherence to Shutdown Procedures: Evaluates how closely learners follow the prescribed shutdown steps without skipping or incorrectly performing any actions.

- Thoroughness of Cleaning: Assesses the effectiveness of cleaning the area, including proper storage of tools and removal of all waste and debris.

- Operations Safety Compliance: Measures compliance with safety standards during and after shutdown, including checking for potential hasards.

Through these educational activities, learners will develop a strong understanding of the importance of proper equipment shutdown and workspace management. These practices not only ensure the longevity and reliability of the equipment but also contribute to a safer and more productive work environment.

### Applied Knowledge Learning Content for AK0901: Types of Compressors and Their Uses

Understanding the various types of compressors and their specific applications is crucial for professionals working with pneumatic tools in various industries. This module provides comprehensive insights into the common types of compressors, their characteristics, and how they are best utilised in different settings.

1. Overview of Common Types of Compressors

Compressors are essential for powering pneumatic tools by converting power into potential energy stored as compressed air. The choice of compressor depends on factors like required air pressure, volume, and the nature of the application:

- Reciprocating Compressors: Often used in smaller shops and for tasks requiring intermittent use of air. These compressors work by increasing air pressure using pistons and are available in single-stage and two-stage configurations.

- Rotary Screw Compressors: These are suitable for continuous air demand and are commonly found in industrial environments. They operate quieter than reciprocating compressors and provide a steady air flow.

- Centrifugal Compressors: Used for extremely high demand applications, these compressors are often seen in large industrial operations where vast amounts of air are needed. They utilise a different mechanism involving a rotating disk to accelerate and then decelerate air to build up pressure.

2. Practical Examples

Example 1: Small Furniture Workshop

- Compressor Type: Reciprocating Compressor

- Use Case: Ideal for powering staple guns, nail guns, and small spray guns used in furniture upholstery and assembly. The intermittent nature of these tasks makes the reciprocating compressor suitable due to its on-demand air supply and cost-effectiveness.

Example 2: Auto Repair Shop

- Compressor Type: Rotary Screw Compressor

- Use Case: Used for operating impact wrenches, ratchets, and paint sprayers continuously throughout the day. The rotary screw compressor's ability to run continuously without overheating is essential for such high-demand environments.

Example 3: Manufacturing Facility

- Compressor Type: Centrifugal Compressor

- Use Case: Powers large equipment and multiple pneumatic tools simultaneously, such as assembly line robotic tools and heavy-duty lifting equipment. Its high-capacity air output is critical for maintaining efficient operations in a large-scale manufacturing setting.

3. Real-World Application

Site Visits: Organise field trips to facilities that use different types of compressors. This will allow learners to see how these compressors operate in real-world settings and understand the practical considerations of each type.

Industry Speaker Sessions: Invite experts from industries that utilise heavy pneumatic machinery to discuss the importance of choosing the right compressor for specific applications.

4. Evaluation and Feedback

Knowledge Assessment: Develop an assessment that tests learners on their understanding of different types of compressors, their mechanisms, and suitable applications.

Project-Based Learning: Assign learners to a project where they must propose a compressor setup for a hypothetical new business, justifying their choice based on the business’s specific needs and expected air demand.

By engaging with these educational activities, learners will not only understand the differences between various types of compressors but also how to apply this knowledge in practical, real-world situations. This foundational understanding is crucial for making informed decisions in any professional setting that requires the use of pneumatic tools.

### Applied Knowledge Learning Content for AK0902: Compliance with the Occupational Health and Safety Act Regulations for Vessels Under Pressure

Understanding and adhering to regulations under the Occupational Health and Safety Act (OHSA) concerning vessels under pressure is critical for ensuring workplace safety and legal compliance. This module delves into the specifics of these regulations, providing examples of how to apply them in workplaces that use compressors and other pressurised equipment.

1. Overview of OHSA Regulations for Vessels Under Pressure

The OHSA outlines specific requirements for the installation, operation, and maintenance of pressure vessels to prevent accidents and injuries. Key aspects include:

- Regular Inspections: Pressure vessels must be inspected regularly by certified professionals to ensure they are in good working condition and free from defects.

- Certification Requirements: Vessels must have up-to-date certification that proves they have been inspected and meet all safety standards.

- Safety Valve Checks: All pressure vessels must be equipped with properly functioning safety relief valves that are tested regularly to ensure they operate at the correct pressure settings.

- Record Keeping: Accurate records of inspections, maintenance, and any repairs performed on pressure vessels must be maintained and readily available for review.

2. Practical Examples

Example 1: Workshop Compressor

- Scenario: A small automotive repair shop uses a reciprocating compressor.

- Compliance Actions:

- Conducting bi-annual safety inspections by a certified inspector.

- Installing and maintaining a safety relief valve to prevent over-pressurisation.

- Keeping a maintenance log detailing all service dates, issues found, and corrective actions taken.

Example 2: Manufacturing Plant

- Scenario: A large manufacturing facility uses several high-capacity rotary screw compressors.

- Compliance Actions:

- Ensuring all compressors are certified with the latest safety standards and inspection stickers are visible.

- Regular training for all staff on emergency procedures related to compressor failures.

- Implementing a digital tracking system for all pressure vessels to monitor inspection schedules and maintenance records.

3. Real-World Application

Compliance Workshop: Organise a workshop focusing on the importance of complying with OHSA regulations for vessels under pressure. Include demonstrations on how to perform safety checks and maintain records.

Mock Inspection: Conduct a mock inspection of the workplace’s compressors and other pressurised equipment to practice identifying potential safety violations and learning how to correct them.

4. Evaluation and Feedback

Knowledge Quis: Develop a quis that tests learners on their understanding of OHSA regulations, focusing on key points such as inspection intervals, safety features, and record-keeping requirements.

Performance Review: Evaluate how effectively learners can apply their knowledge during the mock inspection, assessing their ability to identify non-compliance issues and suggest appropriate corrective actions.

By engaging with these educational activities, learners will develop a thorough understanding of the legal requirements and best practices for managing pressure vessels safely. This knowledge is essential for preventing workplace injuries and ensuring that operations involving compressed air are conducted within the bounds of regulatory compliance.

### Applied Knowledge Learning Content for AK0903: Standard Safe Operating Procedures (SOPs)

Understanding and implementing Standard Operating Procedures (SOPs) for safe equipment use is crucial to maintaining a safe work environment. This module offers a detailed guide on creating, understanding, and applying SOPs for various types of equipment and workplace scenarios, emphasising the importance of safety and efficiency.

1. Overview of Standard Safe Operating Procedures

SOPs are detailed, written instructions designed to achieve uniformity in the performance of specific functions. In the context of workplace safety, SOPs are crucial for:

- Minimising Risks: SOPs help to minimise the risk of workplace accidents and injuries by providing clear instructions on how to safely operate machinery and handle materials.

- Improving Efficiency: Well-designed SOPs ensure that tasks are completed in a consistent, efficient manner, reducing the likelihood of errors.

- Ensuring Compliance: Adhering to SOPs helps organisations comply with regulatory requirements and internal safety standards.

2. Practical Examples

Example 1: Using a Pneumatic Staple Gun

- SOP Content:

- Check that the staple gun is clean and free from defects before use.

- Verify that the air pressure is correctly set according to the tool’s specifications.

- Always wear safety goggles and ear protection when operating the staple gun.

- Ensure that no one is in the line of fire or within the area where work is being performed.

- Disconnect the staple gun from the air supply before attempting to clear a jam or when not in use.

Example 2: Operating a Rotary Screw Compressor

- SOP Content:

- Conduct a pre-operation inspection including checking oil levels, inspecting hoses for leaks, and verifying that safety valves are functional.

- Monitor pressure gauges regularly during operation and report any irregularities.

- Follow a strict maintenance schedule, recording all service activities.

- Ensure the area around the compressor is clear of debris and any flammable materials.

- Shut down the compressor following the detailed steps outlined in the SOP to avoid wear on the equipment and potential hasards.

3. Real-World Application

SOP Development Activity: Engage learners in developing or updating SOPs for equipment in their training environment. This could include identifying risks associated with the equipment, determining necessary personal protective equipment (PPE), and outlining step-by-step safe operating procedures.

SOP Implementation Drill: Conduct a simulation where learners must follow the SOPs they have developed to complete a task, such as operating a new piece of machinery or handling hasardous materials.

4. Evaluation and Feedback

Rubric for Exercises:

- SOP Comprehensiveness: Measures the thoroughness of the SOPs developed by learners, assessing whether they address all aspects of operation, safety, and compliance.

- Adherence to SOPs: Evaluates how well learners follow SOPs during practical drills, focusing on their ability to apply the procedures under simulated work conditions.

- Problem Identification and Resolution: Assesses learners' ability to identify when SOPs are not sufficient or need adjustment based on practical application, encouraging continuous improvement.

Through engaging with these educational activities, learners will not only understand the importance of SOPs but also gain the skills necessary to create and implement them effectively. This ensures that they are prepared to maintain a safe and efficient workplace, contributing to overall operational excellence and compliance.

### GUIDELINES

You are required to perform the task below and your Facilitator/Assessor will assess your work based on the observation checklist attached.

Practical Assessment Task for PM-02-PS04: Identify raw materials required for cover fitting operations using the correct terminology

Assessment Title:

Purpose

Guidelines to the Learner:

1. You will be presented with a range of samples or photographs of different wooden furniture products and accessories. These will include various styles, types, and uses.  Refer to Knowledge Module KM13 and the additional brochure with examples of upholstered furniture.

2. You are required to examine each sample or photograph and perform the following tasks:

Assessment Criteria:

Assessment Duration:

- The assessment should be completed within a specified time frame, typically 2-3 hours, to simulate a realistic working environment.

Feedback and Evaluation:

- Detailed feedback will be provided on the accuracy and relevance of your categorisations.

- The assessor will highlight areas of strength and suggest improvements where necessary.

Additional Notes:

- This practical assessment is designed to test not only your ability to recognise and categorise different furniture types and accessories but also your understanding of the broader context in which these items are used.

- It is essential to approach this task with attention to detail and a thorough understanding of the varied aspects of upholstered furniture design and manufacturing.

### Case Study: Comprehensive Equipment Review at Apex Upholstery

Scenario:

You are the newly appointed safety and maintenance supervisor at Apex Upholstery, a company that specialises in high-end upholstery services. Your first task is to assess the company's compressor equipment to ensure everything is compliant with safety standards and operating efficiently. This includes identifying the types of compressors used, verifying their maintenance records and legal certifications, and observing the operational procedures for starting, monitoring, and shutting down the equipment.

Scenario-Based Questions:

Question 1: (IAC0901)

Describe how you would identify the types of compressors at Apex Upholstery and record your findings.

Question 2: (IAC0902)

You notice that one of the compressors lacks up-to-date maintenance records and certification. Outline the steps you would take to address this issue.

Question 3: (IAC0903)

Explain how you would oversee the process of starting, monitoring, and shutting down a compressor and air dryer, noting key checkpoints at each stage.

| Criteria | Exemplary (3 points) | Satisfactory (2 points) | Needs Improvement (1 point) |
| --- | --- | --- | --- |
| Identification and Documentation | Accurately identifies and thoroughly documents all types of compressors with detailed records and photographs. | Identifies most compressors correctly, but documentation lacks some details. | Fails to correctly identify some compressors and documentation is incomplete or inaccurate. |
| Maintenance and Certification | Promptly addresses any gaps in maintenance or certification, ensuring all compressors meet legal and safety standards. | Addresses most issues, but some actions are delayed or not fully compliant with regulatory standards. | Neglects to address maintenance or certification issues adequately, risking non-compliance and safety. |
| Operational Procedures | Flawlessly performs start-up, monitoring, and shut-down procedures, ensuring all safety and operational standards are met. | Generally follows procedures correctly but misses some minor safety checks or operational steps. | Inconsistent with procedural compliance, skipping important steps, and failing to monitor equipment properly. |

Through this case study, learners will demonstrate their capability to manage and oversee the operation of compressors in a professional setting, ensuring compliance with safety standards and operational efficiency. This assessment also highlights the importance of meticulous documentation and proactive maintenance management.

EVALUATION CHECKLIST

| NAME: …………………………………….…. SURNAME: ……………………………….…. ID…………………………………………….... | LEARNER SIGNATURE DATE: |
| --- | --- |

| EVALUATION CRITERIONS | MET REQUIREMENTS | DID NOT MEET REQUIREMENTS | COMMENTS OR ACTION REQUIRED |
| --- | --- | --- | --- |
| Ability of the learner to: |  |  |  |

| GENERAL COMMENTS OF OBSERVER: |  |  |
| --- | --- | --- |
| Assessor Name ……………………………… | Signature ……………………………………. | Date …………………………………. |

## PM-02-PS10: Apply measuring techniques and calculations applicable to frame covering

Scope of Practical Skill

Given measuring equipment such as a tape measure, various frames, consumables such as foam, webbing and springs and standard sizing charts the learner must be able to:

- PA1001 Identify and distinguish between the tape measures, various lengths of steel rules, squares, protractor, compass, drawing board and pencils

- PA1002 Use a tape measure and other measuring equipment

- PA1003 Accurately apply and calculate measuring units related to covering of furniture frames

- PA1004 Accurately apply scale

- PA1005 Accurately measure frames and consumables

- PA1006 Measure and mark covers according to the frame (back and / or seat and / or arms)

- PA1007 Measure and mark to centralizing the cover, spacing of buttons and studs, front borders of chairs

- PA1008 Calculate allowances

Applied Knowledge

Internal Assessment Criteria

- IAC1001 Drawing equipment and tools are identified and used to produce basic technical drawings that meet the industry standard

- IAC1002 Scale is accurately applied and allowances are accurately calculated

- IAC1003 Drawings are complete and accurate

### Practical Application Learning Content for PA1001: Identify and Distinguish Between Measuring Tools

Accurate measurement is foundational in upholstery, where precision is key to achieving a professional finish. This module provides detailed guidelines on how to identify and effectively use various measuring tools, including tape measures, steel rules, squares, protractors, compasses, drawing boards, and pencils.

1. Overview of Measuring Tools

- Tape Measures: Flexible and portable, tape measures are ideal for measuring curved and long straight surfaces. They are commonly used in upholstery for measuring fabric and larger furniture frames.

- Steel Rules: Offer more precision than tape measures for shorter lengths. They are rigid and typically used for measuring and marking straight cuts on smaller pieces.

- Squares: Essential for ensuring angles are precise, squares are used to check the squareness of corners and as guides for marking perpendicular lines.

- Protractors: Used to measure and lay out angles, making them crucial for cutting fabric pieces that need to fit together at specific angles.

- Compasses: Useful for drawing circles or arcs, which are often needed when designing or modifying patterns.

- Drawing Boards: Provide a stable, flat surface for laying out large patterns and fabrics, allowing for precise marking and adjustments.

- Pencils: Used for marking measurements on fabrics and other materials, pencils must be sharp to ensure accuracy and visibility of marks.

2. Practical Exercises

Exercise 1: Tool Identification Relay

- Task: Learners participate in a relay where they quickly identify and correctly categorise tools laid out on a table into their respective uses (measuring length, angles, drawing circles, etc.).

- Objective: To reinforce the recognition and purpose of each tool in a timed, dynamic setting.

- Example: Learners run to the table and pick a tool, such as a protractor, and place it in the 'angle measurement' category.

Exercise 2: Measuring and Marking Workshop

- Task: Using a given set of materials (fabric, foam, wood), learners must choose the appropriate tool for various measuring and marking tasks described in a workshop scenario.

- Objective: To apply the correct use of each tool based on the measurement requirements of the task.

- Example: Measuring the length of a sofa frame with a tape measure, marking cut lines on foam padding with a steel rule, and outlining the placement of buttons using a compass.

3. Real-World Application

Tool Matching Challenge: Set up different stations with specific tasks (such as marking angles, measuring fabric for cushions, etc.). Learners must select the correct tool for each station and demonstrate its use effectively.

Interactive Learning Sessions: Organise small group sessions where learners can demonstrate the use of each tool on actual projects, explaining their choice of tool and technique to peers.

4. Evaluation and Feedback

Rubric for Exercises:

- Accuracy in Tool Identification: Assesses the learner's ability to correctly identify tools and their specific uses.

- Precision in Using Tools: Measures how accurately learners use the tools for measuring and marking, focusing on the correctness of application in upholstery tasks.

- Effectiveness in Tool Selection: Evaluates the appropriateness of tool selection for various types of measurements and the rationale behind the choices made.

Through these engaging and practical activities, learners will become proficient in identifying and using a variety of measuring tools, enhancing their skills in precision and efficiency essential for successful upholstery projects.

### Practical Application Learning Content for PA1002: Use a Tape Measure and Other Measuring Equipment

Efficient use of measuring equipment is critical in upholstery, where precision impacts the fit and appearance of the finished product. This module outlines how to correctly utilise a tape measure and other essential measuring tools, providing practical exercises to enhance skill development.

1. Overview of Essential Measuring Tools

- Tape Measure: Flexible and retractable, ideal for measuring fabric, foam, and irregularly shaped objects. Essential for obtaining accurate dimensions of furniture pieces.

- Steel Rule: Offers precision for short, straight measurements. Commonly used for detailed work on smaller components and in spaces where a tape measure's flexibility is a disadvantage.

- Calipers: Useful for measuring the thickness of materials like foam and small components that require high precision.

2. Practical Exercises

Exercise 1: Accurate Fabric Measurement

- Task: Learners measure various pieces of fabric for different furniture components using a tape measure. They must account for pattern repeats and ensure allowances for seams.

- Objective: To practice using a tape measure accurately in a common upholstery scenario.

- Example: Measuring the length and width of fabric needed to cover a sofa cushion, adding extra material for pattern alignment and seam allowances.

Exercise 2: Detailed Component Measurement

- Task: Using a steel rule and calipers, learners measure smaller components like furniture legs or the thickness of decorative trims.

- Objective: To develop precision in measuring smaller, detailed parts of furniture projects.

- Example: Measuring the diameter of sofa legs with calipers for custom slipcovers and using a steel rule to measure the length of trim needed around the base of a chair.

3. Real-World Application

Group Measurement Challenge: Set up a workshop challenge where learners must measure and cut materials for a small upholstery project, such as a footstool. They must use different measuring tools appropriately and explain their choices.

Fabric Calculation Station: Create stations with various types of furniture pieces or patterns. Learners rotate between stations, calculating fabric requirements using tape measures and adding necessary allowances for each specific piece.

4. Evaluation and Feedback

Rubric for Exercises:

- Accuracy of Measurements: Evaluates how close the learners' measurements align with the actual requirements for the upholstery projects.

- Tool Appropriateness: Assesses the learners’ ability to select the most appropriate measuring tool for the task at hand.

- Precision in Execution: Measures the learners' precision in applying measurements to fabric cutting and material preparation.

Through engaging with these educational activities, learners will master the use of a tape measure and other precise measuring tools, building a foundation of accuracy and efficiency crucial for successful upholstery. This knowledge will help ensure that all fabric and material cuts meet the specifications needed for high-quality upholstery work.

### Practical Application Learning Content for PA1003: Accurately Apply and Calculate Measuring Units Related to Covering of Furniture Frames

Mastering the art of measurement and calculation is crucial for ensuring the precise fit and appearance of upholstered furniture. This module provides learners with essential techniques to accurately apply and calculate measuring units for various furniture covering tasks, incorporating practical examples to solidify these skills.

1. Understanding Measuring Units and Their Applications

Measurements in upholstery involve dimensions (length, width, depth) and calculations for material quantities based on these dimensions. Understanding and accurately applying these units are key to minimising waste and ensuring the proper fit of coverings.

- Linear Measurements: Measuring the length, width, and height of furniture frames to determine the amount of fabric or other covering materials needed.

- Area Calculations: Calculating the surface area to be covered to determine how much material is required, taking into account the width of the fabric roll.

- Volume Measurements: Occasionally needed for calculating quantities of filling materials, such as foam, which may be sold by volume.

2. Practical Exercises

Exercise 1: Measuring for Fabric Covering

- Task: Learners measure a chair for a new fabric cover, including all visible surfaces that require fabric.

- Objective: To practice taking precise measurements from a real piece of furniture and calculate the amount of fabric needed.

- Example: Measuring the height and width of a chair back, the width and depth of the seat, and the dimensions of the armrests. Learners then calculate the total area to be covered and convert these measurements into yards or meters of fabric required.

Exercise 2: Calculating Materials for a Complete Sofa Reupholstery

- Task: Learners calculate the amount of different materials (fabric, batting, and foam) needed to completely reupholster a sofa.

- Objective: To apply comprehensive measuring and calculation skills to a larger, more complex furniture piece.

- Example: Determining the surface area of all sofa components that will be covered and then calculating fabric needs based on fabric width. Additionally, calculating the volume of foam needed by measuring the thickness, width, and depth of cushions.

3. Real-World Application

Collaborative Project: In teams, learners select a furniture item to measure, calculate material needs, and present their findings. This project includes planning the cutting layout to minimise waste and explaining their process and decisions.

Material Estimation Workshop: Host a workshop where learners practice estimating material quantities for various furniture types, using different types of materials and discussing cost-effective cutting strategies.

4. Evaluation and Feedback

Rubric for Exercises:

- Measurement Accuracy: Assesses how accurately learners measure furniture dimensions and calculate the required materials.

- Efficiency of Material Use: Evaluates learners' ability to plan and calculate material use to minimise waste while achieving the desired outcome.

- Clarity and Precision in Calculation: Measures the correctness of calculations and the ability to clearly articulate the rationale behind them.

By engaging with these educational activities, learners will develop the precision and analytical skills necessary for successful furniture upholstery. Accurate measurement and calculation are indispensable for ensuring that projects are completed with professionalism and efficiency, reflecting high-quality craftsmanship in the finished products.

### Practical Application Learning Content for PA1004: Accurately Apply Scale

Understanding and accurately applying scale is essential in upholstery, particularly when working from patterns or technical drawings. This module will guide learners through the processes involved in translating dimensions from scaled drawings into real-world measurements for precise fabrication.

1. Overview of Scale Application

Scale application involves converting measurements from drawings or blueprints that are not full sise into actual dimensions used for cutting fabric and other materials. This skill is crucial for ensuring the accuracy of material cuts based on scaled designs.

2. Practical Exercises

Exercise 1: Understanding Scale on Drawings

- Task: Learners are given various upholstery design drawings with different scales (e.g., 1:10, 1:20). They must identify the scale used and convert one dimension (like the length of a sofa arm) into actual measurements.

- Objective: To practice reading scales on drawings and accurately calculating real-world measurements.

- Example: A drawing of a chair shows a scale of 1:10, where the back height is represented as 9 cm on paper. Learners calculate the actual height as 90 cm.

Exercise 2: Creating a Scaled Drawing

- Task: Learners measure an actual piece of furniture and create a scaled drawing using a chosen scale. They must then use this drawing to calculate fabric needs for a hypothetical reupholstery of the piece.

- Objective: To apply skills in both directions—measuring real objects to create scaled drawings and using scaled drawings to calculate material requirements.

- Example: Measuring a dining chair, creating a 1:20 scale drawing, and then calculating the fabric needed for upholstery based on the scaled dimensions.

3. Real-World Application

Project-Based Learning: Assign learners to develop a complete reupholstery plan for a furniture piece, starting with taking physical measurements, creating scaled drawings, and calculating all materials needed based on their drawings.

Interactive Workshop: Host a workshop with professionals who specialise in CAD (Computer-Aided Design) for upholstery, demonstrating how digital tools can assist in scaling and material estimation.

4. Evaluation and Feedback

Rubric for Exercises:

- Accuracy in Scale Conversion: Assesses how precisely learners can convert dimensions from scaled drawings to actual sises and vice versa.

- Proficiency in Drawing and Measurement: Evaluates the learner's ability to create accurate scaled drawings from real-world measurements.

- Application of Knowledge: Measures how effectively learners can use scaled drawings to estimate material needs, considering efficiency and waste minimisation.

Through engaging with these educational activities, learners will master the critical skill of accurately applying scale, which is essential for translating design concepts into tangible outcomes in upholstery. This knowledge ensures that projects are not only aesthetically pleasing but also material-efficient and cost-effective.

### Practical Application Learning Content for PA1005: Accurately Measure Frames and Consumables

Accurate measurement is the cornerstone of successful upholstery projects, ensuring that materials fit correctly and are used efficiently. This module focuses on the techniques required to measure furniture frames and consumables like foam, webbing, and springs accurately.

1. Understanding Frame and Consumable Measurements

- Frame Measurements: Include dimensions such as height, width, depth, and diagonal measurements of furniture frames. These measurements are crucial for determining the amount of fabric and other materials needed.

- Consumable Measurements: Foam, webbing, and springs must be measured to ensure they fit the frame dimensions properly and fulfill the comfort and design requirements.

2. Practical Exercises

Exercise 1: Measuring Furniture Frames

- Task: Learners measure various types of furniture frames, including chairs, sofas, and ottomans, using appropriate tools like tape measures and calipers.

- Objective: To develop proficiency in measuring complex objects and understanding how these measurements translate into material requirements.

- Example: Measuring the width, depth, and height of a sofa, as well as the dimensions of its cushions and armrests to calculate the amount of upholstery fabric required.

Exercise 2: Measuring Consumables for Upholstery

- Task: Learners measure lengths and widths of webbing and the diameter and compression of springs. Foam thickness and density measurements are also included.

- Objective: To ensure learners can accurately gauge the specifications of various upholstery consumables needed for a project.

- Example: Determining the size of foam required for a chair seat by measuring the seat’s dimensions and adding additional allowance for a snug fit. Calculating the length of webbing needed based on the frame’s width and the layout pattern.

3. Real-World Application

Field Measurement Project: Learners visit a local upholstery workshop or a customer’s home to practice measuring furniture on-site. This experience helps them understand the challenges and nuances of taking precise measurements in varied environments.

Material Estimation Simulation: Using their measurements, learners calculate the total materials needed for an upholstery project, including allowances for cuts and seams, and present their findings in a mock project proposal.

4. Evaluation and Feedback

Rubric for Exercises:

- Measurement Accuracy: Evaluates how close the learners' measurements align with the actual dimensions, considering their ability to handle measuring tools correctly.

- Comprehension of Material Needs: Assesses the learners’ ability to calculate material requirements based on frame measurements accurately.

- Effectiveness in Communication: Measures the learners' ability to document and communicate their findings clearly in the context of project planning or client proposals.

Through these activities, learners will not only become adept at measuring furniture frames and consumables but also at applying these measurements to calculate material needs accurately. This skill set is crucial for minimising waste, reducing costs, and ensuring that upholstery projects meet both aesthetic and functional standards.

### Practical Application Learning Content for PA1006: Measure and Mark Covers According to the Frame (Back, Seat, and Arms)

Accurate measurement and marking of upholstery covers according to the specific parts of furniture frames ensure that the final product fits perfectly and looks professional. This module provides detailed techniques and practical exercises to master these skills.

1. Overview of Measuring and Marking Techniques

Effective upholstery requires precise cutting of materials to cover the different areas of furniture—such as the back, seat, and arms—while accommodating curves, corners, and angles.

- Back Covers: These often need to conform to both vertical and horizontal curves, requiring careful measurement and marking to ensure the fabric covers the entire area smoothly without puckering.

- Seat Covers: Generally require additional material for pulling and securing under the frame to handle the stress of regular use.

- Firense Arm Covers: Might include complex shapes, especially with furniture that has scrolled or flared arms, necessitating precise pattern making and cutting.

2. Practical Exercises

Exercise 1: Measuring and Marking for a Chair Back

- Task: Learners measure the height and width of a chair back, considering any curvature. They then transfer these measurements to fabric, adding allowances for seams and stretching.

- Objective: To develop skills in measuring complex shapes and marking fabric accurately.

- Example: Using a flexible tape measure to follow the curve of a wingback chair, marking these dimensions on upholstery fabric with tailor's chalk.

Exercise 2: Cutting and Preparing Seat Covers

- Task: After measuring the dimensions of a seat, including depth and any additional padding, learners mark and cut the fabric, adding extra material for securing the fabric under the seat.

- Objective: To ensure that learners can prepare materials that fit securely and neatly, accommodating functional needs.

- Example: Calculating the dimensions for a sofa seat cover that wraps under for stapling, ensuring enough excess for tension adjustments.

Exercise 3: Pattern Creation for Arm Covers

- Task: Learners create a pattern for an arm cover, taking into account any stylistic details such as pleats or buttons.

- Objective: To master the art of pattern making for irregular shapes and details.

- Example: Drafting a paper pattern for a rolled arm on a classic armchair, factoring in allowances for seams and the complex curve of the arm.

3. Real-World Application

Group Upholstery Project: Learners apply their skills in a real-world scenario by reupholstering a piece of furniture in teams. They are responsible for measuring, marking, and fitting the fabric for different sections of the furniture.

Interactive Workshop: Conduct a workshop with a professional upholsterer demonstrating advanced techniques in measuring, marking, and fitting upholstery fabrics, especially for challenging furniture designs.

4. Evaluation and Feedback

Rubric for Exercises:

- Accuracy of Measurements and Markings: Evaluates how precise and appropriate the learners' measurements and markings are for the intended upholstery task.

- Quality of Final Fit: Assesses the accuracy of fit in the upholstered piece, particularly looking at how well the fabric aligns and conforms to the furniture contours without unnecessary bulging or sagging.

- Creativity and Problem-Solving: Measures the learner’s ability to handle complex shapes and designs, evaluating their innovative approaches to challenges encountered during the marking and cutting phases.

Through these educational activities, learners will become proficient in the essential skills of measuring and marking upholstery covers, ensuring they can achieve a professional finish in their upholstery projects. These skills are fundamental for delivering high-quality upholstery work that meets both aesthetic and functional standards.

### Practical Application Learning Content for PA1007

Measure and Mark for Centralising the Cover, Spacing of Buttons and Studs, Front Borders of Chairs

Ensuring the precise placement of decorative and functional elements like buttons, studs, and borders is crucial in upholstery. This module provides learners with detailed techniques for accurately measuring and marking these elements, ensuring symmetry and aesthetic appeal in finished furniture pieces.

1. Overview of Decorative Placement Techniques

- Centralising the Cover: Ensures the fabric pattern is symmetrically aligned on the furniture piece, particularly important for patterned fabrics.

- Spacing of Buttons and Studs: Critical for achieving a visually appealing tufted look or decorative detail. Consistent spacing is key to professional upholstery work.

- Measuring for Front Borders: Involves accurate measurement to ensure that borders are straight and proportional, enhancing the finished look of the piece.

2. Practical Exercises

Exercise 1: Centralising the Fabric Cover

- Task: Learners measure and mark the centre point of both the fabric and the furniture piece (like a chair back), ensuring the pattern is centred before attachment.

- Objective: To practice aligning patterns and designs to the middle of a piece, considering both aesthetics and material placement.

- Example: Using a tape measure to find the centre of a floral-patterned fabric and the corresponding centre on a chair back, marking both with tailor's chalk before draping the fabric over the chair to ensure alignment.

Exercise 2: Spacing Buttons for a Tufted Headboard

- Task: Learners calculate and mark the placement for buttons on a tufted headboard, using a ruler or spacing tape.

- Objective: To develop skills in evenly spacing decorative elements according to a specified pattern.

- Example: Determining a diamond tufting pattern, measuring and marking the points for button placement to ensure even spacing and symmetry using a template or homemade jig.

Exercise 3: Marking Front Borders on a Sofa

- Task: Measure and mark the front border of a sofa, ensuring it runs evenly and at the correct height relative to the ground.

- Objective: To practice measuring and marking for horisontal elements that require precise alignment.

- Example: Using a laser level and measuring tape to mark a line where the front border fabric will be attached, ensuring it is parallel to the base of the sofa.

3. Real-World Application

Upholstery Design Challenge: Learners design and execute a small upholstery project (such as an ottoman or side chair) that incorporates centralising patterns, button tufting, and precise borders. They must demonstrate their measuring and marking techniques throughout the project.

Workshop Collaboration: Partner with a local furniture shop or a professional upholsterer for a day-long workshop, where learners can observe and practice advanced measuring and marking techniques on a variety of furniture styles and configurations.

4. Evaluation and Feedback

Rubric for Exercises:

- Precision of Measurement and Marking: Assesses the accuracy of learners' measurements and markings, ensuring they align with design specifications.

- Quality of Aesthetic Execution: Evaluates the aesthetic alignment and appeal of the fabric, buttons, and borders on the furniture.

- Innovativeness and Problem-Solving: Measures the learner's ability to adapt techniques for unique furniture shapes and styles, and their effectiveness in overcoming measuring and marking challenges.

Through these activities, learners will gain a thorough understanding of how precise measurement and careful marking contribute to the high-quality craftsmanship in upholstery. These skills are essential for ensuring that decorative elements enhance rather than detract from the overall design and functionality of upholstered furniture.

### Practical Application Learning Content for PA1008: Calculate Allowances

Accurate calculation of allowances is crucial in upholstery to ensure that fabrics and other materials are cut and fitted correctly, allowing for a professional finish. This module covers the necessary skills to calculate seam allowances, stretching allowances, and other necessary additions to basic measurements.

1. Understanding Different Types of Allowances

- Seam Allowances: Additional fabric added to the measurements to allow for the material that will be sewn together at the seams. This ensures the cover fits snugly without pulling or puckering.

- Stretching Allowances: Extra material to accommodate the stretch of the fabric when it is pulled over cushions and frames, ensuring it fits tightly and retains its shape over time.

- Hemming and Edge Finishes: Material added to the edges of fabric for finishing touches such as hems or piping, which contribute to the durability and aesthetic of the finished product.

2. Practical Exercises

Exercise 1: Calculating Seam Allowances

- Task: Learners measure and calculate seam allowances for different furniture pieces, such as a square cushion, an armchair, and a curved sofa back.

- Objective: To practice adding appropriate seam allowances based on the thickness of the fabric and the type of seam required.

- Example: Adding a 1-inch seam allowance on all sides of upholstery fabric intended for a chair cushion, accounting for the fabric's thickness and the tightness of the fit desired.

Exercise 2: Applying Stretching Allowances

- Task: Learners calculate the stretching allowance needed for a piece of elastic material to cover a round ottoman.

- Objective: To understand how to adjust measurements for the elasticity of the material to prevent sagging and ensure a tight fit.

- Example: Measuring the circumference of the ottoman and adding a 10% stretching allowance to the fabric length to ensure it grips the frame securely when fitted.

Exercise 3: Hemming and Finishing Edges

- Task: Learners calculate additional material required for hemming the bottom of a drapery panel used as part of a furniture set.

- Objective: To ensure learners can provide enough material for proper finishing techniques that enhance the appearance and longevity of the product.

- Example: Adding 3 inches to the length of a drapery fabric to allow for a 1.5-inch double-fold hem at the bottom.

3. Real-World Application

Material Estimation Workshop: Conduct a hands-on workshop where learners estimate and calculate required materials for an actual client project from start to finish, including all allowances for seams, stretching, and hems.

Collaborative Project: Learners team up to reupholster a set of dining room chairs, calculating all necessary allowances and executing the project to assess how well their calculations translate into practice.

4. Evaluation and Feedback

Rubric for Exercises:

- Accuracy of Calculations: Measures how accurately learners perform calculations for allowances, impacting the fit and finish of the upholstery.

- Application of Allowances: Assesses how well learners apply their calculations in practical upholstery tasks.

- Quality of Finished Product: Evaluates the appearance and functionality of the final product, particularly how well the allowances contributed to a professional finish.

By mastering these calculation skills, learners will be able to ensure that all materials used in their upholstery projects are measured and cut precisely, accounting for all necessary allowances. This skill set is essential for achieving high-quality results that meet both functional and aesthetic standards.

### GUIDELINES

You are required to perform the task below and your Facilitator/Assessor will assess your work based on the observation checklist attached.

Practical Assessment Task for PM-02-PS10: Apply measuring techniques and calculations applicable to frame covering

Assessment Title:

Purpose

Guidelines to the Learner:

1. You will be presented with a range of samples or photographs of different wooden furniture products and accessories. These will include various styles, types, and uses.  Refer to Knowledge Module KM13 and the additional brochure with examples of upholstered furniture.

2. You are required to examine each sample or photograph and perform the following tasks:

Assessment Criteria:

Assessment Duration:

- The assessment should be completed within a specified time frame, typically 2-3 hours, to simulate a realistic working environment.

Feedback and Evaluation:

- Detailed feedback will be provided on the accuracy and relevance of your categorisations.

- The assessor will highlight areas of strength and suggest improvements where necessary.

Additional Notes:

- This practical assessment is designed to test not only your ability to recognise and categorise different furniture types and accessories but also your understanding of the broader context in which these items are used.

- It is essential to approach this task with attention to detail and a thorough understanding of the varied aspects of upholstered furniture design and manufacturing.

### Case Study: Technical Drawing Development for Elite Design Upholstery

Scenario:

You are an apprentice upholsterer at Elite Design Upholstery, tasked with creating a series of technical drawings for a new line of custom furniture that the company plans to launch. This range includes a sofa, an armchair, and a set of dining chairs. Your drawings will be used to guide the production team and must be detailed, accurate, and conform to industry standards.

Scenario-Based Questions:

Question 1: (IAC1001)

How would you identify and use the correct drawing equipment and tools to produce basic technical drawings for the new furniture line?

Question 2: (IAC1002)

Explain how you would apply scale and calculate allowances in your technical drawings.

Question 3: (IAC1003)

Describe the steps you would take to ensure that your drawings are complete and accurate.

Marking Memo:

| Criteria | Exemplary (3 points) | Satisfactory (2 points) | Needs Improvement (1 point) |
| --- | --- | --- | --- |
| Tool and Equipment Use | Correctly identifies and skilfully uses all required tools to produce clear and detailed drawings. | Adequately uses tools, but some aspects of the drawings lack clarity or precision. | Ineffectively uses tools, resulting in unclear or inaccurate drawings. |
| Accuracy of Scale and Allowances | Perfectly applies scale and calculates allowances, ensuring all dimensions are proportional and adequately adjusted for production. | Generally applies scale correctly; minor errors in allowance calculations do not significantly impact the final product. | Fails to apply scale correctly or calculate allowances accurately, potentially causing significant issues in production. |
| Completeness and Accuracy of Drawings | Drawings are exceptionally detailed and accurate, with all necessary annotations and specifications clearly noted. | Drawings are complete but could benefit from greater detail or clarity in some annotations. | Drawings are incomplete or contain significant inaccuracies that could mislead the production team. |

Through this case study, learners are assessed on their ability to create detailed and accurate technical drawings that will directly influence the manufacturing process. Mastery of these skills is crucial for ensuring that the final products are built to specifications and meet the high standards expected in custom furniture design.

EVALUATION CHECKLIST

| NAME: …………………………………….…. SURNAME: ……………………………….…. ID…………………………………………….... | LEARNER SIGNATURE DATE: |
| --- | --- |

| EVALUATION CRITERIONS | MET REQUIREMENTS | DID NOT MEET REQUIREMENTS | COMMENTS OR ACTION REQUIRED |
| --- | --- | --- | --- |
| Ability of the learner to: |  |  |  |

| GENERAL COMMENTS OF OBSERVER: |  |  |
| --- | --- | --- |
| Assessor Name ……………………………… | Signature ……………………………………. | Date …………………………………. |

## PM-02-PS11: Plan and prepare for covering of prepared frames

Scope of Practical Skill

In the precision-driven field of upholstery, the initial steps of planning and preparation are as crucial as the actual covering of the furniture frames. PM-02-PS11 is designed to equip learners with the necessary skills to meticulously plan and prepare for the upholstery process, ensuring that each phase of production is executed with accuracy and efficiency. This module provides comprehensive training in interpreting various types of production documentation and effectively setting up for the upholstery of furniture frames.

Learning Objectives

Through detailed instruction and hands-on practice, learners will:

- PA1101: Read and Interpret Production Documentation

- Learners will develop the ability to thoroughly understand and apply information from product specifications, samples, work instructions, technical drawings, and routing sheets. This skill is vital for ensuring that all aspects of the upholstery process meet the required standards and specifications.

- PA1102: Develop a Detailed Work Plan and Cutting List

- From the information interpreted from the specifications, learners will create a detailed work plan that outlines each step of the upholstery process, accompanied by a precise cutting list for materials like fabric, webbing, and springs.

- PA1103: Measure and List Requirements for Internal Upholstery Components

- Practical exercises will involve measuring the inside seat and frame dimensions to prepare a detailed cutting list for internal components such as webbing and springs, crucial for the furniture’s structural integrity and comfort.

- PA1104: Ensure Sufficient Availability of Raw Materials

- Learners will learn to assess and manage the inventory of raw materials, ensuring there is an adequate supply to complete projects efficiently without interruption.

- PA1105: Prepare the Work Area for Operations

- This objective focuses on organising and setting up the work area to optimise the workflow and maintain safety standards, including the arrangement of tools, materials, and equipment.

- PA1106: Comprehension of Production Cards

- Learners will practice reading and understanding production cards that contain essential information about the upholstery tasks, enabling them to follow through with the specific requirements of each project accurately.

- PA1107: Set Up Upholstery Machinery

- Instruction will be provided on how to correctly set up various machines used in the upholstery process, tailored to the specific needs of the project.

- PA1108: Conduct Pre-start-up Checks

- Before beginning any upholstery work, learners will conduct pre-start checks on equipment to ensure everything is in optimal working condition, a crucial step for maintaining both safety and productivity.

Practical Implementation

This module will integrate classroom learning with practical application in a workshop setting. Learners will engage with real-life scenarios that involve planning, measuring, and preparing for upholstery projects. This approach ensures that learners not only understand the theoretical aspects of these preparatory tasks but also gain the practical skills necessary to implement them effectively.

By mastering these planning and preparation processes, learners will ensure that upholstery projects are not only completed to high standards but also executed efficiently, reflecting professionalism and skill in their craft.

### Practical Application Learning Content for PA1101

Read and Interpret Product Specifications, Samples, Work Instructions, Drawings, Routing Sheets, and Other Production Documentation

Accurate interpretation of production documentation is essential in upholstery, as it directs the entire production process. This module will guide learners through the intricacies of understanding and applying information from various documents that dictate the construction and finishing of upholstered furniture.

1. Understanding Different Types of Production Documentation

- Product Specifications: Detailed descriptions of the furniture piece, including dimensions, materials, and special requirements.

- Samples: Physical or visual examples that showcase materials, finishes, or construction methods.

- Work Instructions: Step-by-step guides on how to perform specific tasks or processes in the production line.

- Technical Drawings: Scaled drawings showing different views and sections of the furniture, often detailing complex components.

- Routing Sheets: Documents outlining the path through which the product moves during manufacturing, specifying operations, equipment, and personnel involved at each stage.

2. Practical Exercises

Exercise 1: Document Comprehension and Application

- Task: Learners are provided with a set of documents for a new upholstery project, including product specifications, a sample fabric swatch, and technical drawings. They must extract critical data from these documents to summarise the project requirements.

- Objective: To develop skills in analysing and synthesising information from multiple sources into a coherent project overview.

- Example: Identifying the type of fabric, cushion density specifications from the product specs, understanding the pattern layout from the technical drawing, and summarising the sequence of assembly steps from the work instructions.

Exercise 2: Interpreting Technical Drawings

- Task: Using technical drawings of a complex chair design, learners must list all dimensions, identify materials for different parts, and note any special construction techniques highlighted.

- Objective: To enhance skills in reading and applying data from technical drawings, which are critical for accurate production.

- Example: Deciphering a cross-sectional view of a chair arm to determine the thickness of foam needed and the type of support structure within.

3. Real-World Application

Mock Production Meeting: Conduct a simulation where learners must present the project requirements based on their interpretation of the provided documents. This exercise mimics real-world scenarios where upholsterers must communicate project details accurately to team members.

Documentation Review Sessions: Regular sessions where learners practice critiquing and correcting a set of documents to ensure they meet industry standards and are clear enough for all production staff to follow.

4. Evaluation and Feedback

Rubric for Exercises:

- Comprehension Accuracy: Measures how well learners understand and interpret the details contained in production documents.

- Application of Information: Evaluates the ability of learners to apply the interpreted information in planning and initiating production tasks.

- Communication Skills: Assesses how effectively learners can communicate the requirements and special instructions to peers and supervisors, a key skill in ensuring smooth production workflows.

Through these educational activities, learners will gain essential skills in interpreting complex production documents, ensuring they are well-prepared to handle the demands of professional upholstery projects. This foundational knowledge is crucial for ensuring that all aspects of production are executed accurately, efficiently, and to the highest standards.

### Practical Application Learning Content for PA1102: Interpret the Information in the Specifications into a Detailed Work Plan and Cutting List

Successfully transforming specifications into actionable work plans and precise cutting lists is vital for efficient upholstery production. This module provides learners with the skills to interpret detailed specifications and translate them into comprehensive work plans that guide the upholstery process from start to finish.

1. Understanding Specifications and Work Plan Development

- Specifications Review: Includes a thorough analysis of dimensions, material types, design preferences, and any special instructions outlined in the product specifications.

- Work Plan Creation: Organising the sequence of tasks, assigning responsibilities, estimating timeframes, and determining the workflow to optimise production efficiency.

- Cutting List Preparation: Calculating the amount of fabric and other materials needed based on the furniture dimensions and incorporating allowances for seams, pattern repeats, and waste minimisation.

2. Practical Exercises

Exercise 1: Creating a Work Plan from Specifications

- Task: Learners are given a set of specifications for a custom sofa that includes fabric type, sofa dimensions, and design features like tufting. They must create a detailed work plan that outlines all necessary steps for the project.

- Objective: To practice organising and scheduling upholstery tasks effectively, ensuring all aspects of the specifications are addressed.

- Example: Developing a timeline that includes steps such as frame inspection, material procurement, cutting, sewing, and assembly. Assigning team members to specific tasks like sewing cushion covers or installing springs.

Exercise 2: Developing a Cutting List

- Task: Based on the same sofa specifications, learners must calculate fabric requirements and prepare a cutting list that includes all parts of the sofa: cushions, arms, back, and base.

- Objective: To enhance precision in material estimation and waste reduction.

- Example: Measuring dimensions for each sofa part and calculating the total fabric needed, considering fabric width and pattern matching, especially if the fabric has a distinct design that needs alignment.

3. Real-World Application

Group Project: Learners collaborate on a mock upholstery project where they apply their work plans and cutting lists to create a piece of furniture. This project allows them to put their planning and organisational skills to the test in a controlled, real-world environment.

Peer Review Sessions: Regular sessions where learners present their work plans and cutting lists to peers for feedback. These sessions help improve their ability to critique and refine upholstery plans based on collective input.

4. Evaluation and Feedback

Rubric for Exercises:

- Accuracy in Plan Creation: Assesses how well the work plan addresses all aspects of the specifications and organises tasks in a logical and efficient order.

- Precision in Cutting List Preparation: Evaluates the accuracy of material calculations and the effectiveness of the cutting list in minimising waste while meeting design requirements.

- Implementation and Adaptability: Measures the learner's ability to adapt the plan and cutting list based on feedback and during the actual project execution, highlighting flexibility and problem-solving skills.

Through these activities, learners will not only become adept at creating detailed work plans and cutting lists but also learn to think critically about the organisation and execution of upholstery projects. This skill set is crucial for ensuring that projects are completed efficiently, within budget, and to the client's satisfaction.

### Practical Application Learning Content for PA1103

Measure the Inside Seat Dimensions and Inside Frame Dimensions Top to Bottom and Draw Up a Cutting List for Webbing and Springs

Accurately measuring the dimensions of furniture frames and effectively planning for the required materials such as webbing and springs are critical steps in the upholstery process. This module focuses on the necessary skills to measure internal components and create detailed cutting lists that ensure all materials are correctly sized and accounted for.

1. Overview of Measuring Techniques for Upholstery

- Inside Seat Dimensions: These include the width, depth, and height of the seat area, which are crucial for determining the amount of webbing and the number of springs needed.

- Inside Frame Dimensions: Measurement of the overall interior frame sise, including the top to the bottom depth, to ensure adequate material coverage and support structure integrity.

2. Practical Exercises

Exercise 1: Measuring Interior Dimensions of Furniture

- Task: Learners measure the inside dimensions of different types of furniture, such as a sofa, an armchair, and a dining chair.

- Objective: To accurately capture the necessary dimensions that will inform the cutting list for webbing and springs.

- Example: Using a tape measure to determine the depth and width of a sofa’s seat, and the distance from the top of the seat frame to the bottom to gauge spring length and tension requirements.

Exercise 2: Creating a Cutting List for Webbing and Springs

- Task: Based on the measurements taken, learners will calculate the required lengths and quantities of webbing and the number of springs, then compile these into a cutting list.

- Objective: To apply measurements in a practical scenario, ensuring that all materials are adequately planned for upholstery reconstruction.

- Example: Determining the number of springs based on seat depth and preferred tension, and calculating the total length of webbing needed by considering the width of the frame and the webbing layout pattern (usually a grid pattern).

3. Real-World Application

Material Estimation Project: Learners take on a project where they must measure, calculate, and order materials for an upcoming furniture restoration. This includes sourcing materials based on their cutting list and comparing different suppliers for the best material quality and prices.

Workshop Practice Session: Organise a hands-on session where learners practice measuring and cutting webbing and springs in the workshop. This session allows them to apply their theoretical knowledge and receive immediate feedback on their technique and accuracy.

4. Evaluation and Feedback

Rubric for Exercises:

- Accuracy of Measurements: Evaluates the precision of the learners’ measurements and their ability to capture all relevant dimensions correctly.

- Effectiveness of Material Calculations: Assesses the accuracy of calculations for material quantities, focusing on the learner’s ability to minimise waste while fulfilling structural needs.

- Quality of Cutting List: Measures how comprehensive and clear the cutting list is, including whether it effectively communicates all necessary information for material preparation and use.

Through these exercises, learners will develop essential skills in both measuring and material estimation, crucial for ensuring that upholstery projects are completed with precision and efficiency. These skills are fundamental for anyone looking to excel in the upholstery industry, where accurate material preparation can significantly impact the quality and durability of the finished product.

### Practical Application Learning Content for PA1104: Ensure Availability of Sufficient Raw Materials to Ensure Efficiency

Effective management of raw materials is crucial in upholstery to maintain production flow and minimise delays. This module provides learners with the strategies and skills needed to ensure that all necessary materials are available in the right quantities at the right time.

1. Overview of Raw Material Management

- Inventory Assessment: Regular checks to track the quantity and condition of materials in stock, such as fabrics, foam, webbing, and springs.

- Supply Chain Coordination: Understanding the lead times for ordering and receiving different materials and coordinating with suppliers to ensure timely delivery.

- Waste Minimisation: Techniques for calculating precise material needs to reduce waste and optimise resource use.

2. Practical Exercises

Exercise 1: Inventory Management

- Task: Learners perform an inventory audit of upholstery materials in a workshop setting. They must identify current stock levels and compare these against upcoming project requirements.

- Objective: To develop skills in managing inventory, identifying shortfalls or surpluses, and understanding the implications of inventory levels on production efficiency.

- Example: Assessing the stock of high-density foam and realising it is insufficient for the scheduled production of a series of dining chairs, prompting an urgent reorder.

Exercise 2: Supplier Coordination and Order Placement

- Task: Based on the inventory check, learners must place orders with suppliers, taking into account lead times and project deadlines.

- Objective: To practice coordinating with suppliers to ensure that materials arrive on time and meet quality standards.

- Example: Contacting multiple fabric suppliers to find the best match for a specific upholstery fabric that meets both the aesthetic and durability requirements of a client’s custom sofa project.

3. Real-World Application

Mock Procurement Scenario: Learners engage in a simulation where they are responsible for the entire material procurement process for a month’s worth of upholstery projects. This includes forecasting, ordering, and managing deliveries.

Collaborative Project with Local Suppliers: Arrange for learners to work directly with local suppliers on a small project. This interaction helps them understand real-world challenges and solutions in supply chain management within the upholstery industry.

4. Evaluation and Feedback

Rubric for Exercises:

- Inventory Management Accuracy: Assesses how accurately learners identify and document inventory needs versus available stock.

- Efficiency in Supplier Coordination: Evaluates learners' effectiveness in managing relationships with suppliers and ensuring timely delivery of quality materials.

- Problem-solving and Adaptability: Measures the learners’ ability to handle unexpected shortages or delays in material supply and their effectiveness in implementing alternative plans.

Through these activities, learners will gain critical skills in raw material management, crucial for running a smooth and efficient upholstery operation. Proper management ensures that projects are not delayed due to material shortages, and helps maintain cost efficiency by minimising waste and optimising material usage.

### Practical Application Learning Content for PA1105: Prepare the Work Area for Frame Preparation Operations

Properly preparing the work area is essential for ensuring safety, efficiency, and productivity in upholstery. This module focuses on the methods and best practices for organising and setting up an upholstery workspace, tailored to frame preparation tasks.

1. Overview of Work Area Preparation

- Workspace Layout: Designing the workspace to maximise efficiency and reduce unnecessary movement. Tools, materials, and equipment should be strategically placed to optimise the workflow.

- Safety Protocols: Implementing safety measures, including clear walkways, properly stored tools, and adequate ventilation for tasks that involve fumes or dust.

- Tool and Equipment Organisation: Ensuring that all necessary tools and equipment are cleaned, maintained, and correctly set up before starting the work.

2. Practical Exercises

Exercise 1: Designing an Efficient Workspace

- Task: Learners are given a diagram of an empty workshop space and a list of all tools, materials, and pieces of equipment they will need for frame preparation. They must design a layout that optimises the flow of work.

- Objective: To apply principles of workflow efficiency and safety in organising a work area.

- Example: Placing the cutting table near the storage area for upholstery fabrics to minimise carrying distances; setting up the sewing and stapling stations next to each other for quick transitions between tasks.

Exercise 2: Implementing Safety and Maintenance Checks

- Task: Learners perform a series of safety checks and prepare their tools and equipment for operation, following a checklist provided.

- Objective: To ensure all aspects of the work area are safe and all equipment is functional before beginning any upholstery tasks.

- Example: Checking that all electrical cords are intact and not a tripping hasard, ensuring all sharp tools are stored securely when not in use, and verifying that ventilation systems are operational.

3. Real-World Application

Work Area Setup Competition: Organise a timed competition where learners are judged on how quickly and effectively they can set up their work area according to best practices for both safety and efficiency.

Peer Review Sessions: After setting up their stations, learners conduct peer reviews of each other’s setups, offering feedback and discussing ways to improve space utilisation and safety.

4. Feedback

Rubric for Exercises:

- Workspace Layout Efficiency: Assesses how well the learner utilises space, considering the flow of materials and ease of movement.

- Safety Adherence: Evaluates the implementation of safety measures, checking for any potential hazards overlooked by the learner.

- Tool and Equipment Preparedness: Measures the readiness of tools and equipment for use, including proper maintenance and organisation.

Through these activities, learners will develop the ability to prepare and maintain an organised and safe upholstery workspace. This preparation is crucial for facilitating smooth operations, preventing accidents, and ensuring high productivity in upholstery projects.

### Practical Application Learning Content for PA1106: Read and Understand Production Cards

Effectively interpreting production cards is essential in the upholstery process, as these cards contain crucial information about tasks, materials, timelines, and quality standards. This module provides learners with the tools to understand and utilise production cards effectively, ensuring adherence to production schedules and specifications.

1. Overview of Production Cards

- Content of Production Cards: Typically, these cards include information such as product identification, material requirements, assembly instructions, and checkpoints for quality control.

- Importance of Production Cards: They guide the production process, ensuring that all team members are aware of their roles and responsibilities and that the workflow proceeds smoothly and efficiently.

2. Practical Exercises

Exercise 1: Interpreting Production Cards

- Task: Learners are given several production cards for different pieces of furniture within a simulated upholstery project. They must read and extract critical information from each card, summarising the key tasks and materials listed.

- Objective: To practice the accurate interpretation and application of information provided on production cards.

- Example: Identifying the type of fabric, the amount required for a particular sofa, specific assembly instructions for the sofa's frame, and the quality checks that must be performed before the final product is approved.

Exercise 2: Role-Based Scenario Simulation

- Task: Using production cards, learners assume different roles within the upholstery process (e.g., cutter, sewer, assembler). Each learner follows the instructions relevant to their role as outlined on their assigned production card.

- Objective: To demonstrate how effectively they can apply the instructions from the production cards in a collaborative, real-world scenario.

- Example: One learner, acting as the cutter, interprets the fabric cutting specifications from the card, while another, as the assembler, follows the construction guidelines to assemble the cut pieces.

3. Real-World Application

Mock Production Line Setup: Organise a mock production line where learners must complete a furniture piece by following a series of production cards from start to finish. This hands-on experience helps solidify the importance of each step in the manufacturing process.

Production Card Creation Workshop: Conduct a workshop where learners create their own production cards for a hypothetical new product. This activity includes researching the necessary components and steps, writing clear instructions, and designing a layout for the cards.

4. Evaluation and Feedback

Rubric for Exercises:

- Comprehension of Production Cards: Assesses the learner's ability to correctly understand and summarise the information found on the production cards.

- Application in Practical Scenarios: Evaluates the accuracy and efficiency with which learners apply the card instructions in simulation exercises.

- Collaboration and Communication: Measures how effectively learners communicate and collaborate when following a shared set of instructions, reflecting their ability to work as part of a team.

Through these activities, learners will become proficient in reading and applying the information provided on production cards, a skill essential for maintaining standards and efficiency in the production environment. This capability ensures that each stage of the upholstery process is performed accurately, adhering to the project's specifications and timelines.

### Practical Application Learning Content for PA1107: Set the Machine

Setting up upholstery machinery correctly is crucial for ensuring efficient, high-quality production while minimising the risk of errors or equipment damage. This module will guide learners through the process of preparing various types of upholstery machinery for operation, including sewing machines, staplers, and other specialised equipment used in the upholstery process.

1. Overview of Upholstery Machinery Setup

- Types of Machinery: Common upholstery machines include sewing machines (both industrial and heavy-duty models), automated cutting machines, and pneumatic staplers.

- Machine Setup Requirements: Each type of machine has specific setup requirements that may involve thread loading, tension adjustment, tool attachment, and setting operation speeds.

2. Practical Exercises

Exercise 1: Setting Up a Sewing Machine

- Task: Learners are provided with an industrial sewing machine and must correctly thread the machine, adjust tension settings, and prepare it for sewing different types of upholstery fabrics.

- Objective: To ensure learners can properly set up a sewing machine to handle various upholstery tasks without causing fabric damage or machine malfunctions.

- Example: Adjusting the machine to sew through heavyweight leather, which involves setting the correct needle size, thread type, and tension to avoid breaking the needle or skipping stitches.

Exercise 2: Preparing a Pneumatic Stapler

- Task: Learners must load staples into a pneumatic stapler, connect it to the air supply, and adjust the pressure settings according to the thickness of the material being used.

- Objective: To master the setup of a pneumatic stapler for secure and efficient fastening of upholstery materials.

- Example: Setting the stapler to the appropriate air pressure to attach fabric to a hardwood frame, ensuring that staples are neither too shallow (not holding the fabric securely) nor too deep (damaging the fabric and frame).

3. Real-World Application

Machine Maintenance and Setup Workshop: Organise a workshop where learners practice setting up different machines under the guidance of experienced technicians. This workshop could also cover routine maintenance tasks to keep the machines in optimal working condition.

Time Trial Challenges: Conduct timed challenges where learners compete to see who can set up their assigned machinery the fastest while still adhering to all operational guidelines. This activity promotes not only speed but also accuracy in machine setup.

4. Evaluation and Feedback

Rubric for Exercises:

- Accuracy of Machine Setup: Measures how precisely learners follow the setup instructions and manufacturer guidelines to prepare the machinery.

- Efficiency in Setup Process: Evaluates how quickly and effectively learners can get the machine ready for operation without compromising on setup quality.

- Problem-solving during Setup: Assesses the learner’s ability to troubleshoot setup issues, such as improper tension or incorrect loading of materials.

Through these educational activities, learners will gain the necessary skills to confidently and correctly set up various types of upholstery machinery, ensuring they are well-prepared for any upholstery task. This capability is essential for maintaining productivity and achieving high-quality results in the upholstery industry.

### Practical Application Learning Content for PA1108: Conduct Pre-Start-Up Checks (Operator Will Sign)

Conducting thorough pre-start-up checks is crucial for ensuring the safety and functionality of upholstery machinery before beginning any work. This module provides learners with a detailed guide to perform these checks systematically, with an emphasis on maintaining safety standards and preventing equipment failure.

1. Overview of Pre-Start-Up Checks

Pre-start-up checks involve a series of inspections and tests to verify that machinery is in proper working order and safe to use. These checks are vital for identifying potential issues that could lead to machine malfunctions or safety hasards during operation.

2. Practical Exercises

Exercise 1: Sewing Machine Safety and Functionality Checks

- Task: Learners perform a comprehensive pre-start-up check on an industrial sewing machine.

- Objective: To ensure the sewing machine is set up correctly and all safety features are functioning before use.

- Example:

- Visual Inspection: Check for any loose components, wear on the belts or threads, and ensure that safety guards are in place.

- Functional Tests: Verify that the foot pedal and light work correctly, test the needle alignment, and ensure the thread tension is adjusted properly.

- Safety Sign-Off: Each learner signs off on a checklist that details all the checks performed, confirming the machine’s readiness.

Exercise 2: Pneumatic Stapler Operational Checks

- Task: Learners prepare a pneumatic stapler for use by performing standard operational checks.

- Objective: To validate that the stapler is correctly assembled, connected to the air supply securely, and set to the correct pressure.

- Example:

- Connection Check: Ensure that the air hose is securely attached and free from leaks.

- Pressure Adjustment: Adjust the air pressure according to the manufacturer's recommendations for the specific material and task.

- Test Firing: Perform a test fire into a scrap piece of wood to confirm proper staple deployment.

- Documentation and Sign-Off: Document the checks and sign off on their completion to ensure accountability and traceability.

3. Real-World Application

Routine Safety Audit: Implement regular safety audits where learners must inspect various machines using the pre-start-up checklists and report any issues to the workshop supervisor. This routine helps embed safety as a priority in daily operations.

Peer Review Sessions: Arrange for learners to pair up and conduct pre-start-up checks on each other’s machines. Peer reviews help reinforce the importance of thorough checks and provide a secondary layer of oversight.

4. Evaluation and Feedback

Rubric for Exercises:

- Thoroughness of Checks: Assesses how detailed and meticulous learners are in performing the pre-start-up checks.

- Accuracy of Functional Testing: Evaluates the correctness of settings and functional elements as tested during the checks.

- Compliance with Safety Standards: Measures adherence to safety protocols and proper use of sign-off procedures to ensure accountability.

Through these educational activities, learners will develop a disciplined approach to ensuring equipment safety and functionality. Mastery of these pre-start-up checks is essential for operating upholstery machinery safely and efficiently, thereby reducing the risk of accidents and enhancing overall productivity in the workshop.

### GUIDELINES

You are required to perform the task below and your Facilitator/Assessor will assess your work based on the observation checklist attached.

Practical Assessment Task for PM-02-PS11: Plan and prepare for covering of prepared frames

Assessment Title:

Purpose

Guidelines to the Learner:

1. You will be presented with a range of samples or photographs of different wooden furniture products and accessories. These will include various styles, types, and uses.  Refer to Knowledge Module KM13 and the additional brochure with examples of upholstered furniture.

2. You are required to examine each sample or photograph and perform the following tasks:

Assessment Criteria:

Assessment Duration:

- The assessment should be completed within a specified time frame, typically 2-3 hours, to simulate a realistic working environment.

Feedback and Evaluation:

- Detailed feedback will be provided on the accuracy and relevance of your categorisations.

- The assessor will highlight areas of strength and suggest improvements where necessary.

Additional Notes:

- This practical assessment is designed to test not only your ability to recognise and categorise different furniture types and accessories but also your understanding of the broader context in which these items are used.

- It is essential to approach this task with attention to detail and a thorough understanding of the varied aspects of upholstered furniture design and manufacturing.

### Case Study: Efficient Upholstery Project at Classic Interiors

Scenario:

You are an upholsterer at Classic Interiors, tasked with overseeing the reupholstery of a set of vintage armchairs. Your responsibilities include interpreting the technical specifications, ensuring the availability of all required materials, and accurately measuring the chairs to prepare them for new upholstery.

Scenario-Based Questions:

Question 1: (IAC1101)

How would you interpret and apply the technical specifications provided for the reupholstery of the vintage armchairs?

Question 2: (IAC1102)

What steps would you take to ensure that all raw materials necessary for the project are available before beginning the reupholstery process?

Question 3: (IAC1103)

Describe how you would use measuring equipment to accurately measure the various areas of the armchair frame for reupholstery.

Marking Memo:

| Criteria | Exemplary (3 points) | Satisfactory (2 points) | Needs Improvement (1 point) |
| --- | --- | --- | --- |
| Technical Specifications | Demonstrates a thorough understanding of the specifications and translates them into a detailed work plan. | Generally understands specifications but may overlook some details in the planning phase. | Misinterprets specifications, leading to potential issues in project execution. |
| Material Availability | Proactively checks and secures all materials before the project begins, ensuring no delays in production. | Checks materials but may not plan adequately for shortages or delays. | Fails to secure necessary materials on time, causing delays. |
| Accuracy in Measurement | Measurements are precise and recorded meticulously, leading to flawless fit and finish of the upholstery. | Measurements are mostly accurate but may require minor adjustments during the project. | Inaccurate measurements lead to significant issues in fitting and waste of material. |

Through this case study, learners will demonstrate their capability to manage an upholstery project from planning through execution, focusing on the precision and foresight required to ensure project success. Mastery of these skills is crucial for upholding quality standards and efficient project management in the upholstery industry.

EVALUATION CHECKLIST

| NAME: …………………………………….…. SURNAME: ……………………………….…. ID…………………………………………….... | LEARNER SIGNATURE DATE: |
| --- | --- |

| EVALUATION CRITERIONS | MET REQUIREMENTS | DID NOT MEET REQUIREMENTS | COMMENTS OR ACTION REQUIRED |
| --- | --- | --- | --- |
| Ability of the learner to: |  |  |  |

| GENERAL COMMENTS OF OBSERVER: |  |  |
| --- | --- | --- |
| Assessor Name ……………………………… | Signature ……………………………………. | Date …………………………………. |

## PM-02-PS12: Cover a range of drop-on and drop-in seats of various shapes and sizes

Scope of Practical Skill

Given a range of prepared frames with square, round, and clover leaf shaped seats, a variety of prepared covers with plain, striped, vinyl, patterned tapestry, floral, and checked fabric, and a staple gun and consumables the learner must be able to:

- PA1201 Identify and match the covers to the frames

- PA1202 Use a staple gun to temporarily attach shaped covers with plain, striped, vinyl, patterned, and checked patterns to square, round and clover shaped drop on and drop in seat frames

- PA1203 Use the correct hand straining techniques to ensure that the main thread lines from left to right, and back to front, are straight.

- PA1204 Strain the covers on square seats to the sides and corners and, when correctly positioned, add tension and staple home

- PA1205 Strain the covers on round seats from the centre of the circle so that the tension is equally matched in all directions. Staple the cover onto the frame

- PA1206 Make relief cuts so that the strains to each side of the clover shape don’t drag the threads over from the opposite leaf

- PA1207 Attach bottom cloth neatly so that all staples, fabric edges, and threads are hidden.

Applied Knowledge

- AK1201 Covering techniques for drop on and drop in seats

- AK1202 Tacking and straining techniques

- AK1203 Fault finding techniques

Internal Assessment Criteria

- IAC1201 Cover components attached in the correct sequence for the design specification

- IAC1202 Thread lines of the fabric of the cover components are kept straight and evenly tensioned.

- IAC1203 Cover is stretched and strained over the frame using the magnetic tack hammer and tacks, pins, staple gun and staples, needles and thread to firmly secure the cover to the prepared frame

- IAC1204 Fabric tension is suitable and smooth, patterns match, pile runs in the correct direction, and there is no distortion of the seats.

### GUIDELINES

You are required to perform the task below and your Facilitator/Assessor will assess your work based on the observation checklist attached.

Practical Assessment Task for PM-02-PS12: Cover a range of drop-on and drop-in seats of various shapes and sizes

Assessment Title:

Purpose

Guidelines to the Learner:

1. You will be presented with a range of samples or photographs of different wooden furniture products and accessories. These will include various styles, types, and uses.  Refer to Knowledge Module KM13 and the additional brochure with examples of upholstered furniture.

2. You are required to examine each sample or photograph and perform the following tasks:

Assessment Criteria:

Assessment Duration:

- The assessment should be completed within a specified time frame, typically 2-3 hours, to simulate a realistic working environment.

Feedback and Evaluation:

- Detailed feedback will be provided on the accuracy and relevance of your categorisations.

- The assessor will highlight areas of strength and suggest improvements where necessary.

Additional Notes:

- This practical assessment is designed to test not only your ability to recognise and categorise different furniture types and accessories but also your understanding of the broader context in which these items are used.

- It is essential to approach this task with attention to detail and a thorough understanding of the varied aspects of upholstered furniture design and manufacturing.

### Case Study: Restoration of a Set of Antique Dining Chairs

Scenario:

You are tasked with restoring a set of six antique dining chairs that require complete reupholstery. The chairs are valuable both historically and monetarily, making precision and adherence to original design specifications crucial. The fabric selected is a high-quality damask with a distinct directional pattern, which must be handled carefully to maintain its aesthetic appeal.

Scenario-Based Questions:

Question 1: (IAC1201)

How would you ensure that the cover components are attached in the correct sequence according to the design specification?

Model Answer:

- Plan and Sequence: Before starting the upholstery process, review the design specification for each chair. Begin by attaching the seat cover, followed by the back, and then the side panels. This sequence will ensure that all components overlap correctly and maintain the integrity of the chair's original design.

- Documentation Review: Regularly consult the design specification documents during the upholstery process to verify that each step conforms to the outlined sequence, making adjustments as necessary.

Question 2: (IAC1202)

Describe how you would maintain straight and evenly tensioned thread lines of the fabric.

Question 3: (IAC1203)

Explain how you would use tools like the magnetic tack hammer, staple gun, and others to securely attach the cover to the frame.

Question 4: (IAC1204)

How would you ensure the fabric tension is suitable and smooth, the patterns match, the pile runs in the correct direction, and there is no distortion?

Marking Memo:

| Criteria | Points Possible | Criteria Met | Comments |
| --- | --- | --- | --- |
| Cover components sequence adherence | 25 | [score] | [comments] |
| Thread line straightness and tension | 25 | [score] | [comments] |
| Secure attachment of the cover | 25 | [score] | [comments] |
| Fabric tension and pattern alignment | 25 | [score] | [comments] |

Evaluation Notes: Each criterion is evaluated based on how closely the learner adheres to best practices and the specific instructions detailed in the design specifications. Points are awarded for precision, technique, and the ability to solve problems that arise during the upholstery process.

This case study challenges learners to apply their skills in a real-world scenario that demands high levels of craftsmanship and attention to detail, crucial for success in professional upholstery projects.

EVALUATION CHECKLIST

| NAME: …………………………………….…. SURNAME: ……………………………….…. ID…………………………………………….... | LEARNER SIGNATURE DATE: |
| --- | --- |

| EVALUATION CRITERIONS | MET REQUIREMENTS | DID NOT MEET REQUIREMENTS | COMMENTS OR ACTION REQUIRED |
| --- | --- | --- | --- |
| Ability of the learner to: |  |  |  |

| GENERAL COMMENTS OF OBSERVER: |  |  |
| --- | --- | --- |
| Assessor Name ……………………………… | Signature ……………………………………. | Date …………………………………. |

## PM-02-PS13: Cover wedge shaped box arm seat frames, with and without corner pleats

Scope of Practical Skill

In the specialised field of upholstery, mastering the techniques for covering complex furniture shapes is essential. PM-02-PS13 focuses on the skills required to upholster wedge-shaped box arm seat frames, a common yet challenging design found in modern and contemporary furniture. This module is designed to equip learners with the ability to handle various materials and styles, specifically focusing on vinyl and checked fabrics, each presenting unique challenges in terms of alignment and finishing.

Learning Objectives

Learners will engage with a set of specific tasks that will enhance their proficiency in upholstery, preparing them to meet the demands of both aesthetic and functional aspects of furniture design:

- PA1301: Identify and Match the Fabric Covers to the Specified Frames

- Learners will develop the ability to discern which types of fabrics best complement different frame designs, focusing on the physical and visual requirements of wedge-shaped box arm seat frames.

- PA1302: Apply Vinyl and Checked Fabrics to Wedge Shaped Seats

- This objective involves the application of vinyl fabric to a frame without corner pleats and a checked fabric to a frame with equal single corner pleats, requiring precision in handling and aesthetics.

- PA1303: Positioning Checked Fabric

- Learners will practice positioning checked fabric so that the pattern starts two-thirds of the way up the back of the seat. This technique is crucial for compensating for visual distortion when the seat is viewed from above.

- PA1304: Sequential Fabric Attachment Using a Staple Gun

- Training will include using a staple gun to temporarily attach fabric covers in a specific sequence that ensures optimal tension and alignment. The sequence will cover all parts of the frame from the inside arms to the bottom cover cloth.

- PA1305: Hand Straining Techniques

- Learners will use hand straining techniques to ensure that the main thread lines of the fabric are straight across the frame, an essential skill for maintaining the fabric’s aesthetic integrity.

- PA1306: Final Straining and Stapling

- The final objective focuses on straining the fabric to the sides and corners of the seat frame. Learners will practice adding tension and securing the fabric permanently, ensuring a smooth and tailored finish.

Practical Implementation

Each session in this module is designed to provide hands-on experience with both tools and materials, guiding learners through the complexities of upholstery work specific to wedge-shaped seats. By mastering these techniques, learners will be able to enhance the functional longevity and visual appeal of the furniture they work on, meeting the high standards expected in professional upholstery projects.

### GUIDELINES

You are required to perform the task below and your Facilitator/Assessor will assess your work based on the observation checklist attached.

Practical Assessment Task for PM-02-PS13: Cover wedge shaped box arm seat frames, with and without corner pleats

Assessment Title:

Purpose

Guidelines to the Learner:

1. You will be presented with a range of samples or photographs of different wooden furniture products and accessories. These will include various styles, types, and uses.  Refer to Knowledge Module KM13 and the additional brochure with examples of upholstered furniture.

2. You are required to examine each sample or photograph and perform the following tasks:

Assessment Criteria:

Assessment Duration:

- The assessment should be completed within a specified time frame, typically 2-3 hours, to simulate a realistic working environment.

Feedback and Evaluation:

- Detailed feedback will be provided on the accuracy and relevance of your categorisations.

- The assessor will highlight areas of strength and suggest improvements where necessary.

Additional Notes:

- This practical assessment is designed to test not only your ability to recognise and categorise different furniture types and accessories but also your understanding of the broader context in which these items are used.

- It is essential to approach this task with attention to detail and a thorough understanding of the varied aspects of upholstered furniture design and manufacturing.

### Case Study: Reupholstery of a Wedge-Shaped Box Arm Sofa

Scenario:

You are tasked with reupholstering a classic wedge-shaped box arm sofa for a boutique hotel lobby. The design specification requires the use of a high-end checked fabric for the main body and a durable vinyl for the seating surfaces. The project's success depends on meticulous attention to fabric handling, alignment, and attachment techniques to ensure the final product is both visually appealing and structurally sound.

Scenario-Based Questions:

Question 1: (IAC1301 and IAC1305)

Describe how you would ensure that cover components are attached in the correct sequence according to the design specifications.

Question 2: (IAC1302)

How would you ensure that the thread lines of the fabric are kept straight and evenly tensioned?

Question 3: (IAC1303)

Explain your approach to stretching and straining the cover over the frame using tools effectively.

Question 4: (IAC1304)

What steps would you take to ensure the fabric tension is suitable, the patterns match, the pile runs in the correct direction, and there is no distortion of the seats?

Marking Memo:

| Criteria | Exemplary (3 points) | Satisfactory (2 points) | Needs Improvement (1 point) |
| --- | --- | --- | --- |
| Sequence of Attachment | Correctly follows the detailed sequence as specified, verifying each step against design documents. | Sequence mostly followed but with minor deviations that do not affect the overall quality. | Fails to adhere to the specified sequence, resulting in potential rework or quality issues. |
| Thread Line Alignment and Tension | Fabric is perfectly aligned; tension is even across all surfaces with no misalignments. | Minor misalignment or uneven tension, but not significantly impacting overall aesthetics. | Significant issues with alignment or tension, affecting the functionality or appearance of the sofa. |
| Use of Tools | Tools used expertly to ensure fabric is securely and evenly attached without damage. | Competent use of tools, though some areas may require re-securing or adjustments. | Incorrect or inefficient use of tools, leading to potential damage or loose fabric. |
| Fabric Handling | Fabric pattern matches perfectly at seams; correct pile direction and no distortion. | Mostly good pattern matching and pile direction; minor distortion that can be corrected. | Poor pattern matching, incorrect pile direction, and noticeable distortion of fabric. |

This case study assesses the learner's ability to integrate technical skills with a detailed understanding of material properties and design requirements, crucial for high-end upholstery projects where precision directly impacts the final product's quality and durability.

EVALUATION CHECKLIST

| NAME: …………………………………….…. SURNAME: ……………………………….…. ID…………………………………………….... | LEARNER SIGNATURE DATE: |
| --- | --- |

| EVALUATION CRITERIONS | MET REQUIREMENTS | DID NOT MEET REQUIREMENTS | COMMENTS OR ACTION REQUIRED |
| --- | --- | --- | --- |
| Ability of the learner to: |  |  |  |

| GENERAL COMMENTS OF OBSERVER: |  |  |
| --- | --- | --- |
| Assessor Name ……………………………… | Signature ……………………………………. | Date …………………………………. |

## PM-02-PS14: Fit covers to prepared frames using visible and concealed finishing techniques

Scope of Practical Skill

Upholstery is not only about comfort and durability but also about the beauty and precision of the finishing touches. PM-02-PS14 focuses on the essential skills needed to fit covers to furniture frames using both visible and concealed finishing techniques, crucial for achieving a polished and professional look. Learners will engage with a comprehensive set of tools and materials that allow for a variety of finishing styles suited to different design specifications and customer preferences.

Learning Objectives

This module aims to equip learners with advanced upholstery techniques, enabling them to execute both aesthetic and functional finishes on a variety of furniture pieces:

- PA1401: Interpret Specifications and Identify Finishing Methods

- Learners will analyse project specifications to determine whether visible or concealed finishing methods are preferred, based on design requirements and client expectations.

- PA1402: Apply Concealed Finishing Techniques

- The focus will be on mastering techniques such as slip stitching, blind tacking, cardboard back tacking, pli-grip, and cross trim. These methods ensure that staples and fabric cuts are effectively hidden, enhancing the item's overall elegance and craftsmanship.

- PA1403: Implement Visible Finishing Methods

- Training will include the application of visible finishing elements like gimp, upholstery studs, and banding, which not only conceal fastenings but also serve as decorative details. Learners will practice techniques on both straight and contoured edges, learning to choose the appropriate method based on the piece's design and the location of the panel.

- PA1404: Attachment of Decorative Elements

- Participants will attach various decorative elements such as gimp pins, upholstery studs, and continuous studding strips to straight edges of fabric and vinyl panels. This process requires precision to ensure that all elements are aligned and contribute to the overall design.

- PA1405: Application of Finishes to Contoured Shapes

- This objective involves applying finishing details to contoured shapes, a more challenging task that demands careful handling and placement to ensure that the final product looks seamless and professionally executed.

Applied Knowledge Areas

- AK1401: Cover Fitting Techniques

- Gain a deep understanding of different cover fitting methods and when to apply each technique to meet specific functional and aesthetic needs.

- AK1402: Aesthetics

- Learn how to enhance the visual appeal of upholstered furniture through the strategic use of finishing techniques that complement the furniture’s style and the decor of the space where it will be placed.

Practical Implementation

This module will combine theoretical learning with hands-on practice, allowing learners to apply their skills on actual furniture items. By working with a variety of materials and tools, participants will develop the versatility and precision needed to excel in the field of upholstery. Practical assessments will challenge learners to demonstrate their ability to interpret specifications accurately and execute finishing techniques that meet high standards of quality and aesthetics.

### GUIDELINES

You are required to perform the task below and your Facilitator/Assessor will assess your work based on the observation checklist attached.

Practical Assessment Task for PM-02-PS14: Fit covers to prepared frames using visible and concealed finishing techniques

Assessment Title:

Purpose

Guidelines to the Learner:

1. You will be presented with a range of samples or photographs of different wooden furniture products and accessories. These will include various styles, types, and uses.  Refer to Knowledge Module KM13 and the additional brochure with examples of upholstered furniture.

2. You are required to examine each sample or photograph and perform the following tasks:

Assessment Criteria:

Assessment Duration:

- The assessment should be completed within a specified time frame, typically 2-3 hours, to simulate a realistic working environment.

Feedback and Evaluation:

- Detailed feedback will be provided on the accuracy and relevance of your categorisations.

- The assessor will highlight areas of strength and suggest improvements where necessary.

Additional Notes:

- This practical assessment is designed to test not only your ability to recognise and categorise different furniture types and accessories but also your understanding of the broader context in which these items are used.

- It is essential to approach this task with attention to detail and a thorough understanding of the varied aspects of upholstered furniture design and manufacturing.

### Case Study: Reupholstery of a Mid-Century Modern Armchair

Scenario:

You are an upholsterer tasked with restoring a mid-century modern armchair that has significant sentimental value to a client. The armchair requires a full upholstery refresh while maintaining its vintage aesthetic. The fabric chosen is a high-end, durable tweed, and the design specifications call for a mix of visible and concealed finishing techniques to enhance its mid-century modern appearance.

Scenario-Based Questions:

Question 1: (IAC1401)

How would you identify the most suitable finishing methods to attach the selected tweed fabric to various panels of the armchair?

Question 2: (IAC1402)

Describe how you would ensure that the covered upholstered items meet the specification and are visually pleasing.

Marking Memo:

| Criteria | Points Possible | Criteria Met | Comments |
| --- | --- | --- | --- |
| Finishing Methods Suitability | 50 | [score] | Evaluate the appropriateness of selected finishing methods. |
| Specification and Visual Appeal | 50 | [score] | Assess the execution's accuracy and the overall visual appeal. |

Evaluation Notes:

- Each criterion is scored out of 50 points. For full marks, the learner must demonstrate a comprehensive understanding of the fabric's properties and the armchair's design requirements, selecting and applying finishing techniques that are both appropriate for the fabric type and design era, and executed flawlessly.

- Deductions may occur for any misalignment, improper use of finishing techniques, or deviations from the design specifications that affect the functionality or aesthetic quality of the armchair.

This case study tests the learner's ability to integrate technical skills with an understanding of historical design aesthetics, ensuring that the reupholstered armchair not only meets the functional needs but also retains its sentimental and design value.

EVALUATION CHECKLIST

| NAME: …………………………………….…. SURNAME: ……………………………….…. ID…………………………………………….... | LEARNER SIGNATURE DATE: |
| --- | --- |

| EVALUATION CRITERIONS | MET REQUIREMENTS | DID NOT MEET REQUIREMENTS | COMMENTS OR ACTION REQUIRED |
| --- | --- | --- | --- |
| Ability of the learner to: |  |  |  |

| GENERAL COMMENTS OF OBSERVER: |  |  |
| --- | --- | --- |
| Assessor Name ……………………………… | Signature ……………………………………. | Date …………………………………. |

## PM-02-PS15: Fit covers to prepared frames using relief cuts to relieve tension on the fabric and to achieve the specified finish

Scope of Practical Skill

Upholstery is not just a craft but an art that requires precision and attention to detail, especially when working with complex furniture shapes that require fabric to be manipulated into tight or curved spaces. PM-02-PS15 focuses on the specialised skills needed to fit covers to prepared frames using relief cuts—a technique essential for reducing fabric tension and achieving a flawless finish. This module equips learners with the expertise to handle challenging upholstery scenarios, enhancing their ability to deliver high-quality, durable furniture upholstery.

Learning Objectives

This module will guide learners through various techniques for applying fabric to furniture frames with complex contours, ensuring that each piece not only meets functional needs but also achieves aesthetic excellence:

- PA1501: Applying Fabric to Inside Arm Panels on Scroll Arm Shapes

- Learners will practice the technique of making precise relief cuts to fit fabric smoothly over the curved surfaces of scroll armchairs. This task requires careful planning and execution to maintain the integrity of the fabric while conforming to the arm's intricate shape.

- PA1502: Applying Patterned Fabric to an Upholstered Child’s Chair

- The focus here is on correctly aligning and fitting patterned fabric on a smaller scale, such as a child’s chair, which may have playful, irregular shapes. Relief cuts will be used to ensure the fabric covers all areas evenly without distorting the pattern, especially around corners and edges.

- PA1503: Applying Fabric to an Upholstered Footstool Around the Legs

- This objective teaches learners how to strategically make relief cuts around the legs of a footstool to ensure the fabric lays flat and remains secure, without bunching or sagging, which is crucial for both appearance and durability.

Applied Knowledge

- AK1501: Fault Finding and Quality Control

- As learners work through the practical applications, they will also be trained in fault finding and quality control techniques. This knowledge is critical to ensuring that every piece of upholstered furniture not only looks great but is also free from defects that could affect its longevity and comfort.

Practical Implementation

Through hands-on practice and critical evaluation, learners will be exposed to a variety of furniture types and fabric materials, challenging them to adapt their techniques to meet specific design requirements and customer expectations. They will learn how to:

- Evaluate the needs of each piece of furniture based on its design and usage.

- Choose the appropriate type of relief cuts based on the furniture shape and fabric type.

- Implement these cuts effectively to enhance the fit and finish of the upholstery.

- Conduct thorough inspections and make adjustments as necessary to meet high-quality standards.

Conclusion

By mastering the skills taught in PM-02-PS15, learners will enhance their upholstery capabilities, allowing them to tackle complex projects with confidence. The ability to make effective relief cuts and adjust fabric tension appropriately is essential for achieving professional-level results that are both visually appealing and functionally sound.

### GUIDELINES

You are required to perform the task below and your Facilitator/Assessor will assess your work based on the observation checklist attached.

Practical Assessment Task for PM-02-PS15: Fit covers to prepared frames using relief cuts to relieve tension on the fabric and to achieve the specified finish

Assessment Title:

Purpose

Guidelines to the Learner:

1. You will be presented with a range of samples or photographs of different wooden furniture products and accessories. These will include various styles, types, and uses.  Refer to Knowledge Module KM13 and the additional brochure with examples of upholstered furniture.

2. You are required to examine each sample or photograph and perform the following tasks:

Assessment Criteria:

Assessment Duration:

- The assessment should be completed within a specified time frame, typically 2-3 hours, to simulate a realistic working environment.

Feedback and Evaluation:

- Detailed feedback will be provided on the accuracy and relevance of your categorisations.

- The assessor will highlight areas of strength and suggest improvements where necessary.

Additional Notes:

- This practical assessment is designed to test not only your ability to recognise and categorise different furniture types and accessories but also your understanding of the broader context in which these items are used.

- It is essential to approach this task with attention to detail and a thorough understanding of the varied aspects of upholstered furniture design and manufacturing.

### Case Study: Upholstering a Vintage Scroll Arm Chair

### Case Study: Upholstering a Vintage Scroll Arm Chair

Scenario:

You have been assigned the task of upholstering a vintage scroll armchair. The design specifications require that the fabric tension be uniform, pleats on the arm radius curve be even, and there are no drags or wrinkles on the straight edges. The chosen fabric is a patterned damask that needs precise handling to ensure that the pattern is aligned and undistorted.

Scenario-Based Questions:

Question 1: (IAC1501)

How would you ensure that the upholstered item meets the specification and achieves suitable fabric tension, even pleats on the arm radius curve, and a finish without drags or wrinkles on the straight edges?

2. Fabric Cutting and Alignment:

3. Applying Relief Cuts:

4. Attaching the Fabric:

5. Creating Even Pleats:

6. Final Adjustments and Quality Control:

Marking Memo:

| Criteria | Points Possible | Criteria Met | Comments |
| --- | --- | --- | --- |
| Suitable Fabric Tension | 25 | [score] | Assess if the fabric is uniformly taut without being overstretched. |
| Even Pleats on Arm Radius Curve | 25 | [score] | Evaluate the consistency and evenness of the pleats around the curve. |
| Finish Without Drags or Wrinkles | 25 | [score] | Inspect for smooth, wrinkle-free fabric on straight edges. |
| Overall Adherence to Specifications | 25 | [score] | Review the final product against the design specifications. |

Evaluation Notes:

- Suitable Fabric Tension: Full points are awarded if the fabric is uniformly taut without over-tightening. Deduct points for any areas where the fabric appears loose or overly stretched.

- Even Pleats on Arm Radius Curve: Full points are awarded if the pleats are consistent in size and spacing. Deduct points for any pleats that are uneven or misaligned.

- Finish Without Drags or Wrinkles: Full points are awarded for smooth, wrinkle-free fabric on straight edges. Deduct points for any visible drags or wrinkles.

- Overall Adherence to Specifications: Full points are awarded if the final product meets all design specifications. Deduct points for any deviations that impact the appearance or functionality of the chair.

This case study assesses the learner's ability to integrate technical skills with an understanding of fabric properties and design aesthetics. Mastery of these skills is crucial for ensuring high-quality results in professional upholstery projects.

EVALUATION CHECKLIST

| NAME: …………………………………….…. SURNAME: ……………………………….…. ID…………………………………………….... | LEARNER SIGNATURE DATE: |
| --- | --- |

| EVALUATION CRITERIONS | MET REQUIREMENTS | DID NOT MEET REQUIREMENTS | COMMENTS OR ACTION REQUIRED |
| --- | --- | --- | --- |
| Ability of the learner to: |  |  |  |

| GENERAL COMMENTS OF OBSERVER: |  |  |
| --- | --- | --- |
| Assessor Name ……………………………… | Signature ……………………………………. | Date …………………………………. |

## PM-02-PS16: Identify and inspect covered frames against specifications and repair and report defects ensuring quality

Scope of Practical Skill

Ensuring the quality and durability of upholstered furniture requires meticulous inspection and attention to detail at every stage of the process. PM-02-PS16 focuses on the critical skills needed to identify and inspect frames, both before and after covering, against product specifications. Learners will be trained to detect defects, assess the quality of materials and workmanship, and perform necessary repairs to meet high standards of quality and customer satisfaction.

Learning Objectives

This module aims to equip learners with comprehensive inspection techniques and quality control measures to ensure that upholstered furniture meets the required specifications and industry standards:

- PA1601: Inspect Sprung and Webbed Frames for Faults and Defects

- Learners will develop the ability to identify common issues in frames such as bad joints, knots in the wood, and protruding staples. These defects can compromise the integrity and longevity of the furniture.

- PA1602: Inspect Foam for Correct Densities and Bonding

- Training will include checking foam for proper density, sise, shape, profile, and bonding. Ensuring the foam meets these criteria is essential for the comfort and durability of the upholstered item.

- PA1603: Inspect Prepared Frames for Quality of Padding and Foam Positioning

- Learners will inspect the positioning and appearance of padding and foam, ensuring they are correctly aligned and centered according to the specifications. This includes verifying that the divisions of the couch are accurate.

- PA1604: Inspect Covered Frames for Quality

- This objective focuses on assessing the stretch, tension, and deflection of the fabric, the spacing of tacks, nails, and staples, and the uniformity of components such as arms and wings. Proper pleating, folding, buttoning, cover positioning, and pattern matching are also evaluated.

- PA1605: Inspect Covered Frame for Correct Buttoning

- Learners will ensure that buttoning is correctly executed, with buttons pulled to the specified depth to achieve the desired look and function.

- PA1606: Inspect Covered Frame for Correct Shaping and Tension

- This involves checking that the edges and corners are shaped correctly and that the fabric tension is even across the entire frame, preventing sagging or misalignment.

Applied Knowledge Areas

- AK1601: Fault Finding and Quality Control

- Develop skills to systematically identify faults and defects in frames and upholstery, understanding the impact of these issues on the overall quality and durability of the furniture.

- AK1602: Repair Techniques and Reporting

- Learn effective repair techniques for addressing identified defects and how to document and report these issues to maintain a high standard of quality control.

Practical Implementation

Through hands-on practice and critical evaluation, learners will gain the skills necessary to conduct thorough inspections and quality assessments. They will learn to:

- Apply detailed inspection techniques to identify a wide range of defects.

- Utilise appropriate tools and methods for repairing identified faults.

- Maintain detailed records of inspections and repairs to ensure continuous quality improvement.

Conclusion

By mastering the skills taught in PM-02-PS16, learners will be well-equipped to ensure that all upholstered furniture meets stringent quality standards. This ability to identify, inspect, and repair defects is crucial for maintaining the integrity, durability, and aesthetic appeal of the final product, thereby ensuring customer satisfaction and upholding the reputation of the upholstery business.

### GUIDELINES

You are required to perform the task below and your Facilitator/Assessor will assess your work based on the observation checklist attached.

Practical Assessment Task for PM-02-PS16: Identify and inspect covered frames against specifications and repair and report defects ensuring quality

Assessment Title:

Purpose

Guidelines to the Learner:

1. You will be presented with a range of samples or photographs of different wooden furniture products and accessories. These will include various styles, types, and uses.  Refer to Knowledge Module KM13 and the additional brochure with examples of upholstered furniture.

2. You are required to examine each sample or photograph and perform the following tasks:

Assessment Criteria:

Assessment Duration:

- The assessment should be completed within a specified time frame, typically 2-3 hours, to simulate a realistic working environment.

Feedback and Evaluation:

- Detailed feedback will be provided on the accuracy and relevance of your categorisations.

- The assessor will highlight areas of strength and suggest improvements where necessary.

Additional Notes:

- This practical assessment is designed to test not only your ability to recognise and categorise different furniture types and accessories but also your understanding of the broader context in which these items are used.

- It is essential to approach this task with attention to detail and a thorough understanding of the varied aspects of upholstered furniture design and manufacturing.

### Case Study: Quality Inspection and Repair of Upholstered Furniture Frames

Scenario:

You are an upholsterer at a high-end furniture manufacturing company. You have received a batch of frames that need to be inspected for defects before they can be upholstered. The frames include a mix of sprung and webbed structures intended for sofas and armchairs. Your task is to inspect these frames meticulously, identify and record any defects, follow workplace instructions to report these defects, and ensure the frames comply with product and workplace specifications. Additionally, you need to use a rasp to smooth any sharp edges that might damage the upholstery fabric.

Scenario-Based Questions:

Question 1: (IAC1601)

How would you inspect the frames according to product specifications, and what steps would you take to identify and record defects?

Question 2: (IAC1602)

Explain how you would follow workplace instructions and procedures to report the specified defects.

Question 3: (IAC1603)

How would you use a rasp to remove sharp edges from the frame, and why is this step important?

Question 4: (IAC1604)

After inspecting and repairing the frames, how would you ensure they comply with product and workplace process specifications?

| Criteria | Points Possible | Criteria Met | Comments |
| --- | --- | --- | --- |
| Inspection and Identification of Defects (IAC1601) | 25 | [score] | Evaluate the thoroughness and accuracy of the inspection process. |
| Following Reporting Procedures (IAC1602) | 25 | [score] | Assess the adherence to workplace procedures for reporting defects. |
| Using a Rasp to Remove Sharp Edges (IAC1603) | 25 | [score] | Evaluate the effectiveness and safety of using the rasp to smooth edges. |
| Ensuring Compliance with Specifications (IAC1604) | 25 | [score] | Assess the final inspection and verification process for compliance with specifications. |

Marking Memo:

Evaluation Notes:

- Inspection and Identification of Defects: Full points are awarded for a comprehensive and accurate inspection, with all defects properly identified and recorded. Deductions are made for missed defects or incomplete records.

- Following Reporting Procedures: Full points are awarded for correctly following all reporting procedures and accurately documenting defects. Deductions are made for incomplete or incorrect reports.

- Using a Rasp to Remove Sharp Edges: Full points are awarded for effectively using the rasp to remove sharp edges without causing new defects. Deductions are made for improper technique or safety violations.

- Ensuring Compliance with Specifications: Full points are awarded for a thorough final inspection that ensures all frames meet specifications. Deductions are made for any non-compliance or overlooked issues.

This case study assesses the learner's ability to integrate inspection, repair, and quality control skills, which are essential for maintaining high standards in upholstered furniture production.

EVALUATION CHECKLIST

| NAME: …………………………………….…. SURNAME: ……………………………….…. ID…………………………………………….... | LEARNER SIGNATURE DATE: |
| --- | --- |

| EVALUATION CRITERIONS | MET REQUIREMENTS | DID NOT MEET REQUIREMENTS | COMMENTS OR ACTION REQUIRED |
| --- | --- | --- | --- |
| Ability of the learner to: |  |  |  |

| GENERAL COMMENTS OF OBSERVER: |  |  |
| --- | --- | --- |
| Assessor Name ……………………………… | Signature ……………………………………. | Date …………………………………. |

## PM-02-PS17: Apply health and safety, housekeeping, ergonomics and productive processes when fitting covers to upholstery frames

Scope of Practical Skill

Ensuring a safe and efficient working environment is paramount in any upholstery workshop. PM-02-PS17 focuses on integrating health and safety practices, effective housekeeping, ergonomic principles, and productive processes when fitting covers to upholstery frames. This module equips learners with the necessary skills to maintain a safe, organised, and productive workspace, thereby enhancing both the quality of their work and their personal well-being.

Learning Objectives

This module aims to provide learners with comprehensive knowledge and practical skills to apply health and safety measures, maintain good housekeeping, and adopt ergonomic practices in the upholstery workshop. Learners will also learn how to set up their workstations to maximise productivity and minimise the risk of injury.

- PA1701: Identify Aisles and Stacking Areas, Electricity Isolation Switches

- Learners will be trained to recognise the designated aisles and stacking areas in the workshop to ensure safe movement and storage of materials. They will also learn to locate and understand the use of electricity isolation switches for emergency purposes.

- PA1702: Identify Emergency Exits and Gathering Places

- This objective focuses on ensuring that learners can quickly and accurately locate emergency exits and designated gathering places in case of an evacuation.

- PA1703: Identify Safety Equipment in the Workplace

- Learners will identify the location and use of safety equipment such as first aid boxes, emergency alarms, fire extinguishers, fire hose reels, and overhead sprinkler systems, enhancing their preparedness for emergency situations.

- PA1704: Identify and Report Hazards and Unsafe Acts

- Training will include recognising and reporting health and safety hazards, and unsafe acts to designated safety personnel like first aiders, fire officers, and SHE (Safety, Health, and Environment) representatives.

- PA1705: Select and Use Appropriate Personal Protective Equipment (PPE)

- Learners will be trained on selecting and correctly using PPE, including ear muffs, ear plugs, respirators, dust masks, goggles, and gloves, to protect themselves from workplace hazards.

- PA1706: Position Raw Materials, Consumables, Tools, and Equipment Ergonomically

- This objective covers the organisation of the workstation to minimise repetitive movements and unnecessary stretching, lifting, bending, and reaching. Proper positioning of materials and tools helps in reducing physical strain and increasing efficiency.

Applied Knowledge

- AK1701: Occupational Health and Safety Act and its Regulations

- Gain a thorough understanding of the Occupational Health and Safety Act and its regulations, ensuring compliance with legal requirements and promoting a culture of safety in the workplace.

- AK1702: Ergonomics, Good Housekeeping, and Productivity

- Learn the principles of ergonomics to set up a workstation that supports good posture and reduces physical strain. Understand the importance of good housekeeping practices in maintaining an organised and efficient work environment, thereby enhancing overall productivity.

Practical Implementation

Through practical exercises and real-world applications, learners will develop the skills necessary to create and maintain a safe, organised, and productive workspace. They will learn to:

- Conduct regular safety inspections and promptly address any hazards or unsafe acts.

- Use personal protective equipment correctly and consistently.

- Arrange their workstations to promote ergonomic efficiency and reduce the risk of injury.

- Implement effective housekeeping practices to ensure a clean and organised workshop.

Conclusion

By mastering the skills taught in PM-02-PS17, learners will be well-equipped to uphold high standards of health and safety, housekeeping, and ergonomics in their upholstery work. This comprehensive approach not only enhances the quality and efficiency of their work but also ensures a safer and more sustainable working environment.

### GUIDELINES

You are required to perform the task below and your Facilitator/Assessor will assess your work based on the observation checklist attached.

Practical Assessment Task for PM-02-PS17: Apply health and safety, housekeeping, ergonomics and productive processes when fitting covers to upholstery frames

Assessment Title:

Purpose

Guidelines to the Learner:

1. You will be presented with a range of samples or photographs of different wooden furniture products and accessories. These will include various styles, types, and uses.  Refer to Knowledge Module KM13 and the additional brochure with examples of upholstered furniture.

2. You are required to examine each sample or photograph and perform the following tasks:

Assessment Criteria:

Assessment Duration:

- The assessment should be completed within a specified time frame, typically 2-3 hours, to simulate a realistic working environment.

Feedback and Evaluation:

- Detailed feedback will be provided on the accuracy and relevance of your categorisations.

- The assessor will highlight areas of strength and suggest improvements where necessary.

Additional Notes:

- This practical assessment is designed to test not only your ability to recognise and categorise different furniture types and accessories but also your understanding of the broader context in which these items are used.

- It is essential to approach this task with attention to detail and a thorough understanding of the varied aspects of upholstered furniture design and manufacturing.

### Case Study: Health and Safety, Housekeeping, Ergonomics, and Productivity in an Upholstery Workshop

Scenario:

You are an upholsterer in a busy workshop that specialises in high-end custom furniture. Your daily tasks involve fitting covers to various upholstery frames. Recently, the workshop has implemented new health and safety policies to improve overall safety and productivity. You are required to demonstrate your understanding and application of these policies by identifying and adhering to emergency procedures, reporting hazards, using personal protective equipment (PPE), and maintaining an organised and ergonomic workspace.

Scenario-Based Questions:

Question 1: (IAC1701)

How would you identify and obey emergency signs, symbols, safety equipment, safety personnel, and procedures in your workshop?

Question 2: (IAC1702)

Describe how you would identify and report hazards and unsafe acts in your workshop.

Question 3: (IAC1703)

Explain how you would use personal protective equipment (PPE) correctly at all times in the workshop.

Question 4: (IAC1704)

How would you handle and position materials in a manner that demonstrates understanding of good housekeeping, workplace ergonomics, and productivity?

Marking Memo:

| Criteria | Points Possible | Criteria Met | Comments |
| --- | --- | --- | --- |
| Identification and Obedience of Safety Procedures (IAC1701) | 25 | [score] | Assess the ability to recognise and follow emergency signs, symbols, equipment, and procedures. |
| Identification and Reporting of Hasards (IAC1702) | 25 | [score] | Evaluate the thoroughness in identifying and reporting hasards and unsafe acts. |
| Correct Use of PPE (IAC1703) | 25 | [score] | Assess the proper selection, use, and maintenance of PPE. |
| Material Handling and Positioning (IAC1704) | 25 | [score] | Evaluate the understanding and application of good housekeeping, ergonomics, and productivity principles. |

Evaluation Notes:

- Identification and Obedience of Safety Procedures: Full points are awarded for demonstrating a comprehensive understanding of emergency signs, symbols, safety equipment, and procedures. Deductions are made for any missed elements or incorrect actions.

- Identification and Reporting of Hazards: Full points are awarded for effectively identifying and accurately reporting hazards and unsafe acts. Deductions are made for any missed hazards or incomplete reports.

- Correct Use of PPE: Full points are awarded for consistently selecting and using the appropriate PPE correctly. Deductions are made for improper use or failure to use PPE.

- Material Handling and Positioning: Full points are awarded for demonstrating proper material handling techniques and maintaining an organised and ergonomic workstation. Deductions are made for unsafe handling practices or disorganised workspaces.

This case study assesses the learner's ability to integrate health and safety practices, housekeeping, ergonomics, and productivity in the upholstery workshop. Mastery of these skills is essential for maintaining a safe, efficient, and productive work environment.

EVALUATION CHECKLIST

| NAME: …………………………………….…. SURNAME: ……………………………….…. ID…………………………………………….... | LEARNER SIGNATURE DATE: |
| --- | --- |

| EVALUATION CRITERIONS | MET REQUIREMENTS | DID NOT MEET REQUIREMENTS | COMMENTS OR ACTION REQUIRED |
| --- | --- | --- | --- |
| Ability of the learner to: |  |  |  |

| GENERAL COMMENTS OF OBSERVER: |  |  |
| --- | --- | --- |
| Assessor Name ……………………………… | Signature ……………………………………. | Date …………………………………. |

## PM-02-PS18: Perform routine cleaning, quality change cleaning and housekeeping of the work area

Scope of Practical Skill

Maintaining a clean, organised, and well-maintained work environment is crucial in the upholstery industry. PM-02-PS18 focuses on the essential skills required for routine cleaning, quality change cleaning, and effective housekeeping of the work area. By ensuring that the workspace is safe and efficient, learners will be better equipped to produce high-quality work and maintain the longevity of their tools and equipment.

Learning Objectives

This module aims to provide learners with the practical skills needed to conduct thorough cleaning and maintenance of their work area and tools. These activities are essential for creating a productive and safe working environment.

- PA1801: Conduct General Cleaning of the Work Area

- Learners will be trained to perform general cleaning tasks to ensure that the workspace is free from debris and hazards, promoting safety and efficiency.

- PA1802: Clean Tools and Equipment

- This objective focuses on removing dust, dirt, and other contaminants from tools and equipment to maintain their functionality and extend their lifespan.

- PA1803: Sharpen or Replace Blunted Edges and Perform Minor Maintenance

- Learners will practice sharpening or replacing blunted edges on tools and performing minor maintenance tasks to keep equipment in optimal working condition.

- PA1804: Perform Dust Extraction Procedures

- Training will include the use of dust extraction systems to remove airborne particles and maintain a clean air environment, reducing health risks associated with dust inhalation.

- PA1805: Perform Basic Routine Maintenance of Tools and Equipment

- This objective covers routine maintenance tasks such as oiling moving parts, tightening loose screws, and handling tools and equipment correctly to prevent damage and ensure longevity.

- PA1806: Check Tools and Equipment for Working Condition

- Learners will verify that all tools and equipment are in proper working condition after cleaning and maintenance procedures are completed, ensuring readiness for use.

Applied Knowledge

- AK1801: Importance of Regular Cleaning and Maintenance

- Understand the significance of regular cleaning and maintenance in prolonging the life of tools and equipment, ensuring safety, and maintaining a high standard of workmanship.

- AK1802: Health and Safety Regulations

- Gain knowledge of health and safety regulations related to workplace cleanliness and equipment maintenance, ensuring compliance with industry standards.

Practical Implementation

Through hands-on practice, learners will develop the skills necessary to maintain a clean and organised work area. They will learn to:

- Perform daily cleaning tasks to keep the work area free from hazards and debris.

- Clean and maintain tools and equipment regularly to ensure they are in good working condition.

- Use dust extraction systems effectively to maintain a healthy work environment.

- Conduct minor maintenance tasks to prevent equipment failure and extend the lifespan of tools.

- Inspect tools and equipment to ensure they are ready for use after cleaning and maintenance.

Conclusion

By mastering the skills taught in PM-02-PS18, learners will be well-equipped to maintain a high standard of cleanliness and organisation in their work environment. These practices are essential for ensuring safety, efficiency, and the production of high-quality upholstered products.

### GUIDELINES

You are required to perform the task below and your Facilitator/Assessor will assess your work based on the observation checklist attached.

Practical Assessment Task for PM-02-PS18: Perform routine cleaning, quality change cleaning and housekeeping of the work area

Assessment Title: Performing a routine clean, quality change cleaning and housekeeping of the work area

Purpose: This assessment aims to evaluate the learner’s ability to perform routine cleaning, quality change cleaning and housekeeping of the work area

Guidelines to the Learner:

1. You will be presented with a range of samples or photographs of different wooden furniture products and accessories. These will include various styles, types, and uses.  Refer to Knowledge Module KM13 and the additional brochure with examples of upholstered furniture.

2. You are required to examine each sample or photograph and perform the following tasks:

- PA1801: Conduct General Cleaning of the Work Area

- PA1802: Clean Tools and Equipment

- PA1803: Sharpen or Replace Blunted Edges and Perform Minor Maintenance

- PA1804: Perform Dust Extraction Procedures

- PA1805: Perform Basic Routine Maintenance of Tools and Equipment

- PA1806: Check Tools and Equipment for Working Condition

Assessment Criteria:

- IAC1801 Different waste in the workshop is managed and disposed safely and according to company policies and regulation.

- IAC1802 Tools and equipment are clean and in good and safe working condition at all times

- IAC1803 Dust is removed regularly to avoid accumulation and the creation of fire hazards

- IAC1804 Waste is minimized during the operations and offcuts that can be reused are marked and neatly stored

Assessment Duration:

- The assessment should be completed within a specified time frame, typically 2-3 hours, to simulate a realistic working environment.

Feedback and Evaluation:

- Detailed feedback will be provided on the accuracy and relevance of your categorisations.

- The assessor will highlight areas of strength and suggest improvements where necessary.

Additional Notes:

- This practical assessment is designed to test not only your ability to recognise and categorise different furniture types and accessories but also your understanding of the broader context in which these items are used.

- It is essential to approach this task with attention to detail and a thorough understanding of the varied aspects of upholstered furniture design and manufacturing.

### Case Study: Maintaining a Clean and Safe Upholstery Workshop

Scenario:

You are responsible for maintaining cleanliness and safety in a busy upholstery workshop. The workshop handles a variety of materials, including fabrics, foams, and wood, which generate different types of waste. Your tasks include managing waste, ensuring tools and equipment are clean and in safe working condition, removing dust to prevent fire hazards, and minimising waste during operations. Additionally, you must identify and store reusable offcuts appropriately.

Scenario-Based Questions:

Question 1: (IAC1801)

How would you manage and dispose of different types of waste in the workshop safely and according to company policies and regulations?

Question 2: (IAC1802)

Describe how you would ensure that tools and equipment are clean and in good and safe working condition at all times.

Question 3: (IAC1803)

How would you ensure that dust is removed regularly to avoid accumulation and the creation of fire hazards?

Question 4: (IAC1804)

How would you minimise waste during operations and handle offcuts that can be reused?

Marking Memo:

| Criteria | Points Possible | Criteria Met | Comments |
| --- | --- | --- | --- |
| Waste Management and Disposal (IAC1801) | 25 | [score] | Assess the effectiveness of waste segregation and disposal procedures. |
| Clean and Safe Tools and Equipment (IAC1802) | 25 | [score] | Evaluate the cleanliness and maintenance of tools and equipment. |
| Dust Removal (IAC1803) | 25 | [score] | Assess the frequency and thoroughness of dust removal practices. |
| Waste Minimisation and Offcuts Handling (IAC1804) | 25 | [score] | Evaluate the methods used to minimise waste and handle reusable offcuts. |

Evaluation Notes:

- Waste Management and Disposal: Full points are awarded for proper segregation, safe disposal, and compliance with regulations. Deductions are made for improper disposal or non-compliance.

- Clean and Safe Tools and Equipment: Full points are awarded for consistently clean and well-maintained tools and equipment. Deductions are made for dirty or poorly maintained tools.

- Dust Removal: Full points are awarded for regular and thorough dust removal practices that prevent accumulation and fire hasards. Deductions are made for insufficient dust removal.

- Waste Minimisation and Offcuts Handling: Full points are awarded for effective waste minimisation practices and proper handling of reusable offcuts. Deductions are made for excessive waste or improper storage of offcuts.

This case study assesses the learner's ability to integrate cleaning, maintenance, and waste management practices to ensure a safe, efficient, and productive upholstery workshop. Mastery of these skills is essential for maintaining high standards of cleanliness and organisation in the workplace.

EVALUATION CHECKLIST

| NAME: …………………………………….…. SURNAME: ……………………………….…. ID…………………………………………….... | LEARNER SIGNATURE DATE: |
| --- | --- |

| EVALUATION CRITERIONS | MET REQUIREMENTS | DID NOT MEET REQUIREMENTS | COMMENTS OR ACTION REQUIRED |
| --- | --- | --- | --- |
| Ability of the learner to: |  |  |  |

| GENERAL COMMENTS OF OBSERVER: |  |  |
| --- | --- | --- |
| Assessor Name ……………………………… | Signature ……………………………………. | Date …………………………………. |

## PM-02-PS19: Record and report frame covering production information

Scope of Practical Skill

Accurate recording and reporting of production information are crucial for maintaining efficiency, quality control, and continuous improvement in the upholstery industry. PM-02-PS19 focuses on developing the skills necessary to document and report all relevant production data related to frame covering operations. This module equips learners with the ability to systematically record production metrics, identify and report issues, and contribute to the overall productivity and quality assurance processes.

Learning Objectives

This module aims to provide learners with comprehensive knowledge and practical skills to accurately record and report frame covering production information. By mastering these skills, learners will ensure that all production activities are documented, enabling effective monitoring and decision-making.

- PA1901: Identify the Type of Information to be Recorded in the Production Document

- Learners will understand the various types of information that need to be recorded, including production rates, downtime, quality faults, and production history.

- PA1902: Record Legibly the Hourly Production, Downtime, Quality Faults, and Production History

- This objective focuses on teaching learners how to document hourly production metrics, track downtime, note any quality faults, and maintain a comprehensive production history.

- PA1903: Record Any Production Related Problems Experienced During the Shift

- Learners will be trained to accurately record any issues or problems encountered during their shift, providing valuable data for troubleshooting and process improvement.

- PA1904: Accurately Record Waste

- This objective emphasises the importance of documenting waste generated during production, which is essential for managing resources and reducing costs.

- PA1905: Accurately Record Defects and Faults (Raw Material and Covered Frames)

- Learners will learn to accurately identify and record any defects or faults in raw materials and covered frames, contributing to quality control and assurance.

- PA1906: Accurately Record Production and Achieving Targets

- This objective covers the accurate recording of production output and the achievement of production targets, ensuring that performance metrics are met and documented.

Applied Knowledge

- AK1901: Importance of Accurate Record-Keeping

- Understand the significance of maintaining accurate and detailed production records for operational efficiency, quality control, and regulatory compliance.

- AK1902: Reporting Procedures and Documentation Standards

- Gain knowledge of standard reporting procedures and documentation practices within the industry, ensuring consistency and reliability in recorded data.

Practical Implementation

Through hands-on practice, learners will develop the skills necessary to effectively record and report production information. They will learn to:

- Identify and document all relevant production data accurately and legibly.

- Maintain detailed records of production activities, including downtime and quality faults.

- Report production-related problems promptly to facilitate quick resolution.

- Track and document waste, defects, and faults to support continuous improvement efforts.

- Ensure that production targets are met and accurately recorded for performance monitoring.

Conclusion

By mastering the skills taught in PM-02-PS19, learners will be equipped to contribute to the efficient operation and continuous improvement of the upholstery production process. Accurate recording and reporting are essential for maintaining high standards of quality, efficiency, and accountability in the workplace.

### GUIDELINES

You are required to perform the task below and your Facilitator/Assessor will assess your work based on the observation checklist attached.

Practical Assessment Task for PM-02-PS19: Record and report frame covering production information

Assessment Title: Recording and reporting frame covering production information

Purpose: This assessment aims to evaluate the learner’s ability to record and report frame covering production information

Guidelines to the Learner:

1. You will be presented with a range of samples or photographs of different wooden furniture products and accessories. These will include various styles, types, and uses.  Refer to Knowledge Module KM13 and the additional brochure with examples of upholstered furniture.

2. You are required to examine each sample or photograph and perform the following tasks:

- PA1901: Identify the Type of Information to be Recorded in the Production Document

- PA1902: Record Legibly the Hourly Production, Downtime, Quality Faults, and Production History

- PA1903: Record Any Production Related Problems Experienced During the Shift

- PA1904: Accurately Record Waste

- PA1905: Accurately Record Defects and Faults (Raw Material and Covered Frames)

- PA1906: Accurately Record Production and Achieving Targets

Resources Provided:

- A collection of samples or high-resolution photographs of various upholstered furniture products and accessories.

- Writing materials for note-taking and categorisation.

- Reference materials on furniture types, styles, and manufacturing materials (if necessary).

Assessment Criteria:

- IAC1901 - The relevant information during the production shift is recorded and reported via the correct reporting channels

- IAC1902 - Production problems experienced during shift are recorded correctly, neatly and legibly

Assessment Duration:

- The assessment should be completed within a specified time frame, typically 2-3 hours, to simulate a realistic working environment.

Feedback and Evaluation:

- Detailed feedback will be provided on the accuracy and relevance of your categorisations.

- The assessor will highlight areas of strength and suggest improvements where necessary.

Additional Notes:

- This practical assessment is designed to test not only your ability to recognise and categorise different furniture types and accessories but also your understanding of the broader context in which these items are used.

- It is essential to approach this task with attention to detail and a thorough understanding of the varied aspects of upholstered furniture design and manufacturing.

### Case Study: Recording and Reporting Frame Covering Production Information

Scenario:

You are working as an upholsterer in a production facility that manufactures high-end furniture. During your shift, you are responsible for covering frames with fabric and ensuring that all production activities are accurately recorded and reported. The facility uses a production logbook to track hourly production, downtime, quality faults, and any production-related problems. Today, your shift encountered several issues, including a temporary halt in production due to a machine malfunction and a batch of defective fabric. You need to document these events clearly and legibly in the production logbook and report them through the correct channels.

Scenario-Based Questions:

Question 1: (IAC1901)

How would you record and report the relevant information during your production shift via the correct reporting channels?

Question 2: (IAC1902)

Describe how you would record production problems experienced during your shift correctly, neatly, and legibly.

Marking Memo:

| Criteria | Points Possible | Criteria Met | Comments |
| --- | --- | --- | --- |
| Recording and Reporting Information (IAC1901) | 50 | [score] | Assess the accuracy, completeness, and legibility of recorded information and adherence to correct reporting channels. |
| Recording Production Problems (IAC1902) | 50 | [score] | Evaluate the clarity, neatness, and legibility of entries documenting production problems. |

Evaluation Notes:

- Recording and Reporting Information: Full points are awarded for accurately and comprehensively recording all relevant information during the shift and reporting it through the correct channels. Deductions are made for incomplete or unclear entries and failure to follow reporting procedures.

- Recording Production Problems: Full points are awarded for neatly and legibly recording all production problems, including detailed descriptions and accurate documentation of actions taken. Deductions are made for illegible writing, missing details, or poorly documented issues.

This case study assesses the learner's ability to effectively document and report production activities and problems, ensuring that all relevant information is accurately recorded and communicated. Mastery of these skills is essential for maintaining operational efficiency and addressing issues promptly in the production environment.

EVALUATION CHECKLIST

| NAME: …………………………………….…. SURNAME: ……………………………….…. ID…………………………………………….... | LEARNER SIGNATURE DATE: |
| --- | --- |

| EVALUATION CRITERIONS | MET REQUIREMENTS | DID NOT MEET REQUIREMENTS | COMMENTS OR ACTION REQUIRED |
| --- | --- | --- | --- |
| Ability of the learner to: |  |  |  |

| GENERAL COMMENTS OF OBSERVER: |  |  |
| --- | --- | --- |
| Assessor Name ……………………………… | Signature ……………………………………. | Date …………………………………. |